| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4635 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Seoul, 3 - 7 November 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Ericsson |  |  |  |
| Title: | H.248 model for SCTP streams |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution from Ericsson addresses the mapping of SCTP streams to H.248 streams and makes a proposal based on the use cases which are in scope of this Recommendation.

# Introduction

In the last SG16 meeting in Sapporo, Q3 had some initial considerations on how SCTP streams within one SCTP association may be mapped to H.248 streams. This contribution analyses the suitability of the different mapping alternatives depending on the type of user messages carried over SCTP and makes a proposal for this Recommendation.

# Discussion

One SCTP association may consist of multiple SCTP streams, up to 65535. SCTP was originally created for the transport of signalling protocols (such as BICC, ISUP, H.248, etc.) over IP. The transport of signalling protocols still represents today the most common case of usage of SCTP.

When SCTP is used for that purpose, it is common to make use of the multi-streaming capability of SCTP. In these cases, ordered delivery is normally only required for messages related to the same call or session. Those are therefore sent in the same SCTP stream. Signalling messages related to different calls or sessions don’t have such a strict requirement on ordered delivery and can be therefore distributed over different SCTP streams. This avoids or mitigates the head-of-line blocking problem, i.e. that the loss or delay of a single message may block or delay all traffic.

Recently new usage scenarios of SCTP have been described where SCTP is used for the transport of data. The best known is probably the “data channel” which is used by WebRTC and by CLUE. In these scenarios one data channel uses one single SCTP stream in each direction.

The mapping of SCTP streams within one SCTP association to H.248 streams could theoretically be done in different ways:

- All SCTP streams are mapped to the same H.248 stream

- Each SCTP stream is mapped to one H.248 stream within the same termination

- Each SCTP stream is mapped to one H.248 stream in a separate termination

The first type of mapping would seem to be appropriate in those cases where the traffic of one application or one single user part is load-shared among multiple streams as described above for the transport of signalling protocols.  The user part layer dynamically distributes the traffic over the streams.

The use of SCTP for the transport of signalling protocols is not considered today to be subject of H.248 control and no similar case of one application distributing traffic over multiple streams has been identified yet. Therefore it is proposed to leave this type of mapping for further studies until an appropriate use case is identified.

The second type of mapping seems appropriate for the data channel used in WebRTC and CLUE. CLUE specifies that only one data channel is used for one CLUE session whereas WebRTC allows for multiple data channels within the same WebRTC session. In this case, each data channel may be subject of different handling. For instance, one may be used to carry instant text messaging while the other is used to carry game control information. The separation into different H.248 streams facilitates the separate control actions by the MGC and MG as well as the interworking with other terminations in the H.248 context, in particular in WebRTC to non-WebRTC interworking scenarios.

Using one single termination is preferable, or even necessary, to allow for control actions on the SCTP association (e.g. initiate, terminate, etc.). For this reason the third type of mapping, a different termination per SCTP stream, is not proposed.

There is however a caveat with mapping nr. 2. The data channel framework, as specified in IETF, mandates to negotiate the maximum number of SCTP streams: 65,535 even though most likely only a few data channels will be established per WebRTC session and only one per CLUE session.

It would not be practical to create 65,536 H.248 streams when establishing an SCTP association for WebRTC or CLUE. Instead it is proposed that H.248 streams are created dynamically as needed.

# Conclusion

Based on the use cases with SCTP bearers that have been identified at this stage as being subject of H.248 control, it is proposed to use a separate H.248 stream for each SCTP stream within an SCTP association. In fact an H.248 stream will handle one SCTP stream in each direction, since the SCTP streams are uni-directional and the H.248 streams are, in general, bi-directional. All H.248 streams handling one SCTP endpoint of an SCTP association are part of the same H.248 termination.

It is proposed to leave a potential case of mapping all SCTP streams within an association to a single H.248 stream for further studies with a dependency on the identification of other use cases subject of H.248 control.

The concrete proposed changes are shown below.

[Begin Correction]

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (2013), Gateway control protocol: Version 3.

[ITU-T H.248.4]	Recommendation ITU-T H.248.4 (12/2009), Gateway contro protocol: Transport over Stream Control Transmission Protocol (SCTP).

[ITU-T H.248.67]	Recommendation ITU-T H.248.67 (12/2009), Gateway control protocol: Transport mode indication package.

[ITU-T H.248.93]	Recommendation ITU-T H.248.93 (07/2014), Gateway control protocol: H.248 support for control of transport security using DTLS.

[IETF RFC 4960]	IETF RFC 4960 (2007), Stream Control Transmission Protocol.

[IETF RFC 5061]	IETF RFC 5061 (2007), Stream Control Transmission Protocol (SCTP) Dynamic Address Reconfiguration.

[IETF RFC 6525]	IETF RFC 6525 (2012), Stream Control Transmission Protocol (SCTP) Stream Reconfiguration.

[IETF DRAFT WEBRTCHANN] draft-ietf-rtcweb-data-channel-12.txt. WebRTC Data Channels

[IETF DRAFT WEBRTCDCEPPROT] draft-ietf-rtcweb-data-protocol-08.txt. WebRTC Data Channel Establishment Protocol

{Editor's note (2013-10): following references might be needed

- draft-ietf-mmusic-sctp-sdp

- draft-ietf-tsvwg-sctp-dtls-encaps

- draft-marcon-msrp-over-webrtc-data-channels

- H.248.DTLS

- others

}

[End Correction]

[Begin Correction]

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| DCEP | Data Channel Establishment Protocol |
| --- | --- |
| DTLS | Datagram Transport Layer Security |
| IP | Internet Protocol |
| IPv4 | Internet Protocol Version 4 |
| IPv6 | Internet Protocol Version 6 |
| L3 | Layer three |
| L4 | Layer four |
| L4+ | Above layer four |
| MG | Media Gateway |
| MGC | Media Gateway Controller |
| SCTP | Stream Control Transmission Protocol |
| SDP | Session Description Protocol |
| SEP | Stream Endpoint |
| SEPP | Stream Endpoint Pair |
| SIP | Session Initiation Protocol |
| UDP | User Datagram Protocol |

[End Correction]

[Begin Correction]

#### 6.2.2.3	SCTP Associations in single-SCTP-Stream or multi-SCTP-Stream mode

The served user instance of SCTP Associations may use a single bidirectional SCTP Stream (i.e. a pair of uni-directional SCTP Streams) or multiple instances thereof (e.g., multiple WebRTC data channels). There might be correspondent use case variations.

[End Correction]

[Begin Correction]

Contributor’s note: existing 7.2 chapter in the draft would be renumbered to 7.3 if the proposal is accepted.

7.2 H.248 model for SCTP multi-stream

One SCTP association may consist of multiple SCTP streams, up to 65535. The mapping of SCTP streams within one SCTP association to H.248 streams could theoretically be done in different ways:

- All SCTP streams are mapped to the same H.248 stream

- Each SCTP stream is mapped to one H.248 stream within the same termination

- Each SCTP stream is mapped to one H.248 stream in a separate termination

7.2.1 One H.248 stream per SCTP stream

The use cases supported by this version of this Recommendation, WebRTC and CLUE, use a “data channel” which consists of one single SCTP stream in each direction.

WebRTC allows for multiple data channels within one WebRTC session, thus multiple SCTP streams in each direction over the same SCTP association. In the general case, these different data channels will carry different types of user messages. For example, one data channel may be used for instant text messaging and one for game controlling information. Therefore in the general case they may be subject of different processing by the MG and require different control actions by the MGC with H.248. They may also interwork differently with the other side of the session, for example having a different transport protocol stack mapping in the context.

CLUE only describes the use of one data channel per CLUE session.

To facilitate the different handling of different data channels within the H.248 context, this version of this Recommendation only considers the handling of one SCTP stream in each direction per H.248 stream. All H.248 streams related to the same end point of an SCTP association are part of the same termination.

7.2.1.1 SCTP streams vs. H.248 streams dynamics

An SCTP association may be created with a much higher number of SCTP streams than what will be actually needed. [IETF DRAFT WEBRTCHANN] recommends to negotiate the maximum number: 65,535 streams.

Creating such a number of H.248 streams would consume resources unnecessarily. Therefore, although this Recommendation does not restrict the number of H.248 streams to be created initially, i.e. at the time of establishment of the SCTP association, it is recommended that the MGC only creates H.248 streams for those SCTP streams that will start to send or receive user messages.

Contributor’s note: Since the reception of an incoming stream is in general out of the control of the endpoint, this presents an issue of when new SEPs within a Termination should be created. There can be multiple ways to resolve this.This contribution proposes one possible solution (in the Package updates) which is SCTP generic. There may be DCEP specific solutions which couple the creation of new streams with the sending or reception of the DCEP DATA_CHANNEL_OPEN and DATA_CHANNEL_ACK messages [IETF DRAFT WEBRTCDCEPPROT].

All H.248 SEPs corresponding to SCTP streams which are part of the same SCTP association belong to the same Termination. This allows for MGC control actions at SCTP association level, e.g. start an association.

Contributor’s note: this clause (7.2.1.1) may be moved to the package procedure chapter later on, when the package is defined.

7.2.2 One H.248 stream for all SCTP streams

When SCTP is used for the transport of signaling protocols (e.g. BICC, ISUP, H.248, etc.) all user messages over an SCTP association are of the same nature. The use of multiple SCTP streams is motivated in order to avoid head-of-line blocking problems, i.e. a lost or delayed single message  blocks or delays all traffic. This type of applications usually utilize a high number of streams with a balanced load distribution.

If an MGC were responsible to control an SCTP association in a MG for this purpose, the use of separate H.248 stream per SCTP stream would cause unnecessary overhead. Therefore one single H.248 stream could be enough for all SCTP streams.

However this case is out of scope of this version of this Recommendation.

[End Correction]

The use of the term “connectionless” in relation to SCTP, even if applied to an individual stream, is confusing and may be misleading. The text should avoid an interpretation of SCTP as a two layer protocol. Rather than talking about “connectionless” SCTP streams it is more meaningful to highlight the fact that the SCTP streams do not require bearer control procedures to come into existence. Furthermore, it is relevant to define when an SCTP is “existing”.

[Begin Correction]

### A.2.2		SCTP Stream

The SCTP Stream is stateless. An SCTP Stream is either existing  or not. It is existing if its stream identifier is between 0 and the negotiated number of streams minus 1, regardless of whether a user message has been sent already or not. Thus there are no bearer control procedures for the establishment of individual streams.

Contributor’s note: However, in the case of data channel, a big number of SCTP streams exist as soon as the SCTP association is established and they will never be used to carry user messages. H.248 signaling procedures will be needed to control the related H.248 SEPs, in particular the creation of them.

[End Correction]

The following updates proposals present a possible package solution that allow the MGC to create new SEPs within  a Termination in an optimal way when creating a number of SEPs equal the number of negotiated SCTP streams is not reasonable.

[Begin Correction]

### III.1.1.2	SCTP stream Identifier

|  | Property Name: | SCTP Stream Identifier |
| --- | --- | --- |
|  | Property ID: | sctpsid (0x0002) |
|  | Description: | This property indicates the SCTP stream Identifier of both the incoming and outgoing SCTP streams associated with the SEP. For an outgoing SCTP stream, this property must have been set before any outgoing SCTP user message may be sent. For an incoming SCTP stream: a) if the property is set, it must match the Stream Identifier included in the received stream, in order for the SCTP user messages to be received at the SEP b) if the property is not set, an incoming SCTP stream with an identifier not yet allocated in any SEP will be bound to the SEP and the SCTP user messages received at the SEP. |
|  | Type: | Integer |
|  | Possible values: | 0-65535 Contributor’s note: do we need an additional value to indicate “undefined” or “not allocated yet”? |
|  | Default: | None |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |

[End Correction]

[Begin Correction]

### III.1.2.2	New SCTP ID Reception

|  | Event Name: | New SCTP Identfier reception |
| --- | --- | --- |
|  | Event ID: | newSCTPIDrec (0x0002) |
|  | Description: | This event indicates that an SCTP message has been received in an SCTP stream with an SCTP stream identifier which is not allocated yet –with the sctpsid property- in any of the SEPs of the Termination. The Stream Identifier must be a valid one, i.e. within the range negotiated with the remote SCTP endpoint. The generation of the event also represents a binding of the SCTP stream with the SEP. The event occurs only once per SCTP Stream Identifier. |
|  | Type: | Integer |
|  | Possible values: | 0-65535 |
|  | Default: | None |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |

### III.1.2.2.1	EventsDescriptor parameters

### None

### III.1.2.2.2	ObservedEventsDescriptor parameters

### III.1.2.2.2.1	SCTP Stream Identifier

| Parameter Name: | SCTP Stream Identfier |
| --- | --- |
| Parameter ID: | (0x0001) |
| Description: | This parameter indicates the Stream Identifier contained in the received SCTP message. |
| Type: | Integer |
| Possible values: | 0-65535 |
| Default: | None |

[End Correction]

[Begin Correction]

### III.1.6.1	SCTP endpoint creation

FIXTHIS

The MGC must set the sctpsid property of the SEP with a valid Stream Identifier, i.e. within the range negotiated with the remote peer, before an SCTP user message can be sent.

An incoming SCTP user message may be received in a SEP that has no value allocated yet to the sctpsid property, if that stream identifier is a valid one and it has not been allocated to any other SEP in the Termination.

If this occurs, the SCTP stream with the received identifier is bound to this H.248 stream and  the newSCTPIDrec event is generated. The MGC shall then set the sctpsid property with that value. If the MGC tries to give a different value, an error will be generated.

Once the SCTP stream has been bound to a SEP at reception of the first received user message the SEP can not be used to receive SCTP user messages with different identifiers, even if the MGC has not set the sctpsid property yet. It is therefore the responsibility of the MGC to create new SEPs if more SCTP streams are expected or may potentially be received.

[End Correction]

__________________