| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4636 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Seoul, 3 - 7 November 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Ericsson |  |  |  |
| Title: | H.248.SCTP Requirements updates |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution from Ericsson proposes a number of updates in the requirements on the H.248 solution for the control of SCTP bearers.

# Introduction

In the last SG16 meeting in Sapporo, an appendix was added to H.248.SCTP with the requirements that should drive the H.248 solution for the control of SCTP bearers. It is acknowledged that the requirements are still provisional since neither the list of interworking scenarios, the use cases or the functionality required from the SEP which is in the scope of  this version of this Recommendation are final.

Ericsson has made some further analysis on the use cases which are agreed to be in scope at this stage and derived some updates of the requirements.

# Discussion

1. Multi-homing is not possible in the CLUE and WebRTC use cases due to the underlying DTLS transport. Therefore related requirements may be considered optional at this stage.

2. Mapping of streams: as described in AVD-4635 for the use cases contemplated in this version of the Recommendation, a mapping of one to one SCTP stream to H.248 stream is desired.

This is reflected in a new proposed requirement. The mechanism should allow for the MGC to identify the individual SCTP streams in order to perform control actions individually on each of them. The requirement also reflects the fact that in the general case an H.248 stream will not be created for each SCTP stream negotiated during the SCTP association establishment.

3. In the data channel, which is used for both CLUE and WebRTC, the ordered or unordered delivery is a characteristic of the channel, thus all user messages on the corresponding SCTP stream will be used with the same criterion. This is indicated during the data channel establishment.

A mechanism must exist so that the MGC can indicate the desired delivery mechanism. At this stage is not defined whether DCEP messages will be terminated in the MGC or in the MG.

4. Likewise when establishing the data channel, the endpoints indicate whether reliable or partially reliable transmission will be applied. A corresponding mechanism must exist so that the MGC can provide this indication to the MG.

5. The lost and recovery of the communication is of interest for the MGC. Thus corresponding requirements are added.

6. The closure of a data channel is initiated with the resetting of the SCTP outgoing stream. A mechanism to allow the MGC to reset a stream is required.

# Conclusion

The following changes to H.248.SCTP are proposed in order to capture above discussion items in the requirements.

[Begin Correction]

Appendix IV

Requirements on the H.248 signaling solution

(This appendix collects requirements on the H.248 signaling solution. It should be removed before document approval. The requirements are not fully agreed at this stage)

## IV.1	Mandatory requirements

In order to allow the MGC to control an SCTP endpoint in a MG, H.248 must provide mechanisms to:

1.	Configure the SEP as an SCTP endpoint:

- Indicate wanted number of outbound streams

- Indicate wanted number of inbound streams

When the SCTP association with the peer is established a number of streams will be available in each direction within the association, taking into account both peers’ capabilities. The number of streams may be different in each direction.

2.	Request the SEP to initiate an SCTP association towards the peer:

3.	Block incoming SCTP association requests

4.	Notify an established SCTP association

- Include information on number of streams in each direction

5.	Request the SEP to initiate a graceful close of an SCTP association (SCTP shutdown)

6.	Request the SEP to abort an SCTP association

7.	Notify an SCTP association release

- Graceful (locally initiated)

- Graceful (remotely initiated)

- Aborted by remote peer

- Aborted by local peer

- Lower layer failure

8.	Configure SCTP association parameters

- RTO.Initial

- RTO.min

- RTO.max

- Path.Max.Retrans (maximum nr of unacknowledged retransmissions before considering the path as unavailable)

NOTE - the last two parameters specially influence the responsiveness to switch to a secondary path when primary fails.

- Association.Max.Retrans

- HB.interval

- HB.Max.Burst

- Maximum delay time to send an acknowledgement

## 9	Map each SCTP stream carrying user messages to an H.248 stream. The mechanism must ensure that the MGC can identify the individual SCTP streams and perform control actions on each stream separately, including how the stream is connected between terminations within the context. The mechanism must also allow for the dynamic creation of resources, in particular SEPs, in the MG as needed during the life time of the SCTP association, i.e. creation of SEPs to handle the number of streams negotiated in the SCTP association establishment process  may be deferred to the moment when it is considered necessary.

## 10.	Indicate whether ordered or unordered delivery is required for all sent user messages

## NOTE: Although in an SCTP stream the user can indicate ordered or unordered delivery per message with the use of the U flag  [IETF RFC 4960], in the data channel, which is the base of the use cases considered in this Recommendation, all messages within one data channel will be deliver according to the same criterion.

## 11.	Indicate whether reliable or partially reliable message transfer is required for all sent user messages and, in the latter case, whether the partial reliability is due to a limit in the number of retransmissions or to time retransmission limit.

12.	Notification that communication with the remote end point has been lost

13.	Notification that communication with the remote end point has been recovered

## 14.	Reset an SCTP stream: the closure of a data channel (webrtc) is initiated by resetting the corresponding outgoing stream.

## IV.2	Optional requirements

There could be potentially further requirements on an H.248 signaling solution. However, it is not clear at this stage if they are needed, i.e. if an MGC involvement is needed in relation to this functions.  H.248 should provide mechanisms to:

1.	Indicate the wanted number of IP addresses

2.	Notify remote IP addresses

3.	Select the primary path

4.	Notify that the connection has switched to secondary path

5.	Notify that the primary path has become available/unavailable

6.	Notify that the connection has switched back to primary path

7.	Notify that a destination has become available/unavailable

NOTE: In the two use cases considered at this stage in this Recommendation multi-homing will not be used. Therefore requirements 1-7 are not needed at this point of time.

8.	Request the SEP not to bundle chunks
This request may be part of the SAP API and up to the application layer in the user plane, thus eventually out of the MGC scope

9.	Request not to have ordered delivery for a particular user message.

This request capability may be part of the SAP API and up to the application layer in the user plane, thus eventually out of the MGC scope. Furthermore for the use cases considered for this Recommendation, which are based on the data channel, all messages are sent according to the same criterion (ordered delivery or unordered).

## IV.3	Work in progress

There is ongoing work in IETF related to quick failover mechanism. Its support may result on further requirements on H.248.

1.	Quick Failover algorithm

- Configure the SEP to support it or not (draft-ietf-tsvwg-sctp-failover-03)

[End Correction]

__________________