| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4640 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Seoul, 3 - 7 November 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Huawei Technologies Co., Ltd. |  |  |  |
| Title: | H.248.50 / H.248.WebRTC: Support of STUN consent freshness |  |  |  |
| Purpose: | Proposal |  |  |  |

1.	Introduction

This Contribution discusses the support of “STUN usage for Consent Freshness” in H.248.

2.	Discussion

IETF draft [draft-ietf-rtcweb-stun-consent-freshness] (http://tools.ietf.org/html/draft-ietf-rtcweb-stun-consent-freshness-07) introduces a requirement for WebRTC implementations to perform a combined consent freshness and session liveness test using STUN request/response procedures.

Therefore gateways interworking with WebRTC implementations will also need to support these procedures.

The main impact of the draft is that a WebRTC endpoint must not send application data over any transport protocol on an ICE-initiated connection unless the receiving endpoint consents to receive the data. After a successful ICE connectivity check on a particular transport address, subsequent consent MUST be obtained following the procedures described the draft.

The procedures of clause 4.1/[draft-ietf-rtcweb-stun-consent-freshness] effectively cause STUN binding requests to be sent at a recommended random interval of between 4000 – 6000 milliseconds. Once a valid binding response is received the consent to send application data remains valid for 30 seconds. This consent relates to the ability to send application data and is considered separate to connection keepalives which may also be used in conjunction with the draft.

ITU-T H.248.50 currently supports several methods for generating STUN bindings and keepalives:

1. The “Originate STUN Connectivity Check” Package (clause 8.2/[ITU-T H.248.50]) allows the MGC to request that the MG perform ICE connectivity checks by sending STUN binding requests. This procedure is to identify which candidates have connectivity and relates to the initial ICE connectivity check procedures as outlined in [draft-ietf-rtcweb-stun-consent-freshness]. It doesn’t address keepalives or consent.

2. The “MGC originated STUN request” Package (clause 9.1/[ITU-T H.248.50]) allows the MGC to request the MG to send a STUN binding request from a local address to a remote address with a particular retransmission time. There is no relation to controlling when to send media. The MGC must also signal each STUN binding request instance due to the “brief” nature of the signal.

3. The “Keep alive request” Package (clause 9.2/[ITU-T H.248.50])  enables the MGC to request sending of keep alive packets of type “STUN binding indication” to maintain bindings for a transport connection. It allows the MG to send the keep alives at a regular interval. Again it does not control when to send application data.

Note: The keepalive signal is of type “brief”. Given the nature of the retransmission interval it seems more suitable for the signal to be of type “on/off”. Currently there is no way to turn “off” the signal once applied. This may be considered for addition to H.248.50.

As can be seen above none of the currently defined packages supporting STUN binding requests support the functionality outlined in [draft-ietf-rtcweb-stun-consent-freshness].  Thus new work needs to be untaken to allow MGs to support this functionality.

In developing a solution the following requirements are considered:

- Given the nature of the interval being a random period of 0.8N and 1.2N where N is recommended to be 5000ms, it is appropriate for the MG to control the sending of STUN binding requests rather than burdening the MGC with this regular signalling. Therefore the method supported by the “MGC originated STUN request” Package wouldn’t be suitable.

- [draft-ietf-rtcweb-stun-consent-freshness] indicates that STUN binding requests are NOT retransmitted. This means that the “MGC originated STUN request” Package wouldn’t be suitable because it supports a retransmission interval of a minimum of 15 seconds.

- As [draft-ietf-rtcweb-stun-consent-freshness] explicitly states that keepalives may be used in addition, it would not be appropriate to link a solution to the current keepalive mechanisms. i.e. not suited to a revision of the “Keep alive request” Package.

- The usage of the STUN consent mechanism is tied to the sending of application data. None of the current packages support this behaviour. New procedures / elements would need to be developed and indicate possible relation to other H.248 elements, e.g. Streammode.

- Clause 7/ [draft-ietf-rtcweb-stun-consent-freshness] indicates what a Javascript API should support. Given the decomposition of a MGC and MG, the H.248 interface should also support the ability for:

  - The MG to notify the MGC when consent freshness has failed and the MG has stopped sending.

  - The MGC to start and stop liveness testing and to set the interval.

  - The MG to notify the MGC that a liveness test has failed.

Given the above considerations a new “STUN Consent Freshness” package is proposed. The package has been proposed against H.248.50 however given that it is also related to WebRTC support it could instead be incorporated in H.248.WEBRTC.

3.	Proposal

It is proposed to accept the following changes. The changes are against TD 147r2/WP1 from the Sapporo, July Q3/16 Rapporteurs meeting.  For readability the package itself is not marked up.

Draft revised Recommendation ITU-T H.248.50

Gateway control protocol: NAT traversal toolkit packages

…

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published. The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (2005), Gateway control protocol: Version 3.

[ITU-T H.248.14]	Recommendation ITU-T H.248.14 (2009), Gateway control protocol: Inactivity timer package.

[ITU-T H.248.37]	Recommendation ITU-T H.248.37 (2008), Gateway control protocol: IP NAPT traversal package.

[ITU-T H.248.40]	Recommendation ITU-T H.248.40 (03/2013), Gateway control protocol: Application data inactivity detection package  .

[ITU-T H.248.90]	Recommendation ITU-T H.248.90 (2014), Gateway control protocol: H.248 packages for control of transport security using TLS.

[ITU-T Y.2111]	Recommendation ITU-T Y.2111 (2008), Resource and admission control functions in next generation networks.

[IETF RFC 3489]	IETF RFC 3489 (2003), STUN – Simple Traversal of User Datagram Protocol (UDP) Through Network Address Translators (NATs).

[IETF RFC 3556]	IETF RFC 3556 (2003), Session Description Protocol (SDP) Bandwidth Modifiers for RTP Control Protocol (RTCP) Bandwidth.

[IETF RFC 3605]	IETF RFC 3605 (2003), Real Time Control Protocol (RTCP) attribute in Session Description Protocol (SDP).

[IETF RFC 4566]	IETF RFC 4566 (2006), SDP: Session Description Protocol.

[IETF RFC 4787]	IETF RFC 4787 (01/2007), Network Address Translation (NAT) Behavioral Requirements for Unicast UDP.

[IETF RFC 5245]	IETF RFC 5245 (2010), Interactive Connectivity Establishment (ICE): A Protocol for Network Address Translator (NAT) Traversal for Offer/Answer Protocols.

[IETF RFC 5389]	IETF RFC 5389 (2008), Session Traversal Utilities for NAT (STUN).

[IETF RFC 5766]	IETF RFC 5766 (2010), Traversal Using Relays around NAT (TURN): Relay Extensions to Session Traversal Utilities for NAT (STUN).

[IETF RFC 5780]	IETF RFC 5780 (05/2010), NAT Behavior Discovery Using Session Traversal Utilities for NAT (STUN).

[IETF RFC 6544]	IETF RFC 6544 (03/2012), TCP Candidates with Interactive Connectivity Establishment (ICE).

[IETF STUNCONFRES]	IETF draft-ietf-rtcweb-stun-consent-freshness-07 (201x), STUN Usage for Consent Freshness.

…

## x.1	STUN Consent Freshness

|  | Package name: | STUN Consent Freshness |
| --- | --- | --- |
|  | Package ID: | stnconfres (0x????) |
|  | Description: | This package allows an MGC to request an MG to perform the STUN usage for Consent procedures described in [IETF STUNCONFRES]. This allows the MG to send STUN binding requests and receive STUN binding responses in order to determine whether application data may be sent on the Stream. |
|  | Version: | 1 |
|  | Extends: | None |

### x.1.1	Properties

None.

## x.1.2	Events

#### x.1.2.1	Consent State

|  | Event name: | Consent State |
| --- | --- | --- |
|  | Event ID: | constate (0x0001) |
|  | Description: | This Event allows a MG to indicate to the MGC whether consent has been granted or not for a particular stream. It is only reported if the state changes between “granted” and “not granted” (i.e. revoked) states and vice versa. In consent “not granted” state application data stops flowing. In consent “granted” state application data flows. |

##### x.1.2.1.1	EventsDescriptor parameters

###### x.1.2.1.1.1	Request States

|  | Parameter name: | Request States |
| --- | --- | --- |
|  | Parameter ID: | reqstate (0x0001) |
|  | Description: | The parameter allows the MGC to request notification of a change to “granted” and/or “not granted” state. |
|  | Type: | Enumeration |
|  | Optional: | Yes |
|  | Possible values: | G Notify a transition to state “granted” N Notify a transition to state “not granted” B Notify a transition to state “granted” or “not granted” {Contributor’s note: The solution could be simplified to remove the ability to request a certain state transition and always report all transitions. In such case the parameter can be removed.} |
|  | Default: | B |

##### x.1.2.1.2	ObservedEventsDescriptor parameters

###### x.1.2.1.2.1	State

|  | Parameter name: | State |
| --- | --- | --- |
|  | Parameter ID: | state (0x0001) |
|  | Description: | This parameter indicates the consent state that the transport has transitioned to. |
|  | Type: | Enumeration |
|  | Optional: | No |
|  | Possible values: | G Consent has been granted and application is flowing. N Consent is not granted and application is not flowing. |
|  | Default: | None. |

#### x.1.2.2	STUN Consent Request Failure

|  | Event name: | STUN Consent Request Failure |
| --- | --- | --- |
|  | Event ID: | confail (0x0002) |
|  | Description: | This Event allows a MG to indicate to the MGC when a response to a STUN binding request indicating the failure to renew consent. |

##### x.1.2.2.1	EventsDescriptor parameters

##### None.

##### x.1.2.2.2	ObservedEventsDescriptor parameters

### None.

### x.1.3	Signals

#### x.1.3.1	Consent Test

|  | Signal name: | Consent Test |
| --- | --- | --- |
|  | Signal ID: | contst (0x0001) |
|  | Description: | This signal initiates the combined consent freshness and session liveness test using STUN request/response procedures as outlined in clause 4.1/[IETF STUNCONFRES]. |
|  | Signal Type: | On/Off |
|  | Duration: | Not applicable. |

##### x.1.3.1.1	Additional parameters

###### x.1.3.1.1.1	Test Interval

|  | Parameter name: | Test Interval |
| --- | --- | --- |
|  | Parameter ID: | tstint (0x0001) |
|  | Description: | This parameter indicates the consent check interval, the parameter “N” as defined by clause 4.1/[IETF STUNCONFRES]. The MG sends consent checks at random intervals between 0.8N and 1.2N. {Contributor’s note: An alternative is to specify this parameter as a sub-list of Integer with two values, a minimum time period and a maximum time period. This would allow for more flexibility than the 0.8N and 1.2N definition.} |
|  | Type: | Integer |
|  | Optional: | Yes |
|  | Possible values: | 1 millisecond upwards |
|  | Default: | 5000ms |

### x.1.4	Statistics

None.

### x.1.5	Error Codes

None.

### x.1.6	Procedures

#### x.1.6.1 STUN Consent Freshness initiation

To initiate the STUN consent freshness procedures from [IETF STUNCONFRES] the MGC shall send the “Consent Test” (stnconfres/contst) Signal on the H.248 Stream where the consent is required. The stnconfres/contest Signal should be sent prior to or in the same H.248 command as any signals initiating ICE connectivity checking to ensure that consent is granted as a result of successful initial ICE procedures.

On reception of the stnconfres/contest signal the MG shall initiate the consent procedures in clause 4.1/[IETF STUNCONFRES]. As described the MG must not send application data (e.g., RTP, RTCP, SCTP, DTLS), over any transport protocol (e.g., UDP, TCP) on an ICE-initiated connection unless the receiving endpoint consents to receive the data.  After a successful ICE connectivity check on a particular transport address, subsequent consent shall be obtained following the procedures described [IETF STUNCONFRES].  The MG shall send a STUN binding request consent check at the random interval indicated by the “Test Interval” (tstint) parameter in order to maintain consent.

When consent is granted the MG may send application data on the Stream. The granting of consent does not override other methods for controlling the flow of application or transport related data such as the Local Control StreamMode property [ITU-T H.248.1].

In order to be notified about the consent state the MGC should set the “Consent State” (stnconfres/contest) Event with the required states on the applicable Stream. As the Event indicates state transitions, the MGC should set the Event when sending the stnconfres/contest Signal to ensure the initial transition from consent “not-granted” (i.e. the default state before the completion of ICE connectivity check when the stnconfres/contest Signal is active) to a granted state after initial consent checking is reported.

The state transitions to a consent “granted” state on receipt of a valid STUN binding response. The state transitions to a consent “not-granted” state when a valid STUN binding response corresponding to one of the STUN requests sent in the last 30 seconds has not been received from the remote peer's Transport Address or the MG detects an immediate revocation of consent via one of the methods in clause 4.2/[IETF STUNCONFRES]. A MGC may also be notified of the revocation of consent based on the detection of Events related to a closure of the transport connection, i.e. a TLS BNC Change Event indicating “bearer release” [ITU-T H.248.90].

#### x.1.6.2 STUN consent request failure

In order to be notified of a failed STUN consent refresh the MGC shall set the “STUN Consent Request Failure” (stnconfres/confail) Event on the applicable Stream. When the MG detects a failure of a STUN consent refresh to renew the consent it shall send a Notify.Req with the stnconfres/confail) ObservedEvent. The MGC may then decide to continue to allow the MG to send STUN consent refreshes, or stop STUN consent refreshes or to take another action as appropriate.

#### x.1.6.3 STUN Consent Freshness deactivation

To stop sending STUN consent requests the MGC should turn the stnconfres/contest Signal “off” by removing it from the applicable stream via a Modify.req command. On reception of the signal the MG shall stop sending STUN consent refreshes. If any of the stnconfres Package Events are active on the Stream, then the MG shall stop sending application data at the expiry of the consent period. If no stnconfres Package Signals or Events are active on the Stream then the STUN consent refresh procedures are no longer applicable to the Stream and application data may flow.

#### x.1.6.4 STUN Consent Freshness response

In order for a MG to respond to STUN binding requests used for consent refresh the MGC shall use the “MG Act-as STUN Server” Package (clause 8.1/[ITU-T H.248.50]).

#### x.1.6.5 Connection liveness

If the MGC requires notification of application data inactivity (as per clause 5/[IETF STUNCONFRES] it should use the “Application Data Inactivity Detection Package” [ITU-T H.248.40]. Keepalives can be initiated through the use of the “Keep alive request” Package (clause 9.2/[ITU-T H.248.50]) .

___________________