| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4641 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Seoul, 3 - 7 November 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Huawei Technologies Co., Ltd. |  |  |  |
| Title: | H.248.PAURES: New work item for RTP stream pause and resume |  |  |  |
| Purpose: | Proposal |  |  |  |

1.	Introduction

This Contribution proposes a new work item to allow the usage of remote media stream pause and resume.

2.	Discussion

The IETF are currently working on draft-ietf-avtext-rtp-stream-pause which introduces the ability for an endpoint to send a message to the remote RTP sender requesting that it pause (or resume) the transmission of RTP packets. It does this by extending the codec control messages (CCM) in the RTCP feedback package. The purpose is to allow endpoints to manage RTP flows without the need to use call control signalling to stop the flows and later add the flows. It allows the endpoints to optimise the use of network bandwidth and to control inputs to mixing. See clauses 1 and 3/[ draft-ietf-avtext-rtp-stream-pause] for more details on the usage scenarios. As H.248 media gateways may act as a source/sink for RTP media and may act as a RTP mixer it is important that gateways are able to interact with endpoints supporting the capabilities in the IETF draft.

To allow for the negotiation of the remote pause/resume capability, [draft-ietf-avtext-rtp-stream-pause] extends SDP by adding a new CCM feedback value:

Pause: Indicates whether the endpoint supports pause/resume related feedback messages.

There are sub-parameters “nowait” and “config” which further define the functionality.

In order to communicate between the RTP receiver and RTP sender several new feedback control information (FCI) blocks are defined. These new types are: PAUSE request, RESUME request, PAUSED indication and a REFUSED response.

The PAUSE request is used by the RTP receiver to request that the RTP sender pause the RTP stream. In response to the request the RTP sender may initiate the pause and send a PAUSED indication to the RTP receiver or continue transmission and issue a REFUSED response.

The RESUME request is used by the RTP receiver to request that the RTP sender resume the RTP stream. In response to the request the RTP sender may resume sending RTP packets on the stream or remain is a paused state and issue a REFUSED response.

Gateways utilising H.248 currently have several methods to pause the transmission of media:

- H.248.1: Contains the Local Control Descriptor ‘Streammode’ property that allows the MGC to effectively stop sending a RTP media stream (i.e. change from send/receive to receive only) whilst still maintaining an RTCP control flow. To resume the flow of media the MGC can change the Streammode back to send/receive. However this is a local MG action rather than a request to a remote endpoint. The action is also triggered from a call control level stimulus.

- H.248.1: The connection model (including the TopologyDescriptor) allows Terminations/Streams to be isolated from other Terminations/Streams in the Context. This allows a MGC to isolate a media stream from other input streams effectively stopping the flow of media. The MGC may then move the Termination/Stream back into a Context to resume media flow.

- H.248.9: Contains the “Play Collect” package that enables a user to request (via a DTMF digit) that the playout of media associated with a signal be paused. It also enables the user to resume the media flow via a DTMF digit.

- H.248.66: Contains the “Signal pause package” which is a generic package that allows the MGC to request a pause in the playout of media and later resume media flow. It is suited to scenarios where media playout has been initiated through the use of a H.248 Signals rather than real time media flows. It again is a local MG action rather than a request to a remote endpoint and the action is also triggered from a call control level stimulus.

None of the above methods match the functionality supported by [draft-ietf-avtext-rtp-stream-pause]. H.248.1 and H.248.66 do not support user originated requests being terminated at the MG and H.248.9 and H.248.66 do not allow the pause of media streams not related to the playout of signals. Therefore to support the functionality of [draft-ietf-avtext-rtp-stream-pause] an update is needed to the H.248.x Sub-series of Recommendations. This Contribution proposes the addition of a new Recommendation ITU-T H.248.PAURES “Support of remote media pause and resume” to the series.

References:

[draft-ietf-avtext-rtp-stream-pause] 	https://datatracker.ietf.org/doc/draft-ietf-avtext-rtp-stream-pause/

3.	Proposal

It is proposed to adopt a new work item on the support of remote media pause and resume as detailed in the work item template below:

Proposed new ITU-T Recommendation H.248.PAURES

| Question: | 3/16 | Proposed new ITU-T Recommendation | Seoul, 30 4-6 November 2014 |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference and title: | Recommendation ITU-T H.248.PAURES "Gateway control protocol: Support of remote media pause and resume" |  |  |  |  |
| Base text: | AVD-xxxx |  | Timing: | 2015 |  |
| Editor(s): | Christian Groves (Christian.Groves@nteczone.com) |  |  |  |  |
| Scope: |  |  |  |  |  |
| The support RTP Stream Pause and Resume functionality as defined by [IETF STREAM PAUSE] on H.248 controlled gateways: a) describe considerations with respect to RTP stream pause and resume implementation on gateways; b) describe relationship to existing H.248 Recommendations that incorporate pause/resume functionality; c) define H.248 protocol extensions for the support of RTP stream pause/resume; d) describe the mapping between call control (SIP) SDP and H.248.SDP describing pause/resume capabilities e) provide examples. |  |  |  |  |  |
| Summary |  |  |  |  |  |
| Support of remote RTP pause and resume in H.248 media gateways |  |  |  |  |  |
| Relations to ITU-T Recommendations or to other standards (approved or under development): |  |  |  |  |  |
| H.248.1, H.248.9, H.248.66 and H.248.STGROUP |  |  |  |  |  |
| Liaisons with other study groups or with other standards bodies: |  |  |  |  |  |
| Possibly with IETF, 3GPP. |  |  |  |  |  |
| Supporting members that have committed to contribute in an active manner to this work item: |  |  |  |  |  |
| Huawei |  |  |  |  |  |

Furthermore it is proposed to accept the draft revised Recommendation as a baseline text for ITU-T H.248.PAURES “Support of remote media pause and resume”.

Draft revised Recommendation ITU-T H.248.PAURES

Gateway control protocol: Support of remote media pause and resume

Summary

This Recommendation allows the RTP Stream Pause and Resume functionality as defined by [IETF STREAM PAUSE] to be supported on H.248 controlled gateways.

# 1	Scope

This Recommendation describes the support of RTP Stream Pause and Resume functionality as defined by [IETF STREAM PAUSE] on H.248 controlled gateways. It describes the mapping of call control SDP related to pause/resume to H.248 SDP and vice versa. It also contains a package that allows several different implementation types to be supported.

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (2013), Gateway control protocol: Version 3.

[ITU-T H.248.9]	Recommendation ITU-T H.248.1 (12/09), Gateway control protocol: Advanced media server packages .

[ITU-T H.248.66]	Recommendation ITU-T H.248.66 (xxxx), Gateway control protocol: Packages for RTSP and H.248 interworking.

[IETF STREAM PAUSE]	IETF draft-ietf-avtext-rtp-stream-pause-04 (xxxx), RTP Stream Pause and Resume.

# 3	Definitions

## 3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	<Term 1> [Reference]: <optional quoted definition>

3.1.2	<Term 2> [Reference]: <optional quoted definition>

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	<Term 3>: <definition>

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| CCM | Codec Control Message |
| --- | --- |
| DTMF | Dual Tone Multi Frequency |
| MG | Media Gateway |
| MGC | Media Gateway Controller |
| RTCP | Real Time Control Protocol |
| RTP | Real Time Protocol |
| SDP | Session Description Protocol |

# 5	Conventions

Elements of the ITU-T H.248 protocol model, e.g., Context, Termination, Stream, Event are represented using the first letter capitalized. Property, Event, Signal and Parameter identities are given in italics.

The suffix ".req" added to an ITU-T H.248 command name stands for a command request, while the suffix ".rep" stands for a command reply. For example "Notify.req" represents a Notify Request.

Capitalised names PAUSE, RESUME, REFUSED, PAUSED refer to Codec Control (CCM) messages defined in [IETF STREAM PAUSE].

# 6	Relation to other H.248 Recommendations

Gateways utilising H.248 currently have several methods to pause the transmission of media.

[ITU-T H.248.1]: Contains the Local Control Descriptor ‘Streammode’ property that allows the MGC to effectively stop sending a RTP media stream (i.e. change from send/receive to receive only) whilst still maintaining an RTCP control flow. To resume the flow of media the MGC can change the Streammode back to send/receive. However this is a local MG action rather than a request to a remote endpoint. The action is also triggered from a call control level stimulus.

[ITU-T H.248.1]: The H.248 connection model (including the TopologyDescriptor) allows Terminations/Streams to be isolated from other Terminations/Streams in the Context. This allows a MGC to isolate a media stream from other input streams effectively stopping the flow of media. The MGC may then move the Termination/Stream back into a Context to resume media flow.

[ITU-T H.248.9]: Contains the “Play Collect” package that enables a user to request (via a DTMF digit) that the playout of media associated with a signal be paused. It also enables the user to resume the media flow via a DTMF digit.

[ITU-T H.248.66]: Contains the “Signal pause package” which is a generic package that allows the MGC to request a pause in the playout of media and later resume media flow. It is suited to scenarios where media playout has been initiated through the use of a H.248 Signals rather than real time media flows. It again is a local MG action rather than a request to a remote endpoint and the action is also triggered from a call control level stimulus.

None of the above methods match the functionality supported by [IETF STREAM PAUSE]. [ITU-T H.248.1] and [ITU-T H.248.66] do not support user originated requests being terminated at the MG and [ITU-T H.248.9] and [ITU-T H.248.66] do not allow the pause of media streams not related to the playout of signals.

Pause/resume relates to real time media flow. Therefore the resumption of media flow is based on the media at the time of the resume request rather than at the pause point. [ITU-T H.248.66] assumes that the playout occurs from the pause point unless another point is indicated in the resumption request.

# 7	Considerations

{Editor’s note: It is expected this clause would address any gateway consideration for the support of pause/resume. E.g. consider how the different requirements from draft-ietf-avtext-rtp-stream-pause are met.

# 8	Remote Pause and Resume Package

|  | Package name: | Remote Pause and Resume |
| --- | --- | --- |
|  | Package ID: | rempr (0x????) |
|  | Description: | This package allows a MGC to request that the MG issue a request to a remote endpoint to pause (and subsequently resume) the transmission of an RTP media stream. |
|  | Version: | 1 |
|  | Extends: | None |

8.1	Properties

{Editor’s note: TBD}

8.2	Events

{Editor’s note: TBD}

8.3	Signals

{Editor’s note: TBD}

8.4	Statistics

{Editor’s note: TBD}

8.5	Error Codes

{Editor’s note: TBD}

8.6	Procedures

{Editor’s note: TBD}

# 9 Examples

{Editor’s note: TBD}

Annex A

<Annex Title>

(This annex forms an integral part of this Recommendation.)

<Body of annex A>

Appendix I

<Appendix Title>

(This appendix does not form an integral part of this Recommendation.)

<Body of appendix I>

Bibliography

[b-ITU-T X.yyy]	Recommendation ITU-T X.yyy (date), Title

____________________