| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4642 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Seoul, 3 - 7 November 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Huawei Technologies Co., Ltd. |  |  |  |
| Title: | H.248.WEBRTC: Potential Updates to H.248.x Recommendations for WEBRTC support |  |  |  |
| Purpose: | Proposal |  |  |  |

1.	Introduction

This Contribution discusses potential updates to H.248 Recommendations as a result of supporting RTCWEB on a MGC/MG.

2.	Discussion

This contribution analyses the current IETF drafts related to the IETF RTCWEB working group to see if there any impacts to existing H.248 Recommendations and to see if new items are required to support communication with WebRTC clients by H.248 controlled gateways.

Appendix I below contains a list of drafts from the RTBWEB WG page (https://datatracker.ietf.org/doc/search/?name=rtcweb&activeDrafts=on&rfcs=on). Most independent submissions have been removed. Each draft from the list is a level one heading. These drafts may contain references to other drafts that need to be considered. These drafts have been included in level 2 and 3 headings. Drafts with potential impacts are highlighted in yellow.

3.	Proposal

There are no specific updates proposed. This contribution only serves for discussion to trigger possible future contribution.

# 1. draft-alvestrand-rtcweb-gateways-00 “WebRTC Gateways”

This draft indicates that a WebRTC gateway may support ICE-Lite. This can be supported through the use of H.248.50.

# 2. draft-ietf-rtcweb-alpn-00 “Application Layer Protocol Negotiation for Web Real-Time Communications (WebRTC)”

This draft introduces the ability to indicate the usage of ALPNs for “webrtc” and “c-webrtc”. Application Layer Protocol   Negotiation (ALPN) [RFC7301] enables an endpoint to positively identify WebRTC uses and distinguish them from other DTLS uses. H.284.WEBRTC should address how this is communicated between the MGC and MG. It may require updates to H.248.93 on DTLS to support signalling the ALPN or discuss how to determine it.

# 3. draft-ietf-rtcweb-audio-06 “WebRTC Audio Codec and Processing Requirements”

This draft indicates a baseline set of audio requirements. H.248 can support the signalling of the required information. E.g. Opus, G.711 codecs, comfort noise, audio/telephone events, audio level and echo cancelling.

# 4. draft-ietf-rtcweb-audio-codecs-for-interop-00 “Additional WebRTC audio codecs for interoperability with legacy networks”

This draft presents additional audio codecs. The use of these codecs may be signalled via H.248.

# 5. draft-ietf-rtcweb-constraints-registry-00 “IANA Registry for RTCWeb Constrainable Properties”

The draft introduces a registry for media track constraints. The draft in itself does not relate to H.248 however the media track constraints (http://www.w3.org/TR/2013/WD-mediacapture-streams-20130903/#idl-def-MediaTrackConstraints) do relate to the description of media and thus have a relation to SDP for media. There may need to be some mapping between the constraints to appropriate SDP. This is possibly a topic for the JSEP draft.

# 6. draft-ietf-rtcweb-data-channel-12 “WebRTC Data Channels”

The support of this is already being discussed as part of the work on H.248.WEBRTC, H.248.SCTP and H.248.STGROUP.

# 7. draft-ietf-rtcweb-data-protocol-08 “WebRTC Data Channel Establishment Protocol”

The support of this is already being discussed as part of the work on H.248.WEBRTC, H.248.SCTP and H.248.STGROUP.

# 8. draft-ietf-rtcweb-jsep-07 “Javascript Session Establishment Protocol”

This draft provides a mapping between the WebRTC Javacript API and SDP. It defines what SDP is used for WebRTC sessions. It is expected that a H.248 Gateway acting as a WebRTC endpoint would utilise this draft. The SDP requirements from the draft would be translated to H.248 SDP. It is expected this would be addressed by H.248.WEBRTC.

# 9. draft-ietf-rtcweb-overview-11 “Overview: Real Time Protocols for Browser-based Applications”

This draft gives an overview of the requirements and architecture of WebRTC. The data transport, framing and security requirements are detailed by the other drafts in this Contribution.

# 10. draft-ietf-rtcweb-rtp-usage-17 “Web Real-Time Communication (WebRTC): Media Transport and Use of RTP”

This draft outlines requirements for the use of RTP in WEBRTC. Much of the required information can be signalled between a MGC and MG via SDP however other requirements relate to specific handling of RTP that is typically not signalled by the MGC. A MG must know whether the H.248 Termination relates to a WebRTC client to ensure that all the requirements are implemented. This area requires further investigation.

# 11. draft-ietf-rtcweb-security-07 “Security Considerations for WebRTC”

This draft outlines security issues related to WebRTC. There’s no direct impact related to H.248 controlled gateways.

# 12. draft-ietf-rtcweb-security-arch-10 “WebRTC Security Architecture”

This draft outlines the security architecture including protocols for WebRTC. The main requirement is the support of DTLS which is supported in H.248.93. The use of a=identity and a=fingerprint is also indicated which in supported via SDP in H.248. Verification and consent are also discussed. These issues are discussed by other drafts in this Contribution.

# 13. draft-ietf-rtcweb-stun-consent-freshness-07 “STUN Usage for Consent Freshness”

This draft introduces the notion of periodic STUN binding requests into order to determine whether a sender may sent application data for a period 30 seconds. H.248.50 allows the MGC/MG to send binding requests but does not support sending periodic requests. This would need to be added to H.248.50.

# 14. draft-ietf-rtcweb-transports-06 “Transports for WebRTC”

## 14.1 System provided interfaces

### 14.1.1 draft-ietf-tsvwg-rtcweb-qos-02 “DSCP and other packet markings for RTCWeb QoS”

No explicit impact other than indicating what DSCP values the MGC should set.

## 14.2 IPv4/IPv6 Usage

### 14.2.1 draft-reddy-mmusic-ice-happy-eyeballs-07 “Happy Eyeballs Extension for ICE”

This draft updates the calculation of candidate priority for connectivity checks from ICE RFC5245. It introduces a new formula. H.248.50 is currently based on RFC5245. Gateways used for WEBRTC should use the new formula. H.248.50 clause 8 allows the MGC to provide the priority or to wildcard choose the priority. In the MGC provided case as it knows whether the Termination relates to an WEBRTC endpoint it can apply the new formula. In the case of the wildcarded CHOOSE case, the MG would need an indication of whether to apply the WebRTC or base ICE priority formula. This indication would need to be added to H.248. In either case the section could be updated to indicate that the priority formula from this draft could be used in the MGC provided case.

## 14.2.2 RFC6724 “Default Address Selection for Internet Protocol Version 6 (IPv6)”

This RFC indicates that temporary addresses are preferred over public addresses for source address selection. RTCWEB clients should use this selection choice. Clause 5 indicates that there must be a means to override the preference. This RFC also references RFC5104 which is the IPv6 socket API that allows different address “types” to be indicated. H.248 currently does not support these functions when a MGC requests address either through normal H.248 wildcarding procedures or via address gathering in H.248.50.

## 14.3 Middle Box Related

### 14.3.1 RFC5245 “Interactive Connectivity Establishment (ICE): A Protocol for Network Address Translator (NAT) Traversal for Offer/Answer Protocols”

ICE is supported by H.248.50.

### 14.3.2 RFC5766  “Traversal Using Relays around NAT (TURN): Relay Extensions to Session Traversal Utilities for NAT (STUN)”

TURN is supported by H.248.50.

### 14.3.3 RFC6156  “TURN Extension for IPv4/IPv6 Transition”

H.248.50 supports TURN however it currently does not specify the usage of RFC6156 which allows the use of the REQUESTED-ADDRESS-FAMILY attribute (e.g. clause 4.1/RFC6156). RTCWeb indicates this must be supported. H.248.50 should be updated to indicate that this may be used with a possible new codepoint in clause 7.3 for the MG TURN client package.

### 14.3.4 RFC6062  “Traversal Using Relays around NAT (TURN) Extensions for TCP Allocations”

The RFC defines additional STUN methods and attributes to be used with TURN for TCP address allocations. H.248.50 currently doesn’t make reference to this RFC. The support of RFC6062 should be considered by revised H.248.50.

### 14.3.5 RFC6544  “TCP Candidates with Interactive Connectivity Establishment (ICE) ”

This RFC introduces the possibility to use TCP candidates in ICE procedures. The support of RFC6544 has been added to the draft revised H.248.50.

## 14.4 Transport Protocol Related

### 14.4.1 RFC4571  “Framing Real-time Transport Protocol (RTP) and RTP Control Protocol (RTCP) Packets over Connection-Oriented Transport ”

This RFC allows the transport of RTP over TCP. The use of RTP over TCP may be indicated through SDP and thus may be used by H.248.

### 14.4.2 RFC5389  “Session Traversal Utilities for NAT (STUN)”

This RFC defines Session Traversal Utilities for NAT known as “STUN”. H.248.50 already supports STUN.

# 15. draft-ietf-rtcweb-use-cases-and-requirements-14 “Web Real-Time Communication Use-cases and Requirements”

This draft introduces a number of use cases of web applications that are executed in a browser utilising real-time requirements. A number of requirements are generated as a result. As the requirements are addressed by other more detailed drafts there’s no direct impact to H.248 from this draft.

# 16. draft-ietf-rtcweb-video-00 “WebRTC Video Processing and Codec Requirements”

As per the audio requirements, the use of a particular video codec (along with its parameters) may be indicated via SDP. No additional work is envisaged for H.248.

# 17. draft-ietf-tsvwg-rtcweb-qos-02 “DSCP and other packet markings for RTCWeb QoS”

As per clause 14.1.1 above.

# 18. draft-ietf-xrblock-rtcweb-rtcp-xr-metrics-00 “Considerations for Selecting RTCP Extended Report (XR) Metrics for the RTCWEB Statistics API”

This draft indicates which RTCP XR metrics may be used by a WEBRTC endpoint. H.248.48 allows the detection and reporting of RTCP XR metric.

# 19. draft-jennings-rtcweb-deps-04 “WebRTC Dependencies”

This draft contains the full list of WEBRTC dependencies on IETF documents. It may provide a further checklist.

# 20. draft-nandakumar-rtcweb-sdp-05 “SDP for the WebRTC”

This draft contains sample SDP Offer/Answers used by WEBRTC endpoints. This also provides a good checklist to see if H.248 may support the media described by the SDP. H.248.WEBRTC could consider this.

____________________