| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4643 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 5/16 | Seoul, 3 - 7 November 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Huawei Technologies Co., Ltd. |  |  |  |
| Title: | H.TPS-SIG: Capability Exchange |  |  |  |
| Purpose: | Proposal |  |  |  |

1.	Introduction

This Contribution discusses the editor’s note in clause 8.3 on “Capability Exchange”.

2.	Discussion

At the Sapporo meeting there was a discussion on the use of H.245 TerminalCapabilitySet messages and how these related to SDP offers when transmitting CLUE related material. There was no resolution of the issue so it was agreed to add an editor’s note capturing the discussion:

“{Editor’s note: CLUE proposes use of SDP m-lines with encoding ID tags to bind CLUE captures to encodings. Q5/16 experts propose use of H.323 capabilities – which are similar to SDP m-lines but not identical. In addition, H.323 has three capability classes – receive, transmit, and receive-and-transmit.  Use of the receive capability seems closest to the CLUE proposal, but is problematic since the encoding ID tags are chosen by the sender.  Receive-and-Transmit is not suitable.  Transmit is possible, but could create interworking issues with existing H.323 equipment. More analysis is needed, and contributions are solicited }”

Clause 6.2.8.1/[ITU-T H.323] describes the capability exchange procedures for H.323 terminals. The capabilities are described as:

- Receive capabilities: Describe the terminal's ability to receive and process incoming information streams. Transmitters shall limit the content of their transmitted information to that which the receiver has indicated it is capable of receiving. The absence of a receive capability indicates that the terminal cannot receive (is a transmitter only).

- Transmit capabilities: Describe the terminal's ability to transmit information streams. Transmit capabilities serve to offer receivers a choice of possible modes of operation, so that the receiver may request the mode which it prefers to receive. The absence of a transmit capability indicates that the terminal is not offering a choice of preferred modes to the receiver (but it may still transmit anything within the capability of the receiver).

- Receive-and-Transmit capabilities: Describe the terminal's ability to receive and transmit information streams when these capabilities are not independent and are required to be the same in both directions.

As the editor’s note indicates the use “Receive-and-Transmit” capabilities is not suited for use with CLUE because it mandates that the receive and transmit capabilities must be the same. The very nature of CLUE means that different sets of media may be used in the send and receive directions. This capability is not considered further by this Contribution.

Also noted in the editor’s note is the use of SDP m-lines by CLUE. The SDP is used as a means of capability description. Clause 4.5.1/[draft-ietf-clue-signaling] describes the sending of an initial SDP offer to initiate a CLUE session. It indicates that an endpoint should include at least one non-CLUE controlled m-line for interoperability reasons. It also indicates that the CLUE enabled endpoint should not include CLUE controlled m-lines unless it knows the capabilities of the peer endpoint.

For the non-CLUE controlled m-lines the default behaviour (as described in the SDP Offer/Answer RFC in clause 5.1/RFC3254) is a sent/receive m-line. The offerer sends the capabilities it supports. The SDP Answerer (the peer endpoint) may accept or reject an m-line (and thus the capabilities) as per (clause 6.1/RFC3254). The SDP Answerer may also add additional m-lines. This is usually achieved via the SIP INVITE, 2xx INVITE response messages. 1xx series and ACK messages are used to acknowledge the receipt of the INVITE and 2xx response. See [IETF RFC 6337 for details on SIP usage of SDP offer/answer). For H.323 devices the capabilities are usually sent as “receive” capabilities. The assumption is that endpoints describe what they are willing to receive.  The endpoint receiving a terminalCapabilitySet uses the receive capabilities and then sends the capabilities it supports in return (see clause 8.2/H.323). The TerminalCapabilitySet message sent by the initiating endpoint in terms of capability exchange can be seen as a SIP INVITE with an SDP offer and the TerminalCapabilitySet message sent by the peer can be seen as the 2xx INVITE RESPONSE. The TerminalCapabilitySetAck messages can been seen as a SIP 1xx and ACK response. Thus there is effectively the same behaviour for SIP/SDP and H.323 endpoints with regards to capability negotiation for standard media. An Offer/TerminalCapabilitySet is sent and an Answer/TerminalCapabilitySet is returned with chosen capabilities.

For CLUE controlled m-lines the behaviour with respect to indicating capabilities is different. CLUE controlled m-lines are advertised as “Sendonly” (or inactive). Rather than the SDP Offer/Answer procedures being used to determine which m-line is used, the CLUE Advertisement/Configure procedures are used. The media consumer chooses the media capture and encoding (m-line) it wants to receive. Both endpoints indicate the media that they can send through the SDP Offer/Answer procedures and use CLUE to choose which encoding/m-line they want. So this represents a change in behaviour over today’s normal procedures for media establishment. The endpoints can successfully complete this negotiation and establish media because they have both agreed on the use of CLUE.

If we parallel the SDP “Sendonly” behaviour with H.323 we can see that “Transmit” capabilities result in similar behaviour. An endpoint sends capabilities to the peer and the peer chooses what it wants to receive. There is no inference as to what it can send. It would seem that this is an ideal choice for use with CLUE enabled H.323 endpoints. However the editor’s note indicates “Transmit is possible, but could create interworking issues with existing H.323 equipment”. This is true and is also true of CLUE enabled SIP endpoints interworking with non-CLUE enabled endpoints.

The solution for SIP has been to indicate that an endpoint should only offer CLUE enabled SDP lines if it has “reasonable knowledge” of whether the peer supports CLUE. If it doesn’t know that a peer supports CLUE it waits until the successful negotiation of the CLUE protocol before sending an additional offer with the CLUE controlled m-lines. Thus in reality there is no interworking between CLUE enabled or non-CLUE enabled endpoints. The CLUE enabled endpoint will fall back to non-CLUE usage if the negotiation fails.

A similar approach is proposed for H.323. In order to successfully interwork with a CLUE enabled endpoint an existing endpoint would need to be upgraded to support the CLUE protocol and new capabilities model of indicating what it can send. A non-CLUE endpoint can’t be expected to alter its behaviour. Thus sending a TerminalCapabilitySet message with transmit only capabilities may cause problems because the endpoint may not support “transmit capabilities”.

Like SIP, the assumption H.323 can use is that capabilities related to CLUE controlled media streams are not sent in a TerminalCapabilitySet message until the endpoint is aware that the peer endpoint supports CLUE. This can be determined through the CLUEControlCapability (as per clause 7.2.1.1/[ITU-T H.TPS-SIG]). If it is not possible to determine whether the peer endpoint supports CLUE during the H.225 signalling phase, the use of CLUE can be certain once the endpoints have negotiated a method for carrying CLUE messages (as per as per clause 7.2.1.1/[ITU-T H.TPS-SIG]). As H.323 allows a TerminalCapabilitySet message to be sent whenever the capabilities of an endpoint change, then the endpoints may exchange TerminalCapabilitySet messaging including “Transmit” capabilities. This procedure would mean that there is no possibility that a non-CLUE enabled H.323 endpoint would receive a TerminalCapabilitySet message with “transmit” capabilities from a CLUE enabled endpoint.

Even if a non-CLUE endpoint was to receive a TerminalCapabilitySet with “transmit” capabilities it would not result in any OpenLogical signalling as the CLUE enabled transmitting endpoint would not establish media until it received a CLUE configure which would not be forthcoming from the non-CLUE endpoint.

It is proposed to capture these negotiation procedures in H.TPS-SIG.

3.	Proposal

It is proposed to accept the following changes. Changes are marked up against TD 169 R3/WP1 from the July 2014 Q.5/16 Rapporteurs’ meeting in Sapporo.

Recommendation ITU-T H.TPS-SIG Signalling for telepresence-enabled conferencing

...

# References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[F.TPS-Reqs]	Recommendation ITU-T F.yyy (date), Definitions, Requirements, and use cases for Telepresence Systems.

[F/H.TPS-Arch]	Recommendation ITU-T F/H.TPS-Arch (date), Telepresence System Architecture.

[H.TPS-AV]	Recommendation ITU-T H.yyy (date), Audio/Video Parameters for Telepresence systems.

[ITU-T H.323]	Recommendation ITU-T H.323 v7 (12/2009), Packet-based multimedia communications systems.

[ITU-T H.245]	Recommendation ITU-T H.245 v16 (05/2011), Control protocol for multimedia communication.

[ITU-T H.225.0]	Recommendation ITU-T H.225.0 v7 (12/2009), Call signalling protocols and media stream packetization for packet-based multimedia communication systems.

[ITU-T H.460.1]	Recommendation ITU-T H.460.1 (03/2002), Guidelines for the use of the generic extensible framework.

[IETF CLUE DM]	IETF draft-ietf-clue-data-model-schema-04, An XML Schema for the CLUE data model.

[IETF CLUE FW]	IETF draft-ietf-clue-framework-15, Framework for  Telepresence Multi-Streams.

[IETF CLUE SIG] 	IETF draft-ietf-clue-signaling-01, CLUE Signaling.

[IETF CLUE DC]		IETF draft-ietf-clue-datachannel-00, CLUE Protocol Data Channel.

[IETF CLUE PROT]		IETF draft-ietf-clue-protocol-01, CLUE protocol.

[IETF RTCWEB DC]		IETF draft-ietf-rtcweb-data-channel-07, WebRTC Data Channels.

[IETF DCEP]		IETF draft-ietf-rtcweb-data-protocol-03, WebRTC Data Channel Establishment Protocol.

...

## 8.3 Capability exchange

Capability exchange for telepresence endpoints is based on H.323 capability exchange procedures in clause 8.2/[ITU-T H.323] with the addition of CLUE protocol messaging to provide further information regarding media. The telepresence capability exchange procedures have two parts. The capability exchange related to the establishment of the CLUE protocol and the capability exchange related to the CLUE controlled media.

### 8.3.1 CLUE protocol capability exchange

Clause 7.2.1 indicates that an endpoint may use a separate data connection or use H.245 for the transport of CLUE messages.

If the initiating H.323 telepresence endpoint decides to use a separate data connection, in the initial TerminalCapabilitySet message it should set a bidirectional DataApplicationCapability indicating the use of “CLUE” over a WebRTC Data Channel as defined by Annex U/[ITU-T H.245]. If the peer endpoint is a H.323 telepresence endpoint then it should return a TerminalCapabilitySet message also indicating the support of a data connection for the CLUE protocol. The initiating endpoint shall then establish a DTLS/SCTP association containing the CLUE protocol as per Annex U/[ITU-T H.245].

Once the initiating endpoint is ready to send CLUE messages (either via H.245 transport or data channel transport) it shall initiate messaging according to [IETF CLUE PROT]. The endpoints shall exchange CLUE OPTIONs messaging and each provide an initial CLUE ADVERTISEMENT. The CLUE ADVERTISEMENTs shall contain encodingIDs related to CLUE controlled media capabilities communicated via H.245 TerminalCapabilitySet messages. This may relate to already communicated capabilities or capabilities yet to be communicated.

### 8.3.2 CLUE controlled media capability exchange

In order to align with the CLUE protocol which describes the media captures sent by an endpoint, “transmit” capabilities (see clause 6.2.8.1/[ITU-T H.323]) are used in the TerminalCapabilitySet messages to represent capabilities related to CLUE controlled media.

An endpoint should not include CLUE related capabilities in a TerminalCapabilitySet message unless it is able to determine that the peer supports the use of the CLUE protocol. This can be determined through the CLUEControlCapability (as per clause 7.2.1.1). If it is not possible to determine whether the peer endpoint supports CLUE during the H.225 signalling phase, capability exchange via an initial TerminalCapabilitySet exchange may occur indicating the use of the CLUE protocol (as per 8.3.1) but without CLUE controlled media capabilities being exchanged. Once the endpoints have negotiated a method for carrying CLUE messages (as per as per clause 7.2.1) then they can complete a CLUE controlled media capability negotiation by resending TerminalCapabilitySet messages including the “transmit” capabilities related to the CLUE controlled media.

Both endpoints involved in the capability exchange send “transmit” capabilities based on their own capabilities and not based on information received from their peer. The set of capabilities related to one CLUE controlled media stream shall have an associated CLUE Correlation Capability containing a unique encodingID (see clause 7.2.3.1) for the call. The CLUE protocol ADVERTISEMENT/CONFIGURE messages are used to choose the media (with associated capabilities) that is established. The endpoint acting as a CLUE media consumer will use the media capture and encoding information from a CLUE ADVERTISEMENT message and the capability set information related to the encodings to determine which media captures and encodings it requires. The selected media capture and encodings are communicated to the endpoint acting as the media provider. The endpoint acting as a media consumer shall not send a CLUE CONFIGURE indicating the use of a particular media capture and encoding unless it has received a TerminalCapabilitySet with a capability set containing the applicable encodingID. The endpoint acting as the media provider then establishes the media via H.245 OpenlogicalChannel procedures.  An endpoint shall not establish CLUE controlled media unless it has received a CLUE CONFIGURE indicating the applicable capability set via an encodingID.

### 8.3.3 Example capability exchange flows

### Note: For brevity the example flows below do not show non-CLUE controlled media. The encodingIDs used do not imply any numbering or sequencing.

### 8.3.3.1 CLUE over WebRTC Datachannel, CLUE support negotiated via H.225

Figure 6 shows the scenario where the support of CLUE between endpoints A and B has been negotiated and the CLUE protocol over a WebRTC Datachannel is being used. The CLUE controlled media capabilities are communicated via the initial TerminalCapabilitySet messaging.

Figure 6 – CLUE over WebRTC Datachannel, CLUE support negotiated via H.225Capability Exchange Call Flow

### 8.3.3.2 CLUE over WebRTC Datachannel, CLUE controlled media capabilities provided after CLUE protocol establishment

Figure 7 shows the scenario where the CLUE protocol over a WebRTC Datachannel is being used and the CLUE controlled media capabilities are communicated after the CLUE protocol establishment. The TerminalCapabilitySet signalling with the CLUE controlled media capabilities could occur at any time after the initial TerminaCapabilitySet signalling confirms the use of CLUE.

Figure 7 – CLUE over WebRTC Datachannel, CLUE support negotiated via H.225Capability Exchange Call Flow

_______________________