| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4649 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Seoul, 3 - 7 November 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Ericsson |  |  |  |
| Title: | H.Sup.OpenFlow: OpenFlow protocol analysis |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution from Ericsson provides further input to the H.248 vs. OpenFlow Supplement with emphasis on some of the differences between the protocols which were not visible so far in the draft.

# Discussion

## Scope

The current draft contains statements such as “H.248 is much more powerful than OpenFlow”,  “it can be considered a superset of OpenFlow”  or “the capabilities of OpenFlow may be essentially provided by H.248”.

In Ericsson’s view these are not fair statements. While it is true that H.248 was designed to cover a wider range of scenarios than OpenFlow, normally requiring extensions (packages) for specific scenarios, OpenFlow’s high specialization in the programmatic control of Ethernet switches and IP routers make it today a more powerful protocol than H.248 in this specific field.

As suggested in other paragraph in the draft, the scope of the study would be to assess the OpenFlow emulation possibilities of  H.248. Thus, whether OpenFlow capabilities can be provided by H.248 or not should be the result of this analysis and can not be answered positively a priori.

## Terminology relations

The current draft provisionally proposes several terminology relationships between OpenFlow concepts and H.248 concepts which in Ericsson’s view are arbitrary or incorrect.

The current draft relates the OpenFlow “Action” to the H.248 command, “action bucket” to H.248 descriptor, “action set” to H.248 transaction and “group” to H.248 context.

There is no explicit concept of “action” in H.248 but based on the OpenFlow semantics of this concept it could be related to the operation of applying a descriptor (Media, Signal, Event, Topology) on a stream. The same relation is applicable for action bucket, action and bucket group since these three concepts represents groups or sets of actions and an H.248 descriptor or set of descriptors could imply one or several operations applied on a stream.

Commands and transactions are syntactical means to convey the instructions to perform such operations in H.248. Therefore the mapping action/command or action set/ transaction seems incorrect.

## Architecture comparison

The draft derives from two rather simplified architecture pictures that “both models largely overlap and consider identical scopes related to the network forwarding plane and the separation line between controlling and controlled entity”.

In Ericsson’s view this is an overstatement which ignores the rather different dynamics of both protocols in terms of context vs. flow tables life cycles and the fact that the H.248 model is primarily stream based, where stream is related to a session, call or conference.

## Pull and Push modes

The statement about pull and push modes seems unnecessary for the illustration of the control modes which is clearly master/slave. H.248.55 only applies to specific type of actions related to policy enforcement. A generic statement such as “H.248 supports push as well as pull mode” in the context of clause 6.4 (which is general about the MGC-MG control relationship) is misleading or wrong.

## Pipeline processing

OpenFlow defines a clear pipeline processing model where the controller has full control of the order in which the matching rules and the resulting actions are applied. The draft states that in H.248 the MG internal processing pipeline is quite similar.

Ericsson can agree that in a MG implementation there is likely to be ultimately a “pipeline” or serialized processing model although this is not explicit or mandated in the H.248 model and implementations may use some forms of parallel processing. There are attempts in the H.248 Recommendations series to define the recommended order in which some of the most basic actions and matching criteria on packets should be performed. But in general the order of actions is not fully determined by the H.248 protocol. There is no similar flexibility in H.248 for the controller to manage the sequencing of matching criteria and actions as there is in OpenFlow.

## Capabilities comparison

Tables in clause 10 contain an initial attempt to compare OpenFlow and H.248 capabilities. However these tables are not much more than a check list at slogan level. A “Yes” in a particular capability in both protocols should not be interpreted as if they support a similar level of functionality. The tables lack clarifications of how a certain capability is supported with H.248.

# Conclusion

The following changes are proposed in order to capture above discussion points.

[Begin Correction]

# 1	Scope

The "vertical" decomposition of network elements in packet based communication infrastructures, driven by network evolution scenarios which will demand for "open control" interfaces between the packet transport, forwarding and routing domain and the overlay domain of application/service control, demands for technologies for controlling of packet flows in the most general sense.

There are a number of candidate technologies for such network solutions. Purpose of this document is an attempt in comparing H.248 with OpenFlow (as defined by [OPENFLOW SS]).

[…]

It has to be noted that an evaluation between H.248 and OpenFlow - from overall protocol point of view - is not meaningful due to the  narrower scope of OpenFlow compared to H.248. OpenFlow  only considers Ethernet switches and IP routers as network elements. It is hence a packet technology dependent protocol, tightly coupled to packet forwarding and routing functions. H.248 is more generic and potentially covers all type of packet network entities related to packet switching, packet forwarding/routing, packet modification, etc. usually by means of extensions (H.248 packages). However, with very few exceptions, H.248 protocol is designed to operate on session-based flows.

The objective of this study is to analyse if and how the capabilities of the OpenFlow protocol may be provided by H.248.

The results of any assessment are dependent on the considered context, which might be

- protocol Pi should satisfy the requirements given by network solution Nj (i.e., an evaluation against a specific, concrete set of required protocol capabilities);

- protocols Pi and Pj are compared with focus on the common set of capabilities (i.e., looking only at the intersection of Pi ∩ Pj);

- protocol Pi should be able to emulate the capabilities of protocol Pj (or vice versa) (i.e., an evaluation against a set of supported protocol capabilities); or

- any other objectives.

Architects of new networks would be interested in evaluation context 1. Technology replacement strategies would consider evaluation context 2. The scope of this technical report is primarily the third evaluation context.

[…]

The structure of this TR is as follows:

- protocol origins and histories (clause 6);

- packet forwarding models (clause 7);

- stage 1 (services, use cases) consideration (clause 8);

- stage 2 (protocol requirements) consideration (clause 9);

- stage 3 (protocol capabilities) consideration (clause 10);

and

- complementary information to OpenFlow (Appendix I);

- complementary, high-level assessment of H.248 vs ForCES (Appendix II) and

- reference to other work on OpenFlow vs ForCES.

{Editor's note (2013-11 meeting): clause 1 is not yet complete … }

[End Correction]

[Begin Correction]

## 6.2	Terminology relations

Each control technology uses their specific terminology. Table 1 provides a rough comparison of OpenFlow specific terms and their relation in H.248.

OpenFlow and H.248 are nevertheless based on different models. In H.248 the operations performed on the user plane are specified by Descriptors. It is therefore of no surprise that many of the OpenFlow concepts relate loosely to the H.248 descriptor concept. This relationship, as well as this table as a whole, does not imply in any way that the OpenFlow concepts can in the general case be emulated by the indicated H.248 concept.

Table 1 – Terminology relations between OpenFlow and H.248

| Term | OpenFlow | H.248 | Comments |
| --- | --- | --- | --- |
| Action: | An operation that forwards the packet to a port or modiﬁes the packet, such as decrementing the TTL ﬁeld. Actions may be speciﬁed as part of the instruction set associated with a ﬂow entry or in an action bucket associated with a group entry. Actions may be accumulated in the Action Set of the packet or applied immediately to the packet | Relate to the operation of applying one or multiple descriptors to a stream. |  |
| Action Bucket: | A set of actions and associated parameters, deﬁned for groups. |  |  |
| Action Set | A set of actions associated with the packet that are accumulated while the packet is processed by each table and that are executed when the instruction set instructs the packet to exit the processing pipeline |  |  |
| Controller: | An entity interacting with the OpenFlow switch using the OpenFlow protocol. | MGC |  |
| Flow Entry: | An element in a ﬂow table used to match and process packets. It contains a set of match ﬁelds for matching packets, a priority for matching precedence, a set of counters to track packets, and a set of instructions to apply. | Relates to the descriptors, as they contained the data from which context determination and eventually packet matching rules (e.g. for packet filtering, statistics, etc.) are derived as well as for describing the action to be performed. |  |
| Flow Table: | A stage of the pipeline, contains ﬂow entries. | No corresponding concept in H.248. |  |
| Group: | A list of action buckets and some means of choosing one or more of those buckets to apply on a per-packet basis. | As per bucket |  |
| Instruction: | Instructions are attached to a ﬂow entry and describe the OpenFlow processing that happen when a packet matches the ﬂow entry. An instruction either modiﬁes pipeline processing, such as direct the packet to another ﬂow table, or contains a set of actions to add to the action set, or contains a list of actions to apply immediately to the packet. | Relate to a descriptor or descriptors as these contain the instructions in H.248 to perform operations on a stream e.g. , policy rule actions ( [ITU-T H.248.79]) |  |
| Match Field: | A ﬁeld against which a packet is matched, including packet headers, the ingress port, and the metadata value. A match ﬁeld may be wildcarded (match any value) and in some cases bitmasked | An information element in policy rule conditions, against which a packet is matched, either for context determination (see [ITU-T H.248.79]) or for action determination within a context (e.g.[ITU-T H.248.43], [ITU-T H.248.86], etc.) |  |
| Metadata: | A maskable register value that is used to carry information from one table to the next. | No corresponding concept in H.248. |  |
| Meter: | A switch element that can measure and control the rate of packets. The meter trigger a meter band if the packet rate or byte rate passing through the meter exceed a predeﬁned threshold. If the meter band drops the packet, it is called a Rate Limiter. | a combination of - H.248.53 policer and - H.248 statistics |  |
| OpenFlow switch | A network node implementing the packet forwarding and manipulation actions specified by the Controller. | MG |  |
| Outermost Tag: | The tag that appears closest to the beginning of a packet. |  |  |
| Packet: | An Ethernet frame, including header and payload. | Lx-PDU in general | OF is tightly coupled to ET! |
| Pipeline: | The set of linked ﬂow tables that provide matching, forwarding, and packet modiﬁcations in an OpenFlow switch. | Relates to a Context |  |
| Port: | Where packets enter and exit the OpenFlow pipeline. May be a physical port, a logical port deﬁned by the switch, or a reserved port deﬁned by the OpenFlow protocol. | Relates to a Termination |  |
| Tag: | A header that can be inserted or removed from a packet via push and pop actions. | Lx-PCI in general |  |

[End Correction]

[Begin Correction]

## 6.3	Objectives in terms of network architecture

Figure 1 illustrates the high level decomposed control model for OpenFlow versus H.248.

Both architecture models are similar in terms of layer separation (a control layer and a bearer plane or forwarding plane) and the positioning of the protocol (H.248 and OpenFlow) as the communication mechanism between the two layers.

The internal components of each model (contexts, terminations and descriptors in H.248 and flow tables in OpenFlow) have in general very different dynamics and life cycles.

For completeness, Figure 2 depicts the correspondent ForCES model versus H.248. There are actually similar observations and conclusions as in case of OpenFlow.

Figure 1 – Decomposed control models – OpenFlow vs H.248

Contributor’s note: It is also proposed to replace “Packet flows”in the ingress and outgress of the MG in the right-hand picture by “Stream”.

Figure 2 – Decomposed control models – ForCES vs H.248

Contributor’s note: It is also proposed to replace “Packet flows” in the ingress and outgress of the MG in the right-hand picture by “Stream”.

[End Correction]

[Begin Correction]

## 6.4	Relationship between controlling and controlled entities

The relationship between the controlling and controlled entity is asymmetrical, given by a master/slave or master/agent relation. Thus, there is not any peer-to-peer or client/server relation (with flexible client/server role assignments).

It may be concluded that OpenFlow, H.248 and ForCES follow a master/slave asymmetry, from perspective of a control/management plane protocol.

[End Correction]

[Begin Correction]

## 6.5	Control dynamics

One of the biggest differences between the H.248 and OpenFlow protocols and models is related to the different control and connectivity dynamics. H.248 can be said to be session-based. In H.248 connections are established and removed dynamically in conformity with the sessions, calls or conferences. The packet forwarding is the result of connections. Connections are determined by Terminations, Contexts and Topologies that have a session-tied life cycle. The MGC handles also a session protocol.

The OpenFlow model is not session-based. Its main focus is L2 and L3 packet forwarding and routing, rather than connections. In OpenFlow the packet forwarding is determined by relatively static, not session-tied, flow tables.

NOTE: [ITU-T H.248.64] represents a rather exceptional case in the H.248 model as it is session-less. This Recommendation provides means for the control of packet forwading, in particular for the control of an IP router and therefore has a bigger analogy with the OpenFlow model.

## 6.6	Time scale of network control: call-dependent vs call-independent control actions

The fundamental procedures between the controlling and controlled entity are "control actions" in direction from controller (master) down to the controlled entity (slave). Control actions are normally "pushed" (push mode), i.e., the initial stimuli originates at controlling entity level, or could be also "pulled" (pull mode), i.e., the initial stimuli originates at the controlled entity.

There are a plethora of use cases, possibilities, service scenarios concerning the stimuli type behind such a control action. Table 2 indicates typical timescales between "high dynamical", call dependent packet path (node) control till "static", "semi-permanent", call-independent packet path (node) control.

Table 2 – Timescale of network control actions – Order and examples

| O. | Timescale of "control action" | Origin of stimuli for control action (examples) | H.248 | OpenFlow |
| --- | --- | --- | --- | --- |
| 1 | microseconds | event detection in packet flow (user plane), notified to controlling entity | Yes | ? |
| 2 | milliseconds | call control signalling (control plane) | Yes | ? |
| 3 | seconds | security policy signalling (control plane) related to filter updates | Yes | ? |
| 4 | 10's of seconds | security policy signalling (control plane) related to filter updates | Yes | ? |
| 5 | minutes | packet flow individual performance monitoring (control plane) (NOTE 1) | Yes | ? |
| 6 | 10's of minutes | packet flow individual configurations (management plane, NOTE 2). | Yes | Yes |
| 7 | hours | packet flow independent configurations related to "day time specific network traffic profiles", e.g. related to QoS engineering (control plane (NOTE 3), management plane). | Yes | Yes |
| 8 | days | packet flow independent configurations (e.g., driven by temporary routing decisions) | Yes | Yes |
| 9 | weeks | packet flow individual configurations as "semi-permanent" connection (management plane, NOTE 4). | Yes | Yes |
| NOTE 1 – E.g., either conditional reporting by controlled entity or/and auditing (e.g. interval based) by controlling entity. NOTE 2 – There are basically two options for triggering a "control action" via management plane: either via the management entity of the "controlling entity" (indirect) or via the management entity of the "controlled entity", - under the prerequisite of coordinated management entities in the management plane. In case of H.248, see e.g. illustration in Figure 1/[ETSI TS 183 025] and example data object (as managed/controlled object) in Annex B/[ETSI TS 183 025]. NOTE 3 – E.g., a centralized "QoS control server". NOTE 4 – See e.g., [ITU-T H.248.21] in case of H.248 related use case. |  |  |  |  |

For H.248: is basically applicable at all timescales as outlined in above Table 2. For instance, H.248-controlled deep packet inspection (DPI; [ITU-T H.248.86]) could be actually applied on all above time scales (see also Appendices I and IV in [ITU-T H.248.86]).

However, in the vast majority of applications and scenarios H.248 operates in the upper, thus highly dynamic, end of the time scale table.

[End Correction]

[Begin Correction]

## 7.1	OpenFlow processing model

Figure 3 provides a copy of the packet processing model of OpenFlow. The processing pipeline represents the common basic architecture: incoming packets are compared against a set of policy rule conditions, and the subsequent execution of associated policy rule action(s) in case of match events. The conditions are ordered by the usual concept of hierarchy and priorities. The performance objective of high packet throughput is addressed by the usual concept of a "staged pipeline", which represents a hierarchical internal packet path.

In the OpenFlow model the OpenFlow controller has full control of the ordering of actions as it is the controller who populates the tables.

Figure 3 – Packet ﬂow through the processing pipeline (see Fig. 2/[OPENFLOW SS])

In H.248 the MG usually also applies an internal packet processing pipeline, i.e. a serialized processing. Some application specific models are illustrated in [ITU-T H.248.43] (with scope on filtering), [ITU-T H.248.53] (with scope on traffic policing) or [ITU-T H.248.86] (with scope on packet inspection). The illustration of an example of an IP processing model with a mixture of filtering, QoS tagging, address translation, policing, etc. is  contained in Annex D/[ETSI TR 183 068]. [ITU-T H.248.79] provides some guidelines for the sequencing of packet processing actions in the MG. They cover some of the most common actions like statistics collection, filtering and traffic management. It also recommends to follow the processing order of the packets according to the OSI model, i.e. from lowest to highest layer in the ingress and from highest to lowest in the outgress.

However, the ordering of actions in H.248 is in general not fully determined by the protocol and to a large extent left to the implementation. It is for example in general not possible for an MGC to change the order of packet processing actions or determine priorities within matching conditions.

[End Correction]

[Begin Correction]

# 10	Stage 3 – Protocol capabilities

{Editor's note (2013-11 meeting): just some initial areas  …}

Tables in the following subclauses attempt to show a comparison of the capabilities of both protocols at high level. A capability listed as supported by both protocols may in fact be supported in a very different way and with a different level of functionality.

The tables also indicate how, e.g. with which extensions, some of the capabilities are supported with H.248.

Table 10 – Protocol capabilities – H.248 vs OpenFlow – Basic Capabilities

| No. | Capability | H.248 | OpenFlow | Comments |
| --- | --- | --- | --- | --- |
|  |  | Yes | Yes |  |

[End Correction]

[Begin Correction]

## 10.1	Termination types: supported protocol stacks in forwarding plane

Table 11 – Protocol capabilities – H.248 vs OpenFlow 
– Termination types for packet-oriented bearer technologies

| No. | Capability | H.248 | OpenFlow | Comments |
| --- | --- | --- | --- | --- |
|  | L2: Ethernet | Yes | Yes (NOTE 1) |  |
|  | L2: other than Ethernet | Yes | No |  |
|  | L3: IP (V4 and V6) | Yes | Yes (NOTE 1) |  |
|  | L3: other than IP | Yes | No |  |
|  | L4: IP transport protocols TCP, UDP, SCTP | Yes | No |  |
|  | Tunneled IP stacks | Yes | No | OpenFlow just IETF-BRM |
| NOTE 1 – The OpenFlow basic network element is an Ethernet switch! Thus, the notion of 'OpenFlow switch', rather than IP router or gateway etc. The OpenFlow protocol stack is fundamentally IP-over-Ethernet. |  |  |  |  |

Table 12 – Protocol capabilities – H.248 vs OpenFlow 
– Termination types for Circuit-oriented bearer technologies

| No. | Capability | H.248 | OpenFlow | Comments |
| --- | --- | --- | --- | --- |
|  | circuit-switched bearer | Yes | No (NOTE 1) |  |
| NOTE 1 – OpenFlow is not considering any interworking between IP or Ethernet network domains with circuit switched networks. |  |  |  |  |

[End Correction]

[Begin Correction]

## 10.2	Connection models: IP mode of operation in forwarding plane

Table 13 – Protocol capabilities – H.248 vs OpenFlow 
– Connection topologies

| No. | Capability | H.248 | OpenFlow | Comments |
| --- | --- | --- | --- | --- |
|  | Ethernet termination | Yes | No | In H.248 this could occur in IP (over Ethernet) to non-IP scenarios. |
|  | IP termination | Yes | No | H.248: (IP, non-IP) topology |
|  | ET-to-ET (switched Ethernet) | No | Yes | There are currently no H.248 extension to control the forwarding of frames based on MAC addresses. |
|  | IP-to-IP (in IP router mode) | Yes | Yes | In H.248 the control of an IP router would require the H.248 extensions from [ITU-T H.248.64]. |
|  | IP-to-IP (in IP router mode plus local NAT) | Yes | No |  |
|  | IP-to-IP (in B2BIH mode) | Yes | No |  |
|  | IP-to-IP (with protocol version translation) | Yes | No | such as V4-to-V6 |
|  | tunnelled to untunnelled IP interworking | Yes | No |  |
|  | IP multipoint topologies | Yes | No |  |
| NOTE 1 – # |  |  |  |  |

Table 14 – Protocol capabilities – H.248 vs OpenFlow 
– IP connection models

| No. | Capability | H.248 | OpenFlow | Comments |
| --- | --- | --- | --- | --- |
|  | IPR native | Yes | Yes | Contributor’s note: is there any difference to C#4? |
|  | IPR with NAT | Yes | No | Contributor’s note: is there any difference to C#5? |
|  | B2BIH native | Yes | No | V4-to-V4 ¦ V6-to-V6 |
|  | B2BIH V4-to-V6 | Yes | No |  |
| NOTE 1 – # |  |  |  |  |

Table 15 – Protocol capabilities – H.248 vs OpenFlow 
– Policy rule actions – Generic view

| No. | Capability | H.248 | OpenFlow | Comments |
| --- | --- | --- | --- | --- |
|  | Packet Forwarding (called Output in OF) | Yes | Yes | Packet forwarding in H.248 is usually determined by a session-tied connection. An exception is the extension specified in [ITU-T H.248.64] for the control of an IP Router. |
|  | Packet Discarding (called Drop in OF) | Yes | Yes | Can be supported in H.248 with IP filtering [ITU-T H.248.43] |
|  | Packet Scheduling (called Set-Queue in OF) | Yes | Yes | See clause 7.1.6/[ITU-T Y.1221] Contributor’s note: the H.248 mechanism that supports this should be stated. |
|  | Packet Marking | Yes | Yes (part of Set-Queue)? | See clause 7.1.4/[ITU-T Y.1221] DS Packet marking is supported in H.248 with [ITU-T H.248.52] extensions. |
|  | Traffic shaping | No (NOTE 1) | Yes (part of Set-Queue) | See clause 7.1.5/[ITU-T Y.1221] |
|  | Traffic policing | Yes | No | See clause 7.1.2/[ITU-T Y.1221] Ingress traffic policing is supported in H.248 with [ITU-T H.248.53] |
|  | Complete packet header removal or addition (called Push-Tag/Pop-Tag in OF) | Partially | Yes | H.248 MGs are able to generate or remove an IP header in IP to non-IP session based scenarios |
|  | Partial packet header modification (called Set-Field in OF) | Partially | Yes | IP header packet modification is usually a result of native H.248 packet processing through a context. |
|  | Packet lifetime modification (called Change-TTL in OF) | No | Yes | Limited to IP and MPLS TTL. |
|  | Packet-, Flow- and Flow-aggregate-related Measurements |  |  |  |
| NOTE 1 – Currently not supported, but in scope of a new H.248 extension |  |  |  |  |

[End Correction]

__________________