| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4653 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 5/16 | Seoul (KR), 3 - 7 Nov 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ZTE Corporation |  |  |  |
| Title: | H.TPS-SIG: Carry encodingID in OLC message |  |  |  |
| Purpose: | Proposal |  |  |  |

1	Introduction

This contribution proposes to add the description of how to carry encodingID in OLC message.

2	Discussion

After the finish of capability exchange the endpoints should open logical channels for media transport. The logical channel which correlates with CLUE should carry the encodingID to link the particular Capture otherwise it’s a traditional logical channel. Since encodingID has been defined in section 7.2.3 it’s necessary to find a suitable place to carry encodingID in OpenLogicalChannel structure.

dataType.VideoCapability.genericVideoCapability and dataType.AudioCapability. genericAudioCapability are prepared for the generic parameter without extra changes to H.245.

So we prefer to use dataType.VideoCapability.genericVideoCapability.collapsing and dataType.AudioCapability. genericAudioCapability.collapsing carrying video and audio encodingIDs.

# 3	Proposal

It is proposed to make these changes in H.TPS-SIG. Change is marked up against TD 169 R3/WP1 from the last meeting.

[Begin Proposal]

## 8.5	Open Logical Channel

After the finish of capability exchange the endpoints should open logical channels for media transport. The logical channel which correlates with CLUE should carry the encodingID to link the particular Capture otherwise it’s a traditional logical channel.

dataType.VideoCapability.genericVideoCapability.collapsing and dataType.AudioCapability.genericAudioCapability.collapsing in OpenLogicalChannel.forwardLogicalChannelParameters and OpenLogicalChannel. reverseLogicalChannelParameters is used to carry video and audio encodingIDs defined in section 7.2.3.

[End Proposal]

_________________