| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | TD-10 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | South Korea, (KR), 3 - 7 Nov 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Editor H.248.WEBRTC |  |  |  |
| Title: | H.248.WEBRTC "Guidelines on the use of H.248 capabilities for web-based real-time communication services in H.248 profiles" (New): Output draft Ed. 0.14 |  |  |  |
| Purpose: | Meeting input (Draft Recommendation) |  |  |  |

Summary

This document contains the editor's output draft for draft new H.248.WEBRTC "Guidelines on the use of H.248 capabilities for web-based real-time communication services in H.248 profiles" (Ed. 0.14).

# Changes versus Ed. 0.13:

- agreed discussed proposals from contributions AVD-4615, 4616, 4642 & 4660

__________________

Document History

| Document History |  |
| --- | --- |
| Issue | Notes |
| H.248.WEBRTC Ed. 0.1 | Input (AVD-4275) to WP 2/16 Brisbane meeting (2012-09) Location: http://ftp3.itu.int/av-arch/avc-site/2009-2012/1209_Bri/AVD-4275.zip |
| H.248.WEBRTC Ed. 0.2 | Output (TD-26) from WP 2/16 Brisbane meeting (2012-09) Location: http://ftp3.itu.int/av-arch/avc-site/2009-2012/1209_Bri/TD-26.zip |
| H.248.WEBRTC Ed. 0.3 | Input (TD 22/WP1) from SG16 Geneva meeting, January 2013 Location: http://itu.int/md/T13-SG16-130114-TD-WP1-0022 |
| H.248.WEBRTC Ed. 0.4 | Output (TD 22R1/WP1) from SG16 Geneva meet., Jan 2013 Location: http://itu.int/md/T13-SG16-130114-TD-WP1-0022 |
| H.248.WEBRTC Ed. 0.5 | Input (AVD-4369) to WP 1/16 Oslo meeting (2013-06) Location: http://ftp3.itu.int/av-arch/avc-site/2013-2016/1306_Osl/AVD-4369.zip |
| H.248.WEBRTC Ed. 0.6 | Output (TD-19) from WP 1/16 Oslo meeting (2013-06) Location: http://ftp3.itu.int/av-arch/avc-site/2013-2016/1306_Osl/TD-19.zip |
| H.248.WEBRTC Ed. 0.7 | Input (TD 86/WP1) to SG16 Geneva meeting, Oct/Nov 2013 Location: https://www.itu.int/md/dologin_md.asp?lang=en&id=T13-SG16-131028-TD-WP1-0086!MSW-E |
| H.248.WEBRTC Ed. 0.8 | Output (TD 86R1/WP1) from SG16 Geneva meeting, Oct/Nov 2013 Location: https://www.itu.int/md/dologin_md.asp?lang=en&id=T13-SG16-131028-TD-WP1-0086!R1!MSW-E |
| H.248.WEBRTC Ed. 0.9 | Input (AVD-4478) to WP 1/16 Geneva meeting (2014-03) Location: http://wftp3.itu.int/av-arch/avc-site/2013-2016/1403_Gen/AVD-4478.zip |
| H.248.WEBRTC Ed. 0.10 | Output (TD-18) from WP 1/16 Geneva meeting (2014-03) Location: http://wftp3.itu.int/av-arch/avc-site/2013-2016/1403_Gen/TD-18.zip |
| H.248.WEBRTC Ed. 0.11 | Input (TD 154/WP1) to SG16 Sapporo meeting, June/July 2013 Location: http://www.itu.int/md/T13-SG16-140630-TD-WP1-0153/en |
| H.248.WEBRTC Ed. 0.12 | Output (TD154R1/WP1) from SG16 Sapporo meeting, June/July 2013 Location: https://www.itu.int/md/dologin_md.asp?lang=en&id=T13-SG16-140630-TD-WP1-0154!R1!MSW-E |
| H.248.WEBRTC Ed. 0.13 | Input (AVD-4594) to WP 1/16 South Korea meeting (2014-11) Location: http://wftp3.itu.int/av-arch/avc-site/2013-2016/1411_Seo/AVD-4594.zip |
| H.248.WEBRTC Ed. 0.14 | Output (TD-09) from WP 1/16 South Korea meeting (2014-11) Location: http://wftp3.itu.int/av-arch/avc-site/2013-2016/1411_Seo/TD-11.zip |
| H.248.WEBRTC Ed. 0.15 |  |

CONTENTS

Page

1 Scope 9 2 References 9 3 Definitions 11 3.1 Terms defined elsewhere 11 3.2 Terms defined in this Recommendation 12 4 Abbreviations and acronyms 12 5 Conventions 13 5.1 Acronym usage 13 5.2 Prescriptive language 13 6 WEBRTC core technology overview 14 6.1 WEBRTC gateways 14 6.2 Capabilities 14 6.3 Transport of user plane WebRTC traffic 15 6.3.1 Objective 15 6.3.2 Protocol stack for UDP-based transport 15 6.3.3 Protocol stack for TCP-based transport 15 7 H.248 WEBRTC gateways: example use cases 18 7.1 Point-to-point communication 18 7.1.1 Use case #1: WebRTC-to-WebRTC interworking 18 7.1.2 Use case #2: WebRTC-to-NGN/IMS interworking 18 7.1.3 Use case #3: WebRTC-to-PSTN/ISDN interworking 19 7.2 Multipoint communication 19 8 Functional requirements for H.248 WEBRTC gateways 20 8.1 Requirements related to NAT traversal support 20 8.1.1 Media latching based NAT traversal support 20 8.1.2 ICE based NAT traversal support 20 8.2 Requirements related to communication topologies 20 8.2.1 H.248 WEBRTC gateway for point-to-point interworking 20 8.2.2 H.248 WEBRTC gateway as WEBRTC conferencing point 21 8.3 Requirements related to bearer traffic multiplexing 21 8.4 Requirements related to RTP-to-RTP type interworking 21 8.4.1 RTCP control flow component 21 8.4.2 RTP profiles 21 8.4.3 RTP multiplexing 21 8.4.4 RTP topologies 21 8.4.5 RTCP based services, report types and packet formats 22 8.4.6 Resource allocation rules for RTP/RTCP 22 8.5 Requirements related to TCP-based media transport 22 8.5.1 Requirements related to ### 23 8.5 Requirements related to media-aware type of interworking 23 8.5.1 Requirements related to WEBRTC audio 23 8.5.2 Requirements related to WEBRTC video 23 8.5.3 Requirements related to WEBRTC data 23 8.5.4 Requirements related to WEBRTC text 24 8.6 Requirements related to bearer security 24 8.6.1 Requirements related to transport security 24 8.6.2 Requirements related to media security 25 8.7 Requirements related to WEBRTC emergency services 26 8.8 Requirements related to QoS support 26 8.8.1 IP traffic control: QoS marking 26 8.8.2 IP traffic control: traffic policing – IP byterate policing 26 8.8.3 Improved transport robustness for RTP 26 8.9 Requirements related to bearer-path coupled congestion controls 27 8.10 Requirements related to performance monitoring 27 8.10.1 Requirements related to measurement point (MP) 27 8.10.2 Requirements related to collection point (CP) 28 8.10.3 Requirements related to reporting point (RP) 28 8.10.4 Requirements related to filtering point (FP) 28 8.10.5 Requirements related to loopback point (LP) 28 8.11 Requirements related to SDP-based description, declaration and negotiation of WEBRTC media configurations 28 8.11.1 Requirements related to supported SDP elements – Overall media descriptions 28 8.11.2 Requirements related to supported SDP elements – Codec configurations 31 8.11.3 Requirements related to SDP Offer/Answer 33 9 H.248 profile specification guidelines 33 9.1 Profile Identification 33 9.2 Summary 33 9.3 Gateway Control Protocol Version 33 9.4 Connection Model 33 9.5 Context Attributes 33 9.6 Terminations 34 9.7 Descriptors 34 9.7.1 TerminationState Descriptor 34 9.7.2 Stream Descriptor 34 9.7.3 Events Descriptor 34 9.7.4 EventBuffer Descriptor 34 9.7.5 Signals Descriptor 34 9.7.6 DigitMap Descriptor 34 9.7.7 Statistics Descriptor 34 9.7.8 ObservedEvents Descriptor 34 9.7.9 Topology Descriptor 34 9.7.10 Error Descriptor 34 9.8 Command API 35 9.8.1 Add 35 9.8.2 Modify 35 9.8.3 Subtract 35 9.8.4 Move 35 9.8.5 AuditValue 35 9.8.6 AuditCapabilities 35 9.8.7 Notify 35 9.8.8 ServiceChange 35 9.8.9 Manipulating and auditing context attributes 35 9.9 Generic command syntax and encoding 35 9.10 Transactions 35 9.11 Messages 35 9.12 Transport 35 9.13 Security 36 9.14 Packages 36 9.14.1 Mandatory Package 36 9.14.2 Optional Package 36 9.14.3 Package usage information 37 9.15 Mandatory support of SDP and ITU-T H.248.1 Annex C information elements 38 9.16 Optional support of SDP and ITU-T H.248.1 Annex C information elements 38 9.17 Procedures 38 Bibliography 39 Appendix I Use case specific capability sets – Two examples 40 I.1 Overview 40 Appendix L1 Living List – WebRTC media component "data" – Discussion of interworking scenarios and H.248 connection models 42 L1.1 Scope 42 L1.2 Possible interworking models by H.248 WebRTC gateways 42 L1.3 Relevant documentation 43 Appendix L2 Living List – H.248 WebRTC gateway capabilities for 3GPP Rel-12 45 L2.1 3GPP references 45 L2.2 H.248 WebRTC media gateway: media plane interworking scenarios 45 Appendix L3 Living List – DiffServ for WebRTC multimedia traffic – Discussion of IETF proposal "DiffServ for Real-time communication" 47 L3.1 Item of discussion 47 L3.2 DiffServ models 47 L3.3 Impact on H.248 WebRTC gateways 48 L3.4 Conclusions 48 Appendix L4 Living List – Potential updates to H.248.x Recommendations for WEBRTC support 49 L4.1 Purpose 49 L4.2 Discussion 49 L4.3 Status of analysis (2014-11) 49 L4.3.1 IETF WG RTCWEB 49 L4.3.2 Other IETF WGs 53

List of Tables

Page

Table 1 – Requirements related to supported SDP elements – Part I SDP "m="-line 28 Table 2 – Requirements related to supported SDP elements – Part II SDP "a="-line 29 Table 3 – Requirements related to supported SDP elements – Part III other SDP line s 30 Table 4 – SDP parameters for IANA registered media type "audio/opus" 32 Table L4.1 – IETF WebRTC documents identified by WG RTCWEB 49

List of Figures

Page

Figure 1 – User plane WebRTC protocol stack in case of UDP-based transport 15 Figure 2α – User plane WebRTC protocol stack in case of TCP-based transport 16 Figure 2β – User plane WebRTC protocol stack in case of TCP-based transport 16 Figure 2 – User plane WebRTC protocol stack in case of TCP-based transport 17 Figure 2γ – User plane WebRTC protocol stack in case of TCP-based transport 18 Figure 3 – Use case #1: WebRTC-to-WebRTC interworking 18 Figure 4 – Use case #2: WebRTC-to-NGN/IMS interworking 19 Figure 5 – Use case #3: WebRTC-to-PSTN/ISDN interworking 19 Figure L1.1 – H.248 WebRTC gateway – Interworking and scope on Data Channel (multiple IM sessions) 42 Figure L1.2 – H.248 WebRTC gateway – Example model for H.248 SEPs 43 Figure L1.3 – Landscape of related IETF Documents ("WebRTC data channel") 44 Figure L2.1 – 3GPP Rel-12 – H.248 WebRTC media gateway: media plane interworking scenarios 46 Figure L3.1 – DiffServ models 47

Draft new Recommendation ITU-T H.248.xx (ex H.248.WEBRTC)

Guidelines on the use of H.248 capabilities for 
Web-based Real-Time Communication Services in H.248 Profiles

AAP Summary

<Mandatory material to be provided before Consent>

Summary

<Mandatory material>

# 1	Scope

Web-based real-time communication is a standardized service by the IETF and W3C (called WEBRTC or RTCWEB service) and defines a particular protocol suite for IP-based communication in web browser environments. This application is related to multimedia conversational services which cover typical service components such as telephony, conferencing, instant messaging, etc. The native IETF / W3C WEBRTC service (as defined for the public Internet by the IETF) will be embedded in other IP communication infrastructures such as NGN/IMS.

This Recommendation

- describes example use cases with H.248 WEBRTC gateway involvement;

- identifies H.248 capabilities for such gateways; and

- provides guidelines for the specification of H.248 profiles for dedicated H.248 WEBRTC gateway types.

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (03/2013), Gateway control protocol: Version 3.

[ITU-T H.248.37]	Recommendation ITU-T H.248.37 (06/2008), Gateway control protocol: IP NAPT traversal package.

[ITU-T H.248.39]	Recommendation ITU-T H.248.39 (07/2014), Gateway control protocol: H.248 SDP parameter identification and wildcarding.

[ITU-T H.248.48]	Recommendation ITU-T H.248.48 (02/2012), Gateway control protocol: RTCP XR block reporting package.

[ITU-T H.248.50]	Recommendation ITU-T H.248.50 (09/2010), Gateway control protocol: NAT traversal toolkit packages.

[ITU-T H.248.52]	Recommendation ITU-T H.248.52 (06/2008), Gateway control protocol: QoS support packages.

[ITU-T H.248.53]	Recommendation ITU-T H.248.53 (03/2009), Gateway control protocol: Traffic management packages.

[ITU-T H.248.57]	Recommendation ITU-T H.248.57 (07/2014), Gateway control protocol: RTP control protocol package.

[ITU-T H.248.71]	Recommendation ITU-T H.248.71 (02/2010), Gateway control protocol: RTCP support packages.

[ITU-T H.248.77]	Recommendation ITU-T H.248.77 (09/2010), Gateway control protocol: Secure real-time transport protocol (SRTP) package and procedures.

[ITU-T H.248.78]	Recommendation ITU-T H.248.78 (03/2013), Gateway control protocol: Bearer-level application level gateway.

{Editor’s note (2014-07): the reference should be updated when H.248.78Rev.1 is approved (due to a possible B-ALG for MSRP-over-Data channel in  WebRTC H.248 gateways).}

[ITU-T H.248.87]	Recommendation ITU-T H.248.87 (01/2014), Gateway control protocol: Guidelines on the use of ITU-T H.248 capabilities for performance monitoring in RTP networks in ITU-T H.248 profiles.

[ITU-T H.248.88]	Recommendation ITU-T H.248.88 (01/2014), Gateway control protocol: RTP topology dependent RTCP handling by ITU-T H.248 media gateways with IP terminations.

[ITU-T H.248.92]	Recommendation ITU-T H.248.92 (07/2014), Gateway control protocol: Stream endpoint interlinkage package.

[ITU-T H.248.93]	Recommendation ITU-T H.248.93 (07/2014), Gateway control protocol: H.248 support for control of transport security using DTLS.

[ITU-T H.248.RTPMUX]	Draft Recommendation ITU-T H.248.RTPMUX (XX/2014), Gateway control protocol: H.248 support for RTP multiplexing.

[ITU-T H.248.SCTP]	Draft Recommendation ITU-T H.248.SCTP (XX/2014), Gateway control protocol: H.248 support for control of SCTP bearer connections.

[ITU-T H.248.STGROUP]	Draft Recommendation ITU-T H.248.STGROUP (XX/2014), Gateway control protocol: H.248 Stream grouping and aggregation.

[ITU-T H.320]	Recommendation ITU-T H.320 (03/2004), Narrow-band visual telephone systems and terminal equipment.

[ITU-T H.324]	Recommendation ITU-T H.324 (04/2009), Terminal for low bit-rate multimedia communication.

[ITU-T T.140]	Recommendation ITU-T T.140 (02/1998), Protocol for multimedia application text conversation.

[IETF RFC 3550]	IETF RFC 3550 (2003), RTP: A Transport Protocol for Real-Time Applications.

[IETF RFC 4585]	IETF RFC 4585 (2006), Extended RTP Profile for Real-time Transport Control Protocol (RTCP)-Based Feedback (RTP/AVPF).

[IETF RFC 4961]	IETF RFC 4961 (2007), Symmetric RTP / RTP Control Protocol (RTCP).

[IETF RFC 5109]	IETF RFC 5109 (2007), RTP Payload Format for Generic Forward Error Correction.

[IETF RFC 5124]	IETF RFC 5124 (2008), Extended Secure RTP Profile for Real-time Transport Control Protocol (RTCP)-Based Feedback (RTP/SAVPF).

[IETF RFC 5506]	IETF RFC 5506 (2009), Support for Reduced-Size Real-Time Transport Control Protocol (RTCP): Opportunities and Consequences.

[IETF RFC 5576]	IETF RFC 5576 (2009), Source-Specific Media Attributes in the Session Description Protocol (SDP).

[IETF RFC 5764]	IETF RFC 5764 (2010), Datagram Transport Layer Security (DTLS) Extension to Establish Keys for the Secure Real-time Transport Protocol (SRTP).

[IETF RFC 7007]	IETF RFC 7007 (2013), Update to Remove DVI4 from the Recommended Codecs for the RTP Profile for Audio and Video Conferences with Minimal Control (RTP/AVP).

[IETF rtcweb-overview]	IETF draft-ietf-rtcweb-overview (2014), Overview: Real Time Protocols for Brower-based Applications.

[IETF rtp-usage]	IETF draft-ietf-rtcweb-rtp-usage (2014), Web Real-Time Communication (WebRTC): Media Transport and Use of RTP.

[IETF rtcweb-secur]	IETF draft-ietf-rtcweb-security (2014), Security Considerations for WebRTC.

[IETF rtcweb- security-arch]	IETF draft-ietf-rtcweb-security-arch (2014), WebRTC Security Architecture.

[IETF rtcweb-transports]	IETF draft-ietf-rtcweb-transports (2014), Transports for RTCWEB.

[IETF xrblock-webrtc]	IETF draft-singh-xrblock-webrtc-additional-stats (201#), Additional RTP Control Protocol (RTCP) Extended Report (XR) Metrics for WebRTC Statistics API.

# 3	Definitions

## 3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	media path [IETF rtcweb-overview]: The path that media data follows from one WebRTC device to another.

NOTE – Relates to the bearer plane interface (at MG) in case of WEBRTC H.248 gateway.

3.1.2	web browser [ITU-T H.351]: A software application capable of rendering HTML and XHTML documents.

3.1.2	signalling path [IETF rtcweb-overview]: The communication channels used between entities participating in signaling to transfer signaling. There may be more entities in the signaling path than in the media path..

NOTE – Relates to the signalling plane call control interface (at MGC) in case of a WEBRTC H.248 gateway.

3.1.3	web browser [ITU-T H.351]: A software application capable of rendering HTML and XHTML documents.

3.1.4	WebRTC Browser [IETF rtcweb-overview]: Browser that conforms to the WebRTC protocol specifications and offer the WebRTC Javascript APIs.

3.1.5	WebRTC Device [IETF rtcweb-overview]: An unit (software, hardware or combinations) that conforms to the WebRTC protocol specifications, but does not offer the WebRTC Javascript APIs.

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	WEBRTC H.248 gateway: a decomposed gateway according H.248 (i.e., a MGC-MG tandem), using an H.248 profile with support of web-based real-time communication services.

NOTE – The particular gateway type may be further qualified, - based on the application specific profile -, such as WEBRTC PSTN gateway, WEBRTC border gateway, WEBRTC IMS access gateway, etc.

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| API | Application Programming Interface |
| --- | --- |
| BFCP | Binary Floor Control Protocol |
| DTLS | Datagram Transport Layer Security |
| HTTP | Hyper-text transfer protocol |
| ICE | Interactive Connectivity Establishment |
| IMS | IP multimedia subsystem |
| IP | Internet protocol |
| IPv4 | Internet protocol version 4 |
| IPv6 | Internet protocol version 6 |
| LD | Local descriptor |
| MG | Media gateway |
| MGC | Media gateway controller |
| MSRP | Message Session Relay Protocol |
| MTI | Mandatory To Implement |
| NACK | (RTP/RTCP) Negative Acknowledgements |
| NAT | Network address translation |
| NAT-T | NAT Traversal |
| NGN | Next generation network |
| PSTN | Public Switched Telephone Network |
| RD | Remote descriptor |
| RMCAT | RTP Media Congestion Avoidance Techniques (IETF WG) |
| RTCP | RTP Control Protocol |
| RTCWEB | Real-Time Communication in WEB-browsers (as work item in IETF) |
| RTP | Real-time Transport Protocol |
| SCTP | Stream Control Transmission Protocol |
| SDP | Session description protocol |
| SEP | (H.248) Stream endpoint |
| SEPP | (H.248) Stream endpoint pair |
| SIP | Session initiation protocol |
| SSRC | Synchronization Source |
| SRTP | Secure RTP |
| STUN | Session Traversal Utilities for NAT |
| TCP | Transmission control protocol |
| TLS | Transport Layer Security |
| TURN | Traversal Using Relays around NAT |
| UDP | User datagram protocol |
| VP9 |  |
| W3C | World Wide Web Consortium (http://www.w3.org/) |
| WEB | "WWW” (World Wide Web) |
| WEBRTC | Web Real-Time Communication = Real-Time Communication in WEB-browsers (as work item in IETF) |

# 5	Conventions

## 5.1	Acronym usage

The two acronyms RTCWEB and WEBRTC denote the works to support real-time communication in Web-browsers in the two different SDOs IETF and W3C: RTCWEB relates to a "protocol specification" (note: it is actually a suite of protocols) while WebRTC provides an API specification. Both are synonym from perspective of H.248 entities. This Recommendation uses the acronym WEBRTC only for the underlying communication service.

## 5.2	Prescriptive language

This document provides a list of items, labelled as R-x/y, where x refers to the clause number and y a number within that clause. Such items use the following keywords with meanings as prescribed below:

The keywords "is required to" indicate a requirement which must be strictly followed and from which no deviation is permitted if conformance to this document is to be claimed.

The keywords "is prohibited from" indicate a requirement which must be strictly followed and from which no deviation is permitted if conformance to this document is to be claimed.

The keywords "is recommended" indicate a requirement which is recommended but which is not absolutely required.  Thus this requirement need not be present to claim conformance.

The keywords "can optionally" indicate an optional requirement which is permissible, without implying any sense of being recommended. This term is not intended to imply that the vendor’s implementation must provide the option and the feature can be optionally enabled by the network operator/service provider.  Rather, it means the vendor may optionally provide the feature and still claim conformance with the specification.

# 6	WEBRTC core technology overview

## 6.1	WEBRTC gateways

The WEBRTC communication model considers usage of WEBRTC gateways besides the native browser-to-browser scenarios. [b-IETF rtcweb-gateway] describes such gateways in general, whereas the specific instance of an H.248 WEBRTC gateway is in scope of this Recommendation.

## 6.2	Capabilities

The H.248 WEBRTC gateway is peering to WEBRTC endpoints, given by IP host entities with "WEBRTC capable web browsers" as application instance. WEBRTC endpoints are characterized by following mandatory (and optional) capabilities (according to the "RTCWEB protocol specification" [IETF rtcweb-overview]):

- WEBRTC as multimedia conversational communication service with application components audio, video and data (which covers again a bunch of "data services");

- Application data transport (see [IETF rtcweb-transports]):

  - audio, video: RTP/RTCP in specific capability sets (see [IETF rtp-usage])

  - data:

- Application data framing and securing (see [IETF rtcweb-security-arch], [IETF rtcweb-secur]):

  - audio, video: SRTP

  - data: (SCTP over) DTLS

- Application data formats:

  - audio codec types (see [IETF rtcweb-audio]): Opus [b-IETF RFC 6716] and G.711 PCM µ-/A-law [b-ITU-T G.711]

  - video codec types: H.264 [b-ITU-T H.264], VP8 [b-IETF RFC 6386] (and their successor technologies H.265 [b-ITU-T H.265] and VP9 probably in future)

  - real-time text conversation (based on [ITU-T T.140])

  - instant messaging (using MSRP session-mode)

  - conference / floor control (based BFCP)

- WebRTC data channel services:

  - two data services are under consideration (see [IETF rtcweb-data-channel]):

    - unreliable data  (non-critical information includes state information)

    - reliable data  (both real-time and non-realtime data)

  - Real-time text conversation (based on [ITU-T T.140])

- NAT traversal techniques for end-to-end IP media path connectivity

  - ICE for UDP-based media transport

  - ICE for alternative TCP-based media transport

  - Latching (in case of hosted NAT traversal support [b-IETF latching])

- Quality of service

- Performance monitoring

{Editor’s note (2014-07): the overview needs to be cross-checked again in future meetings).}

## 6.3	Transport of user plane WebRTC traffic

### 6.3.1	Objective

There is the basic assumption that a WebRTC device could be located behind a NAT/FW device, and thus needs NAT/FW traversal in order to get end-to-end connectivity at IP network and transport layer. This objective results in mainly in two design aspects:

- usage of multiplexing methods in order to minimize the number of required IP transport connections;

- usage of TCP-based transport as a last resort when the preferred option of UDP-based transport is blocked due to NAT/FW behaviour.

Thus, there are two protocol stacks of consideration from H.248 WEBRTC gateway perspective.

### 6.3.2	Protocol stack for UDP-based transport

See Figure 1:

Figure 1 – User plane WebRTC protocol stack in case of UDP-based transport

Notably, there is

- SCTP/DTLS used for the transport of text conversion according to [ITU-T T.140] (instead of the native RTP-based transport according to [IETF RFC 4103]

### 6.3.3	Protocol stack for TCP-based transport

{Editor’s note (2014-11): our understanding, - RTP|RTCP-over-DTLS and NOT RTP|RTCP-over-SRTP -, if yes, then also NO DTLS-SRTP, hence not a stack like this:

Figure 2α – User plane WebRTC protocol stack in case of TCP-based transport

Status: above protocol stack is NOT used for TCP-based WebRTC -  to be confirmded}

possible alternative? =>

Figure 2β – User plane WebRTC protocol stack in case of TCP-based transport

Status: above protocol stack is NOT used for TCP-based WebRTC -  to be confirmded}

See Figure 2:

Figure 2 – User plane WebRTC protocol stack in case of TCP-based transport

Notably, there is

- DTLS-over-TCP used ("despite the fact of TLS-over-TCP as native transport security protocol"), and

- DTLS used for securing RTP ("instead of SRTP").

Hence, there are two DTLS session / DTLS connection pairs, i.e., one DTLS connection used for the transfer of all kind of SCTP traffic and the other one for RTP / RTCP traffic. Thus, the RFC 4571 Shim header is supposed to support the multiplexing / demultiplexing of three traffic components (twice DTLS plus STUN). To Be Confirmed!

{Editor’s note (2014-11): above protocol stack looks odd in two dimensions:
a) DTLS/TCP instead of native TLS/TCP and
b) RTP/SRTP/ Shim/TCP instead of RTP/DTLS/Shim/TCP.
=> below such protocol layering:

Figure 2γ – User plane WebRTC protocol stack in case of TCP-based transport

Status: above protocol stack is NOT used for TCP-based WebRTC -  to be confirmded}

# 7	H.248 WEBRTC gateways: example use cases

Some example use cases are described in this clause.

{Editor’s note (2014-11): the notion of "WebRTC endpoint" needs to be revisited due to recent discussions in IETF. Either we find some replacement or need to provide a term definition.}

## 7.1	Point-to-point communication

All examples here are characterized by a two-party call (between A and B) and that party A is always using WebRTC browser environment.

### 7.1.1	Use case #1: WebRTC-to-WebRTC interworking

See Figure 3.

Figure 3 – Use case #1: WebRTC-to-WebRTC interworking

Use case #1 relates to a browser-to-browser scenario: the WEBRTC H.248 gateway may be needed e.g. due to different NAT-T requirements in the two networks where the WebRTC endpoints reside, due to codec mismatch between both WebRTC endpoints, due to different QoS architecture of the networks where the WebRTC endpoints reside, etc.

### 7.1.2	Use case #2: WebRTC-to-NGN/IMS interworking

Figure 4 depicts an interworking scenario with NGN/IMS networks.

Figure 4 – Use case #2: WebRTC-to-NGN/IMS interworking

Use case #2 is fundamentally motivated by the fact that NGN/IMS IP user equipment is supposed to be not fully identical to WebRTC endpoints (in terms of capabilities as listed in clause 8).

### 7.1.3	Use case #3: WebRTC-to-PSTN/ISDN interworking

Figure 5 outlines the general request in supporting of still existing PSTN/ISDN infrastructures.

Figure 5 – Use case #3: WebRTC-to-PSTN/ISDN interworking

Use case #3 provides again many variants, such as

- the down-negotiation of an initial multimedia call request to an audio-only call,

- the transcoding between different audio codecs,

- the support of PSTN/ISDN multimedia terminals (e.g. [ITU-T H.324], [ITU-T H.320]).

## 7.2	Multipoint communication

Multiparty calls are in scope of WEBRTC and may demand for "RTP mixer”, as e.g. provided by media servers. Such a network scenario may therefore relate to H.248 profile types with regards to media servers (aka media resource processors).

# 8	Functional requirements for H.248 WEBRTC gateways

There are general and use case specific functional requirements for H.248 WEBRTC gateways. WEBRTC gateway requirements could be classified in subsequent functional areas.

## 8.1	Requirements related to NAT traversal support

### 8.1.1	Media latching based NAT traversal support

R-8.1.1/1: The H.248 WEBRTC gateway is required to support media latching according to the H.248 IP NAPT traversal package [ITU-T H.248.37].

NOTE – Latching as hosted NAT traversal mechanism (see [b-IETF latching]) translates to [ITU-T H.248.37] when provided by a decomposed H.248 gateway.

R-8.1.1/2: The H.248 WEBRTC gateway is recommended to support the media latching associated address reporting capability according to the H.248 address reporting package [ITU-T H.248.37].

{Editor’s note (2014-03 meeting): depends on further anlaysis related to ICE-based address reporting …}

### 8.1.2	ICE based NAT traversal support

IP transport protocols:

R-8.1.2/1: The H.248 WEBRTC gateway is required to support ICE for UDP connections, according to [ITU-T H.248.50].

R-8.1.2/2: The H.248 WEBRTC gateway is required to support ICE for TCP connections, according to [ITU-T H.248.50].

{Editor’s note (2014-03 meeting): the ICE related requirements should be more detailed … as well as required associated H.248.50 building blocks…}

ICE mode:

R-8.1.2/3: The H.248 WEBRTC gateway is required to support basically ICE-full mode (see clause 3.4/[IETF rtcweb-transports]), according to [ITU-T H.248.50].

R-8.1.2/4: The H.248 WEBRTC gateway is required to support ICE-lite mode only (according to [ITU-T H.248.50]), dependent on the location of the H.248 WEBRTC gateway in the end-to-end path and/or the "NAT traversal" relevant network architecture.

Trickle ICE:

R-8.1.2/5: The H.248 WEBRTC gateway is required to support the ICE extensions "trickle ICE" (see [b-IETF trickle-ice]), according to [ITU-T H.248.50].

IPv4/IPv6 dual stack fairness extension for ICE:

R-8.1.2/5: The H.248 WEBRTC gateway can optionally support the ICE extensions "IPv4/IPv6 dual stack fairness" (see [b-IETF ice-dualstack]), according to [ITU-T H.248.50].

{Editor’s note (2014-11 meeting): premature requirement due to IETF status of this proposal.}

## 8.2	Requirements related to communication topologies

### 8.2.1	H.248 WEBRTC gateway for point-to-point interworking

R-8.2.1/1: The H.248 WEBRTC gateway is required to support an H.248 (IP, IP) or (IP, non-IP connection models (use cases #1, 2 and #3).

### 8.2.2	H.248 WEBRTC gateway as WEBRTC conferencing point

R-8.2.2/1: The H.248 WEBRTC gateway is required to support the conferencing extensions for RTP media as indicated in clause 5.1/[IETF rtp-usage].

## 8.3	Requirements related to bearer traffic multiplexing

R-8.3.1/1: The H.248 WEBRTC gateway is required to support at least two UDP transport connections for a single, multiplexed WEBRTC multimedia call.

NOTE – This is the bearer configuration with a maximum degree of multiplexing. All audio and video traffic share a single UDP transport connection by usage of RTP transport and media multiplexing. And all data traffic is sharing a single UDP transport connection with a single DTLS session with a single SCTP association.

## 8.4	Requirements related to RTP-to-RTP type interworking

### 8.4.1	RTCP control flow component

R-8.4.1/1: The H.248 WEBRTC gateway is required to support always RTCP control flow besides the RTP media flow (see clause 4.1/[IETF rtp-usage]).

Support of RTCP (i.e., the handling and processing of specific RTCP packet types) is dependent on the RTP topology used for the H.248 WEBRTC gateway (see clause 8.4.4). Following requirements are applicable for the example of the RTP topology "Back-to-back RTP end system (B2BRE)":

R-8.4.1/2: Basic RTCP services: The H.248 WEBRTC gateway is required to support sending and receiving RTCP SR, RR, SDES, and BYE packet types.

R-8.4.1/3: RTP profile dependent RTCP services: The H.248 WEBRTC gateway is required to support sending and receiving RTCP RTPFB (Generic RTP Feedback) and PSFB (Payload-specific Feebdack) packet types.

R-8.4.1/4: Supplementary RTCP services: The H.248 WEBRTC gateway could optionally support sending and receiving RTCP XR (extended report) packet types with respect to performance monitoring of RTP traffic (see also [ITU-T H.248.87]).

### 8.4.2	RTP profiles

R-8.4.2/1: The H.248 WEBRTC gateway is required to support RTP profile "RTP/SAVPF" according to [IETF RFC 5124, IETF RFC 7007] (see clause 4.2/[IETF rtp-usage]).

### 8.4.3	RTP multiplexing

R-8.4.3/1: The H.248 WEBRTC gateway is required to support RTP media multiplexing ("SSRC multiplexing") (see clause 4.4/[IETF rtp-usage]).

NOTE 1 – The multiplexing mode will only be used when all participants agree. If not, unmultiplexed mode will be used. Thus, any MGC involved in the call control level end-to-end capability negotiations could actually downgrade to unmultiplexed mode.

R-8.4.3/2: The H.248 WEBRTC gateway is required to support RTP transport multiplexing ("RTP / RTCP transport multiplexing") (see clause 4.5/[IETF rtp-usage]).

NOTE 2 – Same comment as in NOTE 1: backward compatibility requires to support unmultiplexed mode as well.

### 8.4.4	RTP topologies

NOTE – All required RTP topologies are already outlined by [ITU-T H.248.88].

R-8.4.4/1: Use case independent: the H.248 WEBRTC gateway is required to comply with the "RTP topology" behaviour according to [ITU-T H.248.88].

R-8.4.4/2: Use case #1: the H.248 WEBRTC gateway is required to support topology

- "RTP transparent forwarding",

- "RTP transport translator" or

- "RTP media translator",

dependent on the call-level end-to-end negotiation of the used "RTP configuration" for WebRTC.

R-8.4.4/3: Use case #2: The H.248 WEBRTC gateway is required to support, in addition to RTP topologies of use case #1, the toplogy

- "Back-to-back RTP end system (B2BRE)"

due to the "legacy usage of RTP" in the "NGN/IMS IP domain".

NOTE – All existing H.248 profiles for border gateways are not explicit on RTP topologies in detail. Thus, the B2BRE topology represents a default topology (due to the back-to-back IP host configuration of an RTP-RTP context).

R-8.4.4/4: Use case #3: The H.248 WEBRTC gateway is required to support the "RTP endsystem" (RTPE) topology (see clause 7.1/[ITU-T H.248.88]).

### 8.4.5	RTCP based services, report types and packet formats

R-8.4.5/1: The H.248 WEBRTC gateway is required to support reduced size RTCP according to [IETF RFC 5506] (see clause 4.6/[IETF rtp-usage]).

### 8.4.6	Resource allocation rules for RTP/RTCP

#### 8.4.6.1	Port allocation rules for RTP and RTCP

R-8.4.6.1/1: The H.248 WEBRTC gateway is required to support symmetric RTP/RTCP according to [IETF RFC 4961] (see clause 4.7/[IETF rtp-usage]), which relates to the H.248-controlled port allocation as defined by [ITU-T H.248.57].

#### 8.4.6.2	SSRC allocation rules for RTP

R-8.4.6.2/1: The H.248 WEBRTC gateway is required to support signalled RTP synchronisation source (SSRC) identifiers according to [IETF RFC 5576] (see clause 4.8/[IETF rtp-usage]).

#### 8.4.6.3	CNAME allocation rules for RTP/RTCP

R-8.4.6.2/1: The H.248 WEBRTC gateway is required to support unique RTCP CNAME value(s) (see clause 4.9/[IETF rtp-usage]).

NOTE – A single CNAME might be sufficient for a single H.248 MG entity.

R-8.4.6.2/2: The H.248 WEBRTC gateway can optionally be requested to report received RTCP CNAME value(s) from remote WebRTC clients (according to the RTCP Source Description package as defined by [ITU-T H.248.71].

## 8.5	Requirements related to TCP-based media transport

{Editor’s note (2014-11 meeting): this sub-clause is a placeholder, still dependent on the finally required protocol stack, see discussion in clause 6.3.3 …}

### 8.5.1	Requirements related to ###

#### 8.5.1.1	Requirements related to ####

R-8.5.1.1/1: ###

[IETF rtcweb-transports]

RFC 6544:

ICE-TCP candidates [RFC6544] MUST be supported; this may allow

applications to communicate to peers with public IP addresses across

UDP-blocking firewalls without using a TURN server.

If TCP connections are used, RTP framing according to [RFC4571] MUST

be used, both for the RTP packets and for the DTLS packets used to

carry data channels.

## 8.5	Requirements related to media-aware type of interworking

### 8.5.1	Requirements related to WEBRTC audio

#### 8.5.1.1	Requirements related to media format (audio)

R-8.5.1.1/1: The H.248 WEBRTC gateway is required to support audio transcoding between the WEBRTC MTI audio codec(s) and other audio codecs as typically used in NGNs.

#### 8.5.1.2	Requirements related to media adaptation and rate control (audio)

Requirements are subject of clause 7/[IETF rtp-usage], however, there are not yet any explicit capabilities identified for the first phase of WebRTC service deployment. Correspondent capabilities are currently developed by IETF working group RMCAT (RTP Media Congestion Avoidance Techniques), see e.g. requirements in [b-IETF rmcat-cc-requirements], which are planned to be integrated later on in WEBRTC.

### 8.5.2	Requirements related to WEBRTC video

#### 8.5.2.1	Requirements related to media format (video)

R-8.5.2.1/1: The H.248 WEBRTC gateway as IP-IP media gateway is not required to support video transcoding (because, if required at all, provided by centralized media servers).

R-8.5.2.1/2: The H.248 WEBRTC gateway as media server can optionally support video transcoding between the WEBRTC video codec(s) and other video codecs as typically used in NGNs.

NOTE – The optional tagging of this requirement is due to the fact that it is expected that potential scenarios for video transcoding might be fairly low and exceptional. It is rather expected that WebRTC endpoints will negotiate a common video codec, or fallback to a default codec, or even not use video.

#### 8.5.2.2	Requirements related to media adaptation and rate control (video)

Status: see clause 8.5.1.2.

### 8.5.3	Requirements related to WEBRTC data

#### 8.5.3.1	Requirements for MSRP-based instant messaging

R-8.5.3.1/1: The H.248 WEBRTC gateway is required to support transparent forwarding of MSRP protocol data units ("MSRP messages"), independent of the underlying (WebRTC specific or legacy) transport (protocol stack). This includes interworking between WebRTC clients or a WebRTC and non-WebRTC client.

R-8.5.3.1/2: The H.248 WEBRTC gateway is required to support the underlying transport relay function in interworking between "SCTP/DTLS/UDP" to "TCP" for MSRP traffic.

R-8.5.3.1/3: The H.248 WEBRTC gateway can optionally support a bearer level application gateway (B-ALG) at MSRP layer according to [ITU-T H.248.78]. Such a capability relates to a specific MSRP B2BUA function with the purpose of a L4+ level NAT traversal support.

#### 8.5.3.2	Requirements for BFCP-based conference control

R-8.5.3.2/1: The H.248 WEBRTC gateway is required to support transparent forwarding of BFCP protocol data units ("BFCP messages"), independent of the underlying (WebRTC specific or legacy) transport (protocol stack). This includes interworking between WebRTC clients or a WebRTC and non-WebRTC client.

R-8.5.3.2/2: The H.248 WEBRTC gateway is required to support the underlying transport relay function in interworking between "SCTP/DTLS/UDP" to "TCP" for BFCP traffic.

### 8.5.4	Requirements related to WEBRTC text

WebRTC text telephony (text conversation) is considered as "data traffic" (despite the fact of the usual RTP-based transport in non-WebRTC environments).

R-8.5.4/1: The H.248 WEBRTC gateway is required to support transparent forwarding of T.140 protocol data units (according to [ITU-T T.140]), independent of the underlying (WebRTC specific or legacy) transport (protocol stack). This includes interworking between WebRTC clients or a WebRTC and non-WebRTC client.

R-8.5.4/2: The H.248 WEBRTC gateway is required to support the underlying transport relay function in interworking between "SCTP/DTLS/UDP" to "RTP/UDP" for T.140 traffic.

## 8.6	Requirements related to bearer security

### 8.6.1	Requirements related to transport security

### 8.6.1.1	Protocol stacks with DTLS

R-8.6.1.1/1: The H.248 WEBRTC gateway is required to support transport security for UDP using DTLS, according to [ITU-T H.248.93].

R-8.6.1.1/2: The H.248 WEBRTC gateway is required to use a self-contained DTLS session for SCTP-encapsulated WebRTC data traffic, according to [ITU-T H.248.93, H.248.SCTP].

R-8.6.1.1/3: The H.248 WEBRTC gateway is required to a separate DTLS session for SRTP related key exchange procedures (for WebRTC audio and video traffic), according to [ITU-T H.248.93, H.248.77].

{Editor’s note (2014-11 meeting): the update of above requirements is related to the (D)TLS terminology clarification and framework, see AVD-4600. There are basically following mapping models concerning the dedicated traffic of above two requirements:

Figure:  WebRTC service – Possible mapping models (for discussion)

Conclusion: either mapping I or II, but not III, due to the assumption of two, separate DTLS sessions. This conclusion needs to be confirmed.

NOTE – discussion was progressed in recent 3GPP meeting, see C4-141698
http://www.3gpp.org/ftp/tsg_ct/WG4_protocollars_ex-CN4/TSGCT4_66bis_Sophia_Antipolis/Docs/C4-141698.zip }

#### 8.6.1.2	DTLS interworking scenarios

Transport of data traffic in a native WEBRTC domain relates to the protocol layering of "data-over-SCTP/DTLS/UDP/IP", whereas transport of data traffic in the native Internet uses basically "data-over-TCP/IP". The H.248 WEBRTC gateway needs to address interworking between both network paradigms (see also use cases #1 and #2 in clause 7.1), leading to following requirements:

R-8.6.1.2/1: The H.248 WEBRTC gateway is required to support DTLS transparent forwarding for the interworking between two network domains with SCTP/DTLS/UDP based WEBRTC data transport. (relates to the use case according to clause 6.2.2.1/[ITU-T H.248.93]).

R-8.6.1.2/2: The H.248 WEBRTC gateway is required to support DTLS-to-non-DTLS interworking for the interworking between two network domains with SCTP/DTLS/UDP and (TLS)/TCP based WEBRTC data transport. (relates to the use case #1.2 according to clause 6.2.1/[ITU-T H.248.93]).

#### 8.6.1.3	TLS support

R-8.6.1.3/1: The H.248 WEBRTC gateway is required to support TLS termination in case of the DTLS interworking scenario according to R-8.6.1.2/2 and transport security usage in the TCP domain.

### 8.6.2	Requirements related to media security

R-8.6.2/1: The H.248 WEBRTC gateway is required to support media security for RTP using SRTP, according to [ITU-T H.248.77].

{Editor’s note (2014-03 meeting): the specific H.248.77 use cases should be detailed …
(2014-11 meeting): looks like an extension for srtp package is required, see AVD-4619 …}

R-8.6.2/2: The H.248 WEBRTC gateway is required to support the media-path coupled SRTP key exchange (using DTLS-SRTP [IETF RFC 5764], see [b-IETF RFC 5763]) according to requirement [R-8.6.1/2].

R-8.6.2/3: The H.248 WEBRTC gateway can optionally support the signalling-path coupled SRTP key exchange, i.e., the "SDES" (SDP security descriptions) related method according to [ITU-T H.248.77].

## 8.7	Requirements related to WEBRTC emergency services

From IETF side, requirements for WEBRTC emergency services are so far contained in the (expired) draft [b-IETF rtcweb-ecrit]. There may not any additional requirements for H.248 WEBRTC gateways derived, - taking this initial document as baseline.

## 8.8	Requirements related to QoS support

### 8.8.1	IP traffic control: QoS marking

R-8.8.1/1: The H.248 WEBRTC gateway is required to support DiffServ based QoS marking according to [ITU-T H.248.52] (see clause x.x/[IETF rtp-usage]).

NOTE – The H.248 WEBRTC MG would then provide the so-called "DS pre-marker" role (see Appendix IV/[ITU-T H.248.52]).

### 8.8.2	IP traffic control: traffic policing – IP byterate policing

R-8.8.2/1: The H.248 WEBRTC gateway is required to support IP byterate policing according to [ITU-T H.248.53].

### 8.8.3	Improved transport robustness for RTP

See clause 6/[IETF rtp-usage].

#### 8.8.3.1	Negative acknowledgements

In general:

R-8.8.3.1/1: The H.248 WEBRTC gateway is recommended to support negative acknowledgements (NACKs) for RTP data packets [IETF RFC 4585] (see clause 6.1/[IETF rtp-usage]).

In particular:

The origination and termination of RTCP Feedback (FB) messages is subject of RTP sender and RTP receiver entities [IETF RFC 4585], thus tightly coupled to "RTP topologies" (see clause 8.4.8). Hence, an H.248 WEBRTC gateway, as located within an end-to-end RTP media path, is normally only partially involved in the processing of RTCP FB messages. Following high level behaviour is supposed:

R-8.8.3.1/2: The H.248 WEBRTC gateway is recommended to support the forwarding of RTCP FB messages when enabled in RTP translator topology.

R-8.8.3.1/3: The H.248 WEBRTC gateway is recommended to originate and terminate RTCP FB messages when enabled in Back-to-back RTP end system topology.

#### 8.8.3.2	Forward Error Correction

In general:

R-8.8.3.2/1: The H.248 WEBRTC gateway can optionally support Forward Error Correction (FEC) for RTP [IETF RFC 5109] (see clause 6.2/[IETF rtp-usage]).

In particular:

Roughly, an RTP sender generates the usual RTP "media packets" plus RTP "FEC packets", which are evaluated again by the RTP receiver in case of lost RTP "media packets". And there is an underlying assumption of unmodified RTP packets along the RTP media path. Thus, FEC becomes useless in case of interim RTP entities which would modify RTP packets (to be confirmed). Leading to following high-level behaviour:

R-8.8.3.2/2: The H.248 WEBRTC gateway is required to support the transparent forwarding of RTP traffic when enabled in RTP translator topology, - i.e., when FEC is required, then only RTP transport translator topology would be allowed.

R-8.8.3.2/3: The H.248 WEBRTC gateway is recommended to originate and terminate RTP "FEC packets" when enabled in Back-to-back RTP end system topology.

## 8.9	Requirements related to bearer-path coupled congestion controls

These requirements are tightly coupled with the capability of "media adaptation and rate control" (see clause 8.5). Specific requirements are not yet settled from IETF side for the first release of this Recommendation.

## 8.10	Requirements related to performance monitoring

The requirements framework in this clause follows the [ITU-T H.248.87] "guidelines on the use of ITU-T H.248 capabilities for performance monitoring in RTP networks".

Browser-based WEBRTC endpoints may likely support the local generation of measurements for performance metrics according to [b-IETF rtcweb-xr]. The bowser-embedded measurement point and the set of supported WebRTC's statistics is defined by [b-W3C webrtc-stats].

### 8.10.1	Requirements related to measurement point (MP)

Generally, the set of performance measurements differs typically between WEBRTC endpoints (such as a WEBRTC browser) and WEBRTC gateways, e.g., because the observation of application level performance metrics is primary subject of communication endpoints and normally questionable or even meaningless in case of "in-path equipment".

R-8.10.1/1: The H.248 WEBRTC gateway is recommended to execute measurements for  transport level performance metrics (term definition see clause 3.2.6/[ITU-T H.248.87]).

R-8.10.1/2: The H.248 WEBRTC gateway can optionally execute measurements for application level performance metrics (definition see clause 3.2.1/[ITU-T H.248.87]), - under the condition of a valuable semantic of the concerned metric.

Example metrics:

- RTP based WEBRTC traffic:

  - see Appendix III/[ITU-T H.248.87] for a general overview

  - RTCP XR related metrics are recommended (e.g., H.248 nt / rtp package statistics are deprecated)

- Non-RTP based WEBRTC traffic: e.g. transport level metrics related to

  - DTLS (see [ITU-T H.248.93]),

  - SCTP (see [b-ITU-T H.248.SCTP]),

  - TLS (see [ITU-T H.248.90]) and

  - TCP (see [ITU-T H.248.84, ITU-T H.248.90]).

The actual set of performance metrics for a particular H.248 "WEBRTC gateway" profile depends, as usual, on the specific purpose, such as

- usage metering (relates typically to the traffic volume on the application level);

- reporting of user experience related metrics (i.e., QoS/QoE related metrics);

- reporting of network condition related metrics (i.e., GoS related metrics);

- recording of successful, unsuccessful or specific policing actions; or

- validation of network capacity allocations (relates typically to the traffic volume on the lowest layer of transport capacity reservation)

The primary focus is most likely (c) in case of a H.248 WEBRTC gateway due to its positioning at the border of network domains (with typically different Grade of Service conditions).

### 8.10.2	Requirements related to collection point (CP)

The set of supported performance metrics, which should be measured by WebRTC endpoints (as peered by H.248 WEBRTC gateway), is still undefined at release of this Recommendation. It is e.g. expected that [b-IETF rtcweb-xr] might be supported for the RTP based WEBRTC traffic components. Performance metrics for non-RTP WEBRTC traffic is still unclear.

When WebRTC endpoints support the measurement of RTCP XR based metrics, then it is expected that they report their measurements via RTCP along the WebRTC media path.

R-8.10.2/1: The H.248 WEBRTC gateway is required to collect remote measurement data via incoming RTCP reports.

### 8.10.3	Requirements related to reporting point (RP)

R-8.10.3/1: The H.248 WEBRTC gateway is required to support the reporting of RTCP XR related H.248 statistics according to the RTCP XR block reporting package [ITU-T H.248.48].

### 8.10.4	Requirements related to filtering point (FP)

Measurement data carried by RTCP XR reports could be basically filtered by H.248 WEBRTC gateways, see [ITU-T H.248.87]. However, the particular H.248 WEBRTC gateway filter behaviour is subject of operator policies, thus out of scope of this Recommendation.

### 8.10.5	Requirements related to loopback point (LP)

None.

## 8.11	Requirements related to SDP-based description, declaration and negotiation of WEBRTC media configurations

### 8.11.1	Requirements related to supported SDP elements – Overall media descriptions

The H.248 WEBRTC gateway is required to support SDP elements (for the purpose of media gateway control) according to Tables 1, 2 and 3:

Table 1 – Requirements related to supported SDP elements – Part I SDP "m="-line

| Requirement |  |  | SDP "m=" line element |  |  | Purpose |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Media type values (in case of media type aware interworking modes): |  |  |  |  |  |  |
| R-8.11.1.1/1: | media type = audio |  |  | WEBRTC audio component |  |  |
| R-8.11.1.1/2: | media type = video |  |  | WEBRTC video component |  |  |
| R-8.11.1.1/3: | media type = application |  |  | WEBRTC data components |  |  |
| Transport protocol (stack) values: |  |  |  |  |  |  |
| R-8.11.1.1/4: | proto = RTP/SAVPF |  |  | RTP profile for all RTP-based traffic components |  |  |
| R-8.11.1.1/5: | proto = DTLS/SCTP |  |  | WEBRTC data traffic components (NOTE 1) |  |  |
| Media format values (in case of media format aware interworking modes) (NOTE 2): a) audio formats (from WebRTC endpoint perspective, see [b-IETF rtcweb-audio]): |  |  |  |  |  |  |
| R-8.11.1.1/6: | OPUS | Internet audio code [b-IETF RFC 6716] |
| R-8.11.1.1/7: | PCMA / PCMU & CN | voice G.711 µ/A-law with optional comfort noise |
| R-8.11.1.1/8: | telephone-event (event codepoints 0 to 11) | audio/telephone-event media format as specified in [b-IETF RFC 4733] for DTMF |
| b) audio formats (optional, from NGN/IMS endpoint perspective, see use case #2): |  |  |
| R-8.11.1.1/9: | AMR-WB | Adaptive Multi-Rate Wideband [b-IETF RFC 4867] |
| R-8.11.1.1/10: | EVRC | Enhanced Variable Rate Codec [b-IETF RFC 4788] |
| c) video formats (for consideration, see [b-IETF rtcweb-video]): |  |  |
| R-8.11.1.1/11: | VP8 | video codec [b-IETF RFC 6386], [b-IETF rtp-vp8] |
| R-8.11.1.1/12: | H264 | video codec [b-IETF RFC 6184] |
| R-8.11.1.1/13: | VP9 | video codec [b-IETF codec-vp9] |
| R-8.11.1.1/14: | H265 | video codec [b-IETF rtp-h265] |
| d) text formats (for consideration): |  |  |
| R-8.11.1.1/15: | t140 | conversational text [b-IETF RFC 4103] |
| e) WebRTC data channel formats |  |  |
| R-8.11.1.1/16: | webrtc-datachannel | generic data channel, realized as SCTP stream over a DTLS session (NOTE 3). |
| NOTE 1 – SDP element defined by [b-IETF sdp-sctp]. NOTE 2 – Codec technologies are continuously improving, leading to a limited timeline of usage and their future replacement by evolved codecs. Thus, all listed media format specific types are not mandatory, but are listed in order to provide concrete use cases for the SDP-based parameterization of such codecs in H.248 WEBRTC gateways (see clause 8.11.2). Mandatory to implement (MTI) type of media formats are out of scope of this Recommendation, rather subject of concrete H.248 profile specifications. NOTE 2 – SDP "m="-line media format value is defined by [b-IETF sdp-sctp]. (to be confirmed) |  |  |

{Editor’s note (2014-11 meeting): above tables are still in progress, dependent on H.248.SCTP, H.248.STGROUP, H.248.RTPMUX}

Table 2 – Requirements related to supported SDP elements – Part II SDP "a="-line

| Requirement | SDP element |  | Purpose |  |
| --- | --- | --- | --- | --- |
| NAT traversal support for media / bearer path: |  |  |  |  |
| R-8.11.1.2/1: | a=ice-ufrag | ICE credentials for media level STUN authentication procedure |
| R-8.11.1.2/2: | a=ice-pwd |  |
| R-8.11.1.2/3: | a=candidate: … TCP … | in case of last resort of TCP-based media transport (instead of UDP)) |
| Security in the media / bearer path: |  |  |  |  |
| R-8.11.1.2/4: | a=fingerprint |  | Authentication procedure for WEBRTC DTLS sessions. |  |
| R-8.11.1.2/5: | a=crypto |  | in case of optional SDES-based SRTP |  |
| Traffic multiplexing in the media / bearer path: |  |  |  |  |
| R-8.11.1.2/6: | a=group:BUNDLE |  | General media grouping using "bundle" (NOTE 1). |  |
| R-8.11.1.2/7: | a=mid: |  |  |  |
| R-8.11.1.2/8: | a=rtcp-mux |  | for transport multiplexed RTP traffic [ITU-T H.248.57] |  |
| R-8.11.1.2/9: | a=rtcp |  | optional, if transport unmultiplexed mode for RTP and explicit RTCP transport address allocation [ITU-T H.248.57] |  |
| R-8.11.1.2/10: | a=ssrc: | Not required at H.248 interface (to be confirmed) |
| WEBRTC usage of RTP / RTCP: |  |  |
| R-8.11.1.2/11: | a=rtcp-rsize | Reduced-size RTCP [IETF RFC 5506] |
| R-8.11.1.2/12: | a=rtcp-fb | [IETF RFC 5104] (to be confirmed) |
| WEBRTC data traffic configuration: {Editor’s note (2014-11 meeting): below information is preliminary, not yet mature due to the ongoing discussion in the IETF (MMUSIC, DISPATCH) on SDP elements related to the configuration of WEBRTC data traffic (see draft-ejzak-mmusic-data-channel-sdpneg, draft-ejzak-dispatch-msrp-data-channel, draft-ietf-clue-datachannel)} |  |  |
| R-8.11.1.2/13: | a=fmtp:webrtc-datachannel … |  | Usage of an SCTP stream as "WEBRTC data channel" (NOTE 2) |  |
| R-8.11.1.2/14: | a=sctp-port:… | The actual SCTP port value of the SCTP association carried over DTLS/UDP, which is used for "WEBRTC data channels" (NOTE 2) |
| R-8.11.1.2/15: | a) application-agnostic: a=dcmap:<SCTP Stream ID> |  | Assignment of a particual SCTP stream to a WEBRTC data traffic component (NOTE 3) without any indication of the subprotocol. |  |
| R-8.11.1.2/16: | a) application-aware a=dcmap:<SCTP Stream ID> subprotocol="…" | Assignment of a particual SCTP stream to a WEBRTC data traffic component (NOTE 3) with indication of the subprotocol. |
| R-8.11.1.2/17: | a=dcsa:… |  | application-specific information of a particular subprotocol asWEBRTC data channel (NOTE 4, NOTE 5). |  |
| NOTE 1 – SDP element defined by [b-IETF sdp-bundle]. NOTE 2 – SDP element defined by [b-IETF sdp-sctp]. NOTE 3 – SDP element defined by [b-IETF data-channel-sdpneg]. NOTE 4 – SDP element defined by [b-IETF data-channel-sdpneg]. NOTE 5 – See [b-IETF data-channel-msrp] concerning the SDP profiling for the example usage of a WebRTC data channel for application protocol "MSRP". |  |  |  |  |

Table 3 – Requirements related to supported SDP elements – Part III other SDP line s

| Requirement | SDP element | Purpose |
| --- | --- | --- |
| Transport capacity reservation and allocation – H.248 Stream admission control: |  |  |
| R-8.11.1.3/1: | "b="-line | see [IETF rtp-usage] {which bandwidth modifiers? RR, RS, AS, CT, …} |

{Editor’s note (2014-11 meeting): above tables are still in progress, dependent on H.248.SCTP, H.248.STGROUP, H.248.RTPMUX}

### 8.11.2	Requirements related to supported SDP elements – Codec configurations

Table 1 introduced the required media formats ("codecs"), which usually include a set of parameters concerning their configuration. These parameters are signalled via SDP from MGC to MG and affect the RTP packet encapsulation ("internal structure of the protocol data unit"), codec processing and RTCP handling, - dependent on media-aware / media-agnostic modes of operation of the H.248 WEBRTC gateway.

### 8.11.2.1	SDP elements related the parameterization of audio codec "OPUS"

See clause 2.1/[b-IETF RFC 6716]:

The Opus codec includes a number of control parameters that can be changed dynamically during regular operation of the codec, without interrupting the audio stream from the encoder to the decoder. These parameters only affect the encoder since any impact they have on the bitstream is signaled in-band such that a decoder can decode any Opus stream without any out-of-band signaling. Any Opus implementation can add or modify these control parameters without affecting interoperability. The most important encoder control parameters in the reference encoder are listed below.

The H.248 WEBRTC gateway, when enabled for media aware interworking, would provide such an encoder and decoder function, hence is required to support SDP elements (for the purpose of media gateway control) according to Table 4. [b-IETF rtp-opus] defines the formal codec parameters (in clause 6.1) and their mapping on SDP elements (in clause 6.2).

Table 4 – SDP parameters for IANA registered media type "audio/opus"

| Requirement | Codec parameter | SDP parameter |  | M/O |  | Value range |  | Default |  | Comments and H.248 profile |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| R-8.11.2.1/1: | rate ("sampling rate")) | a=rtpmap:… opus/48000/2 | M | 8000, 12000, 16000, 24000, 48000 | 48000 (?) | all |
| R-8.11.2.1/2: | maxplaybackrate | a=fmtp: (NOTE 1) | O |  |  | all (tbd) |
| R-8.11.2.1/3: | sprop-maxcapturerate | a=fmtp: (NOTE 1) | O |  | 48000 |  |
| R-8.11.2.1/4: | maxptime | a=maxptime | O | 3, 5, 10, 20, 40, 60, or an arbitrary multiple of an frame size rounded up to the next full integer value, up to a max. value of 120 |  | When "media transcoding" mode, then it is recommended to align these parameter values between the OPUS and non-OPUS RTP domain in order to minimize end-to-end latency and to avoid interworking complexity. |
| R-8.11.2.1/5: | ptime | a=ptime | O |  |  |  |
| R-8.11.2.1/6: | minptime | a=fmtp: (NOTE 1) | O |  |  |  |
| R-8.11.2.1/7: | maxaveragebitrate | a=fmtp: (NOTE 1) |  | O |  | Any positive integer is allowed, but values outside the range 6000 to 510000 SHOULD be ignored |  |  |  |  |  |
| R-8.11.2.1/8: | stereo | a=fmtp: (NOTE 1) | O | 0 (mono), 1 (stereo) | 0 (mono) |  |
| R-8.11.2.1/9: | sprop-stereo | a=fmtp: (NOTE 1) | O |  | 0 (mono) |  |
| R-8.11.2.1/10: | cbr | a=fmtp: (NOTE 1) |  | O |  | 1 (constant bitrate), 0 (variable bitrate) |  | 0 (vbr) |  |  |  |
| R-8.11.2.1/11: | useinbandfec | a=fmtp: (NOTE 1) |  | O |  | 0 (no FEC), 1 (FEC) |  | 0 (no FEC) |  |  |  |
| R-8.11.2.1/12: | usedtx | a=fmtp: (NOTE 1) |  | O |  | 0 (no DTX), 1 (DTX) |  | 0 (no DTX) |  |  |  |
| NOTE 1 – Media type string in the form of a semicolon-separated list of parameter=value pairs, as part of the "a=fmtp" SDP attribute. E.g., a=fmtp:101 maxplaybackrate=16000; sprop-maxcapturerate=16000; maxaveragebitrate=20000; stereo=1; useinbandfec=1; usedtx=0 |  |  |  |  |  |  |  |  |  |  |  |  |

{Editor’s note (2014-11 meeting): above table format may be subject of revisions. E.g., there might be alternative table structures (see parallel proposal AVD-4633 "H.248.CodecSDPprofile: Profiling codec configurations via SDP in H.248 profiles – Guidelines and examples (initial skeleton draft)")}

### 8.11.3	Requirements related to SDP Offer/Answer

WEBRTC does not mandate a specific call control signalling protocol. Thus, there are not any guidelines for SDP offer/answer from IETF side in case of SIP as WEBRTC call control protocol.

In case of SIP networks with H.248 WEBRTC gateways, handling of SDP offer/answer procedures between call control level and their representation at H.248 gateway control level is outlined by [ITU-T H.248.80], particular when the revised SDP offer/answer model would be preferred.

# 9	H.248 profile specification guidelines

This clause provides guidelines for H.248 profile specifications. The structure follows the profile template according to Appendix III/[ITU-T H.248.1].

The template elements which are not applicable in this Recommendation are indicated by "Subject to profile specification".

Any profile guidelines are primarily dependent on the concerned network configuration and use case. The guidelines in this clause are therefore basically conditional. Two exemplary use cases are considered (as described in Appendix I), termed as capability set CSA and CSB.

## 9.1	Profile Identification

Subject to profile specification.

## 9.2	Summary

Subject to profile specification.

## 9.3	Gateway Control Protocol Version

Subject to profile specification.

## 9.4	Connection Model

| Maximum number of contexts: | Subject to profile specification. |
| --- | --- |
| Maximum number of terminations per context: | Subject to profile specification. Examples: IF CSA THEN "2". IF CSB THEN "2". |
| Allowed termination type combinations in a context: | Subject to profile specification. |

## 9.5	Context Attributes

Subject to profile specification.

## 9.6	Terminations

Subject to profile specification.

## 9.7	Descriptors

### 9.7.1	TerminationState Descriptor

Subject to profile specification.

### 9.7.2	Stream Descriptor

Subject to profile specification.

### 9.7.3	Events Descriptor

IF CSA OR CSB THEN following table:

FIXTHIS

All other aspects related to Events Descriptor (e.g., EventBuffer Control, KeepActive, Notification Behaviour) are "Subject to profile specification".

### 9.7.4	EventBuffer Descriptor

Subject to profile specification.

### 9.7.5	Signals Descriptor

IF CSA OR CSB THEN following table:

FIXTHIS

All other aspects related to Signals Descriptor (e.g., Signal Direction, Signal List) are "Subject to profile specification".

### 9.7.6	DigitMap Descriptor

Subject to profile specification.

### 9.7.7	Statistics Descriptor

IF CSB THEN none (tbc).

IF CSA THEN following table entries:

FIXTHIS

### 9.7.8	ObservedEvents Descriptor

Subject to profile specification.

### 9.7.9	Topology Descriptor

Subject to profile specification.

### 9.7.10	Error Descriptor

Subject to profile specification.

FIXTHIS

## 9.8	Command API

NOTE – It is assumed that an Error Descriptor may be returned in any command reply.

### 9.8.1	Add

Subject to profile specification.

### 9.8.2	Modify

Subject to profile specification.

### 9.8.3	Subtract

Subject to profile specification.

### 9.8.4	Move

Subject to profile specification.

### 9.8.5	AuditValue

Subject to profile specification.

### 9.8.6	AuditCapabilities

Subject to profile specification.

### 9.8.7	Notify

Subject to profile specification.

### 9.8.8	ServiceChange

Subject to profile specification.

### 9.8.9	Manipulating and auditing context attributes

Subject to profile specification.

## 9.9	Generic command syntax and encoding

Subject to profile specification.

## 9.10	Transactions

Subject to profile specification.

NOTE – There is no impact on Transactions.

## 9.11	Messages

Subject to profile specification.

## 9.12	Transport

Subject to profile specification.

NOTE – Usage of bearer security may demand for an secured H.248 transport mode, too.

## 9.13	Security

Subject to profile specification.

## 9.14	Packages

### 9.14.1	Mandatory Package

Mandatory: specifies the packages that shall be supported in this profile.

Examples:

IF CSA OR CSB THEN "following table":

| Mandatory packages: |  |  |  |
| --- | --- | --- | --- |
| Package name | PackageID | Version | Termination Types Supported |
| "IP NAT traversal package" [ITU-T H.248.37] | ipnapt (0x0099) | v1 | ANY |
| "MG Act-as STUN server package" [ITU-T H.248.50] | mgastuns (0x00c2) | v1 | ANY |
| "Originate STUN continuity check package" [ITU-T H.248.50] | ostuncc (0x00c3) | v1 | ANY |
| "TLS basic session control package" [ITU-T H.248.90] | tlsbsc (0x0117) | v1 | ANY; Stream type: "DTLS-enabled UDP transport connection" |
| FIXTHIS … H.248.SCTP |  |  |  |

### 9.14.2	Optional Package

Examples:

IF CSA OR CSB THEN "following table":

| Optional packages: |  |  |  |
| --- | --- | --- | --- |
| Package name | PackageID | Version | Termination Types Supported |
| "Stream endpoint interlinkage package" [ITU-T H.248.92] | seplink (0x011b) | v1 | ANY |

Examples:

IF CSA THEN "following table":

| Optional packages: |  |  |  |
| --- | --- | --- | --- |
| Package name | PackageID | Version | Termination Types Supported |
| "Address reporting package" [ITU-T H.248.37] {Editor's note (2014-07): still under discussion … related to ICE procedures …} | adr (0x00ac) | v1 | ANY |
| "Statistics for discarded packets due to latching package" [ITU-T H.248.37] | lstat (0x00e4) | v1 | ANY |
| "Advanced SDP wildcarding package" [ITU-T H.248.39] (NOTE 1) | aswp (0x011c) | v1 | ANY |
| "MGC controlled bearer level ALG package" [ITU-T H.248.78] (NOTE 2) | mcbalg (0x0108) | v1 | ANY |
| "TLS capability negotiation package" [ITU-T H.248.90] (NOTE 3) | tlscn (0x0118) | v1 | ANY |
| "TLS session maintenance package" [ITU-T H.248.90] (NOTE 4) | tlsm (0x0119) | v1 | ANY |
| "TLS traffic volume metrics package" [ITU-T H.248.90] (NOTE 4) | tlstv (0x011a) | v1 | ANY |
| FIXTHIS … H.248.STGROUP |  |  |  |
| FIXTHIS … H.248.RTPMUX |  |  |  |
| "Secure RTP Package" [ITU-T H.248.77] (NOTE 5) | srtp (0x0107) | v1 (v2?) | ANY |
| NOTE 1 – When MGC and MG agree to benefit from advanced SDP wildcarding. NOTE 2 – For WEBRTC data services (if required; dependent on application protocol and other criteria). NOTE 3 – Applied for DTLS-enabled stream endpoints (in context of SCTP/DTLS/UDP protocol layering). NOTE 4 – Related to DTLS layer. NOTE 5 – Selection of key management scheme for SRTP. |  |  |  |

{Editor’s note (2014-07): the requirement of "ICE for TCP" would imply support of TCP-enabled SEPs and consequently lead to the optional support of TCP-related H.248 packages according to H.248.84 and H.248.89. To be clarified.}

Examples:

IF CSB THEN "following table":

FIXTHIS

### 9.14.3	Package usage information

The following is a non-exhaustive list of package usage indications.

#### 9.14.3.1	xxx

FIXTHIS

## 9.15	Mandatory support of SDP and ITU-T H.248.1 Annex C information elements

At least the following:

| Supported Annex C and SDP information elements: |  |  |
| --- | --- | --- |
| Information Element | Annex C Support | SDP Support |
| "m="-line <proto> | not supported (in this example) | Value(s): ALL IANA registered codepoints with "x" protocol Purpose: ### Dependent on ### - application-aware: ### - application-agnostic: ### |
|  |  | Value(s): ALL IANA registered codepoints with "y" protocol Purpose: ### Dependent on ### - application-aware: ### - application-agnostic: ### |
| "a=fingerprint:" | not supported (in this example) | Value(s): all Purpose: DTLS authentication procedures |
| … |  |  |
| FIXTHIS |  |  |

Rest: subject to profile specification.

## 9.16	Optional support of SDP and ITU-T H.248.1 Annex C information elements

At least the following:

| Supported Annex C and SDP information elements: |  |  |
| --- | --- | --- |
| Information Element | Annex C Support | SDP Support |
| "a=setup:" | not supported (in this example) | FIXTHIS |
| … FIXTHIS |  |  |
| NOTE 1 – FIXTHIS |  |  |

Rest: subject to profile specification.

## 9.17	Procedures

The initial release of this Recommendation is focusing on the previous profile elements. Specific guidelines for the procedural section are for further studies.

Bibliography

[b-ITU-T H.264]	Recommendation ITU-T H.264 (02/2014), Advanced video coding for generic audiovisual services.

[b-ITU-T H.265]	Recommendation ITU-T H.265 (04/2013), High efficiency video coding.

[b-IETF RFC 4347]	IETF RFC 4347 (2006), Datagram Transport Layer Security.

[b-IETF RFC 4788]	IETF RFC 4788 (2007), Enhancements to RTP Payload Formats for EVRC Family Codecs.

[b-IETF RFC 4867]	IETF RFC 4867 (2007), RTP Payload Format and File Storage Format for the Adaptive Multi-Rate (AMR) and Adaptive Multi-Rate Wideband (AMR-WB) Audio Codecs.

[b-IETF RFC 5763]	IETF RFC 5763 (05/2010), Framework for Establishing a Secure Real-time Transport Protocol (SRTP) Security Context Using Datagram Transport Layer Security (DTLS).

[b-IETF RFC 6184]	IETF RFC 6184 (05/2011), RTP Payload Format for H.264 Video.

[b-IETF RFC 6386]	IETF RFC 6386 (2011), VP8 Data Format and Decoding Guide.

[b-IETF RFC 6716]	IETF RFC 6716 (2012), Definition of the Opus Audio Codec.

[b-IETF clue-datachannel]	IETF draft-ietf-clue-datachannel (2014), CLUE Protocol Data Channel.

[b-IETF latching]	IETF draft-ietf-mmusic-latching (201#), Latching: Hosted NAT Traversal (HNT) for Media in Real-Time Communication.

[b-IETF rmcat-cc-requirements]	IETF draft-ietf-rmcat-cc-requirements (2014), Congestion Control Requirements For RMCAT.

[b-IETF rtcweb-gateway]	IETF draft-alvestrand-rtcweb-gateways (2014), WebRTC Gateways.

[b-IETF trickle-ice]	IETF draft-ietf-mmusic-trickle-ice (2014), Trickle ICE: Incremental Provisioning of Candidates for the Interactive  Connectivity Establishment (ICE) Protocol.

[b-IETF ice-dualstack]	IETF draft-martinsen-mmusic-ice-dualstack-fairness (##/201#), ICE IPv4/IPv6 Dual Stack Fairness.

[b-IETF rtp-opus]	IETF draft-ietf-payload-rtp-opus (2014), RTP Payload Format for Opus Speech and Audio Codec.

[b-IETF rtp-vp8]	IETF draft-ietf-payload-vp8 (2014), RTP Payload Format for VP8 Video.

[b-IETF codec-vp9]	IETF draft-grange-vp9-bitstream (2013), A VP9 Bitstream Overview.

[b-IETF rtp-h265]	IETF draft-ietf-payload-rtp-h265 (2014), RTP Payload Format for High Efficiency Video Coding.

[b-IETF rtcweb-ecrit]	IETF draft-aboba-rtcweb-ecrit-01 (2013), Emergency Services Support in WebRTC.

[b-IETF rtcweb-audio]	IETF draft-ietf-rtcweb-audio (2014), WebRTC Audio Codec and Processing Requirements.

[b-IETF rtcweb-video]	IETF draft-ietf-rtcweb-video (2014), WebRTC Video Processing and Codec Requirements.

[b-IETF rtcweb-xr]	IETF draft-huang-xrblock-rtcweb-rtcp-xr-metrics (2014), Considerations for Selecting RTCP Extended Report (XR) Metrics for the RTCWEB Statistics API.

[b-IETF sdp-bundle]	IETF draft-ietf-mmusic-sdp-bundle-negotiation (2014), Negotiating Media Multiplexing Using the Session Description Protocol (SDP).

[b-IETF sdp-sctp]	IETF draft-ietf-mmusic-sctp-sdp (2014), Stream Control Transmission Protocol (SCTP)-Based Media Transport in the Session Description Protocol (SDP).

[b-IETF data-channel-sdpneg]	IETF draft-ejzak-mmusic-data-channel-sdpneg (2014), SDP-based "SCTP over DTLS" data channel negotiation.

[b-IETF data-channel-msrp]	IETF draft-ejzak-mmusic-msrp-usage-data-channel (2014), MSRP over SCTP/DTLS data channels.

[b-W3C webrtc-stats]	W3C draft webrtc-stats (2014), Identifiers for WebRTC's Statistics API.
URL: http://dev.w3.org/2011/webrtc/editor/webrtc-stats.html

Appendix I

Use case specific capability sets – Two examples

(This appendix does not form an integral part of this Recommendation)

## I.1	Overview

Profile content is mainly use case dependent (given be the inherent motivation of "protocol profiling"). In order to demonstrate profile specification guidelines in clause 9, this Recommendation considers two exemplary use cases:

- Capability set 'A' (CSA) – a WEBRTC H.248 gateway as positioned in use case #1 (see clause 7.1.1):

Use case #1 is based on the assumption that the WebRTC endpoint could request and use all mandatory WEBRTC capabilities as defined by the IETF / W3C. The WEBRTC H.248 gateway does then need to support all mandatory requirements as listed in clause 8.

  - … FIXTHIS … list major capabilities

  - …

- Capability set 'B' (CSB) – a WEBRTC H.248 gateway as positioned in use case #2 with scope on the NGN/IMS domain located WEBRTC endpoint (see clause 7.1.2):

  - … FIXTHIS … list major capabilities (see Annex U/23.228) …

  - …

  - ICE-lite mode sufficient

  - .ICE for UDP might be sufficient (tbc)

  - . no RTP media multiplexing (R-8.4.3/1)

  - . no RTP transport multiplexing (R-8.4.3/2)

  - …

Capability set 'A' (CSA) is therefore a superset of capability set 'B' (CSB).

Appendix L1
Living List – WebRTC media component "data" – Discussion 
of interworking scenarios and H.248 connection models

(This appendix does only contain working material.)

{Editor's note (2013-01 meeting):

Purpose: this Appendix parks potential revision material for H.248.77 as long as evaluation still ongoing.

Ref: T13-SG16-C-0050

Initial proposal plus possible solution (in yellow highlighted), see below:

## L1.1	Scope

WebRTC media component data and discussion of interworking options by H.248 WebRTC gateways.

## L1.2	Possible interworking models by H.248 WebRTC gateways

Figure L1.1 illustrates an example of WebRTC endpoints (EP) with four media components: audio, video and two instant messaging (IM) sessions. IM is MSRP based in this example, which represents a WebRTC data application. The two indicated H.248 contexts (1 and 2) are located in different H.248 MG entities. A H.248 WebRTC gateway application is indicated by the connection model in H.248 context 1:

- audio and video: possibly multiplexed RTP is unmultiplexed (but out of scope of this contribution);

- data: SCTP/DTLS/UDP transport is interworked to TCP transport.

Figure L1.1 indicates the various EP types.

Figure L1.1 – H.248 WebRTC gateway – Interworking and scope on Data Channel 
(multiple IM sessions)

There are a couple of questions related to the outlined interworking model in Figure L1.1. The most important one is related to the SCTP Association endpoint ("STCP Assoc EP a1") at Termination Ta1. The SCTP association contains two SCTP Streams ("StrP a1.3" and "StrP a1.4"), which are terminated at Ta and interworked to the two TCP endpoints in Termination Tc1. The basic question is related to applicable H.248 SEP modeling?

Figure L1.2 indicates one possible solution.

Figure L1.2 – H.248 WebRTC gateway – Example model for H.248 SEPs

Comments:

- Solution A "hierarchical SEPs" (= Figure L1.2): here SEP 255 represents the "tunnel endpoint" (SCTP Association/DTLS/UDP/IP) and SEP 3 and SEP 4 the two bearer connections for WebRTC data (MSRP over SCTP Streams plus data channel control);

- Solution B "one-SEP-per-Data channel": would lead to a bundling based model;

- It should be noted that the outlined example provides a worst case scenario because we got a) multiple WebRTC data channels, which are multiplexed in a single b) L4-over-L4 tunnel.

- However, the question/problem of H.248 SEP modelling remains if we would just consider single WebRTC data channels …

- … keeping in mind the different lifetime and bearer connection control procedures at the various protocol layers:

- UDP transport connection

- DTLS session

- SCTP association

- SCTP stream and

- WebRTC data control channel (tbc)

Thus, a distribution of hierarchical (associated) SEPs seems to be beneficial from H.248 control (and MGC) perspective.

- Solution A seems to be similar than H.248 Mux descriptor? (tbc)

## L1.3	Relevant documentation

Figure L1.3 attempts an overview (status: 2013-10):

Figure L1.3 – Landscape of related IETF Documents ("WebRTC data channel")

Appendix L2
Living List – H.248 WebRTC gateway capabilities for 3GPP Rel-12

(This appendix does only contain working material.)

{Editor's note (2014-02 meeting): purpose: this Appendix provides the link to correspondent 3GPP documents (without repeating that information).}

## L2.1	3GPP references

See (status 2014-06):

- 23.228 Annex U "WebRTC access to IMS - network-based architecture" (normative)

- 23.701  "Study on Web Real Time Communication (WebRTC) access to IP Multimedia Subsystem (IMS); Stage 2":
Annex A  "WebRTC access to IMS - network-based architecture" (informative)

- 23.228 Annex U "WebRTC access to IMS - network-based architecture" (normative)

- 23.706 "Study on enhancements to Web Real Time Communication (WebRTC) access to IP Multimedia Subsystem (IMS); Stage 2; (Release 13)"

- 24.371 "Web Real Time Communication (WebRTC) Access to IMS; Stage 3 Protocol Specification (Release 12)"

## L2.2	H.248 WebRTC media gateway: media plane interworking scenarios

WebRTC supports three main media components (Figure L2.1): 1) audio including DTMF and 2) video , which are both using SRTP/UDP transport, and 3) data using WebRTC DataChannels (based on SCTP/DTLS/UDP transport). Hence any specific media other than audio and video uses WebRTC DataChannels for transport.

Figure L2.1 – 3GPP Rel-12 – H.248 WebRTC media gateway: media plane interworking scenarios

Appendix L3
Living List – DiffServ for WebRTC multimedia traffic – Discussion of IETF proposal "DiffServ for Real-time communication"

(This appendix does only contain working material.)

{Editor's note (2014-11): this item is relevant for H.248 WEBRTC gateways, but documented as living list item because 1st the requirement situation is still unclear and 2nd the technology (DiffServ extensions) are yet in discussion (IETF WG DART).}

## L3.1	Item of discussion

H.248 WEBRTC gateway is required to support "QoS marking" for WEBRTC bearer traffic. QoS marking relates to capabilities as defined by [ITU-T H.248.52].

For the DiffServ-based QoS marking method are discussions ongoing how WEBRTC could benefit from DiffServ, under the specific conditions of

Condition 1: WEBRTC = multiple application traffic flows multiplexed on IP transport connections;

Condition 2: WEBRTC = different application traffic flows benefit from different DSCPs (DiffServ codepoint);

Such a DiffServ model is not yet supported.

## L3.2	DiffServ models

Figure L3.1 illustrates three DiffServ models:

Figure L3.1 – DiffServ models

The existing DiffServ technology, as well as H.248.52, was designed for model (I), but would be also applicable for model (II).

Model (III) is in scope of IETF draft draft-york-dart-dscp-rtp, and considered for WEBRTC.

Draft "draft-york-dart-dscp-rtp" considers the case of a) multiplexed applications as transported via b) RTP/UDP (or RTP/TCP), at the example of WebRTC (i.e., audio/RTP and video/RTP share the same IP transport connection, but would benefit from different DS PHB). There would be consequently different DSCP assignments for the same IP transport connection.

Further information, see:

[b-IETF dscp-rtp]	IETF draft-york-dart-dscp-rtp (2014), Differentiated Services (DiffServ) and Real-time Communication.
URL: http://tools.ietf.org/html/draft-york-dart-dscp-rtp-02

## L3.3	Impact on H.248 WebRTC gateways

There are various roles of H.248 IP media gateways concerning support and handling of DiffServ, see Appendix IV/[ITU-T H.248.52]. Thus, the possible set of "DiffServ" roles is primarily limited by the "IP mode of operation", see [ITU-T H.248.64]: Back-to-Back IP Host (B2BIH) mode or IP router (IPR) mode. True DS PHB is associated to the IPR mode only!

Status H.248 WebRTC gateways:

Existing H.248 profiles for IP-IP gateways, which are subject of WEBRTC support (such as 3GPP eIMS-AGW), enforce the Back-to-Back IP Host (B2BIH) mode.

Such B2BIH configured H.248 border gateways act in a DiffServ environment either

- in DSCP transparent forwarding mode (see H.248.52, § 7.6.1.2 Copying DSCP values from the ingress to the egress);

- as DS boundary node (see H.248.52, § IV.3.2  H.248 MG as "DS boundary node"); or

- as DSCP translator (see H.248.52, § IV.3.3  H.248 MG between two DS domains).

Thus, such H.248 border gateways don't provide any kind of actual DS PHB.

## L3.4	Conclusions

On a first glance, when the B2BIH mode would be maintained for H.248 WEBRTC gateways, then:

- there seems to be no real impact from the extended DiffServ model (draft-york-dart-dscp-rtp) on existing H.248 border gateway profiles;

- if there are multiple DSCPs used per IP transport connection, then it would also not affect the DSCP translator function.

- impact on the current H.248.52 supported DiffServ model needs to be investigated;

However, a final evaluation is dependent on the finally required H.248 WEBRTC gateway interworking models for WEBRTC RTP traffic components (as dependent on draft RTCWEB-RTP-USAGE).

Appendix L4
Living List – Potential updates to H.248.x Recommendations 
for WEBRTC support

(This appendix does only contain working material.)

{Editor's note (2014-11): this item is driven by AVD-4642.}

## L4.1	Purpose

This living list discusses potential updates to H.248 Recommendations as a result of supporting RTCWEB on a MGC/MG.

## L4.2	Discussion

This Appendix analyses the current IETF drafts related to the IETF RTCWEB working group to see if there any impacts to existing H.248 Recommendations and to see if new items are required to support communication with WebRTC clients by H.248 controlled gateways.

Below Table contains a list of drafts from the RTCWEB WG page (https://datatracker.ietf.org/doc/search/?name=rtcweb&activeDrafts=on&rfcs=on ). Most independent submissions have been removed. Each draft from the list is a level one heading. These drafts may contain references to other drafts that need to be considered. These drafts have been included in level 2 and 3 headings. Drafts with potential impacts are highlighted in yellow.

## L4.3	Status of analysis (2014-11)

## L4.3.1	IETF WG RTCWEB

See Table L4.1:

Table L4.1 – IETF WebRTC documents identified by WG RTCWEB

| IETF doc | Summary |  | H.248 impact? |  |
| --- | --- | --- | --- | --- |
| 1. draft-alvestrand-rtcweb-gateways-00 “WebRTC Gateways” | This draft indicates that a WebRTC gateway may support ICE-Lite. |  | This can be supported through the use of H.248.50. |  |
| # 2. draft-ietf-rtcweb-alpn-00 “Application Layer Protocol Negotiation for Web Real-Time Communications (WebRTC)” | This draft introduces the ability to indicate the usage of ALPNs for “webrtc” and “c-webrtc”. Application Layer Protocol Negotiation (ALPN) [RFC7301] enables an endpoint to positively identify WebRTC uses and distinguish them from other DTLS uses. H.284.WEBRTC should address how this is communicated between the MGC and MG. | It may require updates to H.248.93 on DTLS to support signalling the ALPN or discuss how to determine it. |
| 3. draft-ietf-rtcweb-audio-06 “WebRTC Audio Codec and Processing Requirements” | This draft indicates a baseline set of audio requirements. | H.248 can support the signalling of the required information. E.g. Opus, G.711 codecs, comfort noise, audio/telephone events, audio level and echo cancelling. |
| 4. draft-ietf-rtcweb-audio-codecs-for-interop-00 “Additional WebRTC audio codecs for interoperability with legacy networks” | This draft presents additional audio codecs. | The use of these codecs may be signalled via H.248. |
| 5. draft-ietf-rtcweb-constraints-registry-00 “IANA Registry for RTCWeb Constrainable Properties” | The draft introduces a registry for media track constraints. The draft in itself does not relate to H.248 however the media track constraints (http://www.w3.org/TR/2013/WD-mediacapture-streams-20130903/#idl-def-MediaTrackConstraints) do relate to the description of media and thus have a relation to SDP for media. | There may need to be some mapping between the constraints to appropriate SDP. This is possibly a topic for the JSEP draft. |
| 6. draft-ietf-rtcweb-data-channel-12 “WebRTC Data Channels” |  | The support of this is already being discussed as part of the work on H.248.WEBRTC, H.248.SCTP and H.248.STGROUP. |
| 7. draft-ietf-rtcweb-data-protocol-08 “WebRTC Data Channel Establishment Protocol” |  | The support of this is already being discussed as part of the work on H.248.WEBRTC, H.248.SCTP and H.248.STGROUP. |
| 8. draft-ietf-rtcweb-jsep-07 “Javascript Session Establishment Protocol” | This draft provides a mapping between the WebRTC Javacript API and SDP. It defines what SDP is used for WebRTC sessions. It is expected that a H.248 Gateway acting as a WebRTC endpoint would utilise this draft. | The SDP requirements from the draft would be translated to H.248 SDP. It is expected this would be addressed by H.248.WEBRTC. |
| 9. draft-ietf-rtcweb-overview-11 “Overview: Real Time Protocols for Browser-based Applications” | This draft gives an overview of the requirements and architecture of WebRTC. The data transport, framing and security requirements are detailed by the other drafts in this Table. |  |
| 10. draft-ietf-rtcweb-rtp-usage-17 “Web Real-Time Communication (WebRTC): Media Transport and Use of RTP” | This draft outlines requirements for the use of RTP in WEBRTC. | Much of the required information can be signalled between a MGC and MG via SDP however other requirements relate to specific handling of RTP that is typically not signalled by the MGC. A MG must know whether the H.248 Termination relates to a WebRTC client to ensure that all the requirements are implemented. This area requires further investigation. |
| 11. draft-ietf-rtcweb-security-07 “Security Considerations for WebRTC” | This draft outlines security issues related to WebRTC. | There’s no direct impact related to H.248 controlled gateways. |
| 12. draft-ietf-rtcweb-security-arch-10 “WebRTC Security Architecture” | This draft outlines the security architecture including protocols for WebRTC. | The main requirement is the support of DTLS which is supported in H.248.93. The use of a=identity and a=fingerprint is also indicated which in supported via SDP in H.248. Verification and consent are also discussed. These issues are discussed by other drafts in this Table. |
| 13. draft-ietf-rtcweb-stun-consent-freshness-07 “STUN Usage for Consent Freshness” | This draft introduces the notion of periodic STUN binding requests into order to determine whether a sender may sent application data for a period 30 seconds. | H.248.50 allows the MGC/MG to send binding requests but does not support sending periodic requests. This would need to be added to H.248.50. |
| 14. draft-ietf-rtcweb-transports-06 “Transports for WebRTC” |  |  |
| 14.1 System provided interfaces |  |  |
| 14.1.1 draft-ietf-tsvwg-rtcweb-qos-02 “DSCP and other packet markings for RTCWeb QoS” |  | No explicit impact other than indicating what DSCP values the MGC should set. (see also Appendix L3) |
| 14.2 IPv4/IPv6 Usage |  |  |
| 14.2.1 draft-martinsen-mmusic-ice-dualstack-fairness, ICE IPv4/IPv6 Dual Stack Fairness. | This draft updates the calculation of candidate priority for connectivity checks from ICE RFC5245. It introduces a new formula. | H.248.50 is currently based on RFC5245. Gateways used for WEBRTC should use the new formula. H.248.50 clause 8 allows the MGC to provide the priority or to wildcard choose the priority. In the MGC provided case as it knows whether the Termination relates to an WEBRTC endpoint it can apply the new formula. In the case of the wildcarded CHOOSE case, the MG would need an indication of whether to apply the WebRTC or base ICE priority formula. This indication would need to be added to H.248. In either case the section could be updated to indicate that the priority formula from this draft could be used in the MGC provided case. |
| 14.2.2 RFC6724 “Default Address Selection for Internet Protocol Version 6 (IPv6)” | This RFC indicates that temporary addresses are preferred over public addresses for source address selection. RTCWEB clients should use this selection choice. Clause 5 indicates that there must be a means to override the preference. This RFC also references RFC5104 which is the IPv6 socket API that allows different address “types” to be indicated. | H.248 currently does not support these functions when a MGC requests address either through normal H.248 wildcarding procedures or via address gathering in H.248.50. |
| 14.3 Middle Box Related |  |  |
| 14.3.1 RFC5245 “Interactive Connectivity Establishment (ICE): A Protocol for Network Address Translator (NAT) Traversal for Offer/Answer Protocols” |  | ICE is supported by H.248.50. |
| 14.3.2 RFC5766 “Traversal Using Relays around NAT (TURN): Relay Extensions to Session Traversal Utilities for NAT (STUN)” |  | TURN is supported by H.248.50. |
| 14.3.3 RFC6156 “TURN Extension for IPv4/IPv6 Transition” |  | H.248.50 supports TURN however it currently does not specify the usage of RFC6156 which allows the use of the REQUESTED-ADDRESS-FAMILY attribute (e.g. clause 4.1/RFC6156). RTCWeb indicates this must be supported. H.248.50 should be updated to indicate that this may be used with a possible new codepoint in clause 7.3 for the MG TURN client package. |
| 14.3.4 RFC6062 “Traversal Using Relays around NAT (TURN) Extensions for TCP Allocations” | The RFC defines additional STUN methods and attributes to be used with TURN for TCP address allocations. | H.248.50 currently doesn’t make reference to this RFC. The support of RFC6062 should be considered by revised H.248.50. |
| 14.3.5 RFC6544 “TCP Candidates with Interactive Connectivity Establishment (ICE) ” | This RFC introduces the possibility to use TCP candidates in ICE procedures. | The support of RFC6544 has been added to the draft revised H.248.50. |
| 14.4 Transport Protocol Related |  |  |
| 14.4.1 RFC4571 “Framing Real-time Transport Protocol (RTP) and RTP Control Protocol (RTCP) Packets over Connection-Oriented Transport ” | This RFC allows the transport of RTP over TCP. | The use of RTP over TCP may be indicated through SDP and thus may be used by H.248. |
| 14.4.2 RFC5389 “Session Traversal Utilities for NAT (STUN)” | This RFC defines Session Traversal Utilities for NAT known as “STUN”. | H.248.50 already supports STUN. |
| 15. draft-ietf-rtcweb-use-cases-and-requirements-14 “Web Real-Time Communication Use-cases and Requirements” | This draft introduces a number of use cases of web applications that are executed in a browser utilising real-time requirements. A number of requirements are generated as a result. | As the requirements are addressed by other more detailed drafts there’s no direct impact to H.248 from this draft. |
| 16. draft-ietf-rtcweb-video-00 “WebRTC Video Processing and Codec Requirements” | As per the audio requirements, the use of a particular video codec (along with its parameters) may be indicated via SDP. | No additional work is envisaged for H.248. |
| 17. draft-ietf-tsvwg-rtcweb-qos-02 “DSCP and other packet markings for RTCWeb QoS” | As per clause 14.1.1 above. |  |
| 18. draft-ietf-xrblock-rtcweb-rtcp-xr-metrics-00 “Considerations for Selecting RTCP Extended Report (XR) Metrics for the RTCWEB Statistics API” | This draft indicates which RTCP XR metrics may be used by a WEBRTC endpoint. | H.248.48 allows the detection and reporting of RTCP XR metric. |
| 19. draft-jennings-rtcweb-deps-04 “WebRTC Dependencies” | This draft contains the full list of WEBRTC dependencies on IETF documents. It may provide a further checklist. |  |
| 20. draft-nandakumar-rtcweb-sdp-05 “SDP for the WebRTC” | This draft contains sample SDP Offer/Answers used by WEBRTC endpoints. | This also provides a good checklist to see if H.248 may support the media described by the SDP. H.248.WEBRTC could consider this. |

### L4.3.2	Other IETF WGs

For further studies (such as MMUSIC, AVTCORE, XRBLOCK, …)

______________________

Annex A/23.701 was moved to Annex U/23.228 at the end oft he study phase.

DiffServ: There's the basic assumption that the IP L4 payload contains unmultiplexed application data traffic, i.e., a single DS codepoint is assignment per "application IP flow".