| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | TD-17 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 5 | Seoul, 3 - 7 November 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Cisco |  |  |  |
| Title: | Comments on contributions AVD-4631, AVD-4647 and AVD-4653 |  |  |  |
| Purpose: | Discussion |  |  |  |

# Introduction

We apologize for not being able to join Q5 experts in Seoul for this Rapporteur meeting. However, we have some comments on a few contributions that we would like to bring to the attention of the meeting.

# AVD-4631 – H.TPS-AV: Signalling audio level

We support the idea of sending the actual time-varying audio level in the audio stream. This can be very useful for mixers, forwarders and receivers. However, we believe that time-averaging between RTP packets should not be used by the sender before signalling. The signalled level should simply be the RMS value of the samples in each packet as described in section 3 of RFC6464.

To remove uncertainties and differences caused by parameter choices and implementation differences, the averaging should be done either by the mixer, the forwarder or the receiver, depending on their usage of the audio level information.

The most common use for dynamic audio level information is to choose which stream is active and should be mixed, re-sent or rendered. The procedures defined for speech level averaging in ITU-T P.56 are not necessarily suitable for such real-time use, for instance the detection of new active speakers.

Security issues as described in section 6 of RFC6464 should also be considered. The level auditing described in that section would become more complicated to perform if senders are allowed to perform averaging.

The dynamic audio level should be signalled using RFC6464 as well and this is adequately covered in the Appendix. “May be signalled” leaves the sender the choice of signalling the level information or not.

## Proposed change in 7.1.3.5

We propose to delete the following sentence in 7.1.3.5: “Dynamic audio level shall be measured using method B of  [ITU-T P.56], which may be implemented using the tool sv-p56.c in the Software Tools Library [ITU-T G.191].” and replace it with this sentence: “Dynamic audio level shall be measured using RFC6464.”

# AVD-4647 – LS/i on new Recommendation on QoE requirements of telepresence services [from ITU-T SG12]

This liaison from SG12 comes in a little late since G.1091 (ex G.qrtp) was just approved on 29 October 2014. Unfortunately, some text in clause 7 and 11, in particular, is not aligned with the text of the Q5’s AV parameter draft.

Clauses 11.1 and 11.2 define background noise and reverberation, but the definitions in the AV parameter draft seem better and define those parameters more precisely.

In 11.4, sound uniformity is actually a good characteristic, but it offers little meaning without further details on frequency bands, assessment methods, etc.

In 11.8, we believe that audibility is meant to be a privacy characteristic (not allowing a nearby room to overhear a conversation in the telepresence room) and this, which is a building requirement, is better stated in the AV parameter draft.

Regarding clause 7, most of the audio characteristics are defined in the AV parameter draft. To avoid differences in the definition of those characteristics, the parameters that are identical should make a reference to the AV parameter Recommendation (once approved).

Therefore, we recommend that Q5 sends a short reply LS to SG12 informing them of some misalignments between our work on AV parameters and G.1091 and tell them that SG16 plans to send them a detailed reply from the February 2015 meeting when the AV parameter draft will be proposed for Consent.

# AVD-4653 – H.TPS-SIG: Carry encodingID in OLC message

As we understand the proposal, the intent is to open a media flow for a particular encodingID using “dataType” in H.245.  It is not clear exactly what the generic capability structures would contain, but we are concerned with using the “dataType” for this purpose where each generic parameter capability type might contain different values (representing the encodingIDs). Historically, “dataTypes” that an endpoint might open are advertised in the TCS message and are often grouped into simultaneous capabilities. The notion of an “encodingID” does not, on the surface, seem appropriate for a “dataType”.

That said, we appreciate the desire to use CLUE to negotiate media flows and then to use OLCs to establish those flows.  In the spirit of trying to accomplish what we think the contribution aims to do, perhaps it might be better to do these things:

- Define a single generic audio and a single generic video capability that represents a “CLUE” audio and “CLUE” video flow

- Use that single capability in the TCS messages (with one or more instances, as appropriate)

- Use the OLC.genericInformation field to convey the specific “encodingID” value associated with a particular channel

Appreciating that what is proposed in the contribution might be the right solution, we encourage further dialog to help establish a mutual understanding so that we arrive at the right conclusion.

__________________