| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | TD-25 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 5/16 | Seoul, 3 - 7 November 2014 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Rapporteur |  |  |  |
| Title: | Refined Text from AVD-4651, AVD-4652, AVD-4653 |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This comprises the refined produced by Q5 experts AVD-4651, AVD-4652, and AVD-4653.  This text is proposed for incorporation into H.TPS-SIG.

[Begin Proposal]

## 8.2	Call session setup

If two endpoints both support CLUE signalling an exemplary call session setup procedure is shown in the figure below.

Figure ? Exemplary CLUE call establishment procedure

Telepresence endpoint A carries the CLUEControlCapability in supportedFeatures field in order to signal that it supports CLUE signalling. When telepresence endpoint B receives the setup message it knows the peer supports CLUE signalling.  Therefore B responds with the Alerting message which also carries the supported feature CLUEControlCapability.  This informs A that B also supports CLUE signalling. The CLUEControlCapability feature also can be carried in Call proceeding or Connect messages. After the Connect message a CLUE call is established.

If the support for CLUEControlCapability is not received from B, then either B does not support CLUE, or the GEF component of the message was removed somewhere along the signalling path. In that case, a non-CLUE traditional H.323 call setup occurs as shown below.

Figure ? Non-CLUE call establishment procedure

However, if the H.245 TCS messages indicate the CLUE is in fact supported by B, then the CLUE protocol can still be used.

[End Proposal]

[Begin Proposal]

## 8.4   Interaction of CLUE protocol and H.323 protocol stack

Information about media streams controlled by CLUE is split between two message types: H.245 capability message, which defines basic media types and limits, and the CLUE messages, which defines properties of capture devices available, scene information and additional constraints. As a result certain operations, such as advertising support for a new transmissible capture with associated stream, require both H.245 and CLUE messaging.

A telepresence endpoint needs the ability to get or update protocol states from both the H.323 (H.245 included) and CLUE protocol stack, and to determine how to implement the coming operations according to both protocol states above and the signalling procedures.

This section defines some principles for interaction between H.245 and CLUE protocols, provides some recommendations on dealing with intermediary stages in non-atomic operations, and mandates additional constraints on when CLUE-configured media can be sent.

### 8.4.1   Independence of H.245 TCS and CLUE Advertisements

H.245 and CLUE protocols are independent, and therefore there are transitory periods where the CLUE Encoding Correlations transported over H.245 are not synchronized with the CLUE Advertisements.   In order to minimize the impact of these transitory periods the following is recommended:

Providers should ensure that any inconsistencies between H.245 CLUE Encoding Correlations and CLUE signalling are temporary by sending an updated H.245 TCS or CLUE Advertisements as soon as the relevant state machines and other constraints permit.

Receivers shall be able to receive CLUE Advertisement messages independently from the transmission of the CLUE Encoding Correlation included in the TCS, and be able to handle states in which CLUE messages make reference to EncodingIDs that do not match the most recently received H.245 TCS.

As a general rule, receivers will not send a CLUE configure message until they have both the Advertisement and the CLUE encoding Correlation.  However, if a CLUE Configure message is received before the CLUE encoding Correlation is sent, the provider may reject the message with via the CLUE “Invalid Sequencing” response code.

Providers may update their CLUE Encoding Correlations at any time, but providers should avoid reusing encodingID numbers for different media capabilities.

### 8.4.2   Constraints on sending media

CLUE controlled media shall not be sent unless the provider’s most recent H.245 capability exchange contains the correct CLUE encoding correlation and the consumer has sent a CLUE Configure message specifying a valid capture for that encoding.

## 8.5	Open Logical Channel

After a CLUE Configure is received, the provider opens logical channels for media transport. Each logical channel’s dataType shall carry the encodingID to identify the particular Capture, using the extendedVideoCapability and extendedAudioCapability structures in H.245.  The video media encoding shall be carried in the videoCapability field of the extendedVideoCapability. The audio media encoding shall be carried in the audioCapability field of the extendedAudioCapability.

The video encodingID shall be carried in the videoCapabilityExtension, and the audio encodingID shall be carried in the audioCapabilityExtension.

The videoCapabilityExtension and audioCapabilityExtension shall carry the CLUE Correlation Capability defined in Table 4 with the appropriate encodingID parameter.

{Editor’s Note:  extendedAudioCapability needs to be added to H.245}

[End Proposal]

__________________