| INTERNATIONAL TELECOMMUNICATION UNION |  | STUDY GROUP 16 |  |
| --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  | TD 214 R1 (WP 1/16) |  |
|  |  | English only Original: English |  |
| Question(s): | 3/16 | Geneva, 9 - 20 February 2015 |
| TD |  |  |
| Source: | Editor H.248.50 |  |
| Title: | H.248.50 "Gateway control protocol: NAT traversal toolkit packages" (Rev.): Output draft Ed. 0.12 (Geneva, 9 - 20 February 2015) |  |

1.	Introduction

This document contains the editor's output draft ed. 0.12 for draft new H.248.50 "Gateway control protocol: NAT traversal toolkit packages" (Rev.) from the SG16 meeting in Geneva, 9 - 20 February 2015.

[NOTE – Annex L is a provisional placeholder that contains the living list of issues for this text and will be removed before Consent.]

Document History

| Document History |  |
| --- | --- |
| Issue | Notes |
| H.248.50 Rev. Ed. 0.0 | Proposal (AVD-4273a) to WP 2/16 Brisbane meeting (2012-09) Location: http://wftp3.itu.int/av-arch/avc-site/2009-2012/1209_Bri/AVD-4273a.zip |
| H.248.50 Rev. Ed. 0.1 | Input (C-0030) from ITU-T SG16 Geneva meeting, Jan 2013 Location: http://www.itu.int/md/T13-SG16-C-0030 |
| H.248.50 Rev. Ed. 0.2 | Output (TD 49 (WP1/16)) from ITU-T SG16 Geneva meeting, Jan 2013 Location: http://www.itu.int/md/T13-SG16-130114-TD-WP1-0049 |
| H.248.50 Rev. Ed. 0.3 | Input (AVD-4362) to WP 1/16 Oslo meeting (2013-01) Location: http://wftp3.itu.int/av-arch/avc-site/2013-2016/1306_Osl/AVD-4362.zip |
| H.248.50 Rev. Ed. 0.4 | Output (TD 121/WP1) from SG16 Geneva meeting, Oct/Nov 2013 Location: http://www.itu.int/md/T13-SG16-131028-TD-WP1-0121 |
| H.248.50 Rev. Ed. 0.5 | Input (AVD-4471) to WP 1/16 Geneva meeting (2014-03) Location: http://wftp3.itu.int/av-arch/avc-site/2013-2016/1403_Gen/AVD-4471.zip |
| H.248.50 Rev. Ed. 0.6 | Output (TD-11) from WP 1/16 Geneva meeting (2014-03) Location: http://wftp3.itu.int/av-arch/avc-site/2013-2016/1403_Gen/TD-11.zip |
| H.248.50 Rev. Ed. 0.7 | Input (TD 147/WP1) to SG16 Sapporo meeting, June/July 2013 Location: http://www.itu.int/md/T13-SG16-140630-TD-WP1-0147/en |
| H.248.50 Rev. Ed. 0.8 | Output (TD 147R2/WP1) from SG16 Sapporo meeting, June/July 2013 Location: https://www.itu.int/md/dologin_md.asp?lang=en&id=T13-SG16-140630-TD-WP1-0147!R2!MSW-E |
| H.248.50 Rev. Ed. 0.9 | Input (AVD-4592) to WP 1/16 South Korea meeting (2014-11) Location: http://wftp3.itu.int/av-arch/avc-site/2013-2016/1411_Seo/AVD-4592.zip |
| H.248.50 Rev. Ed. 0.10 | Output (TD-08) from WP 1/16 South Korea meeting (2014-11) Location: http://wftp3.itu.int/av-arch/avc-site/2013-2016/1411_Seo/TD-08.zip |
| H.248.50 Rev. Ed. 0.11 | Input (TD 214/WP1) to ITU-T SG16 Geneva meeting, Feb 2015 Location: http://www.itu.int/md/T13-SG16-150209-TD-WP1-0214/en |
| H.248.50 Rev. Ed. 0.12 | Output (TD 214R1/WP1) from ITU-T SG16 Geneva meeting, Feb 2015 Location: http://www.itu.int/md/T13-SG16-150209-TD-WP1-0214/en |
| H.248.50 Rev. Ed. 0.13 |  |
| H.248.50 Rev. Ed. 0.14 |  |

CONTENTS

Page

1 Scope 8 1.1 Relation to other H.248 related NAT traversal mechanisms 9 2 References 9 3 Definitions 11 3.1 Terms defined elsewhere 11 3.2 Terms defined in this Recommendation 11 4 Abbreviations and acronyms 12 5 Conventions 13 6 Toolkit usage 13 6.1 ITU-T H.248.50 usage in different network models 13 6.1.1 ITU-T H.248 MGC/MG as interim nodes in the end-to-end path between user equipment (with STUN client/server and/or ICE support) 13 6.1.2 ITU-T H.248 MGC/MG emulates a user agent (with STUN client/server and/or ICE support) 15 6.1.3 ITU-T H.248 MGC/MG provides STUN/TURN server functionality 16 6.1.4 ITU-T H.248 MG provides TURN media relay functionality 18 6.1.5 Note to "STUN server" capability 18 6.1.6 ICE capability set: ICE full versus ICE lite 19 6.1.7 Principal network models concerning location of ICE endpoints 19 6.2 Overview of toolkit NAT traversal techniques 20 6.3 ITU-T H.248 call/bearer separation, connection model and IP addresses for ephemeral terminations 20 6.4 Specific SDP information elements 21 6.4.1 SDP for vanilla ICE 21 6.4.2 SDP for extended ICE 21 6.5 Overview of NAT traversal support mechanisms (by ITU-T H.248 entities) 22 6.5.1 Address latching support 22 6.5.2 Basic STUN/TURN support (ICE-less) 22 6.5.3 ICE-controlled STUN/TURN 22 7 STUN and TURN support 24 7.1 STUN base package 24 7.1.1 Properties 24 7.1.2 Events 25 7.1.3 Signals 25 7.1.4 Statistics 25 7.1.5 Error codes 25 7.1.6 Procedures 25 7.2 MG STUN client package 25 7.2.1 Properties 26 7.2.2 Events 28 7.2.3 Signals 28 7.2.4 Statistics 28 7.2.5 Error codes 28 7.2.6 Procedures 28 7.3 MG TURN client package 29 7.3.1 Properties 29 7.3.2 Events 32 7.3.3 Signals 32 7.3.4 Statistics 32 7.3.5 Error codes 32 7.3.6 Procedures 32 7.4 MGC STUN client package 33 7.4.1 Properties 33 7.4.2 Events 33 7.4.3 Signals 33 7.4.4 Statistics 34 7.4.5 Error codes 34 7.4.6 Procedures 34 7.5 STUN information package 34 7.5.1 Properties 34 7.5.2 Events 35 7.5.3 Signals 36 7.5.4 Statistics 37 7.5.5 Error codes 37 7.5.6 Procedures 37 8 ICE support 37 8.1 MG act-as STUN server package 39 8.1.1 Properties 40 8.1.2 Events 40 8.1.3 Signals 40 8.1.4 Statistics 40 8.1.5 Error codes 41 8.1.6 Procedures 41 8.2 Originate STUN continuity check package 41 8.2.1 Properties 41 8.2.2 Events 42 8.2.3 Signals 45 8.2.4 Statistics 46 8.2.5 Error codes 46 8.2.6 Procedures 46 8.3 STUN consent freshness package 47 8.3.1 Properties 48 8.3.2 Events 48 8.3.3 Signals 49 8.3.4 Statistics 50 8.3.5 Error Codes 50 8.3.6 Procedures 50 9 Keep-alive and pinhole support 51 9.1 MGC-originated STUN request package 51 9.1.1 Properties 51 9.1.2 Events 51 9.1.3 Signals 52 9.1.4 Statistics 53 9.1.5 Error codes 53 9.1.6 Procedures 53 9.2 Keep alive request package 53 9.2.1 Properties 54 9.2.2 Events 54 9.2.3 Signals 54 9.2.4 Statistics 55 9.2.5 Error codes 55 9.2.6 Procedures 55 10 Package-independent NAT-T procedures 56 10.1 Support for MG terminated STUN-based connectivity checks 56 10.1.1 Overview 56 10.1.2 Required MG support functions 56 10.1.3 Indication of bearer type "STUN/L4/IP" 56 10.1.4 Signalling of credentials 57 10.1.5 ICE conclusion process (STUN-specific media latching) 57 Appendix I Example signalling scenarios for Package-independent NAT-T procedures 59 I.1 Example #1: ICE/STUN support by H.248 IP access gateways 59 I.1.1 Network model 59 I.1.2 Example signalling at SIP and H.248 interfaces 59 I.2 Example #2: … 61 Appendix II ICE for TCP 62 II.1 Introduction 62 II.2 MG bearer interface: protocol stack 62 II.2 Bearer establishment 62 II.3 TCP address candidates 62 Appendix III ICE mode support by gateway-embedded ICE agents 64 III.1 Background 64 III.2 Main differences between ICE-full and ICE-lite 64 Appendix IV ICE extensions: "Trickle ICE" 66 II.1 Introduction 66 Bibliography 67 Appendix L Living List related to enhancements and clarifications 68 L.3 Item #3: Package-independent NAT-T procedures 69 L.4 Item #4: H.248.50 References to IETF ICE documents 69 "" 69 L.4.2 Item #4.2: IETF RFC 6724 "IPv6 address selection" 70 L.5 Item #5: Recent ICE updates by IETF 70 L.5.0 Overview (Status 11/2014) 70 L.5.1 Item #5.1: IETF document "Update on Candidate Address Selection" 71 L.5.2 Item #5.2: IETF document "Trickle ICE" 71 L.5.4 Item #5.4: IETF document "Media level ice-options SDP attribute" 73 L.5.5 Item #5.5: IETF document "Mobility with ICE" 74

List of Tables

Page

Table 1 – Network models concerning location of ICE endpoints 20 Table 2 – Usage of SDP-defined credentials in H.248 LD and RD 57 Table I.1 – Example command encoding– (SIP) SDP Offer 60 Table I.2 – Example command encoding – MGC request 60 Table I.3 – Example command encoding– MG reply 61 Table II.1 – Type extensions for TCP-based candidates 63 Table II.2 – TCP candidate options 63 Table III.1 – High-level comparison of ICE modes 64

List of Figures

Page

Figure 1 – Landscape of NAT traversal toolkit packages, categorized into three application areas 9 Figure 2 – Network model – ITU-T H.248 MGC/MG as interim nodes in the end-to-end path between user equipment (with STUN client/server and/or ICE support) 14 Figure 3 – Network model – ITU-T H.248 MGC/MG emulates a user agent (with STUN client/server and/or ICE support) 15 Figure 4 – Network model – Scenario when MG and MGC are located in different IP realms – Keep-alive and pinhole support 16 Figure 5 – Network model – ITU-T H.248 MGC/MG provides STUN/TURN server functionality 17 Figure 6 – Network model – ITU-T H.248 MG provides TURN media relay functionality 18 Figure I.1 – Network model for H.248 IP access gateways (derived from Fig. 2 with scope on half call only) 59 Figure II.1 – ICE for TCP – protocol stack (MG bearer interface) 62

Draft revised Recommendation ITU-T H.248.50

Gateway control protocol: NAT traversal toolkit packages

AAP Summary

[To be added]

Summary

Recommendation ITU-T H.248.50 contains a series of ITU-T H.248 packages that enable various network address translator (NAT) traversal techniques to be employed in order to facilitate media flow between networks. Any of these packages may be utilized in any order to gather and map addresses, as well as maintain connectivity with and through NATs.

This revision of Recommendation ITU-T H.248.50 adds clarifications, ###

# 1	Scope

This Recommendation describes packages to enable various network address translator (NAT) traversal techniques to be employed in order to facilitate media flow between networks. The media gateway controller (MGC) may utilize any of the packages in any order to gather addresses, map them and then maintain connectivity with and through NATs.

The packages described in this Recommendation allow an ITU-T H.248 MGC and media gateway (MG) to use the techniques defined by:

–	Simple STUN reflexive address mapping as defined in [IETF RFC 3489] and [IETF RFC 5389].

–	Relayedaddress mapping using the TURN techniques as described in [IETF RFC 5766].

–	Comprehensive NAT traversal ICE techniques as described in [IETF RFC 5389].

In order to maintain backward compatibility, packages have been produced for both STUN as defined by [IETF RFC 3489] and by [IETF RFC 5389].

Throughout this Recommendation it is assumed that the media gateway performs STUN server discovery through the use of domain name system (DNS) lookup.

Figure 1 summarizes the various packages as defined by this Recommendation. Every package is self-contained and does not use the extension principle.

Figure 1 – Landscape of NAT traversal toolkit packages, 
categorized into three application areas

## 1.1	Relation to other H.248 related NAT traversal mechanisms

{Editor's note (2014-06 meeting): this text overlaps somehow with the scope of clause 6.5. However, complete restructuring of clause 6.5 is difficult in a Revision. But the resulting redundancy seems to be a minor issue …}

The following covers the applicability of this Recommendation versus other H.248 supported NAT-T mechanisms:

- Transport protocol generic NAT-T method "media latching" [ITU-T H.248.37]: this is an orthogonal mechanism and may be used in combination with this Recommendation; some of the mechanisms in this Recommendation make "media latching" unnecessary.

- Transport protocol specific NAT-T method "TCP merge mode" [ITU-T H.248.84]: this represents a TCP simultaneous open from end-to-end perspective, which is, if applicable, used before any "ICE for TCP" procedures would be selected.

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published. The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (2005), Gateway control protocol: Version 3.

[ITU-T H.248.14]	Recommendation ITU-T H.248.14 (2009), Gateway control protocol: Inactivity timer package.

[ITU-T H.248.37]	Recommendation ITU-T H.248.37 (2008), Gateway control protocol: IP NAPT traversal package.

[ITU-T H.248.40]	Recommendation ITU-T H.248.40 (03/2013), Gateway control protocol: Application data inactivity detection package.

[ITU-T H.248.90]	Recommendation ITU-T H.248.90 (10/2014), Gateway control protocol: H.248 packages for control of transport security using TLS.

[ITU-T Y.2111]		Recommendation ITU-T Y.2111 (2008), Resource and admission control functions in next generation networks.

[IETF RFC 3489]	IETF RFC 3489 (2003), STUN – Simple Traversal of User Datagram Protocol (UDP) Through Network Address Translators (NATs).

[IETF RFC 3556]	IETF RFC 3556 (2003), Session Description Protocol (SDP) Bandwidth Modifiers for RTP Control Protocol (RTCP) Bandwidth.

[IETF RFC 3605]	IETF RFC 3605 (2003), Real Time Control Protocol (RTCP) attribute in Session Description Protocol (SDP).

[IETF RFC 4566]	IETF RFC 4566 (2006), SDP: Session Description Protocol.

[IETF RFC 4787]	IETF RFC 4787 (01/2007), Network Address Translation (NAT) Behavioral Requirements for Unicast UDP.

[IETF RFC 5245]	IETF RFC 5245 (2010), Interactive Connectivity Establishment (ICE): A Protocol for Network Address Translator (NAT) Traversal for Offer/Answer Protocols.

[IETF RFC 5389]	IETF RFC 5389 (2008), Session Traversal Utilities for NAT (STUN).

[IETF RFC 5766]	IETF RFC 5766 (2010), Traversal Using Relays around NAT (TURN): Relay Extensions to Session Traversal Utilities for NAT (STUN).

[IETF RFC 5780]	IETF RFC 5780 (05/2010), NAT Behavior Discovery Using Session Traversal Utilities for NAT (STUN).

[IETF RFC 6544]	IETF RFC 6544 (03/2012), TCP Candidates with Interactive Connectivity Establishment (ICE).

[IETF RFC 6888]	IETF RFC 6888 (04/2013), Common Requirements for Carrier-Grade NATs (CGNs).

[IETF STUNCONFRES]	IETF draft-ietf-rtcweb-stun-consent-freshness-11 (201x), STUN Usage for Consent Freshness.

# 3	Definitions

## 3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	NAT traversal [ITU-T Y.2111]: The operation of adapting the IP addresses so that the packets in the media flow can pass through a far-end (remote) NAT.

3.1.2	network address translation [ITU-T Y.2111]: The operation by which IP addresses are translated (mapped) from one address domain to another address domain.

3.1.3	pinhole [ITU-T H.248.37]: A configuration of two associated H.248 IP terminations within the same context, which allows/prohibits unidirectional forwarding of IP packets under specified conditions (e.g., address tuple).

3.1.4	symmetric NAT [IETF RFC 3489]: A symmetric NAT is one where all requests from the same internal IP address and port, to a specific destination IP address and port, are mapped to the same external IP address and port.

3.1.5	STUN agent [IETF RFC 5389]: an entity that implements the STUN protocol. The entity can be either a STUN client or a STUN server.

3.1.6	STUN client [IETF RFC 5389]: an entity that sends STUN requests and receives STUN responses. A STUN client can also send indications.

3.1.7	STUN server [IETF RFC 5389]: an entity that receives STUN requests and sends STUN responses. A STUN server can also send indications.

3.1.8	Vanilla ICE [b-IETF trickle-ice]: The Interactive Connectivity Establishment protocol as defined in [IETF RFC 5245].

3.1.9	Half trickle ICE mode [b-IETF trickle-ice]: FIXTHIS ….

3.1.10	Full trickle ICE mode [b-IETF trickle-ice]: FIXTHIS ….

3.1.11	STUN consent freshness [IETF STUNCONFRES]: Maintaining and renewing consent over time. Consent relates to the mechanism of obtaining permission to send to a remote transport address.

NOTE – Initial consent is obtained using ICE or a TCP handshake.

## 3.2	Terms defined in this Recommendation

3.2.1	ICE-full (mode, implementation): The ICE protocol capability set that performs the complete set of ICE requirements according to clause 4.1/[IETF RFC 5245].

NOTE – This definition is based on clause 3/[IETF RFC 5245]: "Full: An ICE implementation that performs the complete set of functionality defined by this specification." The RFC uses the term "full ICE" to reference this functionality.

3.2.2	ICE-lite (mode, implementation): The ICE protocol capability set that performs the subset of ICE requirements according to clause 4.2/[IETF RFC 5245].

NOTE – This definition is based on clause 3/[IETF RFC 5245]: "Lite: An ICE implementation that omits certain functions, implementing only as much as is necessary for a peer implementation that is full to gain the benefits of ICE.  Lite implementations do not maintain any of the state machines and do not generate connectivity checks.".

3.2.3	Trickle ICE: The extended ICE protocol capabilities, beyond vanilla ICE, as defined in [b-IETF trickle-ice]. Trickle ICE includes half trickle and full trickle ICE modes.

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| ABNF | Augmented Backus-Naur Form |
| --- | --- |
| B2BIH | Back-to-Back IP Host |
| B2BUA | Back-to-Back User Agent |
| DCCP | Datagram Congestion Control Protocol |
| DNS | Domain Name System |
| FW | FireWall |
| ICE | Interactive Connectivity Establishment |
| IMS | IP Multimedia Subsystem |
| IP | Internet Protocol |
| ISDN | Integrated Services Digital Network |
| IUA | ISDN User Adaptation |
| MG | Media Gateway |
| MGC | Media Gateway Controller |
| NAT | Network Address Translation |
| PES | PSTN/ISDN Emulation Subsystem |
| PSTN | Public Switched Telephone Network |
| RMG | Residential Media Gateway |
| RMGC | Residential Media Gateway Controller |
| RTCP | RTP Control Protocol |
| RTP | Real Time Protocol |
| SCTP | Stream Control Transmission Protocol |
| SDP | Session Description Protocol |
| SIP | Session Initiation Protocol |
| STUN | Session Traversal Utilities for NAT |
| TCP | Transmission Control Protocol |
| TLS | Transport Layer Security |
| TURN | Traversal Using Relays around NAT |
| UDP | User Datagram Protocol |
| UE | User Equipment |
| UNSAF | UNilateral Self-Address Fixing |
| VPN | Virtual Private Network |

# 5	Conventions

None.

# 6	Toolkit usage

## 6.1	ITU-T H.248.50 usage in different network models

The ITU-T H.248 packages of this Recommendation may be applied in various network configurations. The clauses below illustrate some main scenarios.

### 6.1.1	ITU-T H.248 MGC/MG as interim nodes in the end-to-end path between user equipment (with STUN client/server and/or ICE support)

The NAT traversal support protocols STUN, TURN and ICE are IP application protocols. They are fundamentally deployed in IP hosts (behind NAT device(s)) and IP network servers. The IP host function is located in the user equipment (UE) in Figure 2. The end-to-end IP bearer-path goes through an ITU-T H.248 media gateway (with an ITU-T H.248 IP-IP Context). There may be multiple NAT devices in the IP bearer-path (and IP signalling path(s)). Figure 2 shows an example with four IP realms, separated by two standalone NAT devices. The third NAT function is provided by the ITU-T H.248 MG.

NOTE – The MG-internal NAT function is realized by the so-called back-to-back IP host (B2BIH) mode.

Figure 2 – Network model – ITU-T H.248 MGC/MG as interim nodes in the end-to-end path 
between user equipment (with STUN client/server and/or ICE support)

The ITU-T H.248 gateway (MGC and MG entity) may be requested to support particular STUN/TURN/ICE scenarios. For instance, the ITU-T H.248 gateway may be requested to act in a "proxy role" of the STUN client function (e.g., in case that the MG and UE would be both located in the same IP realm and when the UE could not provide the STUN client function itself).

It may be noted that the relation between the assist protocols (STUN, TURN and ICE) and the call/session control protocol (like SIP) may vary between a loose and a tighter coupling mode, e.g., some examples with regard to the time relation:

– 	timely tightly coupled: e.g., the ICE address gathering phase and subsequent SIP INVITE phase (before connectivity check phase);

–	timely loosely coupled: e.g., the STUN keep pinhole open mechanism and NAT pinhole timer settings (which may vary between 30 seconds and the timescale of minutes and hours);

–	timely de-coupled: e.g., the basic STUN mechanism for analysing the mode of operation(s) of the installed base of NAT devices.

The function of the last bullet item may be decoupled from call/session control phases and done in advance (because network topology and NAT behaviour changes rather slowly). The ITU-T H.248 gateway may be used for a NAT traversal support function on each time-scale.

### 6.1.2	ITU-T H.248 MGC/MG emulates a user agent (with STUN client/server and/or ICE support)

The evolution from the legacy PSTN/ISDN to IP-based networks like the PSTN emulation subsystem (PES) or the IP multimedia subsystem (IMS) may lead to a network model as depicted in Figure 3. The ITU-T H.248 gateway may be located at the border between the circuit-switched and IP networks. The ITU-T H.248 gateway may be requested to emulate the behaviour of IP user equipment with regard to the NAT traversal support function.

Figure 3 – Network model – ITU-T H.248 MGC/MG emulates a user agent 
(with STUN client/server and/or ICE support)

It may be noted that the TURN media relay function is outside of the ITU-T H.248 gateway and provided by a separate network server in the previous two examples.

#### 6.1.2.1	Scenario when MG and MGC located in different IP realms

Where the MGC and MG are in different IP realms, the ITU-T H.248 signalling and media flows may traverse the same NAT/Firewall (FW) (see also Figure 4, using the example of a residential media gateway (RMG)). The techniques described in this Recommendation may be used for media flow NAT/FW traversal. Signalling (call/session) NAT traversal is generally not in the scope of this Recommendation, however ITU-T H.248 NAT traversal may benefit from the use of ITU-T H.248-based peer-to-peer polling mechanisms (like e.g., [ITU-T H.248.14] or empty audits on ITU-T H.248 level, or e.g., SCTP indications on transport connection level) in order to maintain NAT bindings. Opening of the NAT binding for ITU-T H.248 traffic would be subject of the initial ServiceChange messages for MG registration.

Figure 4 – Network model – Scenario when MG and MGC are located in different IP realms – Keep-alive and pinhole support

Keep "pinhole open" mechanisms for media IP flows are in scope of this Recommendation. Keep "pinhole open" mechanisms for signalling IP flows are out of scope of this Recommendation; such methods may be e.g., addressed by correspondent ITU-T H.248 profile specifications for such ITU-T H.248 gateways.

### 6.1.3	ITU-T H.248 MGC/MG provides STUN/TURN server functionality

The STUN/TURN server function could be principally embedded in a MG, MGC or gateway (MGC/MG tandem), see Figure 5.

Figure 5 – Network model – ITU-T H.248 MGC/MG provides 
STUN/TURN server functionality

The STUN server function requires the processing of incoming STUN Binding Request messages and the reply by correspondent STUN Binding Response messages. In case of NAT devices in "symmetric NAT" mode, additional support by TURN is required. The TURN server function requires the processing of TURN messages (Allocate Request, Allocate Response and Send Request).

The STUN server and TURN server functions may be provided by an ITU-T H.248 gateway on a MG or MGC level (as indicated in Figure 5). However, the TURN media relay function should be out of scope of MGC nodes (see clause 6.1.4).

The advantage of a STUN/TURN server at MGC level is the close location to call/session control. For instance, it may be beneficial to have information available concerning interim NAT devices and their behaviour, NAT binding lifetime information (e.g., a TURN attribute) or IP address usage. A close location of possible SIP server function (like a SIP proxy, application level gateway or B2BUA function) and STUN/TURN servers may also be beneficial.

The advantage of a STUN/TURN server at MG level may be e.g., driven by a functional sharing performance model for off-loading the MGC node from STUN/TURN message processing functions, or e.g., the coupling of a TURN server and TURN media relay in an ITU-T H.248 MG node.

### 6.1.4	ITU-T H.248 MG provides TURN media relay functionality

The TURN media relay function may be embedded in a MG itself, see Figure 6. The TURN media relay function is inserted in the end-to-end IP bearer-path. The decision for routing the IP bearer connection over a TURN media relay is either already known before call/session establishment, or done during that phase due to a call/session-driven STUN process. The TURN media relay function may require substantial resources for processing and forwarding IP bearer path packets. This is the primary reason for excluding such a function from MGC entities.

There are on the other side many good reasons for combining the TURN media relay function and ITU-T H.248 MG function. For example, this is because the MG may inherently provide the TURN media relay NAT function, or because of a "simpler" IP bearer-path routing process, or due to QoS reasons (e.g., the native TURN media relay provides only coarse support for bearer-path resource reservation by the single attribute "Bandwidth", but is lacking support for session-dependent policing functions per se), or the ability of the MGC to access TURN media relay server information.

Figure 6 – Network model – ITU-T H.248 MG provides TURN media relay functionality

### 6.1.5	Note to "STUN server" capability

A network domain may provide one or multiple STUN server(s). A STUN server may be realized as non-H.248 network element or as H.248 entity-embedded STUN server functions (in context of this Recommendation). For instance, the STUN agent provided by the MG is then called "MG act-as STUN server" function.

The STUN service(s) may be provided by a single or multiple STUN servers; typically dependent on the network configuration and/or STUN application.

The internal modeling of the STUN protocol in "service categories" (protocol functions) is difficult, however, STUN server functions could be classified in call-independent and call-dependent categories. It should be noted that such a classification scheme is not useful in general, but definetely suited in context of H.248 gateways.

The call-dependent category relates to the ICE connectivity check and has to be performed by the entity in the call/session control path that terminates ICE, either the ICE end point located in a terminal or a gateway, (e.g. a session border gateway). Providing a single STUN server in the network domain thus implies that call control signalling corresponding to all call/sessions using ICE has to be routed through that gateway (such as in Figure 5), unless the end point itself provides the server functionality.

An example of STUN server function distributions for STUN application 'ICE' according to the network model in Figure 2 (see also description of the scenario in clause 6.1.1 or Appendix I.1):

A stand-alone "STUN server" is involved during the STUN-based address gathering phase. And the H.248 MG as "STUN server" is involved during the STUN-based connectivity check phase. The first STUN server function is "call-independent" because actually happening before any call control signalling (from MGC perspective), and the second STUN server function is "call-dependent" because associated to call control signalling.

The STUN server involved during STUN-based address gathering phase could also be provided by an H.248 MG entity (such as in case of Figure 5).

### 6.1.6	ICE capability set: ICE full versus ICE lite

ICE allows two explicit mode of operations, where "ICE full" represents the complete capability set and "ICE lite" a subset of "ICE full". [IETF RFC 5245] defines an explicit signalling element (SDP "a=ice-lite" attribute) for ICE mode support indication. The omission / inclusion of this SDP attribute in SIP signalling indicates ICE full / lite support. However, [IETF RFC 5245] does not provide a detailed description of the behaviour of the ICE lite mode; the information is rather distributed across the entire ICE RFC.

NOTE – The rationale behind this is the expectation that ICE lite is only a temporary implementation step which should evolve to ICE full support. This assumption might be valid for terminal-embedded ICE agents, but not necessarily valid for gateway-embedded ICE support as in scope of this Recommendation.

Appendix III describes the differences between the two ICE modes at the level of detail of interest to H.248 gateways.

### 6.1.7	Principal network models concerning location of ICE endpoints

ICE represents a single algorithm, distributed over two ICE agents in a network. There are two main models concerning the mapping of the two ICE agents to the network topology (Table 1):

Table 1 – Network models concerning location of ICE endpoints

| Model | Characteristics | Comments |
| --- | --- | --- |
| Terminal-to-terminal (peer-to-peer) | Both ICE endpoints are locate in terminal embedded IP host entities | Base model of IETF for the design of ICE. All IETF "ICE RFCs" use this model. |
| Terminal-to-network | One ICE endpoint is located in IP network infrastructure equipment, such as an H.248 IP-to-IP media gateway (acting as B2BIH or IP router) within the end-to-end IP media path. This leads to following consequences: - The end-to-end IP media path is divided (by the H.248 MG) in two segments: - "ICE segment" and - "another non-ICE based segment". - Special case: both segments use an ICE-based NAT traversal independent of each other. | Major difference to the IETF base model: the MG might not be "located behind a NAT" (from perspective of ICE), and thus be always reachable ("full IP connectivity without ICE-based NAT traversal support"). |

This aspect should be taken into account when looking at the use cases in clauses 6.1.1 to 6.1.4.

## 6.2	Overview of toolkit NAT traversal techniques

This Recommendation describes the various mechanisms used in order to secure media flow traversal of networks where NATs are used. In essence, the packages described provide a toolkit for the MGC to use in order to: gather a list of potential NAT mapped address (address gathering), check connectivity associated with those addresses, and maintain the connectivity associated with those addresses. The Recommendation is structured around the various techniques that have been defined:

1)	STUN usage according to [IETF RFC 3489].

2)	STUN usage according to [IETF RFC 5389].

3)	TURN usage according to [IETF RFC 5766].

4)	ICE techniques according to [IETF RFC 5245].

5)	Other techniques used to maintain connectivity.

These techniques have typically not considered the scenario where the media flows are initiated from a split MGC and MG.

## 6.3	ITU-T H.248 call/bearer separation, connection model and IP addresses for ephemeral terminations

The splitting of the so-called "user agent" functionality into a control and media components means that there needs to be coordination between the two components. As the MGC usually constructs call and session control messages, it must be able to request information from the MG in order to build these messages. For instance, it would need address and media information to place in an SDP offer.

Media gateways typically support several different source/destination IP address/port combinations per peer-to-peer media connection. This may be due to different media (e.g., audio or video), different single media capabilities (codec A versus codec B) and to handle events (e.g., RTCP). These capability sets may be advertised to a remote party which selects the appropriate media set to use. In order for media connectivity to be successful for any of the sets, any media stream that encounters a NAT should have its address mapped. At a call/session level, a correlation is given between the local "native" IP address/ports of the MG and the mapped addresses. Such a scheme is detailed in clause 4.3 of [IETF RFC 5245]. The use of ITU-T H.248 between a MGC and MG introduces a connection model that needs to be considered when correlating local "native" addresses and mapped addresses using ICE techniques. Furthermore, some NAT traversal techniques do not use ICE, thus this particular candidate scheme is not appropriate for them.

As such in this Recommendation, where ICE is not used, the correlation is in the form of an ordered list of values, where:

The first position on the list corresponds to the first address in the first group associated with a particular stream. The second position is associated with either a second address in the first group or, if there are no further addresses, it is associated with the first address in the second group. The parameter "Address Correlation" in the Base STUN package describes the mapping between the list position and the place in the local descriptor.

## 6.4	Specific SDP information elements

### 6.4.1	SDP for vanilla ICE

## 6.4.1.1	UDP-based address candidates

Where ICE is used, the SDP CANDIDATE attribute [IETF RFC 5245] is used.

The so-called "candidate scheme" does not alter the actual media flows. That is, a MG will use the source address in the local descriptor to send from, and the destination address in the remote descriptor to send to. In the text encoding, this is the SDP connection address (c=) and media (m=) lines. Therefore, if the MGC chooses a candidate other than the local address, this shall be reflected in the local and remote descriptors. This may have implications on how the media is sent. For example, if this address is a relay address, then the data may have to be sent using TURN send indications.

### 6.4.1.2	TCP-based address candidates

Where ICE is used for TCP [IETF RFC 6544] (see also Appendix II), the SDP CANDIDATE attribute from [IETF RFC 5245] is used with its grammar extended according to clause 4.5/[IETF RFC 6544].

### 6.4.2	SDP for extended ICE

#### 6.4.2.1	Trickle ICE

{Editor’s note (2014-11 meeting): trickle ICE introduces 
a) a new SDP codepoint for the "ice-options" SDP attribute:
	a=ice-options:trickle
and
b) a new SDP attribute:
	a=end-of-candidates. 
Logically, such SDP information belongs to clause 6.4. However, clause 6.4 is still not comprehensive concerning all SDP elements as defined for ICE (see IANA registry). It might be worth to think about a Table which lists all ICE relevant SDP elements, indicated in separate columns their usage at call control signalling level and relevance for H.248 interface?
Contributions are solicited.}

## 6.5	Overview of NAT traversal support mechanisms (by ITU-T H.248 entities)

### 6.5.1	Address latching support

This is covered in the scope of [ITU-T H.248.37].

### 6.5.2	Basic STUN/TURN support (ICE-less)

See clause 7.

### 6.5.3	ICE-controlled STUN/TURN

See clause 8. The ICE (interactive connectivity establishment) methodology defines a "superior" protocol that uses the STUN/TURN mechanism and other protocols. The use of ICE is also tightly coupled with SIP and in particular with SIP/SDP Offer/Answer procedures. ICE is executed between a pair of ICE agents. There are two device types concerning the embodiment of ICE agents with respect to their location in the IP network infrastructure:

- terminal-embedded ICE agent: user equipment with ICE agent function and SIP UA function;

- gateway-embedded ICE agent: a decomposed H.248 gateway provides the ICE agent function in a distributed manner (with the SIP signalling function located at MGC level).

This clause describes the ICE process from the perspective of a generic ICE agent, i.e., it does not differentiate between the above two embodiments (but indicating possible H.248 impact for the gateway-provided ICE agent).

The execution of ICE may be basically divided into several consecutive phases:

1)	Gather addresses

- Purpose: Address gathering.

- This phase may be based on STUN and/or TURN procedures as required.

- Used ITU-T H.248.50 packages: None.

2)	Prioritizing addresses

- Purpose: optimize final IP bearer path (routing).

- Function provided either by

- MG, in case of FIXTHIS; or

- MGC, in case of FIXTHIS; or

- external (SIP) server, in case of FIXTHIS;

- Used ITU-T H.248.50 packages: FIXTHIS.

- Address prioritization rules: The basic rules for address prioritization are described in clause 4.1.2.1/[IETF RFC 5245]. The purpose of ICE IPv4/IPv6 dual stack fairness support may demand for other prioritization algorithms, see [b-IETF ice-dualstack]. The applicable prioritization rules should be explicitly part of an H.248 profile specification in case that an H.248 entity is responsible for that ICE function.

{Editor’s note (2014-11 meeting): above list item is generic, but mentions [b-IETF ice-dualstack] as example. That reference might be removed again dependent on progress in IETF.}

3)	Call/session control signalling: generate SDP Offer (for SIP signalling)

- Purpose: generate SDP Offer content (if originating side).

- Multiple SDP attributes may be involved: FIXTHIS.

NOTE – The SDP attribute values may be requested from MGC and provided by MG, in case of network scenario FIXTHIS.

- Used ITU-T H.248.50 packages: None (due to application control protocol related function).

4)	Call/session control signalling: Negotiations – Offering and Answering (initiate and accept SIP messages

–	Purpose: Address advertisement.

–	This phase is typically related to the initiation of a SIP INVITE (thus, out of scope of this Recommendation).

NOTE – The peer user (called party, invited party) may also start a correspondent "address gathering" procedure (based on STUN and/or TURN as required). These remote procedures would not be visible for the local MGC/MG entities.

–	The peer (SIP) user may reply with a SIP OK (thus, out of scope of this Recommendation).

–	Used ITU-T H.248.50 packages: None (due to application control protocol related function).

5)	Connectivity checks

–	This phase is again based on STUN and/or TURN procedures as required.

–	The connectivity checks are based on the same 4-tuple(s) as (later) used for the ITU-T H.248 stream.

–	The STUN/TURN messages may be thus multiplexed into the RTP/RTCP packet flow of the checked ITU-T H.248 stream (of an ITU-T H.248 RTP termination).

–	Used ITU-T H.248.50 packages: mgastuns, ostuncc.

6)	Completing ICE procedures

–	One agent assumes ICE controlling role, other agent assumes ICE controlled role.

–	Used ITU-T H.248.50 packages: FIXTHIS.

{Editor’s note (2013-01 meeting): aspect of "Gateway Free Operation (GrFO)" – there might be ICE variants which may conclude that the routed IP bearer path could possibly finally bypass the MG instance, which was initially involved in above ICE procedural steps. The initially created H.248 context would be released again, the MG is deallocated (which may be termed as GrFO, similar to TrFO). 
In case of GrFO, then the next step would be different. 
Note: GrFO could also lead to just an "MG transparent forwarding packet forwarding mode", i.e., the initial IP bearer path would be still routed via the MG, but agnostic packet forwarding …
Contributions are solicited.}

7)	Bearer connection: check/selection of media streams

–	Used ITU-T H.248.50 packages: None.

The outlined, high level ICE process is not detailed concerning possible differences from ICE mode perspective. The ICE mode impact on gateway-embedded ICE agents is described in Appendix III.

# 7	STUN and TURN support

The packages defined in this clause allow the use of STUN and TURN techniques without the need for support of ICE and associated SDP, e.g., Candidates are not used.

## 7.1	STUN base package

|  | Package name: | STUN Base |
| --- | --- | --- |
|  | Package ID: | stunb (0x00bd) |
|  | Description: | This package describes the mapping between transport addresses defined in the ITU-T H.248 local descriptor and the list position when used for STUN purposes. |
|  | Version: | 1 |
|  | Extends: | None |

### 7.1.1	Properties

#### 7.1.1.1	Address correlations

|  | Property name: | Address Correlation |
| --- | --- | --- |
|  | Property ID: | ac (0x0001) |
|  | Description: | This property details the list position of each IP Address in the H.248 Local Descriptor. This package may be used by the MGC to determine the mapping between local IP addresses and list positions for STUN processing purposes. NOTE – It is not mandatory to implement this package as the MGC may determine the list positions given the logic below. However it provides a mechanism in order to check the list positions. |
|  | Type: | List of String |
|  | Possible values: | Each element in the list of string SHALL be type AddressCorrelation according to the following ABNF: AddressCorrelation = Listposition "¦" Groupnumber "¦" Instance "¦" ComponentID Where: List position is an integer. The first position of the list shall be 1 and sequentially rising by one. Groupnumber is an integer and is the ITU-T H.248 group number. Instance is an integer. The first instance of a media format (as per clause 5.14 of [IETF RFC 4566]) <fmt> field in "m=" line in a particular group shall be 1. For each subsequent media format the instance shall be incremented by one. ComponentID is the identifier of a component. A component is a piece of a media flow requiring a single transport address. For RTP-based media flows, the RTP itself has a component ID of 1, and RTCP has a component ID of 2. For non RTP-based media flow, the component ID is 1. The list SHALL contain an element for each IP address in the local descriptor. For example, the values: "1¦1¦1¦1" "2¦1¦1¦2" "3¦1¦2¦1" "4¦1¦2¦2" "5¦2¦1¦1" "6¦2¦1¦2" Would relate to the following SDP in the local descriptor:: v=0 c=IN IP4 192.168.1.100 m=audio 10000 RTP/AVP 4 18 v=0 c=IN IP4 192.168.1.200 m=audio 20000 RTP/AVP 0 The values of ReserveValue and ReserveGroup are "on". |
|  | Default: | Empty String if no IP addresses are present in the local descriptor. |
|  | Defined in: | LocalControl |
|  | Characteristics: | ReadOnly |

### 7.1.2	Events

None.

### 7.1.3	Signals

None.

### 7.1.4	Statistics

None.

### 7.1.5	Error codes

None.

### 7.1.6	Procedures

In order for the MGC to determine in a timely manner the correlation between local IP Address and the STUN processing list positions, the MGC shall perform a CHOOSE ($) on the "stunb/ac" property whenever an IP address is added/changed/deleted to/from the ITU-T H.248 local descriptor on a particular Stream/Termination.

## 7.2	MG STUN client package

|  | Package name: | MG STUN Client |
| --- | --- | --- |
|  | ### Package ID: | mgstunc (0x00be) |
|  | ### Description: | This package enables an MGC to determine the mapped IP address and port that will be routed back to the media component that sent the request. This package applies to both [IETF RFC 3489] and [IETF RFC 5389]. |
|  | ### Version: | 1 |
|  | ### Extends: | None |

### 7.2.1	Properties

#### 7.2.1.1	STUN address

{Editor’s note (2014-11 meeting): the package property concerning "STUN address" seems to be not impacted by [b-IETF ice-dualstack], - to be confirmed. If yes, then no changes to clause 7.2.}

|  | ### Property name: | STUN Address |
| --- | --- | --- |
|  | ### Property ID: | stuna (0x0001) |
|  | ### Description: | This property indicates that the MG shall return a STUN mapped IP address and port for each local address indicated. The MGC may send this STUN mapped IP address and port to the peer as its remote IP address and port. NOTE – Implementors should be aware that each STUN message to a STUN server may imply a 9.5-second delay. This may impact the time in which an ITU-T H.248 command reply can be sent to the MGC. In cases where the STUN delay is excessive the use of TransactionPending is encouraged. |
|  | Type: | List of String |
|  | Possible values: | In an ITU-T H.248 command request: Each element in the list of string SHALL be of type StunAddressReq in ABNF format: StunAddressReq = "L"/("B"[COLON Stun-Transport-type])/ ("S"[COLON Stun-Transport-type]) Stun-Transport-type = "UDP"/"TCP"/"TLS" Where: "L": Local Address – Do not perform STUN address mapping for the address in this position. "B": Binding Request – Perform a STUN binding request on the address in this list position. STUN clients can communicate with a TURN server using UDP, TCP, or TLS over TCP. "S": Shared Secret/Binding Request – Perform Shared Secret STUN messages before performing a STUN binding request on the address in this list position. The value of STUN-TRANSPORT-TYPE is "UDP" , "TCP" or "TLS". These values are for "Binding Requests". "Shared Secret Requests" are always sent over TLS. In an ITU-T H.248 command reply: Each element of the string SHALL be of type StunAddressReply in ABNF format: StunAddressReply = (IP4-address COLON PortNumber)/ (IP6-address COLON PortNumber)/EToken [COLON ErrorCode] PortNumber = UINT16 EToken = "E" ErrorCode = 1*3(DIGIT); could be extended The ABNF syntax of IP4-address and IP6-address is defined in [IETF RFC 4566]. The MG may reply with an IP4 or IP6 address, for example "192.168.1.10:10000" and "FF1E:03AD::7F2E:172A:1E24:20000", or reply with an error information in which the Class and Number of the error (see clause 11.2.9 of [IETF RFC 3489] or clause 15.6 of [IETF RFC 5389]) is included. |
|  | Default: | Empty List |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |

#### 7.2.1.2	NAT lifetime

|  | Property name: | NAT Lifetime |
| --- | --- | --- |
|  | Property ID: | natl (0x0002) |
|  | Description: | This property requests the MG to return the NAT binding lifetime associated with the transport address at a particular list position. |
|  | Type: | List of String |
|  | Possible values: | In an ITU-T H.248 command request: Each element may be one of the following characters: T: "Time Request"; N: "No time request". In an ITU-T H.248 command reply: An integer (in string form) representing a Lifetime value, Or: E: "Error in list position", followed by the Class and Number of the error (clause 11.2.9 of [IETF RFC 3489] or clause 15.6 of [IETF RFC 5389]) is included in the response. |
|  | Default: | Empty List |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |

#### 7.2.1.3	RTO value

|  | Property name: | RTO Value |
| --- | --- | --- |
|  | Description: | This property requests the MG to set the initial retransmission timer RTO interval as defined in clause 7.2.1 of [IETF RFC 5389]. |
|  | Property ID: | rto (0x0003) |
|  | Type: | Integer |
|  | Possible values: | 1 ms to 600000 ms (10 minutes) |
|  | Default: | 100 ms |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |

### 7.2.2	Events

None.

### 7.2.3	Signals

None.

### 7.2.4	Statistics

None.

### 7.2.5	Error codes

None.

### 7.2.6	Procedures

To determine a STUN mapped address, the MGC shall issue an ITU-T H.248 Add/Modify/Move.req command containing the "stuna" property on the relevant termination/stream. If the MG wants to modify the value of RTO, the "rto" property should also be set on the relevant termination/stream. The MGC shall include a value in each position of list of string for each element described in the "stunb/ac" property.

For list positions designated with the value "L", the MG shall simply return an empty string for that position of the list.

For list positions designated with the value "B", the MG shall perform a STUN binding request messaging. If the Binding request is successful, the MG shall return the mapped address in the list position of the reply. If the binding request is unsuccessful, then the STUN error shall be returned in the list position.

For list positions designated with the value "S", the MG shall perform a STUN Shared Secret request before issuing binding request messaging. The MG may perform a single Shared Secret request for all Binding requests or issue multiple Shared Secret requests. STUN server implementations based on [IETF RFC 3489] do not support Shared Secret requests.

If the MGC wishes to reset the mappings, then it shall resend the parameter in an ITU-T H.248 request indicating "L", "B" or "S".

If the MGC wishes to receive the period of the binding lifetime with a NAT for an address it shall issue a "natl" property requesting "T" for each list element it wants a lifetime for and "N" for each element it does not want the time for. On receipt of the request, the MG will determine if a binding lifetime has been determined for a particular address. It will return the binding lifetime value if received from the NAT otherwise it will return "0". It is recommended that the "natl" property be included after the "stuna" property to maximize the possibility that a binding lifetime period is returned.

If the peer side returns error code 300 (Try Alternate), the transport address of the alternate server is in the ALTERNATE-SERVER attribute. The MG should use this transport address to resend the same request message to the alternate server. If the MG cannot send or the request still fails, the MG should indicate the error code to the MGC.

STUN server implementations based on [IETF RFC 3489] may not return a binding lifetime. The STUN information package should be used in this case.

## 7.3	MG TURN client package

|  | Package name: | MG TURN Client |
| --- | --- | --- |
|  | Package ID: | mgturnc (0x00bf) |
|  | Description: | This package enables an MGC to determine the mapped IP address and port used for data relaying through a TURN server. It enables the procedures defined by [IETF RFC 5766] to operate in a split MGC/MG environment. |
|  | Version: | 1 |
|  | Extends: | None |

### 7.3.1	Properties

#### 7.3.1.1	TURN address

|  | Property name: | TURN Address |
| --- | --- | --- |
|  | Property ID: | turna (0x0001) |
|  | Description: | This property requests for a particular local transport address and port that the MG perform a TURN allocate request with a remote TURN server in order to reserve an address. As per clause 6 of [IETF RFC 5766] the MGC may send this particular local transport address and Port to the peer as its remote IP address and port. |
|  | Type: | List of String |
|  | Possible values: | In an ITU-T H.248 command request: Each element in the list of string SHALL be of type TurnAddressReq in ABNF format: TurnAddressReq = ("A" [RequestedProps][TurnTransportType] [Lifetime][ChannelSend])/"N" RequestedProps = SP "rp" COLON ReqP TurnTransportType = SP "ttp" COLON TurnTranT Lifetime = SP "l" COLON LifeT ChannelSend = SP "c" COLON ChannelS ReqP = 3* BinChar BinChar = "0"/"1" TurnTranT = "UDP"/"TCP"/"TLS" LifeT = UINT32 ChannelS = "SEND"/"CHANNEL" "A": Allocate Request – Perform a TURN Allocation request on the address in this list position. The REQUESTED-PROPS attribute may be indicated at the same time. The value of REQUESTED-PROPS is "000" to "111". The value of TURN-TRANSPORT-TYPE is "UDP" , "TCP" or "TLS". LIFETIME is the value of the LIFETIME attribute. The value of ChannelS is "SEND" or "CHANNEL". This flag is used to request the MG to send application data using ChannelData messages or using Send and Data indications. "N": No change – Do not perform TURN address mapping for the address in this position. In an ITU-T H.248 command reply: Each element of the string contains a relay address, reflexive address and LIFETIME. Each element in the list of string SHALL be of type TurnAddressReply in ABNF format: TurnAddressReply = ((IP4RelayAddr SP IP4ReflexiveAddr) / (IP6RelayAddr SP IP6ReflexiveAddr) SP LifeT ) / (EToken [COLON ErrorCode]) IP4RelayAddr = IP4-address COLON PortNumber IP4ReflexiveAddr = IP4-address COLON PortNumber IP6RelayAddr = IP6-address COLON PortNumber IP6ReflexiveAddr = IP6-address COLON PortNumber PortNumber = UINT16 LifeT = UINT32 EToken = "E" The ABNF syntax of IP4-address and IP6-address is defined in [IETF RFC 4566]. The MG may reply with the relay address, reflex address and the value of LIFETIME attribute, for example "192.168.1.10:10000 192.168.2.100:20000 11000" and "FF1E:03AD::7F2E:172A:1E24:20000 FF1E:03AD::7F2E:2AB3:9AA8:20000 30000". Or it may reply with an error indication in which the class and number of the error (see clause 11.2.9 of [IETF RFC 3489] or clause 15.6 of [IETF RFC 5389]) is included. |
|  | Default: | Empty List |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |

#### 7.3.1.2	TURN refresh

|  | Property name: | TURN Refresh |
| --- | --- | --- |
|  | Property ID: | turnr (0x0002) |
|  | Description: | This property requests for a particular local transport address and port that the MG perform a TURN refresh request with a remote TURN server in order to keep the allocation and change the bandwidth or lifetime. As per clause 7 of [IETF RFC 5766], a Refresh transaction can be used to either (a) refresh an existing allocation and update its time-to-expire, or (b) delete an existing allocation. MG should send periodic refresh request for each local transport address and port automatically. The MGC may request the MG to send a special refresh request to change bandwidth or lifetime via this property. |
|  | Type: | List of String |
|  | Possible values: | In an ITU-T H.248 command request: Each element in the list of string SHALL be of type RefreshReq in ABNF format: RefreshReq = ("R"[COLON LifeT])/"N" LifeT = UINT16 "R": Refresh Request – Perform a TURN refresh request on the address in this list position. Lifetime contains the value of the LIFETIME attribute. If the value of LIFETIME attribute is zero, this will cause the IP termination to remove the allocation, and all associated permissions and channel numbers. If a value for lifetime is not included a default lifetime as per [IETF RFC 5766] shall be assumed. "N": No change – Do not perform TURN refresh request for the address in this position. In an ITU-T H.248 command reply: Each element of the string may be LIFETIME. Each element in the list of string SHALL be of type RefreshReply in ABNF format: RefreshReply = LifeT/ EToken [:ErrorCode] Lifetime contains the value of the LIFETIME attribute. As per clause 7.2 of [IETF RFC 5766], in a successful response, the LIFETIME attribute indicates the amount of additional time (the number of seconds after the response is received) that the allocation will live without being refreshed. Or reply with an error information in which the Class and Number of the error (see clause 11.2.9 of [IETF RFC 3489] or clause 15.6 of [IETF RFC 5389]) is included. |
|  | Default: | Empty List |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |

#### 7.3.1.3	NAT lifetime

|  | Property name: | NAT Lifetime |
| --- | --- | --- |
|  | Property ID: | natl (0x0003) |
|  | Description: | This property requests the MG to return the NAT binding lifetime associated with the transport address at a particular list position. |
|  | Type: | List of String |
|  | Possible values: | In an ITU-T H.248 command request: Each element may be one of the following characters: T:"Time Request"; N:"No time request". In an ITU-T H.248 command reply: A string based integer representing a Lifetime value, Or: E:"Error in list position", followed by the class and number of the error (clause 11.2.9 of [IETF RFC 3489]) is included in the response. |
|  | Default: | Empty List |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |

### 7.3.2	Events

None.

### 7.3.3	Signals

None.

### 7.3.4	Statistics

None.

### 7.3.5	Error codes

None.

### 7.3.6	Procedures

To determine a TURN relayed address the MGC shall issue an ITU-T H.248Add/Modify/Move.req command containing the "turna" property on the relevant termination/stream. The MGC shall include a value in each position of list of string for each element described in the "stunb/ac" property.

For list positions designated with the value "N", the MG shall simply return an empty string for that position of the list.

For list positions designated with the value "A", the MG shall perform a TURN allocate request messaging. If the allocate request is successful the MG shall return the relayed address, reflex address and the value of LIFETIME attribute in the list position of the reply. The following attributes may be derived from the "turna" property:

EVEN-PORT, clause 14.6 of [IETF RFC 5766]

LIFETIME, clause 14.2 of [IETF RFC 5766]

Communication between the TURN client and the TURN server can run over UDP, TCP or TLS. "TurnTransportType" in the syntax of the "turna" property is used to indicate the desired transport type.

When the TURN client has data to send to a peer, it may use either a ChannelData message or a Send indication. "ChannelSend" in the syntax of the "turna" property is used to indicate which method should be used.

If the allocate request is unsuccessful then the TURN error shall be returned in the list position.

If the MGC wishes to reset the mappings, then it shall resend the "turna" property in an ITU-T H.248 request indicating "N" or "A".

To refresh an existing allocation and update its time-to-expire, or to delete an existing allocation, the MGC shall issue a Modify/Move.req command containing the "turnr" property on the relevant termination/stream.

For list positions designated with the value "R" the MG shall perform TURN refresh request messaging. If the MGC wishes the TURN server to set the time-to-expire timer to something other than the default lifetime, the MGC indicates to the MG to include a LIFETIME attribute with the requested value. If the value of the LIFETIME attribute is 0, the TURN server immediately deletes the allocation. If the refresh request is successful the MG shall return the LIFETIME attribute in the list position of the reply. If the refresh request is unsuccessful then the TURN error shall be returned in the list position.

## 7.4	MGC STUN client package

|  | Package name: | MGC STUN Client |
| --- | --- | --- |
|  | Package ID: | mgcstunc (0x00c0) |
|  | Description: | This package enables an MGC to determine the STUN mapped IP address and port for a particular media component. This package allows the STUN client to remain at the MGC level rather than being implemented in the MG. NOTE – This package has been included for backward compatibility reasons. |
|  | Version: | 1 |
|  | Extends: | None |

### 7.4.1	Properties

None.

### 7.4.2	Events

None.

### 7.4.3	Signals

#### 7.4.3.1	MGC initiated STUN request

|  | Signal name: | MGC Initiated STUN Request |
| --- | --- | --- |
|  | Signal ID: | mgcistunr (0x0001) |
|  | Description: | The MGC shall send a signal for each media component address whose address is to be mapped. These signals may occur in the same ITU-T H.248 message. The STUN response messages are not used in this case. It is used to fulfil the case described in clause 10.3 of [IETF RFC 3489]. |
|  | Signal type: | Brief |
|  | Duration: | Not applicable |

##### 7.4.3.1.1	Binding request message

|  | Parameter name: | Binding Request Message |
| --- | --- | --- |
|  | Parameter ID: | brm (0x0001) |
|  | Description: | This parameter contains the MGC constructed STUN request message. |
|  | Type: | Octet String |
|  | Optional: | No |
|  | Possible values: | Any valid STUN request message. |
|  | Default: | None |

##### 7.4.3.1.2	Transport address

|  | Parameter name: | Transport Address |
| --- | --- | --- |
|  | Parameter ID: | ta (0x0002) |
|  | Description: | This parameter indicates the IP address of the media component that is to have its address STUN mapped. |
|  | Type: | Integer |
|  | Optional: | No |
|  | Possible values: | Any valid list position from the "stunb/ac" parameter. |
|  | Default: | None |

### 7.4.4	Statistics

None.

### 7.4.5	Error codes

None.

### 7.4.6	Procedures

The MGC provides the STUN message contents, STUN server address and media address to the MG which will send a packet from the media address containing the STUN message contents to the STUN server address. The response will be provided to the MGC, thus any protocol or error handling can remain at the MGC level. This package enables the scenario described in clause 10.3 of [IETF RFC 3489].

## 7.5	STUN information package

|  | Package name: | STUN Information |
| --- | --- | --- |
|  | Package ID: | stuni (0x00c1) |
|  | Description: | This package enables an MGC to determine the type of NAT the MG is behind and the binding lifetime associated with the STUN server. Signals and events may be applied to the Root Termination. |
|  | Version: | 1 |
|  | Extends: | None |

### 7.5.1	Properties

None

### 7.5.2	Events

#### 7.5.2.1	NAT type determination

|  | Event name: | NAT Type Determination |
| --- | --- | --- |
|  | Event ID: | nattd (0x0001) |
|  | Description: | This event detects the end of the NAT Type determination procedure (initiated by a NAT Determination signal) and returns the detected type. |

##### 7.5.2.1.1	EventsDescriptor parameters

None

##### 7.5.2.1.2	ObservedEventsDescriptor parameters

###### 7.5.2.1.2.1	NAT Type

|  | Parameter name: | NAT Type |
| --- | --- | --- |
|  | Parameter ID: | natt (0x0001) |
|  | Description: | The determined NAT type (scenario) as per clause 10.1 of [IETF RFC 3489]. NOTE - The original STUN protocol [IETF RFC 3489] was obsoleted by [IETF RFC 5389]. The original NAT type discovery process as per clause 10.1 of [IETF RFC 3489] was moved to a self-contained specification [IETF RFC 5780]. Clause 4/[IETF RFC 5780] describes the NAT type discovery process using NAT type naming according to [IETF RFC 4787]. [IETF RFC 6888] also uses the NAT type naming according to [IETF RFC 4787]. |
|  | Type: | Enumeration |
|  | Optional: | No |
|  | Possible values: | 0x0000: On the open Internet; 0x0001: Firewall that blocks UDP; 0x0002: Firewall that allows UDP out, and responses have to come back to the source of the request (like a symmetric NAT, but no translation). A symmetric UDP Firewall; 0x0003: Full-cone NAT; 0x0004: Symmetric NAT; 0x0005: Restricted cone NAT; 0x0006: Restricted port cone NAT. |
|  | Default: | None |

#### 7.5.2.2	Binding lifetime determination

|  | Event name: | Binding Lifetime Determination |
| --- | --- | --- |
|  | Event ID: | bld (0x0002) |
|  | Description: | This event detects the end of the Binding Lifetime determination procedure (initiated by a Binding Lifetime Determination signal) and returns the time period of the binding. |

##### 7.5.2.2.1	EventsDescriptor parameters

None.

##### 7.5.2.2.2	ObservedEventsDescriptor parameters

###### 7.5.2.2.2.1	Binding lifetime

|  | Parameter name: | Binding Lifetime |
| --- | --- | --- |
|  | Parameter ID: | bl (0x0001) |
|  | Description: | The period of the life of the binding. |
|  | Type: | Integer |
|  | Optional: | No |
|  | Possible values: | 0-3600 seconds |
|  | Default: | None |

### 7.5.3	Signals

#### 7.5.3.1	NAT type determination

|  | Signal name: | NAT Type Determination |
| --- | --- | --- |
|  | Signal ID: | nattd (0x0001) |
|  | Description: | This signal instructs the MG to detect the type of NAT that a particular media component IP address is behind. |
|  | Signal type: | Brief |
|  | Duration: | NA |

##### 7.5.3.1.1 Additional parameters

###### 7.5.3.1.1.1	Transport address

|  | Parameter name: | Transport Address |
| --- | --- | --- |
|  | Parameter ID: | ta (0x0001) |
|  | Description: | This parameter indicates the IP address of the media component that is to have its address STUN mapped. |
|  | Type: | Integer |
|  | Optional: | No |
|  | Possible values: | Any valid list position from the "stunb/ac" parameter. |
|  | Default: | None |

#### 7.5.3.2	Binding lifetime determination

|  | Signal name: | Binding Lifetime Determination |
| --- | --- | --- |
|  | Signal ID: | bld (0x0002) |
|  | Description: | This signal instructs the MG to detect the binding lifetime with the NAT. |
|  | Signal type: | Brief |
|  | Duration: | NA |

##### 7.5.3.2.1	Additional parameters

##### 7.5.3.2.1.1		Transport address

|  | Parameter name: | Transport Address |
| --- | --- | --- |
|  | Parameter ID: | ta (0x0001) |
|  | Description: | This parameter indicates the IP address of the media component that is to have its address STUN mapped. |
|  | Type: | Integer |
|  | Optional: | No |
|  | Possible values: | Any valid list position from the "stunb/ac" parameter. |
|  | Default: | None |

### 7.5.4	Statistics

None.

### 7.5.5	Error codes

None.

### 7.5.6	Procedures

#### 7.5.6.1	NAT behaviour discovery as defined for original STUN protocol

This package is used to support the procedures as outlined in clause 10 of [IETF RFC 3489].

#### 7.5.6.2	NAT behaviour discovery as defined for updated STUN protocol

This package could be also used to support the procedures according to [IETF RFC 5780].

# 8	ICE support

{Editor’s note (2012-09 meeting): this section is apparently structured along individual SDP elements related to ICE. Thus, there’s a prerequisite that the reader is fully familiar with ICE. Readability might be enhanced by introducing briefly the concerned SDP elements and/or provide a list or subclause structure. 
Contributions are solicited.}

The support of ICE implies that the SDP CANDIDATE attribute is supported. This can facilitate address gathering and description of candidates. However, due to the nature of the ITU-T H.248 connection model, several procedural changes are required.

Address gathering

The MGC may request an MG to perform address discovery through the use of the wildcarding mechanism on the CANDIDATE attribute. In this case the MG is responsible for STUN server discovery. Fully specifying the CANDIDATE attribute will not result in any action. The candidate-attribute is specified by the following ABNF [IETF RFC 5245]:

candidate-attribute = "candidate" ":" foundation SP component-id SP

transport SP

priority SP

connection-address SP     ;from [IETF RFC 4566]

port     ;port from [IETF RFC 4566]

SP cand-type

[SP rel-addr]

[SP rel-port]

*(SP extension-att-name SP

extension-att-value)

NOTE 1 – Clause 4.5/[IETF RFC 6544] defines an extended ABNF grammar for the SDP "a=candidate:" attribute for TCP.

NOTE 2 – Trickle ICE reuses above vanilla ICE syntax for the SDP "a=candidate:" attribute (see clause 9.2/[b-IETF ice-trickle].

NOTE 3 – The priority value is the result of a particular address prioritization algorithm (see clause 4.1.2.1/[IETF RFC 5245] for basic rules and [b-IETF ice-dualstack] rule related to ICE IPv4/IPv6 dual stack fairness).

{Editor’s note (2014-11): [b-IETF ice-dualstack] is still preliminary. Whether the prioritization algorithm will be impacted depends on the progress of this draft.}

The MGC shall either include values for, or wildcard mandatory fields. Including values provides a way of narrowing down the selection. For example:

If the MGC wanted a particular candidate type it could specify:

a=candidate:1 1 UDP  $ $ typ host

a=candidate:2 1 UDP  $ $ srflx raddr  $ rport $

and the host address and server reflexive address would be returned.

If the MGC wanted all candidate types it could specify:

a=candidate:$ $ $ $ $ $ typ *

and the local, server reflexive address and relay addresses would be returned.

If a request results in multiple candidates then these shall be returned in multiple "a=" lines.

As per clause 4.3 of [IETF RFC 5245], if the MG utilizes RTCP, the MGC and MG MUST encode the RTCP candidate using the "a=rtcp" attribute as defined in [IETF RFC 3605]. If RTCP is not in use, the MGC and MG MUST signal that by using "b=RS:0" and "b=RR:0" as defined in [IETF RFC 3556].

If the MGC requires a MG to allocate a port for RTCP, the MGC will send the required information to request an RTCP port (e.g., "a=" line in SDP, a=rtcp $).

A further complication is provided by the selection of the "In-Use candidate" in the SDP media (m=) and connection (c=) lines. In the ITU-T H.248 local descriptor this is a local transport address in the MG. Whereas in ICE this may be another address. Therefore, in order to construct an SDP offer, the MGC shall determine the "In-Use candidate" from the CANDIDATE attributes returned in the response.

The remote-candidate-att is related to the SDP (SIP) offer and provides the identity of the remote candidates that the offerer wishes the answerer to use in its answers. It is assumed that the MGC (acting as the offerer) will populate this attribute based on candidate information from the MG.

Similarly the ICE SDP "ice-pwd-att" attribute may be wildcarded or provided.

If the MGC asks for candidates, it should provide or ask for credentials in order for the MG to be able to perform the necessary signalling.

If MGC provides credentials, the MGC will send the required "a=" lines in SDP. For example:

a=ice-pwd:asd88fgpdd777uzjYhagZg

a=ice-ufrag:8hhY

If the MGC asks the MG to provide the credentials, the MGC will send the required "a=" lines in SDP. For example:

a=ice-pwd:$

a=ice-ufrag:$

If MGC does not supply the necessary "a=" lines in SDP, the MG may determine these credentials and return them to the MGC in a reply message. This is in order for the MG to be able to send a correct STUN message.

The use of the "a=ice-lite" attribute indicates whether or not the agent implements a lite version of ICE (see also clause 6.1.6 and Appendix III). As the agent resides in the MG this information needs to be provided to the MGC in order for the information to be communicated to a peer. Given the structure of the "a=ice-lite" attribute the MGC is unable to perform a CHOOSE wildcard operation on the attribute. As such where the MG implements a "lite" version of ICE it shall include the "a=ice-lite" attribute in any command response containing the candidate attribute.

{Editor’s note (2012-09 meeting): this clause is a placeholder (item #1) for providing use case details on ICE full and ICE lite and impact on H.248 entities. 
Contributions are solicited.}

Where a binary encoding of ITU-T H.248 is used, the ICE-related attributes may be realized through the use of [ITU-T H.248.1] Annex C.11 properties. In order to facilitate multiple values the "sub-list of" form should be used.

{Editor’s note (2012-09 meeting): this clause is a placeholder related to ICE control mode details (see item #2). 
Contributions are solicited.}

Connectivity checking

In order for an MG to perform some ICE operations, the remote candidate list is required. This remote candidate list can be provided to the MG in the form of candidate attributes being placed in the appropriate remote descriptors. The MGC may be required to reformat the SDP answer/offer received (i.e., via SIP) in order for it to be applicable to ITU-T H.248 structures. The setting of the candidate attributes in the remote descriptor will not in itself generate any ICE operation such as Connectivity Checking or keep alive mechanisms. ITU-T H.248 packages are used to trigger these operations.

## 8.1	MG act-as STUN server package

|  | Package name: | MG Act-as STUN Server |
| --- | --- | --- |
|  | Package ID: | mgastuns (0x00c2) |
|  | Description: | This package enables an MGC to request that a particular address on an MG act as a STUN server in order to process, receive binding requests and return STUN binding responses. The purpose of the package is to enable the procedures defined in clause 7.2 of [IETF RFC 5389]. |
|  | Version: | 1 |
|  | Extends: | None |

### 8.1.1	Properties

#### 8.1.1.1	Act-as STUN server

|  | Property name: | Act-as STUN Server |
| --- | --- | --- |
|  | Property ID: | astuns (0x0001) |
|  | Description: | This property requests for a particular local transport address that the MG be prepared to receive STUN binding requests for the purpose of connectivity checking. |
|  | Type: | List of String |
|  | Possible values: | In an ITU-T H.248 command request: Each element in the list of string SHALL be of type MGActServer in ABNF format: MGActServer = GroupID "¦" Foundation "¦" Component-id "¦" RequestType GroupID = UINT16 Foundation = UINT16 Component-id = UINT16 RequestType = "N"/"S" GroupId is as per UINT16 in clause B.2 of [ITU-T H.248.1]. Component-id is as per <component-id> in clause 15.1 of [IETF RFC 5245]. Foundation is as per <foundation> in clause 15.1 of [IETF RFC 5245]. RequestType is: N ("No request"): do not process STUN requests; S ("STUN Server"): act as a STUN server receiving binding requests. |
|  | Default: | "STUN Server" |
|  | Defined in: | LocalControl |
|  | Characteristics: | Read/Write |

### 8.1.2	Events

None.

### 8.1.3	Signals

None.

### 8.1.4	Statistics

None.

### 8.1.5	Error codes

None.

### 8.1.6	Procedures

To request the MG to act as a STUN server, the MGC should issue an ITU-T H.248 Add/Modify/Move.req command containing the "astuns" property on the relevant Termination/Stream.

## 8.2	Originate STUN continuity check package

|  | Package name: | Originate STUN Continuity Check |
| --- | --- | --- |
|  | Package ID: | ostuncc (0x00c3) |
|  | Description: | This package enables an MGC to initiate a STUN continuity check binding request process. The purpose of the package is to enable the procedures defined in clause 7 of [IETF RFC 5389]. |
|  | Version: | 1 {Editor's note (2014-06): package should be upversioned to 2 in order to indicate additional support of TCP.} |
|  | Extends: | None |

### 8.2.1	Properties

#### 8.2.1.1	Host candidate realm

|  | Property name: | Host Candidate Realm |
| --- | --- | --- |
|  | Property ID: | hcr (0x0001) |
|  | Description: | This property indicates the realm the MG allocates IP address and port for host candidate. As per clause 1 of [IETF RFC 5245], because ICE exchanges a multiplicity of IP addresses and ports for each media stream, it also allows for address selection for multi-homed and dual-stack hosts. As per clause 4.1.1.1 of [IETF RFC 5245], host candidates are obtained by binding to ports (typically ephemeral) on an IP address attached to an interface (physical or virtual, including VPN interfaces) on the host. The information of host candidate is described in local SDP. If MGC requires the MG to gather more than one host candidate via local SDP, it uses this property to indicate to the MG to gather each of those host candidates in the indicated IP realm or VPN. |
|  | # Type: | List of String |
|  | # Possible values: | Each element in the list of string SHALL be of type HostCandRealm in ABNF format: HostCandRealm = GroupID "¦" Foundation "¦" Component-id "¦" Realm GroupID = UINT16 Foundation = UINT16 Component-id = UINT16 Realm = ALPHA 0*63(ALPHA/ DIGIT) ALPHA = %x41-5A / %x61-7A ; A-Z / a-z DIGIT = %x30-39; 0-9 Where: GroupId is as per UINT16 in clause B.2 of [ITU-T H.248.1]. Component-id is as per <component-id> in clause 15.1 of [IETF RFC 5245]. Foundation is as per <foundation> in clause 15.1 of [IETF RFC 5245]. Realm is a string used to discriminate overlapping IP address spaces. It is the identifier of the IP domain or VPN. e.g., "1¦1¦1¦realm1", "1¦2¦1¦realm2" And MGC sends such a local SDP to a MG: v=0 c=IN IP4 $ a=ice-pwd:YH75Fviy6338Vbrhrlp8Yh a=ice-ufrag:9uB6 m=audio $ RTP/AVP 0 a=candidate:1 1 UDP 2130706431 $ $ typ host a=candidate:2 1 UDP 2113929215 $ $ typ host The MG should gather two host candidates. The first one is in IP realm "realm1" and the second one is in IP realm "realm2". |
|  | # Default: | None |
|  | # Defined in: | LocalControl |
|  | # Characteristics: | Read/Write |

{Editor’s note (2014-06 meeting): this property may also support TCP-based candidates [IETF RFC 6544] without any syntactical & semantical extensions. To be cross-checked and confirmed. If yes, then an explicit Note might be beneficial. 
Contributions are solicited.}

### 8.2.2	Events

#### 8.2.2.1	Connectivity check result

|  | # Event name: | Connectivity Check Result |
| --- | --- | --- |
|  | # Event ID: | ccr (0x0001) |
|  | # Description: | This event returns the result of an ICE connectivity check. |

##### 8.2.2.1.1	EventsDescriptor parameters

None.

##### 8.2.2.1.2	ObservedEventsDescriptor parameters

###### 8.2.2.1.2.1	Candidate/transport pair

|  | # Parameter name: | Candidate/Transport Pair |
| --- | --- | --- |
|  | # Parameter ID: | ctp (0x0001) |
|  | # Description: | The list of the successful candidate/transport pairs. |
|  | # Type: | List of String |
|  | # Optional: | No |
|  | # Possible values: | Each element in the list of string SHALL be of type CandPair in ABNF format: CandPair = StreamID "¦" GroupID "¦" Foundation-l "¦" Foundation-r "¦" Component-id [Lp-connection- address] [Rp-connection-address] [Rc] StreamID = UINT16 GroupID = UINT16 Foundation-l = UINT16 Foundation-r = UINT16 Component-id = UINT16 Lp-connection-address = "¦lp-" Connection-address COLON PortNumber Rp-connection-address = "¦rp-" Connection-address COLON PortNumber Connection-address = IP4-address Rc = "¦RC" PortNumber = UINT16 The ABNF syntax of IP4-address is defined in [IETF RFC 4566]. Where: StreamID is as per StreamId in clause B.2 of [ITU-T H.248.1]. GroupID is as per UINT16 in clause B.2 of [ITU-T H.248.1]. Foundation-l (Local Foundation) is as per <foundation> in clause 15.1 of [IETF RFC 5245]. Foundation-r (Remote Foundation) is as per <foundation> in clause 15.1 of [IETF RFC 5245]. Component-id is as per <component-id> in clause 15.1 of [IETF RFC 5245]. Lp (Local) Peer reflexive candidate as described in clause 7.1.3.2 of [IETF RFC 5245]. Rp Peer reflexive remote candidate as described in clause 7.2.1.3 of [IETF RFC 5245]. Rc (Role Change) is a flag to indicate that the control role has changed (i.e., controlled or controlling). Control role is described in clause 7.2.1.1 of [IETF RFC 5245]. e.g., "1¦2¦3¦4¦1¦lp-202.2.3.4:1000" Means that StreamID is 1, GroupID is 2, Local Foundation is 3, Remote Foundation is 4 , Component-id is 1, and there is a local peer reflexive candidate in this candidate pair. The IP address of this local peer reflexive candidate is 202.2.3.4, and the port is 1000. |
|  | # Default: | None |

{Editor’s note (2014-06 meeting): this parameter may also support TCP-based candidates [IETF RFC 6544] without any syntactical & semantical extensions. To be cross-checked and confirmed. If yes, then an explicit Note might be beneficial. 
Contributions are solicited.}

#### 8.2.2.2	New peer reflexive candidate

|  | # Event name: | New Peer Reflexive Candidate |
| --- | --- | --- |
|  | # Event ID: | nprc (0x0002) |
|  | # Description: | This event indicates that a new peer reflexive candidate was discovered during a connectivity check. |

##### 8.2.2.2.1	EventsDescriptor parameters

None.

##### 8.2.2.2.2	ObservedEventsDescriptor parameters

###### 8.2.2.2.2.1	Candidate

|  | # Parameter name: | Candidate |
| --- | --- | --- |
|  | # Parameter ID: | can (0x0001) |
|  | # Description: | A list of the newly discovered peer reflexive candidates. |
|  | # Type: | Sub-list of String |
|  | # Optional: | No |
|  | # Possible values: | Each element in the list of string SHALL be of type peerCand in ABNF format: peerCand = StreamID "¦" GroupID "¦" Foundation "¦" Component-id "¦" Connection-address COLON PortNumber StreamID = UINT16 GroupID = UINT16 Foundation = UINT16 Component-id = UINT16 Connection-address = IP4-address PortNumber = UINT16 The ABNF syntax of IP4-address is defined in [IETF RFC 4566]. Where: StreamID is as per StreamId in clause B.2 of [ITU-T H.248.1]. GroupID is as per UINT16 in clause B.2 of [ITU-T H.248.1]. Foundation is as per <foundation> in clause 15.1 of [IETF RFC 5245]. Component-id is as per <component-id> in clause 15.1 of [IETF RFC 5245]. A newly discovered peer reflexive candidate as described in clause 7.1.3 of [IETF RFC 5245]. e.g., "1¦2¦3¦4¦202.2.3.4:1000" Means that the StreamID is 1, GroupID is 2, Foundation is 3, Component-id is 4, and a new peer reflexive candidate was discovered. The IP address of this peer reflexive candidate is 202.2.3.4, and the port is 1000. |
|  | # Default: | None |

{Editor’s note (2014-06 meeting): this parameter may also support TCP-based candidates [IETF RFC 6544] without any syntactical & semantical extensions. To be cross-checked and confirmed. If yes, then an explicit Note might be beneficial. 
Contributions are solicited.}

### 8.2.3	Signals

#### 8.2.3.1	Send Connectivity Check

|  | # Signal name: | Send Connectivity Check |
| --- | --- | --- |
|  | # Signal ID: | scc (0x0001) |
|  | # Description: | This signal initiates connectivity checking procedures as defined in clause 7 of [IETF RFC 5245]. |
|  | # Signal type: | Brief |
|  | # Duration: | NA |

##### 8.2.3.1.1 Additional parameters

##### 8.2.3.1.1.1 	Control

|  | # Parameter name: | Control |
| --- | --- | --- |
|  | # Parameter ID: | cntrl (0x0001) |
|  | # Description: | This parameter indicates the controlling role defined in clause 7.1.2.2 of [IETF RFC 5245]. |
|  | # Type: | Enumeration |
|  | # Optional: | Yes |
|  | # Possible values: | Controlling (0x0001): MG acts as controlling role Controlled (0x0002): MG acts as controlled role |
|  | # Default: | Controlling |
|  | # Characteristics: | Read |

#### 8.2.3.2	Send additional connectivity check

|  | # Signal name: | Send Additional Connectivity Check |
| --- | --- | --- |
|  | # Signal ID: | sacc (0x0002) |
|  | # Description: | This signal instructs a MG to initiate additional ICE connectivity check procedures in the case additional candidate/transport address pairs are identified. The MG shall use the CANDIDATE attributes in both the local and remote descriptors to determine the candidate/transport address pairs to use for the connectivity check. The MG shall resume and only perform checks not already performed. |
|  | # Signal type: | Brief |
|  | # Duration: | NA |

##### 8.2.3.2.1	Additional parameters

###### 8.2.3.2.1.1	Control

|  | # Parameter name: | Control |
| --- | --- | --- |
|  | # Parameter ID: | cntrl (0x0001) |
|  | # Description: | This parameter indicates the controlling role defined in clause 7.1.2.2 of [IETF RFC 5245]. |
|  | # Type: | Enumeration |
|  | # Possible values: | Controlling (0x0001): MG acts as controlling role Controlled (0x0002): MG acts as controlled role |
|  | # Default: | Controlling |
|  | # Characteristics: | Read |

### 8.2.4	Statistics

None.

### 8.2.5	Error codes

None.

### 8.2.6	Procedures

#### 8.2.6.1	Connectivity check procedures for UDP

When the "Send Connectivity Check" (scc) signal is sent, the MGC should initiate the STUN continuity check procedures as outlined in clause 7 of [IETF RFC 5245]. As the ICE agent functionality is split between a MGC and MG and the MG is responsible for sending the connectivity checks, the agent role (controlled or controlling, see clause 5.2 of [IETF RFC 5245]) should be provided. In order for the MGC to obtain the result of the connectivity check, it shall set the "Connectivity Check Result" (ccr) event.

The usage of the "Send Connectivity Check" signal shall clear the results of any previous connectivity checking on the Termination/Stream.

If the ccr event is set and the scc signal is received, the MG shall apply the procedures of clause 7.1.2 of [IETF RFC 5245] to form and prioritize (NOTE 1) a checklist.

NOTE 1 – The calculation of the priority value follows basically clause 4.1.2.1/[IETF RFC 5245], though, there might be other policies such as [b-IETF ice-dualstack].

Once the procedures of clause 7.1.3 of [IETF RFC 5245] have been completed, the MG shall then notify the MGC of the result. The MG shall deem the procedures of clause 5.8 of [IETF RFC 5245] completed once all checks have reached the state "succeeded" or "failed". In order to uniquely identify the checks on the termination, the MG shall provide the StreamID, GroupID, foundation of the local candidate, foundation of the remote candidate and the component-id. If the control role of this candidate pair changes, an optional flag "Rc" is included in the end. The MG shall maintain the results of the checking for the lifetime of the Termination/Stream or until they are cleared by a subsequent "Send Connectivity Check" signal.

Where there is only one stream or group the respective ids default to 1.

If there are no successful candidate pairs for one of the particular component-ids in the list, then the connectivity check for that component-id has failed.

If there are more than one successful candidate pairs for a particular component-id in the list, only the candidate pair having the highest priority will be reported. The "Sub-list of" shall be in the same order as the checklist in order to maintain relative priority.

Depending on its connection role, "controlled" or "controlling", the MGC may use the result in the ObservedEventsDescriptor parameter ccr/ctp to determine which transport pair to use for the media connection.

If a new peer reflexive candidate is discovered as per the procedure in clause 7.1.3.2.1 of [IETF RFC 5245], and if the "New Peer Reflexive Candidate" (nprc) event is set, the MG will notify the MGC with the new peer reflexive candidate. In order to identify this peer reflexive candidate, the MG shall provide the StreamID, GroupID, Foundation, Component-id and the transport IP address and port number of this peer reflexive candidate. If the MGC requires that the peer reflexive candidate be paired with other remote candidates besides the one in the valid pair that will be generated, the MGC may generate an updated offer which includes the peer reflexive candidate. This will cause it to be paired with all other remote candidates. The list of the candidate pairs is updated in this case.

When a new peer reflexive candidate is discovered, the Connectivity Checks procedure continues. If the MGC updates the local candidates via the SDP in the ITU-T H.248 message, the MG will continue with any connectivity checks that are still in progress. It will not perform checks on the new candidates. The MGC shall send the "Send Additional Connectivity Check" (sacc) signal to perform checking of the new candidates.

In some kinds of conditions, where the list of the connectivity check candidate pairs is changed (i.e., a new stream is added, an existing stream is modified or a new peer reflexive candidate is discovered) additional connectivity check procedures may be initiated. The MGC may send the "Send Additional Connectivity Check" signal to initiate a connectivity check procedure on any candidate/transport pair not previously checked. In this procedure, the MG shall only perform checks not already performed. Another example is where a new stream is added after the connectivity checks for the existing stream have finished. In this case, the additional connectivity checks will only check the candidate pairs of the newly added stream.

#### 8.2.6.2	Connectivity check procedures for TCP

This package may be also applied for TCP based; the procedures of previous clause 8.2.6.1 are applicable under consideration of TCP specific aspects according to clause 7/[IETF RFC 6544].

## 8.3	STUN consent freshness package

|  | Package name: | STUN Consent Freshness |
| --- | --- | --- |
|  | Package ID: | stnconfres (0x0120) |
|  | Description: | This package allows an MGC to request an MG to perform the STUN usage for Consent procedures described in [IETF STUNCONFRES]. This allows the MG to send STUN binding requests and receive STUN binding responses in order to determine whether application data may be sent on the Stream. |
|  | Version: | 1 |
|  | Extends: | None |

### 8.3.1	Properties

None.

## 8.3.2	Events

#### 8.3.2.1	Consent State

|  | Event name: | Consent State |
| --- | --- | --- |
|  | Event ID: | constate (0x0001) |
|  | Description: | This Event allows a MG to indicate to the MGC whether consent has been granted or not for a particular stream. It is only reported if the state changes between "granted" and "not granted" (i.e. revoked) states and vice versa. In consent "not granted" state application data stops flowing. In consent "granted" state application data flows. |

##### 8.3.2.1.1	EventsDescriptor parameters

###### 8.3.2.1.1.1	Request States

|  | Parameter name: | Request States |
| --- | --- | --- |
|  | Parameter ID: | reqstate (0x0001) |
|  | Description: | The parameter allows the MGC to request notification of a change to "granted" and/or "not granted" state. |
|  | Type: | Enumeration |
|  | Optional: | Yes |
|  | Possible values: | G Notify a transition to state "granted" N Notify a transition to state "not granted" B Notify a transition to state "granted" or "not granted" |
|  | Default: | B |

##### 8.3.2.1.2	ObservedEventsDescriptor parameters

###### 8.3.2.1.2.1	State

|  | Parameter name: | State |
| --- | --- | --- |
|  | Parameter ID: | state (0x0001) |
|  | Description: | This parameter indicates the consent state that the transport has transitioned to. |
|  | Type: | Enumeration |
|  | Optional: | No |
|  | Possible values: | G Consent has been granted and application data is flowing. N Consent is not granted and application data is not flowing. |
|  | Default: | None. |

#### 8.3.2.2	STUN Consent Request Failure

|  | Event name: | STUN Consent Request Failure |
| --- | --- | --- |
|  | Event ID: | confail (0x0002) |
|  | Description: | This Event allows a MG to indicate to the MGC when a response to a STUN binding request indicating the failure to renew consent. |

##### 8.3.2.2.1	EventsDescriptor parameters

##### None.

##### 8.3.2.2.2	ObservedEventsDescriptor parameters

### None.

### 8.3.3	Signals

#### 8.3.3.1	Consent Test

|  | Signal name: | Consent Test |
| --- | --- | --- |
|  | Signal ID: | contst (0x0001) |
|  | Description: | This signal initiates the combined consent freshness test using STUN request/response procedures as outlined in clause 4.1/[IETF STUNCONFRES]. |
|  | Signal Type: | On/Off |
|  | Duration: | Not applicable. |

##### 8.3.3.1.1	Additional parameters

###### 8.3.3.1.1.1	Test Interval

|  | Parameter name: | Test Interval |
| --- | --- | --- |
|  | Parameter ID: | tstint (0x0001) |
|  | Description: | This parameter indicates the consent check interval, the parameter "N" as defined by clause 4.1/[IETF STUNCONFRES]. The MG sends consent checks at random intervals between 0.8N and 1.2N. |
|  | Type: | Integer |
|  | Optional: | Yes |
|  | Possible values: | 1 millisecond upwards NOTE – The typical test interval is at the time scale of seconds and tens of seconds. Small interval sizes are not recommended due to significant messaging and processing overhead that this would introduce. |
|  | Default: | 5000 ms |

### 8.3.4	Statistics

None.

### 8.3.5	Error Codes

None.

### 8.3.6	Procedures

#### 8.3.6.1 STUN Consent Freshness initiation

To initiate the STUN consent freshness procedures from [IETF STUNCONFRES] the MGC shall send the "Consent Test" (stnconfres/contst) Signal on the H.248 Stream where the consent is required. The stnconfres/contest Signal should be sent prior to or in the same H.248 command as any signals initiating ICE connectivity checking to ensure that consent is granted as a result of successful initial ICE procedures.

On reception of the stnconfres/contest signal the MG shall initiate the consent procedures in clause 4.1/[IETF STUNCONFRES]. As described the MG must not send application data (e.g., RTP, RTCP, SCTP, DTLS), over any transport protocol (e.g., UDP, TCP) on an ICE-initiated connection unless the receiving endpoint consents to receive the data. After a successful ICE connectivity check on a particular transport address, subsequent consent shall be obtained following the procedures described [IETF STUNCONFRES]. The MG shall send a STUN binding request consent check at the random interval indicated by the "Test Interval" (tstint) parameter in order to maintain consent.

When consent is granted the MG may send application data on the Stream. The granting of consent does not override other methods for controlling the flow of application or transport related data such as the Local Control StreamMode property [ITU-T H.248.1].

In order to be notified about the consent state the MGC should set the "Consent State" (stnconfres/contest) Event with the required states on the applicable Stream. As the Event indicates state transitions, the MGC should set the Event when sending the stnconfres/contest Signal to ensure the initial transition from consent "not-granted" (i.e. the default state before the completion of ICE connectivity check when the stnconfres/contest Signal is active) to a granted state after initial consent checking is reported.

The state transitions to a consent "granted" state on receipt of a valid STUN binding response. The state transitions to a consent "not-granted" state when a valid STUN binding response corresponding to one of the STUN requests sent in the last 30 seconds has not been received from the remote peer's Transport Address or the MG detects an immediate revocation of consent via one of the methods in clause 4.2/[IETF STUNCONFRES]. A MGC may also be notified of the revocation of consent based on the detection of Events related to a closure of the transport connection, i.e. a TLS BNCChange Event indicating "bearer release" [ITU-T H.248.90].

#### 8.3.6.2 STUN consent request failure

In order to be notified of a failed STUN consent refresh the MGC shall set the "STUN Consent Request Failure" (stnconfres/confail) Event on the applicable Stream. When the MG detects a failure of a STUN consent refresh to renew the consent it shall send a Notify.Req with the stnconfres/confail) ObservedEvent. The MGC may then decide to continue to allow the MG to send STUN consent refreshes, or stop STUN consent refreshes or to take another action as appropriate.

#### 8.3.6.3 STUN Consent Freshness deactivation

To stop sending STUN consent requests the MGC should turn the stnconfres/contest Signal "off" by removing it from the applicable stream via a Modify.req command. On reception of the signal the MG shall stop sending STUN consent refreshes. If any of the stnconfres Package Events are active on the Stream, then the MG shall stop sending application data at the expiry of the consent period. If no stnconfres Package Signals or Events are active on the Stream then the STUN consent refresh procedures are no longer applicable to the Stream and application data may flow.

#### 8.3.6.4 STUN Consent Freshness response

In order for a MG to respond to STUN binding requests used for consent refresh the MGC shall use the "MG Act-as STUN Server" Package (clause 8.1/[ITU-T H.248.50]).

#### 8.3.6.5 Connection liveness

If the STUN consent freshness mechanism is used, no additional keepalives are needed to maintain liveness. E.g. such as those initiated through the use of the "Keep alive request" package (clause 9.2]).

Keepalives can be initiated through the use of the "Keep alive request" Package (clause 9.2/[ITU-T H.248.50]).

# 9	Keep-alive and pinhole support

The packages in this clause provide a means of opening a pinhole through a NAT and to maintain a binding with the NAT. It does not rely on ICE techniques.

## 9.1	MGC-originated STUN request package

|  | # Package name: | MGC Originated STUN Request |
| --- | --- | --- |
|  | # Package ID: | mgcostunr (0x00c4) NOTE 1 – Clause 10 of [IETF RFC 5245] makes use of this mechanism. |
|  | # Description: | The MGC may also periodically request the MG to send a STUN request in order to keep the binding with the NAT "alive". This should be done before the expiry of the keep-alive period. NOTE 2 – This package is an exception to the rule in the scope that the MG determines the address to which a Binding Request is sent. |
|  | # Version: | 1 |
|  | # Extends: | None |

### 9.1.1	Properties

None

### 9.1.2	Events

#### 9.1.2.1	STUN binding request failure

|  | # Event name: | STUN Binding Request Failure |
| --- | --- | --- |
|  | # Event ID: | fail (0x0001) |
|  | # Description: | This event is triggered if a STUN binding request has failed. |

##### 9.1.2.1.1	EventsDescriptor parameters

###### 9.1.2.1.1.1	From address

|  | # Parameter name: | From Address |
| --- | --- | --- |
|  | # Parameter ID: | fa (0x0001) |
|  | # Type: | List of String |
|  | # Optional: | No |
|  | # Possible values: | "Y" (Yes): Failure reporting is required for this list position. "N" (No): Failure Reporting is not required for this list position. |

##### 9.1.2.1.2	ObservedEventsDescriptor parameters

###### 9.1.2.1.2.1	Failure

|  | # Parameter name: | Failure |
| --- | --- | --- |
|  | # Parameter ID: | fail (0x0001) |
|  | # Description: | This parameter contains the reason for the failure. |
|  | # Type: | String |
|  | # Optional: | No |
|  | # Possible values: | If the response of STUN binding request is "time out", error code 1000 follows the <list position>. If the mapping address in the response of STUN binding request changes, error code 1001 and the new mapping address follows <list position>. The format is "<list position>:IP address ":" port. e.g., "1:202.1.2.3:1000" |
|  | # Default: | None |

### 9.1.3	Signals

#### 9.1.3.1	Send STUN request

|  | # Signal name: | Send STUN Request |
| --- | --- | --- |
|  | # Signal ID: | sstunr (0x0001) |
|  | # Description: | This signal instructs a MG to send a binding request from the local address to the remote address contained in the remote descriptor. |
|  | # Signal type: | Brief |
|  | # Duration: | NA |

##### 9.1.3.1.1	From address

|  | # Parameter name: | From Address |
| --- | --- | --- |
|  | # Parameter ID: | fa (0x0001) |
|  | # Description: | This parameter contains the addresses where the binding request should be sent from, to the corresponding remote address. |
|  | # Type: | List of String |
|  | # Optional: | No |
|  | # Possible values: | N ("No Request"): do not perform a STUN binding request from the local address. |
|  | # Default: | None |

##### 9.1.3.1.2	Retransmission time interval

|  | # Parameter name: | Retransmission Time Interval |
| --- | --- | --- |
|  | # Parameter ID: | rti (0x0002) |
|  | # Description: | This parameter contains the initial retransmission time interval for the STUN request, as RTO defined in clause 7.2.1 of [IETF RFC 5389]. |
|  | # Type: | Integer |
|  | # Optional: | Yes |
|  | # Possible values: | 1 ms to 600000 ms (10 minutes) |
|  | # Default: | 100 ms |

### 9.1.4	Statistics

None.

### 9.1.5	Error codes

None.

### 9.1.6	Procedures

The MGC should initiate a STUN Binding Request to the remote address when it suspects that the local address is behind a restricted cone NAT. This will have the effect of opening a pinhole, allowing the remote end to send packets to the local end. If the MGC initiates a STUN Binding Request to the remote address, it can keep NAT bindings active. If there are no packets sent between the local and remote address pairs being used for media for Tr seconds (where packets include media and previous keep-alives), the MG MUST generate a keep-alive on that pair. To detect STUN Binding Request failures the fail event should be set.

This procedure is distinct from the ICE continuity check procedures.

## 9.2	Keep alive request package

{Editor’s note (2013-01 meeting): there is a general question still open: the existing keep-alive package did (?) just focus on connectionless transport UDP, but missed the consideration of connection-oriented transports such as TCP or SCTP. To be checked/confirmed.

Two further aspects:

- The "CRLF keep-alive technique" (RFC 5626) may need to be supported for e.g. TCP or TLS/TCP.

- Connection Reuse in the SIP (RFC 5923)

Both items are driven by NAT-T, and both might be relevant in case of networks with H.248.TCP and/or H.248.TLS. Contributions are solicited.}

|  | Package name: | Keep Alive Request |
| --- | --- | --- |
|  | Package ID: | kar (0x00c5) |
|  | Description: | This package enables the MGC to request the MG to send a packet in order to open a pinhole through a server or to maintain a binding. This packet goes through the same way as the media flows. For example it allows the techniques as defined in clause 10 of [IETF RFC 5245]. NOTE – This package is an exception to the rule in the scope that the MG determines the address to which a Binding Request is sent. |
|  | Version: | 1 |
|  | Extends: | None |

### 9.2.1	Properties

None.

### 9.2.2	Events

None

### 9.2.3	Signals

#### 9.2.3.1	Send keepalive packet

|  | Signal name: | Send Keepalive Packet |
| --- | --- | --- |
|  | Signal ID: | skap (0x0001) |
|  | Description: | This signal instructs a MG to send a keepalive packet from the local address to the remote address contained in the remote descriptor. |
|  | Signal type: | Brief |
|  | Duration: | NA |

##### 9.2.3.1.1	From address

|  | Parameter name: | From Address |
| --- | --- | --- |
|  | Parameter ID: | fa (0x0001) |
|  | Description: | This parameter contains the addresses where the keepalive packet should be sent from, to the corresponding remote address. The MGC shall include a value in each position of list of string for each element described in the "stunb/ac" property. |
|  | Type: | List of String |
|  | Optional: | No |
|  | Possible values: | "Send" (S): send a keepalive packet from the local address. "Not Send" (N): do not send a keepalive packet from the local address. |
|  | Default: | None |

##### 9.2.3.1.2	Keep alive transmission interval

|  | Parameter name: | Keep Alive Transmission Interval |
| --- | --- | --- |
|  | Parameter ID: | ti (0x0002) |
|  | Description: | This parameter contains transmission time interval for sending the keepalive packet. The value of this parameter corresponds to the Tr timer from [IETF RFC 5245]. |
|  | Type: | Integer |
|  | Optional: | Yes |
|  | Possible values: | 15000 ms or more |
|  | Default: | 15000 ms (15 seconds), unless provisioned otherwise. |

##### 9.2.3.1.3	Keep alive packet type

|  | Parameter name: | Keep Alive Packet Type |
| --- | --- | --- |
|  | Parameter ID: | kapt (0x0003) |
|  | Description: | This parameter indicates the type of keep alive packet type. |
|  | Type: | Enumeration |
|  | Optional: | Yes |
|  | Possible values: | et (0x0000): Transport (i.e., UDP, DCCP) packet of 0-byte; rm (0x0001): RTCP packets multiplexed with RTP packets; sbi (0x0002): STUN binding indication; cn (0x0003): RTP packet with comfort noise payload; no (0x0004): Reserved for RTP packet with No-Op payload; iv (0x0005): RTP packet with incorrect version number; up (0x0006): RTP packet with unknown payload type. NOTE – Value no (0x0004) is reserved for a future payload type. |
|  | Default: | up (0x0006): RTP packet with unknown payload type |

### 9.2.4	Statistics

None.

### 9.2.5	Error codes

None.

### 9.2.6	Procedures

#### 9.2.6.1	MGC stimuli for IP bearer path keep-alive requests

The IP bearer path related keep-alive functions are part of overall NAT traversal solutions. The MGC may trigger keep-alive procedures due to

- local policy (provisioned policy, e.g. in case that all calls of an IP network domain require keep-alive support) or/and

- call-individual requests (and parameterization) by application control protocol signalling (see e.g. [b-IETF RFC 6223], [b-IETF RFC 5626])

Example: A SIP network with usage of [b-IETF RFC 6223]: the MGC could (inter alia) derive the signal parameters "keep alive transmission interval" (kar/skap/ti) and "keep alive packet type" (kar/skap/kapt) from SIP signalling.

#### 9.2.6.2	Triggering keep- alive at MG by MGC

In order to open or maintain a binding, the MGC should send the "Send Keep Alive" (skap) signal to the MG to initiate sending a keep-alive packet to the remote address. This will have the effect of opening a pinhole (or keeping a binding open) allowing the remote end to send packets to the local end. The "Keep Alive transmission interval" (ti) parameter is used in order for the MG to autonomously send a keep-alive packet when no packets have been detected for the address for time interval Tr.

Where a MGC utilizes a Keep alive Request signal with the "RTP packet with unknown payload type", it may be required to communicate this to a peer MGC via the SDP. An MGC on reception of this attribute should then set this attribute in the remote descriptor of the applicable MG Termination/Stream. It indicates to the MG that the remote end will utilize a keep-alive using an RTP packet with an unknown payload type.

# 10	Package-independent NAT-T procedures

## 10.1	Support for MG terminated STUN-based connectivity checks

### 10.1.1	Overview

STUN-based connectivity checks can be originated or terminated by the MG. Clause 8.2 defines support for MG originated STUN connectivity checks. This clause covers network scenarios where the MG is requested to terminate the STUN-based connectivity check procedure of STUN application ICE (e.g., model according Figure 2). Appendix I.1 illustrates an example use case in more detail.

NOTE – A MG which only supports termination of STUN-based connectivity check procedures  necessarily only supports ICE-lite.

### 10.1.2	Required MG support functions

The MG is required to support

- the STUN protocol [IETF RFC 5389] at the IP bearer interface of the MG (clause 10.1.3);

- the signalled credentials (for the bearer plane STUN-based connectivity checks) in H.248 (clause 10.1.4);

- the ICE conclusion process (including media latching) (clause 10.1.5)

and optionally

- reporting of latched addresses to the MGC (clause 10.1.5).

### 10.1.3	Indication of bearer type "STUN/L4/IP"

The MGC may indicate a STUN-enabled termination/stream endpoint by ICE-specific SDP in the LD/RD (see next clause).

### 10.1.4	Signalling of credentials

### 10.1.4.1	General behaviour

The motivation for credentials is described in clauses 2.5 and 7 in [IETF RFC 5245], e.g.:

Each STUN connectivity check is covered by a message authentication code (MAC) computed using a key exchanged in the signaling channel.  This MAC provides message integrity and data origin authentication, thus stopping an attacker from forging or modifying connectivity check messages.

The two ICE-specific SDP attributes "ice-pwd" and "ice-ufrag" are used for that purpose. The MGC shall signal these SDP elements to the MG.

NOTE – See also clause 7.1.2.3/[IETF RFC 5245], which specifies that STUN’s "short-term credentials" need to be used by ICE, and clause 10.1/[IETF RFC 5245], which defines this short-term credential mechanism, and which explains why username and password need to be exchanged via the signalling channel.

#### 10.1.4.2	Detailed H.248 signalling of credentials

The usage of credentials in the H.248 Local descriptor and Remote descriptor, in direction from MGC to MG (command request) and vice versa (command reply), dependent on the applied ICE protocol variant, and dependent whether the resource management of ICE credential is provided by the MGC or MG, is summarized in Table 2:

Table 2 – Usage of SDP-defined credentials in H.248 LD and RD

| No. | Resourcemanagement | ICE variant | SDP attributes (credentials) | MGC-to-MG (request) |  | MG-to-MGC (reply) |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | LD | RD | LD | RD |
| 1.1 | MG (NOTE 1) | ICE-lite | "a=ice-ufrag:" "a=ice-pwd:" | Yes Yes | No No | Yes Yes | No No |
| 1.2 |  | full ICE | "a=ice-ufrag:" "a=ice-pwd:" | Yes Yes | Yes Yes | Yes Yes | No No |
| 2.1 | MGC (NOTE 2) | ICE-lite | "a=ice-ufrag:" "a=ice-pwd:" | Yes Yes | No No | O O | No No |
| 2.2 |  | full ICE | "a=ice-ufrag:" "a=ice-pwd:" | Yes Yes | Yes Yes | O O | No No |
| NOTE 1 – SDP would be wildcarded CHOOSE in case of MG level resource management. NOTE 2 – To the optional (O) tag in the MG reply: The MGC chooses ICE ufrag and passwd values and sends them to the MG in the requested Local descriptor. If the MGC locally stores these values, then the MG is not required to reply them back to the MGC. Thus it may be optional for the MG to reply ice-ufrag and ice-pwd values to MGC. |  |  |  |  |  |  |  |

### 10.1.5	ICE conclusion process (STUN-specific media latching)

The STUN connectivity check procedure provides an explicit indication to the MG about the existence of a remote NAT device in the IP bearer path, leading to the conclusion to perform latching.

NOTE – Background: this requirement relates to the ICE conclusion process as described in clause 8/[IETF RFC 5245]. This conclusion process depends on the supported ICE variant and, in case of full-ICE, on the ICE role (controlling or controlled).

The MG shall autonomously perform the ICE conclusion process which implies such STUN-specific latching, i.e., without any explicit indication by the MGC (as in case of [ITU-T H.248.37] stimulated latching).

The MGC may subscribe for optional address reporting (according to clause 7/[ITU-T H.248.37]).

Appendix I
Example signalling scenarios for 
Package-independent NAT-T procedures

(This appendix does not form an integral part of this Recommendation)

## I.1	Example #1: ICE/STUN support by H.248 IP access gateways

### I.1.1	Network model

Figure I.1 illustrates a "half call" model, which is consistent with the network model of clause 6.1.1. An H.248 IP-IP gateway is located between access and core network level. It's a widespread model for many IP-based network access technologies.

Figure I.1 – Network model for H.248 IP access gateways 
(derived from Fig. 2 with scope on half call only)

The user equipment (UE) provides ICE-controlled STUN/TURN procedures according clause 6.5.3.

### I.1.2	Example signalling at SIP and H.248 interfaces

Figure I.1 highlights the most important phases during the establishment of an outgoing call. The communication between the UE and STUN/TURN server as well as the UE and SIP servers is out of scope of this Appendix. Only some significant SIP/SDP is indicated here.

The UE starts to gather addresses by contacting a STUN server (phase I; the UE receives the outging address of the NAT/FW (so called server reflexive address) by the STUN binding request/response cycle). After the address prioritization step (by the UE), an outgoing SIP message inclusive of an SDP offer is sent (phase II) with the purpose of advertisement of the candidate address(es). Table I.1 indicates the significant SDP for ICE:

Table I.1 – Example command encoding– (SIP) SDP Offer

| (SIP) SDP encoding | Comments |
| --- | --- |
| v=0 o=... s=... t=... m=audio <UE_A_port_audio_rtp> RTP/AVP … c=IN IP6 <UE_A_IP_addr_audio_rtp> … a=candidate:... a=ice-ufrag:<UE_A_ice_ufrag_audio> a=ice-pwd:<UE_A_ice_pwd_audio> … | Two type of ICE-related SDP information: - Address advertisement ("candidate(s)") - Authentication and message-integrity mechanisms for STUN |

The IP bearer path is routed through the MG. The subsequent STUN based connectivity checks will be therefore processed by the MG. The MGC prepares the MG corresspondingly (phase III) by creating a context and stream endpoint (SEP). The SEP requires the information for STUN authentication, see Table I.2:

Table I.2 – Example command encoding – MGC request

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MGC to MG: MEGACO/3 [11.9.19.65]:55555 Transaction = 1 { Context = $ { Add = ip/$/$/$ { Media { Stream = 1 { LocalControl { ipdc/realm = <access realm>, Mode = Inactive, ... }, Local { v=0 c=IN IP6 $ m=audio $ RTP/AVP - a=ice-ufrag:<IP_A_ice_ufrag_audio> ; NOTE 1 a=ice-pwd:<IP_A_ice_pwd_audio> ; NOTE 1 }, Remote { v=0 c=IN IP6 <UE_A_IP_addr_audio_rtp> m=audio <UE_A_port_audio_rtp> RTP/AVP - ## a=ice-ufrag:<UE_A_ice_ufrag_audio> ## a=ice-pwd:<UE_A_ice_pwd_audio> } … } | NOTE 1 – Or wildcard CHOOSE in case of MG level management of resources "ICE username fragment" and "ICE password" (see also Table 1 in clause 10.1.4.2): a=ice-ufrag: $ a=ice-pwd:$ |

A possible example MG reply is indicated in Table I.3:

Table I.3 – Example command encoding– MG reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MG to MGC: MEGACO/3 [125.125.125.111]:55555 Reply = 1 { Context = <C1> { Add = <IP_A> { Media { Stream = 1 { Local { v=0 c=IN IP6 <IP_A_IP_addr_audio> m=audio <IP_A_port_audio_rtp> RTP/AVP - a=ice-ufrag:<IP_A_ice_ufrag_audio> ; ice-pwd attribute might not be returned as MG does not perform STUN binding request authentication initially (NOTE 1) … }, Remote { v=0 c=IN IP6 <UE_A_IP_addr> m=audio <UE_A_port_audio_rtp> RTP/AVP - ## a=ice-ufrag:<UE_A_ice_ufrag_audio> ## a=ice-pwd:<UE_A_ice_pwd_audio> }, | NOTE 1 – The "a=ice-pwd" value in this SEP’s Local descriptor is used by the UE when generating STUN binding requests (clause 7.1.2.3/[IETF RFC 5245], e.g. with agent "L = UE" and with agent "R = MG"). This SEP then, when receiving such STUN binding requests (thus as STUN server) needs to use it’s own password (in the Local descriptor) to check the received STUN binding request’s validity (clause 7.2/[IETF RFC 5245] and clause 10.1.2/[IETF RFC 5389]). Therefore the "a=ice-pwd" attribute in Local is actually needed. But, if the MG is an ICE-lite endpoint, it may ignore the" a=ice-pwd" attribute in the Remote descriptor, because in this case it never generates and sends STUN binding requests itself (see also Table 1 in clause 10.1.4.2). |

The MG monitors the IP bearer path and terminates STUN connectivity checks (phase V). The learnt addresses are used for latching, i.e., the adaption of the remote destination address (phase VI).

This kind of latching (as part of the "ICE conclusion process") is called (here) STUN-specific latching, similar to H.248.37-stimulated latching. Both belong conceptionally to the overall function of "media latching" (as a kind of NAT traversal mechanism), but differ from H.248 perspective (NOTE).

NOTE – Difference:

- "H.248.37 latching": explicitly triggered latching via an H.248 signal;

- "STUN-specific latching" (i.e., MG autonomously performing the ICE conclusion process (which incorporates latching of the peer’s transport address, which is actually chosen by the controlling side)): implictly triggered latching via "ICE/STUN" specific SDP.

The MGC could enable the address reporting capability (according [ITU-T H.248.37]), which is normally not required for the progress of the call establishment process.

## I.2	Example #2: …

<placeholder>

Appendix II

ICE for TCP

(This appendix does not form an integral part of this Recommendation.)

## II.1	Introduction

The purpose of this Appendix is to highlight major differences of ICE for TCP versus the original ICE for UDP. Only aspects relevant for H.248 entities are indicated.

## II.2	MG bearer interface: protocol stack

The "ICE for TCP" protocol stack (Figure II.1) is defined in clause 3/[IETF RFC 6544]:

Figure II.1 – ICE for TCP – protocol stack (MG bearer interface)

The framing protocol (according to [b-IETF RFC 4571]) on top of TCP is mandatory. The transport level security protocols are optional. The option of using an UDP tunnel for TCP (see [IETF RFC 6544], clause 5.4 "UDP-Tunneled Candidates") is not indicated in Figure II.1.

## II.2	Bearer establishment

The STUN procedures (address gathering, connectivity checks, etc.) are executed over TCP bearer connection(s). The MG-side TCP-enabled H.248 stream endpoints are controlled according to [b-ITU-T H.248.84] or/and [b-ITU-T H.248.89]. Additional (D)TLS transport level security may require support of [b-ITU-T H.248.90] or [b-ITU-T H.248.93], dependent on how far the MG is involved in (D)TLS traffic processing.

## II.3	TCP address candidates

The UDP-candidate types [IETF RFC 5245],

candidate-types = "host" / "srflx" / "prflx" / "relay" / token

are further qualified by an extension type in case of ICE for TCP [IETF RFC 6244]:

tcp-type = "active" / "passive" / "so"

which is important concerning the direction of TCP bearer establishment (from MG perspective). Table II.1 summarizes the semantic of the TCP extension types:

Table II.1 – Type extensions for TCP-based candidates

| Type extension for candidates | Semantic |
| --- | --- |
| active | An active candidate is one for which the agent will attempt to open an outbound connection but will not receive incoming connection requests. |
| passive | A passive candidate is one for which the agent will receive incoming connection attempts but not attempt a connection. |
| simultaneous-open (S-O) | An S-O candidate is one for which the agent will attempt to open a connection simultaneously with its peer. |

This leads to the following TCP candidate type variations (Table II.2):

Table II.2 – TCP candidate options

| Main candidate types | Variations |
| --- | --- |
| host | {active¦passive¦simultaneous-open} host candidate |
| server reflexive | {active¦passive¦simultaneous-open} server reflexive candidate |
| peer reflexive | {active¦passive¦simultaneous-open} peer reflexive candidate |
| relayed | {active¦passive¦simultaneous-open} relayed candidate |
| NAT-assisted | SDP encoded as "server reflexive", but with higher priority value |
| UDP-tunneled | The "TCP(/IP) packet over UDP" tunnel leads to "ICE for UDP". SDP encoded as "relayed" or "server reflexive". |

Example SDP grammar (ICE for TCP):

a=candidate:<> <> TCP <> <> typ host tcptype active

a=candidate:<> <> TCP <> <> typ host tcptype passive

a=candidate:<> <> TCP <> <> typ host tcptype so

a=candidate:<> <> TCP <> <> typ srflx raddr <> rport <> tcptype active

a=candidate:<> <> TCP <> <> typ srflx raddr <> rport <> tcptype passive

a=candidate:<> <> TCP <> <> typ srflx raddr <> rport <> tcptype so

a=candidate:<> <> TCP <> <> typ prflx raddr <> rport <> tcptype active

a=candidate:<> <> TCP <> <> typ prflx raddr <> rport <> tcptype passive

a=candidate:<> <> TCP <> <> typ prflx raddr <> rport <> tcptype so

a=candidate:<> <> TCP <> <> typ relay raddr <> rport <> tcptype active

a=candidate:<> <> TCP <> <> typ relay raddr <> rport <> tcptype passive

a=candidate:<> <> TCP <> <> typ relay raddr <> rport <> tcptype so

Appendix III

ICE mode support by gateway-embedded ICE agents

(This appendix does not form an integral part of this Recommendation.)

## III.1	Background

The differences between ICE modes have to be extracted from [IETF RFC 5245] due to lack of an explicit description for an "ICE lite" capability set. Complementary ICE documentation provides only a very high level description, such as following expired IETF draft [b-IETF ice-lite]:

"A Lite Ice Implementation (LII) behaves much like a normal ICE

implementation, with three major differences:

o  It only gathers candidate addresses from its own interfaces.

o  It cannot be a controlling endpoint.

o  It does not generate checks but only responds to periodic checks

from other endpoints."

## III.2	Main differences between ICE-full and ICE-lite

The following Table III.1 provides an attempt to indicate the differences at a lower level, applicable for ICE implementations by H.248 entities. Whenever there is a discrepancy between the information in this Appendix and [IETF RFC 5245], the IETF RFC takes precedence. It should be noted that this Table provides some abstraction. More differences could be identified when looking at a more detailed protocol level.

Table III.1 – High-level comparison of ICE modes

|  | Capability | Full mode | Lite mode | Comment |
| --- | --- | --- | --- | --- |
| 1 | Gathering Candidates: |  |  |  |
| 1.1 | Host candidates | Yes | Yes (NOTE 1) | "no real candidate gathering (because STUN/TURN not required)" |
| 1.2 | Server reflexive candidates | Yes | No | STUN/TURN based |
| 1.3 | Relayed candidates | Yes | No | TURN based |
| 1.4 | Peer reflexive candidates | Yes | No | Possible result of connectivity checks |
| 1.5 | TCP-specific candidates: |  |  | [IETF RFC 6544] |
| 1.5.1 | NAT-assisted candidates (TCP) | Yes | No | [IETF RFC 6544] |
| 1.5.2 | UDP-tunneled candidates (TCP) | Yes | No | [IETF RFC 6544] |
| 2 | Prioritizing Candidates |  |  |  |
| 2.1 | Priority assignment | Yes | Yes |  |
| 2.2 | Robust IP address selection (V4 / V6) | Yes (NOTE 2) | No |  |
| 3 | SIP: generate SDP Offer (or SDP Answer) |  |  |  |
| 3.1 | Choosing default candidates | Yes | Yes |  |
| 3.2 | Insert SDP attribute "a=ice-lite" | N.A. | Yes |  |
| 4 | SIP: negotiation based SDP O/A |  |  |  |
| 4.1 | eliminating redundant candidates | Yes | No |  |
| 4.2 | forming check lists (when Offer received) | Yes | No |  |
| 5 | Connectivity checks |  |  |  |
| 5.1 | Initiator role of connectivity checks | Yes | No |  |
| 5.2 | Responder role of connect. checks | Yes | Yes |  |
| 5.3 | STUN role: "STUN client" | Yes | No |  |
| 5.4 | STUN role: "STUN server" | Yes | Yes |  |
| 6 | Completing ICE procedures: |  |  |  |
| 6.1 | Agent role "ICE controlling" | Yes | No (NOTE 3) |  |
| 6.2 | Agent role "ICE controlled" | Yes | Yes |  |
| 6.3 | Role conflict detection & repair | Yes | No |  |
| 6.4 | Nominating pairs (regular, agressive) | Yes | No (tbc) |  |
| 6.5 | Updating state of candidate pairs | Yes | No |  |
| 6.6 | Updating state of check lists | Yes | No |  |
| 6.7 | Freeing candidates | Yes | Yes |  |
| 7 | Media handling: |  |  |  |
| 7.1 | Unconditional start of media sent | Yes | No | [IETF RFC 5245], clause 11.1 |
| 8 | Interoperability / Extensibility: |  |  |  |
| 8.1 | Interoperate with full mode | Yes | Yes |  |
| 8.2 | Interoperate with lite mode | Yes | No | Lite-to-lite = conditional (NOTE 4) |
| 8.3 | Support of ICE extensions | Yes | No |  |
| 9 | Performance: |  |  |  |
| 9.1 | Call setup time (minimal) | Yes (NOTE 5) | No | Aggressive vs regular nomination |
| 10 | Security: |  |  |  |
| 10.1 | Security benefits (unrelated to NAT-T) | Yes | No | see Appendix A in [IETF RFC 5245] |
| 11 | IP network architecture evolution: |  |  |  |
| 11.1 | Deployment stability | Yes | No (NOTE 6) |  |
| NOTE 1 – A lite implementation doesn't gather candidates; it includes only host candidates for any media stream. NOTE 2 – See Lite (single IPv4 host address, multiple IPv6 addresses; IPv6 connectivity preferred in general (future safe)); References:[IETF RFC 5245], Appendix A "Lite and Full Implementations" and [b-IETF RFC 6724] NOTE 3 – [IETF RFC 5245], clause 5.2: "For a lite implementation, being the controlling agent means selecting a candidate pair based on the ones in the offer and answer (for IPv4, there is only ever one pair), and then generating an updated offer reflecting that selection, when needed (it is never needed for an IPv4-only host)." NOTE 4 – Lite-to-lite represents a theoretical NAT-less end-to-end path, i.e., there would be no NAT traversal required, hence also no ICE required. NOTE 5 – [IETF RFC 5245]: "A full implementation will reduce call setup times, since ICE's aggressive mode can be used." NOTE 6 – [IETF RFC 5245]: "it is often the case that a device that finds itself with a public address today will be placed in a network tomorrow where it will be behind a NAT. It is difficult to definitively know, over the lifetime of a device or product, that it will always be used on the public Internet. Full implementation provides assurance that communications will always work." |  |  |  |  |

The identified capabilities should cover the major aspects. A precise differentiation between both ICE modes would require a formal specification of the complete ICE protocol logic ("which does not exist, niether in IETF nor in other SDOs"). However, - in scope of this Recommendation -, it should be sufficient to identify ICE mode differences at the level of H.248 protocol and procedural specification.

{Editor’s note: pending living list items:

#1.1:	H.248 call-dependent procedures for each ICE capability set. Are they identical or different? Explicit text would be beneficial on that aspect.

#1.2:	The remote "ICE agent" could provide the same or a different ICE capability set. Any impact on the MGC or/and MG behaviour? Explicit documentation would be beneficial.

Contributions solicited.}

Appendix IV

ICE extensions: "Trickle ICE"

(This appendix does not form an integral part of this Recommendation.)

## II.1	Introduction

Purpose of this Appendix is to illustrate signalling flows with trickle ICE in order to highlight principle impact on H.248 gateway control. ### FIXTHIS ###.

Bibliography

[b-ITU-T H.248.84]	Recommendation ITU-T H.248.84 (07/2012), Gateway control protocol: NAT-traversal for peer-to-peer services.

[b-ITU-T H.248.89]	Recommendation ITU-T H.248.89 (10/2014), Gateway control protocol: TCP support packages.

[b-ITU-T H.248.90]	Recommendation ITU-T H.248.90 (10/2014), Gateway control protocol: H.248 packages for control of transport security using TLS.

[b-ITU-T H.248.93]	Recommendation ITU-T H.248.93 (10/2014), Gateway control protocol: H.248 packages for control of transport security using DTLS.

[b-IETF RFC 4571]	IETF RFC 4571 (07/2006), Framing Real-time Transport Protocol (RTP) and RTP Control Protocol (RTCP) Packets over Connection-Oriented Transport.

[b-IETF RFC 5626]	IETF RFC 5626 (10/2009), Managing Client-Initiated Connections in the Session Initiation Protocol (SIP).

NOTE 1 – Two keep-alive techniques: CRLF keep-alive technique for TCP and SCTP, and STUN keep-alive technique for UDP.

[b-IETF RFC 6223]	IETF RFC 6223 (04/2011), Indication of Support for Keep-Alive.

{Editor’s note (2013-01 meeting): possible further "keep alive" reference:
"Indication of support for reverse keep-alive" 
(draft-holmberg-sipcore-rkeep). Contributions solicited.}

[b-IETF RFC 6724]	IETF RFC 6724 (09/2012), Default Address Selection for Internet Protocol Version 6 (IPv6).

[b-IETF ice-lite]	IETF draft-rescorla-mmusic-ice-lite (02/2007), Implementing Interactive Connectivity Establishment (ICE) in Lite Mode.
URL: http://tools.ietf.org/html/draft-rescorla-mmusic-ice-lite-00

[b-IETF trickle-ice]	IETF draft-ietf-mmusic-trickle-ice (##/201#), Trickle ICE: Incremental Provisioning of Candidates for the Interactive Connectivity Establishment (ICE) Protocol.

[b-IETF trickle-sip]	IETF draft-ietf-mmusic-trickle-ice-sip (##/201#), A Session Initiation Protocol (SIP) usage for Trickle ICE.

[b-IETF ice-dualstack]	IETF draft-martinsen-mmusic-ice-dualstack-fairness (##/201#), ICE IPv4/IPv6 Dual Stack Fairness.

Appendix L
Living List related to enhancements and clarifications

(This appendix will be again removed at the stage of consent/approval)

|  | - "" - "" """" "" |  |
| --- | --- | --- |

## L.3	Item #3: Package-independent NAT-T procedures

| Motivation: | It turns out that network use cases may be identified with H.248 MGC/MG involvement in ICE/STUN/TURN related NAT-T, but which do not necessarily required H.248.50 defined protocol elements. Just H.248 LD/RD-embedded SDP information element might be sufficient for controlling the NAT-T service. This aspect is similar to H.248.84, which provides NAT-T service support with and without H.248.84 defined packages. #3.1: Correspondent use cases are conditional on particular behaviour, role assignment and/or network configurations, and should be described. |  |
| --- | --- | --- |
| Impacted H.248.50 text: | - Clause 6.1? - New clause on package-independent NAT-T procedures? |  |

## L.4	Item #4: H.248.50 References to IETF ICE documents

The core RFC on ICE [IETF RFC 5245] was augmented and updated by some extensions, which were not referenced by H.248.50 (note: work on H.248.50 was already technically stable in 2007). This clause provides a reference check.

### ""

|  | """" |
| --- | --- |
|  | "" |

### L.4.2	Item #4.2: IETF RFC 6724 "IPv6 address selection"

Missing H.248.50 reference:

| [IETF RFC 6724] IETF RFC 6724 (09/2012), Default Address Selection for Internet Protocol Version 6 (IPv6). |  |
| --- | --- |
| Motivation: | - ICE recommends to use the rules defined in [RFC 3484] (= obsoleted by [RFC 6724]) as part of the prioritization formula for IPv6 candidates |
| H.248.50 impact? | - No impact |

## L.5	Item #5: Recent ICE updates by IETF

{Editor’s note (2012-09 meeting): there are a number of IETF activities for improving ICE in different ways (see also email "[MMUSIC] Poll for path on ICE extensions and updates/corrections", http://www.ietf.org/mail-archive/web/mmusic/current/msg09558.html ):
- draft-keranen-mmusic-ice-address-selection-01
- draft-petithuguenin-mmusic-ice-attributes-level-03
- draft-wing-mmusic-ice-mobility-01
- draft-elwell-mmusic-ice-updated-offer-02
There is, in addition, a long discussion in the RTCWEB mailing list about "trickle ICE", and a couple of other ICE-related drafts and discussions in RTCWEB.

Most likely, we can categorize these documents in two big classes:
a) Documents that identify underspecified areas or errors in the spec, which are often driven by implementation experience
b) Documents that update or extend ICE with new functionality, such as "tricke ICE" (see e.g. the e-mail thread at http://www.ietf.org/mail-archive/web/rtcweb/current/msg05121.html) .
Such documents may involve normative updates to the ICE specification or they may simply be ICE extensions.".

Conclusions: there are basically impacts on H.248.50 expected. 
Contributions are solicited.}

### L.5.0	Overview (Status 11/2014)

IETF WG MMUSIC (= technology owner of ICE):

- draft-ietf-mmusic-trickle-ice & draft-ietf-mmusic-trickle-ice-sip
"Trickle ICE: Incremental Provisioning of Candidates for the Interactive Connectivity Establishment (ICE) Protocol"

- replaced by: draft-martinsen-mmusic-ice-dualstack-fairness
"ICE IPv4/IPv6 Dual Stack Fairness"""

- draft-petithuguenin-mmusic-ice-attributes-level-04 (2012-10-08) 
"Media level ice-options SDP attribute"

- draft-keranen-mmusic-ice-address-selection-01 (2012-07-16) 
"Update on Candidate Address Selection for Interactive Connectivity Establishment (ICE)"

- draft-wing-mmusic-ice-mobility-07 (2014-06) 
"Mobility with ICE (MICE)"

- others?

### L.5.1	Item #5.1: IETF document "Update on Candidate Address Selection"

| Reference: | draft-keranen-mmusic-ice-address-selection-01 (2012-07-16) "Update on Candidate Address Selection for Interactive Connectivity Establishment (ICE)" URL: http://tools.ietf.org/id/draft-keranen-mmusic-ice-address-selection-01.txt |
| --- | --- |
| Motivation: | - update of rules for candidate address selection - primarily driven by IPv6 aspects related to ICE HOST CANDIDATES (differences between IPv4 and IPv6 based local interfaces) - # |
| H.248.50 impact? | - No impact (because rule selection algorithm is "ICE internal" functionality without any impact on "H.248 MG reporting of successful candidate/transport pairs") TO BE CHECKED |

### L.5.2	Item #5.2: IETF document "Trickle ICE"

| Reference: | draft-ietf-mmusic-trickle-ice & draft-ietf-mmusic-trickle-ice-sip "Trickle ICE: Incremental Provisioning of Candidates for the Interactive Connectivity Establishment (ICE) Protocol" |
| --- | --- |
| Motivation: | - Issue: process of candidate gathering, prioritization and final selection may significantly delay communication establishment ("relatively lengthy session establishment times and degraded user experience"). Such delays do negatively impact primarily realtime conversation services. - Trickle ICE: alternative mode by incremental exchange of candidates already early during session establishment process - trickle ICE = proven technology, included in specifications such as XMPP Jingle - ICE agent: additional capability "candidate harvester" |
| H.248.50 impact? | - Yes: trickle ICE is not backward compatible to standard ICE, hence both ICE agents needs firstly to check "trickle ICE" support or not - the "trickle ICE" capability declaration and negotiation, as well as fallback process is delegated to the IP signalling plane (such as SDP Offer/Answer), thus subject of the H.248 interface in case of ICE involved MGs - concrete H.248.50 changes: to be done |
| update by AVD-4617: | The capability of trickle ICE is part of the overall WEBRTC solution, hence relevant for H.248 WEBRTC gateways (see H.248.WEBRTC). Trickle ICE significantly impacts the existing H.248.50 specification because - the information about vanilla ICE is distributed across multiple places in H.248.50 and - trickle ICE is inherently not backward compatible to vanilla ICE. Thus, an urgent clarification and update of H.248.50 would be beneficial. Such work implies at least following three tasks: T1: Study of signalling scenarios (for trickle ICE only and combined trickle/vanilla ICE) in order to separate "trickle ICE" information at call control signalling level from the remaining part which finally will be visible at the H.248 interface at all. T2: Impact of trickle ICE on all existing H.248.50 packages ("still unclear if at all, but obvious that some package procedure would benefit from more detailled vanilla ICE related information")? T3: Impact of trickle ICE on package-less procedures (caluse 10)? This contribution does not address any of these tasks, the questions are still open. With regards to H.248.WEBRTC gateways it might be that the first H.248 WEBRTC gateway profile versions survive with the model according Appendix I/H.248.50. Taking the high-level assumptions that Assumption 1: trickle ICE will affect primarily the address gathering phase (= step I in Fig. I.1) (to be confirmed) and Assumption 2: subsequent STUN connectivity check phase(s) (= step IV in Fig. I.1) might be independent of the parallel and still ongoing call level trickle ICE signalling (to be confirmed), then the impact might be limited on H.248 packages mgastuns and ostuncc ("which are both subject of first H.248 WEBRTC gateway profile versions"). And there seems to be now impact on the mgastuns package, but impact on the ostuncc package is not obvious due to the "candidate information" used. |

|  | "" |
| --- | --- |
|  | - "" |

### L.5.4	Item #5.4: IETF document "Media level ice-options SDP attribute"

| Reference: | draft-petithuguenin-mmusic-ice-attributes-level-04 (2012-10-08) "Media level ice-options SDP attribute" URL: http://tools.ietf.org/id/draft-petithuguenin-mmusic-ice-attributes-level-04.txt |
| --- | --- |
| Motivation: | - Disaggregated media: refers to the ability for a user to create a multimedia session combining different media streams, coming from different devices under his or her control, so that they are treated by the far end of the session as a single media session - Redefinition of ICE SDP attributes: - ice-options (as a session-level and media-level attribute) special attention to value: "a=ice-options:rtp+ecn" - ice-lite: not redefined - ice-mismatch - ## |
| H.248.50 impact? | - # TO BE CHECKED |

### L.5.5	Item #5.5: IETF document "Mobility with ICE"

| Reference: | draft-wing-mmusic-ice-mobility-02 (2012-10-06) "Mobility with ICE (MICE)" URL: http://tools.ietf.org/id/draft-wing-mmusic-ice-mobility-02.txt |
| --- | --- |
| Motivation: | - Issue: mobility may lead to location changes which lead to address changes - ICE does not expect either the local host or the remote host to change their IP addresses. If … then "ICE restart" … (but takes time, service interruption) - Two mobility mechanisms: - Both endpoints ICE capable - Not …, with TURN server - STUN extensions: - new STUN Attribute MOBILITY-TICKET - new STUN Error Response Code - # |
| H.248.50 impact? | - # TO BE CHECKED |

________________

“Addresses obtained from local network interfaces, called host candidates, are recommended as high-priority ones to be tested first since if they work, they provide usually the best path between the two hosts.“

Topic = http://tools.ietf.org/html/draft-loreto-splices-disaggregated-media-02 (see clause 3.2.1. Megaco issues)