| Rapporteur Meeting of Questions 2, 3, 5 and 21/16 |  |  |  | AVD-4678 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Guilin, 8 - 12 June 2015 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Editor H.248.PAURES |  |  |  |
| Title: | H.248.PAURES “Gateway control protocol: Support of remote media pause and resume” (New): Input draft |  |  |  |
| Purpose: | Proposal |  |  |  |

1.	Introduction

This document contains the Editor's input draft for H.248.PAURES “Gateway control protocol: Support of remote media pause and resume” (New).

2.	Discussion

It is a copy of TD 231r1/WP1 from the Geneva, 9-20 February 2015 meeting. There are no changes.

3.	Proposal

It is proposed to accept this document as the baseline text for the meeting.

Document History

| Document History |  |
| --- | --- |
| Issue | Notes |
| H.248.PAURES Ed. 0.1 | Initial baseline text for H.248.PAURES. Based on AVD-4641 from the Seoul, November 2014 ITU-T SG16 meeting. |
| H.248.PAURES Ed. 0.2 | Input draft to the Geneva, 9-20 February 2015 meeting. TD 231/WP1: http://www.itu.int/md/T13-SG16-150209-TD-WP1-0231/en |
| H.248.PAURES Ed. 0.3 | Output draft from the Geneva, 9-20 February 2015 meeting. Changes due to: C.731 |
| H.248.PAURES Ed. 0.4 | Input draft for the Guilin, 8 - 12 June 2015 meeting. |

CONTENTS

Page

1 Scope 4 2 References 4 3 Definitions 5 3.1 Terms defined elsewhere 5 3.2 Terms defined in this Recommendation 5 4 Abbreviations and acronyms 5 5 Conventions 5 6 Relation to other Recommendations in the ITU-T H.248 sub-series 6 7 Considerations 6 7.1 Real-time nature 6 7.2 Message direction 7 7.3 Apply to individual sources 7 7.4 Consensus 7 7.5 Acknowledgements 7 7.6 Retransmitting Requests 8 7.7 Sequence Numbering 8 8 Remote pause and resume package 8 8.1 Properties 8 8.1.1 Autonomous Response 8 8.2 Events 9 8.2.1 RTP Pause State 9 8.2.2 Detect Pause / Resume Request 10 8.2.3 Detect Pause / Resume Result 11 8.3 Signals 13 8.3.1 Local Pause 13 8.3.2 Local Resume 14 8.3.3 Refuse 14 8.3.4 Remote Pause 15 8.3.5 Remote Resume 16 8.4 Statistics 17 8.5 Error Codes 17 8.6 Procedures 17 8.6.1 General - SDP parameter setting 17 8.6.2 General – Event and Signal Scope 18 8.6.3 MG autonomous handling of remote requests 18 8.6.4 MGC reception of pause and resume requests 18 8.6.5 MGC initiation of remote pause and resume requests 19 8.6.6 MGC initiation of local pause and resume 19 8.6.7 MG initiation of remote pause and resume 20 8.6.8 Error handling 20 9 Examples 21 Annex A <Annex Title - placeholder> 22 Appendix I <Appendix Title - placeholder> 22 Bibliography (placeholder) 22

List of Tables

Page

No table of figures entries found.

List of Figures

Page

No table of figures entries found.

Draft new Recommendation H.248.xxx (ex ITU-T H.248.PAURES)

Gateway control protocol: Support of remote media pause and resume

Summary

This Recommendation allows the RTP Stream Pause and Resume functionality as defined by [IETF STREAM PAUSE] to be supported on H.248 controlled gateways.

# 1	Scope

This Recommendation describes the support of RTP Stream Pause and Resume functionality as defined by [IETF STREAM PAUSE] on H.248 controlled gateways. It describes the mapping of call control SDP related to pause/resume to H.248 SDP and vice versa. It also contains a package that allows several different implementation types to be supported. This allows the Media Gateway (MG) to operate in an autonomous manner by receiving, making pause/resume decisions and sending RTCP level pause/resume messages with minimal MGC involvement. A Media Gateway Controller (MGC) controlled configuration is also supported whereby the reception of pause/request requests/responses is indicated to the MGC for it to make decisions and determine a response. It also allows the MGC to initiate pause/resume requests.

It also contains considerations related to the support of RTP Pause/Resume in H.248 controlled gateways.

The Recommendation also details the relationship to other existing H.248.x Sub-series Recommendations where pause/resume functions are supported.

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (2013), Gateway control protocol: Version 3.

[ITU-T H.248.9]	Recommendation ITU-T H.248.1 (12/09), Gateway control protocol: Advanced media server packages.

[ITU-T H.248.66]	Draft Recommendation ITU-T H.248.66 (xxxx), Gateway control protocol: Packages for RTSP and H.248 interworking.

[ITU-T H.248.STGROUP]	Recommendation ITU-T H.248.STGROUP (xxxx), Gateway control protocol: H.248 Stream grouping and aggregation.

[IETF RFC 3264]	IETF RFC 3264 (2002), An Offer/Answer Model with Session Description Protocol (SDP).

[IETF RFC 3550]	IETF RFC 3550 (2003), RTP: A Transport Protocol for Real-Time Applications.

[IETF RFC 5104]	IETF RFC 5104 (2008), Codec Control Messages in the RTP Audio-Visual Profile with Feedback (AVPF).

[IETF RFC 5576]	IETF RFC 5576 (2009), Source-Specific Media Attributes in the Session Description Protocol (SDP).

[IETF BUNDLE]	draft-ietf-mmusic-sdp-bundle-negotiation-16 (xxxx), Negotiating Media Multiplexing Using the Session Description Protocol (SDP).

[IETF STREAM PAUSE]	IETF draft-ietf-avtext-rtp-stream-pause-05 (xxxx), RTP Stream Pause and Resume.

# 3	Definitions

## 3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	<Term 1> [Reference]: <optional quoted definition>

3.1.2	<Term 2> [Reference]: <optional quoted definition>

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	<Term 3>: <definition>

{Editor’s note: A definition of RTP stream should be added.}

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| CCM | Codec Control Message |
| --- | --- |
| DTMF | Dual Tone Multi Frequency |
| MG | Media Gateway |
| MGC | Media Gateway Controller |
| RTCP | Real Time Control Protocol |
| RTP | Real Time Protocol |
| SDP | Session Description Protocol |
| SSRC | Synchronisation Source Identifier |

# 5	Conventions

Elements of the ITU-T H.248 protocol model, e.g., Context, Termination, Stream, Event are represented using the first letter capitalized. Property, Event, Signal and Parameter identities are given in italics.

The suffix ".req" added to an ITU-T H.248 command name stands for a command request, while the suffix ".rep" stands for a command reply. For example "Notify.req" represents a Notify Request.

Capitalised names PAUSE, RESUME, REFUSED, PAUSED refer to Codec Control (CCM) messages defined in [IETF STREAM PAUSE].

# 6	Relation to other Recommendations in the ITU-T H.248 sub-series

Gateways utilising H.248 currently have several methods to pause the transmission of media.

[ITU-T H.248.1]: Contains the Local Control Descriptor ‘Streammode’ property that allows the MGC to effectively stop sending a RTP media stream (i.e. change from send/receive to receive only) whilst still maintaining an RTCP control flow. To resume the flow of media the MGC can change the Streammode back to send/receive. However this is a local MG action rather than a request to a remote endpoint. The action is also triggered from a call control level stimulus.

[ITU-T H.248.1]: The H.248 connection model (including the TopologyDescriptor) allows Terminations/Streams to be isolated from other Terminations/Streams in the Context. This allows a MGC to isolate a media stream from other input streams effectively stopping the flow of media. The MGC may then move the Termination/Stream back into a Context to resume media flow.

[ITU-T H.248.9]: Contains the “Play Collect” package that enables a user to request (via a DTMF digit) that the playout of media associated with a signal be paused. It also enables the user to resume the media flow via a DTMF digit.

[ITU-T H.248.66]: Contains the “Signal pause package” which is a generic package that allows the MGC to request a pause in the playout of media and later resume media flow. It is suited to scenarios where media playout has been initiated through the use of a H.248 Signals rather than real time media flows. It again is a local MG action rather than a request to a remote endpoint and the action is also triggered from a call control level stimulus.

{Editor’s note: ITU-T H.248.88 should possibly be considered here as it relates to RTCP handling. The RTP pause / resume mechanism utilises RTCP so there may be a relation.}

None of the above methods match the functionality supported by [IETF STREAM PAUSE]. [ITU-T H.248.1] and [ITU-T H.248.66] do not support user originated requests being terminated at the MG and [ITU-T H.248.9] and [ITU-T H.248.66] do not allow the pause of media streams not related to the playout of signals.

Pause/resume relates to real time media flow. Therefore the resumption of media flow is based on the media at the time of the resume request rather than at the pause point. [ITU-T H.248.66] assumes that the playout occurs from the pause point unless another point is indicated in the resumption request.

# 7	Considerations

Clause 3/[IETF STREAM PAUSE] describes the main use cases for RTP pause and resume. It lists a number of RTP topologies. A H.248 gateway may act as a media sender, media receiver and/or mixer.  As such the design criteria listed in clause 4/[IETF STREAM PAUSE] should be considered when implementing RTP pause/resume on gateways.

{Editor’s note: Further details regarding how the SSRC is determined should be added. For example H.248.71 Source Description package, Call Control Signalling or via the rempr package are possibilities.}

## 7.1	Real-time nature

Given the real time aspects described in clause 4.1/[IETF STREAM PAUSE] a gateway should ensure prompt handling of resume requests, i.e. a MG and optionally MGC should process and act upon any requests to resume a RTP stream in an expedited manner.

## 7.2	Message direction

A gateway may initiate a local pause or resume an RTP stream, request a remote RTP sender to pause or resume an RTP stream or respond to a request from a remote RTP receiver to pause or resume and RTP stream. Clause 8 provides functionality for an MG to act autonomously or for it to co-ordinate with the MGC to implement RTP pause/resume. Furthermore it allows the specification of which pause CCM messages are used.

## 7.3	Apply to individual sources

Clause 4.3/[IETF STREAM PAUSE] indicates that the pause CCM messages relate to a single RTP Stream, i.e. relates to a single synchronization source identifier (SSRC). This usually relates to a single H.248 Stream. An RTP session may have multiple RTP streams A single SDP m=line typically describes an RTP session. However where media multiplexing is used (e.g. as described by [IETF BUNDLE]) multiple SDP m=lines can describe an RTP session. Thus a H.248 Stream using a SDP m=line may describe one or more RTP streams. [IETF RFC 5576] provides attributes for source specific parameters.

This has implications for Signals and Events related to pause and resume on H.248 streams. In order for any Signals and Events to be correctly scoped where a single SDP m=line describes multiple sources the Signal or Event must be able to reference the source. Clause 8 allows the SSRC to be associated with pause / resume related Signals and Events. The MGC shall include the ssrc parameter in rempr Package Signals when there are multiple SSRCs in a H.248 Stream to uniquely identify the RTP Stream instance.

Alternatively the MGC may consider alternate means (such as through stream grouping [ITU-T H.248.STGROUP]) to ensure that there is a single RTP stream per H.248 stream.

{Editor’s note: The scope of the CCM messages needs to be clarified. E.g. do they relate to a particular payload type.}

## 7.4	Consensus

Clause 4.4/[IETF STREAM PAUSE] indicates that there must be consensus between RTP receivers before an RTP sender can pause a RTP stream. This requires that all RTP receivers be aware of the current RTP stream state (i.e. paused or active) and that all receivers be aware of the pause request. As described in clause 5.2/[IETF STREAM PAUSE] the RTP sender should delay the pausing of a stream for a wait period. As the gateway acts as an RTP sender it should ensure that pausing meets these requirements. If MG autonomous response is indicated, the MG shall be responsible for maintaining the wait period. If MGC controlled mode is indicated, the MGC shall be responsible for maintaining the wait period.

If a new RTP receiver that supports pause messages is added whilst an RTP stream is in the paused state, the gateway shall send it a PAUSED indication as per clause 5.3/[IETF STREAM PAUSE]. The MG is responsible for sending the PAUSED indication. In the case of MGC controlled mode the MG shall only send a PAUSED indication if the MGC had previously sent a “Local Pause” (rempr/lpause) Signal for the particular RTP stream.

## 7.5	Acknowledgements

As described in clause 4.5/[IETF STREAM PAUSE] the RTP and RTCP do not guarantee reliable data transmission, meaning that the PAUSE, RESUME, PAUSED and REFUSED messages may be lost. H.248 MGCs and MGs should ensure that they follow the procedures in [IETF STREAM PAUSE] with respect to sending PAUSED and REFUSED and detecting paused and resumed states. Clause 8 provides Events for detecting when a RTP media stream has been paused or resumed to allow co-ordination between MGC and MG.

## 7.6	Retransmitting Requests

As per clause 4.6/[IETF STREAM PAUSE], PAUSE and RESUME requests sent from a H.248 gateway to an RTP stream sender may result in no response (either the receipt of a PAUSED or REFUSED, media action (pause/resume)) being detected from the sender. The gateway may assume that the PAUSE or RESUME was lost and thus may need to retransmit the message. Depending on the configuration the MGC or MG is responsible for the retransmission once the timeout period is met. If MG autonomous response is indicated, the MG shall be responsible for retransmitting the messages. If MGC controlled mode is indicated, the MGC shall be responsible for retransmitting the message (i.e. by sending a new signal to the MG indicating pause or resume).

## 7.7	Sequence Numbering

Clause 4.7/[IETF STREAM PAUSE] indicates that PAUSE request messages have a sequence number associated with them. This number is then referenced by the other pause CCM messages. As the pause identity is updated for each PAUSE request the gateway needs to manage the identities. If MG autonomous mode is indicated, then the MG shall be responsible for assigning the identities. If MGC controlled mode is indicated, then the MGC shall be responsible for assigning the identities.

# 8	Remote pause and resume package

|  | Package name: | Remote Pause and Resume |
| --- | --- | --- |
|  | Package ID: | rempr (0x????) |
|  | Description: | This package allows a MGC to request that the MG issue a request to a remote endpoint to pause (and subsequently resume) the transmission of an RTP media stream. |
|  | Version: | 1 |
|  | Extends: | None |

## 8.1	Properties

### 8.1.1	Autonomous Response

|  | Property name: | Autonomous Response |
| --- | --- | --- |
|  | Property ID: | ar (0x0001) |
|  | Description: | The property indicates whether the MG shall provide an autonomous response to PAUSE and RESUME requests or whether it should notify the MGC and await confirmation from the MGC on what action to take. |
|  | Type: | Boolean |
|  | Possible values: | On MG autonomous: The MG responds autonomously Off MGC controlled: The MG notifies the MGC and awaits a response |
|  | Default: | On (unless provisioned otherwise) |
|  | Defined in: | LocalControl or TerminationState |
|  | Characteristics: | Read/Write |

## 8.2	Events

### 8.2.1	RTP Pause State

|  | Event name: | RTP Pause State |
| --- | --- | --- |
|  | Event ID: | rtpps (0x0001) |
|  | Description: | This event allows the MG to indicate when the state associated with an RTP Pause / Resume changes. |

#### 8.2.1.1	EventsDescriptor parameters

##### 8.2.1.1.1	State

|  | Parameter name: | State |
| --- | --- | --- |
|  | Parameter ID: | state (0x0001) |
|  | Description: | This parameter indicates which pause states the MGC wishes to be notified about. |
|  | Type: | Enumeration |
|  | Optional: | Yes |
|  | Possible values: | paused The MG has paused media flow as a result of the receipt PAUSE request from a remote endpoint. resumed The MG has resumed media flow as a result of the receipt of a RESUME request from a remote endpoint localPause The MG has paused media flow based on a local decision. localResume The MG has resumed media flow based on a local decision. |
|  | Default: | All (paused, resumed, localpause, localresume) |

##### 8.2.1.1.2	RTP SSRC

|  | Parameter name: | ssrc |
| --- | --- | --- |
|  | Parameter ID: | ssrc (0x0002) |
|  | Description: | This parameter indicates on which SSRCs of the H.248 Stream the Event is detected. {Editor's note (2015-02): how does the MGC know this information. Possible solution see E.N. in clause 8.3.1.1.2.} |
|  | Type: | Sub-list of Integer (32 bit) |
|  | Optional: | Yes |
|  | Possible values: | As per clause 3/[IETF RFC 3550]. |
|  | Default: | The event is detected against all SSRCs that support pause/resume on the H.248 Stream. |

#### 8.2.1.2	ObservedEventsDescriptor parameters

##### 8.2.1.2.1	Observed State

|  | Parameter name: | Observed State |
| --- | --- | --- |
|  | Parameter ID: | obstate (0x0001) |
|  | Description: | This parameter indicates the detected pause state. It is reported whenever there is a transition between the states. |
|  | Type: | Enumeration |
|  | Optional: | No |
|  | Possible values: | As per “states” clause 8.2.1.1.1 |
|  | Default: | None |

##### 8.2.1.2.2	RTP SSRC

|  | Parameter name: | ssrc |
| --- | --- | --- |
|  | Parameter ID: | ssrc (0x0002) |
|  | Description: | This parameter indicates on which SSRC of the H.248 Stream the Event has been detected. |
|  | Type: | Integer (32 bit) |
|  | Optional: | No |
|  | Possible values: | As per clause 3/[IETF RFC 3550]. |
|  | Default: | None. |

### 8.2.2	Detect Pause / Resume Request

|  | Event name: | Detect Pause Resume Request |
| --- | --- | --- |
|  | Event ID: | dprreq (0x0002) |
|  | Description: | This event allows the MG to indicate when a PAUSE or RESUME request has been detected. |

#### 8.2.2.1	EventsDescriptor parameters

##### 8.2.2.1.1	RTP SSRC

|  | Parameter name: | ssrc |
| --- | --- | --- |
|  | Parameter ID: | ssrc (0x0001) |
|  | Description: | This parameter indicates on which SSRCs of the H.248 Stream the Event is detected. {Editor's note (2015-02): how does the MGC know this information. Possible solution see E.N. in clause 8.3.1.1.2.} |
|  | Type: | Sub-list of Integer (32 bit) |
|  | Optional: | Yes |
|  | Possible values: | As per clause 3/[IETF RFC 3550]. |
|  | Default: | The event is detected against all SSRCs that support pause/resume on the H.248 Stream. |

#### 8.2.2.2	ObservedEventsDescriptor parameters

##### 8.2.2.2.1	Pause Identity

|  | Parameter name: | Pause Identity |
| --- | --- | --- |
|  | Parameter ID: | pauseid (0x0001) |
|  | Description: | This parameter indicates the pauseID contained the detected request. |
|  | Type: | Integer |
|  | Optional: | No |
|  | Possible values: | 0 - 65535 |
|  | Default: | None |

##### 8.2.2.2.2	Request Type

|  | Parameter name: | Request Type |
| --- | --- | --- |
|  | Parameter ID: | reqt (0x0002) |
|  | Description: | This parameter indicates whether a PAUSE or a RESUME is requested for the RTP Stream. |
|  | Type: | Enumeration |
|  | Optional: | No |
|  | Possible values: | “PAUSE” A PAUSE has been requested. “RESUME” A RESUME has been requested. |
|  | Default: | None |

##### 8.2.2.2.3	RTP SSRC

|  | Parameter name: | ssrc |
| --- | --- | --- |
|  | Parameter ID: | ssrc (0x0003) |
|  | Description: | This parameter indicates on which SSRC of the H.248 Stream the Event has been detected. |
|  | Type: | Sub-list of Integer (32 bit) |
|  | Optional: | No |
|  | Possible values: | As per clause 3/[IETF RFC 3550]. |
|  | Default: | None. |

### 8.2.3	Detect Pause / Resume Result

|  | Event name: | Detect Pause Resume Result |
| --- | --- | --- |
|  | Event ID: | dprres (0x0003) |
|  | Description: | This event allows the MG to detect and indicate whether the remote RTP sender has: sent a PAUSED indication, a REFUSE indication or resumed RTP transmission in response to a request initiated by the MGC. It also detects a failure of the RTP sender to take action on a PAUSE or RESUME request. |

#### 8.2.3.1	EventsDescriptor parameters

##### 8.2.3.1.1	RTP SSRC

|  | Parameter name: | ssrc |
| --- | --- | --- |
|  | Parameter ID: | ssrc (0x0001) |
|  | Description: | This parameter indicates on which SSRCs of the H.248 Stream the Event is detected. |
|  | Type: | Sub-list of Integer (32 bit) |
|  | Optional: | Yes |
|  | Possible values: | As per clause 3/[IETF RFC 3550]. |
|  | Default: | The event is detected against all SSRCs that support pause/resume on the H.248 Stream. |

#### 8.2.3.2	ObservedEventsDescriptor parameters

##### 8.2.3.2.1	Pause Identity

|  | Parameter name: | Pause Identity |
| --- | --- | --- |
|  | Parameter ID: | pauseid (0x0001) |
|  | Description: | This parameter indicates the pauseID contained in the detected PAUSED or REFUSED indication. It may also contain the pauseID related to a failed PAUSE or RESUME request. |
|  | Type: | Integer |
|  | Optional: | No |
|  | Possible values: | 0 - 65535 |
|  | Default: | None |

##### 8.2.3.2.2	Response Type

|  | Parameter name: | Response Type |
| --- | --- | --- |
|  | Parameter ID: | rest (0x0002) |
|  | Description: | This parameter indicates whether a PAUSED or a REFUSED indication has been received. It also indicates whether a RESUME request as resulted in the flow of RTP packets from the RTP sender. |
|  | Type: | Enumeration |
|  | Optional: | No |
|  | Possible values: | “paused” A PAUSED indication has been received. “refused” A REFUSED indication has been received. “resumed” The MG has detected a resumed RTP flow. “failed” No response from the RTP sender has been detected |
|  | Default: | None |

##### 8.2.2.2.3	RTP SSRC

|  | Parameter name: | ssrc |
| --- | --- | --- |
|  | Parameter ID: | ssrc (0x0003) |
|  | Description: | This parameter indicates on which SSRC of the H.248 Stream the Event has been detected. |
|  | Type: | Sub-list of Integer (32 bit) |
|  | Optional: | No |
|  | Possible values: | As per clause 3/[IETF RFC 3550]. |
|  | Default: | None. |

## 8.3	Signals

### 8.3.1	Local Pause

|  | Signal name: | Local Pause |
| --- | --- | --- |
|  | Signal ID: | lpause (0x0001) |
|  | Description: | This signal indicates that the MG shall pause the RTP stream and send a PAUSED indication to the remote RTP receiver(s). |
|  | Signal Type: | Brief |
|  | Duration: | None |

#### 8.3.1.1	Additional parameters

##### 8.3.1.1.1	Pause Identity

|  | Parameter name: | Pause Identity |
| --- | --- | --- |
|  | Parameter ID: | pauseid (0x0001) |
|  | Description: | This parameter indicates the pauseID of the PAUSE to which the PAUSED indication relates. |
|  | Type: | Integer |
|  | Optional: | No |
|  | Possible values: | 0 - 65535 |
|  | Default: | None |

##### 8.3.1.1.2	RTP SSRC

|  | Parameter name: | ssrc |
| --- | --- | --- |
|  | Parameter ID: | ssrc (0x0002) |
|  | Description: | This parameter indicates to which SSRC the PAUSE relates. {Editor's note (2015-02): there are basically three sources of information at MGC level with regards to SSRC values: a) as detect and reported by the MG via events rtpps/ssrc or dpreq/ssrc; b) via SDP attributes in SIP signalling (tbc); or/and c) via H.248.71 "statistic" rtcpdes/rssrc. It is supposed that method (a) is used in context of this package.} |
|  | Type: | Integer (32 bit) |
|  | Optional: | Yes |
|  | Possible values: | As per clause 3/[IETF RFC 3550]. |
|  | Default: | None. |

### 8.3.2	Local Resume

|  | Signal name: | Local Resume |
| --- | --- | --- |
|  | Signal ID: | lresume (0x0002) |
|  | Description: | This signal indicates that the MG shall resume transmission of the RTP stream. |
|  | Signal Type: | Brief |
|  | Duration: | None |

#### 8.3.2.1	Additional parameters

##### 8.3.2.1.1	Pause Identity

|  | Parameter name: | Pause Identity |
| --- | --- | --- |
|  | Parameter ID: | pauseid (0x0001) |
|  | Description: | This parameter indicates the pauseID indicating the PAUSE to which the RESUME relates. |
|  | Type: | Integer |
|  | Optional: | No |
|  | Possible values: | 0 - 65535 |
|  | Default: | None |

##### 8.3.2.1.2	RTP SSRC

|  | Parameter name: | ssrc |
| --- | --- | --- |
|  | Parameter ID: | ssrc (0x0002) |
|  | Description: | This parameter indicates to which SSRC the RESUME relates. |
|  | Type: | Integer (32 bit) |
|  | Optional: | Yes |
|  | Possible values: | As per clause 3/[IETF RFC 3550]. |
|  | Default: | None. |

### 8.3.3	Refuse

|  | Signal name: | Refuse |
| --- | --- | --- |
|  | Signal ID: | refuse (0x0003) |
|  | Description: | This signal indicates that the MG shall not change the state of the RTP stream. If the MG had received a remote request to change the state, it should send a REFUSE indication to the remote end. |
|  | Signal Type: | Brief |
|  | Duration: | None |

#### 8.3.3.1	Additional parameters

##### 8.3.3.1.1	Pause Identity

|  | Parameter name: | Pause Identity |
| --- | --- | --- |
|  | Parameter ID: | pauseid (0x0001) |
|  | Description: | This parameter indicates the pauseID indicating the PAUSE to which the REFUSE relates. |
|  | Type: | Integer |
|  | Optional: | No |
|  | Possible values: | 0 - 65535 |
|  | Default: | None |

##### 8.3.3.1.2	RTP SSRC

|  | Parameter name: | ssrc |
| --- | --- | --- |
|  | Parameter ID: | ssrc (0x0002) |
|  | Description: | This parameter indicates to which SSRC the REFUSE relates. |
|  | Type: | Integer (32 bit) |
|  | Optional: | Yes |
|  | Possible values: | As per clause 3/[IETF RFC 3550]. |
|  | Default: | None. |

### 8.3.4	Remote Pause

|  | Signal name: | Remote Pause |
| --- | --- | --- |
|  | Signal ID: | rpause (0x0004) |
|  | Description: | This signal indicates that the MG shall send a PAUSE request to the remote RTP sender. |
|  | Signal Type: | Brief |
|  | Duration: | None |

#### 8.3.4.1	Additional parameters

##### 8.3.4.1.1	Pause Identity

|  | Parameter name: | Pause Identity |
| --- | --- | --- |
|  | Parameter ID: | pauseid (0x0001) |
|  | Description: | This parameter indicates the pauseID contained in the PAUSE request. |
|  | Type: | Integer |
|  | Optional: | No |
|  | Possible values: | 0 - 65535 |
|  | Default: | None |

##### 8.3.4.1.2	RTP SSRC

|  | Parameter name: | ssrc |
| --- | --- | --- |
|  | Parameter ID: | ssrc (0x0002) |
|  | Description: | This parameter indicates to which SSRC the PAUSE relates. |
|  | Type: | Integer (32 bit) |
|  | Optional: | Yes |
|  | Possible values: | As per clause 3/[IETF RFC 3550]. |
|  | Default: | None. |

### 8.3.5	Remote Resume

|  | Signal name: | Remote Resume |
| --- | --- | --- |
|  | Signal ID: | rresume (0x0005) |
|  | Description: | This signal indicates that the MG shall send a RESUME request to the remote RTP sender. |
|  | Signal Type: | Brief |
|  | Duration: | None |

#### 8.3.5.1	Additional parameters

##### 8.3.5.1.1	Pause Identity

|  | Parameter name: | Pause Identity |
| --- | --- | --- |
|  | Parameter ID: | pauseid (0x0001) |
|  | Description: | This parameter indicates the pauseID contained in the RESUME request. |
|  | Type: | Integer |
|  | Optional: | No |
|  | Possible values: | 0 - 65535 |
|  | Default: | None |

##### 8.3.5.1.2	RTP SSRC

|  | Parameter name: | ssrc |
| --- | --- | --- |
|  | Parameter ID: | ssrc (0x0002) |
|  | Description: | This parameter indicates to which SSRC the RESUME relates. |
|  | Type: | Integer (32 bit) |
|  | Optional: | Yes |
|  | Possible values: | As per clause 3/[IETF RFC 3550]. |
|  | Default: | None. |

## 8.4	Statistics

{Editor’s note: There may be a need for a statistic to indicate the ratio of inactivity (due to a pause) to activity. Contributions are solicited.}

None

## 8.5	Error Codes

None

## 8.6	Procedures

This Package allows the gateway to act in four main modes:

- MG autonomous handling of remote pause and resume requests where the MGC is notified of RTP stream state change.

- MGC controlled handling of remote pause and resume requests where the MGC is notified of pause and resume messages from a remote RTP sender and provides a response to the MG regarding the action to take.

- MGC initiated remote pause/resume requests where the MGC receives the pause and resume request results from MG.

- MGC initiated local pause/resume requests where the MGC receives the pause and resume request results from MG.

The procedures for these modes are described in clauses 8.6.3 to 8.6.7. Clauses 8.6.1 and 8.6.2 provide general procedures these modes.

### 8.6.1	General - SDP parameter setting

On H.248 Streams where RTP Pause / Resume is supported, for all three modes of operation the MGC shall set SDP media descriptions in the Local and/or Remote Descriptors indicating the support of “pause” codec control message (CCM) [IETF RFC 5104] as per clause 9/[IETF STREAM PAUSE]. This allows the MGC to indicate to the MG the support of MG initiated local pause and resume without the need for H.248 Signals and Events.

#### 8.6.1.1	Pause

The SDP for “pause” allows a “config” parameter to be set against an m-line which indicates what Pause/Resume CCM messages it supports. This description is based on the m-line describing a bi-directional bearer however the Local and Remote Descriptor describes a unidirectional bearer. Rather than mapping the “config” value to a separate value for the Local and Remote Descriptor the same value shall be set on both these Descriptors.

### 8.6.2	General – Event and Signal Scope

Given that pause and resume relate to a single RTP stream, Signals and Events should be set in such a way that the affected RTP streams are unambiguously identified. Where a H.248 Stream contains a single RTP stream (i.e. a single SSRC) the MGC may omit the ssrc parameter when setting any Signals and Events in this package. Where a H.248 Stream contains multiple RTP streams the MGC shall include the ssrc parameter indicating the SSRC of the applicable RTP stream.

Where there are multiple H.248 Streams on a Termination the MGC shall set the StreamID related to the RTP Strean in the Events and Signals in this package.

### 8.6.3	MG autonomous handling of remote requests

The MGC controller may confer handling of RTP Pause/Resume requests to the MG by setting the “Autonomous Response” (rempr/ar) property to “on” indicating that the MG shall autonomously process and respond to any RTP pause/resume messages as defined by [IETF STREAM PAUSE]. If the MGC requires to be notified of any changes in RTP transmission state the MGC shall set the “RTP Pause State” (rempr/rtppps) event with the states that it requires to be informed about. The MGC shall also set the ‘pause’ parameter in the Local and Remote Descriptor as appropriate according to clause 8.6.1.

On recept of a PAUSE request message the MG shall as RTP sender act according to clauses 5.2 and 5.3/[IETF STREAM PAUSE]. This may result in the MG pausing the RTP stream and the sending of a PAUSED indication or it sending a REFUSE message and continuing the transmission of the RTP stream. If requested when entering the PAUSED state the MG will generate a Notify.req with a “RTP Pause State” (rempr/rtpps) ObservedEvent indicating that the RTP Stream has entered the “paused” state (obstate=“paused”).

On receipt of a RESUME request message the MG shall as RTP sender act according to clause 5.4 /[IETF STREAM PAUSE]. This may result in the MG resuming the RTP stream or the sending of a REFUSE message and the continuation of the paused state. If requested when resuming media flow the MG will generate a Notify.req with a “RTP Pause State” (rempr/rtpps) ObservedEvent indicating that the RTP Stream has been resumed (obstate=“resumed”).

According to clause 5.3/[IETF STREAM PAUSE] an MG may pause an RTP Stream at any time. If the MG initiates a local pause and it has been requested to report this state, it shall generate a Notify.req with a “RTP Pause State” (rempr/rtpps) ObservedEvent indicating that the RTP Stream has been paused (obstate=“localpause”) due to a local decision.

According to clause 5.4/[IETF STREAM PAUSE] an MG may resume an RTP Stream at any time. If the MG initiates a local resume and it has been requested to report this state, it shall generate a Notify.req with a “RTP Pause State” (rempr/rtpps) ObservedEvent indicating that the RTP Stream has been resumed (obstate=“localresume”) due to a local decision.

### 8.6.4	MGC reception of pause and resume requests

The MGC may require that it be responsible for the decision of whether or not to pause or resume an RTP Stream. In this case it shall set the “Autonomous Response” (rempr/ar) property to “off” indicating to the MG that it shall defer the decision to pause/resume an RTP Stream to the MGC. The MGC shall also set the “Detect Pause/Resume Request” (rempr/dprreq) to request notification of pause or resume requests.

On receipt of a PAUSE request the MG shall send a Notify.req with the “Detect Pause/Resume Request” (rempr/dprreq) ObservedEvent indicating a request type indicating a pause (reqt=”PAUSE”). The MGC shall take into any topology (see clause 7), service or other local considerations to determine whether to grant the pause request. In order to grant the request the MGC shall send the “pause” (rempr/lpause) signal to the MG. On receipt of the signal the MG shall pause the RTP Stream and send a PAUSED indication as per clause 5.3/[IETF STREAM PAUSE]. To deny the request, the MGC shall send the “refuse” (rempr/refuse) signal to the MG. On receipt of the signal the MG shall send a REFUSED indication as per clauses 5.2 and 5.3/[IETF STREAM PAUSE].

On receipt of a RESUME request the MG shall send a Notify.req with the “Detect Pause/Resume Request” (rempr/dprreq) ObservedEvent indicating a request type indicating a resume (reqt=”RESUME”). The MGC shall take into account any topology (see clause 7), service or other local considerations to determine whether to grant the resume request. In order to grant the request the MGC shall send the “resume” (rempr/lresume) signal to the MG. On receipt of the signal the MG shall resume the RTP Stream as per clause 5.4/[IETF STREAM PAUSE]. To deny the request, the MGC shall send the “refuse” (rempr/refuse) signal to the MG. On receipt of the signal the MG shall send a REFUSED indication as per clauses 5.4/[IETF STREAM PAUSE].

The MG shall not send a Notify.req with “Detect Pause/Resume Request” (rempr/dprreq) ObservedEvent with a request type for a pause state that the RTP stream is already in. I.e. the MG shall not send a request type equal to pause if the RTP stream is already paused.

### 8.6.5	MGC initiation of remote pause and resume requests

The MGC may require that for a particular Stream that the remote RTP sender pauses RTP transmission. In order to request the MG to send a PAUSE request to the remote RTP sender, the MGC shall send the “Remote Pause” (rempr/rpause) Signal to the MG. On reception of the signal the MG shall initiated a PAUSE request on the applicable RTP Stream. The MGC may provide or wildcard CHOOSE a Pause Identity (pauseid). If the pauseid is wildcarded, the MG shall return the chosen value in a command reply. The MG will send the pause identity in the PAUSE request. The use of the pause identity allows the MGC and MG to correlate pause/resume requests and replies.

The MGC may require that for a particular Stream that the remote RTP sender resumes RTP transmission. In order to request the MG to send a RESUME request to the remote RTP sender, the MGC shall send the “Remote Resume” (rempr/rresume) Signal to the MG. On reception of the signal the MG shall initiate a RESUME request on the applicable RTP Stream. The MGC shall provide a fully specified a Pause Identity (pauseid).

In order that the MGC may be notified of the response from the remote RTP sender regarding MGC initiated PAUSE and RESUME requests it shall set the “Detect Pause Resume Result” (rempr/dprres) Event. On reception of a rempr/rpause signal with a pause identity, the MG arms the Event and sends a Notify.req to the MGC when it detects a PAUSED or REFUSED indication for the applicable pause identity. On reception of a rempr/rresume signal with a pause identity, the MG arms the Events and sends a Notify.req to the MGC when it detects a REFUSED indication or when it detects a resumed RTP flow for the applicable RTP stream. The Notify.req contains the dprres ObservedEvent containing the Response Type (rest). If after a suitable timeout period (see clause 4.6/[IETF STREAM PAUSE] the MG does not detect any response from the RTP sender (either by signaling pause CCM messages or via a modification in the sending of the RTP stream) the MG shall send a Notify.req containing the dprres ObservedEvent containing the Response Type indicating a failure.

### 8.6.6	MGC initiation of local pause and resume

The MGC may require that for a particular Stream that the MG as RTP sender pauses RTP transmission. In order to request the MG to pause RTP sending, the MGC shall send the “Local Pause” (rempr/lpause) Signal to the MG. On reception of the signal the MG shall initiate a PAUSE on the applicable RTP Stream. The MG shall send a PAUSED indication to the RTP receivers. The MGC may provide or wildcard CHOOSE a Pause Identity (pauseid). If the pauseid is wildcarded, the MG shall return the chosen value in a command reply. The MG will send the pause identity in the PAUSED indication.

The MGC may require that for a particular Stream that the MG as RTP sender resumes RTP transmission. In order to request the MG to resume RTP sending, the MGC shall send the “Local Resume” (rempr/lresume) Signal to the MG. On reception of the signal the MG shall initiate a RESUME request on the applicable RTP Stream. The MGC shall provide a fully specified a Pause Identity (pauseid).

In order that the MGC may be notified of the response of the MG to MGC initiated local PAUSE and RESUME requests it shall set the “Detect Pause Resume Result” (rempr/dprres) Event. On reception of a rempr/lpause signal with a pause identity, the MG sends a Notify.req to the MGC when it enters the local paused state. On reception of a rempr/lresume signal with a pause identity, the MG sends a Notify.req to the MGC when it resumes RTP flow for the applicable RTP stream. The Notify.req contains the dprres ObservedEvent containing the Response Type (rest).

### 8.6.7	MG initiation of remote pause and resume

{Editor’s note: MG autonomous initiation of remote pause and resume should be considered.}

### 8.6.8	Error handling

The MGC shall not send signals or set events that are incompatible with the SDP “config” CCM parameters. For example, the MGC shall not request the “rpause” signal on a Stream if it has not set the “config” parameter equal to 1,2 or 4. The table below indicates the allowed settings:

Table 2 – Allowed setting of “rempr” package signals and events

| Package “rempr” Signals | Valid “configs” | Comments |
| --- | --- | --- |
| Local Pause (lpause) | 1,2,3,5,6,8 |  |
| Local Resume (lresume) | 1,2,4,5 |  |
| Refuse (refuse) | 1,2,5 |  |
| Remote Pause (rpause) | 1,2,4 |  |
| Remote Resume (rresume) | 1,2,4 |  |
| Events |  |  |
| RTP Pause State (rtpps) | 1,2,3,4,5,6,7,8 | As the event may detect in state it may be set against any configuration. |
| Detect Pause Resume Request (dprres) | 1,2,3,4,6,7 |  |
| Detect Pause Resume Result (dprres) | 1,3,5 |  |

If the MGC sends a Signal or Event without having also set the having set the ‘pause’ or ‘paused’ SDP parameter in a Local or Remote Descriptor then the MG shall respond with error 472 “Required Information Missing”. If the MGC sends a Signal or Event on a Stream where the ‘pause’ or ‘paused’ SDP parameter is set contravening table 2, then the MG shall respond with error 473 “Conflicting Property Values.

# 9	Examples

{Editor’s note: TBD}

Annex A

<Annex Title - placeholder>

(This annex forms an integral part of this Recommendation.)

<Body of annex A>

Appendix I

<Appendix Title - placeholder>

(This appendix does not form an integral part of this Recommendation.)

<Body of appendix I>

Bibliography (placeholder)

[b-ITU-T X.yyy]	Recommendation ITU-T X.yyy (date), Title.

____________________