| Rapporteur Meeting of Questions 2, 3, 5 and 21/16 |  |  |  | AVD-4680 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3 | Guilin, 8 - 12 June 2015 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Editor H.248.STGROUP |  |  |  |
| Title: | H.248.STGROUP “Gateway Control Protocol: H.248 Stream grouping and aggregation” (New): input draft |  |  |  |
| Purpose: | Proposal |  |  |  |

1.	Introduction

This document contains the Editor’s input draft for H.248.STGROUP “Gateway Control Protocol: H.248 Stream grouping and aggregation” (New).

2.	Discussion

It is a copy of TD 233r2/WP1 from the Geneva, 9-20 February 2015 meeting. There are no changes.

3.	Proposal

It is proposed to accept this document as the baseline text for the meeting.

Document History

| Document History |  |
| --- | --- |
| Issue | Notes |
| H.248.STGROUP Ed. 0.1 | Initial baseline text for H.248.STGROUP. Based on the output TD-22 from the RGM in Geneva, 10 - 14 March 2014 and C.523r1, C.582r1 and C.459 from the Sapporo, July 2014 ITU-T SG16 meeting. |
| H.248.STGROUP Ed. 0.2 | Input text to the Seoul, 3 – 7 November 2014 meeting. |
| H.248.STGROUP Ed. 0.3 | Output draft from the Seoul, 3 – 7 November 2014 meeting. Changes due to: AVD-4623, AVD-4624 |
| H.248.STGROUP Ed. 0.4 | Input draft to the Geneva, 9-20 February 2015 meeting. TD 233/WP1: http://www.itu.int/md/T13-SG16-150209-TD-WP1-0233/en |
| H.248.STGROUP Ed. 0.5 | Output draft for the Geneva, 9-20 February 2015 meeting. Changes due to: TSB comments, C.676r1, C.677, C.678, C.684, C.734 |
| H.248.STGROUP Ed. 0.6 | Input draft for the Guilin, 8 - 12 June 2015 meeting. |

CONTENTS

Page

1 Scope 6 2 References 7 3 Definitions 8 3.1 Terms defined elsewhere 8 3.2 Terms defined in this Recommendation 8 4 Abbreviations and acronyms 9 5 Conventions 10 6 H.248.1 defined stream grouping, aggregation and multiplexing 10 6.1 Reserve Group 10 6.2 Stream multiplexing using transport address information 10 7 Further issues with existing H.248 stream grouping mechanisms 12 7.1 Scope of Signals / Events and Statistics 12 7.2 Scope of Topology 12 8 Media Grouping package 12 8.1 Properties 13 8.1.1 Group Semantics 13 8.1.2 Stream Aggregation 13 8.1.3 Stream De-aggregation 14 8.2 Events 14 8.3 Signals 14 8.4 Statistics 14 8.5 Error Codes 14 8.5.1 Invalid aggregation and/or de-aggregation 14 8.6 Procedures 15 8.6.1 Support of media grouping 15 8.6.2 Mapping of group attributes in H.248 16 8.6.3 MG Behaviour 17 8.6.4 Error handling 21 8.6.5 Usage of stream aggregation 21 8.6.6 Usage of stream de-aggregation 28 8.6.7 Interaction of stream grouping methods "aggregation" and "de-aggregation" 30 8.6.8 Stream Level Interactions 31 Annex A Codepoints for parameter Semantics of H.248 property Group Semantics 31 A.1 Introduction and purpose 32 A.2 IETF defined codepoints 32 A.3 ITU-T defined codepoints 32 Bibliography 32 Appendix L Living List – H.248.STGROUP Possible use cases 34 L.1 Use case #1: alternate speech-text telephony 34 L.1.1 Use case #1.1: alternate speech-text telephony in IP-to-IP gateways (without RTP bundling) 34 L.1.2 Use case #1.2: alternate speech-text telephony in IP-to-IP gateways with RTP bundling 42 L.1.3 Use case #1.3: additional performance monitoring 42 L.2 Use case #2: WebRTC 42 L.2.1 Use case #2.1: WebRTC with RTP media multiplexing and separated H.248 streams for WebRTC DTLS session and underyling IP transport connection 42 L.2.2 Use case #2.2: WebRTC without RTP media multiplexing and a single SEP for the "SCTP/DTLS/L4/IP" stack segment 55 L.3 Use case #3: Telepresence media server 55 L.3.1 Use case #3.1: Protocol stack for application control mapped on group of three streams 55 L.3.2 Use case #3.2: Protocol stack for application control mapped on group of two streams 60 L.3.3 Use case #3.3: Protocol stack for application control mapped on a single stream 60 L.3.4 Benefits of stream grouping for CLUE-based telepresence 61 L.4 Use case #4: alternate IP connectivity (ALTC) 61 L.4.1 Use case #4.1: with support of early media for preferred IP connections 61 L.4.2 Benefits of stream grouping for ALTC 65 L.5 Use case #5: SCTP transport modes 65 L.5.1 Background 65 L.5.2 Guidelines for the derivation of H.248 streams per SCTP transport mode 66 L.5.3 Use case #5.1: Domain with "tunneled SCTP via switched {DTLS/UDP ¦ DTLS/RFC4571/TCP}" 68 Living List Bibliography 72

List of Tables

Page

Table 1 – Profile guidelines: package property combinations 15 Table 2 – Example command encoding– (SIP) SDP Offer 16 Table 3 – Example command encoding – MGC request 16 Table A.1 – Codepoints – Semantics for the "group" SDP Attribute (status: 12/2014) 32 Table A.2 – Codepoints – Semantics for the "group" SDP Attribute 32

List of Figures

Page

Figure 1 – Example single and multiplexed RTP streams 11 Figure 2 – Aggregated Stream 22 Figure 3 – Signals on an aggregation stream with FID semantic 23 Figure 4 – Events on an aggregation stream with SRF semantic 24 Figure 5 – Events on an aggregation stream with ANAT semantic 25 Figure 6 – Statistics on an aggregation stream with ANAT semantic 25 Figure 7 – Interworking between grouped streams and single stream 26 Figure 8 – Example of a multiple stream aggregation into a single stream 27 Figure 9 – Example of invalid direct stream and aggregation stream connection between two terminations 27 Figure 10 – De-aggregation stream 28 Figure 11 – Half context model – Principle use cases of stream aggregation (1) and stream de-aggregation (2) 30 Figure 12 – Cascaded de-aggregation to aggregation 31 Figure L.1 – Use case #1.1: alternate speech-text telephony in IP-to-IP gateways – Traffic flow models 34 Figure L.2 – Use case #1.1: alternate speech-text telephony in IP-to-IP gateways – Context stream grouping model 35 Figure L.3 – Overall message sequence diagram for a call in PES to IMS direction 36 Figure L.3 – Use case #1.1: summary of the example evolution of the considered context stream grouping model 41 Figure L.4 – Use case #2.1: WebRTC with RTP media multiplexing and separated H.248 streams for WebRTC DTLS session and underyling IP transport connection – Context stream grouping model 44 Figure L.5 – Use case #3.1: Telepresence server – Protocol stack for application control mapped on group of three streams 56 Figure L.6 – Use case #3.2: Telepresence server – Protocol stack for application control mapped on group of two streams 60 Figure L.7 – Use case #3.3: Telepresence server – Protocol stack for application control mapped on a single stream 61 Figure L.8 – Use case #4.1: Alternate IP connectivity – Context stream grouping model 62 Figure L.9 – Generic protocol stack model for mapping of SCTP streams to applications (here at the example of SCTP transport mode "tunneled SCTP via DTLS/L4") 66 Figure L.10 – Example decision logic for the derivation of H.248 streams per SCTP transport mode 67 Figure L.11 – Use case #5.1: SCTP transport modes – "tunneled SCTP via switched {DTLS/UDP ¦ DTLS/RFC4571/TCP}" 68

Draft new Recommendation ITU-T H.248.xx (ex H.248.STGROUP)

Gateway Control Protocol: 
H.248 Stream grouping and aggregation

AAP Summary

This Recommendation discusses how H.248 streams may be grouped according to a particular semantic and how they are aggregated or de-aggregated internally in the Context for the execution of H.248 control actions. It defines a H.248 package that allows an explicit grouping semantic to be assigned to a group of streams. It also introduces the concept and support for a new type of H.248 stream, the “aggregation stream”, the “de-aggregation stream” and the “component stream”. This allows H.248 to better support multiple media streams which share a common transport, the “de-aggregation stream” and the “component stream”.

Summary

This Recommendation discusses how H.248 streams may be grouped according to a particular semantic and how they are aggregated or de-aggregated internally in the Context for the execution of H.248 control actions. It defines a H.248 package that allows an explicit grouping semantic to be assigned to a group of streams. It also introduces the concept and support for a new type of H.248 stream, the “aggregation stream”.

The ability to group streams and to provide an explicit semantic allows the media gateway controller to control the processing of multiple streams. The aggregation and de-aggregation streams allow existing H.248 Descriptors to be applied to component streams as well as the aggregated/de-aggregation stream. That means Signals, Events, Statistics and Topology may be applied to aggregated/de-aggregation stream with benefits including for example: the ability to report cumulative Statistics rather than multiple individual statistics or report a single Event rather multiple events on the component streams. The ability to group streams and to provide an explicit semantic for these streams is also supported.

# 1	Scope

As described by the H.248 connection model (clause 6/[ITU-T H.248.1) a termination sources and/or sinks one or more streams. In a multimedia conference, a termination can be multimedia and sources or sinks multiple media streams. The H.248 protocol uses the stream concept to model individual media flows. That is typically there is one media (and optional control flow) per H.248 stream. The characteristics of each stream is described through the use of the local control, local and remote descriptors. Each termination contains a group of streams. H.248 makes no assumptions as to the relationship between the streams on a termination other than indicating that streams exiting a termination shall be synchronised with each other (clause 7.1.4/ITU-T H.248.1).

Certain network technologies may require a media gateway (MG) to have knowledge of how two or more streams relate to each other beyond simply knowing that they shall be synchronised. Where there is a multiplicity of streams there may be a different relation between different sets of streams. For the purposes of this Recommendation the relation between streams is defined as the “grouping semantic”. Depending on the grouping semantic the MG may be required to apply certain processing to the media related to the streams. For example: one semantic may indicate that the media is to be duplicated on separate streams, another semantic may indicate media is only sent on one of two streams at a time. Some grouping semantics require that media flows from multiple streams are aggregated into a single flow.

H.248.1 does not support a means to describe grouping semantics. This Recommendation defines the “media grouping package” that allows a media gateway controller (MGC)  to group media from two or more H.248 streams according to a particular semantic and optionally to aggregate/de-aggregate the media from the grouped streams according to the semantic.

The grouping framework in [IETF RFC5888] can in principle be applied to any transport. It is the actual grouping stream semantics that imposes limitations. I.e., BUNDLE may only be applied to IP transports. As such, the “media grouping package” and the concept of a H.248 aggregation stream can be in principle applied to any transport. The majority of defined stream semantics focus on IP based transports. As such, this Recommendation describes the semantics with respect to IP. Whilst some semantics may be used for any type of bearer technology the use of the semantics for these bearer technologies are not covered in this Recommendation.

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (2013), Gateway Control Protocol: Version 3.

[ITU-T H.248.7]	Recommendation ITU-T H.248.7 (2004), Gateway Control Protocol: Generic Announcement package.

[ITU-T H.248.47]	Recommendation ITU-T H.248.47 (2008), Gateway Control Protocol: Statistic conditional reporting package.

[ITU-T H.248.58]	Recommendation ITU-T H.248.58 (2008), Gateway Control Protocol: Packages for application level H.248 statistics.

[ITU-T H.248.61]	Recommendation ITU-T H.248.61 (2013), Gateway Control Protocol: Packages for network level ITU-T H.248 statistics.

[ITU-T H.248.65]	Recommendation ITU-T H.248.65 (2009), Gateway control protocol: Support of the resource reservation protocol.

[ITU-T H.248.SCTP]	Recommendation ITU-T H.248.SCTP (xx/xxxx), Gateway control protocol: H.248 support for control of SCTP bearer connections.

[IETF RFC 3524]	IETF RFC 3524 (2003), Mapping of Media Streams to Resource Reservation Flows.

[IETF RFC 4566]	IETF RFC 4566 (2006), SDP: Session Description Protocol.

[IETF RFC 4588]	IETF RFC 4588 (2006), RTP Retransmission Payload Format.

[IETF RFC 5583]	IETF RFC 5583 (2009), Signaling Media Decoding Dependency in the Session Description Protocol (SDP).

[IETF RFC 5888]	IETF RFC 5888 (2010), The Session Description Protocol (SDP) Grouping Framework.

[IETF RFC 5956]	IETF RFC 5956 (2010), Forward Error Correction Grouping Semantics in the Session Description Protocol.

[IETF RFC 7104]	IETF RFC 7104 (2014), Duplication Grouping Semantics in the Session Description Protocol.

[IETF BUNDLE]	IETF draft-ietf-mmusic-sdp-bundle-negotiation (201X), Negotiating Media Multiplexing Using the Session Description Protocol (SDP).

[IETF GRPTAX]	IETF draft-ietf-avtext-rtp-grouping-taxonomy (201X), A Taxonomy of Grouping Semantics and Mechanisms for Real-Time Transport Protocol (RTP) Sources.

[IETF RTPMUX]	IETF draft-ietf-avtcore-rtp-multi-stream (201X), Sending Multiple Media Streams in a Single RTP Session.

# 3	Definitions

## 3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1		Stream [ITU-T H.248.1]: Bidirectional media or control flow received/sent by a media gateway as part of a call or conference.

3.1.2		Stream Semantics [IETF RFC 5888]: the purpose for which the media streams are grouped.

{Editor’s note: the notion of STREAM in the two above term definitions are not necessarily the same. I guess that there might be H.248 Stream descriptor configurations with 1:1 and 1:N mappings between "H.248 (media) stream" and "RFC 5888 (media) stream".}

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	Aggregation: The MG internal process of combining two or more component streams on entering the H.248 context in the MG into one single stream, called aggregation stream. The way the component streams are combined is dependent on the grouping stream semantics defined for the component streams. It may consist of selecting one of the component streams, applying a reparation procedure to one of the component streams based on the information from another component stream, etc. In the reverse direction it may consist on delivering media to only one of them at a time, providing a replica, generating repairing or redundancy information, etc.

3.2.2	Aggregation stream: a stream that sources media to two or more component streams and/ or sinks media from two or more component streams which are related to each other with a certain grouping semantics. The aggregation stream exists only in the interior of the context. Within one context a stream may be defined as an aggregation stream in one termination and as a normal stream in another termination.

3.2.3	Component stream: A stream that acts as a source or sink to an aggregation or de-aggregation stream. Also, each of the streams defined as belonging to a semantic group.

3.2.4	De-aggregation: The MG internal process of splitting a stream (called the de-aggregation stream) on entering the H.248 context in the MG into multiple component streams. The way component streams are generated from a aggregation stream is dependent of the grouping semantics. It may consist on delivering media to only one of them at a time, etc. In the reverse direction it may consist of sending multiple component streams over the same de-aggregation stream.

3.2.5	De-aggregation stream: a stream acting as a media sink to two or more component streams and/ or sources media from two or more component streams that are related to each other with a certain grouping semantics. The component streams exist on the interior of the context whereas a de-aggregation stream represents an external stream. Within one context, a stream may be defined as a component stream in one termination and as a normal stream in another termination.

3.2.6	Group: Streams that share the same stream semantic are part of the same group. A stream may be part of several groups. Whether the streams in a group are aggregated or not depends on the stream semantic. Primarily [IETF RFC 5888] is utilised to group the streams and to provide the semantics.

3.2.7	Multiplexed stream: Multiple media streams are combined into a lower layer protocol.

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| AMR | Adaptive Multi Rate |
| --- | --- |
| ANAT | Alternative Network Address Types |
| CLUE | (IETF working group) ControLling mUltiple streams for tElepresence |
| CS | Circuit-Switched (network) |
| DDP | Decoding Dependency |
| DUP | Duplication |
| ID | Identity |
| FEC | Forward Error Correction |
| FEC-FR | Forward Error Correction Flow Repair |
| GTT | (3GPP) Global Text Telephony |
| ID | Identity |
| IP | Internet protocol |
| IPv4 | Internet protocol version 4 |
| IPv6 | Internet protocol version 6 |
| LD | Local descriptor |
| LS | Lip Synchronization |
| MG | Media gateway |
| MGC | Media gateway controller |
| NAT | Network address translation |
| NAT-T | NAT Traversal |
| NGN | Next generation network |
| PCM | Pulse Code Modulation |
| RD | Remote descriptor |
| RSVP | Resource Reservation Protocol |
| RTCP | RTP Control Protocol |
| RTP | Real Time Protocol |
| SDP | Session description protocol |
| SRF | Single Reservation Flow |
| SEP | (H.248) Stream endpoint |
| SEPP | (H.248) Stream endpoint pair |
| SIP | Session initiation protocol |
| UDP | User datagram protocol |
| WEBRTC | Real-Time Communication in WEB-browsers (as work item in W3C) |

# 5	Conventions

This Recommendation uses the terms “Grouping”, “Aggregation”, “De-aggregation” and “Multiplexing”. As per the definitions:

- Grouping describes the process using H.248 to group streams together that share a common stream semantic.

- Aggregation describes the process of using H.248 to indicate that the media of several streams is aggregated in a single stream.

- De-aggregation describes the process of using H.248 to indicate that the media of a single stream is deaggregated in multiple streams.

- Multiplexing is the process used to carry multiple streams over the same transport.

# 6	H.248.1 defined stream grouping, aggregation and multiplexing

This clause describes the existing H.248.1 mechanisms related to stream grouping, aggregation and multiplexing.

## 6.1	Reserve Group

Within a single stream, H.248.1 allows multiple sets of local and/or remote parameters to be specified. This is accomplished through the use of the ReserveGroup attribute (clause 7.1.7.1.2/‌[ITU-T H.248.1]). This allows a MGC to request that a MG reserve resources for functions described in each group. Media packets may be sent or received according to any of the groups although only a single alternative may be active at any given time. The Reserve Group concept does not describe any grouping semantic other than an implicit semantic indicating that the different groups may only be sent one at a time As such the Reserve Group concept does not lead to aggregated streams. The use of aggregation on streams related to each other by reserve group is not possible unless they are also grouped with other grouping semantics. Clause 8.6.3 describes several grouping semantics where the reserve group concept may be used.

## 6.2	Stream multiplexing using transport address information

Typically, a single media stream is assigned to a single H.248 Stream endpoint by specifying a unique address/port combination in the Local Descriptor per H.248 Stream on a Termination along with the associated media properties. In general, to indicate the use of a multiplexed media stream where multiple streams are sent using the same transport address, the MGC requests the same address/port in the Local Descriptor for all H.248 Streams that are to make up the multiplexed media stream.

As such when the same transport address/port is used for several streams there is an implicit grouping semantic which effectively multiplexes several streams over the same transport.

For text-based H.248, SDP is used for each Local Descriptor. In order to request to the same address/port the following conditions must be must:

- The value of the c-line must be the same for each Local Descriptor

- The value of m-line [IETF RFC 4566] <port> field is the same for each Local Descriptor.

A MGC may fully specify the address/port information for multiple H.248 Streams in a single Termination in an Add.req command. However in many cases the MGC will wildcard “choose” this information, in order for the MG to provide the address/port. In the case where wildcarding is used the MGC can:

- request the address/port information for one of the H.248 Streams involved in the multiplex and then use the same address/port information in subsequent Mod.req/s for additional H.248 Streams, or

- request address/port information for all the H.248 Streams involved in the multiplex and then select the address/port from one of the H.248 Streams and then send a subsequent Modify.request to modify the address/port information. This has the disadvantage over option a) that some resources may have been allocated to the previous address/port and these allocations would have to be changed.

Figure 1 shows example syntax for a Termination (T1) utilising two RTP Streams and Termination (T2) utilising one RTP Stream for two H.248 Streams.

| Two H.248 Streams, Two RTP Streams | Two H.248 Streams, One RTP Stream |
| --- | --- |
| MEGACO/3 [123.123.123.4]:55555 Transaction = 10003 { Context = 1 { Add = T1 { Media { Stream = 1 { Local { v=0 c=IN IP4 124.124.124.222 m=audio 2222 RTP/AVP 4 } } Stream = 2 { Local { v=0 c=IN IP4 124.124.124.222 m=video 2223 RTP/AVP 34 } } } } } } | MEGACO/3 [123.123.123.4]:55555 Transaction = 10004 { Context = 1 { Add = T2 { Media { Stream = 1 { Local { v=0 c=IN IP4 124.124.124.222 m=audio 2222 RTP/AVP 4 } } Stream = 2 { Local { v=0 c=IN IP4 124.124.124.222 m=video 2222 RTP/AVP 34 } } } } } } |

Figure 1 – Example single and multiplexed RTP streams

On reception of an Add, Modify or Move request command with several Local Descriptors from the same Termination containing the same address/port information the MG shall treat the associated H.248 Streams as a multiplexed media stream. Multiplexing is applied according to the type or profile of the stream. For example, in the case that a media profile RTP/AVP is given RTP multiplexing is applied. Each media stream component of the overall multiplexed RTP Stream is coded according to the information Local Descriptor / H.248 Stream for that particular media.

RTP multiplexing is described by [IETF RTPMUX].

This multiplexed media stream may contain more than one media type.

Multiple multiplexed media streams are also possible on a Termination.

Individual components of the media multiplex can be modified by changing the Local and/or Remote Descriptors of the related H.248 Stream. If the address/port information of a H.248 Stream is changed to be different from the other components of the multiplexed media stream then the media defined by the H.248 Stream will longer be part of the multiplex and it will be its own media stream.

Clause 8.6.3 describes several grouping semantics where the use of transport address for multiplexing may be used.

# 7	Further issues with existing H.248 stream grouping mechanisms

This clause describes issues associated with the stream grouping described in clause 6.

## 7.1	Scope of Signals / Events and Statistics

Signals, Events and Statistics may be set at either a Termination or Stream level.  Generally when set at a Termination level the scope of this elements applies to all of the H.248 Streams on the Termination. For example: the octets sent statistic would be the sum of octets sent by each H.248 Stream and by corollary each media stream.

When these elements are set at a Stream level the scope of the element is given by the StreamID. In the case of the octets sent statistic it would contain only the octets sent by the H.248 Stream given by the StreamID.

When applying grouping to media Streams the above behaviour should be considered. The grouped Streams may consitute a sub-set of the total H.248 Streams in a Termination. For example: StreamID(1) could be a single stream, StreamIDs(2&3) could be grouped media Streams. This leads to complications regarding the scope of how Signals, Events and Statistics are applied. For example: A Termination level octets sent Statistic would correctly give the number of octets sent by the Termination however there is no way to request an octets sent Statistic related to the grouped media streams. The MGC would have to request a statistic for StreamID(2) and another statistic for StreamID(3) and add them together.

This issue is also applicable to Signals and Events. This is no way to indicate that a Signal or Event is only applicable to those grouped streams

The Media Grouping package defined in this Recommendation introduces an aggregation stream concept that allows signals, events and statistics to be set against an aggregation stream. What effect these elements have is based on the semantics associated with the aggregation.

## 7.2	Scope of Topology

For some of the media grouping semantics a MG may be required to interconnect a group of media streams at one interface with a single stream or a group of a different number of streams at another interface. This type of connectivity cannot be described with the existing topology descriptor.

The Media Grouping package defined in this Recommendation introduces an aggregation stream concept. This allows an MGC to indicate a topology for the aggregation stream.

# 8	Media Grouping package

|  | Package Name: | Media Grouping |
| --- | --- | --- |
|  | Package ID: | mgroup (0x011f) |
|  | Description: | This package provides a general way for the MGC to instruct the MG to group media streams defined in different stream descriptors within the same media descriptor. It also introduces the “aggregation” and “de-aggregation” H.248 Stream concept and provides properties to indicate these types of H.248 Strreams. |
|  | Version: | 1 |
|  | Extends: | None |

## 8.1	Properties

### 8.1.1	Group Semantics

|  | Property Name: | Group Semantics |
| --- | --- | --- |
|  | Property ID: | groupse (0x0001) |
|  | Description: | This property allows a MGC to indicate which media streams to group and for which purpose they are grouped. This allows the MG to allocate resources accordingly. For example, providing a common address/port in response to a wildcarded request from the MGC in case multiplexing is supported. In addition, the optional “Direction” element allows indicating whether the grouping is applicable in both directions or in one particular direction. When not included a default of “bothway” is assumed. Several sets of grouped streams are allowed per Termination. |
|  | Type: | Sub-list of String |
|  | Possible values: | Each element of the sub-list follows the group-attribute syntax in [IETF RCF 5888], here represented using type “Group”: Group = Semantics 1*WSP StreamIDList[1*WSP Direction] ; (NOTE 1) WSP = SP/HTAB StreamIDList = StreamID *(WSP StreamID) StreamID = UINT16 Semantics = token token-char = %x21 / %x23-27 / %x2A-2B / %x2D-2E / %x30-39/ %x41-5A / %x5E-7E token = 1*(token-char) Direction = Bothtoken / Sendtoken / Receivetoken Bothtoken = “B” Sendtoken = “S” Receivetoken = “R” (NOTE 2) As defined by [ITU-T H.248.1 Annex B]. NOTE 1 – The syntax functionally duplicates the Grouping attribute syntax from [IETF RFC 5888] with the optional addition of a direction parameter. NOTE 2 – This syntax definition allows for a Stream ID list containing one single value. This case is only allowed in a reply to an AuditCapabilities command, where the StreamId shall be set to 0. |
|  | Default: | None. |
|  | Defined in: | TerminationState |
|  | Characteristics: | Read/Write |

### 8.1.2	Stream Aggregation

|  | Property Name: | Stream Aggregation |
| --- | --- | --- |
|  | Property ID: | stragg (0x0002) |
|  | Description: | This property indicates that the stream that this property is set on is an aggregation of the component streams specified in the property. The component streams must be grouped according to the grouping semantics indicated by the groupse property. |
|  | Type: | Sub-list of integer |
|  | Possible values: | Each value in the list is a Stream ID |
|  | Default: | None |
|  | Defined in: | Local Control |
|  | Characteristics: | Read/Write |

### 8.1.3	Stream De-aggregation

|  | Property Name: | Stream De-aggregation |
| --- | --- | --- |
|  | Property ID: | strdeagg (0x0003) |
|  | Description: | This property indicates that the data from the stream that this property is set on is de-aggregated to the component streams specified in the property. The component streams must be grouped according to the grouping semantics indicated by the groupse property. |
|  | Type: | Sub-list of integer |
|  | Possible values: | Each value in the list is a Stream ID. |
|  | Default: | None |
|  | Defined in: | Local Control |
|  | Characteristics: | Read/Write |

## 8.2	Events

None.

## 8.3	Signals

None.

## 8.4	Statistics

None.

## 8.5	Error Codes

### 8.5.1	Invalid aggregation and/or de-aggregation

|  | Error Code #: | 489 |  |
| --- | --- | --- | --- |
|  | Name: | Invalid aggregation and/or de-aggregation |  |
|  | Definition: | The media stream components received in the Stream Aggregation and/or Stream De-aggregation results in an invalid configuration on the Termination. |  |
|  | Error text in the Error Descriptor: |  | None |
|  | Comment: | - |  |

## 8.6	Procedures

### 8.6.1	Support of media grouping

Usage and support of media grouping may be pre-agreed in a profile specification or may be discovered via the auditing capability.

An MGC may audit the Packages Descriptor to determine if a MG supports media grouping. If the response contains the “Media Grouping Package” (mgroup) it can assume that the MG support media grouping. The MGC may perform an H.248 AuditCapability request command on the groupse property to determine which semantics are supported by the MG. In this case, the MG shall respond with the supported semantic tags and the Stream ID set to 0.

An MGC controlling a MG that supports the Media Grouping package can instruct the MG to group two or more sent and/or received media streams. Each media stream (including the sent and received directions) is described in its own Stream Descriptor and all are included in the same Media Descriptor.

The MGC can indicate to the MG:

- Which sent media streams to group

- Which received media streams to group and for which purpose the media streams are grouped.

The mgroup package defines three properties, which may be used in various combinations. Table 1 indicates valid combinations:

Table 1 – Profile guidelines: package property combinations

NOTE – The notion of "self-contained" means that the H.248 Media Descriptor contains only the groupse property, without property stragg or strdeagg.

The groupse property can be used in a self-contained manner, i.e., used in a particular H.248 Termination without the stream-level properties. The stream-level properties stragg and strdeagg are always used in conjunction with the groupse property, i.e., there are (grouping semantic specific) tandems of {groupse|stragg} or {groupse|strdeagg} in the description of a particular H.248 Termination.

In general the mgroup properties can be used together with the H.248 Reserved Group concept (see clause 7.1.7/[ITU-T H.248.1] as the ReserveGroup property applies to alternatives related to a particular H.248 stream. The mgroup package relates multiple H.248 streams. If ReserveGroup is used with the mgroup package properties, the MGC should ensure that any semantic is applicable to each of reserved groups on the applicable stream.

### 8.6.2	Mapping of group attributes in H.248

An MGC that sends or receives SDP media grouping related attributes in an SDP Offer/Answer dialogue should map those attributes as follows when sending the command to the MG:

- Each m-block is mapped into one Stream Descriptor.

- The “a=mid” attribute of each m-block is dropped in the Local/Remote Descriptor of the corresponding Stream Descriptor, instead the StreamID, is used for the index in the group.

- The “a=group” attribute is mapped to the mgroup/groupse property and included in the TerminationState Descriptor. The content of the property, a sub-list of string, should be follow a copy of the “a=group” attribute. The semantic is retained and the mid value is replaced with the corresponding StreamID.

Example:

An MGC sends an SDP Offer with the following media description (see Table 2).

Table 2 – Example command encoding– (SIP) SDP Offer

| (SIP) SDP encoding | Comments |
| --- | --- |
| v=0 o=Alice 289083124 2890844526 IN IP4 124.124.124.222 c=IN IP4 124.124.124.222 a=group:LS 1 2 m=audio 30002 RTP/AVP 97 a=mid 1 a=rtpmap:97 AMR/8000 a=fmtp:97 mode-set=0,2,5,7; mode-change-period=2; \ mode-change-neighbor = 1; mode-change-capability=2; max-red=0 m=video 52738 RTP/AVP 99 a=mid 2 b=TIAS:256000 b=AS:270 a=rtpmap:99 H264/90000 a=fmtp:99 profile-level-id=428014; a=sendrecv | It describes a session containing one video stream and one audio stream. They use the “LS” [IETF RF 5888] grouping semantics to indicate that the play-out of both media must be synchronized. |

The MGC maps this description into the Media Description in Table 3.

Table 3 – Example command encoding – MGC request

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| Media { TerminationState { mgroup/groupse = ["LS 1 2"]}, Stream = 1 { Remote { c=IN IP4 124.124.124.222 m=audio 30002 RTP/AVP 97 a=rtpmap:97 AMR/8000 a=fmtp:97 mode-set=0,2,5,7; mode-change-period=2; \ mode-change-neighbor = 1; mode-change-capability=2; max-red=0 } }, Stream = 2 { Remote { c=IN IP4 124.124.124.222 m=video 52738 RTP/AVP 99 b=TIAS:256000 b=AS:270 a=rtpmap:99 H264/90000 a=fmtp:99 profile-level-id=428014; a=sendrecv } } } | NOTE - A similar mapping would apply to the Local Descriptor. The Local Descriptor is not shown but it is also part of the Media Descriptor and will be sent to the MG. |

### 8.6.3	MG Behaviour

The behaviour of the MG with respect to the grouping is largely dependent on the semantic associated with the grouping, e.g. the value of semantic in the mgroup/groupse properties.

The SDP parameter IANA registry (http://www.iana.org/assignments/sdp-parameters/) contains a list of defined SDP group semantics. In general the MG should adopt the behaviour defined by the RFC (or other standard) that defines the semantic taking into consideration H.248 protocol behaviour. The following sub-clauses discuss several semantics.

#### 8.6.3.1	Lip Synchronisation (LS)

The “Lip Synchronisation” semantic is defined in [IETF RFC 5888]. As per clause 7/[IETF RFC 5888], the MG must synchronise the play out of H.248 Streams listed together in the same “LS” marked group.

##### 8.6.3.1.1	Directionality

In most scenario where LS is used both the video and audio would be bi-directional. However there are some scenarios such as synchronization of source audio and simultaneous translation, here lip sync may only be applied in one direction. [IETF RFC 5888] does not define an SDP mechanism to negotiate a unidirectional use of the LS semantics between peers other than the one implicitly given by defining one of the streams as unidirectional. The mgroup/groupse property allows for defining a unidirectional use of the LS semantics also if the grouped streams are all bi-directional. However, as this use does not have an SDP counterpart between SDP peers, it is discouraged.

#### 8.6.3.2	Flow identifier (FID)

The “Flow Identifier” semantic is defined in [IETF RFC 5888]. The use of the flow identifier with H.248 is in some cases redundant due to the H.248 connection model and H.248 usage of SDP. H.248 has the StreamID identifier which uniquely identifies media flows. Each H.248 stream may utilise the Reserve Group/Reserve Value (see 7.1.8.2.3/[ITU-T H.248.1]) construct which enables multiple configurations to be set for a H.248 stream. This allows a single H.248 stream to have multiple sets of characteristics of which one set is active at a particular point in time.

The following example described in 8.4.1/[IETF RFC 5888] can be emulated with the use of the Reserve Group mechanism:

Example 1:

Two audio streams are negotiated between two peers with different audio codecs. Only one is active at a time while the other one is muted. The peers indicate the same IP address for receiving both streams but different ports.

The following example also described in 8.4.1/[IETF RFC 5888] cannot be emulated with the use of the Reserve Group/Reserve Value constructs and therefore requires the use of the H.248 mgroup/groupse property:

Example 2:

Three audio streams are negotiated between two peers. The first two are bi-directional and use different audio codecs. Only one of these two is active at a time while the other one is muted. The third is unidirectional and has two payload types, one corresponding to each of the codecs used by the other two streams. This third stream always carries a copy of the active stream, either of the first or the second stream. Both active streams are received at the same IP address at different ports. The third stream is received at a different IP address.

NOTE – Reserve Group/Reserve Value can apply to alternatives using the same or different IP addresses or ports.

Another example use of the FID semantics that cannot be emulated with the ReserveGroup/Reserve Value constructs is the one described in 8.7/[IETF RFC 4588]:

Example 3:

Two SDP peers negotiate one audio and one video stream. In addition they negotiate another audio and another video streams used for retransmission. The audio source and retransmission streams are grouped with the FID semantics and the video source and retransmission streams are grouped with the FID semantics.

##### 8.6.3.2.1	Directionality

[IETF RFC 5888] does not define a mechanism to negotiate a unidirectional use of the FID semantics between peers other than the one implicitly given by defining one of the streams as unidirectional. The mgroup/groupse property allows for defining a unidirectional use of the FID semantics also if all the grouped streams are bi-directional. However, as this use does not have an SDP counterpart between SDP peers, it is discouraged.

#### 8.6.3.3	Single Reservation flow (SRF)

The “Single Reservation flow” semantic is defined in [IETF RFC 3524]. It indicates that media (described by m=lines) should be mapped to the same resource reservation flow. [ITU-T H.248.65] defines a package for the support of RSVP. The initiation of a resource reservation request is typically explicitly requested through the use of a H.248 signal. Any identification of flow is part of the parameters to the signal.

If the wanted behaviour is the allocation and subsequent policy enforcement of a predefined total bandwidth for a number of combined streams, the use of the SRF semantics is required as opposed to allocate individual bandwidths for each stream. The use of [ITU-T H.248.65] may also be applicable in combination with the mgroup package. In that case sent or received RSVP messages will refer to all grouped media combined. For that reason, signals and events of the rsvp package should be sent or activated in the aggregation stream when they are conveying RSVP messages related to a grouped media streams (see clauses 8.6.5.1 and 8.6.5.2).

##### 8.6.3.3.1	Directionality

[IETF RFC 3524] does not define a mechanism to negotiate a unidirectional use of the SRF semantics between peers other than the one implicitly given by defining one of the streams as unidirectional. Thus, it must be assumed that the negotiation of multiple streams grouped with the SRF semantics between peers is bi-directional if the grouped streams are also bi-directional.

However, this semantic is commonly associated with RSVP to reserve the required bandwidth. RSVP is unidirectional in nature. Thus, the SRF semantics may require two RSVP sessions. The use of RSVP in a MG requires [ITU-T H.248.65], which has the inherent mechanisms to determine the applicable direction, e.g. sending of an RSVP Path message with the H.248 rsvp/path signal is related to the outgoing stream while sending an RSVP Resv message with the H.248 rsvp/resv signal is related to the incoming stream.

If the mgroup/groupse property is used with the direction parameter together with the SRF semantics, the sent or received RSVP messages should apply or not to all grouped streams depending on the direction. For example, if the MGC sends mgroup/groupse = “SRF 1 2 R” to the MG, an RSVP Resv message sent by that termination will refer to both streams 1 and 2 combined, and the MGC should set the rsvp/resv signal in the aggregation stream; however, an RSVP Path message will refer only to the individual stream where the signal rsvp/path is set..

#### 8.6.3.4	Alternative Network Address Types (ANAT)

The “Alternative Network Address Types” semantic is defined in [b-IETF RFC 4091]. The ANAT semantic allows an SDP offerer to indicate that multiple network address types may be used for a single logical media stream. This semantic is redundant in H.248 as it offers multiple methods for indicating or requesting address types. A MGC may use “over specification” (see 7.1.8.2.2/[ITU-T H.248.1) to provide a list of network address types for a particular H.248 Stream. A MG will then chose an appropriate network address type.

An MGC that wants to initiate an SDP offer with the ANAT semantics can reserve resources for both transport types using two property groups. Both property groups will contain the same information except for the IP address information (e.g. the c= attribute) which will be partially wildcarded, and will request an IPv4 address in one of the groups and an IPv6 address in the other group. The MGC will set ReserveGroup to true. Thereafter it will send the SDP offer.

The MGC that receives the SDP offer with the ANAT semantics may be able to determine by itself which of the transport types to use, e.g. by certain call or traffic distribution criteria. In that case it will only indicate one address type when adding the termination in the MG. The MGC may also delegate that decision to the MG. In that case it will send two property groups, like the initiating MGC, but in this case, it will set ReserveGroup to false. The MG will only allocate resources for one of the transport types and indicate the chosen one to the MGC in the ADD reply.

The initiating MGC, when receiving the SDP answer which contains the chosen transport type, may send a H.248 Modify command to the MG, this time excluding the property group with the transport type that was not chosen.

#### 8.6.3.5	Forward Error Correction (FEC)

The “Forward Error Correction” semantic is defined in [IETF RFC 5956]. The use of “FEC” has been deprecated and should not be used with H.248.

#### 8.6.3.6	Forward Error Correction FR (FEC-FR)

The “Forward Error Correction FR” semantic is defined in [IETF RFC 5956]. This is used to associate source and repair flows for the purposes of forward error correction (FEC). If this semantic is received the MG should utilise the source and repairs H.248 Streams listed together in the same “FEC-FR” marked group for the purposes of FEC.

The MGC should indicate whether a flow is a source or repair flow through the use of coding related parameters (i.e. a=rtpmap:111 1d-interleaved-parityfec/90000 and a=fmtp:111 L=10; D=10; repair-window=400000).

If the session consists only of one source stream and one repair stream, the MG can unambiguously identify by means of the above attribute, which is the source and which the corresponding repair stream. The mgroup/groupse property may be omitted in this case. However, if there are multiple source streams and/or multiple repair streams, it may not be possible to unambiguously identify to which source each repair stream corresponds. Furthermore, the forward error correction mechanism allows for different types of correction schemes: n repair streams may be related to m source streams. In all these cases the SRF semantics is required to indicate the association between repair and source streams.

#### 8.6.3.6.1	Directionality

[IETF RFC 5956] does not define a mechanism to negotiate a unidirectional use of the FEC-FR semantics between peers other than the one implicitly given by defining one of the streams as unidirectional. The mgroup/groupse property allows for defining a unidirectional use of the FEC-FR semantics also if all the grouped streams are bi-directional. However, as this use does not have an SDP counterpart between SDP peers, it is discouraged.

Thus, the preferred method to specify the use of forward error correction only in one direction is with the use of StreamMode (SendOnly or RecvOnly) or the SDP direction attribute (a=sendonly or a=recvonly) in the repair stream(s).

#### 8.6.3.7	Decoding Dependency (DDP)

The “Decoding Dependency” semantic is defined in [IETF RFC 5583]. This semantic may be used with H.248 as well as other SDP attributes (e.g. a=depend) to describe decoding dependencies for layered codings.

[IETF RFC 5583] does not define a mechanism to negotiate a unidirectional use of the DDP semantics between peers other than the one implicitly given by defining one of the streams as unidirectional. The mgroup/groupse property allows for defining a unidirectional use of the DDP semantics also if all the grouped streams are bi-directional. However, as this use does not have an SDP counterpart between SDP peers, it is discouraged.

#### 8.6.3.8	Multiplexing Negotiation Using Session Description Protocol (BUNDLE)

The “BUNDLE” semantic is defined in [IETF draft-ietf-mmusic-sdp-bundle-negotiation]. It used to describe that two or more RTP streams use a single 5-tuple (i.e. multiplex multiple RTP streams onto a single 5-tuple). As [IETF BUNDLE] only describes the use of BUNDLE for IP transports, it shall not be used with Streams using non-IP transports.

As described in clause 7 it is possible to request that a MG use a multiplexed RTP stream however there are a number of deficiencies.

Firstly the MGC is unaware if the MG supports RTP multiplexing. In order to determine if the MG supports RTP multiplexing the MGC may audit the media grouping package to determine if the “BUNDLE” semantic is supported (see clause 8.6.1).

Secondly there is no way to wildcard CHOOSE the address/port information from several H.248 Streams and request that a single transport connection (i.e. multiplexed RTP stream) be used. The media grouping package contains the “group semantics” property that, when including the BUNDLE semantic allows the MGC to indicate that two or more H.248 Streams are multiplexed together. MGs that support the media grouping package should check the “group semantics” property when processing wildcarding requests for multiple H.248 Streams. If the H.248 Streams match the association given by this property then the same value should be returned for those H.248 Streams.

##### 8.6.3.8.1	Directionality

BUNDLE is symmetric in nature. If it is used, it applies to source and destination.

In practice, however, the required behaviour from the MG specific to this semantics is related to the Local Descriptor, i.e. to the received media. The use of the same address and port for multiple streams in the Remote Descriptor is usually not problematic in the MG, also when BUNDLE is not used.

Therefore, the use of the direction parameter in the mgroup/groupse property is superfluous in case of the BUNDLE semantics.

#### 8.6.3.9	Duplication (DUP)

The “DUP” semantic is defined in [IETF RFC 7104]. It indicates that the MG should duplicate packets and send them in separate redundant streams. The RFC allows the “DUP” semantic to be used within a single m-line where multiple redundant streams use the same IP destination address. For an example see clause 4.1/[IETF RFC 7104]). For this usage a MGC can use the SDP to indicating the grouping rather than using the mgroup package.

The RFC also allows the “DUP” semantic to be used across m-lines where the redundant streams have different destination addresses. For an example see clause 4.2/[IETF RFC 7104]. For this usage, the MGC shall use the mgroup/groupse to assign the DUP semantic to the relevant streams. Where a gateway is required to interwork between a redundant and non-redundant media stream the MGC shall use an aggregation stream to aggregate/de-aggregate the main and redundant streams.

##### 8.6.3.8.1	Directionality

The “DUP” semantic implicitly describes sending behaviour. Thus, the DUP semantic is associated with the sending direction. It is possible that whilst a Termination sends redundant media streams it receives a single stream in return. It is likely that these redundant stream would have a stream mode of send only so setting the DUP semantic with “both” direction would have no effect given the overriding stream mode. Even if the main stream was set with the DUP semantic with a “both” direction and the stream mode was send/receive there would be no effect on the incoming stream, due to the DUP semantics.

### 8.6.4	Error handling

In the following error cases, the MG should return an error code 473 “Conflicting Property Values”:

- If any of the stream identities included in the groupse property does not exist in the termination.

- If the stragg property is set without a value in the groupse property indicating the semantic associated with the aggregation stream, i.e. a value in the groupse property with the same streamID list as in the aggregation.

- If the strdeagg property is set without a value in the groupse property indicating the semantic associated with the aggregation stream, i.e. a value in the groupse property with the same streamID list as in the aggregation.

- Depending on the grouping semantics if there is a mismatch between the mgroup/groupse property and the Local or Remote Descriptor.

For example: An error may be generated if the MGC uses the BUNDLE media grouping and specifies a StreamID of a H.248 Stream whose Local Descriptor does not have the same address/port as the other H.248 Streams in the group (and is not updated in the current command). Another example is if an error may be generated if MGC uses the “LS” semantic and specifies the StreamIDs of H.248 streams where the MG cannot maintain lip sync. A more specific error code may be returned if available. The underlying reasons for generating an error may depend on the semantic associated with the group.

### 8.6.5	Usage of stream aggregation

In some grouping scenarios, multiple media streams may be aggregated when they enter the context into one media stream and, in the other direction, may result from the reverse operation of splitting one media stream into several media streams. Clause 6.2/[ITU-T H.248.1] describes the multiplexing of Terminations through the use of a multiplex termination. However, it does not support multiplexing of streams within a Termination.

The mgroup package defines the concept of an Aggregation Stream. By setting the stragg property on an aggregation stream indicating the component streams it allows stream aggregation to be modelled internally in the MG. Media is then aggregated to/from the component streams to the aggregation stream according to the applicable grouping semantics.

Figure 2 illustrates the concept.

Figure 2 – Aggregated Stream

In Figure 2 media streams labelled media 1, 2, and 3 received as described by the Stream descriptors Sid1, Sid2, Sid3 will be aggregated according to the groupse semantic for those streams. This aggregation will be sent via stream Sid4 to Termination Tb. It is then sent externally from the media gateway as described by the Stream descriptor for Stream Sid4 on Termination Tb.  The MG may apply any interworking/adaptation etc. functions that would normally apply when the same media stream is set on two Terminations. The aggregation stream may also remain isolated to one Termination in a Context.

The behaviour of the aggregation stream is well determined by the grouping semantics and the media descriptors of the corresponding component streams. Thus, the local control, local, and remote descriptors are not needed in the aggregation stream. If set the values of the properties should be compatible with those defined to the component streams and in no case should override any value set in those streams.

When entering or leaving the context, the component streams have a send and receive relationship with the aggregation stream. Thus, the stream mode does not need to be set in the aggregation stream. If set, it will be ignored by the MG.

A MGC may apply signals, events, statistics and topology to the aggregation stream as per normal streams. The behaviour is discussed in more detail in the sub-clauses below.

#### 8.6.5.1	Signals and aggregation streams

A signal applied to the aggregation stream will have the effect of being played out on one or more component streams. Which component streams the signal is played out over depends on the grouping semantic associated with the aggregation. Signals may still be specified to be played on the individual component streams. In this case, the playout is isolated to that particular component stream.

Figure 3 illustrates an example scenario using the Flow Identification semantic.

Figure 3 – Signals on an aggregation stream with FID semantic

Example 1:

Termination Ta has four streams, including an aggregation stream. It has two audio streams with identities 1 and 2 that use the same codec. They are grouped with the FID (Flow Identification) semantics [IETF RFC 5888]. Thus, they carry identical content. One is a replica of the other, usually sent to a different destination address. The termination has in addition other media stream with identity 3, which is not part of the media group. Termination Ta has an aggregation stream with streams 1 and 2 as the component. Requesting an an/apv [ITU-T H.248.7] signal on stream 4 to play an audio announcement should result in the announcement being sent synchronously to streams 1 and 2.

Example 2:

Termination Ta has 4 streams, including an aggregation stream. It has two audio streams with identities 1 and 2 that use different codecs. They are grouped with the FID semantics [IETD RFC 5888].  At a time, only one of the streams carries data in each direction while the other one is muted in that direction. This is dependent on the codec which is chosen dynamically by the end-user equipment. Termination Ta has an aggregation 4 stream with streams 1 and 2 as the component. Requesting an an/apv [ITU-T H.248.7] signal on stream 4 to play an audio announcement should result in the announcement being sent in the stream (1 or 2) which carries media at that moment.

Example 3:

A termination Ta has one video stream and two audio streams with identities 1, 2 and 3 grouped with the SRF (Single Reservation Flow) semantics [IETF RFC 3524]. Thus, resource reservation using RSVP and policy enforcing is applied to the complete group. The MGC defines an aggregation stream 4. In order to send RSVP messages related to all grouped streams, the MGC should set the signals of the rsvp package [ITU-T H.248.65], e.g. rsvp/path on the aggregation stream 4.

NOTE – RSVP sessions are normally defined by the triplet destination address, protocol ID, destination port. For the identification of the session related to grouped media, see [IETF RFC 3524].

#### 8.6.5.2	Events and aggregation streams

An event applied to the aggregation stream will have the effect of detecting events on the component streams as a result of stimulus usually from outside the context. How the event is reported is based on the grouping semantic associated with the aggregation stream. Events may still be specified on the component streams. In this case, detection is isolated to that particular component stream.

Where an event is set at the termination level on a termination that has an aggregation stream the effect will be that it is only set against the aggregation streams and stream not involved in an aggregation.

Example 1:

Figure 4 shows an example where a Context has a Termination Ta with one video stream and two audio streams with identities 1, 2 and 3 grouped with the SRF (Single Reservation Flow) semantics [IETF RFC 3524]. Thus, resource reservation using RSVP and policy enforcement is applied to the complete group. The MGC has defined an aggregation stream with stream identity 4. In order to receive RSVP messages related to all grouped streams, the MGC should request the events of the rsvp package [ITU-T H.248.65], e.g. rsvp/pathr event on stream 4. The detection of a RSVP path message related to stream1, 2 and 3 results in the notification of a single ObservedEvent from the aggregation stream.

NOTE – RSVP sessions are normally defined by the triplet destination address, protocol ID, destination port. For the identification of the session related to grouped media, see [IETF RFC 3524].

Figure 4 – Events on an aggregation stream with SRF semantic

Example 2:

Figure 5 shows an example where a Context has a termination Ta. It has two audio streams which use different codecs. They are grouped with the FID (Flow Identification) semantics [IETF RFC 5888]. Thus, at any point in time, only one of them carries data while the other one is muted. The termination has in addition other media stream with identity 3, not part of the media group. The MGC has defined an aggregation stream 4. The MGC requests the MG to collect data volume related statistics on the aggregation stream. The MGC uses also the conditional reporting functionality [ITU-T H.248.47] and activates therefore the scr/cr event on the aggregation stream and refers to the relevant statistics in the si parameter.

Figure 5 – Events on an aggregation stream with ANAT semantic

#### 8.6.5.3	Statistics and aggregation streams

When statistics are set against an aggregation stream, the statistics are collected as if they had been set against all the component streams. For example, if the rtp/pr “RTP packets received” statistic is set on an aggregation stream it will count the sum of RTP packets received from all the component streams.

When statistics are set at the termination level on a termination that has an aggregation stream the effect will be that it is only set against the aggregation streams and streams not involved in an aggregation.

Figure 6 – Statistics on an aggregation stream with ANAT semantic

Example:

Figure 6 shows an example where a Context has a termination Ta that handles three audio streams with identities 1, 2 and 3 grouped with the BUNDLE semantic. The termination has in addition another media stream with identity 4, not part of the media group. The MGC has defined an aggregation stream with identity 5. The MGC wishes to collect statistics on the data volumes sent and received in the bundled group at different levels (network, transport, application). It uses the Network Package os and or [ITU-T H.248.1], the IP Layer Octets Counts Statistics Package ipos, ipor, ipps and ippr [ITU-T H.248.61] and the RTP Application Data Package payloados and payloador [ITU-T H.248.58] statistics. It requests these statistics by including them in the statisticsDescriptor of the streamDescriptor of stream 5. The MG should collect statistics in each of the three component streams and sum up the values.

#### 8.6.5.4	Topology and aggregation streams

An aggregation stream allows the MG to transfer the media from/to several component media streams into/‌from one single stream or a different set of component media streams

The requested behaviour from the MG depends on the grouping semantics but it will usually involve some dynamic switching of the connectivity between the terminations included in the topology command based on external conditions. This dynamic switching will not be controlled by the MGC.

Example 1:

Figure 7 illustrates a scenario where a context connects two terminations. One of them has three audio streams: stream with identity 1 (sendrecv) uses PCM A-law, stream with identity 2 (sendrecv) uses AMR and stream with identity 3 (sendonly) uses PCM A-law. The three streams are grouped with the FID [IETF RFC 5888] semantics. The other termination has one single stream with two payload types, PCM A-law and AMR.

Figure 7 – Interworking between grouped streams and single stream

In order to connect the set of streams at Ta with the single stream at Tb, the MGC defines stream identity 4 as an aggregation stream on termination Ta. It then sets stream identity 4 on termination Tb. Terminations Ta and Tb are then by default bothway connected through stream 4 (Figure 9).

Figure 8 – Example of a multiple stream aggregation into a single stream

NOTE – The direction attributes shown in the figure correspond to the MG terminations’ views.

P1 (Figure 7) can change the codec dynamically to send data. According to the FID semantics, when PCM encoded audio is received externally in Tb, Ta forwards it to P2 in stream 1 and as a replica in stream 3 –it may be with a different IP destination address if specified like that in the Remote Descriptor- and stream 2 remains muted. When AMR encoded audio is received in T1, T2 forwards it to P2 in stream 2 while stream 1 and stream 2 remain muted.

When PCM encoded audio is received externally in Ta in stream 1 it is forwarded to Tb via stream4. When AMR encoded audio is received externally in Ta stream 2 it is forwarded to Tb via stream 4.

It is possible for a stream to be connected to other Terminations directly of via an aggregation stream.

Figure 9 – Example of invalid direct stream and aggregation stream connection between two terminations

It is not possible for a normal stream to be directly connected to other terminations in a Context and be connected at the same time to the same termination through an aggregation stream. Figure 9 shows an example of such a case where two terminations are connected via a normal stream (stream identity 1) and an aggregation stream (stream identity 4).

There may be multiple aggregation streams on a Termination. A stream cannot be a component of more than one  aggregation stream if the aggregation streams are connected between the same terminations, i.e. it is not possible for a stream to be connected to another termination through more than one aggregation stream. However a component can be source of multiple aggregation streams if at most one (either the component or one of the aggregation streams) is connected to another specific termination. This can be the case e.g. with aggregation streams defined for statistics gathering, sending signals or detecting events, which can have an isolated topology from other terminations.

The MGC can modify topology related to an aggregation stream by using its identity in a topology descriptor. For example using figure 8 the topology may be set Topology(Ta,Tb,oneway,Sid4), Topology(Tb,Ta, Oneway, Sid4), Topology(Ta,Tb, isolate, Sid4,) or Topology(Ta,Tb,bothway, Sid4).

### 8.6.6	Usage of stream de-aggregation

In some grouping scenarios a single media stream on entering the context may be de-aggregated into multiple component media streams and, in the other direction, may result from the reverse operation of merging multiple component media streams into a single media stream.

The mgroup package defines the concept of a de-aggregation Stream. By setting the mgroup/strdeagg property on a H.248 Stream indicating the component streams it allows stream de-aggregation to be modelled internally in the MG. Media is then de-aggregated to/from the de-aggregation stream to the component streams according to the applicable grouping semantics.

The mgroup/strdeagg property may be set on multiple H.248 streams on a Termination. However the component H.248 StreamIDs shall only be assigned to one de-aggregation stream. If the MGC assigns a component H.248 StreamID to more than one mgroup/strdeagg property on a Termination then error code 489 “Invalid aggregation and/or de-aggregation” shall be returned by the MG.

Figure 10 illustrates the concept.

Figure 10 – De-aggregation stream

In figure 10 media stream labelled media 1 described by Stream Descriptor Sid1 will be de-aggregated into components 1a, 1b and 1c according to the groupse semantic for those streams. This de-aggregation will be sent via streams Sid2, Sid3 and Sid4 to Termination Tb. Streams Sid2, Sid3 and Sid4 are the component streams. They are then sent externally from the media gateway as described by the Stream descriptor for Streams Sid2, Sid3 and Sid4 on Termination Tb.  The MG may apply any interworking/adaptation etc. functions that would normally apply when the same media stream is set on two Terminations. The de-aggregation stream may also remain isolated to one Termination in a Context.

The behaviour of the de-aggregation stream is determined by the grouping semantics and the media descriptor of de-aggregation streams and any media descriptors of the component streams. If set the values of the properties in the component streams should be compatible with those defined in the de-aggregation stream and in no case should override any value set in the de-aggregation stream.

When entering or leaving the context through the de-aggregation stream, the component streams have a send and receive relationship with the de-aggregation stream. Thus the stream mode does not need to be set in the component stream. If set, it will be ignored by the MG.

A MGC may apply Signals, Events, Statistics and Topology to the de-aggregation and component streams. The behaviour is discussed in more detail in the sub-clauses below.

#### 8.6.6.1	Signals and de-aggregation streams

A Signal applied to a de-aggregation stream will have the effect of being played out externally. Its effect is dependent on the semantic of the signal. A Signal applied to the internal direction will have the effect of playing on each of the component streams.

A Signal applied to a component stream will have the effect of being played out externally on the de-aggregation stream in the part of that the component relates to.

#### 8.6.6.2	Events and de-aggregation streams

An Event applied to the component stream will have the effect of detecting events as a result of stimulus usually from outside the context only on the component of the de-aggregation. How the Event is reported is based on the grouping semantic associated with the de-aggregation stream. Events may be specified on the de-aggregation streams. Detection is based on the de-aggregation stream has a whole or any of the components of the stream.

Where an Event is set at the Termination level on a Termination that has a de-aggregation stream the effect will be that it is only set against the de-aggregation stream and streams not involved in a de-aggregation.

#### 8.6.6.3	Statistics and aggregation streams

When Statistics are set against a de-aggregation stream, the statistics are collected as if they had been set against all the component streams. If statistics are set against a component stream, the statistics are collected only for that component of the de-aggregation stream.

When statistics are set at the Termination level on a Termination that has a de-aggregation stream the effect will be that it is only set against the de-aggregation streams and streams not involved in a de-aggregation.

#### 8.6.6.4	Topology and de-aggregation streams

As per clause 8.6.5.4.

#### 8.6.6.5	Principle use cases of stream aggregation and deaggregation

Figure 11 illustrates the principle models for use case stream aggregation (1) and stream deaggregation (2).

Figure 11 – Half context model – Principle use cases of stream aggregation (1) 
and stream de-aggregation (2)

### 8.6.7	Interaction of stream grouping methods "aggregation" and "de-aggregation"

{Editor’s note: Further investigation is required to analyse if cascading of aggregation and de-aggregation is of use and allowed by the model. The current model presents the following difficulty for such a cascading: components streams of a de-aggregation stream are internal to the context, they don’t’ exist externally whereas components of an aggregation stream are external to the context.A de-aggregation stream represents an external media stream. As such the component streams of the de-aggregation are a representation of part of the external media stream. These component streams may form an input into an aggregation stream forming a cascaded de-aggregation to aggregation configuration. This allows an external media stream to be de-aggregated according to one semantic and then aggregated according to another semantic. Figure 11 shows an example configuration.

Figure 12 – Cascaded de-aggregation to aggregation

it must be analysed if a configuration where a de-aggregation stream is a component of an aggregation stream is valid or not. For example adding Sid1 as a component to the mgroup/stragg property in the above figure. In principle, the current model would not preclude it, since the de-aggregation stream is an external stream which could be grouped with other external streams into an aggregation stream. However, no use case has been identified so far.

The  mgroup/stragg and mgroup/strdeagg properties shall not be set on the same H.248 Stream. If the MG receives a Local Descriptor with both the mgroup/stragg and mgroup/strdeagg properties set it shall return error code 473 “Conflicting Property Values”.}

### 8.6.8	Stream Level Interactions

{Editor’s note: There may be H.248 stream level interactions, i.e., the contents between Stream Descriptors of component streams and the Stream Descriptor of aggregation / de-aggregation streams could overlap, leading to semantical interaction issues. Ee.g. with property StreamMode, there might be following valid use cases:

UC#1 ("stream group global settings"): StreamMode set on (de-)aggregation stream level and omitted at component stream level:
=> semantic: all component streams inherit the setting from the (de-)aggregation stream;

UC#2 ("component stream local settings"): StreamMode set on component stream level and omitted at (de-)aggregation stream level:
=> semantic: all component streams could apply individual stream modes;

Interaction issues: IF StreamMode is signalled at both stream levels AND IF StreamMode settings would contradict.

This is a concrete example for one element of a Stream Descriptor, however, in general it could affect all elements of Stream Descriptors. For further studies..}

Annex A

Codepoints for parameter Semantics of H.248 property Group Semantics

(This annex forms an integral part of this Recommendation.)

## A.1	Introduction and purpose

Property mgroup/groupse (clause 8.1.1) defines essentially a list of H.248 StreamID values which is qualified by semantics according to the Semantics (SDP) parameter as defined by [IETF RFC 5888]. The purpose of this Annex is to summarize IANA registered codepoints as well as proposed codepoints by IETF working documents and ITU-T defined codepoints. This information should be reviewed and updated for each Release of this Recommendation.

## A.2	IETF defined codepoints

IANA registered values for Semantics see [b-IANA group semantics]. Table A.1 provides a copy of this information, augmented with additional, yet unregistered codepoint information:

Table A.1 – Codepoints – Semantics for the "group" SDP Attribute (status: 12/2014)

| Semantics | Token | Reference | IANA registered |
| --- | --- | --- | --- |
| Alternative Network Address Types | ANAT | [IETF RFC 4091] | Yes |
| Bundling | BUNDLE | [ETF BUNDLE] | Not yet |
| Controlling multiple streams for telepresence | CLUE | [b-IETF CLUESIG] | Not yet |
| Composite Session | CS | [b-IETF FLUTE] | Yes |
| Decoding Dependency | DDP | [IETF RFC 5583] | Yes |
| Duplication | DUP | [IETF RFC 7104] | Yes |
| Flow Identification | FID | [IETF RFC 5888] | Yes |
| Forward Error Correction (Deprecated) | FEC | [IETF RFC 5956] | Yes |
| Forward Error Correction FR | FEC-FR | [IETF RFC 5956] | Yes |
| Lip Synchronization | LS | [IETF RFC 5888] | Yes |
| Single Reservation flow | SRF | [IETF RFC 3524] | Yes |

## A.3	ITU-T defined codepoints

Table A.2 summarizes codepoints which are specific to H.248-based gateway control:

Table A.2 – Codepoints – Semantics for the "group" SDP Attribute

| Semantics | Token | Reference | IANA registered |
| --- | --- | --- | --- |
| SCTP Stream De-aggregation | SCTP | [ITU-T H.248.SCTP] | No |

Bibliography

[b-IANA group semantics]	IANA registry for "Semantics for the "group" SDP Attribute".
URL: http://www.iana.org/assignments/sdp-parameters/sdp-parameters.xhtml#sdp-parameters-13

[b-IETF RFC 4091]	IETF RFC 4091 (07/2005), The Alternative Network Address Types (ANAT) Semantics for the Session Description Protocol (SDP) Grouping Framework.

[b-IETF CLUESIG]	IETF draft-ietf-clue-signaling (xx/xxxx), CLUE Signaling.

[b-IETF FLUTE]		IETF draft-ietf-rmt-flute-sdp-03 (09/2012), SDP Descriptors for FLUTE.

Appendix L
Living List – H.248.STGROUP Possible use cases

(This appendix only contains working material.)

## L.1	Use case #1: alternate speech-text telephony

### L.1.1	Use case #1.1: alternate speech-text telephony in IP-to-IP gateways (without RTP bundling)

#### L.1.1.1	High-level description of the gateway interworking service

The alternate speech-text telephony communication (see e.g. [b-ITU-T F.703], [b-ITU-T F.790], [b-ETSI ETR 333], [b-ETSI EG 202 320]) is a regulated service, e.g., as defined for 3GPPs global text telephony (GTT) service ([b-ETSI TS 122 226], [b-ETSI TS 123 226]). That service has the peculiarities that at least one communication party [b-ITU-T AVA TR2] is using audio and text quasi in parallel, but only one media format at a particular point in time and also only in unidirectional manner (e.g., a mute-only person gets audio in receive only mode and text in send only mode, or deaf-only person a vice versa configuration). Figure L.1 illustrates combinations of interest in this Appendix, i.e., one termination with audio and embeed text as voiceband data (using [b-ITU-T V.18] modem), and one termination with separate stream endpoints for native VoIP (audio) and ToIP (text). Again, combinations when one communcation party could use audio or text in bidirectional manner is out of scope because effectively leading to an audio-only or text-only call. The H.248 context model in scope are combinations with unidirectional audio and text (Figure L.1).

This use case is hence a good example for demonstrating stream grouping with scope on

- stream aggregation: two unidirectional traffic flows (carried in bidirectional H.248 media streams) are combined to a single bidirectional one; and

- directionality token usage in property groupse.

Figure L.1 – Use case #1.1: alternate speech-text telephony in IP-to-IP gateways 
– Traffic flow models

The basic question relates to the modeling of audio and text at the left hand side, e.g., whether both could be merged into a single H.248 stream? This is basically possible by usage of the H.248 ReserveGroup property and the definition of two H.248 media groups. There are deficiencies with this model, primarily that both media groups share all common stream endpoint characteristics such as traffic directionality configuration (StreamMode). Other constraints are stream-level statistics. Therefore an enhanced context model based on stream grouping is discussed.

The IP application protocol for H.248 stream endpoint T2(S3) is [b-ITU-T V.152], which relates to a dual mode VoIP / VBDoIP configuration (with an audio mode and VBD mode). The example media formats are

- V.152 audio mode: G.711 µ-law without silence suppression

- V.152 VBD mode: G.711 µ-law as VBD codec plus RTP packet redundancy in order to provide an assured transport service (see clause 6.3.2/[b-ITU-T V.152]).

#### L.1.1.2	Selected stream grouping model

See Figure L.2.

Figure L.2 – Use case #1.1: alternate speech-text telephony in IP-to-IP gateways 
– Context stream grouping model

One call leg is supposed to be located in an IMS (IP multimedia subsystem) domain, and the other call led in a PES (PSTN emulation subsystem) domain.

Desired MG behaviour:

- If SEP TC(S2) receives an AMR/RTP packet, then the AMR encoded voice sample is transcoded to a G.711 encoded voice sample that then SEP TA(S1) transmits an RTP packet with RTP "PT = 0".

- If SEP TC(S3) receives a T.140/RTP packet, then the encoded text is transformed into a corresponding series of baudot tones, which is G.711 encoded and packetized into a series of G.711/VBD/RTP packets (based on [b-ITUT-T V.152], which then are transmitted by SEP TA(S1) within RFC 2198 ("PT=102") RTP packets.

- If SEP TA(S1) receives a G.711/RTP packet with encoded voice sample, then this voice sample is AMR encoded and a corresponding AMR/RTP packet is transmitted by SEP TC(S2). Such an IWF relates to "audio transcoding",

- If SEP TA(S1) receives an RFC 2198 RTP packet ("PT=102"), then it's extracts the contained "PT=98" VBD payloads, then it interprets the VBD payload as a fraction of text part encoding baudot tone and tries to transform the baudot tone into the corresponding text characters (several VBD payloads may need to be concatenated before doing this), then it transforms these text characters into the corresponding T.140 format, which is then encoded and packetized into T.140/RTP packets as per RFC 4103, which then are transmitted by SEP TC(S3). Such an IWF relates to "text transcoding".

Thus, termination TC would have to "know" whether to use its H.248 stream S2 or its S3 for transmission of an RTP packet. This would depend on the payload type format of the RTP packet(s) TA(S1) receives and on the payload type(s) which the IWF uses when generating RTP packets. Hence, the MG is expected to aggregate streams according to the "FID" semantic.

Furthermore, termination TA implies the usage of a single H.248 stream in "dual mode" usage, which is inherently satisfied by V.152 (and its two state model of "audio mode" and "VBD mode").

#### L.1.1.3	Overall signalling scenario (PES-to-IMS call)

The following message sequence diagram (Figure L.3) shows the messages and message numbering scheme which is used by all example message flows in this use case, which are related to the PES network  domain being the communication session originator.

Figure L.3 – Overall message sequence diagram for a call in PES to IMS direction

There is call control signalling between the PES domain and the MGC (1, 8), and the IMS domain and the MGC (4, 5, 9, 12). Such call control signalling would be SIP based, but the specific call control protocol syntax is out of scope here.

There is the basic assumption in above scenario that the context C1 is initially created and prepared for audio-only, - and extended to additional text support after a correspondent update from IMS side. The rationale behind is the expected distribution between "audio only" and "audio & text" calls (e.g., an estimated propability of 99.99 % of type "audio only").

#### L.1.1.4	Signaling steps

Notes:

- Some GTT independent SDP attributes are not shown in the following example messages. Such additional SDP attributes would typically be present in Local and Remote descriptors. Also, some GTT independent LocalControl descriptor properties are deliberately not shown.

- Events and Signals descriptors, which would potentially be used in some cases, are also not shown.

Step 1: PES call control signalling (e.g., based on SIP-I)

- request for a "speech" call

Steps 2 and 3: H.248 transaction request/reply {Create Ctxt, Add terminations "IP_A" and "IP_C"}

See Table L.1.1.

Table L.1.1 – Example H.248 signalling for steps 2 and 3

| H.248/SDP encoding (shortened H.248 Media Descriptor) | Comments: |
| --- | --- |
| Transaction = 1 { Context = $ { Add = ip/$/$/$ { ; "PES termination" Media { Stream = 1 { LocalControl { Mode = SendReceive, ReservedValue = ON ; NOTE 1 }, Local { rtpt/rtptopo = PP ; B2BRE topo enforced ; start of SDP-defined H.248 media group v=0 c=IN IP4 $ m=audio $ RTP/AVP 0 98 102 a=rtpmap:98 PCMU/8000 a=gpmd:98 vbd=yes a=rtpmap:102 red/8000 ; packet redun. a=fmtp:102 98/98/98/98 a=… }, Remote { rtpt/rtptopo = PP ; B2BRE topo enforced v=0 c=IN IP4 <PES_IP_addr_audio> m=audio <PES_port_audio> 0 98 a=rtpmap:98 PCMU/8000 a=gpmd:98 vbd=yes a=rtpmap:102 red/8000 ; packet redun. a=fmtp:102 98/98/98/98 a=… }}}}, Add = ip/$/$/$ { ; "IMS termination" Media { Stream = 1 { LocalControl { Mode = SendReceive, ReservedValue = ON }, Local { rtpt/rtptopo = PP ; B2BRE topo enforced v=0 c=IN IP6 $ m=audio $ RTP/AVP 103 0 13 100 a=rtpmap:100 telephone-event/8000 a=fmtp:100 0-15 a=rtpmap:103 AMR/8000/1 a=fmtp:103 mode-set=0,2,5,7 a=ptime:20 a=silenceSupp:on - - - - }}}}}} | Initial speech only configuration with media formats AMR at IMS side and G.711 µ-law at PES side. Hence, the MG is requested for an interworking function "audio transcoding". The PES termination is additional enabled for potential VBD, i.e., an - audio configuration 'PCMU' and - VBDoIP configuration with 'PCMU' as VBD codec. |
| Reply = 1 { …} | positive acknowledgement |
| NOTE 1 – The V.152 dual mode configuration implies a ReserveValue 'true' setting. |  |

Steps 4 and 5: SIP call control signalling – SDP offer/answer cycle

- request for a "audio-only" call with AMR as codec which is confirmed by IMS UE

Steps 6 and 7: H.248 transaction request/reply for completion of the initial audio-only context configuration

Step 8: PES call control signalling – positive response

The IMS terminal decides (e.g., based on configuration data) to change from audio-only to parallel usage of audio and text, leading to a SIP UPDATE or re-INVITE request (step 9).

Step 9: SIP call control signalling – SDP offer

- with an additional media description for a media type 'text'.

Steps 10 and 11: H.248 transaction request/reply {modification of termination "IP_C"}

See Table L.1.2.

Table L.1.2 – Example H.248 signalling for steps 10 and 11

| H.248/SDP encoding (shortened H.248 Media Descriptor) | Comments: |
| --- | --- |
| Transaction = 3 { Context = <C1> { Modify = <IP_C> { ; NOTE 1 TerminationState { mgroup/groupse = ["FID 2 3"] {Editor's note (2015-02): there is a question for clarification whether the groupse "Direction" parameter might be useful (due to unidirectional traffic flows)? And IF, how would the streams individually be indicated if eg. '2' would be sendonly and '3 would be receive only? For further studies.} }, Media { Stream = 1 { LocalControl { mgroup/stragg = [2, 3], ReservedValue = ON }, Local {}, Remote {} }, Stream = 2 { LocalControl { Mode = SendReceive, ReservedValue = ON }, Local { rtpt/rtptopo = PP ; B2BRE topo enforced v=0 c=IN IP6 $ m=audio $ RTP/AVP 0 103 100 a=rtpmap:103 AMR/8000/1 a=rtpmap:100 telephone-event/8000 a=fmtp:103 mode-set=0,2,5,7; max-red=0 a=ptime:20 a=maxptime:30 a=silenceSupp:off - - - - }, Remote { rtpt/rtptopo = PP ; B2BRE topo enforced v=0 c=IN IP6 <IMS_IP_addr_audio> m=audio <IMS_port_audio> RTP/AVP 0 110 100 a=rtpmap:110 AMR/8000/1 a=rtpmap:100 telephone-event/8000 a=fmtp:110 mode-set=0,2,5,7 a=ptime:20 a=maxptime:30 a=silenceSupp:off - - - - } }, Stream = 3 { LocalControl { Mode = SendReceive, ReservedValue = OFF }, Local { rtpt/rtptopo = PP ; B2BRE topo enforced v=0 c=IN IP6 $ m=text $ RTP/AVP 120 121 a=rtpmap:120 red/1000/1 a=fmtp:120 121/121/121 ; NOTE 2 a=rtpmap:121 t140/1000/1 }, Remote { rtpt/rtptopo = PP ; B2BRE topo enforced v=0 c=IN IP6 <IMS_IP_addr_text> m=text <IMS_port_text> 120 121 a=rtpmap:120 red/1000/1 a=fmtp:120 121/121/121 ; NOTE 2 a=rtpmap:121 t140/1000/1 } }}}}} | MGC adds TerminationState descriptor containing the mgroup/groupse property in order to request the MG to group the two new SEPs 2 and 3 with semantic "FID" as per RFC 5888. Stream S1: MGC changes this SEP to the aggregating SEP. This allows keeping SEP <IP_A>.S1 without any modifications. MGC sets RV = ON in order to avoid SEP 1 being deleted. MGC replaces Local and Remote with empty descriptors, as the aggregation SEP does not need any own Local or Remote descriptor content and especially does not have an own externally visible transport address. Stream S2: MGC creates SEP 2 which takes over the previous role of <IP_C>'s SEP 1 (audio media towards IMS). However, new SEP 2 is created with choose wildcards in the Local descriptor, as the MG does not support moving SEP 1's transport address to SEP 2. Stream S3: New SEP 3 for the new text media component towards IMS. |
| NOTE 1 - In Step (10) the MGC does not modify the PES side termination <IP_A>.Thus especially it's SEP 1 remains PCMU-based for audio and VBD signals. But due to the modification of <IP_C>'s SEP 1 to an SEP aggregating the new SEPs 2 and 3, and due to the new SEPs 2 and 3 describe audio and RTT text based media components, respectively, the MG now knows that it needs to insert an interworking function into the internal data path between <IP_A>'s SEP 1 on the one hand side and <IP_C>'s SEPs 2 and 3 on the other hand side. The IWF provides: - audio transcoding between AMR and PCMU; - text transcoding between RFC 4103 based RTT to text/baudot/PCMU/VBD; and - "mixing" of the two internal PCMU-based streams to PCMU/RTP/UDP/IP packets in PES direction. NOTE 2 – The example shows packet redundacy levels of two at SEP TC(S3) for native text-over-RTP and of three at SEP TA(S1) for text in VBD encoding (see Table L.1.1.). The higher redundancy might be justified by the different encoding schemes or/and different GoS conditions in the two IP domains. |  |
| Reply = 3 { …} | positive acknowledgement |

Step 12: SIP call control signalling – SDP answer

- completion of call establishment

#### L.1.1.5	Observations

Figure L.3 summarizes again the transition from the initial audio-only context to additional text support.

Figure L.3 – Use case #1.1: summary of the example evolution of the considered context stream grouping model

Steps 9 to 12 are only executed for the rare cases of additional text support. The H.248 "PES" termination would be then not affected by a "context reconfiguration".

The aspect of "RTP topology" [b-ITU-T H.248.88] should be carefully considered:

- Phase 1 "audio only" (steps 1 to 8):

- either B2BRE or RTPMT topology (due to enforced audio transcoding);

- Phase 2 "additional text" (steps 9 to 12):

- The RTP topology between the three external H.248 media streams looks like an "RTP mixer", but the overall communication topology relates still to an two-party call only, underlined by the fact that there are only two H.248 terminations and the H.248 topology descriptor is not used.

- The ultimate requirement for RTCP is the isolation of RTCP control flows to their RTP session legs only (from an end-to-end perspective). Thus, any incoming RTCP report needs to be fully terminated in its H.248 SEP. Such a behaviour is only guaranteed by a B2BRE topology configuration, which is therefore explicitly enforced on each stream and each traffic direction.

- The RTP topology (in the H.248 MG) should not change (tbc) by a transition from phase 1 to phase 2 from an end-to-end communication perspective. That's the reason why the B2BRE topology was already explicitly enforced in both phases.

{Editor's note (2015-02): the rtpt/rtptopo property is located at LD-/RD-level (and not at LCD level) (see H.248.88), which might be in conflict with the LD-/RD-embedded SDP lines for media description, hm?}

### L.1.2	Use case #1.2: alternate speech-text telephony in IP-to-IP gateways with RTP bundling

Use case #1.2 provides RTP bundling in the IMS domain in addition to use case #1.1.

For further studies:

- bundling: shared IP interface (network address only) between TC(S2) and TC(S3)
=> what kind of information needs to be added to TC(S1)  description?

- "groupse" semantic 'FID': what about LIP or BUNDLE?

### L.1.3	Use case #1.3: additional performance monitoring

The transition from initial audio-only mode to "global text telephony" mode could demand for correspondent performance monitoring support (e.g., due to a regulated service).

In order to minimize cost, the number of H.248 statistics might be reduced by consideration of media stream specific characteristics. For instance by support of stream-level statistics.

a) Number of H.248 statistics: at least one performance metric is selected per media type (audio & text) in order to characterize the performance of the media stream.

b) Type of H.248 statistic: RTP packet loss for audio in order to get feedback about Grade of Service, and RTP payload traffic volume in octets for text as a measure for "characters". Furthermore, a performance metric such as information loss isn't really helpful for media type "text" due to enabled packet redundancy mode.

Hence, in order to reduce the cost of performance monitoring to the absolute minimum, following stream-level statistics might be considered ("the Statistics descriptors would be inserted in the H.248 transaction request in step 10"):

Statistics{  ; Stream 2 = audio

rtp/pl,     ; "RFC3550 packet loss" ingress

recrtcp/rpl ; "RFC3550 packet loss" egress

}

Statistics{  ; Stream 3 = text

rtpad/payloados, ; "T.140 text" octets sent

rtpad/payloador  ; "T.140 text" octets recv.

}

## L.2	Use case #2: WebRTC

### L.2.1	Use case #2.1: WebRTC with RTP media multiplexing and separated H.248 streams for WebRTC DTLS session and underyling IP transport connection

The user plane protocol stack for WebRTC (see also clause 6.3/[b-ITU-T H.248.WEBRTC]) represents a hierarchical layered protocol architecture with multiplexing options at various level. The general WebRTC protocol stack mandates the usage of stream grouping capabilities in order to model such an H.248 termination in H.248 WebRTC gateways. The first example (#2.1) tries to model the most complex scenario, constituted by a maximum degree of multiplexing as well as multiple, dynamic WebRTC data channels. The considered WebRTC communication service is characterized by

- usage of RTP / RTCP transport multiplexing;

- usage of RTP media multiplexing (aka RTP SSRC multiplexing);

- usage of a single IP transport connection (UDP or TCP) for all WebRTC application flow components;

- usage of a single DTLS session for a) DTLS-SRTP based key management for SRTP and b) DTLS transport of SCTP protocol data units; and

- multiple WebRTC data application instances.

The WebRTC protocol stack and its traffic flows could be mapped on a number of H.248 streams (see Figure L.4) using following stream grouping principles:

- shared use of an (RTP)-connection for bundling audio and video;

- shared use of an (SCTP)-association for WebRTC data channels; and

- protocol stack segment realizing the tunnel endpoint (DTLS/L4).

{Editor's note (2015-02): there is an ongoing discussion about the set of entities which constitute actually the "tunnel endpoint" (TEP). There might be three meanigful solutions:

S1: TEPS1 = "DTLS protocol" only,
justified by the fact that the tunnel layer start above the native stack (L.e., "L4/IP/…") and ends before the served user instance of the tunnel, which would be the SCTP (because SCTP provides the multiplexing option for DCs based on the SCTP stream concept; or

S2: TEPS2 = "DTLS/L4 protocol stack segment",
due to the native coupling of the transport level security protocol DTLS with the transport layer; or

S3: TEPS3 = "(SCTP)-association/(DTLS)-session/(L4)-connection" (using a X.200 type of convention in denoting layer-dependent "protocol objects"),  justified by the fact that the X.200 convention allows to model the SCTP (layer) in the sub-entities of (SCTP)-association-endpoint and (SCTP)-stream-endpoint objects, which could be mapped on separated H.248 SEPs. Hence we would get a more fine granular object model for the "WEBRTC TEP" as located at H.248 WEBRTC gateway side.

Discussion is ongoing. The present text denotes the tunnel according to solution S2.}

The first two grouping principles are rather straighforward because a single protocol (RTP, SCTP) is used for multiple, separated application traffic flows. And the actual application flows are mapped on individual H.248 SEPs and the protocol layer used for aggregation gets a separate (but optional) SEP assignment.

The 3rd list item isn't straightforward because conditional, primarily dependent on NAT traversal aspects (see clause L.5 on "SCTP model").

Figure L.4 – Use case #2.1: WebRTC with RTP media multiplexing and separated H.248 streams for WebRTC DTLS session and underyling IP transport connection 
– Context stream grouping model

Figure L.4 higlights already the usage of the three mgroup properties. There are three SEPs which serve stream deaggregation (S8, S9 and S10) and one SEP for stream aggregation (S3).

Table L.2.1 illustrate example H.248 signalling with regards to the initial creation of the context and all four stream endpoints:

Table L.2.1 – Example H.248 media descriptor

| H.248/SDP encoding (shortened H.248 Media Descriptor) | Comments: |
| --- | --- |
| ; Termination T1 Context = C1 { ; Context C1 Add = ip/$/$/$ { ; Termination T1 Media{ TerminationState { mgroup/groupse = ["BUNDLE 1 2", "WRTC-DC 4 5 6 7", "SCTP-TRANSPORT 8", "DTLS-TRANSPORT 9"] ; Semantics "WRTC-DC", "SCTP-TRANSPORT", ; "DTLS-TRANSPORT are not yet specified; }, Stream = 1 { ; Audio/SRTP LocalControl { ipdc/realm = <webrtc_realm>, rtcph/rsb = On, ; transport multiplexed RCTP Mode = SendReceive, ReservedValue = On, ... }, Local { v=0 c=IN IP6 $ m=audio $ UDP/TLS/RTP/SAVP 101 0 13 126 a=rtcp-mux a=rtpmap:101 opus/48000/2 a=rtpmap:0 PCMU/8000 a=rtpmap:13 CN/8000 a=rtpmap:126 telephone-event/8000 a=ssrc:$ cname:$ a=ice-ufrag:$ a=ice-pwd:$ a=fingerprint:$ $ }, Remote { v=0 c=IN IP6 <UE_IP_BUNDLE> m=audio <UE_port_BUNDLE> UDP/TLS/RTP/SAVP 101 0 13 126 a=rtcp-mux a=rtpmap:101 opus/48000/2 a=rtpmap:0 PCMU/8000 a=rtpmap:13 CN/8000 a=rtpmap:126 telephone-event/8000 a=ssrc:<UE_SSRC_audio> cname:<UE_CNAME> a=ice-ufrag:<UE_ICE_ufrag_BUNDLE> a=ice-pwd:<UE_ICE_pwd_BUNDLE> a=fingerprint: <UE_hash_func_BUNDLE> <UE_fingerprint_BUNDLE> } }, Stream = 2 { ; Video/SRTP LocalControl { ipdc/realm = <webrtc_realm>, rtcph/rsb = On, ; transport multiplexed RCTP Mode = SendReceive, ReservedValue = On, ... }, Local { v=0 c=IN IP6 $ m=video $ UDP/TLS/RTP/SAVPF 96 97 a=rtcp-mux a=rtpmap:96 VP8/90000 a=rtpmap:97 H264/90000 a=ssrc:$ cname:$ a=ice-ufrag:$ a=ice-pwd:$ a=fingerprint:$ $ }, Remote { v=0 c=IN IP6 <UE_IP_BUNDLE> m=video <UE_port_BUNDLE> UDP/TLS/RTP/SAVPF 96 97 a=rtcp-mux a=rtpmap:96 VP8/90000 a=rtpmap:97 H264/90000 a=ssrc:<UE_SSRC_video> cname:<UE_CNAME> a=ice-ufrag:<UE_ICE_ufrag_BUNDLE> a=ice-pwd:<UE_ICE_pwd_BUNDLE> a=fingerprint: <UE_hash_func_BUNDLE> <UE_fingerprint_BUNDLE> } }, Stream = 3 { ; Ctxt internal aggregation stream ; for statistics LocalControl { ipdc/realm = <webrtc_realm>, Mode = SendReceive, ## ; Stream mode intentionally set to SendReceive ; in order to activate statistics counting. ; This was clarified in the H.248.1 IG. mgroup/stragg = [1,2] ... }, ; Assumption: Local and Remote descriptors ; are not needed, and hence are left ; unspecified. Statistics { ; traffic volume in octets rtpad/payloados, ; for audio & video at app- rtpad/payloador ; lication-level } }, Stream = 4 { ; Text/T.140/DC LocalControl { ; Mode = SendReceive, ... }, Local { v=0 c=- - - m=application – DTLS/SCTP webrtc-datachannel ; How do we know that this SEP does not ; model L4 and DTLS connection endpoints? ; Should we just use "SCTP" as <proto> value? ; But this is already defined in ; draft-ietf-mmusic-sctp-sdp for the ; plain SCTP-over-IP case. ; Above m-line does not contain a UDP port, ; and the c-line also does not contain any ; L3 information. ; Also, there is no further DTLS related ; information, like fingerprint value. ## ; Potentially this could be used as indication ## ; that no L4 and DTLS connection endpoints are ; modeled by this SEP. ; How do we know that this SEP does also ; not model the SCTP association endpoint? ; Because it is part of the SEP list ; associated with the "WRTC-DC" semantic in ; T1's groupse value. ; The fmtp attribute is not present, as its ; only parameter is the "max-message-size" ; parameter. However, this is an SCTP ; association level parameter, which occurs ; just once in S8, which models the SCTP ; association endpoint. a=dcmap:0 subprotocol="text-t.140" ; Do we need a subprotocol value at all? ; Assumingly yes, if we want to use the ; "data channel" m-line above. ; In this case the subprotocol value ; would need to unambiguously define ; the exact sub-protocol and application ; protocol formats. Proposing "text-t.140". ; "text-t.140" is not defined so far. ; The dcmap attribute does not contain ; further data channel parameters. ; This especially implies ordered and ; reliable transmission, such that no ; other redundancy schemes are required. ; Similar to ; draft-ejzak-mmusic-msrp-usage-data-channel ; a dedicated "T.140-usage-data-channel" ; document would be needed. }, Remote { v=0 c=- - - m=application – DTLS/SCTP webrtc-datachannel a=dcmap:0 subprotocol="text-t.140" } }, Stream = 5 { ; IM(text)/MSRP/DC LocalControl { ipdc/realm = <webrtc_realm>, Mode = SendReceive, ... }, Local { v=0 c=- - - m=application – DTLS/SCTP webrtc-datachannel a=dcmap:1 subprotocol="msrp" a=dcsa:1 path:msrps://<MG_authority_webrtc>/\ <Remote_session_chat>;dc ; Assumption: MG operates in MSRP BL ALG mode, ; thus it does not have an own MSRP session id ; such that the Local's path value references ; the MSRP session id used by the remote far ; end MSRP session endpoint. ; For the same reason no a=dcsa attribute ; is present containing the ; accept-types attribute. }, Remote { v=0 c=- - - m=application – DTLS/SCTP webrtc-datachannel a=dcmap:1 subprotocol="msrp" a=dcsa:1 path:msrps://<UE_authority>/\ <UE_session_chat>;dc } }, Stream = 6 { ; IM(file)/MSRP/DC LocalControl { ipdc/realm = <webrtc_realm>, Mode = SendReceive, ... }, Local { v=0 c=- - - m=application – DTLS/SCTP webrtc-datachannel a=dcmap:2 subprotocol="msrp"; a=dcsa:2 path:msrps://<MG_authority_webrtc>/\ <Remote_session_file_transfer>;dc ; Similar comments as to T1.S5 }, Remote { v=0 c=- - - m=application – DTLS/SCTP webrtc-datachannel a=dcmap:2 subprotocol="msrp"; a=dcsa:2 path:msrps://<UE_authority>/\ <UE_session_file_transfer>;dc } }, Stream = 7 { ; FloorCtrl/BFCP/DC LocalControl { ipdc/realm = <webrtc_realm>, Mode = SendReceive, ... }, Local { v=0 c=- - - m=application – DTLS/SCTP webrtc-datachannel a=dcmap:3 subprotocol="bfcp"; ; TBC - Further BFCP related attributes? ; Sub-protocol "bfcp" is not defined yet. ; The dcmap attribute does not contain ; further data channel parameters. ; This especially implies ordered and ; reliable transmission. ; Similar to ; draft-ejzak-mmusic-msrp-usage-data-channel ; a dedicated "bfcp-usage-data-channel" ; document would be needed. }, Remote { v=0 c=- - - m=application – DTLS/SCTP webrtc-datachannel a=dcmap:3 subprotocol="bfcp"; ; TBC - Further BFCP related attributes? } }, Stream = 8 { ; SCTP association LocalControl { Mode = SendReceive, mgroup/strdeagg = [4,5,6,7] ; deaggregation ... }, Local { v=0 c=- - - m=application – DTLS/SCTP webrtc-datachannel ; Similar comment as to T1.S4: ; How do we know that this SEP does not ; model L4 and DTLS connection endpoints? ; Should we just use "SCTP" as <proto> value? ; But this is already defined in ; draft-ietf-mmusic-sctp-sdp-08 for the ; plain SCTP-over-IP case. ; Above m-line does not contain a UDP port, ; and the c-line also does not contain any ; L3 information. ; Also, there is no further DTLS related ; information, like fingerprint value. ; Potentially this could be used as indication ; that no L4 and DTLS connection endpoints are ; modeled by this SEP. ; How do we know that this SEP does ; model the SCTP association endpoint? ; Because of its strdeagg value, which ; associates it to the SEP list in T1's ; groupse value, whose semantic is "WRTC-DC". a=fmtp:webrtc-datachannel \ max-message-size=$ a=sctp-port=$ ; Assumption: MGC lets MG choose local SCTP ; port and maximal message size. }, Remote { v=0 c=- - - m=application – DTLS/SCTP webrtc-datachannel a=fmtp:webrtc-datachannel \ max-message-size=<UE_SCTP_max_message_size> a=sctp-port=<UE_SCTP_port> } }, Stream = 9 { ; DTLS connection LocalControl { Mode = SendReceive, tlsbsc/bceb="unblocked", mgroup/strdeagg = [8] ; Seems we cannot use the seplink v1 (!) linktopo ; property in order to request T1.S8 to ; trigger SCTP association establishment as ; soon as the DTLS session / connection is ; establshed. Reason: The linktopo property ; allows for vertical interlinkage of ; different protocol layers, but only within ; the same SEP. But here SCTP is handled by ; S8, whereas DTLS is handled by S9. ; Potential seplink extension: Allow the ; linktopo property to optionally trigger ; establishment or release procedures on ; another H.248 stream (only within same ; termination or also in other ; termination(s)?). => see proposal C-685 ... }, Local { v=0 c=- - - m=- – DTLS/SCTP – ; Media format list intentionally agnostic ; as this SEP does not need to know ; anything regarding SCTP usage. ; Similar comment as to T1.S4: ; How do we know that this SEP does not ; model the L4 connection endpoints? ; Should we just use "DTLS" as <proto> value? ; Would be an H.248 specific <proto> value. ; Above m-line does not contain an L4 port, ; and the c-line also does not contain any ; L3 information. ; Potentially this could be used as indication ; that no L4 connection endpoint is ; modeled by this SEP. ; How do we know that this SEP does ; model the DTLS session or connection ; endpoint? ; Because of its strdeagg value, which ; associates it to the SEP list in T1's ; groupse value, whose semantic is ; "SCTP-TRANSPORT", and because its <proto> ; value indicates "DTLS/..." and the ; fingerprint attribute is present and valid. ; How do we know that this SEP does not model ; an SCTP association endpoint? ; This could also be derived from the above ; mentioned strdeagg value. (But just using ; "DTLS" as m-line <proto> value might make ; this clearer.) ; Media type is intentionally agnostic as this ; SEP does not need to know anything related ; to upper layer application. a=fingerprint:$ $ ; TBC – Do we need "setup"? If yes, is ; "actpass" the correct value (in order to ; indicate DTLS endpoint mode, see H.248.84)? ; H.248.93 re-uses .90's tlsbsc package, which ; contains the bcep property and the ; EstBNC signal for (D)TLS role specific ; signaling and processing ("package-full ; approach"). }, Remote { v=0 c=- - - m=- – DTLS/SCTP – a=fingerprint:<UE_hash_func_data> <UE_fingerprint_data> } }, Stream = 10 { ; L4 connection: UDP or TCP LocalControl { ipdc/realm = <webrtc_realm>, Mode = SendReceive, mgroup/strdeagg = [9] ... }, Local { v=0 c=IN IP6 $ m=- $ udp – ; Media type and media format agnostic, as ; this SEP does not need to know anything ; related to UDP payload formats. ; <proto> value "udp" as per RFC 4566. ; Assumption: ICE might also conclude with ; a TCP candicate pair, which would result ; in an SCTP/DTLS/RFC 4571/TCP/IP ; transport protocol stack. a=ice-ufrag:$ a=ice-pwd:$ }, Remote { v=0 c=IN IP6 <UE_IP_data> m=- <UE_port_data> udp – a=ice-ufrag:<UE_ICE_ufrag_data> a=ice-pwd:<UE_ICE_pwd_data> } } } }} | General comments: SDP "a=setup:" attribute: This SDP attribute could be used for multiple self-contained protocol layers (here TCP, DTLS and SCTP) from IETF side, e.g., in call control signalling. This is not possible here in case of an hierarchical layered protocol architecture due to the above mentioned semantical overloading of this SDP attribute. Therefore, the control of the bearer control procedure for establishment of a a) (TCP)-connection, b) (DTLS)-session/-connection, and c) (SCTP)-association is subject of other signalling elements (H.248 event/signal based control, or ICE-related SDP in case of TCP). Stream-specific comments: Stream 1: - audio with codec options OPUS and G.711 µ-law; - RTCP transport multiplexed (according to the rules of clause [b-ITU-T H.248.57]; Stream 2: - video with codec options VP8 and H.264; - RTCP transport multiplexed (according to the rules of clause [b-ITU-T H.248.57]; Stream 3: - represents an context-internal aggregation stream (for all RTP traffic); - allows a simple configuration related to the performance monitoring of the entire RTP traffic for audio and video; Stream 4: - WebRTC data channel with sub-protocol 'T.140' for WebRTC application flow "text conversation"; Stream 5: - WebRTC data channel with sub-protocol 'MSRP' for WebRTC application flow "instant messaging (text-based data)"; Stream 6: - WebRTC data channel with sub-protocol 'MSRP' for WebRTC application flow "instant messaging (file-based data)"; Stream 7: - WebRTC data channel with sub-protocol 'BFCP' for WebRTC application flow "conference/floor control)"; Stream 8: - represents an context-external aggregation stream (for all SCTP traffic); - the SEP represents the actual H.248 (SCTP)-bearer-connection-endpoint, i.e., handles all real SCTP packets with the MG-external network interface; and "forwards" the SCPT traffic to the SCTP-Stream-dependent MG-internal IWFs (as configured by H.248 streams S4 to S7); - thus, the usual protocol configuration of the (SCTP)-connection-endpoint is subject of this SEP; Stream 9: - SEP realizes the H.248 (DTLS)-bearer-connection-endpoint; - this SEP is used for a) DTLS-SRTP key exchange and b) DTLS application traffic 'SCTP'; - thus, represents an context-external aggregation stream (for all SCTP traffic); Stream 10: - SEP realizes the H.248 (L4)-bearer-connection-endpoint; |
| ; Termination T2 Context = C1 { ; Context C1 Add = ip/$/$/$ { ; Termination T2 Media{ TerminationState { ... }, Stream = 1 { ; Audio/RTP LocalControl { ipdc/realm = <core_realm>, Mode = SendReceive, ... }, Local { v=0 c=IN IP6 $ m=audio $ RTP/AVP 101 0 13 126 a=rtpmap:101 opus/48000/2 a=rtpmap:0 PCMU/8000 a=rtpmap:13 CN/8000 a=rtpmap:126 telephone-event/8000 }, Remote { v=0 c=IN IP6 <Remote_IP_audio> m=audio <Remote_port_audio> RTP/AVP 101 0 13 126 a=rtpmap:101 opus/48000/2 a=rtpmap:0 PCMU/8000 a=rtpmap:13 CN/8000 a=rtpmap:126 telephone-event/8000 } }, Stream = 2 { ; Video/RTP LocalControl { ipdc/realm = <core_realm>, Mode = SendReceive, ... }, Local { v=0 c=IN IP6 $ m=audio $ RTP/AVPF 96 97 a=rtpmap:96 VP8/90000 a=rtpmap:97 H264/90000 }, Remote { v=0 c=IN IP6 <Remote_IP_video> m=audio <Remote_port_video> RTP/AVPF 96 97 a=rtpmap:96 VP8/90000 a=rtpmap:97 H264/90000 } }, Stream = 4 { ; Text/T.140/RTP LocalControl { ipdc/realm = <core_realm>, Mode = SendReceive, ... }, Local { v=0 c=IN IP6 $ m=text $ RTP/AVP 94 95 a=rtpmap:95 t140/1000/1 a=rtpmap:94 red/1000/1 a=fmtp:94 95/95/95 }, Remote { v=0 c=IN IP6 <Remote_IP_text> m=text <Remote_port_text> RTP/AVP 94 95 a=rtpmap:95 t140/1000/1 a=rtpmap:94 red/1000/1 a=fmtp:94 95/95/95 } }, Stream = 5 { ; IM(text)/MSRP/TCP LocalControl { ipdc/realm = <core_realm>, Mode = SendReceive, ... }, Local { v=0 c=IN IP6 $ m=text $ TCP/MSRP – a=path:msrp://<MG_authority_core>/\ <UE_session_chat>;tcp a=setup:actpass ; Similar comments as to T1.S5 }, Remote { v=0 c=IN IP6 <Remote_IP_chat> m=text <Remote_port_chat> TCP/MSRP – a=path:msrp://<Remote_authority>/\ <Remote_session_chat>;tcp } }, Stream = 6 { ; IM(file)/MSRP/TCP LocalControl { ipdc/realm = <core_realm>, Mode = SendReceive, ... }, Local { v=0 c=IN IP6 $ m=text $ TCP/MSRP – a=path:msrp://<MG_authority_core>/\ <UE_session_file_transfer>;tcp a=setup:actpass ; Similar comments as to T1.S5 }, Remote { v=0 c=IN IP6 <Remote_IP_chat> m=text <Remote_port_chat> TCP/MSRP – a=path:msrp://<Remote_authority>/\ <Remote_session_file_transfer>;tcp } }, Stream = 7 { ; FloorCtrl/BFCP/TCP LocalControl { ipdc/realm = <core_realm>, Mode = SendReceive, ... }, Local { v=0 c=IN IP6 $ m=application $ TCP/BFCP – ; TBC – further BFCP related attributes? ; a=floorctrl might not be needed as we want ; to establish an end-to-end BFCP session. ; "confid" or "userid" attributes required? }, Remote { v=0 c=IN IP6 <Remote_IP_floor_ctrl> m=application <Remote_port_floor_ctrl> \ TCP/BFCP – ; TBC – further BFCP related attributes? } } } }} |  |
| NOTE 1 – |  |

{Editor's note (2015-02): above signalling example is just a first draft and needs further work because dependent on parallel discussions related to
1) stream deaggregation (C-698),
2) codepoint for parameter Semantics (C-682),
3) seplink enhancements (C-685) and
4) "IETF SDP drafts".
Furthermore, the example is expected to be extended in order to illustrate as well the bearer control procedures related to
5) DTLS establishment, 
6) SCTP establishment and
7) SCTP StreamID handling in more detail.

Furthermore, the creation of the ten SEPs in T1 might be mapped on multiple H.248 command request/reply cycles in order to illustrate signalling dynamics (see H.248.SCTP). For instance:
Step 1: creation of S1 to S3, S9 and S10 ("the WebRTC communication is initially starting with audio and video, DTLS session is immediately established in order to execute DTLS-SRTP");
Step 2: creation of S8 ("additional WebRTC data components are expected for future use, thus the SCTP association is established despite the fact of still lacking concrete WebRTC data channel open requests");
Step 3: creation and removal of S4 to S7 ("dependent on dynamic usage of WebRTC DCs").

Furthermore, the example could be extended to illustrate a L4 protocol switchover from UDP to TCP (or vice versa) during the active communication phase ("which results in an H.248 MODify.request for S10").

Discussion are ongoing.}

Some observations:

- IP address realm indication is mandatory for H.248 WebRTC gateways. The isolation of the (L4)-connection-endpoint (UDP or TCP here) inclusive the underlying IP (IPv4 or IPv6) in a self-contained SEP (here S10) allows an unambiguous IP realm indication (via ipdc/realm property), i.e., avoids redundancy by repeating that information in all SEPs "at upper layer protocols".

- Protocol replacement within a particular SEP (here: L4 protocol switchover process from UDP to TCP or vice versa during active communication phase, driven by NAT traversal issues: the isolation of the (L4)-connection-endpoint in a self-contained SEP seems to be beneficial from H.248 signalling perspective ("by limiting the required signalling information on the parts only, which are actually affected");

- FIXTHIS

### L.2.2	Use case #2.2: WebRTC without RTP media multiplexing and a single SEP for the "SCTP/DTLS/L4/IP" stack segment

{Editor's note (2015-02): use case #2.1 illustrates the most granular model concerning the mapping on individual H.248 SEPs. Such a mapping is feasible (thanks to H.248.STGROUP) and beneficial for specific H.248 WebRTC gateway scenarios. However, there are "WebRTC domains" without the full-fledged usage of the WebRTC service suite, a simplified NAT traversal context, a data-less WebRTC service profile, etc etc. Such scenarios could lead to simplified stream grouping models. Such a use case is subject of this clause.}

FIXTHIS.

## L.3	Use case #3: Telepresence media server

### L.3.1	Use case #3.1: Protocol stack for application control mapped on group of three streams

Telepresence essentially relates to an realtime multimedia conferencing service associated with a bearer-level control protocol, called CLUE (see [b-IETF clue-prot], [b-IETF clue-sign], [b-IETF clue-dc]).

The example focuses on a single termination, related to one conference party, of a context with usually multiple terminations. Only the stream modelling for the application control traffic, as processed at H.248 MG level, is in scope of this example. Thus, the usually numerous H.248 media streams for audio, video, image, text, data, etc of telepresence based conferencing services are not considered here. The mapping of application control on three, grouped H.248.SEPs is studied in this example, see Figure L.5:

Figure L.5 – Use case #3.1: Telepresence server – Protocol stack for application control mapped on group of three streams

Such a fine granular stream group mapping is motivated by following objectives or assumptions in this example:

- UDP is the primary IP transport protocol, but it cannot be excluded that detected NAT problems could lead to a replacement by TCP during the active conferencing phase.

- The SCTP association and DTLS session/connection are decoupled concerning the directionality of establishment.

- The SCTP association and DTLS session/connection are decoupled concerning the timeline of establishment.

- The DTLS protocol is profiled, primarily due to supported protocol versions and the explicit notification of possible error alerts.

- SCTP association establishment is conditional, dependent on successful DTLS authorization and established DTLS session.

- Thus, SCTP protocol procedures initially blocked. SCTP association establishment is rather triggered in a subsequent step, after confirmation of trusted user.

- Only dial-in conference parties are supported, hence DTLS server role expected, and H.248 SEP prepared for incoming DTLS session establishment procedure.

- SCTP protocol failures could lead to a shut-down of the SCTP association, but should not lead to the release of a DTLS session/connection, rather a re-establishment of an SCTP association.

- CLUE handling for telepresence support: relay of CLUE signalling information between MGC and MG requires a message backhauling service (based on [b-ITU-T H.248.78]), which is tight to a particular SEP here.

- Termination prepared for incoming CLUE protocol messages.

- There are two mechanisms (still in discussion) for establishing a CLUE data channel (see [b-IETF clue-dc]): the Data Channel Establishment Protocol (DCEP) and the SDP-based "SCTP over DTLS" data channel negotiation mechanism. The discussed example is using the SDP approach.

- Performance monitoring: a few protocol specific statistics are requested, which result in stream-level statistics.

The considered context model uses three H.248 SEPs (see Figure L.5, "stream group" SG1 in termination T1). Table L.3.1 illustrate example H.248 signalling with regards to the initial creation of the context and termination T1:

Table L.3.1 – Example H.248 media descriptor – step 1 for initial DTLS establishment

| H.248/SDP encoding (shortened H.248 Media Descriptor) | Comments: |
| --- | --- |
| ; Termination T1 Context = C1 { ; Context C1 Add = ip/$/$/$ { ; Termination T1 Media{ ; stream group for application control traffic TerminationState { mgroup/groupse = ["CLUE 1 2 3"] ; NOTE 1 } ; CLUE control group ## Stream = 3 { ; IP transport stack LocalControl{ ipdc/realm = <realm_X_IPv6>, ; NOTE 2 Mode = SendRecv }, Local{ v=0 ; c=IN IP6 $ … m=application $ UDP – b= … ; bitrate of IP transport connection …}, Remote{ v=0 ; c=IN IP6 <network address> … m=application <L4_port> UDP – b= … ; bitrate of IP transport connection …}, Statistics{ ; IP layer specific statistics ipocs/ipos, ; NOTE 3 ipocs/ipor ; NOTE 3 } } /* end of Stream 3 */ Stream = 2 { ; DTLS session/connection LocalControl{ ; DTLS protocol profiling tlsbsc/bceb = "unblocked", ; NOTE 4 tlscn/tlsv = ["1.2" "1.0"], ; NOTE 4 tlscn/srsc = OFF, ; NOTE 5 Mode = SendRecv }, Local{ v=0 ; no "s="-, "t="-, "o="- and "c="-lines! ## m=application - DTLS – ; NOTE 6 a=fingerprint:$ (to be confirmed) }, Remote{ v=0 ; no "s="-, "t="-, "o="- and "c="-lines! m=application - DTLS – a=fingerprint:SHA-256 <UE_A_DTLS_fingerprint> }, Events = 1234 { ; DTLS layer specific events tlsbsc/BNCChange [Type = Est} ; NOTE 7 tlsm/mgea ; NOTE 8 } Statistics{ ; DTLS layer specific statistics ## tlstv/rectmfrag, ; NOTE 9 ## tlstv/senttmfrag ; NOTE 9 } } /* end of Stream 2 */ Stream = 1 { ; CLUE & SCTP association LocalControl{ sctpbcc/bceb = "blocked", ; NOTE 10 sctpbcc/sctpid = <SCTP StreamID>, ; NOTE 11 Mode = Inactive }, Local{ v=0 ; no "s="-, "t="-, "o="- and "c="-lines! m=application – DTLS/SCTP webrtc-datachannel a=fmtp:webrtc-datachannel max-message-size=… a=sctp-port:- }, Remote{ v=0 ; no "s="-, "t="-, "o="- and "c="-lines! m=application – DTLS/SCTP webrtc-datachannel a=fmtp:webrtc-datachannel max-message-size=… a=sctp-port:<UE_A_port_SCTP> }, Events = 5678 { ; CLUE message backhauling mcbalg/det{ stream=1, ; NOTE 12 ## ehpf ="webrtc-datachannel", ## label="CLUE" } } Statistics{ ; SCTP specific statistics … } } /* end of Stream 1 */ ; end of stream group for application control ; streams for application data traffic ## Stream = 10 { ; audio stream 1 … etc, out of scope of this example | Example traffic flow configuration from Figure L.v1. Termination T1: - Group semantics: - Which streams: S1, S2 and S3 are grouped; - Semantics: "CLUE" (or "CLUE*") (to be confirmed) (NOTE 1) - Directionality: unspecified, hence default "bidirectional" at TerminationState level, however, finally profile at Stream level Stream endpoint T1(S3): - IP transport stack - UDP-based (but might be dynamically replaced by TCP in case of NAT issues (TBC)) - IP address realm indication (NOTE 2) Stream endpoint T1(S2): - DTLS layer Stream endpoint T1(S1): - SCTP layer and CLUE protocol |
| ; other Terminations T2, … |  |
| NOTE 1 –The "grouping semantic" CLUE is defined/proposed for call control signaling, but not yet for H.248 gateway control. Following semantic is supposed her: the streams which are grouped constitute together the protocol stack as used for MG-level CLUE signalling. The distribution on multiple H.248 SEPs facilitates the control of protocol layer specific operations. NOTE 2 – IP domain indication is required. The control scope could be limited on SEP T1(S3) only. NOTE 3 – The IP layer traffic volume in octets is measured in order to get real data concerning a particular telepresence conference user configuration. The measurements are subject of fine-tuned bitrate estimations for traffic control of future telepresence calls. These H.248 statistics are tight to the "IP" SEP in order to avoid any ambiguities due to the overall, hierarchical protocol layering of "L4-over-L4" (here: SCTP/…/L4/IP"). NOTE 4 – DTLS protocol profiling: might be necessary when telepresence mandates any DTLS protocol version (which is the case in WebRTC, hence might be applicable here too) (TBC). NOTE 5 – DTLS protocol profiling: furthermore, DTLS renegotiation is explicitly disabled due to multiple reasons ("ongoing discussion in IETF RTCWEB / CLUE / MMUSIC concerning the usage of that procedure in context of SCTPoDTLS layering; and unclear semantic of this (D)TLS procedure from H.248 perspective"). The expected MG behavour should be the blocking of any DTLS handshake procedure after the initial, successful establishment of the DTLS connection. NOTE 6 – The SDP-based media description for the "DTLS" SEP requires only the indication of protocol "DTLS" and security information. NOTE 7 – Incoming DTLS session establishment request(s) are expected. NOTE 8 – DTLS alerts could lead to the automatic release of a DTLS connection (and loss of (SCTP)-association-endpoint and loss of (CLUE)-endpoint). The MGC wants to be notified about the particular DTLS alert type. NOTE 9 – The MGC observes the "DTLS control traffic" by enabling statistics related to "DTLS messages" (see Annex B and clause 12 in [b-ITU-T H.248.90]), which cover essentially all DTLS protocol data units without any SCTP data. NOTE 10 – Preparation of SCTP layer by initial blocking of any SCTP bearer control procedures (due to first confirmation of trusted user and successfully established DTLS connection inline with allowed DTLS protocol profile). NOTE 11 – SCTP StreamID handling: for further studies. NOTE 12 – Handling of CLUE information for telepresence support. It may be noted that events parameter "stream=1" relates to an H.248 stream and not to an SCTP stream. |  |

When the remote user equipment answers, then the MGC will modify the configuration of context C1. There might be following actions:

Table L.3.2 – Example H.248 media descriptor – step 2 for completion

| H.248/SDP encoding (shortened H.248 Media Descriptor) | Comments: |
| --- | --- |
| FIXTHIS… Stream = 3 { ; IP transport stack => no changes Stream = 2 { ; DTLS session/connection => no changes Stream = 1 { ; CLUE & SCTP association => SCTP association establishment procedure |  |

### L.3.2	Use case #3.2: Protocol stack for application control mapped on group of two streams

{Editor's note (2015-02): use case #3.1 illustrates the most granular model concerning the mapping on individual H.248 SEPs. Such a mapping is feasible (thanks to H.248.STGROUP) and beneficial for specific H.248 telepresence media server scenarios. However, there are network solutions without the need of a full-fledged usage of the outlined telepresence transport protocol stack, a simplified NAT traversal context, c etc. Such scenarios could lead to simplified stream grouping models. Such use cases are subject of this and the next sub-clause.}

This approach distributes the protocol stack for application control on two SEPs, which both are grouped together using stream grouping. The primary purpose is to separate SCTP and its upper layer protocol CLUE from the underyling tunneled transport stack, see Figure L.6:

Figure L.6 – Use case #3.2: Telepresence server – Protocol stack for application control mapped on group of two streams

### L.3.3	Use case #3.3: Protocol stack for application control mapped on a single stream

This model should be basically feasible and might be default for H.248 profiles with telepresence support. The model might be sufficient particularily when all the assumptions of use case #3.1 would be not relevant (i.e., in case of a reduced set of requirements).

{Editor's note: "proof of assumption still missing"}

The resulting context model is depicted in see Figure L.7, using a single stream for the entire protocol stack for application control:

Figure L.7 – Use case #3.3: Telepresence server – Protocol stack for application control mapped on a single stream

It has to be noted that use case #3.3 does not use any kind of stream grouping.

### L.3.4	Benefits of stream grouping for CLUE-based telepresence

Some observations related to the stream grouping based solutions #3.1 and #3.2:

- aggregation stream looks beneficial for additional services (here: H.248 statistics for performance monitoring support);

- isolation of individual bearer-connection-endpoints (here L4, DTLS, SCTP) in self-contained SEPs looks attractive from H.248 signalling side ("e.g., because subsequent modification operations could limit the signalling on information only affected within a H.248 media descriptor").

## L.4	Use case #4: alternate IP connectivity (ALTC)

### L.4.1	Use case #4.1: with support of early media for preferred IP connections

A dual media call (audio and video) is considered. The H.248 MG requires an (IP, IP) connection model, i.e., there are typically two IP-enabled H.248 terminations. It is supposed that there is support for IPv4 and IPv6 at each termination, however, the ALTC capability is only used at one termination (e.g., because there might be dual stack capable IP user equipment). Figure L.8 illustrates the evaluated stream grouping model:

Figure L.8 – Use case #4.1: Alternate IP connectivity – Context stream grouping model

The preferred ALTC IP connectivity is IPv6 based and IPv4 serves as alternate. It is known that the majority of registered user equipment is already IPv6 capable as well as the network operator wants to benefit from his IPv6 infrastructure. Hence, there is not an equal balance between IPv4 and IPv6 resources, rather a high preference for IPv6.

H.248 stream S1 is used for audio. SEP T1(S1) is supposed to be connected to an IPv4 domain. SEP T2(S1) serves as aggregation instance in ALTC direction. H.248 stream T2(S3) e.g., provides the IPv6-based preferred IP connection endpoint description.

Similar setting for video, but SEP T1(S2) is supposed to be connected to an IPv6 domain.

Table L.4.1 illustrates example H.248 signalling with regards to the initial creation of the context and all six stream endpoints, at the example of an outgoing call (i.e., Remote descriptor information not yet available at ALTC side of the context):

Table L.4.1 – Example H.248 media descriptor

| H.248/SDP encoding (shortened H.248 Media Descriptor) | Comments: |
| --- | --- |
| ; Termination T1 Context = C1 { ; Context C1 Add = ip/$/$/$ { ; Termination T1 Media{ Stream = 1 { ; audio LocalControl{ ipdc/realm = <realm_X_IPv4>, ; NOTE 1 Mode = SendRecv, ReservedGroup = OFF, ReservedValue = OFF }, Local{ v=0 ; start of SDP-defined H.248 media group c=IN IP4 $ … m=audio $ RTP/SAVPF … …}, Remote{ v=0 ; start of SDP-defined H.248 media group c=IN IP4 … … m=audio 530 RTP/SAVPF … …}, } /* end of Stream 1 */ Stream = 2 { ; video LocalControl{ ipdc/realm = <realm_X_IPv6>, ; NOTE 1 Mode = SendRecv, ReservedGroup = OFF, ReservedValue = OFF }, Local{ v=0 ; start of SDP-defined H.248 media group c=IN IP6 $ … m=video $ RTP/SAVPF … …}, Remote{ v=0 ; start of SDP-defined H.248 media group c=IN IP6 … … m=video 530 RTP/SAVPF … …}, } /* end of Stream 2 */ | Termination T1 does not use alternate IP connectivity, hence only single IP connection endpoints are reserved and allocated. In the example: audio is using IPv4 transport and video IPv6. The IP address realm is explicitly indicated via the ipdc/realm property (NOTE 1). |
| ; Termination T2 Context = C1 { ; Context C1 Add = ip/$/$/$ { ; Termination T2 Media{ TerminationState { mgroup/groupse = ["ALTC 3 4"], ; audio group mgroup/groupse = ["ALTC 5 6"]} ; video group Stream = 1 { ; audio stream group LocalControl{ mgroup/stragg = [3, 4] } } /* end of context-int. aggregated Stream 1 */ Stream = 3 { ; preferred audio media stream LocalControl{ ipdc/realm = <realm_Y_IPv6>, ; NOTE 3 Mode = SendRecv, ; NOTE 4 ReservedGroup = OFF, ; NOTE 5 ReservedValue = OFF }, Local{ v=0 ; start of SDP-defined H.248 media group c=IN IP6 $ … m=audio $ RTP/AVP … …}, /* end of Stream 3 */ Stream = 4 { ; alternate audio media stream LocalControl{ ipdc/realm = <realm_Y_IPv4>, ; NOTE 3 Mode = ReceiveOnly, ; NOTE 4 ReservedGroup = OFF, ; NOTE 5 ReservedValue = OFF}, Local{ v=0 ; start of SDP-defined H.248 media group c=IN IP4 $ … m=audio $ RTP/AVP … …}, /* end of Stream 4 */ ; 2nd stream group Stream = 2 { ; video stream group LocalControl{ mgroup/stragg = [5, 6] } /* end of context-int. aggregated Stream 2 */ Stream = 5 { ; preferred video media stream LocalControl{ ipdc/realm = <realm_Y_IPv6>, ; NOTE 3 Mode = SendRecv, ; NOTE 4 ReservedGroup = OFF, ; NOTE 5 ReservedValue = OFF}, Local{ v=0 ; start of SDP-defined H.248 media group c=IN IP6 $ … m=video $ RTP/AVP … …}, /* end of Stream 5 */ Stream = 6 { ; alternate video media stream LocalControl{ ipdc/realm = <realm_Y_IPv4>, ; NOTE 3 Mode = ReceiveOnly, ; NOTE 4 ReservedGroup = OFF, ; NOTE 5 ReservedValue = OFF}, Local{ v=0 ; start of SDP-defined H.248 media group c=IN IP4 $ … m=video $ RTP/AVP … …}, /* end of Stream 6 */ } | Example traffic flow configuration from Figure L.w1. Termination T2: - Group semantics: - Which streams: S3 and S4 as part of the audio stream group and S5 and S6 as part of the video stream group; - Semantics: "ALTC" (alternate connectivity) (to be confirmed) (NOTE 2) - Directionality: unspecified, hence default "bidirectional" at TerminationState level, however, finally profile at Stream level Stream endpoint T2(S1): - serves as aggregate for audio. Stream endpoint T2(S2): - serves as aggregate for video. |
| NOTE 1 – IP address realm indication via the ipdc/realm property (see [b-ITU-T H.248.41]). NOTE 2 – The "grouping semantic" ALTC isn't yet defined. Following semantic would be beneficial: the token ALTC indicates the MG a) that finally one of the two SEPs within the "ALTC stream group" will be removed again, b) that the StreamID list is ordered, first list item relates to "preferred IP connectivity" and the second one to "alternate IP connectivit", and c) that the final selection of the "preferred IP connection endpoint" is more likely. The MG could take into account that semantic concerning the underlying reservation and allocation of MG resources. NOTE 3 – The IP address realm indication element is actually mandated in all H.248 profiles for IP-IP media gateways, therefore part of this example here (due to its tight semantical correlation with ALTC). NOTE 4 – Allows to support "early media". NOTE 5 – ReserveGroup is explicitly disabled because not applied for IP transport resources. However, there might be on top multiple application flows, which could lead to the additional definition of multiple H.248 media groups (feasibility to be confirmed). |  |

When the remote user equipment answers the "ALTC" request, then the MGC will modify the configuration of context C1. There might be following actions:

- in case of IPv6 selection:

- modification of group semantics at TerminationState level (or even complete removal of mgroup/groupse specification information (to be discussed));

- removal of SEPs T2(S4) and T2(S6);

- modification of grouping at LocalControl level of aggregated streams SEPs T2(S1) and T2(S2) (i.e., mgroup/stragg setting); and

- the usual completion of lacking information in LD/RDs of finally used IP connection endpoints SEPs T2(S3) and T2(S5);

- in case of IPv4 selection:

- similar steps as above.

### L.4.2	Benefits of stream grouping for ALTC

The evaluated ALTC stream grouping model is based on the usage of self-contained H.248 SEPs for preferred and alternate IP connection endpoint resources, and the possible decoupling of context-internal and context-external stream endpoint specifications, leading to following advantages:

- unambiguous usage of IP address realm indication possible, as required in conjunction with ALTC;

- ALTC grouping semantics could allow a different weighting of MG resources (see NOTE 2 in Table L.4.1);

- SEP-individual StreamMode settings for external streams;

- additional usage of ReserveGroup enabled multiple H.248 media groups should be principally feasible, e.g., related to protocol stack configurations on top of the IP transport connection (e.g., there might be two media groups enabled on T1(S1), which might be finally also used at e.g., T2(S2)); and

- finally, ALTC implies always removal of SEP(s) during the context establishment phase, which implies the creation of a SEP pair, - and the SEPP could be simply fixed via modification of property values of mgroup/stragg.

## L.5	Use case #5: SCTP transport modes

### L.5.1	Background

Protocol stacks for valid SCTP transport modes are outlined in clause 6.1/[b-ITU-T H.248.SCTP]. Figure L.9 illustrates the most complex SCTP transport mode, given by "tunneled SCTP via DTLS/L4", and the option of dynamic L4 replacements between UDP and TCP due to potential NAT issues during the lifetime of the communication service.

Figure L.9 – Generic protocol stack model for mapping of SCTP streams to applications
(here at the example of SCTP transport mode "tunneled SCTP via DTLS/L4")

This SCTP transport mode is characterized by

- option of L4 protocol and dynamic replacement(s);

- option of tunneled stack (L4 over L4); and

- option of single- vs multi-stream-SCTP mode.

Such a protocol stack requires usually multiple H.248 streams, but might be served in a specific case by a single H.248 stream only. The next sub-clause tries to evaluate such decision criteria.

### L.5.2	Guidelines for the derivation of H.248 streams per SCTP transport mode

There are theoretically eight combinations taken the assumption that each of the above three conditions leads finally to an "H.248 stream yes/no?" decisions. Figure L.10 summarizes a sample decision logic:

NOTE – The example serial execution of decisions could be parallized as well.

Figure L.10 – Example decision logic for the derivation 
of H.248 streams per SCTP transport mode

Whether there are finally really explicit H.248 streams requested by the MGC for each decision point is basically conditional. The default case would be to minimize the number of H.248 streams in order to reduce "signalling overhead", which means the merging of multiple protocol layers in single H.248 streams, whenever possible. The division in separated H.248 streams would be only done in case of semantical ambiguities, stream-specific functionality and/or control activities with limited scope on streams only.

The SCTP association serves multiple, N SCTP streams in parallel, with a value N between one ("the single-SCTP-stream-mode case") and usually "smaller than ten" for typical, present communication services.

The resulting amount of H.248 streams is between one as minimum and 'N+3' as theoretical maximum.

It has to be noted that the decision logic of Figure L.10 could be actually applied to all known SCTP transport modes. For instance:

- SCTP transport mode "native SCTP": amount of H.248 streams = 'N+1' (because a) without L4-over-L4 tunneling, and b) without DTLS); or

- SCTP transport mode "transport secured SCTP": amount of H.248 streams = '1' (due single-SCTP-stream-mode because DTLS connection is not shared by multiple application protocols).

### L.5.3	Use case #5.1: Domain with "tunneled SCTP via  switched {DTLS/UDP | DTLS/RFC4571/TCP}"

A possible H.248 context model and example H.248 signalling are discussed for the use case between the interworking of the two network domains with different transport modes as illustrated in Figure L.11. The example stream grouping model takes into account multiple application level traffic flows, which are mapped on individual data channels (DC):

Figure L.11 – Use case #5.1: SCTP transport modes – "tunneled SCTP via 
switched {DTLS/UDP | DTLS/RFC4571/TCP}"

Such a fine granular stream group mapping is motivated by following objectives or assumptions in this example:

- UDP is the primary IP transport protocol, but it cannot be excluded that detected NAT problems could lead to a replacement by TCP during the active conferencing phase.

- The SCTP association and DTLS session/connection might be decoupled concerning the directionality of establishment.

- The SCTP association and DTLS session/connection are decoupled concerning the timeline of establishment.

- The DTLS protocol is profiled, primarily due to supported protocol versions and the explicit notification of possible error alerts.

- SCTP association establishment is conditional, dependent on successful DTLS authorization and established DTLS session.

- Thus, SCTP protocol procedures initially blocked. SCTP association establishment is rather triggered in a subsequent step, after confirmation of trusted user.

- H.248 SEP for DTLS is able for both DTLS roles concerning incoming and outgoing DTLS session establishment procedures.

- SCTP protocol failures could lead to a shut-down of the SCTP association, but should not lead to the release of a DTLS session/connection, rather a re-establishment of an SCTP association.

- Performance monitoring: a few protocol specific statistics are requested, which result in stream-level statistics.

The considered context model uses three H.248 SEPs. Table L.5.1 illustrate example H.248 signalling with regards to the initial creation of the context and ###:

Table L.5.1 – Example H.248 media descriptor – step 1 for initial DTLS establishment

| H.248/SDP encoding (shortened H.248 Media Descriptor) | Comments: |
| --- | --- |
| ; Termination T1 Context = C1 { ; Context C1 Add = ip/$/$/$ { ; Termination T1 Media{ TerminationState { mgroup/groupse = ["WRTC-DC K,K+1 ,… ,K+N", "SCTP-TRANSPORT 3", "DTLS-TRANSPORT 2"] ; Semantics "WRTC-DC", "SCTP-TRANSPORT", ; "DTLS-TRANSPORT are not yet specified; }, ; --- STREAM 1 --------------------------------- Stream = 1 { ; L4 connection: UDP or TCP LocalControl { ipdc/realm = <IP_realm>, Mode = SendReceive, mgroup/strdeagg = [2] ; -> Stream 2 'DTLS' ... }, Local { v=0 c=IN IP6 $ m=- $ udp – ; Media type and media format agnostic, as ; this SEP does not need to know anything ; related to UDP payload formats. ; <proto> value "udp" as per RFC 4566. ; Assumption: ICE might also conclude with ; a TCP candicate pair, which would result ; in an SCTP/DTLS/RFC 4571/TCP/IP ; transport protocol stack. a=ice-ufrag:$ a=ice-pwd:$ }, Remote { v=0 c=IN IP6 <UE_IP_data> m=- <UE_port_data> udp – a=ice-ufrag:<UE_ICE_ufrag_data> a=ice-pwd:<UE_ICE_pwd_data> } }, ; --- STREAM 2 --------------------------------- Stream = 2 { ; DTLS connection LocalControl { Mode = SendReceive, tlsbsc/bceb="unblocked", mgroup/strdeagg = [3] ; -> Stream 3 'SCTP' ... }, Local { v=0 c=- - - m=- – DTLS/SCTP – a=fingerprint:$ $ }, Remote { v=0 c=- - - m=- – DTLS/SCTP – a=fingerprint:<UE_hash_func_data> <UE_fingerprint_data> } }, ; --- STREAM 3 --------------------------------- Stream = 3 { ; SCTP association LocalControl { Mode = SendReceive, mgroup/strdeagg = [K,K+1 ,… ,K+N] ; deaggreg. ... }, Local { v=0 c=- - - m=application – DTLS/SCTP webrtc-datachannel a=fmtp:webrtc-datachannel \ max-message-size=$ a=sctp-port=$ ; Assumption: MGC lets MG choose local SCTP ; port and maximal message size. }, Remote { v=0 c=- - - m=application – DTLS/SCTP webrtc-datachannel a=fmtp:webrtc-datachannel \ max-message-size=<UE_SCTP_max_message_size> a=sctp-port=<UE_SCTP_port> } }, ; --- STREAM K, K+1, …-------------------------- Stream = 4 { ; Data application LocalControl { Mode = SendReceive, ... }, Local { v=0 c=- - - m=application – DTLS/SCTP <application usage> … a=dcmap:<SCTP_Stream_ID> subprotocol="…" … }, Remote { v=0 c=- - - m=application – DTLS/SCTP <application usage> … a=dcmap:<SCTP_Stream_ID> subprotocol="…" … } }, | Stream 1: - (L4)-protocol is terminated - (L4)-protocol = 'UDP' (initially) - (L4)-protocol is linked to (L4+)-protocol (here 'DTLS'), which is part of Stream 2 - that deaggregation from the (L4) stream to the DTLS stream is indicated via the strdeagg property Stream 2: - (DTLS)-protocol is terminated - (DTLS)-protocol is linked to (SCTP)-protocol (Stream 3) - that deaggregation from the DTLS stream to the SCTP stream is indicated via the strdeagg property With respect to the establishment of the (DTLS)-bearer-connection: - the SDP "a=setup:" attribute cannot be used due to ambiguity; - the incoming/outgoing establishment procedures are controlled via tlsbsc package elements (not shown in the example) Stream 3: - (SCTP)-protocol is terminated; - SEP represents the (SCTP)-association-endpoint; - (SCTP)-association is linked to (SCTP)-streams (Stream K, K+1, …) - that deaggregation from SCTP association level to SCTP streams is indicated via the strdeagg property With respect to the establishment of the (SCTP)-bearer-connection: - the SDP "a=setup:" attribute cannot be used due to ambiguity; - the incoming/outgoing establishment procedures are controlled via sctpbcc package elements (not shown in the example) Streams K, K+1, etc are just indicated because the intention of this use case is not to emphasize on a particular communication service using this SCTP transport mode. |
| ; other Terminations T2, … |  |

The example H.248 command requests highlights some specifics related to the "stream deaggregation" characteristic:

a) semantical grouping of SEPs within a termination:

- the example protocol stack is characterized by multiple hierarchical relationships, which are reflected usage of property mgroup/groupse at termination level and property mgroup/strdeagg at stream level; and

- deaggration from a particular SEP to other SEP(s) covers the range from one to many;

b) LD-/RD-embedded SDP media description:

- IP transport address:

- the actual IP transport address information is only subject of the SDP "c="- / "m="-lines of H.248 SEPs which serving as protocol endpoint for the IP and L4 protocol;

- other SEPs which are agnostic for that kind of information using the "-" wildcard for correspondent SDP information elements;

- transport protocol (stack segment) indication:

- the "m="-line <proto> should provide a value for the protocol(s) which are assigned to that SEP;

- this would be a single protocol value only in above example (i.e., the SIP level "DTLS/SCTP" value is mapped to self-contained "DTLS" and "SCTP" values)

- SDP media format attributes: only H.248 SEP relevant attributes are sufficient, not relevant SDP attributes should be omitted in order to avoid redundancy.

{Editor's note (2015-02): above observations are still preliminary …}

When the remote user equipment answers, then the MGC will modify the configuration of context C1. There might be following actions:

Table L.5.2 – Example H.248 media descriptor – step 2 for completion

| H.248/SDP encoding (shortened H.248 Media Descriptor) | Comments: |
| --- | --- |
| FIXTHIS |  |

Living List Bibliography

[b-ITU-T V.18]	Recommendation ITU-T V.18 (11/2000), Operational and interworking requirements for DCEs operating in the text telephone mode.

[b-ITU-T F.703]	Recommendation ITU-T F.703 (11/2000), Multimedia conversational services.

[b-ITU-T F.790]	Recommendation ITU-T F.790 (01/2007), Telecommunications accessibility guidelines for older persons and persons with disabilities.

[b-ITU-T AVA TR2]	Technical Report ITU-T FG AVA TR Version 1.0 (10/2013), Part 2: Vocabulary for ITU-T Focus Group on Audiovisual Media Accessibility (FG AVA).

[b-ETSI ETR 333]	ETSI ETR 333, Text Telephony; User requirements and Recommendations.

[b-ETSI EG 202 320]	ETSI EG 202 320 Version 1.2.1 (10/2005), Human Factors (HF); Duplex Universal Speech and Text (DUST) communications.

[b-ETSI TS 122 226]	ETSI  TS 122 226 Version 9.0.0 (01/2010), Digital cellular telecommunications system (Phase 2+); Universal Mobile Telecommunications System (UMTS); LTE; Global text telephony (GTT); Stage 1: Service description (3GPP TS 22.226 version 9.0.0 Release 9).

[b-ETSI TS 123 226]	ETSI  TS 123 226 Version 9.0.0 (01/2010), Digital cellular telecommunications system (Phase 2+); Universal Mobile Telecommunications System (UMTS); LTE; Global text telephony (GTT); Stage 2: Service description (3GPP TS 22.226 version 9.0.0 Release 9).

[b-IETF clue-prot]	IETF draft-ietf-clue-protocol (##/201#), CLUE protocol.

[b-IETF clue-sign]	IETF draft-ietf-clue-signaling (##/201#), CLUE signaling.
(=> SDP "a=group:CLUE")

[b-IETF clue-dc]	IETF draft-ietf-clue-datachannel (##/201#), CLUE Protocol Data Channel. 
(=> SDP-based vs DCEP-based DC control procedures")

[b-IETF RFC 6947]	IETF RFC 6947 (05/2013), The Session Description Protocol (SDP) Alternate Connectivity (ALTC) Attribute.

_______________________

E.g., an initial UDP connection might be replace at a later point in time by a TCP connection, triggered by ICE refreshment activities.