| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4690 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Chengdu (China), 8 - 12 June 2015 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.STGROUP: clause L.4 – discussion and update proposal for "use case #4: ALTC" |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution continues the discussion of a potential stream grouping model for ALTC.

1	Introduction

There wasn't yet any deep technical discussion on this use case at the last meeting due to time reasons. The last meeting did rather focus on the principal handling from H.248.STGROUP documentation perspective.

2	Discussion and conclusions.

2.1	ANAT-related change proposals

C.680R1 provided some ANAT related change requests besides the scope on the ALTC model. The ANAT related update proposals were missed to be implemented in the output draft of H.248.STGROUP.

The ANAT-specific parts are repeated again here, see change proposals #1 to #3.

That changes are ALTC-independent.

2.2	ALTC-related change proposals

The use case in clause L.4 was updated at a few places. There's mainly one open point related to ReserveGroup. However, it could be already concluded that the H.248 stream grouping is basically applicable for ALTC. The main benefits of stream grouping for ALTC are indicated in clause L.4.2.

Below Figure 1 illustrates again the specific ALTC-aspects of H.248 stream grouping (for a monomedia call), which is totally different in comparison to the other stream grouping use cases so far. ALTC effectively leads to a separation of the protocol stack in an upper and lower layer part because the "connection aspect" affects essentially only the transport level resources!

Figure 1 – Benefits of stream grouping for ALTC- MG resource model with and without stream grouping

Figure 1 outlines high-level resource models in a pure qualitative manner (i.e., without quantifying real MG resources, which is always conditional, e.g., service dependent). ALTC-specific H.248 stream grouping is also different to other stream grouping applications due to its dynamic nature during the call establishment phase.

The resource model without stream grouping (top level in Fig. 1) illustrates that all application-level MG resources are logically doubled from signalling point of view, i.e., there's a huge amount of redundant information, especially in case of multimedia call configurations.

The usage of stream grouping would allow to limit the dynamics on the part only, which is affected by ALTC, i.e., the transport level MG resources.

Figure 1 indicates that all control operations happen on a single "monlithic" H.248 stream for the model without stream grouping. The model with stream grouping partitions such control functionality in three different H.248 streams with the benefit that only the H.248 component streams related to transport level resources are subject of modification, whereas the allocation of the application level resources in the H.248 aggregation stream gots two inherent advantages: 1st  static nature from resource management perspective, and 2nd MG-internal IWFcould be prepared as early as possible (because the application level parts are more complex than the lower level IWF parts).

When the meeting would agree to continue with stream grouping support for ALTC, then also a new group semantic 'ALTC' would need to be added in H.248.STGROUP.

3	Proposal

It is proposed to accept the marked up changes below. Changes are marked up against AVD-4680, the input draft for this meeting.

NOTE – Some changes are complemented by electronic comments in order to provide more background information or editorial hints. It's at the discretion of the meeting and/or editor to handle these comments (deletion or conversion to editor note).

BEGIN of ANAT changes

Change proposal #1:

[Begin Proposed Correction]

#### 8.6.3.4	Alternative Network Address Types (ANAT)

The “Alternative Network Address Types” semantic is defined in [b-IETF RFC 4091]. The ANAT semantic allows an SDP offerer to indicate that multiple network address types may be used for a single logical media stream. This semantic is redundant in H.248 as it offers multiple methods for indicating or requesting address types. A MGC may use “over specification” (see 7.1.8.2.2/[ITU-T H.248.1) to provide a list of network address types for a particular H.248 Stream. A MG will then chose an appropriate network address type.

An MGC that wants to initiate at call control level an SDP offer with the ANAT semantics can reserve resources for both transport types using two H.248 media groups (called "Property Group" in ASN.1), using the LocalControl property "ReserveGroup" (see clause 7.1.7.1.2/[ITU-T H.248.1]). Both property groups will contain the same information except for the IP address information (e.g. the SDP connection attribute) which will be partially wildcarded, and will request an IPv4 address in one of the groups and an IPv6 address in the other group. The MGC will set ReserveGroup to true. Thereafter it will send the SDP offer.

The MGC that receives the SDP offer with the ANAT semantics may be able to determine by itself which of the transport types to use, e.g. by certain call or traffic distribution criteria. In that case it will only indicate one address type when adding the termination in the MG. The MGC may also delegate that decision to the MG. In that case it will send two property groups, like the initiating MGC, but in this case, it will set ReserveGroup to false. The MG will only allocate resources for one of the transport types and indicate the chosen one to the MGC in the ADD reply.

The initiating MGC, when receiving the SDP answer which contains the chosen transport type, may send a H.248 Modify command to the MG, this time excluding the property group with the transport type that was not chosen.

[End Proposed Correction]

Change proposal #2:

[Begin Proposed Correction]

#### 8.6.5.2	Events and aggregation streams

An event applied to the aggregation stream will have the effect of detecting events on the component streams as a result of stimulus usually from outside the context. How the event is reported is based on the grouping semantic associated with the aggregation stream. Events may still be specified on the component streams. In this case, detection is isolated to that particular component stream.

Where an event is set at the termination level on a termination that has an aggregation stream the effect will be that it is only set against the aggregation streams and stream not involved in an aggregation.

Example 1:

Figure 4 shows an example where a Context has a Termination Ta with one video stream and two audio streams with identities 1, 2 and 3 grouped with the SRF (Single Reservation Flow) semantics [IETF RFC 3524]. Thus, resource reservation using RSVP and policy enforcement is applied to the complete group. The MGC has defined an aggregation stream with stream identity 4. In order to receive RSVP messages related to all grouped streams, the MGC should request the events of the rsvp package [ITU-T H.248.65], e.g. rsvp/pathr event on stream 4. The detection of a RSVP path message related to stream1, 2 and 3 results in the notification of a single ObservedEvent from the aggregation stream.

NOTE – RSVP sessions are normally defined by the triplet destination address, protocol ID, destination port. For the identification of the session related to grouped media, see [IETF RFC 3524].

Figure 4 – Events on an aggregation stream with SRF semantic

Example 2:

Figure 5 shows an example where a Context has a termination Ta. It has two audio streams which use different codecs (media streams 1 and 2). They are grouped with the FID (Flow Identification) semantics [IETF RFC 5888]. Thus, at any point in time, only one of them carries data while the other one is muted. The termination has in addition another media stream with identity 3, not part of the stream group at Ta. The MGC has defined an aggregation stream 4. The MGC requests the MG to collect data volume related statistics on the aggregation stream. The MGC uses also the conditional reporting functionality [ITU-T H.248.47] and activates therefore the scr/cr event on the aggregation stream and refers to the relevant statistics in the si parameter.

Termination Tb contains two stream endpoints: Sid3 for video traffic (= H.248 media stream 3) and Sid4 for the aggregated audio streams.

Figure 5 – Events on an aggregation stream with FID semantic

{Contributor's / Editor's note (2015-02): we were wondering about the RTP topology for audio. There might be at least three meaningful options, however, firstly it need to be clarified whether Sid4 represents a single or two RTP sessions. This is dependent whether there is a single RTP source or two behind media 1 and 2. In case of "single" would be single SSRC but different PT values for each media format, and in case of two source would be two SSRCs. Further Sid4 could apply RTP media multiplexing. It would be beneficial if we could add further text, something like

It may be noted that the H.248 stream endpoint pair {Ta(Sid4), Tb(Sid4)} represents a back-to-back RTP endsystem (B2BRE) topology because …

}

[End Proposed Correction]

Change proposal #3:

[Begin Proposed Correction]

#### 8.6.5.3	Statistics and aggregation streams

When statistics are set against an aggregation stream, the statistics are collected as if they had been set against all the component streams. For example, if the rtp/pr “RTP packets received” statistic is set on an aggregation stream it will count the sum of RTP packets received from all the component streams.

When statistics are set at the termination level on a termination that has an aggregation stream the effect will be that it is only set against the aggregation streams and streams not involved in an aggregation.

Figure 6 – Statistics on an aggregation stream with BUNDLE semantic

Example:

Figure 6 shows an example where a Context has a termination Ta that handles three audio streams with identities 1, 2 and 3 grouped with the BUNDLE semantic. The termination has in addition another media stream with identity 4, not part of the media group. The MGC has defined an aggregation stream with identity 5. The MGC wishes to collect statistics on the data volumes sent and received in the bundled group at different levels (network, transport, application). It uses the Network Package os and or [ITU-T H.248.1], the IP Layer Octets Counts Statistics Package ipos, ipor, ipps and ippr [ITU-T H.248.61] and the RTP Application Data Package payloados and payloador [ITU-T H.248.58] statistics. It requests these statistics by including them in the statisticsDescriptor of the streamDescriptor of stream 5. The MG should collect statistics in each of the three component streams and sum up the values.

[End Proposed Correction]

END of ANAT changes

BEGIN of ALTC changes

Change proposal #4:

[Begin Proposed Correction]

## L.4	Use case #4: alternate IP connectivity (ALTC)

### L.4.1	Use case #4.1: with support of early media for preferred IP connections

{Editor's note (2015-06): it was questioned whether the example could not be changed from a dualmedia to a monomedia call in order to reduce the signalling syntax. The current example is due to the same type of examples as currently under evaluation in H.Sup.ALTC.
If an ALTC related stream grouping example would be finally incorporated in H.248.STGROUP, then a monomedia type of call should be definetely sufficient.}

A dual media call (audio and video) is considered. The H.248 MG requires an (IP, IP) connection model, i.e., there are typically two IP-enabled H.248 terminations. It is supposed that there is support for IPv4 and IPv6 at each termination, however, the ALTC capability is only used at one termination (e.g., because there might be dual stack capable IP user equipment). Figure L.8 illustrates the evaluated stream grouping model:

Figure L.8 – Use case #4.1: Alternate IP connectivity – Context stream grouping model

The preferred ALTC IP connectivity is IPv6 based and IPv4 serves as alternate. It is known that the majority of registered user equipment is already IPv6 capable as well as the network operator wants to benefit from his IPv6 infrastructure. Hence, there is not an equal balance between IPv4 and IPv6 resources, rather a high preference for IPv6.

H.248 stream S1 is used for audio. SEP T1(S1) is supposed to be connected to an IPv4 domain. SEP T2(S1) serves as aggregation instance in ALTC direction. H.248 stream T2(S3) e.g., provides the IPv6-based preferred IP connection endpoint description.

Similar setting for video, but SEP T1(S2) is supposed to be connected to an IPv6 domain.

Table L.4.1 illustrates example H.248 signalling with regards to the initial creation of the context and all six stream endpoints, at the example of an outgoing call (i.e., Remote descriptor information not yet available at ALTC side of the context):

Table L.4.1 – Example H.248 media descriptor

| H.248/SDP encoding (shortened H.248 Media Descriptor) | Comments: |
| --- | --- |
| ; Termination T1 Context = C1 { ; Context C1 Add = ip/$/$/$ { ; Termination T1 Media{ Stream = 1 { ; audio LocalControl{ ipdc/realm = <realm_X_IPv4>, ; NOTE 1 Mode = SendRecv, ReservedGroup = OFF, ReservedValue = OFF }, Local{ v=0 ; start of SDP-defined H.248 media group c=IN IP4 $ … m=audio $ RTP/SAVPF … …}, Remote{ v=0 ; start of SDP-defined H.248 media group c=IN IP4 … … m=audio 530 RTP/SAVPF … …}, } /* end of Stream 1 */ Stream = 2 { ; video LocalControl{ ipdc/realm = <realm_X_IPv6>, ; NOTE 1 Mode = SendRecv, ReservedGroup = OFF, ReservedValue = OFF }, Local{ v=0 ; start of SDP-defined H.248 media group c=IN IP6 $ … m=video $ RTP/SAVPF … …}, Remote{ v=0 ; start of SDP-defined H.248 media group c=IN IP6 … … m=video 530 RTP/SAVPF … …}, } /* end of Stream 2 */ | Termination T1 does not use alternate IP connectivity, hence only single IP connection endpoints are reserved and allocated. In the example: audio is using IPv4 transport and video IPv6. The IP address realm is explicitly indicated via the ipdc/realm property (NOTE 1). |
| ; Termination T2 Context = C1 { ; Context C1 Add = ip/$/$/$ { ; Termination T2 Media{ TerminationState { mgroup/groupse = ["ALTC 3 4"], ; audio group mgroup/groupse = ["ALTC 5 6"]} ; video group Stream = 1 { ; audio stream group LocalControl{ mgroup/stragg = [3, 4] } } /* end of context-int. aggregated Stream 1 */ Stream = 3 { ; preferred audio media stream LocalControl{ ipdc/realm = <realm_Y_IPv6>, ; NOTE 3 Mode = SendRecv, ; NOTE 4 ReservedGroup = OFF, ; NOTE 5 ReservedValue = OFF }, Local{ v=0 ; start of SDP-defined H.248 media group c=IN IP6 $ … m=audio $ RTP/AVP … …}, /* end of Stream 3 */ Stream = 4 { ; alternate audio media stream LocalControl{ ipdc/realm = <realm_Y_IPv4>, ; NOTE 3 Mode = ReceiveOnly, ; NOTE 4 ReservedGroup = OFF, ; NOTE 5 ReservedValue = OFF}, Local{ v=0 ; start of SDP-defined H.248 media group c=IN IP4 $ … m=audio $ RTP/AVP … …}, /* end of Stream 4 */ ; 2nd stream group Stream = 2 { ; video stream group LocalControl{ mgroup/stragg = [5, 6] } /* end of context-int. aggregated Stream 2 */ Stream = 5 { ; preferred video media stream LocalControl{ ipdc/realm = <realm_Y_IPv6>, ; NOTE 3 Mode = SendRecv, ; NOTE 4 ReservedGroup = OFF, ; NOTE 5 ReservedValue = OFF}, Local{ v=0 ; start of SDP-defined H.248 media group c=IN IP6 $ … m=video $ RTP/AVP … …}, /* end of Stream 5 */ Stream = 6 { ; alternate video media stream LocalControl{ ipdc/realm = <realm_Y_IPv4>, ; NOTE 3 Mode = ReceiveOnly, ; NOTE 4 ReservedGroup = OFF, ; NOTE 5 ReservedValue = OFF}, Local{ v=0 ; start of SDP-defined H.248 media group c=IN IP4 $ … m=video $ RTP/AVP … …}, /* end of Stream 6 */ } | Example traffic flow configuration from Figure L.w1. Termination T2: - Group semantics: - Which streams: S3 and S4 as part of the audio stream group and S5 and S6 as part of the video stream group; - Semantics: "ALTC" (alternate connectivity) (to be confirmed) (NOTE 2) - Directionality: unspecified, hence default "bidirectional" at TerminationState level, however, finally profile at Stream level Stream endpoint T2(S1): - serves as aggregate for audio. Stream endpoint T2(S2): - serves as aggregate for video. |
| NOTE 1 – IP address realm indication via the ipdc/realm property (see [b-ITU-T H.248.41]). NOTE 2 – The "grouping semantic" ALTC isn't yet defined (and the call level group semantic ANAT is not used at the H.248 interface, see clause 8.6.3.4). Following semantic would be beneficial: the token ALTC indicates the MG a) that finally one of the two SEPs within the "ALTC stream group" will be removed again, b) that the StreamID list is ordered, first list item relates to "preferred IP connectivity" and the second one to "alternate IP connectivit", and c) that the final selection of the "preferred IP connection endpoint" is more likely. The MG could take into account that semantic concerning the underlying reservation and allocation of MG resources. NOTE 3 – The IP address realm indication element is actually mandated in all H.248 profiles for IP-IP media gateways, therefore part of this example here (due to its tight semantical correlation with ALTC). NOTE 4 – Allows to support "early media". NOTE 5 – ReserveGroup is explicitly disabled because not applied for IP transport resources. However, there might be on top multiple application flows, which could lead to the additional definition of multiple H.248 media groups (feasibility to be confirmed). {Editor's note (2015-06): usage of ReserveGroup needs to be clarified, still open ...} |  |

When the remote user equipment answers the "ALTC" request, then the MGC will modify the configuration of context C1. There might be following actions:

- in case of IPv6 selection:

- modification of group semantics at TerminationState level (or even complete removal of mgroup/groupse specification information (to be discussed));
{Editor's note (2015-06): Would right now assume that the groupse property should stay be just modified. Because T1 would continue using S1 for the audio related communication with its peer, whereas T2 would use its S3 for the ...(?)
T1.S1 would continue being interconnected to T2.S1 context internally, such that the specific "relationship" between T2.S1 and T2.S3 remains....}

- removal of SEPs T2(S4) and T2(S6);

- modification of grouping at LocalControl level of aggregated streams SEPs T2(S1) and T2(S2) (i.e., mgroup/stragg setting); and

- the usual completion of lacking information in LD/RDs of finally used IP connection endpoints SEPs T2(S3) and T2(S5);

- in case of IPv4 selection:

- similar steps as above.

### L.4.2	Benefits of stream grouping for ALTC

The evaluated ALTC stream grouping model is based on the usage of self-contained H.248 SEPs for preferred and alternate IP connection endpoint resources, and the possible decoupling of context-internal and context-external stream endpoint specifications, leading to following advantages:

- unambiguous usage of IP address realm indication possible, as required in conjunction with ALTC (here: ipdc v1 package used);

- ALTC grouping semantics could allow a different weighting of MG resources (see NOTE 2 in Table L.4.1);

- SEP-individual StreamMode settings for external streams;

- additional usage of ReserveGroup enabled multiple H.248 media groups should be principally feasible, e.g., related to protocol stack configurations on top of the IP transport connection (e.g., there might be two media groups enabled on T1(S1), which might be finally also used at e.g., T2(S2)); and

- finally, ALTC implies always removal of SEP(s) during the context establishment phase, which implies the creation of a SEP pair, - and the SEPP could be simply fixed via modification of property values of mgroup/stragg.

[End Proposed Correction]

END of ALTC changes

____________________