| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4694 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Chengdu (China), 8 - 12 June 2015 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.WEBRTC: update of profile guideline sections |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution provides updates for the profile guideline sections.

1	Discussion

Firstly, we defined some use case specific capability sets in Appendix I, which was so far containing just ideas and editor's notes. We are proposing three media gateway examples and one media server specific CS.

Secondly, we completed the clause on H.248 packages and package usage details.

Thirdly, we checked and updated clauses 11.1 to 11.13 concerning package element usage as well as general H.248.

Finally, we started to update the SDP clauses 11.15 and 11.16.

2	Proposal

It is proposed to accept the marked up changes below. Changes are marked up against AVD-4663, the input draft for this meeting.

Change proposal #1:

[Begin Proposed Correction]

# 11	H.248 profile specification guidelines

This clause provides guidelines for H.248 profile specifications. The structure follows the profile template according to Appendix III/[ITU-T H.248.1].

The template elements which are not applicable in this Recommendation are indicated by "Subject to profile specification".

Any profile guidelines are primarily dependent on the concerned network configuration and use case. The guidelines in this clause are therefore basically conditional. Some exemplary use cases are considered (as described in Appendix I), termed as capability sets CSA, CSB, CSC and CSD. Tag 'CS*' indicates unconditional profile elements, which are either generic because basic WebRTC capabilities or required for all four CS examples.

## 11.1	Profile Identification

Subject to profile specification.

## 11.2	Summary

Subject to profile specification.

## 11.3	Gateway Control Protocol Version

Subject to profile specification.

## 11.4	Connection Model

| Maximum number of contexts: | Subject to profile specification. |
| --- | --- |
| Maximum number of terminations per context: | Subject to profile specification. Examples: IF CSA OR CSB OR CSC THEN "2". IF CSD THEN "N". |
| Allowed termination type combinations in a context: | Subject to profile specification. |

## 11.5	Context Attributes

Subject to profile specification.

## 11.6	Terminations

Subject to profile specification.

## 11.7	Descriptors

### 11.7.1	TerminationState Descriptor

IF CS* THEN following table:

| TerminationState: Groupe Semantics (mgroup/groupse) | See clause 11.14.3.10. |
| --- | --- |
| TerminationState: … | … |

All other aspects related to TerminationState Descriptor (e.g., ServiceState, EventBuffer Control) are "Subject to profile specification".

### 11.7.2	Stream Descriptor

Subject to profile specification.

It has to be noted that an H.248 IP termination for service WebRTC may carry additional H.248 (de-)aggregation streams besides the legacy H.248 component streams. There are then additional, dedicated Stream Descriptors at (de-)aggregation stream level.

### 11.7.3	Events Descriptor

IF CSA OR CSB OR CSC THEN following table:

| Events settable on termination types and stream types: |  | Yes |  |  |
| --- | --- | --- | --- | --- |
| If yes | Event ID | Termination type | Stream type |
|  | See clause 11.14.3.4 - stnconfres/constate - stnconfres/confail | ALL except ROOT | IP-enabled |
|  | See clause 11.14.3.5 - tlsbsc/BNCChange | ALL except ROOT | DTLS-enabled |
|  | See clause 11.14.3.8 - sctpbcc/BNCChange | ALL except ROOT | SCTP-enabled |
|  | See clause 11.14.3.9 - sctpreset/detreset - sctpreset/result | ALL except ROOT | SCTP-enabled |

IF CSA THEN following table:

| Events settable on termination types and stream types: |  | Yes |  |  |
| --- | --- | --- | --- | --- |
| If yes | Event ID | Termination type | Stream type |
|  | See clause 11.14.3.3 - ostuncc/ccr - ostuncc/nprc | ALL except ROOT | IP-enabled |
| NOTE – These ostuncc events are only required for ICE full mode. |  |  |  |

IF CSA OR CSB THEN following table:

| Events settable on termination types and stream types: |  | Yes |  |  |
| --- | --- | --- | --- | --- |
| If yes | Event ID | Termination type | Stream type |
|  | See clause 11.14.3.13 - srtp/mke | ALL except ROOT | SRTP-enabled |
|  | See clause 11.14.3.16 - tlsm/mgea | ALL except ROOT | DTLS-enabled |
|  | See clause 11.14.3.18 - adr/rtac | ALL except ROOT | IP-enabled |

IF CSA OR CSD THEN following table:

| Events settable on termination types and stream types: |  | Yes |  |  |
| --- | --- | --- | --- | --- |
| If yes | Event ID | Termination type | Stream type |
|  | See clause 11.14.3.11 - dcep/detmess | ALL except ROOT | SCTP-enabled |
|  | See clause 11.14.3.10 - mcbalg/det | ALL except ROOT | SCTP-enabled |

All other aspects related to Events Descriptor (e.g., EventBuffer Control, KeepActive, Notification Behaviour) are "Subject to profile specification".

### 11.7.4	EventBuffer Descriptor

Subject to profile specification.

### 11.7.5	Signals Descriptor

IF CSA OR CSB OR CSC THEN following table:

| The setting of signals is dependent on termination or streams types: |  | Yes |  |
| --- | --- | --- | --- |
| If yes | Signal ID | Termination Type | Stream Type/ID |
|  | See clause 11.14.3.1 - ipnapt/latch | ALL except ROOT | IP-enabled |
|  | See clause 11.14.3.4 - stnconfres/contst | ALL except ROOT | IP-enabled |
|  | See clause 11.14.3.5 - tlsbsc/EstBNC - tlsbsc/RelBNC | ALL except ROOT | DTLS-enabled |
|  | See clause 11.14.3.8 - sctpbcc/EstBNC - sctpbcc/RelBNC | ALL except ROOT | SCTP-enabled |
|  | See clause 11.14.3.9 - sctpreset/initreset - sctpreset/resetresp | ALL except ROOT | SCTP-enabled |

IF CSA THEN following table:

| The setting of signals is dependent on termination or streams types: |  | Yes |  |
| --- | --- | --- | --- |
| If yes | Signal ID | Termination Type | Stream Type/ID |
|  | See clause 11.14.3.3 - ostuncc/scc - ostuncc/sacc | ALL except ROOT | IP-enabled |
| NOTE – These ostuncc signals are only required for ICE full mode. |  |  |  |

IF CSA OR CSB THEN following table:

| The setting of signals is dependent on termination or streams types: |  | Yes |  |
| --- | --- | --- | --- |
| If yes | Signal ID | Termination Type | Stream Type/ID |
|  | See clause 11.14.3.16 - tlsm/ mgcea | ALL except ROOT | DTLS-enabled |

IF CSA OR CSD THEN following table:

| The setting of signals is dependent on termination or streams types: |  | Yes |  |
| --- | --- | --- | --- |
| If yes | Signal ID | Termination Type | Stream Type/ID |
|  | See clause 11.14.3.11 - dcep/sndopen - dcep/dcopenresp | ALL except ROOT | SCTP-enabled |
|  | See clause 11.14.3.10 - mcbalg/sblm | ALL except ROOT | SCTP-enabled |

All other aspects related to Signals Descriptor (e.g., Signal Direction, Signal List) are "Subject to profile specification".

### 11.7.6	DigitMap Descriptor

Subject to profile specification.

### 11.7.7	Statistics Descriptor

IF CSB OR CSC OR CSD THEN none.

IF CSA THEN following table entries: see clauses clause 11.14.3.13, 11.14.3.17 and 11.14.3.19.

### 11.7.8	ObservedEvents Descriptor

Subject to profile specification.

### 11.7.9	Topology Descriptor

Subject to profile specification.

### 11.7.10	Error Descriptor

IF CS* THEN following table:

Error Codes sent by the MGC:

| Supported ITU-T H.248.8 Error Codes: | … |
| --- | --- |
| Supported Error Codes defined in packages: | #489 "Invalid aggregation and/or de-aggregation" |

Error Codes sent by the MG:

| Supported ITU-T H.248.8 Error Codes: | … |
| --- | --- |
| Supported Error Codes defined in packages: | #489 "Invalid aggregation and/or de-aggregation" |

IF CSA THEN following table:

Error Codes sent by the MGC:

| Supported ITU-T H.248.8 Error Codes: | … |
| --- | --- |
| Supported Error Codes defined in packages: | #488 "Incorrect stream endpoint interlinkage" |

Error Codes sent by the MG:

| Supported ITU-T H.248.8 Error Codes: | … |
| --- | --- |
| Supported Error Codes defined in packages: | #488 "Incorrect stream endpoint interlinkage" |

All other error codes are "Subject to profile specification".

## 11.8	Command API

NOTE – It is assumed that an Error Descriptor may be returned in any command reply.

### 11.8.1	Add

Subject to profile specification.

### 11.8.2	Modify

Subject to profile specification.

### 11.8.3	Subtract

Subject to profile specification.

### 11.8.4	Move

Subject to profile specification.

### 11.8.5	AuditValue

Subject to profile specification.

### 11.8.6	AuditCapabilities

Subject to profile specification.

### 11.8.7	Notify

Subject to profile specification.

### 11.8.8	ServiceChange

Subject to profile specification.

### 11.8.9	Manipulating and auditing context attributes

Subject to profile specification.

## 11.9	Generic command syntax and encoding

Subject to profile specification.

## 11.10	Transactions

Subject to profile specification.

NOTE – There is no impact on Transactions.

## 11.11	Messages

Subject to profile specification.

## 11.12	Transport

Subject to profile specification.

NOTE – Usage of bearer security may demand for an secured H.248 transport mode, too.

## 11.13	Security

Subject to profile specification.

## 11.14	Packages

### 11.14.1	Mandatory Package

Mandatory: specifies the packages that shall be supported in this profile.

Examples:

IF CSA OR CSB OR CSC THEN "following table":

| Mandatory packages: |  |  |  |
| --- | --- | --- | --- |
| Package name | PackageID | Version | Termination Types Supported (NOTE) |
| "IP NAT traversal package" [ITU-T H.248.37] | ipnapt (0x0099) | v1 | IP |
| "MG Act-as STUN server package" [ITU-T H.248.50] | mgastuns (0x00c2) | v1 | IP |
| "Originate STUN continuity check package" [ITU-T H.248.50] | ostuncc (0x00c3) | v1 | IP |
| "STUN consent freshness package" [ITU-T H.248.50] | stnconfres (0x0120) | v1 | IP |
| "TLS basic session control package" [ITU-T H.248.90] | tlsbsc (0x0117) | v1 | IP; Stream type: "DTLS-enabled UDP or TCP connection" |
| "SCTP basic connection control package" [ITU-T H.248.SCTP] | sctpbcc (0x0121) | v1 | IP; Stream type: "DTLS-enabled L4 connection" |
| "SCTP re-configuration stream reset package" [ITU-T H.248.SCTP] | sctpreset (0x0122) | v1 | IP; Stream type: "DTLS-enabled L4 connection" |
| "Media grouping package" [ITU-T H.248.STGROUP] | mgroup (0x011f) | v1 | IP |
| NOTE – Termination types: an H.248 profile might futher differentiate IP-based H.248 terminations in "WebRTC" and "non-WebRTC" terminations (see also clause 8.12). This note applies also for the tables in following clause 11.14.2. |  |  |  |

### 11.14.2	Optional Package

Examples:

IF CSA OR CSB THEN "following table":

| Optional packages: |  |  |  |
| --- | --- | --- | --- |
| Package name | PackageID | Version | Termination Types Supported |
| "Stream endpoint interlinkage package" [ITU-T H.248.92] | seplink (0x011b) | v2 | IP |
| "TLS capability negotiation package" [ITU-T H.248.90] (NOTE 5) | tlscn (0x0118) | v1 | IP |
| "TLS session maintenance package" [ITU-T H.248.90] (NOTE 6) | tlsm (0x0119) | v1 | IP |
| NOTE 5 – Applied for DTLS-enabled stream endpoints (in context of SCTP/DTLS/UDP protocol layering). NOTE 6 – Related to DTLS layer. |  |  |  |

Examples:

IF CSA OR CSB OR CSC THEN "following table":

| Optional packages: |  |  |  |
| --- | --- | --- | --- |
| Package name | PackageID | Version | Termination Types Supported |
| "Data channel establishment protocol support package" [this Recommendation] (NOTE 3) | dcep (0x0124) | v1 | IP |
| "Secure RTP Package" [ITU-T H.248.77] (NOTE 7) | srtp (0x0107) | v2 | IP |
| NOTE 4 – For WEBRTC data services (if required due to L4+ NAT traversal support; dependent on application protocol and other criteria). NOTE 7 – Selection of key management scheme for SRTP. |  |  |  |

{Editor’s note (2014-07): the requirement of "ICE for TCP" would imply support of TCP-enabled SEPs and consequently lead to the optional support of TCP-related H.248 packages according to H.248.84 and H.248.89. To be clarified.}

Examples:

IF CSA OR CSD THEN "following table":

| Optional packages: |  |  |  |
| --- | --- | --- | --- |
| Package name | PackageID | Version | Termination Types Supported |
| "MGC controlled bearer level ALG package" [ITU-T H.248.78] (NOTE 3, NOTE 4) | mcbalg (0x0108) | v2 | IP |
| "Data channel establishment protocol support package" [this Recommendation] (NOTE 3) | dcep (0x0124) | v1 | IP |
| NOTE 3 – There are two options related to the end-to-end control of WebRTC data channels: a) "out-of-band" method using call control signalling, or b) "inband" method. The first approach seems to be default for WebRTC support in SIP networks (such as IMS), and the baseline in this Recommendation. H.248 WebRTC gateways which would need to support the second method would require an H.248 profile with dcep v1 and mcbalg v2 packages support. NOTE 4 – In case of CLUE-based WebRTC conferencing control. |  |  |  |

Examples:

IF CSA THEN "following table":

| Optional packages: |  |  |  |
| --- | --- | --- | --- |
| Package name | PackageID | Version | Termination Types Supported |
| "Address reporting package" [ITU-T H.248.37] | adr (0x00ac) | v1 | IP |
| "Statistics for discarded packets due to latching package" [ITU-T H.248.37] | lstat (0x00e4) | v1 | IP |
| "Advanced SDP wildcarding package" [ITU-T H.248.39] (NOTE 1) | aswp (0x011c) | v1 | IP |
| "TLS traffic volume metrics package" [ITU-T H.248.90] (NOTE 6) | tlstv (0x011a) | v1 | IP |
| NOTE 1 – When MGC and MG agree to benefit from advanced SDP wildcarding. NOTE 6 – Related to DTLS layer. |  |  |  |

### 11.14.3	Package usage information

The following is a non-exhaustive list of package usage indications.

#### 11.14.3.1	IP NAPT traversal package

Example:

| Properties | Mandatory/ Optional | Used in command: | Supported values: | Provision-ed value: | Termination/ Stream Types Supported: |
| --- | --- | --- | --- | --- | --- |
| None. | - | - | - | - | - |
| Signals | Mandatory/ Optional | Used in command: |  | Duration provisioned value: |  |
| Latching (ipnapt/latch, 0x0099/0x0001) | M | ADD, MOD |  | - |  |
|  | Signal parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | napt (0x0001) | M | ALL | - |  |
| Events | Mandatory/ Optional | Used in command: |  |  |  |
| None. | - | - |  |  |  |
|  | Event parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
|  | ObservedEvent parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
| Statistics | Mandatory/ Optional | Used in command: | Supported values: |  | Termination/ Stream Types Supported: |
| None. | - | - | - |  | - |
| Error Codes | Mandatory/Optional |  |  |  |  |
| None. | - |  |  |  |  |

#### 11.14.3.2	MG act-as STUN server package

Example:

| Properties | Mandatory/ Optional | Used in command: | Supported values: | Provisioned value: | Termination/ Stream Types Supported: |
| --- | --- | --- | --- | --- | --- |
| Act-as STUN Server (mgastuns/astuns, 0x00c2/0x0001) | M | ADD, MOD | ALL | - | - |
| Signals | Mandatory/ Optional | Used in command: |  | Duration provisioned value: |  |
| None. | - | - |  | - |  |
|  | Signal parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
| Events | Mandatory/ Optional | Used in command: |  |  |  |
| None. | - | - |  |  |  |
|  | Event parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
|  | ObservedEvent parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
| Statistics | Mandatory/ Optional | Used in command: | Supported values: |  | Termination/ Stream Types Supported: |
| None. | - | - | - |  | - |
| Error Codes | Mandatory/Optional |  |  |  |  |
| None. | - |  |  |  |  |

#### 11.14.3.3	Originate STUN continuity check package

Example:

| Properties | Mandatory/ Optional | Used in command: | Supported values: | Provisioned value: | Termination/ Stream Types Supported: |
| --- | --- | --- | --- | --- | --- |
| Host Candidate Realm (ostuncc/hcr, 0x00c3/0x0001) | O | ADD, MOD | ALL | Yes | - |
| Signals | Mandatory/ Optional | Used in command: |  | Duration provisioned value: |  |
| Send Connectivity Check (ostuncc/scc, 0x00c3/0x0001) | M | ADD. MOD |  | not applicable |  |
|  | Signal parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | cntrl, (0x0001) | O | controlling, controlled | not applicable |  |
| Send Additional Connectivity Check (ostuncc/sacc, 0x00c3/0x0002) | Mandatory/ Optional | Used in command: |  | Duration provisioned value: |  |
|  | M | MOD |  | not applicable |  |
|  | Signal parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | cntrl, (0x0001) | O | controlling, controlled | not applicable |  |
| Events | Mandatory/ Optional | Used in command: |  |  |  |
| Connectivity Check Result (ostuncc/ccr, 0x00c3/0x0001) | M | ADD, MOD, NOTIFY |  |  |  |
|  | Event parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
|  | ObservedEvent parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | ctp, (0x0001) | M | ALL | not applicable |  |
| New Peer Reflexive Candidate (ostuncc/nprc, 0x00c3/0x0002) | Mandatory/ Optional | Used in command: |  |  |  |
|  | M | ADD, MOD, NOTIFY |  |  |  |
|  | Event parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
|  | ObservedEvent parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | can, (0x0001) | M | ALL | not applicable |  |
| Statistics | Mandatory/ Optional | Used in command: | Supported values: |  | Termination/ Stream Types Supported: |
| None. | - | - | - |  | - |
| Error Codes | Mandatory/Optional |  |  |  |  |
| None. | - |  |  |  |  |

#### 11.14.3.4	STUN consent freshness package

Example:

| Properties | Mandatory/ Optional | Used in command: | Supported values: | Provisioned value: | Termination/ Stream Types Supported: |
| --- | --- | --- | --- | --- | --- |
| None. | - | - | - | - | - |
| Signals | Mandatory/ Optional | Used in command: |  | Duration provisioned value: |  |
| Consent Test (stnconfres/contst, 0x0120/0x0001) | M | ADD. MOD |  | not applicable |  |
|  | Signal parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | tstint, (0x0001) | O | ALL | '5000 ms' |  |
| Events | Mandatory/ Optional | Used in command: |  |  |  |
| Consent State (stnconfres/constate, 0x0120/0x0001) | M | ADD, MOD, NOTIFY |  |  |  |
|  | Event parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | reqstate (0x0001) | O | ALL | 'B' |  |
|  | ObservedEvent parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | state, (0x0001) | M | ALL | not applicable |  |
| STUN Consent Request Failure (stnconfres/confail, 0x0120/0x0002) | Mandatory/ Optional | Used in command: |  |  |  |
|  | M | ADD, MOD, NOTIFY |  |  |  |
|  | Event parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
|  | ObservedEvent parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
| Statistics | Mandatory/ Optional | Used in command: | Supported values: |  | Termination/ Stream Types Supported: |
| None. | - | - | - |  | - |
| Error Codes | Mandatory/Optional |  |  |  |  |
| None. | - |  |  |  |  |

#### 11.14.3.5	TLS basic session control package

Example:

| Properties | Mandatory/ Optional | Used in command: | Supported values: | Provisioned value: | Termination/ Stream Types Supported: |
| --- | --- | --- | --- | --- | --- |
| Incoming security session establishment blocking (tlsbsc/bceb, 0x0117/0x0001) | O (NOTE 1) | ADD, MOD | ALL | "Un-blocked " | DTLS |
| Signals | Mandatory/ Optional | Used in command: |  | Duration provisioned value: |  |
| Establish BNC (tlsbsc/EstBNC, 0x0117/0x0001) | M | ADD. MOD |  | - |  |
|  | Signal parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
| Release BNC (tlsbsc/RelBNC, 0x0117/0x0002) | Mandatory/ Optional | Used in command: |  | Duration provisioned value: |  |
|  | O (NOTE 2) | ADD, MOD |  | - |  |
|  | Signal parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
| Events | Mandatory/ Optional | Used in command: |  |  |  |
| TLS session state change (tlsbsc/BNCChange, 0x0117/0x0001) | O (NOTE 3) | ADD, MOD, NOTIFY |  |  |  |
|  | Event parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | Type | M | Est (0x01), Rel (0x05) | - |  |
|  | ObservedEvent parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | Type | M | Est (0x01), Rel (0x05) | - |  |
| Statistics | Mandatory/ Optional | Used in command: | Supported values: |  | Termination/ Stream Types Supported: |
| None. | - | - | - |  | - |
| Error Codes | Mandatory/Optional |  |  |  |  |
| None. | - |  |  |  |  |
| NOTE 1 – Required for e.g. too early incoming DTLS messages (due to security threat), or delayed DTLS session establishment (due to multiple SDP offer/answer cycles, H.248 two-stage resource reservation, to await firstly successful L4 connectivity, etc.). NOTE 2 – When the MGC wants to explicitly trigger the DTLS bearer session release procedure (instead of the implicit trigger related to the removal of the H.248 stream (via a MODify.request or SUBtract.request command)). NOTE 3 – When the MGC wants to monitor the execution of DTLS bearer control procedures. |  |  |  |  |  |

#### 11.14.3.6	Stream endpoint interlinkage package

Example:

FIXTHIS

#### 11.14.3.7	MG located bearer level ALG package

Example:

FIXTHIS

#### 11.14.3.6	SCTP basic connection control package

Example:

| Properties | Mandatory/ Optional | Used in command: | Supported values: | Provisioned value: | Termination/ Stream Types Supported: |
| --- | --- | --- | --- | --- | --- |
| Incoming bearer connection establishment blocking (sctpbcc/bceb, 0x0121/0x0001) | O (NOTE 1) | ADD, MOD | ALL | "Un-blocked " | SCTP |
| SCTP StreamID (sctpbcc/sctpid (0x0121/0x0002) | M | ADD, MOD | ALL | None. | SCTP |
| Signals | Mandatory/ Optional | Used in command: |  | Duration provisioned value: |  |
| Establish BNC (sctpbcc/EstBNC, 0x0121/0x0001) | M | ADD. MOD |  | - |  |
|  | Signal parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
| Release BNC (sctpbcc/RelBNC, 0x0121/0x0002) | Mandatory/ Optional | Used in command: |  | Duration provisioned value: |  |
|  | O (NOTE 2) | ADD, MOD |  | - |  |
|  | Signal parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
| Events | Mandatory/ Optional | Used in command: |  |  |  |
| SCTP connection state change (sctpbcc/BNCChange, 0x0121/0x0001) | O (NOTE 3) | ADD, MOD, NOTIFY |  |  |  |
|  | Event parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | Type | M | Est (0x01), Rel (0x05) | - |  |
|  | ObservedEvent parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | Type | M | Est (0x01), Rel (0x05) | - |  |
| Statistics | Mandatory/ Optional | Used in command: | Supported values: |  | Termination/ Stream Types Supported: |
| None. | - | - | - |  | - |
| Error Codes | Mandatory/Optional |  |  |  |  |
| None. | - |  |  |  |  |
| NOTE 1 – Required for blocking incoming SCTP association establishment requests. NOTE 2 – When the MGC wants to explicitly trigger the SCTP association shutdown procedure (instead of the implicit trigger related to the removal of the H.248 stream (via a MODify.request or SUBtract.request command)). NOTE 3 – When the MGC wants to monitor the execution of SCTP bearer control procedures. |  |  |  |  |  |

#### 11.14.3.7	SCTP Re-configuration Stream Reset package

Example:

| Properties | Mandatory/ Optional | Used in command: | Supported values: | Provisioned value: | Termination/ Stream Types Supported: |
| --- | --- | --- | --- | --- | --- |
| None. | - | - | - | - | - |
| Signals | Mandatory/ Optional | Used in command: |  | Duration provisioned value: |  |
| Initiate Outgoing SCTP Stream Reset (sctpreset/initreset, 0x0122/0x0001) | M | ADD. MOD |  | - |  |
|  | Signal parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | sctpid (0x0001) | M | ALL | None. |  |
| Outgoing SCTP Stream Reset Response (sctpreset/resetresp, 0x0122/0x0002) | M | ADD. MOD |  | - |  |
|  | Signal parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | sctpid (0x0001) | M | ALL | None. |  |
| Events | Mandatory/ Optional | Used in command: |  |  |  |
| Detect outgoing SCTP stream reset (sctpreset/detreset, 0x0122/0x0001) | M | ADD, MOD, NOTIFY |  |  |  |
|  | Event parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | outresp (0x0001) | O (tbc) | ALL (tbc) | “accept” |  |
|  | ObservedEvent parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | sctpid (0x0001) | M | ALL | None. |  |
| Outgoing SCTP stream reset result (sctpreset/result, 0x0122/0x0002) | M | ADD, MOD, NOTIFY |  |  |  |
|  | Event parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | None. | - | - | - |  |
|  | ObservedEvent parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | sctpid (0x0001) | M | ALL | None. |  |
|  | result (0x0002) | M | ALL (tbc) | None. |  |
| Statistics | Mandatory/ Optional | Used in command: | Supported values: |  | Termination/ Stream Types Supported: |
| None. | - | - | - |  | - |
| Error Codes | Mandatory/Optional |  |  |  |  |
| None. | - |  |  |  |  |

#### 11.14.3.8	Media grouping package

Example:

| Properties | Mandatory/ Optional | Used in command: | Supported values: | Provisioned value: | Termination/ Stream Types Supported: |
| --- | --- | --- | --- | --- | --- |
| Group Semantics (mgroup/groupse, 0x011f/0x0001) | M | ADD, MOD | 'FID', 'BUNDLE’, SCTP’ | None | ALL (NOTE 1) |
| Stream Aggregation (mgroup/stragg, 0x011f/0x0002) | M (NOTE 2) | ADD, MOD | ALL | None | ALL (NOTE 1) |
| Stream De-aggregation (mgroup/strdeagg, 0x011f/0x0003) | M (NOTE 3) | ADD, MOD | ALL | None | ALL (NOTE 1) |
| Signals | Mandatory/ Optional | Used in command: |  | Duration provisioned value: |  |
| None. | - | - |  | - |  |
|  | Signal parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
| Events | Mandatory/ Optional | Used in command: |  |  |  |
| None. | - | - |  |  |  |
|  | Event parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
|  | ObservedEvent parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
| Statistics | Mandatory/ Optional | Used in command: | Supported values: |  | Termination/ Stream Types Supported: |
| None. | - | - | - |  | - |
| Error Codes | Mandatory/Optional |  |  |  |  |
| #489 | M |  |  |  |  |
| NOTE 1 – Some stream grouping semantic values are only applicable for specific protocols (stacks). NOTE 2 – Required for RTP media multiplexing ('BUNDLE'). NOTE 3 – Required for WebRTC data channel stack. |  |  |  |  |  |

#### 11.14.3.9	Data channel establishment protocol support package

Example:

| Properties | Mandatory/ Optional | Used in command: | Supported values: | Provisioned value: | Termination/ Stream Types Supported: |
| --- | --- | --- | --- | --- | --- |
| None. | - | - | - | - | - |
| Signals | Mandatory/ Optional | Used in command: |  | Duration provisioned value: |  |
| Send DCEP Open (dcep/sndopen, 0x0124/0x0001) | M | ADD. MOD |  | - |  |
|  | Signal parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | sctpid (0x0001) | M | ALL | None. |  |
|  | protocol (0x0002) | M (tbc) | ALL | None. |  |
|  | label (0x0003) | O | ALL | None. |  |
|  | chtype (0x0004) | M | ALL | None. |  |
|  | reli (0x0005) | M | ALL | None. |  |
|  | priority (0x0006) | M | ALL | None. |  |
| DCEP Open Response (dcep/ dcopenresp, 0x0124/0x0002) | M | ADD. MOD |  | - |  |
|  | Signal parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | sctpid (0x0001) | M | ALL | None. |  |
| Events | Mandatory/ Optional | Used in command: |  |  |  |
| Detect DCEP Messages (dcep/detmess, 0x0124/0x0001) | M | ADD, MOD, NOTIFY |  |  |  |
|  | Event parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | None. | - | - | - |  |
|  | ObservedEvent parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | sctpid (0x0001) | M | ALL | None. |  |
|  | protocol (0x0002) | M (tbc) | ALL | None. |  |
|  | label (0x0003) | O | ALL | None. |  |
|  | chtype (0x0004) | M | ALL | None. |  |
|  | reli (0x0005) | M | ALL | None. |  |
|  | priority (0x0006) | M | ALL | None. |  |
| Statistics | Mandatory/ Optional | Used in command: | Supported values: |  | Termination/ Stream Types Supported: |
| None. | - | - | - |  | - |
| Error Codes | Mandatory/Optional |  |  |  |  |
| None. | - |  |  |  |  |

#### 11.14.3.10	MGC controlled bearer level ALG package

Example:

| Properties | Mandatory/ Optional | Used in command: | Supported values: | Provisioned value: | Termination/ Stream Types Supported: |
| --- | --- | --- | --- | --- | --- |
| None. | - | - | - | - | - |
| Signals | Mandatory/ Optional | Used in command: |  | Duration provisioned value: |  |
| ## Send bearer level message (mcbalg/sblm, 0x0108/0x0001) | M | ADD. MOD |  | - |  |
|  | Signal parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | Message content (mc) | M | ALL | - |  |
|  | ## Sent application protocol (sap) | ## O | ALL | - |  |
|  | ## Label (lbl) | O | ALL | - |  |
| Events | Mandatory/ Optional | Used in command: |  |  |  |
| Detect bearer level message (mcbalg/det, 0x0108/0x0001) | M | ADD, MOD, NOTIFY |  |  |  |
|  | Event parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | Protocol Filter (pf) | M | ## TBD | ## TBD |  |
|  | Message Filter (mf) | O | ## ALL | "*" |  |
|  | Forwarding Flag (ff) | O | ## ALL | FALSE |  |
|  | Enhanced Protocol Filter (ehpf) | O | ## TBD | TBD |  |
|  | Label (lbl) | O | ## ALL | - |  |
|  | ObservedEvent parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | Message content (mc) | M | ALL | - |  |
|  | Detected protocol (dtp) | O | ALL | - |  |
|  | Label (lbl) | O | ALL | - |  |
| Statistics | Mandatory/ Optional | Used in command: | Supported values: |  | Termination/ Stream Types Supported: |
| None. | - | - | - |  | - |
| Error Codes | Mandatory/Optional |  |  |  |  |
| None. | - |  |  |  |  |

#### 11.14.3.11	Stream endpoint interlinkage package

Example:

| Properties | Mandatory/ Optional | Used in command: | Supported values: | Provisioned value: | Termination/ Stream Types Supported: |
| --- | --- | --- | --- | --- | --- |
| Interlinkage topology (seplink/linktopo, 0x011b/0x0001) | M | ADD, MOD | ALL | _ | - |
| Signals | Mandatory/ Optional | Used in command: |  | Duration provisioned value: |  |
| None. | - | - |  | - |  |
|  | Signal parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
| Events | Mandatory/ Optional | Used in command: |  |  |  |
| None. | - | - |  |  |  |
|  | Event parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
|  | ObservedEvent parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
| Statistics | Mandatory/ Optional | Used in command: | Supported values: |  | Termination/ Stream Types Supported: |
| None. | - | - | - |  | - |
| Error Codes | Mandatory/Optional |  |  |  |  |
| #488 | - |  |  |  |  |

#### 11.14.3.12	MG located bearer level ALG package

Example:

| Properties | Mandatory/ Optional | Used in command: | Supported values: | Provisioned value: | Termination/ Stream Types Supported: |
| --- | --- | --- | --- | --- | --- |
| Protocol type bearer level ALG (mgbalg/ptbalg, 0x011d/0x0001) | M | ADD, MOD | ALL | _"OFF" | - |
| Upper layer protocol filter (mgbalg/ulpf, 0x011d/0x0002) | O (NOTE) | ADD, MOD | ALL | WebRTC related codepoints only. | "0" |
| Upper Layer Enhanced Protocol Filter (mgbalg/ulepf, 0x011d/0x0005) | O (NOTE) | ADD, MOD | ALL | WebRTC related codepoints only. | - |
| Source of replaced source address information part (mgbalg/sosaip, 0x011d/0x0003) | O (NOTE) | ADD, MOD | ALL | "SD" | - |
| Source of replaced destination address information part (mgbalg/sodaip, 0x011d/0x0004) | O (NOTE) | ADD, MOD | ALL | "SD" | - |
| Signals | Mandatory/ Optional | Used in command: |  | Duration provisioned value: |  |
| None. | - | - |  | - |  |
|  | Signal parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
| Events | Mandatory/ Optional | Used in command: |  |  |  |
| None. | - | - |  |  |  |
|  | Event parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
|  | ObservedEvent parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
| Statistics | Mandatory/ Optional | Used in command: | Supported values: |  | Termination/ Stream Types Supported: |
| None. | - | - | - |  | - |
| Error Codes | Mandatory/Optional |  |  |  |  |
| None. | - |  |  |  |  |
| NOTE - When B-ALG service configuration is provisioned in H.248 MG. |  |  |  |  |  |

#### 11.14.3.13	Secure RTP package

{Editor’s note (2015-06): the table content is preliminary due to current revision of H.248.77.}

Example:

| Properties | Mandatory/ Optional | Used in command: | Supported values: | Provisioned value: | Termination/ Stream Types Supported: |
| --- | --- | --- | --- | --- | --- |
| Supported Encryption Transforms (srtp/set, 0x0107/0x0001) | O | AuditValue | ALL | "MG supported encryption transforms" | Root only |
| Supported Authentication Transforms (srtp/sat, 0x0107/0x0002) | O | AuditValue | ALL | "MG supported encryption transforms" | Root only |
| Key Management Scheme (srtp/km, 0x0107/0x0003) | M | ADD, MOD | ALL | "None" or "DTLS-SRTP" | - |
| Key Lifetime Expiry Behaviour (srtp/kleb, 0x0107/0x0004) | O | ADD, MOD | ALL | - | - |
| Signals | Mandatory/ Optional | Used in command: |  | Duration provisioned value: |  |
| None. | - | - |  | - |  |
|  | Signal parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
| Events | Mandatory/ Optional | Used in command: |  |  |  |
| Master Key Expiry (srtp/mke, 0x0107/0x0001) | M | ADD, MOD, NOTIFY |  |  |  |
|  | Event parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | rtpw (0x0001) | O | ALL | - |  |
|  | rtcpw (0x0002) | O | ALL | - |  |
|  | ObservedEvent parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | ke (0x0001) | O | ALL | - |  |
| Statistics | Mandatory/ Optional | Used in command: | Supported values: |  | Termination/ Stream Types Supported: |
| Number of Replayed Packets (srtp/replay, 0x0107/0x0001) | O | ADD, MOD, SUBTRACT, NOTIFY | ALL |  | SRTP-enabled only |
| Number of Authentication Failures (srtp/authfail, 0x0107/0x0002) | O | ADD, MOD, SUBTRACT, NOTIFY | ALL |  | SRTP-enabled only |
| Sent SRTP Packets Protected by Master Key (srtp/srpk, 0x0107/0x0003) | O | ADD, MOD, SUBTRACT, NOTIFY | ALL |  | SRTP-enabled only |
| Sent SRTCP Packets Protected by Master Key (srtp/scpk, 0x0107/0x0004) | O | ADD, MOD, SUBTRACT, NOTIFY | ALL |  | SRTP-enabled only |
| Received SRTP Packets Protected by Master Key (srtp/rrpk, 0x0107/0x0005) | O | ADD, MOD, SUBTRACT, NOTIFY | ALL |  | SRTP-enabled only |
| Received SRTCP Packets Protected by Master Key (srtp/rcpk, 0x0107/0x0006) | O | ADD, MOD, SUBTRACT, NOTIFY | ALL |  | SRTP-enabled only |
| Error Codes | Mandatory/Optional |  |  |  |  |
| None. | - |  |  |  |  |

#### 11.14.3.14	Advanced SDP wildcarding package

Example:

| Properties | Mandatory/ Optional | Used in command: | Supported values: | Provisioned value: | Termination/ Stream Types Supported: |
| --- | --- | --- | --- | --- | --- |
| Advanced Wildcarding Support (aswp/aws, 0x011c/0x0001) | M | AuditValue | ALL | - | Root only |
| Signals | Mandatory/ Optional | Used in command: |  | Duration provisioned value: |  |
| None. | - | - |  | - |  |
|  | Signal parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
| Events | Mandatory/ Optional | Used in command: |  |  |  |
| None. | - | - |  |  |  |
|  | Event parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
|  | ObservedEvent parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
| Statistics | Mandatory/ Optional | Used in command: | Supported values: |  | Termination/ Stream Types Supported: |
| None. | - | - | - |  | - |
| Error Codes | Mandatory/Optional |  |  |  |  |
| None. | - |  |  |  |  |

#### 11.14.3.15	TLS capability negotiation package

Example:

| Properties | Mandatory/ Optional | Used in command: | Supported values: | Provisioned value: | Termination/ Stream Types Supported: |
| --- | --- | --- | --- | --- | --- |
| TLS Domain Profile Identifier (tlscn/dpid, 0x0118/0x0001) | O (NOTE 1) | ADD, MOD, AUDITVALUE | ALL | _ | Root or non-Root '(D)TLS', but not both |
| TLS Versions (tlscn/tlsv, 0x0118/0x0002) | O (NOTE 1) | ADD, MOD, AUDITVALUE | ALL | _ | Root or non-Root '(D)TLS', but not both |
| Chipher Suites (tlscn/cs, 0x0118/0x0003) | O (NOTE 1) | ADD, MOD, AUDITVALUE | ALL | _ | Root or non-Root '(D)TLS', but not both |
| Compression Methods (tlscn/cm, 0x0118/0x0004) | O (NOTE 1) | ADD, MOD, AUDITVALUE | ALL | _ | Root or non-Root '(D)TLS', but not both |
| Support for Renegotiation of the Security Context (tlscn/srsc, 0x0118/0x0005) | O (NOTE 1) | ADD, MOD, AUDITVALUE | ALL | _ | Root or non-Root '(D)TLS', but not both |
| Renegotiation Period (tlscn/rp, 0x0118/0x0006) | O (NOTE 1) | ADD, MOD, AUDITVALUE | ALL | _ | Root or non-Root '(D)TLS', but not both |
| Client Authentication Required (tlscn/car, 0x0118/0x0007) | O (NOTE 1) | ADD, MOD, AUDITVALUE | ALL | _ | Root or non-Root '(D)TLS', but not both |
| Signals | Mandatory/ Optional | Used in command: |  | Duration provisioned value: |  |
| None. | - | - |  | - |  |
|  | Signal parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
| Events | Mandatory/ Optional | Used in command: |  |  |  |
| None. | - | - |  |  |  |
|  | Event parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
|  | ObservedEvent parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
| Statistics | Mandatory/ Optional | Used in command: | Supported values: |  | Termination/ Stream Types Supported: |
| None. | - | - | - |  | - |
| Error Codes | Mandatory/Optional |  |  |  |  |
| None. | - |  |  |  |  |
| NOTE 1 – When tagged as 'optional', then property value(s) needs to be provisioned. |  |  |  |  |  |

#### 11.14.3.16	TLS session maintenance package

Example:

| Properties | Mandatory/ Optional | Used in command: | Supported values: | Provisioned value: | Termination/ Stream Types Supported: |
| --- | --- | --- | --- | --- | --- |
| None. | - | - | - | - | - |
| Signals | Mandatory/ Optional | Used in command: |  | Duration provisioned value: |  |
| MGC triggered TLS error alert (tlsm/mgcea, 0x0119/0x0001) | M | ADD. MOD |  | - |  |
|  | Signal parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | al (0x0001) | M | ALL | - |  |
|  | ad (0x0002) | M | ALL | - |  |
| Events | Mandatory/ Optional | Used in command: |  |  |  |
| TLS error alert (tlsm/mgea, 0x0119/0x0001) | M | ADD, MOD, NOTIFY |  |  |  |
|  | Event parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | None. | - | - | ## - |  |
|  | ObservedEvent parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | blai (0x0001) | M | ALL | - |  |
|  | eat (0x0002) | M | ALL | - |  |
| Statistics | Mandatory/ Optional | Used in command: | Supported values: |  | Termination/ Stream Types Supported: |
| None. | - | - | - |  | - |
| Error Codes | Mandatory/Optional |  |  |  |  |
| None. | - |  |  |  |  |

#### 11.14.3.17	TLS traffic volume metrics package

Example:

| Properties | Mandatory/ Optional | Used in command: | Supported values: | Provisioned value: | Termination/ Stream Types Supported: |
| --- | --- | --- | --- | --- | --- |
| None. | - | - | - | - | - |
| Signals | Mandatory/ Optional | Used in command: |  | Duration provisioned value: |  |
| None. | - | - |  | - |  |
|  | Signal parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
| Events | Mandatory/ Optional | Used in command: |  |  |  |
| None. | - | - |  |  |  |
|  | Event parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
|  | ObservedEvent parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
| Statistics | Mandatory/ Optional | Used in command: | Supported values: |  | Termination/ Stream Types Supported: |
| NOTE 1 (tlstv/*, 0x011a/*) | O | ADD, MOD, SUBTRACT, NOTIFY | ALL |  | (D)TLS enabled |
| Error Codes | Mandatory/Optional |  |  |  |  |
| None. | - |  |  |  |  |
| NOTE 1 – There are twenty statistics available, where usually just a subset is often sufficient. See clause 12.6.1.3/[ITU-T H.248.90] on "Profile selection guidelines". |  |  |  |  |  |

#### 11.14.3.18	Address reporting package

Example:

| Properties | Mandatory/ Optional | Used in command: | Supported values: | Provisioned value: | Termination/ Stream Types Supported: |
| --- | --- | --- | --- | --- | --- |
| Current Remote Transport Address Value (adr/crta, 0x00ac/0x0001) | M | ADD, MOD | ALL | None. | IP |
| Signals | Mandatory/ Optional | Used in command: |  | Duration provisioned value: |  |
| None. | - | - |  | - |  |
|  | Signal parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
| Events | Mandatory/ Optional | Used in command: |  |  |  |
| Remote Transport Address Change (adr/rtac, 0x00ac/0x0003) | M | ADD, MOD, NOTIFY |  |  |  |
|  | Event parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | None. | - | - | - |  |
|  | ObservedEvent parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | nrta (0x0001) | M | ALL | - |  |
| Statistics | Mandatory/ Optional | Used in command: | Supported values: |  | Termination/ Stream Types Supported: |
| None. | - | - | - |  | - |
| Error Codes | Mandatory/Optional |  |  |  |  |
| None. | - |  |  |  |  |

#### 11.14.3.19	Latch statistics package

Example:

| Properties | Mandatory/ Optional | Used in command: | Supported values: | Provisioned value: | Termination/ Stream Types Supported: |
| --- | --- | --- | --- | --- | --- |
| None. | - | - | - | - | - |
| Signals | Mandatory/ Optional | Used in command: |  | Duration provisioned value: |  |
| None. | - | - |  | - |  |
|  | Signal parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
| Events | Mandatory/ Optional | Used in command: |  |  |  |
| None. | - | - |  |  |  |
|  | Event parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
|  | ObservedEvent parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
| Statistics | Mandatory/ Optional | Used in command: | Supported values: |  | Termination/ Stream Types Supported: |
| Discarded Packets (lstat/dp, 0x00e4/0x0001) | M | ADD, MOD, SUBTRACT, NOTIFY | ALL |  | IP |
| Error Codes | Mandatory/Optional |  |  |  |  |
| None. | - |  |  |  |  |

## 11.15	Mandatory support of SDP and ITU-T H.248.1 Annex C information elements

At least the following:

| Supported Annex C and SDP information elements: |  |  |
| --- | --- | --- |
| Information Element | Annex C Support | SDP Support |
| "m="-line <type> | not supported (in this example) | Value(s): - application-aware: 'audio', 'video and 'application' - application-agnostic: '-' |
| "m="-line <proto> | not supported (in this example) | Value(s): 'RTP/SAVPF', '###/SCTP' Purpose: H.248 components streams at H.248 WebRTC termination |
|  |  | Value(s): ALL IANA registered codepoints with "y" protocol Purpose: ### Dependent on ### - application-aware: ### - application-agnostic: ### |
| "m="-line <fmt> | not supported (in this example) | Value: 'webrtc-datachannel' |
| "a=fingerprint:" | not supported (in this example) | Value(s): all Purpose: DTLS authentication procedures |
| " a=fmtp:webrtc-datachannel:" |  | Value(s): all Purpose: Usage of an SCTP stream as "WEBRTC data channel" |
| " a=sctp-port" |  | Value(s): all Purpose: The actual SCTP port value of the SCTP association carried over DTLS/UDP, which is used for "WEBRTC data channels" |
| "a=dcmap:<SCTP Stream ID>" |  | Value(s): all Purpose: application-agnostic WebRTC data channels |
| "a=dcmap:<SCTP Stream ID> subprotocol="…"" |  | Value(s): MSRP, BFCP, T.140 (tbc) Purpose: application-aware WebRTC data channels |
| "a=dcsa:…" |  | Value(s): all Purpose: application-specific information of a particular subprotocol asWEBRTC data channel |

Rest: subject to profile specification.

## 11.16	Optional support of SDP and ITU-T H.248.1 Annex C information elements

At least the following:

| Supported Annex C and SDP information elements: |  |  |
| --- | --- | --- |
| Information Element | Annex C Support | SDP Support |
| "a=setup:" | not supported (in this example) | Not applicable for H.248 streams of H.248 WebRTC terminations (tbc). |
| "a=rtcp-size:" |  |  |
| "a=rtcp-fb:" |  |  |
| NOTE 1 – FIXTHIS |  |  |

IF CSA OR CSB OR CSC THEN "following table":

| Supported Annex C and SDP information elements: |  |  |
| --- | --- | --- |
| Information Element | Annex C Support | SDP Support |
| "a=ice-ufrag:" | not supported (in this example) | Value(s): all Purpose: |
| "a=ice-pwd:" |  | Value(s): all Purpose: |

IF CSA OR CSB THEN "following table":

| Supported Annex C and SDP information elements: |  |  |
| --- | --- | --- |
| Information Element | Annex C Support | SDP Support |
| "a=group:BUNDLE" (tbc) | not supported (in this example) | Value(s): all Purpose: general media grouping using "bundle" |
| "a=mid:" (tbc) |  | Value(s): all Purpose: general media grouping using "bundle" |
| "a=rtcp-mux": |  | Value(s): all Purpose: for transport multiplexed RTP traffic |
| "a=rtcp:" |  | Value(s): all Purpose: optional, if transport unmultiplexed mode for RTP and explicit RTCP transport address allocation |
| "a=ssrc:" |  | Value(s): all Purpose: |

Rest: subject to profile specification.

## 11.17	Procedures

The initial release of this Recommendation focuses on the previous profile elements. Specific guidelines for the procedural section are for further studies with exception of following package-independent procedures.

#### 11.17.1	Package-independent NAT-T procedures

#### 11.17.1.1	Support for MG terminated STUN-based connectivity checks

IF CSA OR CSB OR CSC THEN the procedures according clause 10.1 of [ITU-T H.248.50].

Appendix I

Use case specific capability sets – Some examples

(This appendix does not form an integral part of this Recommendation)

## I.1	Overview

Profile content is mainly use case dependent (given be the inherent motivation of "protocol profiling"). In order to demonstrate profile specification guidelines in clause 11, this Recommendation considers some exemplary use cases (see Table I.1):

- Capability set 'A' (CSA) – a H.248 WEBRTC gateway as positioned in use case #1 (see clause 7.1.1):

Use case #1 is based on the assumption that the WebRTC endpoint could request and use all mandatory WebRTC capabilities as defined by the IETF / W3C. Thus, all mandatory and optional capabilities according to [b-IETF rtcweb-gateway] are basically in scope. The H.248 WebRTC gateway does then need to support all mandatory requirements as listed in clause 8. CSA covers also all optional features for enhanced gateway operation.

- Capability set 'B' (CSB) – a H.248 WebRTC gateway as positioned in use case #2 with scope on the NGN/IMS domain located WEBRTC endpoint (see clause 7.1.2).

- Capability set 'C' (CSC) – a H.248 WebRTC gateway again for use case #2, but a lightweighted capability set in comparison to CSB, which reflects the service focus at market introduction phase..

Capability set 'A' (CSA) is therefore a superset of capability set 'B' (CSB), and 'B' a superset of capability set 'C' (CSC). Hence, reflecting the usual evolution and phased introduction of new communication services.

Furthermore, besides pure H.248 WebRTC gateways with point-to-point connection models due to their network positionings at the edge, access or peering level:

- Capability set 'D' (CSD) – a H.248 WebRTC media server with primary scope on WebRTC-based conferencing topologies and support. The correspondent column in Table I.1 indicates a lot of capabilities as not applicable (n.a.), e.g. due to assumptions that any kind of NAT traversal support is already provided at the network access segment of the IP paths.

Table I.1 – Examples of use case specific capability sets (CS)

| Capability | CSA |  | CSB |  | CSC |  | CSD |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NAT-T I: ICE/STUN for UDP | X | X | X | n.a. |
| NAT-T II: ICE/STUN for TCP | X | X | X | n.a. |
| NAT-T III: ICE refreshs during active call phase | X | X | - | n.a. |
| NAT-T IV: ICE lite mode only | X | X | X | n.a. |
| NAT-T V: ICE full mode | X | - | - | n.a. |
| NAT-T VI: latching | X | X | X | n.a. |
| NAT-T VII: B-ALG for L4+ support for WebRTC data applications | X | X | X | n.a. |
| Multiplexing I: RTP transport multiplexing | X | - | - | X |
| Multiplexing II: RTP media multiplexing | X | - | - | X |
| Multiplexing III: UDP payload multiplexing | X | X | - | X |
| H.248 MG type I: WebRTC gateway | X | X | X | - |
| H.248 MG type II: WebRTC media server (media resource function) | - | - | - | X |
| WebRTC service profile I: audio & video only | X | X | X | X |
| WebRTC service profile II: audio, video and MSRP-based data only | X | X | X | X |
| WebRTC service profile III: audio & video and multiple data channels | X | X | - | X |
| WebRTC service profile IV: CLUE-based conferencing control | X | - | - | X |
| WebRTC service profile V: performance monitoring | X | - | - | - |
| WebRTC data channel control I: Out-of-band control | X | X | X | X |
| WebRTC data channel control II: Inband control | X | - | - | X |
| WebRTC data channel control III: reset of data channels | X | X | - | X |
| Multiple SRTP key management schemes (due to non-WebRTC SRTP) | X | X | X | X |
| Enhanced DTLS support (negotiation, maintenance & monitoring) | X | X | - | - |
| Enhanced H.248 control I: stream interlinkage support | X | X | - | n.a. |
| Enhanced H.248 control II: advanced wildcarding | X | X | - | n.a. |

[End Proposed Correction]

____________________