| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4699 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Chengdu (China), 8 - 12 June 2015 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.Sup13¦H.248.CLOUD: Gateway term |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution provides an update proposal based on a pending action from Draft H.248.CLOUD.

1	Discussion

Term "H.248 gateway" needs to be defined because used in other terms as defined by H.248.CLOUD, see following Editor's Note:

{Editor's note (2014-07): we a definition for H.248 gateway, thus should add following definition to H.Sup.Term (due to the lack of such a term in H.248.1):

H.248 gateway: the two coupled functional entities of an H.248 media gateway (MG) entity (see clause 3.2.3/[ITU-T H.248.1] and its associated H.248 media gateway controller (MGC) entity (see clause 3.2.4/[ITU-T H.248.1].

NOTE – The H.248 gateway represents a specific decomposition model in splitting an original monolithic user/control plane related network element in two, plane specific network elements (MG and MGC), interconnected by the H.248 protocol.

Contributions are solicited.}

The term could be now added to H.Sup13 due to the agreed work item.

2	Proposal

It is proposed to accept the marked up changes below. Changes are marked up against AVD-4669 as input draft to this meeting.

Change proposal #1 – Draft H.248.CLOUD (AVD-4664):

[Begin Proposed Correction]

3.2.6	H.248 packet gateway: an H.248 gateway entirely embedded in an all-packet network environment, hence, with support of H.248 ephemeral terminations only (and without any need for H.248 physical terminations support).

NOTE 1 – The qualified term H.248 packet media gateway denotes the MG part of the packet gateway (and correspondingly for the MGC part).

NOTE 2 – The qualified term H.248 IP gateway denotes the bearer technology specific H.248 packet gateway in packet-switched networks using the IP as network layer protocol.

NOTE 3 – The H.248 packet gateway is also known as H.248 border gateway for specific H.248 profile applications ("the notion of 'border' indicates the gateway positioning between network domains (as consituted by IP address realms)").

[End Proposed Correction]

Change proposal #2 – H.Sup13 (AVD-4669):

[Begin Proposed Correction]

## 3.2	Terms defined in this Supplement

NOTE – The terms in this clause are tabulated in order to maintain the usual alphabetical order when adding additional terms in future releases of this Supplement.

This Supplement defines the terms listed in Table 3.2.

| Table 3.2 – Terms defined in this Supplement |  |
| --- | --- |
| Term | Definition |
| Endpoint | For the purpose of ITU-T H.248.x-Recommendations, an endpoint is an entity that generates and/or terminates information streams. The term endpoint may be further qualified by prefixes or suffixes to provide more detail on the type of endpoint. NOTE – An endpoint is equivalent to an OSI end system. |
| ITU-T H.248 gateway | The two coupled functional entities of an H.248 media gateway (MG) entity (see clause 3.2.3/[ITU-T H.248.1] and its associated H.248 media gateway controller (MGC) entity (see clause 3.2.4/[ITU-T H.248.1]. NOTE – The H.248 gateway represents a specific decomposition model in splitting an original monolithic user/control plane related network element in two, plane specific network elements (MG and MGC), interconnected by the H.248 protocol. |
| ITU-T H.248 master-slave control relationship | The ITU-T H.248 decomposed gateway follows a hierarchical control model in the network control plane (primarily related to call service control and media/bearer control functions). The control hierarchy constitutes a master-slave relationship, with the MGC as master and the MG as the slave entity. NOTE – [b-ITU-T Q.Sup31] and [b-ITU-T Q.Sup32] Annexes C.2 or C.3 provide examples of functional-to-physical mapping models according to the ITU-T H.248 master-slave control relationship. |
| MG autonomous mode | For a particular function, the MGC delegates some (but not all) service specific control decisions down to the MG level. The MG provides a local decision function. The MG autonomous control is limited to decisions with MG available information only. |
| MGC-strictly controlled mode | For a particular function, the control decisions are exclusively under MGC responsibility, the MG degenerates to a pure execution unit. |
| stream endpoint tuple (SEPT) | The generalization of a stream endpoint pair (SEPP) towards multiple associated stream endpoints (SEPs) within the same Context. All SEPs share the same StreamID value. The stream topology is given by the topology descriptor settings. |

[End Proposed Correction]

____________________