| Rapporteur Meeting of Questions 2, 3, 5 and 21/16 |  |  |  | AVD-4707 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Chengdu, 8 - 12 June 2015 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Huawei |  |  |  |
| Title: | H.248.STGROUP: Stream level interactions |  |  |  |
| Purpose: | Proposal |  |  |  |

1.	Introduction

This contribution proposes updates to H.248.STGROUP to describe procedures for Stream property interaction between aggregation/de-aggregation and component streams.

# 2. Discussion

At the February Q3/16 meeting an editor’s note was added to clause 8.6.8/[ITU-T H.248.STGROUP] regarding stream level interactions, i.e.:

{Editor’s note: There may be H.248 stream level interactions, i.e., the contents between Stream Descriptors of component streams and the Stream Descriptor of aggregation / de-aggregation streams could overlap, leading to semantical interaction issues. Ee.g. with property StreamMode, there might be following valid use cases:

UC#1 ("stream group global settings"): StreamMode set on (de-)aggregation stream level and omitted at component stream level:

=> semantic: all component streams inherit the setting from the (de-)aggregation stream;

UC#2 ("component stream local settings"): StreamMode set on component stream level and omitted at (de-)aggregation stream level:

=> semantic: all component streams could apply individual stream modes;

Interaction issues: IF StreamMode is signalled at both stream levels AND IF StreamMode settings would contradict.

This is a concrete example for one element of a Stream Descriptor, however, in general it could affect all elements of Stream Descriptors. For further studies.}

Given the number of existing and possible future H.248 stream related properties in LocalControl, Local and Remote Descriptors rather than describing the interaction between each one, the goal should be to define a general rule for interaction.

H.248.STGROUP currently defines rules for interaction for Signal, Events and Statistics between aggregation/de-aggregation stream and their component streams. In general setting an Event, Signal or Statistic on an aggregation/de-aggregation stream will apply to all the component streams. Setting an Event, Signal or Statistic on a component stream will only apply to that stream.

However this behaviour is problematic when it comes to Stream interactions. Some issues with regards to property inheritance are described below:

- Property defaults: According to H.248 the default state of a H.248 stream is inactive. This would create potential conflicts with the aggregation/de-aggregation stream. For example: if the aggregation stream was “send/receive” and a component stream was added then would it automatically have the value changed by the MG from “inactive” to “send/receive”. This same issue applies to other properties as well, e.g. ReserveValue/Group etc.

- Lack of individual control: If aggregation/de-aggregation streams properties apply to all the component streams then it could be problematic when one of the streams does not need to inherit that property. Procedures would need to be defined allowing properties to be removed from the component streams.

- Mandatory parameters: If component streams inherit properties not applicable to them but the property (or SDP element) is mandatory then an alternate value may need to be set.

- Different internal/external behaviour: The de-aggregation stream and aggregation component streams are typically external streams and thus need transport related information associated with them. Whereas de-aggregation component streams and the aggregation stream are internal streams and therefore do not require transport information. Thus the stream interaction behaviour may need to be different between aggregation and de-aggregation streams.

- Compact messages versus complexity: Part of the reason why H.248.STGROUP defines that Signal/Event/Statistics on aggregation/de-aggregation streams apply to the component stream is to minimise the size of messages. However if the MGC is required to make changes to each component streams due to 1-4 above then it not clear that messages will be significantly smaller for the added complexity.

Given the potential problems of property inheritance between aggregation and de-aggregation streams and their components the Contributors believe that the stream properties should be independent. For a particular property to apply to both the aggregation or de-aggregation streams and a component stream that property must be set on both streams.

This then leads to the issue of what happens if the property values are conflicting. The solution to this issue could be approached by considering how stream data is passed into the H.248, i.e. from the external stream, to the internal stream and then into the Context and vice versa. The data is processing based on a staged approach. The effect of the property setting would depend on the values that were set on the external and internal streams as this dictates the order of processing.

The Contributors propose to include this approach in H.248.STGROUP. More details are found in the proposal clause below.

# 3 Proposal

It is proposed to accept the following changes. The changes are marked up against TD 233R2/WP1 from the Geneva, February Q3/16 Rapporteurs meeting. Note: to facilitate readability only the deletion of the editor’s note has been shown as change marked.

Draft new Recommendation ITU-T H.248.xx (ex H.248.STGROUP)

…

### 8.6.8	Stream Level Interactions

H.248 Stream properties may be sent against aggregation/de-aggregation Streams and/or their component streams. The setting of properties is independent. That is, the inclusion of a property in a LocalControl, Local or Remote Descriptor in an aggregation or de-aggregation Stream does not mean that this property is included in the component Stream’s Descriptors. Properties will be applied to the data stream dependent on the direction (e.g. external->internal->context or context->internal->external) and whether the stream is an aggregation/de-aggregation stream or a component. The sub-clauses below provide more detailed procedures.

Note – The use of Streammode in the sub-clauses below is to illustrate the applied principles. These principles can be applied to any Stream property.

#### 8.6.8.1 De-aggregation Streams

Data received externally at the de-aggregation stream is first processed using the de-aggregation Stream properties. As data is passed to the component streams it is processed according the relevant component Stream’s properties. The data is then passed into the Context.

Data received on a component Stream from the Context is processed against the component’s Stream properties. The data is then passed to the de-aggregation stream and then processed against the de-aggregation Stream properties before being sent externally.

Figure 1 below shows the procedures using the Streammode property with a de-aggregation Stream as an example.

Key:

Sid StreamID

SR Send Receive

SO Send Only

RO Receive Only

Data from External to Context

Data from Internal Context

Figure 1 – De-aggreation Stream property interaction: Streammode

Figure 1 shows how the setting of the Streammode property in a de-aggregation Stream and its component Streams affect data flow into and out of the Context. For successful data flow from the external stream into the Context, the Streammode must be set on both the de-aggregation stream and the component Stream to allow data flow in the desired direction/s across the both of them. Likewise for successful data flow from the Context to the external data stream, the Streammode must be set on both the de-aggregation stream and the component to allow flow in the desired direction/s across the both of them. Figure 1 shows how the setting of the Streammode affects how far data is sent or received in the staged processing.

#### 8.6.8.2 Aggregation Streams

Data received externally at the component stream is first processed using the component Stream’s properties. As data is passed to the aggregation Stream it is processed according to the aggregation Stream’s properties. The data is then passed into the Context.

Data received on an aggregation Stream from the Context is processed against the aggregation Stream’s properties. The data is then passed to the relevant component Stream and then processed against the component Stream properties before being sent externally.

Figure 2 below shows the procedures using Streammode with an aggregation Stream as an example.

Sid2

Sid3

Ta

Sid1

stragg(1,2)

groupse (BUNDLE,1,2)

Internal Stream

External Stream

Context

Key:

Sid StreamID

SR Send Receive

SO Send Only

RO Receive Only

Data from External to Context

Data from Internal Context

Figure 2 – Aggregation Stream property interaction: Streammode

Figure 2 shows how the setting of the Streammode property in an aggregation Stream and its component streams affect data flow into and out of the Context. For successful data flow from the external stream into the context, the Streammode must be set on both the aggregation stream and the component to allow data flow in the desired direction/s across the both of them. Likewise for successful data flow from the context to the external data stream, the Streammode must be set on both the aggregation stream and the component to allow data flow in the desired direction/s across the both of them.

______________________

____________________