| Rapporteur Meeting of Questions 2, 3, 5 and 21/16 |  |  |  | AVD-4709 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 5/16 | Chengdu, 8 - 12 June 2015 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Huawei |  |  |  |
| Title: | H.TPS-AV: Addressing open editor’s notes |  |  |  |
| Purpose: | Proposal |  |  |  |

1.	Introduction

This contribution proposes resolutions to Editor’s notes in clause 3.2 and Appendix I.1.3

2.	Discussion

Clause 3.2 “Terms defined in this Recommendation” currently has two Editor’s notes regarding the terms “consumer”, “provider” and “capability”, i.e.

{Editor's note: The definitions of consumer and provider are consistent with CLUE but are placeholders}

And

{Editor's note: In defining parameters related to capabilities it is important to understand in the general sense what a capability may be. It is expected that this definition will evolve. Further definitions may be required.}

With respect to the first editor’s note, the terms for consumer and provider have been updated in CLUE. The definitions are:

- Consumer: short for Media Consumer.

- Provider: Same as Media Provider.

- Media Consumer: a CLUE-capable device that intends to receive Capture Encodings.

- Media Provider: a CLUE-capable device that intends to send Capture Encodings.

The definitions from clause 3.2/[ITU-T H.TPS-AV] are:

3.2.3	consumer: A device that receives media streams.

3.2.6	provider: A device that sends media streams.

The editor’s note states that the terms are consistent. They are except for the aspect of being CLUE capable. As the draft Recommendation is a list of general telepresence parameters the current definitions in H.TPS-AV are sufficient. Therefore the Contributors propose that the editor’s note be deleted.

The second editor’s note indicates that the definition of “capability” may evolve. The definition was based on previous H.323 usage of the term. The current definition talks about “terminals” and “signals”. The Contributors believe that the use of the term “endpoint” would be more appropriate as the parameters defined in H.TPS-AV could be related to a wide range of endpoint types, not just terminals. Furthermore as H.TPS-AV talks about telepresence “endpoint” characteristics it is proposed to update the definitions of “Provider” and “Consumer to use the term “endpoint” instead of device.

The Contributors also believe that the use of the term “parameter” rather than signal would be more appropriate. This is due to the fact that the draft Recommendation defines telepresence parameters rather than using the term “signal”. The Contributors have proposed a corresponding update below.

The third editor’s note in Appendix I.1.3 addresses the issue that method of signalling of the sendLoudnessRating is TBD.  The Contributors believe that the send loudness rating is a basic parameter associated with audio equipment and not specific to telepresence systems. It previously has not been signalled. Therefore the Contributors propose to mark this parameter as not signalled.

3.	Proposal

It is proposed to accept the following changes. The changes are marked up against TD 247R1/WP1 from the Geneva, February Q5/16 Rapporteurs meeting.

Draft new ITU-T H.??? (ex H.TPS-AV)

Audio/video parameters for telepresence systems

…

3 Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	capability: An endpoint has a particular capability if it is able to support functionality related to a particular telepresence parameter. This may involve encod and transmit or receive and decode of a signal related to the parameter or it  may also relate to how a terminal captures or renders signal to the parameter.

3.2.2	chroma: An adjective specifying that a sample in a digital picture is representing one of the two colour difference signals related to the primary colours.

NOTE – The term chroma is used rather than the term chrominance in order to avoid the implication of the use of linear light transfer characteristics that is often associated with the term chrominance.

3.2.3	consumer: An endpoint that receives media streams.

3.2.4	cycles per degree: The number of alternating white and black lines that can be captured or rendered per degree of visual field.

3.2.5	luma: An adjective specifying that a sample in a digital picture is representing the monochrome signal related to the primary colours.

NOTE – The term luma is used rather than the term luminance in order to avoid the implication of the use of linear light transfer characteristics that is often associated with the term luminance.

3.2.6	provider: An endpoint that sends media streams.

…

7.1.3.5 Sending Loudness Rating

| Identity: | sendLoudnessRating |
| --- | --- |
| Description: | This parameter indicates the acoustic to electric transfer function of the Telepresence audio stream. |
| Format: | Numeric, in dB |
| Possible Values: | +4 to +16 |
| Default: | (+ 13–Fs) |
| Reference: | - |
| Signalling Mechanism: | Not signalled. |
| Supplementary Information: | The purposes of specifying Sending Loudness Rating for the sound capture are: - Set the microphone preamplifiers gain value in order to use the whole dynamic of the A to D converter without saturating it. - Set the microphone preamplifiers gain value of all the microphones in all the rooms in communication in such way that the same sound pressure levels at any conferee position create the same electrical value before the A to D conversion. - Set the nominal electrical sending level from the Telepresence system to be compatible with that from conventional telephones. |

Telepresence systems shall send audio to other telepresence systems with a SendLoudnessRating of (+ 13–Fs) dB, measured in accordance with [ITU-T P.300].

In order to take into account the difference between the reference test positioning and the actual microphone-talker operating distance (ds), for which the terminal is adjusted, the following correction factor Fs is defined:

(ds in meters)

NOTE - The formula for Sending Loudness Rating is currently only defined in an audio bandwidth of up to 7 kHz, whereas Telepresence systems may have audio bandwidths up to 20 kHz. However, loudness ratings are intended for speech, where most energy is below 7 kHz, and any difference in loudness level in wider bandwidths is expected to be small. Adaptation of psychoacoustic based loudness algorithms aiming to determine the loudness for all bandwidths ranging from narrowband to full-band audio signals is an on-going work item in ITU-T SG12, and new results will be incorporated in this Recommendation when available.

…

### I.1.3	Audio parameters

| Parameter | Need for signalling at session initiation | Need for signalling during session | Remarks |
| --- | --- | --- | --- |
| Audio capturePoint | Y | Y | See "Point of Capture" in [IETF CLUE FW]. |
| Audio lineOfCapturePoint | Y | Y | See the "Point on line of Capture" attribute in [IETF CLUE FW]. |
| Audio sensitivityPattern | Y | Y | See the "Audio Capture Sensitivity Pattern" attribute in [IETF CLUE FW]. |
| maxAudioBitrate | Y | Y | See “bandwidth” in [IETF RFC 4566] and “maxBitRate” in [ITU-T H.245]. |
| nominalAudio Level |  |  | [ITU-T H.245] (Not currently available, but may be added in the future.) |
| dynamicAudioLevel | N | Y | See [IETF RFC 6464] |
| sendLoudnessRating |  |  | Not signalled. |
| weighted Microphone-Loudspeaker Coupling Loss | N | N | Not signalled. |

__________________________

____________________