| Rapporteur Meeting of Questions 1, 2, 3, 5 and 21/16 |  |  |  | AVD-4721 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Chengdu (China), 8 - 12 June 2015 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ALCATEL-LUCENT |  |  |  |
| Title: | H.248.88: Living list item "Lip synchronization issue in audio/video RTP topologies" |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

A new living list item for H.248.88 is proposed.

Background

The problem was already verbally mentioned in the last SG16 meeting, actually in the context of telepresence (Q.5). It is directly related to H.248 MGs (Q.3) in case of RTP-to-RTP interworking support, and in context of multimedia calls where a certain synchronization between the multiple RTP streams would be required (at the receiving RTP endpoint).

The detailed description is provided in the proposal.

Proposal

Create a new TD-WP1 for documentation of a living list item related to H.248.88:

1	Problem statement

1.1	End-to-end dualmedia call with synchronized RTP sessions

Lip synchronization is related to the synchronization at receiver side between audio and associated video information. Figure 1 illustrates the end-to-end scenario:

Figure 1 – End-to-end dualmedia call with synchronized RTP sessions
(Copy of Figure 6/[draft-ietf-avtext-rtp-grouping-taxonomy] "Example Point to Point Communication Session with two RTP  Sessions")

1.2	Synchronisation of RTP Flows

IETF RFC 6051 "Rapid Synchronisation of RTP Flows" provides a brief summary about the legacy synchronization (= RFC 3550) (besides rapid synchronization aspects):

## 2.  Synchronisation of RTP Flows

RTP flows are synchronised by receivers based on information that is

contained in RTCP SR packets generated by senders (specifically, the

NTP-format timestamp and the RTP timestamp).  Synchronisation

requires that a common reference clock MUST be used to generate the

NTP-format timestamps in a set of flows that are to be synchronised

(i.e., when synchronising several RTP flows, the RTP timestamps for

each flow are derived from separate, and media specific, clocks, but

the NTP-format timestamps in the RTCP SR packets of all flows to be

synchronised MUST be sampled from the same clock).  To achieve faster

and more accurate synchronisation, it is further RECOMMENDED that

senders and receivers use a synchronised common NTP-format reference

clock with common properties, especially timebase, where possible

(recognising that this is often not possible when RTP is used outside

of controlled environments); the means by which that common reference

clock and its properties are signalled and distributed is outside the

scope of this memo.

For multimedia sessions, each type of media (e.g., audio or video) is

sent in a separate RTP session, and the receiver associates RTP flows

to be synchronised by means of the canonical end-point identifier

(CNAME) item included in the RTCP Source Description (SDES) packets

generated by the sender or signalled out of band [RFC5576].  For

layered media, different layers can be sent in different RTP

sessions, or using different synchronisation source (SSRC) values

within a single RTP session; in both cases, the CNAME is used to

identify flows to be synchronised.  To ensure synchronisation, an RTP

sender MUST therefore send periodic compound RTCP packets following

Section 6 of RFC 3550 [RFC3550].

[…]

Figure 2 – RTCP SR packet – NTP (absolute) and RTP (relative) timestamps

Fundamental requirements:

It could be concluded that the most important requirement is to get the RTCP SR reports for all received RTP streams at the receiving RTP endpoint (because the NTP timestamps (Figure 2) serve as common reference clock across all sent RTP streams from the sending RTP endpoint).

Furthermore, what is the acceptable time shift within synchronized audio and video?

See:

- ITU-T G.1091 QoE Requirements for Telepresence Services
=> clause 10.2  Lip sync 
=> Audio streams should be less than 40 ms ahead of video streams, and less than 60 ms behind video streams.

- ITU-T J.248 Requirements for operational monitoring of video-to-audio delay in the distribution of television programs
=> Appendix I Perceptual limits for lip-sync errors

- ITU-R BT.1359-1 (1998), Relative timing of sound and vision for broadcasting

1.3	End-to-end scenario with interim, RTP-aware middleboxes

Q.3/16 considers end-to-end scenarios where the IP media path is routed across H.248 MGs. Figure 3 illustrates a dualmedia example. A unidirectional traffic flow consideration is sufficient from lip synchronization perspective. There are three RTP entities: the two remote RTP endpoints X and Y besides the RTP-enabled H.248 MG. Hence, there are two RTP streams in X-to-Y direction: the particular media format of each RTP stream is negotiated independently for each other, resulting in the example situation here where the MG is requested to provide a media translation (MT) function for audio and a transparent forwarding (TF) function for video.

There are consequently two different RTP topologies behind each media type specific interworking function in the H.248 MG ("B2BRE" or "RTPMT" for audio and "TF" or "RTPTT" for video).

Figure 3 – End-to-end scenario with interim H.248 IP-IP media gateway

RTCP handling again is coupled to RTP topology behaviour (RFC 5117, H.248.88). Thus, the RTCP SR packets for audio, as sent by X1 are terminated by the MG (A1). The RTP sink receives RTCP SR packets which are originated by the MG (B1). The situation is different for the second RTP stream (video) where the original RTCP SR packets are received by Y.

There are consequently two different time domains (see the coloured fields in Figure 4) due to the different network wide scope of the RTCP basic reports (SR, RR):

Figure 4 – End-to-end scenario with interim H.248 IP-IP media gateway

It should be reminded again that the two RTP endpoints within a B2BRE or RTPMT topology (here A1 and B1) are decoupled from time perspective. Hence, the RTP timestamps of segment B1-to-Y1 are not directly linked to X1-to-A1.

1.4	Problem

The different end-to-end oneway transfer delay of audio and video, from X to Y, as given by delay components τA and τV, does not represent any issue!

The actual problem is related to the different timelines of the RTCP SR reports for audio and video: RTP endpoint tries to provide lip synchronization on basis of the two RTP sources B1 (audio) and X2 (video).

2	Solution proposals

2.1	Proposal 1: "Virtual RTP source"

2.1.1	Required RTP / RTCP capabilities

When RTP source B1 would look like RTP source X1 from time perspective (i.e., NTP timestamp baseline), then lip synchronization should feasible again. Of course, the offset τA,MG due to media translation needs to be considered.

2.1.2	H.248 capabilities

The MG must know whether B1 is a self-contained, real RTP source function or such a virtual RTP source which is linked to the received RTP stream on the peer H.248 SEP within the SEPP.

Such a capability extension would relate to the scope of H.248.88.

2.2	Proposal 2: others?

Any other solution proposals?

____________________