| Rapporteur Meeting of Questions 2, 3, 5 and 21/16 |  |  |  | AVD-4773 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 21/16 | Chengdu, 8 - 12 June 2015 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | China Telecom / MITT of China / ZTE |  |  |  |
| Title: | Proposal for new work item on Content Delivery Network- The architecture of Integration Multimedia Content Delivery Network |  |  |  |

Summary

This contribution proposes to launch a new work item on content delivery network- The architecture of integration multimedia content delivery network.  The background and the proposed skeleton has been introduced in this contribution.

# Background

A content delivery network (CDN) is an infrastructure of network elements operating at layer 4 through layer 7, arranged for the efficient distribution and delivery of digital content. Such content includes, but is not limited to, web pages and images delivered via HTTP, and streaming of continuous media delivered via HTTP, RTSP, RTMP, etc. CDNs typically provide services to multiple content service providers (CSPs).

CDN is not a new concept now, several CDNs for IPTV services have been installed and operated by several companies. After several years’ development,  IPTV services has become a kind service with a large of subscribers around the world with the support of IPTV related  recommendations developed within SG 16, such as Y.1910. As the development of Internet OTT video services, IPTV services face the severe competition from Internet OTT services. In that case, many IPTV service operators begin to provide OTT video services oriented to Internet and mobile Internet. To save the investment, operators hope the CDNs built within large areas supporting IPTV services before to support media services with multi format and multi steam media protocols urgently. The huge need for integration multimedia CDNs occurs.

The CDNs established for IPTV services by operators has the following characters:

- It is designed for IPTV services, and it is complicated to  lead into new services over IPTV CDNs;

- There is a lot of difference to the Internet CDN at supporting steam media protocols， backtracking mode, users’ requests redirection etc;

- There are no interconnecting standards between IPTV CDNs and Internet CDNs, it is hard to install a CDN with the devices of different companies.

As mentioned above, current main IPTV CDNs could not meet the requirements of integration multimedia CDNs. Since 2010, China telecom has experimented to update its IPTV CDNs. The main work includes:

- Standardization of external interfaces of IPTV CDN to achieve the IPTV CDN decoupled  from the backstage of IPTV system;

- Standardization of interconnection interfaces of key components to support the interconnection of devices manufactured by different corporations;

- To update the IPTV CDN media services to support multi stream media, files downloading functions.

The design plan was approved to be successful by the experiment. And the integration multimedia CDN products manufactured by Huawei, ZTE etc. have be installed in integration multimedia CDNs of China Telecom. Now the integration multimedia CDNs of China Telecom operate well and can support IPTV, OTT and file download services.

CDN related standards were studied within IETF CDI Working group around 2003. IETF CDI working group was closed in 2003 after RFC 3466 - A Model for Content Internetworking (CDI) had been released. There is an active working group CDNI within IETF, in which standards related with interworking of CDNs are studied, and several RFCs have be approved.

Two work items about VCDN were set up within Q.21 of ITU-T SG 16 last year. But there is not a standard of the architecture of integration multimedia content delivery network. So we propose to set up a new work item on content delivery network- The architecture of integration multimedia content delivery network within Q.21/16 to standardize the architecture of integration multimedia content delivery (distribution) network.

We propose to establish a work item of to Content Delivery Network- The architecture of integration multimedia Content Delivery Network within Q.21/16 based on the results of our experiment.

# Discussion

The draft document of architecture of integration multimedia content delivery network will define the main function requirements, logical architecture, content access, request redirection, media services, etc. The contents of the proposed new work item are as following:

- Introduction

- Function requirements

This chapter describers the requirements for the content format, media service protocol, networking requirements, extend and secure of integration CDN.

- Architecture

This chapter describers the architecture, the function requirements of logical models (including content database, cache, stream media service, GSLB) and the technical requirement for main interfaces.

- Access modes

This chapter describers the technical requirements for content access of content providers.

- Request redirection

The technical requirements of GSLB are defined.

- Media services

This chapter describers the media service methods supported by integration CDN, including RTSP stream media, HPD, HPD, HLS, DASH, file downloading, etc.

- Resource backtracking

This chapter describers the technical requirements for resource backtracking when the contents are found.

- Operation management

This chapter describers the requirements of operation management of CDN, including operation management, services management, network management and services statistics.

- Interfaces

This chapter describers the main functions of each interface defined in the architecture and its main protocol.

- Services procedures

This chapter describers the main services procedures of content providers access, user requests, resource backtracking.

# Proposal

It is proposed to consider the initiation of a new work item, whose tentative title is “The architecture of Integration Multimedia Content Delivery Network”.

__________________