| Rapporteur Meeting of Questions 2, 3, 5 and 21/16 |  |  |  | TD-17 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 3/16 | Chengdu, 8 - 12 June 2015 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | Editor H.248.RTPMUX |  |  |  |
| Title: | H.248.RTPMUX “Gateway Control Protocol: H.248 support for RTP multiplexing” (New): Output draft |  |  |  |
| Purpose: | Information |  |  |  |

1.	Introduction

This document contains the Editor’s output draft for H.248.RTPMUX “Gateway Control Protocol: H.248 support for RTP multiplexing” (New).

Document History

| Document History |  |
| --- | --- |
| Issue | Notes |
| H.248.RTPMUX Ed. 0.1 | Initial baseline output (TD 59 (WP1/16)) from ITU-T SG16 Geneva meeting, Jan 2013. Based on C.34, C.35, C.41and C.53. Location: http://www.itu.int/md/T13-SG16-130114-TD-WP1-0056/en |
| H.248.RTPMUX Ed. 0.2 | Input draft to Oslo, 17 - 21 June 2013 meeting. Based on TD 56 (WP1/16). |
| H.248.RTPMUX Ed. 0.3 | Output (TD-24) from ITU-T SG16 Oslo meeting, Jun 2013. Based on AVD-4415, AVD-4450 and AVD-4459. Location: http://ftp3.itu.int/av-arch/avc-site/2013-2016/1306_Osl/TD-24.zip |
| H.248.RTPMUX Ed. 0.4 | Input draft to Geneva, 28 October – 8 November 2013 meeting. Based on TD 24 (Oslo, June 2013). |
| H.248.RTPMUX Ed. 0.5 | Output draft to Geneva, 28 October – 8 November 2013 meeting. Changes due to: C.263, C.279, C.287, C.305, C.330, C.343 and C.359. |
| H.248.RTPMUX Ed. 0.6 | Input text to the Geneva, 10 – 14 March 2014 meeting. |
| H.248.RTPMUX Ed. 0.7 | Output draft from the Geneva, 10 – 14 March 2014 meeting. Changes due to: AVD-4540, AVD-4563 and AVD-4564. |
| H.248.RTPMUX Ed. 0.8 | Input text to the Sapporo, 30 June-11 July 2014 meeting. |
| H.248.RTPMUX Ed. 0.9 | Major modification due to the generic grouping and aggregation text being moved to H.248.STGROUP as a result of C 522r1 from the Sapporo, July 2014 ITU-T SG16 meeting. |
| H.248.RTPMUX Ed. 0.10 | Input text to the Seoul, 3 – 7 November 2014 meeting. |
| H.248.RTPMUX Ed. 0.11 | Output draft from the Seoul, 3 – 7 November 2014 meeting. Changes due to: AVD-4629 |
| H.248.RTPMUX Ed. 0.12 | Input draft to the Geneva, 9-20 February 2015 meeting. TD 232/WP1: http://www.itu.int/md/T13-SG16-150209-TD-WP1-0232/en |
| H.248.RTPMUX Ed. 0.13 | Input draft for the Chengdu, 8 - 12 June 2015 meeting. |
| H.248.RTPMUX Ed. 0.14 | Output draft for the Chengdu, 8 - 12 June 2015 meeting. Changes due to: AVD-4679, AVD-4684 and AVD-4685. |

CONTENTS

Page

1 Scope 5 2 References 5 3 Definitions 6 3.1 Terms defined elsewhere 6 3.2 Terms defined in this Recommendation 6 4 Abbreviations and acronyms 8 5 Conventions 9 6 Overview of RTP multiplexing 9 6.1 Introduction 9 6.2 Protocol history: multiplexing framework defined for original RTP 10 6.3 RTP transport multiplexing 10 6.3.1 Limitations of RTP transport multiplexing 10 6.3.2 Control of RTP transport multiplexing 10 6.3.3 Relation to (RTP, non-RTP media) multiplexing in general 10 6.4 RTP media multiplexing 10 6.5 UDP payload multiplexing 10 7 Use cases with RTP multiplexing 10 8 H.248 control of RTP multiplexing 10 9 RTP Multiplexing examples 11 9.1 Video Stream Multiplexing 11 9.2 Audio and Video Stream in separate multiplexes 14 Annex A <Annex Title - Placeholder> 17 Appendix I Existing H.248 support for RTP multiplexing 17 I.1 3GPP-defined multiplexing for IP bearers with 3GPP framing protocol 17 Appendix II Living list related to IETF documents under observation 18 II.1 Important references 18 II.2 Important terminology 19 Bibliography (placeholder) 22

List of Tables

Page

Table 1 – Naming of multiplexing options related to transport and media multiplexing 7 Table 2 – Example H.248 command request for multiplexed streams 12 Table 3 – Example reply to a H.248 command request for multiplexed streams 13 Table 4 – Example H.248 command request for multiple multiplexed streams 15 Table 5 – Example reply to a H.248 command request for multiple multiplexed streams 16

List of Figures

Page

Figure 1 – Illustration of "MG RTP topology" 7 Figure 2 – Different multiplexing methods for RTP traffic and their used RTP header fields 9 Figure 3 – Example: Video Stream Multiplexing connection model 11 Figure 4 – Example: Video Stream Multiplexing connection model 14 Figure I.1 – Terminology of [1] and the underlying hierarchy (“Interpretation correct?”) 21

Draft new Recommendation ITU-T H.248.xx (ex H.248.RTPMUX)

Gateway Control Protocol: 
H.248 support for RTP multiplexing

AAP Summary

<Mandatory material to be provided before Consent>

Summary

<Mandatory material>

Keywords

H.248, GCP, RTP, RTCP

Introduction

<Optional - This clause should appear only if it contains information different from Scope and Summary>

# 1	Scope

Communication services utilising the Real Time Protocol (RTP) as application level framing protocol normally use a native IP-based transport mechanism [IETF RFC 3550], which results in using two separate UDP ports for the RTP and RTCP traffic components. H.248 gateways provide a rich set of gateway services for such kind of RTP traffic, such as defined by [ITU-T H.248.48], [ITU-T H.248.58], [ITU-T H.248.71], [ITU-T H.248.77], [ITU-T H.248.87] and [ITU-T H.248.88].

However in addition, multiple RTP-based traffic flow components could be multiplexed at multiple, different levels (from the perspective of a layered protocol architecture). The purpose of this Recommendation is to clarify how H.248 gateways support such "multiplexing structures".

The scope of this Recommendation addresses following subjects:

•	terminology: clarification of RTP related terms and definitions;

•	multiplexing methods: summary of existing multiplexing schemes in context of RTP-based communication services;

•	H.248 support: provides information on how dedicated multiplexing methods could be supported by H.248 gateways inclusive of references to relevant signalling syntax; and

•	examples: provides some signalling examples as complementary information.

This Recommendation itself does not define any new H.248 protocol extensions.

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (2013), Gateway Control Protocol: Version 3.

[ITU-T H.248.48]	Recommendation ITU-T H.248.48 (02/2012), Gateway control protocol: RTCP XR block reporting package.

[ITU-T H.248.57]	Recommendation ITU-T H.248.57 (03/2013), Gateway control protocol: RTP control protocol package.

[ITU-T H.248.58]	Recommendation ITU-T H.248.58 (06/2008), Gateway control protocol: Packages for application level H.248 statistics.

[ITU-T H.248.71]	Recommendation ITU-T H.248.71 (02/2010), Gateway control protocol: RTCP support packages.

[ITU-T H.248.77]	Recommendation ITU-T H.248.77 (06/2010), Gateway control protocol: Secure real-time transport protocol (SRTP) package and procedures.

[ITU-T H.248.87]	Recommendation ITU-T H.248.87 (01/2014), Gateway control protocol: Guidelines on the use of ITU-T H.248 capabilities for performance monitoring in RTP networks in ITU-T H.248 profiles.

[ITU-T H.248.88]	Recommendation ITU-T H.248.88 (01/2014), Gateway control protocol: RTP topology dependent RTCP handling by ITU-T H.248 media gateways with IP terminations.

[ITU-T H.248.STGROUP]	Recommendation ITU-T H.248.STGROUP (##/201#), Gateway control protocol: H.248 Stream grouping and aggregation.

[IETF RFC 3550]	IETF RFC 3550 (2003), RTP: A Transport Protocol for Real-Time Applications.

[IETF RFC 5761]	IETF RFC 5761 (2010), Multiplexing RTP Data and Control Packets on a Single Port.

[IETF RFC 5764]	IETF RFC 5764 (2010), Datagram Transport Layer Security (DTLS) Extension to Establish Keys for the Secure Real-time Transport Protocol (SRTP).

[IETF RFC 5888]	IETF RFC 5888 (2010), The Session Description Protocol (SDP) Grouping Framework.

[IETF draft taxonomy]	IETF draft-ietf-avtext-rtp-grouping-taxonomy (XX/2015), A Taxonomy of Grouping Semantics and Mechanisms for Real-Time Transport Protocol (RTP) Sources.

{Editor’s note: There are a number of references missing from this section. A review of the baseline should be undertaken of the current draft and the clause updated based on the review.}

# 3	Definitions

## 3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	media (clause 2.1/[IETF draft taxonomy]): a sequence of synthetic or physical Stimuli (sound waves, photons, key-strokes), represented in digital form. Synthesized media is typically generated directly in the digital domain.

3.1.2		media sink (clause 2.1.27/[IETF draft taxonomy])[IETF XYZ]: The media sink receives a source stream that contains, usually periodically, sampled media data together with associated synchronization information. Depending on application, this source stream then needs to be transformed into a raw stream that is conveyed to the media render, synchronized with the output from other media sinks.

3.1.3		media source (clause 2.1.4/[IETF draft taxonomy])[IETF XYZ]: The logical source of a reference clock synchronized, time progressing, digital media stream, called a source stream.  This transformation takes one or more raw streams and provides a source stream as output.  The output is synchronized with a reference clock, which can be as simple as a system local wall clock or as complex as NTP synchronized.

3.1.4		media track (clause 4.10/[IETF draft taxonomy])[IETF XYZ]: Synonym to media source.

NOTE – The term is so far used primarily in context of WebRTC.

3.1.5		(H.248 media) stream [clause 3.2.9/ITU-T H.248.1]: bidirectional media or control flow received/sent by a media gateway as part of a call or conference.

3.1.6		RTP end point (clause 2.2.1/[IETF draft taxonomy]): A single addressable entity sending or receiving RTP packets.  It may be decomposed into several functional blocks, but as long as it behaves as a single RTP stack entity it is classified as a single "Endpoint".

NOTE – The self-contained notion of 'endpoint' is consistent with [b-ITU-T H.Sup.13].

3.1.7		RTP end system [IETF 3550]: An application that generates the content to be sent in RTP packets and/or consumes the content of received RTP packets.  An end system can act as one or more synchronization sources in a particular RTP session, but typically only one.

NOTE – The term is used and important in context of "RTP topologies", see [ITU-T H.248.88]. The term is used in a synonym manner with "RTP endpoint" in [ITU-T H.248.88].

3.1.8		RTP media transport (clause 2.1.13/[IETF draft taxonomy]): FIXTHIS

{Contributor's note: this term is relevant for H.248.STGROUP due to its information about stream multiplexing using a single transport address (clause 6.2). 
On the other side, below description of "media transport" seems to be controversial due to the  linkage with 5-tuples. It might be beneficial to remove the equation of "media transport" with "L4-protocol-specific IP transport connection".}

A Media Transport defines the transformation that the RTP Streams

(Section 2.1.10) are subjected to by the end-to-end transport from

one RTP sender to one specific RTP receiver (an RTP Session

(Section 2.2.2) may contain multiple RTP receivers per sender).  Each

Media Transport is defined by a transport association that is

normally identified by a 5-tuple (source address, source port,

destination address, destination port, transport protocol), but a

proposal exists for sending multiple transport associations on a

single 5-tuple [I-D.westerlund-avtcore-transport-multiplexing].

Characteristics:

o  Media Transport transmits RTP Streams of RTP Packets from a source

transport address to a destination transport address.

o  Each Media Transport contains only a single RTP Session.

o  A single RTP Session can span multiple Media Transports.

The Media Transport concept sometimes needs to be decomposed into

more steps to enable discussion of what a sender emits that gets

transformed by the network before it is received by the receiver.

Thus we provide also this Media Transport decomposition (Figure 5).

3.1.9		RTP stream (clause 2.1.10/[IETF draft taxonomy]): A stream of RTP packets containing media data, source or redundant. The RTP stream is identified by an SSRC belonging to a particular RTP session.

3.1.10	RTP session (clause 2.2.2/[IETF draft taxonomy]): An RTP Session is an association among a group of Participants communicating with RTP.  It is a group communications channel which can potentially carry a number of RTP Streams.  Within an RTP Session, every Participant can find meta-data and control information (over RTCP) about all the RTP Streams in the RTP Session.  The bandwidth of the RTCP control channel is shared between all Participants within an RTP Session.

NOTE – [ITU-T H.248.57], clause 3.1.3 provides following definition:

RTP session: An RTP session comprises a single RTP flow and an optional RTCP flow.

It could be concluded that the "H.248.57 RTP session" is synonym to RTP stream in general.

3.1.11	RTP session group [IETF XYZ]: FIXTHIS

{Editor’s note (2015-06 meeting): this term is introduced by draft-ietf-avtcore-multiplex-guidelines: https://tools.ietf.org/html/draft-ietf-avtcore-multiplex-guidelines-03  defines 

RTP Session Group:  One or more RTP sessions that are used together to perform some function.  Examples are multiple RTP sessions used to carry different layers of a layered encoding.  In an RTP Session Group, CNAMEs are assumed to be valid across all RTP sessions, and designate synchronisation contexts that can cross RTP sessions.

This term is NOT yet linked to the taxonomy draft. The draft-ietf-avtcore-multiplex-guidelines is basically relevant for H.248.RTPMUX, but unfortunately does not yet refer to taxonomy draft.}

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

{Editor’s note (2013-01 meeting): all below listed terminology proposals are just working assumptions and subject of change, primarily dependent on the work progress in IETF on RTP multiplexing models and RTP terminology.}

3.2.1	Media format: a particular pair of associated media encoder (clause 2.1.6/[IETF draft taxonomy]) and media decoder (clause 2.1.25/[IETF draft taxonomy]) entities.

3.2.2	 Media type: a categorization concept for the classification of similar media formats (e.g., 'audio', video', …)

3.2.3	MG RTP topology: The configuration of RTP termination(s) (NOTE 1) for an individual RTP session with a single RTP stream within an RTP node. There are multiple RTP terminations in general besides the single case. The RTP terminations are meshed for the "multiple case" and interconnected via an interworking function (IWF).  There are a number of protocol layer related IWFs; at the RTP layer itself and at lower or upper layers. A particular RTP topology is characterized by a specific IWF.

NOTE 1 – H.248 terminology: an RTP-enabled H.248 termination or stream endpoint.

Important MG RTP topologies are designated by a name.

Figure 1 – Illustration of "MG RTP topology"

NOTE 1 – Figure x1 illustrates the key aspects of the "MG RTP topology" concept. The term "RTP leg" is undefined but not decisive due to the relation to "RTP session".

{Editor's note #2 (2013-11):  term "MG RTP topology" is a work item, the explicit definition could be replaced when a definition for "(IETF) RTP topology" would be available, e.g., by

MG RTP topology: a configuration of RTP-enabled (H.248) stream endpoints (or terminations) in an (H.248) context realizing a specific (IETF) RTP topology.}

3.2.4	RTP media multiplexing: a single IP transport (L4) port for multiple RTP sources. There is a single unique SSRC per RTP source (NOTE 1).

NOTE 1 – Thus, also known as "SSRC multiplexing" (see clause 2.6.2/[IETF draft taxonomy]).

3.2.5	RTP node:

Generic term used to designate a physical or logical network element with processing of RTP traffic, such as an IP host, terminal device, user equipment, gateway, media server, conference server, etc.

3.2.6	RTP sink: a media depacketizer (clause 2.1.23/[IETF draft taxonomy]) which terminates RTP packets, i.e., a sink for received primary and optional received redundancy RTP stream(s).

3.2.7	RTP source: a media packetizer (clause 2.1.9/[IETF draft taxonomy]) which produces RTP packets, i.e., a source RTP stream plus optional associated redundant RTP stream(s). A RTP source uses as primary identifier the SSRC parameter.

3.2.8	RTP topology: expected to be defined by IETF.

{Editor's note (2015-06):  status – it lookslike that there will be not any explicit term "RTP topology" defined by IETF work items
a) draft-ietf-avtext-rtp-grouping-taxonomy
b) draft-ietf-avtcore-rtp-topologies-update
Hence, it will be an implicit concept of the "RTP topologies" (RFCs). }

3.2.9	RTP / RTCP transport multiplexing (briefly "RTP transport multiplexing"): a single IP transport (L4) port for RTP and RTCP packets

3.2.10	RTP multiplexing levels: protocol layer related multiplexing options transport (see clause 3.1.8) and media (see clause 3.2.4) multiplexing could be applied in each combination. Table 1 defines a naming scheme:

Table 1 – Naming of multiplexing options related to transport and media multiplexing

| Multiplexing options | Transport (NOTE 1) | Media (NOTE 2) | Name |
| --- | --- | --- | --- |
| a) | No | No | Fully unmultiplexed |
| b) | No | Yes | Media type multiplexed |
| c) | Yes | No | RTP/RTCP multiplexed |
| d) | Yes | Yes | Fully multiplexed |
| NOTE 1 – Single L4 port for RTP media flow(s) and RTCP control flow(s). NOTE 2 – Single L4 port for multiple RTP sources. There’s a single SSRC per RTP source. |  |  |  |

3.2.11 	UDP payload multiplexing: a single IP transport (L4) port for multiple, UDP-based application (upper layer) protocols. The first octet of the UDP payload is used for multiplexing. The application protocols are used in their native syntax, which limits UDP payload multiplexing only on scenarios which could guarantee unambiguity.

NOTE 1 – Example: [IETF RFC 5764] defines UDP payload multiplexing for the three UDP packet based applications of RTP, STUN and DTLS.

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| DTLS | Datagram Transport Layer Security |
| --- | --- |
| IP | Internet protocol |
| IPv4 | Internet protocol version 4 |
| IPv6 | Internet protocol version 6 |
| IWF | Interworking function |
| Lx | Layer number x |
| LD | Local descriptor |
| LS | Lip Synchronization |
| MG | Media gateway |
| MGC | Media gateway controller |
| NAT | Network address translation |
| NAT-T | NAT Traversal |
| NGN | Next generation network |
| PDU | Packet Data Unit |
| RD | Remote descriptor |
| PT | Packet Type |
| RTCP | RTP Control Protocol |
| RTP | Real Time Protocol |
| SDP | Session description protocol |
| SEP | (H.248) Stream endpoint |
| SEPP | (H.248) Stream endpoint pair |
| SIP | Session initiation protocol |
| SSRC | Sending Source |
| STUN | Session Traversal Utilities for NAT |
| UDP | User datagram protocol |
| WEBRTC | Real-Time Communication in WEB-browsers (as work item in W3C) |

# 5	Conventions

None.

# 6	Overview of RTP multiplexing

## 6.1	Introduction

Multiple, RTP-based traffic flow components may be multiplexed at multiple, different levels (from the perspective of a layered protocol architecture).

A RTP-based traffic flow is given by a packet flow with a common identifier (also known as flow identifier or traffic descriptor). A particular multiplexing method uses specific information elements of such a traffic descriptor, which are mandatory for de-multiplexing again. Figure 1 summarizes the three major multiplexing techniques for RTP traffic, as well as the used header elements.

Figure 2 – Different multiplexing methods for RTP traffic 
and their used RTP header fields

In more detail:

- RTP transport multiplexing (see clause 6.3) uses the 8-bit field of Marker bit (M) and 7-bit Payload Type (PT), which together comprise the 8-bit Packet Type (PT);

- RTP media multiplexing (see clause 6.4) uses the 32-bit SSRC (hence, also known as SSRC multiplexing);

- UDP payload multiplexing (see clause 6.5) is based on the first octet of the UDP payload, which relates to the RTP header field indicated in Figure 2.

## 6.2	Protocol history: multiplexing framework defined for original RTP

Clause 5.2/[IETF RFC 3550] "Multiplexing RTP Sessions" describes the general multiplexing principles for RTP.

## 6.3	RTP transport multiplexing

RTP transport multiplexing represents the case when a RTP media flow and its associated RTCP control flow share a common IP transport address endpoint. In a normal, non-multiplexed scenario, RTP and RTCP use different endpoint identifiers on layer 4 (e.g., UDP ports in case of RTP-over-UDP transport).

Enabling of RTP transport multiplexing is achieved by the allocation of the same L4 port for RTCP as allocated for RTP.

Port allocation rules for RTCP at H.248 interfaces are subject of [ITU-T H.248.57], which covers both, the transport un-multiplexed and multiplexed cases.

RTP transport multiplexing is an extension to [IETF RFC 3550], motivated and defined by [IETF RFC 5761].

### 6.3.1	Limitations of RTP transport multiplexing

Clauses 4 and 5 in [IETF RFC 5761] describe a number of constraints, interactions and conditions to be considered when applying RTP transport multiplexing.

### 6.3.2	Control of RTP transport multiplexing

Control of RTP transport multiplexing:

- Using SDP: via SDP attribute "a=rtcp-mux" (see clause 8/[IETF RFC 5761]). The usage of this SDP element at H.248 interfaces is defined by [ITU-T H.248.57].

### 6.3.3	Relation to (RTP, non-RTP media) multiplexing in general

RTP transport multiplexing is orthogonal to other kinds of multiplexing mechanisms (such as application-specific media level multiplexing options).

RTP transport multiplexing is basically (RTP) application agnostic, only driven by NAT traversal aspects.

## 6.4	RTP media multiplexing

RTP media multiplexing represents the case when multiple RTP streams of different RTP sources share a common IP transport address endpoint. RTP media multiplexing is also known as SSRC multiplexing because the different RTP sources could be separated on SSRC basis. In a normal, non-multiplexed scenario, individual RTP sources use different endpoint identifiers on layer 4 (e.g., UDP ports in case of RTP-over-UDP transport).

Enabling of RTP media multiplexing is achieved at SIP-based call control level via the SDP method "bundling" (see [###]). Bundling is enabled in H.248 via stream grouping [ITU-T H.248.STGROUP].

## 6.5	UDP payload multiplexing

UDP payload multiplexing represents the case:

- A single UDP connection is used plus

- UDP as transport protocol for RTP and RTCP plus

- secure RTP (SRTP and SRTCP) plus

- DTLS-based SRTP key exchange between the SRTP endpoints plus

- ICE/STUN –based NAT traversal support for the UDP connection.

The case referred to as "DTLS-SRTP" is defined by [IETF RFC 5764]. A single UDP connection is then used as the multiplexing structure for the three traffic flow components related to L4+ protocols SRTP, DTLS and STUN. The demultiplexing method is defined in clause 5.1.2 of [IETF RFC 5764], using the leading byte of the UDP packet payload.

Enabling of UDP payload multiplexing is achieved at SIP-based call control level via the indication of "DTLS-SRTP" in RTP related SDP media descriptions (see [###]). H.248 gateway control signalling indicates the application of UDP payload multiplexing at a particular H.248 Termination/Stream implicitly, via the signalling of the SRTP key management scheme [ITU-T H.248.77], usage of ICE/STUN [ITU-T H.248.50] and DTLS related bearer control [ITU-T H.248.93].

# 7	Use cases with RTP multiplexing

## 7.1	General note

There are theoretically at least the following reasons for the application of RTP multiplexing:

- To maximize the likelihood of successful  NAT traversal by minimizing the number of "pinholes" (i.e., number of IP transport connections);

- To minimize the call establishment delay in case of ICE-based NAT traversal procedures by minimizing the number of IP transport connections, because every candidate IP transport connection must firstly be tested for end-to-end connectivity before; or/and

- To "piggy back" the DTLS and STUN packet flows on the same RTP-over-UDP connection in case of DTLS-SRTP.

Reason (3) is already applicable for a single RTP stream ("monomedia call"), but all other multiplexing schemes only take effect for communication services with multiple RTP streams ("multimedia call").

The dominant use cases are driven by NAT traversal (1, 2) for multimedia communication services. This clause focuses on such scenarios.

## 7.2	Use case "multimedia call in NGN/IMS"

FIXTHIS

## 7.3	Use case "Web Real-Time Communication"

[b-ITU-T H.248.WEBRTC] defines an H.248 WebRTC gateway, which is required to support RTP multiplexing as an inherent defined capability of this service. See [b-ITU-T H.248.WEBRTC], e.g. clause 6.3 ("WebRTC protocol stacks inclusive multiplexing usage") and clause 8.4.3 ("RTP multiplexing requirements for WebRTC").

# 8	H.248 control of RTP multiplexing

## 8.1	General principle

Multiplexing of multiple H.248 Streams onto one multiplexed Stream is primarily indicated by setting the same address/port information on the applicable H.248 Streams.

## 8.2	RTP media multiplexing

In order to multiplex several RTP streams the MGC may set the “association” (mgroup/groupse) property indicating which H.248 Streams on a Termination are associated together for the purposes of multiplexing, with the grouping type of “BUNDLE”.

[ITU-T H.248.STGROUP] describes these methods.

## 8.3	RTP transport multiplexing

Solely controlled via [ITU-T H.248.57], any kind of H.248 stream grouping [ITU-T H.248.STGROUP] is not required. It is not possible due to the lack of a correspondent grouping semantic.

# 9	RTP multiplexing examples

{Editor’s note: The scope of the examples need further consideration. The examples below show the use of grouping i.e. the use of the bundle semantic and H.248.STGROUP. It needs to be considered whether the description of RTP multiplexing should focus on the mechanism defined in H.248.STGROUP.}

This clause provides example use cases of RTP multiplexing. In each of the examples, Terminations (T1) and (T2) have five Streams. StreamIDs (1) and (2) are audio streams. StreamIDs (3), (4) and (5) are video streams. These RTP and H.248 streams are bi-directional. The MGC indicates Stream multiplexing by setting the same address/port information on the streams to be multiplexed and by setting the “BUNDLE” grouping semantic tying the Streams together.

There are two basic RTP multiplexing methods, -

- RTP/RTCP transport multiplexing (see clause 6.3) and

- RTP media multiplexing ("SSRC multiplexing") (see clause 6.4).

The examples in clauses 9.1 and 9.2 show RTP media multiplexing.

## 9.1 Video stream multiplexing

In this example the video streams are multiplexed together on Termination (T1) using the “BUNDLE” grouping semantic. The scenario is illustrated in Figure 3.

Figure 3 – Example: Video Stream Multiplexing connection model

Tables 2 and 3 provide an example H.248 command request and response establishing the connection on Termination T1 illustrated in Figure 3.

The MG reply to the command request for multiplexed streams in Table 2 is shown in Table 3.

Table 2 – Example H.248 command request for multiplexed streams

| H.248 encoding | Comments |
| --- | --- |
| MEGACO/3 [123.123.123.4]:55555 Transaction = 10004 { Context = 1 { Add = T1 { Media { TermationState { mgroup/groupse = [“BUNDLE 3 4 5”]}, Stream = 1 { Local { v=0 c=IN IP4 $ m=audio $ RTP/AVP 32 } Remote { v=0 c=IN IP4 193.193.193.193 m=audio 30000 RTP/AVP 32 } } Stream = 2 { Local { v=0 c=IN IP4 $ m=audio $ RTP/AVP 8 } Remote { v=0 c=IN IP4 193.193.193.193 m=audio 30002 RTP/AVP 8 } } Stream = 3 { Local { v=0 c=IN IP4 $ m=video $ RTP/AVP 32 } Remote { v=0 c=IN IP4 193.193.193.193 m=video 30004 RTP/AVP 32 } } Stream = 4 { Local { v=0 c=IN IP4 $ m=video $ RTP/AVP 33 } Remote { v=0 c=IN IP4 193.193.193.193 m=video 30004 RTP/AVP 33 } } Stream = 5 { Local { v=0 c=IN IP4 $ m=video $ RTP/AVP 34 } Remote { v=0 c=IN IP4 193.193.193.193 m=video 30004 RTP/AVP 34 } } } } } } | Termination (T1) is added with the two audio streams and three video streams. The “BUNDLE” grouping semantic is used to indicate that Streams 3, 4 and 5 are multiplexed together. |

Table 3 – Example reply to a H.248 command request for multiplexed streams

| H.248 encoding | Comments |
| --- | --- |
| MEGACO/3 [124.124.124.222]:55555 Reply = 10004 { Context = 1 { Add=T1 { Media { Stream = 1 { Local { v=0 c=IN IP4 200.200.200.200 m=audio 50000 RTP/AVP 32 } Remote { v=0 c=IN IP4 193.193.193.193 m=audio 30000 RTP/AVP 32 } } Stream = 2 { Local { v=0 c=IN IP4 200.200.200.200 m=audio 50002 RTP/AVP 8 } Remote { v=0 c=IN IP4 193.193.193.193 m=audio 30002 RTP/AVP 8 } } Stream = 3 { Local { v=0 c=IN IP4 200.200.200.200 m=video 50004 RTP/AVP 32 } Remote { v=0 c=IN IP4 193.193.193.193 m=video 30004 RTP/AVP 32 } } Stream = 4 { Local { v=0 c=IN IP4 200.200.200.200 m=video 50004 RTP/AVP 33 } Remote { v=0 c=IN IP4 193.193.193.193 m=video 30004 RTP/AVP 33 } } Stream = 5 { Local { v=0 c=IN IP4 200.200.200.200 m=video 50004 RTP/AVP 34 } Remote { v=0 c=IN IP4 193.193.193.193 m=video 30004 RTP/AVP 34 } } } } } } |  |

## 9.2 Audio and Video Stream in separate multiplexes

In this example video streams are multiplexed over a transport connection and the audio streams are multiplexed over a separate transport connection on Termination (T1) using the “BUNDLE” grouping semantic. The scenario is illustrated in figure 4.

Figure 4 – Example: Video Stream Multiplexing connection model

Tables 4 and 5 provide an example H.248 command request and response establishing the connection illustrated in Figure 4. The MG reply is described in Table 5.

Table 4 – Example H.248 command request for multiple multiplexed streams

| H.248 encoding | Comments |
| --- | --- |
| MEGACO/3 [123.123.123.4]:55555 Transaction = 10004 { Context = 1 { Add = T1 { Media { TermationState { mgroup/groupse = [“BUNDLE 1 2”, “BUNDLE 3 4 5”]}, Stream = 1 { Local { v=0 c=IN IP4 $ m=audio $ RTP/AVP 32 } Remote { v=0 c=IN IP4 193.193.193.193 m=audio 30000 RTP/AVP 32 } } Stream = 2 { Local { v=0 c=IN IP4 $ m=audio $ RTP/AVP 8 } Remote { v=0 c=IN IP4 193.193.193.193 m=audio 30000 RTP/AVP 8 } } Stream = 3 { Local { v=0 c=IN IP4 $ m=video $ RTP/AVP 32 } Remote { v=0 c=IN IP4 193.193.193.193 m=video 30002 RTP/AVP 32 } } Stream = 4 { Local { v=0 c=IN IP4 $ m=video $ RTP/AVP 33 } Remote { v=0 c=IN IP4 193.193.193.193 m=video 30002 RTP/AVP 33 } } Stream = 5 { Local { v=0 c=IN IP4 $ m=video $ RTP/AVP 34 } Remote { v=0 c=IN IP4 193.193.193.193 m=video 30002 RTP/AVP 34 } } } } } } | Termination (T1) is added with the two audio streams and three video streams. The “BUNDLE” grouping semantic is used to indicate that Streams 3, 4 and 5 are multiplexed together and that Streams 2 and 3 are multiplexed together. |

Table 5 – Example reply to a H.248 command request for multiple multiplexed streams

| H.248 encoding | Comments |
| --- | --- |
| MEGACO/3 [124.124.124.222]:55555 Reply = 10004 { Context = 1 { Add=T1{ Media { Stream = 1 { Local { v=0 c=IN IP4 200.200.200.200 m=audio 50000 RTP/AVP 32 } Remote { v=0 c=IN IP4 193.193.193.193 m=audio 30000 RTP/AVP 32 } } Stream = 2 { Local { v=0 c=IN IP4 200.200.200.200 m=audio 50000 RTP/AVP 8 } Remote { v=0 c=IN IP4 193.193.193.193 m=audio 30000 RTP/AVP 8 } } Stream = 3 { Local { v=0 c=IN IP4 200.200.200.200 m=video 50002 RTP/AVP 32 } Remote { v=0 c=IN IP4 193.193.193.193 m=video 30002 RTP/AVP 32 } } Stream = 4 { Local { v=0 c=IN IP4 200.200.200.200 m=video 50002 RTP/AVP 33 } Remote { v=0 c=IN IP4 193.193.193.193 m=video 30002 RTP/AVP 33 } } Stream = 5 { Local { v=0 c=IN IP4 200.200.200.200 m=video 50002 RTP/AVP 34 } Remote { v=0 c=IN IP4 193.193.193.193 m=video 30002 RTP/AVP 34 } } } } } } |  |

Annex A

<Annex Title - Placeholder>

(This annex forms an integral part of this Recommendation.)

<Body of annex A>

Appendix I

Existing H.248 support for RTP multiplexing

(This appendix does not form an integral part of this Recommendation)

## I.1	3GPP-defined multiplexing for IP bearers with 3GPP framing protocol

Reference: see clause 6.4/[ETSI TS 129.414].

Motivation: transport capacity reductions (“bandwidth efficiency”).

Multiplexing method: multiple RTP packets mapped into a single PDU augmented by a (3GPP defined) multiplex header (“similar to IETF SHIM approach").

H.248 control: not any signaling support, thus pure provisioning based approach.

Appendix II

Living list related to IETF documents under observation

(This appendix does not form an integral part of this Recommendation)

{Editor’s note: This Appendix would be again deleted as soon as the reference (I.1) and terminology (I.2) discussion becomes stable).

## II.1	Important references

Following IETF documents (from four IETF working groups) are relevant to the RTP multiplexing discussion:

[1]	A Taxonomy of Grouping Semantics and Mechanisms for Real-Time Transport Protocol (RTP) Sources
IETF “RAI Areaa” / WG AVTEXT? (still open)
Draft: draft-lennox-raiarea-rtp-grouping-taxonomy
URL: http://tools.ietf.org/html/draft-lennox-raiarea-rtp-grouping-taxonomy-00

[2]	Guidelines for using the Multiplexing Features of RTP
IETF AVTCORE
Draft: draft-westerlund-avtcore-multiplex-architecture 
URL: http://tools.ietf.org/id/draft-westerlund-avtcore-multiplex-architecture-02.txt

[3]	Multiplexing RTP Data and Control Packets on a Single Port
IETF MMUSIC
RFC 5761

[4]	“BUNDLE”
= Multiplexing Negotiation Using SDP Port Numbers 
IETF MMUSIC
Draft: draft-ietf-mmusic-sdp-bundle-negotiation
URL: http://tools.ietf.org/wg/mmusic/draft-ietf-mmusic-sdp-bundle-negotiation/

[5]	“RTP for WebRTC”
 = Web Real-Time Communication (WebRTC): Media Transport and Use of RTP
IETF RTCWEB
Draft: draft-ietf-rtcweb-rtp-usage
URL: http://tools.ietf.org/wg/rtcweb/draft-ietf-rtcweb-rtp-usage/

[6]	“Multi-Media Concepts and Relations” 
IETF RTCWEB
Draft: draft-burman-rtcweb-mmusic-media-structure
URL: http://tools.ietf.org/id/draft-burman-rtcweb-mmusic-media-structure-00.txt

[7]	“RTP for Telepresence”
 = RTP Usage for Telepresence Sessions
IETF CLUE
Draft: draft-lennox-clue-rtp-usage
URL: http://tools.ietf.org/id/draft-lennox-clue-rtp-usage-04.txt /

Complementary:

[8]	Source-Specific Media Attributes
IETF MMUSIC
RFC 5576

9]	SDP Grouping Framework
IETF MMUSIC
RFC 5888

[10]	Media multiplexing with RTP subsessions
IETF AVTCORE
Draft: draft-ejzak-avtcore-rtp-subsessions
URL: http://tools.ietf.org/id/draft-ejzak-avtcore-rtp-subsessions-01.txt

[11]	Multiple RTP Sessions on a Single Lower-Layer Transport
IETF AVTCORE
Draft: draft-westerlund-avtcore-transport-multiplexing 
URL: http://tools.ietf.org/id/draft-westerlund-avtcore-transport-multiplexing-03.txt

[12]	Multiple Media Types in an RTP Session
IETF AVTCORE
Draft: draft-westerlund-avtcore-multi-media-rtp-session
URL: http://tools.ietf.org/id/draft-westerlund-avtcore-multi-media-rtp-session-00.txt

[13]	RTP Considerations for Endpoints Sending Multiple Media Streams
IETF AVTCORE
Draft: draft-lennox-avtcore-rtp-multi-stream
URL: http://tools.ietf.org/id/draft-lennox-avtcore-rtp-multi-stream-00.txt

[14]	Using Simulcast in RTP sessions
IETF AVTCORE
Draft: draft-westerlund-avtcore-rtp-simulcast 
URL: http://tools.ietf.org/id/draft-westerlund-avtcore-rtp-simulcast-01.txt

[15]	Cross Session Stream Identification in the SDP
IETF RTCWEB
Draft: draft-alvestrand-rtcweb-msid
URL: http://tools.ietf.org/id/draft-alvestrand-rtcweb-msid-02.txt

[16]	H.248.1v4 Living List: New Appendix V – H.248 Streams “Examples for valid media format suites”
ITU-T SG16
Draft: AVD-4241
URL: see http://wftp3.itu.int/av-arch/avc-site/2009-2012/1209_Bri/1209_Bri.html

## II.2	Important terminology

Present work on multiplexed RTP did lead to a revision, modification, extension till the semantical overloading of some RTP related terms. Primarily the terms “media stream” and “RTP session” are used in many slightly different flavours. Following definitions from [1] might be quite useful (“despite the fact that they are not yet fully unambiguous”):

All that terms are either new or extensions/revisions of RFC 1889/RFC 3550 terms:

Endpoint:  A single entity sending or receiving RTP packets.  It may

be decomposed into several functional blocks, but as long as it

behaves a single RTP stack entity it is classified as a single

endpoint.

Comment: an endpoint relates to RTP topology “RTP endsystem” in our understanding (see RFC 5117, H.248.RTPTOPO).

Media Source:  The source of a stream of data of one Media Type, It

can either be a single media capturing device such as a video

camera, a microphone, or a specific output of a media production

function, such as an audio mixer, or some video editing function.

Sending data from a Media Source may cause multiple RTP sources to

send multiple Media Streams.

Media Stream:  A sequence of RTP packets using a single SSRC that

together carries part or all of the content of a specific Media

Type from a specific sender source within a given RTP session.

Comments:

- we need to distinguish between “IETF media stream” and “H.248 media stream”

- 1:1 relationship between “media stream” and “SSRC” due to “single SSRC”, right?

- May use different media streams the same SSRC value? The definition allows both interpretations …

RTP Source:  The originator or source of a particular Media Stream.

Identified using an SSRC in a particular RTP session.  An RTP

source is the source of a single media stream, and is associated

with a single endpoint and a single Media Source.  An RTP Source

is just called a Source in RFC 3550.

Comments:

- 1:1 relationship between “RTP source” and “SSRC”, right?

- May use different RTP sources the same SSRC value? The definition allows both interpretations …

Media Sink:  A recipient of a Media Stream.  The endpoint sinking

media are Identified using one or more SSRCs.  There may be more

than one Media Sink for one RTP source.

Multiplex:  The operation of taking multiple entities as input,

aggregating them onto some common resource while keeping the

individual entities addressable such that they can later be fully

and unambiguously separated (de-multiplexed) again.

RTP Session:  As defined by [RFC3550], the endpoints belonging to the

same RTP Session are those that share a single SSRC space.  That

is, those endpoints can see an SSRC identifier transmitted by any

one of the other endpoints.  An endpoint can receive an SSRC

either as SSRC or as CSRC in RTP and RTCP packets.  Thus, the RTP

Session scope is decided by the endpoints' network interconnection

topology, in combination with RTP and RTCP forwarding strategies

deployed by endpoints and any interconnecting middle nodes.

Comment: this is an extension of the RFC 3550 RTP session concept in our understanding. The RTP session is NOT any global concept across all involved RTP entities (such as endsystems, mixers, translators) of a particular communication application.
The RTP session is rather a local concept, given by the perspective of a particular endpoint.

Examples: a two-party call (= point to point topology) between A and B, and with different multiplexing models of each endpoint may lead to the effect that e.g. A perceives a single RTP session whereas B observes multiple RTP sessions for the same communication service.

NOTE: we may agree to such an “RTP session” concept (given that our interpretation is inline with [1]).

RTP Session Group:  One or more RTP sessions that are used together

to perform some function.  Examples are multiple RTP sessions used

to carry different layers of a layered encoding.  In an RTP

Session Group, CNAMEs are assumed to be valid across all RTP

sessions, and designate synchronisation contexts that can cross

RTP sessions.

Source:  Term that should not be used alone.  An RTP Source, as

identified by its SSRC, is the source of a single Media Stream; a

Media Source can be the source of mutiple Media Streams.

SSRC:  An RTP 32-bit unsigned integer used as identifier for a RTP

Source.

(tbd: The terms "SSRC multiplexing" and "session multiplexing" are

confusing, with unclear historical meanings; they need to be removed

from this document in the interests of clarity)

It might be beneficial to illustrate the multiplexing hierarchy of the defined terminology by [1] (Figure I.1 illustrates the traffic source side only):

Figure I.1 – Terminology of [1] and the underlying hierarchy (“Interpretation correct?”)

NOTE - The terminology in [1] should be further improved. There are already a couple of different interpretations.

Importation clarifications required (by IETF):

- Exact relationship between entities (e.g., 1:1, 1:N, etc.)

- Exact identifier of an entity ("are there any identifiers, and if, which one, given by an n-tuple of …")

It could be concluded that the evaluation of RTP multiplexing model demands for refined RTP terminology as originally introduced by RFC 1889, but the terminology update work by IETF is still not consistent and stable.

Bibliography

[b-ITU-T H.Sup.13]	Draft Supplement ITU-T H.Sup.13 (##/201#), Gateway Control Protocol: ITU-T H.248.x-series - Supplement on common H.248 terminology – Release 2.

{Editor’s note: this reference is not yet confirmed, dependent whether one or multiple common terms would be moved from H.248.RTPMUX to H.Sup13).

_______________________