| INTERNATIONAL TELECOMMUNICATION UNION |  | STUDY GROUP 16 |
| --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  | TD 424 (PLEN/16) |
|  |  | English only Original: English |
| Question(s): | 3/16 | Geneva, 12 - 23 October 2015 |
| TD |  |  |
| Source: | Editor H.248.95 |  |
| Title: | Consent: H.248.95 (ex H.248.RTPMUX) "Gateway Control Protocol: H.248 support for RTP multiplexing" (New) |  |

This TD contains draft new H.248.95 (ex. H.248.RTPMUX) “Gateway Control Protocol: H.248 support for RTP multiplexing”, which is proposed for Consent at this SG16 meeting.

AAP Summary

This Recommendation provides an overview of the types of RTP multiplexing and discusses how these types may be implemented on H.248 gateways. Multiplexing is becoming more prevalent in networks as to minimise the number of ports (pinholes) that need to be open for communication.

CONTENTS

Page

1 Scope 4 2 References 4 3 Definitions 5 3.1 Terms defined elsewhere 5 3.2 Terms defined in this Recommendation 6 4 Abbreviations and acronyms 7 5 Conventions 8 6 Overview of RTP multiplexing 8 6.1 Introduction 8 6.2 Protocol history: multiplexing framework defined for original RTP 9 6.3 RTP transport multiplexing 9 6.3.1 Limitations of RTP transport multiplexing 9 6.3.2 Control of RTP transport multiplexing 10 6.3.3 Relation to (RTP, non-RTP media) multiplexing in general 10 6.4 RTP media multiplexing 10 6.5 UDP payload multiplexing 10 7 Use cases with RTP multiplexing 10 7.1 General note 10 7.2 Use case "multimedia call in NGN/IMS" 11 7.3 Use case "Web Real-Time Communication" 11 8 H.248 control of RTP multiplexing 11 8.1 General principle 11 8.2 RTP media multiplexing 11 8.3 RTP transport multiplexing 11 9 RTP multiplexing examples 11 9.1 Video stream multiplexing 12 9.2 Audio and Video Stream in separate multiplexes 15 Appendix I Existing H.248 support for RTP multiplexing 19 I.1 3GPP-defined multiplexing for IP bearers with 3GPP framing protocol 19 Appendix II Functional processing model for RTP streams 20 II.1 Reference 20 II.2 Purpose 20 II.3 Functional processing model for RTP streams 20 II.4 Functional processing model with scope on RTP multiplexing 22 Bibliography 23

List of Tables

Page

Table 1 – Example H.248 command request for multiplexed streams 12 Table 2 – Example reply to a H.248 command request for multiplexed streams 14 Table 3 – Example H.248 command request for multiple multiplexed streams 15 Table 4 – Example reply to a H.248 command request for multiple multiplexed streams 17

List of Figures

Page

Figure 1 – Illustration of "MG RTP topology" 7 Figure 2 – Different multiplexing methods for RTP traffic and their used RTP header fields 9 Figure 3 – Example: Video Stream Multiplexing connection model 12 Figure 4 – Example: Video Stream Multiplexing connection model 15 Figure II.1 – Functional processing model for RTP streams – Example with coverage of media source mixing, scalable media encoding, RTP packet redundancy decomposed media transport 21 Figure II.2 – Functional processing model with scope on RTP multiplexing at sender side 22

Draft new Recommendation ITU-T H.248.95 (ex H.248.RTPMUX)

Gateway Control Protocol: 
H.248 support for RTP multiplexing

Summary

This Recommendation provides an overview of the types of RTP multiplexing and discusses how these types may be implemented on H.248 gateways.

Keywords

H.248, GCP, RTP, RTCP

# 1	Scope

Communication services utilising the Real Time Protocol (RTP) as application level framing protocol normally use a native IP-based transport mechanism [IETF RFC 3550], which results in using two separate UDP ports for the RTP and RTCP traffic components. H.248 gateways provide a rich set of gateway services for such kind of RTP traffic, such as defined by [ITU-T H.248.48], [ITU-T H.248.58], [ITU-T H.248.71], [ITU-T H.248.77], [ITU-T H.248.87] and [ITU-T H.248.88].

However in addition, multiple RTP-based traffic flow components could be multiplexed at multiple, different levels (from the perspective of a layered protocol architecture). The purpose of this Recommendation is to clarify how H.248 gateways support such "multiplexing structures".

The scope of this Recommendation addresses following subjects:

- terminology: clarification of RTP related terms and definitions;

- multiplexing methods: summary of existing multiplexing schemes in context of RTP-based communication services;

- H.248 support: provides information on how dedicated multiplexing methods could be supported by H.248 gateways inclusive of references to relevant signalling syntax; and

- examples: provides some signalling examples as complementary information.

This Recommendation itself does not define any new H.248 protocol extensions.

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (2013), Gateway Control Protocol: Version 3.

[ITU-T H.248.48]	Recommendation ITU-T H.248.48 (02/2012), Gateway control protocol: RTCP XR block reporting package.

[ITU-T H.248.50]	Recommendation ITU-T H.248.50 (09/2010), Gateway control protocol: NAT traversal toolkit packages.

[ITU-T H.248.57]	Recommendation ITU-T H.248.57 (03/2013), Gateway control protocol: RTP control protocol package.

[ITU-T H.248.58]	Recommendation ITU-T H.248.58 (06/2008), Gateway control protocol: Packages for application level H.248 statistics.

[ITU-T H.248.71]	Recommendation ITU-T H.248.71 (02/2010), Gateway control protocol: RTCP support packages.

[ITU-T H.248.77]	Recommendation ITU-T H.248.77 (06/2010), Gateway control protocol: Secure real-time transport protocol (SRTP) package and procedures.

[ITU-T H.248.87]	Recommendation ITU-T H.248.87 (01/2014), Gateway control protocol: Guidelines on the use of ITU-T H.248 capabilities for performance monitoring in RTP networks in ITU-T H.248 profiles.

[ITU-T H.248.88]	Recommendation ITU-T H.248.88 (01/2014), Gateway control protocol: RTP topology dependent RTCP handling by ITU-T H.248 media gateways with IP terminations.

[ITU-T H.248.93]	Recommendation ITU-T H.248.93 (10/2014), ITU-T H.248 support for control of transport security using the datagram transport layer security (DTLS) protocol.

[ITU-T H.248.96]	Recommendation ITU-T H.248.96 (10/2015), Gateway control protocol: H.248 Stream grouping and aggregation.

[IETF RFC 3550]	IETF RFC 3550 (2003), RTP: A Transport Protocol for Real-Time Applications.

[IETF RFC 5761]	IETF RFC 5761 (2010), Multiplexing RTP Data and Control Packets on a Single Port.

[IETF RFC 5764]	IETF RFC 5764 (2010), Datagram Transport Layer Security (DTLS) Extension to Establish Keys for the Secure Real-time Transport Protocol (SRTP).

# 3	Definitions

## 3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	media (clause 2.1/[b-IETF rtp-taxonomy]): a sequence of synthetic or physical Stimuli (sound waves, photons, key-strokes), represented in digital form. Synthesized media is typically generated directly in the digital domain.

3.1.2	media sink (clause 2.1.27/[IETF rtp-taxonomy]): The media sink receives a source stream that contains, usually periodically, sampled media data together with associated synchronization information. Depending on application, this source stream then needs to be transformed into a raw stream that is conveyed to the media render, synchronized with the output from other media sinks.

3.1.3	media source (clause 2.1.4/[b-IETF rtp-taxonomy]): The logical source of a reference clock synchronized, time progressing, digital media stream, called a source stream.  This transformation takes one or more raw streams and provides a source stream as output.  The output is synchronized with a reference clock, which can be as simple as a system local wall clock or as complex as NTP synchronized.

3.1.4	(H.248 media) stream (clause 3.2.9/[ITU-T H.248.1]): bidirectional media or control flow received/sent by a media gateway as part of a call or conference.

3.1.5	RTP end point (clause 2.2.1/[b-IETF rtp-taxonomy]): A single addressable entity sending or receiving RTP packets.  It may be decomposed into several functional blocks, but as long as it behaves as a single RTP stack entity it is classified as a single "Endpoint".

NOTE – The self-contained notion of 'endpoint' is consistent with [b-ITU-T H.Sup.13].

3.1.6	RTP end system [IETF 3550]: An application that generates the content to be sent in RTP packets and/or consumes the content of received RTP packets.  An end system can act as one or more synchronization sources in a particular RTP session, but typically only one.

NOTE – The term is used and important in context of "RTP topologies", see [ITU-T H.248.88]. The term is used in a synonym manner with "RTP endpoint" in [ITU-T H.248.88].

3.1.7	RTP stream (clause 2.1.10/[IETF rtp-taxonomy]): A stream of RTP packets containing media data, source or redundant. The RTP stream is identified by an SSRC belonging to a particular RTP session.

3.1.8	RTP session (clause 2.2.2/[IETF rtp-taxonomy]): An RTP Session is an association among a group of Participants communicating with RTP.  It is a group communications channel which can potentially carry a number of RTP Streams.  Within an RTP Session, every Participant can find meta-data and control information (over RTCP) about all the RTP Streams in the RTP Session.  The bandwidth of the RTCP control channel is shared between all Participants within an RTP Session.

NOTE – Clause 3.1.3 / [ITU-T H.248.57] provides following definition:

RTP session: An RTP session comprises a single RTP flow and an optional RTCP flow.

It could be concluded that the "H.248.57 RTP session" is synonym to RTP stream in general.

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	Media format: a particular pair of associated media encoder (clause 2.1.6/[b-IETF rtp-taxonomy]) and media decoder (clause 2.1.29/[b-IETF rtp-taxonomy]) entities.

3.2.2	Media type: a categorization concept for the classification of similar media formats (e.g., 'audio', 'video', etc.)

3.2.3	MG RTP topology: The configuration of RTP termination(s) (NOTE 1) for an individual RTP session with a single RTP stream within an RTP node. There are multiple RTP terminations in general besides the single case. The RTP terminations are meshed for the "multiple case" and interconnected via an interworking function (IWF).  There are a number of protocol layer related IWFs; at the RTP layer itself and at lower or upper layers. A particular RTP topology is characterized by a specific IWF.

NOTE 1 – H.248 terminology: an RTP-enabled H.248 termination or stream endpoint.

Important MG RTP topologies are designated by a name.

Figure 1 – Illustration of "MG RTP topology"

NOTE 2 – Figure 1 illustrates the key aspects of the "MG RTP topology" concept. The term "RTP leg" is undefined but not decisive due to the relation to "RTP session".

3.2.4	RTP media multiplexing: a single IP transport (L4) port for multiple RTP sources. There is a single unique SSRC per RTP source.

3.2.5	RTP node: Generic term used to designate a physical or logical network element with processing of RTP traffic, such as an IP host, terminal device, user equipment, gateway, media server, conference server, etc.

3.2.6	RTP source: a media packetizer (clause 2.1.9/[b-IETF rtp-taxonomy]) which produces RTP packets, i.e., a source RTP stream plus optional associated redundant RTP stream(s). A RTP source uses as primary identifier the SSRC parameter.

3.2.7	RTP / RTCP transport multiplexing (briefly "RTP transport multiplexing"): a single IP transport (L4) port for RTP and RTCP packets

3.2.8	UDP payload multiplexing: a single IP transport (L4) port for multiple, UDP-based application (upper layer) protocols. The first octet of the UDP payload is used for multiplexing. The application protocols are used in their native syntax, which limits UDP payload multiplexing only on scenarios which could guarantee unambiguity.

NOTE 1 – Example: [IETF RFC 5764] defines UDP payload multiplexing for the three UDP packet based applications of RTP, STUN and DTLS.

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| CSRC | Contributing Source |
| --- | --- |
| DTLS | Datagram Transport Layer Security |
| ICE | Interactive Connectivity Establishment |
| IMS | IP Multimedia Subsystem |
| IP | Internet protocol |
| IWF | Interworking function |
| MG | Media gateway |
| MGC | Media gateway controller |
| NAT | Network address translation |
| NGN | Next generation network |
| NTP | Network Time Protocol |
| PDU | Packet Data Unit |
| PT | Packet Type |
| RTCP | RTP Control Protocol |
| RTP | Real Time Protocol |
| SDP | Session description protocol |
| SIP | Session initiation protocol |
| SRTCP | Secure RTP Control Protocol |
| SRTP | Secure Real Time Protocol |
| SSRC | Sending Source |
| STUN | Session Traversal Utilities for NAT |
| UDP | User datagram protocol |
| WebRTC | Real-Time Communication in WEB-browsers (as work item in W3C) |

# 5	Conventions

None.

# 6	Overview of RTP multiplexing

## 6.1	Introduction

Multiple, RTP-based traffic flow components may be multiplexed at multiple, different levels (from the perspective of a layered protocol architecture).

A RTP-based traffic flow is given by a packet flow with a common identifier (also known as flow identifier or traffic descriptor). A particular multiplexing method uses specific information elements of such a traffic descriptor, which are mandatory for de-multiplexing again. Figure 1 summarizes the three major multiplexing techniques for RTP traffic, as well as the used header elements.

Figure 2 – Different multiplexing methods for RTP traffic 
and their used RTP header fields

In more detail:

- RTP transport multiplexing (see clause 6.3) uses the 8th-bit field of Marker bit (M) and 7-bit Payload Type (PT), which together comprise the 8-bit Packet Type;

- RTP media multiplexing (see clause 6.4) uses the 32-bit SSRC;

- UDP payload multiplexing (see clause 6.5) is based on the first octet of the UDP payload, which relates to the RTP header field indicated in Figure 2.

## 6.2	Protocol history: multiplexing framework defined for original RTP

Clause 5.2/[IETF RFC 3550] "Multiplexing RTP Sessions" describes the general multiplexing principles for RTP.

## 6.3	RTP transport multiplexing

RTP transport multiplexing represents the case when a RTP media flow and its associated RTCP control flow share a common IP transport address endpoint. In a normal, non-multiplexed scenario, RTP and RTCP use different endpoint identifiers on layer 4 (e.g., UDP ports in case of RTP-over-UDP transport).

Enabling of RTP transport multiplexing is achieved by the allocation of the same L4 port for RTCP as allocated for RTP.

Port allocation rules for RTCP at H.248 interfaces are subject of [ITU-T H.248.57], which covers both, the transport un-multiplexed and multiplexed cases.

RTP transport multiplexing is an extension to [IETF RFC 3550], motivated and defined by [IETF RFC 5761].

### 6.3.1	Limitations of RTP transport multiplexing

Clauses 4 and 5 in [IETF RFC 5761] describe a number of constraints, interactions and conditions to be considered when applying RTP transport multiplexing.

### 6.3.2	Control of RTP transport multiplexing

Control of RTP transport multiplexing:

- Using SDP: via SDP attribute "a=rtcp-mux" (see clause 8/[IETF RFC 5761]).

- The usage of this SDP element at the H.248 interfaces is defined by [ITU-T H.248.57].

### 6.3.3	Relation to (RTP, non-RTP media) multiplexing in general

RTP transport multiplexing is orthogonal to other kinds of multiplexing mechanisms (such as application-specific media level multiplexing options).

RTP transport multiplexing is basically (RTP) application agnostic, only driven by NAT traversal aspects.

## 6.4	RTP media multiplexing

RTP media multiplexing represents the case when multiple RTP streams of different RTP sources share a common IP transport address endpoint. In a normal, non-multiplexed scenario, individual RTP sources use different endpoint identifiers on layer 4 (e.g., UDP ports in case of RTP-over-UDP transport).

Enabling of RTP media multiplexing is achieved at SIP-based call control level via the SDP method "bundling" (see [b-IETF BUNDLE]). Bundling is enabled in H.248 via stream grouping [ITU-T H.248.96].

## 6.5	UDP payload multiplexing

UDP payload multiplexing represents the case:

- A single UDP connection is used plus

- UDP as transport protocol for RTP and RTCP plus

- secure RTP (SRTP and SRTCP) plus

- DTLS-based SRTP key exchange between the SRTP endpoints plus

- ICE/STUN –based NAT traversal support for the UDP connection.

The case referred to as "DTLS-SRTP" is defined by [IETF RFC 5764]. A single UDP connection is then used as the multiplexing structure for the three traffic flow components related to L4+ protocols SRTP, DTLS and STUN. The demultiplexing method is defined in clause 5.1.2 / [IETF RFC 5764], using the leading byte of the UDP packet payload.

Enabling of UDP payload multiplexing is achieved at SIP-based call control level via the indication of "DTLS-SRTP" in RTP related SDP media descriptions (see clause 8 / [IETF RFC 5764]). H.248 gateway control signalling indicates the application of UDP payload multiplexing at a particular H.248 Termination/Stream implicitly, via the signalling of the SRTP key management scheme [ITU-T H.248.77], usage of ICE/STUN [ITU-T H.248.50] and DTLS related bearer control [ITU-T H.248.93].

# 7	Use cases with RTP multiplexing

## 7.1	General note

There are theoretically at least the following reasons for the application of RTP multiplexing:

- To maximize the likelihood of successful  NAT traversal by minimizing the number of "pinholes" (i.e., number of IP transport connections);

- To minimize the call establishment delay in case of ICE-based NAT traversal procedures by minimizing the number of IP transport connections, because every candidate IP transport connection must firstly be tested for end-to-end connectivity before; or/and

- To "piggy back" the DTLS and STUN packet flows on the same RTP-over-UDP connection in case of DTLS-SRTP.

Reason (3) is already applicable for a single RTP stream ("monomedia call"), but all other multiplexing schemes only take effect for communication services with multiple RTP streams ("multimedia call").

The dominant use cases are driven by NAT traversal (1, 2) for multimedia communication services. This clause focuses on such scenarios.

## 7.2	Use case "multimedia call in NGN/IMS"

When multiplexing is used in a legacy NGN/IMS call, then RTP transport multiplexing is used in order to save L4 ports.

## 7.3	Use case "Web Real-Time Communication"

[b-ITU-T H.248.94] defines an H.248 WebRTC gateway, which is required to support RTP multiplexing as an inherent defined capability of this service. See [b-ITU-T H.248.94], e.g. clause 8.3 ("Requirements related to bearer traffic multiplexing ") and clause 8.4.3 ("RTP multiplexing").

# 8	H.248 control of RTP multiplexing

## 8.1	General principle

Multiplexing of multiple H.248 Streams onto one multiplexed Stream is primarily indicated by setting the same address/port information on the applicable H.248 Streams.

## 8.2	RTP media multiplexing

In order to multiplex several RTP streams the MGC may set the “association” (mgroup/groupse) property indicating which H.248 Streams on a Termination are associated together for the purposes of multiplexing, with the grouping type of “BUNDLE”.

[ITU-T H.248.96] describes these methods.

## 8.3	RTP transport multiplexing

Solely controlled via [ITU-T H.248.57], any kind of H.248 stream grouping [ITU-T H.248.95] is not required. It is not possible due to the lack of a correspondent grouping semantic.

# 9	RTP multiplexing examples

There are three basic RTP multiplexing methods, -

- RTP/RTCP transport multiplexing (see clause 6.3),

- RTP media multiplexing (see clause 6.4) and

- UDP payload multiplexing

[ITU-T H.248.57] provides tables on the use of H.248 properties and SDP attributes to enable RTP/RTCP transport multiplexing. No further examples are detailed in this Recommendation.

The examples in clauses 9.1 and 9.2 show RTP media multiplexing.

These clauses provide example use cases of RTP multiplexing. In each of the examples, Terminations (T1) and (T2) have five Streams. StreamIDs (1) and (2) are audio streams. StreamIDs (3), (4) and (5) are video streams. These RTP and H.248 streams are bi-directional. The MGC indicates Stream multiplexing by setting the same address/port information on the streams to be multiplexed and by setting the “BUNDLE” grouping semantic tying the Streams together.

## 9.1	Video stream multiplexing

In this example the video streams are multiplexed together on Termination (T1) using the “BUNDLE” grouping semantic. The scenario is illustrated in Figure 3.

Figure 3 – Example: Video Stream Multiplexing connection model

Tables 1 and 2 provide an example H.248 command request and response establishing the connection on Termination T1 illustrated in Figure 3.

Table 1 – Example H.248 command request for multiplexed streams

| H.248 encoding | Comments |
| --- | --- |
| MEGACO/3 [123.123.123.4]:55555 Transaction = 10004 { Context = 1 { Add = T1 { Media { TermationState { mgroup/groupse = [“BUNDLE 3 4 5”]}, Stream = 1 { Local { v=0 c=IN IP4 $ m=audio $ RTP/AVP 32 } Remote { v=0 c=IN IP4 193.193.193.193 m=audio 30000 RTP/AVP 32 } } Stream = 2 { Local { v=0 c=IN IP4 $ m=audio $ RTP/AVP 8 } Remote { v=0 c=IN IP4 193.193.193.193 m=audio 30002 RTP/AVP 8 } } Stream = 3 { Local { v=0 c=IN IP4 $ m=video $ RTP/AVP 32 } Remote { v=0 c=IN IP4 193.193.193.193 m=video 30004 RTP/AVP 32 } } Stream = 4 { Local { v=0 c=IN IP4 $ m=video $ RTP/AVP 33 } Remote { v=0 c=IN IP4 193.193.193.193 m=video 30004 RTP/AVP 33 } } Stream = 5 { Local { v=0 c=IN IP4 $ m=video $ RTP/AVP 34 } Remote { v=0 c=IN IP4 193.193.193.193 m=video 30004 RTP/AVP 34 } } } } } } | Termination (T1) is added with the two audio streams and three video streams. The “BUNDLE” grouping semantic is used to indicate that Streams 3, 4 and 5 are multiplexed together. |

The MG reply to the command request for multiplexed streams in Table 1 is shown in Table 2.

Table 2 – Example reply to a H.248 command request for multiplexed streams

| H.248 encoding | Comments |
| --- | --- |
| MEGACO/3 [124.124.124.222]:55555 Reply = 10004 { Context = 1 { Add=T1 { Media { Stream = 1 { Local { v=0 c=IN IP4 200.200.200.200 m=audio 50000 RTP/AVP 32 } Remote { v=0 c=IN IP4 193.193.193.193 m=audio 30000 RTP/AVP 32 } } Stream = 2 { Local { v=0 c=IN IP4 200.200.200.200 m=audio 50002 RTP/AVP 8 } Remote { v=0 c=IN IP4 193.193.193.193 m=audio 30002 RTP/AVP 8 } } Stream = 3 { Local { v=0 c=IN IP4 200.200.200.200 m=video 50004 RTP/AVP 32 } Remote { v=0 c=IN IP4 193.193.193.193 m=video 30004 RTP/AVP 32 } } Stream = 4 { Local { v=0 c=IN IP4 200.200.200.200 m=video 50004 RTP/AVP 33 } Remote { v=0 c=IN IP4 193.193.193.193 m=video 30004 RTP/AVP 33 } } Stream = 5 { Local { v=0 c=IN IP4 200.200.200.200 m=video 50004 RTP/AVP 34 } Remote { v=0 c=IN IP4 193.193.193.193 m=video 30004 RTP/AVP 34 } } } } } } |  |

## 9.2	Audio and Video Stream in separate multiplexes

In this example video streams are multiplexed over a transport connection and the audio streams are multiplexed over a separate transport connection on Termination (T1) using the “BUNDLE” grouping semantic. The scenario is illustrated in Figure 4.

Figure 4 – Example: Video Stream Multiplexing connection model

Tables 3 and 4 provide an example H.248 command request and response establishing the connection illustrated in Figure 4.

Table 3 – Example H.248 command request for multiple multiplexed streams

| H.248 encoding | Comments |
| --- | --- |
| MEGACO/3 [123.123.123.4]:55555 Transaction = 10004 { Context = 1 { Add = T1 { Media { TermationState { mgroup/groupse = [“BUNDLE 1 2”, “BUNDLE 3 4 5”]}, Stream = 1 { Local { v=0 c=IN IP4 $ m=audio $ RTP/AVP 32 } Remote { v=0 c=IN IP4 193.193.193.193 m=audio 30000 RTP/AVP 32 } } Stream = 2 { Local { v=0 c=IN IP4 $ m=audio $ RTP/AVP 8 } Remote { v=0 c=IN IP4 193.193.193.193 m=audio 30000 RTP/AVP 8 } } Stream = 3 { Local { v=0 c=IN IP4 $ m=video $ RTP/AVP 32 } Remote { v=0 c=IN IP4 193.193.193.193 m=video 30002 RTP/AVP 32 } } Stream = 4 { Local { v=0 c=IN IP4 $ m=video $ RTP/AVP 33 } Remote { v=0 c=IN IP4 193.193.193.193 m=video 30002 RTP/AVP 33 } } Stream = 5 { Local { v=0 c=IN IP4 $ m=video $ RTP/AVP 34 } Remote { v=0 c=IN IP4 193.193.193.193 m=video 30002 RTP/AVP 34 } } } } } } | Termination (T1) is added with the two audio streams and three video streams. The “BUNDLE” grouping semantic is used to indicate that Streams 3, 4 and 5 are multiplexed together and that Streams 2 and 3 are multiplexed together. |

The MG reply is described in Table 4.

Table 4 – Example reply to a H.248 command request for multiple multiplexed streams

| H.248 encoding | Comments |
| --- | --- |
| MEGACO/3 [124.124.124.222]:55555 Reply = 10004 { Context = 1 { Add=T1{ Media { Stream = 1 { Local { v=0 c=IN IP4 200.200.200.200 m=audio 50000 RTP/AVP 32 } Remote { v=0 c=IN IP4 193.193.193.193 m=audio 30000 RTP/AVP 32 } } Stream = 2 { Local { v=0 c=IN IP4 200.200.200.200 m=audio 50000 RTP/AVP 8 } Remote { v=0 c=IN IP4 193.193.193.193 m=audio 30000 RTP/AVP 8 } } Stream = 3 { Local { v=0 c=IN IP4 200.200.200.200 m=video 50002 RTP/AVP 32 } Remote { v=0 c=IN IP4 193.193.193.193 m=video 30002 RTP/AVP 32 } } Stream = 4 { Local { v=0 c=IN IP4 200.200.200.200 m=video 50002 RTP/AVP 33 } Remote { v=0 c=IN IP4 193.193.193.193 m=video 30002 RTP/AVP 33 } } Stream = 5 { Local { v=0 c=IN IP4 200.200.200.200 m=video 50002 RTP/AVP 34 } Remote { v=0 c=IN IP4 193.193.193.193 m=video 30002 RTP/AVP 34 } } } } } } |  |

Appendix I

Existing H.248 support for RTP multiplexing

(This appendix does not form an integral part of this Recommendation)

## I.1	3GPP-defined multiplexing for IP bearers with 3GPP framing protocol

Reference: see clause 6.4/[b-ETSI TS 129.414].

Motivation: transport capacity reductions (“bandwidth efficiency”).

Multiplexing method: multiple RTP packets mapped into a single PDU augmented by a (3GPP defined) multiplex header (“similar to IETF SHIM approach").

H.248 control: not any signalling support, thus pure provisioning based approach.

Appendix II

Functional processing model for RTP streams

(This appendix does not form an integral part of this Recommendation)

## II.1	Reference

The model provided by this Appendix is based on and aligned with the part models according to Figures 1 to 13 / [b-IETF rtp-taxonomy]. The various part models highlight specific aspects of RTP services and protocol functions.

## II.2	Purpose

The functional model uses the structure of a processing pipeline, considering the entire end-to-end media chain from communication source to communication sink. Such a model illustrates firstly the relationship between many terms. Secondly, H.248 media gateways are typically involved only in a sub-set of the processing pipeline, i.e., an H.248 Context, Termination, Stream Endpoint etc. covering only a few of the processing stages.

## II.3	Functional processing model for RTP streams

Figure II.1 illustrates an example model with coverage of media source mixing, scalable media encoding, RTP packet redundancy decomposed media transport.

Figure II.1 – Functional processing model for RTP streams – Example with coverage of media source mixing, scalable media encoding, RTP packet redundancy decomposed media transport

Stages Media Capturer (S:1) and Media Renderer (R:5)  are normally subject of terminal equipment, thus typically not located in H.248 MG entities. These stages are also not used in cases of H.248 RTP-to-RTP media gateways enforcing RTP topologies "RTP Media Translator" and "Back-to-Back RTP Endsystem" (see [ITU-T H.248.88]). For instance, such a RTP topology might be required due to media format translation (e.g., audio transcoding): the H.248 MG would then support processing stages from/up to the Media Encoder (S:3) and Media Decoder (R:3), or even the additional stages of Media Source (S:2) and Media Sink (R:4) in case of synchronization timing aspects.

Above example model is not exhaustive concerning all possible RTP protocol functions and capabilities (e.g., security is missing).

## II.4	Functional processing model with scope on RTP multiplexing

Figure II.2 focuses on RTP multiplexing at the sender side. The figure shows RTP media multiplexing for audio and video inclusive RTP transport multiplexing for the associated RTCP control flows.

Figure II.2 – Functional processing model with scope on RTP multiplexing at sender side

Stage "Media Transport Sender" (T:1) provides effectively the multiplexing point for RTP transport multiplexing, RTP media multiplexing and UDP payload multiplexing. The present definition of "Media Transport" in [b-IETF rtp-taxonomy] covers therefore un-multiplexed and multiplexed transport.

Bibliography

[b-ITU-T H.248.94]	Recommendation ITU-T H.248.94 (10/2015), Gateway control protocol: Web-based real-time communication services – H.248 protocol support and profile guidelines.

[b-ITU-T H.Sup.13]	Supplement ITU-T H.Sup.1310/2015), Gateway Control Protocol: ITU-T H.248.x-series - Supplement on common H.248 terminology – Release 2.

[b-ETSI TS 129 414]	ETSI TS 129 414 Version 9.0.0 (02/2010), Universal Mobile Telecommunications System (UMTS); Core network Nb data transport and transport signalling.

[b-IETF BUNDLE]	IETF draft-ietf-mmusic-sdp-bundle-negotiation-23 (07/2015), Negotiating Media Multiplexing Using the Session Description Protocol (SDP).

[b-IETF rtp-taxonomy]	IETF draft-ietf-avtext-rtp-grouping-taxonomy-08 (07/2015), A Taxonomy of Grouping Semantics and Mechanisms for Real-Time Transport Protocol (RTP) Sources.

____________________