| INTERNATIONAL TELECOMMUNICATION UNION |  | STUDY GROUP 16 |
| --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  | TD 320 (WP 1/16) |
|  |  | English only Original: English |
| Question(s): | 3/16 | Geneva, 12 - 23 October 2015 |
| TD |  |  |
| Source: | Editor H.248.CodecSDPprofile |  |
| Title: | H.248.CodecSDPprofile "Gateway control protocol: Profiling codec configurations via SDP in H.248 profiles – Guidelines and examples" (New): Output draft Ed. 0.6 |  |

1.	Introduction

This document contains the editor's output draft ed. 0.6 for draft new H.248.CodecSDPprofile "Gateway control protocol: Profiling codec configurations via SDP in H.248 profiles – Guidelines and examples" (New).

NOTE FROM TSB – please replace the images for Figures 1 and 2 with an editable version.

Reply from Editor: both Figures are available as PowerPoint diagrams and inserted as Microsoft Office Graphic Object. Can't find any PasteSpecial options which allow Word-embedded editing. (any guidance?)

| Document History |  |
| --- | --- |
| Issue | Notes |
| H.248.CodecSDPprofile Ed. 0.1 | Initial draft (AVD-4633) to WP 1/16 Seoul (KR) meeting (2014-11) Location: http://wftp3.itu.int/av-arch/avc-site/2013-2016/1411_Seo/AVD-4633.zip |
| H.248.CodecSDPprofile Ed. 0.2 | Output (TD-21) from WP 1/16 Seoul (KR) meeting (2014-11) Location: |
| H.248.CodecSDPprofile Ed. 0.3 | Input (TD 217/WP1) to ITU-T SG16 Geneva meeting, Feb 2015 Location: http://www.itu.int/md/T13-SG16-150209-TD-WP1-0217/en |
| H.248.CodecSDPprofile Ed. 0.4 | Output (TD 217R1/WP1) from ITU-T SG16 Geneva meeting, Feb 2015 Location: http://www.itu.int/md/T13-SG16-150209-TD-WP1-0217/en |
| H.248.CodecSDPprofile Ed. 0.5 | Input (AVD-4668) to WP 1/16 Chengdu meeting (2015-06) Location: http://wftp3.itu.int/av-arch/avc-site/2013-2016/1506_Gui/AVD-4668.zip |
| H.248.CodecSDPprofile Ed. 0.6 | Output (TD 320/WP1) from ITU-T SG16 Geneva meeting, Oct 2015 Location: http://www.itu.int/md/T13-SG16-151012-TD-WP1-0320/en |
| H.248.CodecSDPprofile Ed. 0.7 |  |
| H.248.CodecSDPprofile Ed. 0.8 |  |
| H.248.CodecSDPprofile Ed. 0.9 |  |
| H.248.CodecSDPprofile Ed. 0.10 |  |

CONTENTS

Page

1 Scope 6 2 References 8 3 Definitions 8 3.1 Terms defined elsewhere 8 3.2 Terms defined in this Recommendation 9 4 Abbreviations and acronyms 10 5 Conventions 11 6 Problem statement 11 6.1 Codec handling by H.248 media gateways 11 6.2 Mapping SDP information from call control level to H.248 gateway control 12 6.3 Codec configuration complexity of modern multi-mode codec types 14 6.4 SDP specification level in H.248 profiles 14 6.5 Summary 15 7 Models 15 7.1 Half-call model 15 7.2 Specification models 17 7.2.1 Call control centric model (e.g. 3GPP) 17 7.2.2 Gateway control centric model (ITU-T H.248) 18 7.3 Codec parameter value assignment model 18 7.3.1 Local source media endpoint (local media encoder) 18 7.3.2 Local destination media endpoint (local media decoder) 18 8 Codec profiles 20 8.1 Configuration profiles for audio codecs 20 8.1.1 Parameterization of audio codec "AMR-WB" 20 8.1.2 Parameterization of audio codec "OPUS" 26 8.2 Configuration profiles for video codecs 33 8.3 Configuration profiles for text codecs 33 9 H.248 profile guidelines 33 9.1 Profile Identification 33 9.2 Summary 33 9.3 Gateway Control Protocol Version 33 9.4 Connection Model 33 9.5 Context Attributes 33 9.6 Terminations 33 9.7 Descriptors 33 9.7.1 TerminationState Descriptor 33 9.7.2 Stream Descriptor 33 9.7.3 to 9.7.10 Descriptor types 33 9.7.11 Error Descriptor 34 9.8 Command API 34 9.9 Generic command syntax and encoding 34 9.10 Transactions 34 9.11 Messages 34 9.12 Transport 34 9.13 Security 34 9.14 Packages 34 9.15 Mandatory support of SDP and ITU-T H.248.1 Annex C information elements 34 9.16 Optional support of SDP and ITU-T H.248.1 Annex C information elements 34 9.17 Procedures 35 Appendix I Negotiation of codec configurations – Examples 36 I.1 Example signalling flow for an outgoing call 36 Bibliography 38

List of Tables

Page

Table 8.1– SDP parameters for "audio/AMR-WB" - MG Local and Remote AMR(-WB) parameter semantic 21 Table 8.x1 – SDP parameters for "audio/OPUS" [IETF RFC 6716] 29 Table 8.x2 – SDP parameters for "audio/OPUS" - MG Local and Remote OPUS parameter semantic 31

List of Figures

Page

Figure 1 – Mapping SDP information between call control signaling and H.248 gateway control signaling (copy of Fig 1/[b-ETSI TR 183 046]) 12 Figure 2 – SDP mapping directions - SIP-to-H.248 (1) and H.248-to-SIP (2) (copy of Fig 4/[ b-ETSI TR 183 046]) 13 Figure 3 – Half-call model "ETSI TISPAN Resource Admission Control Subsystem" 14 Figure 4 – Half-call model "IP Multimedia Subsystem" 16 Figure 5 – Call control centric parameter specification model 17 Figure 6 – Gateway control centric parameter specification model 18 Figure 7 – Codec parameter value assignment model: example for local destination media endpoint 19 Figure 8 – Example use cases with "OPUS" 27 Figure I.1 – Example signalling flow for an outgoing call 36

Draft new ITU-T Recommendation H.248.CodecSDPprofile

Gateway control protocol: 
Profiling codec configurations via SDP in H.248 profiles 
– Guidelines and examples

AAP Summary

<to be provided before Consent>

Summary

<Mandatory material>

Keywords

<Optional>

Introduction

<Optional - This clause should appear only if it contains information different from Scope and Summary>

# 1	Scope

{Editor's note (2014-11): scope on H.248 issues, not on general aspects …}

Motivation:

Purpose of such a new Recommendation is to provide guidance concerning the SDP profiling (for the H.248 interface) of media configurations. A media configuration represents the set of parameter/value assignments for a particular media type specific media format. The notion of 'codec' is often used whenever the media format relates to the configuration of encoder/decoder entities. Profiling of such codec configurations via SDP in H.248 profiles need to take into account a couple of aspects. See next item as well as clause 6 for more detailed background information.

Aspects for signalling of codec configurations:

Profiling codec configurations via SDP in H.248 profiles need to take into account at least following aspects:

- mapping of SDP information between call control signalling (here SIP) and H.248;

- SDP semantics might be further precised for gateway control;

- SDP codec parameters might be omitted in call control signalling (in case of default values), but not in H.248 signalling and vice versa;

- SDP elements (with scope on LocalControl descriptor level) needs to be mapped/duplicated to Local / Remote descriptor level;

- alignment (or not) of SDP information between Local and Remote descriptors;

- symmetry conditions concerning traffic directions in case of bidirectional communication services;

- early, partial establishment of media configuration in parallel already to a still ongoing call level end-to-end negotiation process;

- location of control plane mediation function concerning the end-to-end negotiation fo media configurations ("codec broker");

- H.248 default model: MGC-level codec broker, i.e., all information and decisions (as the usual responsibility of a policy decision point) are made by the master entity, providing then straightforward and unambiguous media configuration requests down the H.248 MG as slave entity;

- MG-level codec broker support: there might be network operational or service delivery aspects which demand for basic MG support concerning that capability, e.g., the MGC could be offloaded from SDP mapping (and could then just forward SDP unmodified media configurations), or the fast signaling and early as possible establishment of the end-to-end media path;

- codec restrictions concerning limited support of codec parameters or/and value settings;

- codec profiles defined for terminal and user equipment (e.g., such as by GSMA, 3GPP, etc. for mobile endpoints);

- codec profiles given by network infrastructure support or network operator preferences;

- media interworking functions (IWF): mode of operation of H.248 MG;

- an H.248 MG might be limited concerning the plethora of possible IWFs between the pure media-agnostic forwarding mode towards full-fledged media-aware type of interworking services.

- codec configuration parameter types:

- codec type dependent parameters, i.e., parameters introduced and inherent of a specific codec type (e.g., parameter "maxplaybackrate" of codec type "OPUS"); and

- codec type independent parameters, i.e., generic parameters which are applicable for multiple codec types (e.g., parameters related to packetization times (minimum, nominal, maximum).

Primary focus is codec type dependent parameters; however, comprehensive specifications of complete codec configurations imply often the consideration of codec type independent parameters as well.

Specification content and format:

Hence, there are quite a number of aspects which would be beneficial with respect to SDP usage specification in H.248 profiles. There might be consequently different levels concerning specification detail and different formats how such SDP information could be defined.

However, it looks like that above aspects result at least in two specification formats,

- an overview table which lists all SDP (codec) parameters, associated value range, default values, value restrictions, mandatory/optional tagging, etc; and

- a table with focus on parameter semantics concerning usage in

- MGC-to-MG command requests: Local and Remoter descriptor settings and interpretation;

- subsequent MG-to-MGC command replies: Local and Remoter descriptor settings; and

- resulting MG resource configurations (per traffic direction).

The examples in this Recommendation follow that kind of table structures.

Codec types in scope:

In scope of such a Recommendation are modern codec technologies, associated with a plethora of configuration options. Out of scope are the "simple" legacy codec types.

Guidelines for H.248 profiles:

It is recognized that there is not a "single SDP profile setting" for such codecs. However, many aspects need to be considered when "profiling". Hence, such a new Recommendation should focus on guidelines, the evaluation of important aspects and point out which sections would be impacted in H.248 profiles.

Such a Recommendation may also only focus on example codec types due to the ongoing evolution of codec technologies.

# 2	References

The following ITU-T Recommendations and other references contain provisions which, through reference in this text, constitute provisions of this Recommendation. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; users of this Recommendation are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below. A list of the currently valid ITU-T Recommendations is regularly published.

The reference to a document within this Recommendation does not give it, as a stand-alone document, the status of a Recommendation.

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (03/2013), Gateway control protocol: Version 3.

[ITU-T H.248.39]	Recommendation ITU-T H.248.39 (07/2014), Gateway control protocol: H.248 SDP parameter identification and wildcarding.

[ITU-T H.248.49]	Recommendation ITU-T H.248.49 (08/2007), Gateway control protocol: Session description protocol RFC and capabilities packages.

[ITU-T H.248.80]	Recommendation ITU-T H.248.80 (01/2014), Gateway control protocol: Usage of the revised SDP offer/answer model with ITU-T H.248.

[IETF RFC 5939]	IETF RFC 5939 (2010), Session Description Protocol (SDP) Capability Negotiation.

[IETF RFC 6871]	IETF RFC 6871 (2013), Session Description Protocol (SDP) Media Capabilities Negotiation.

# 3	Definitions

## 3.1	Terms defined elsewhere

This Recommendation uses the following terms defined elsewhere:

3.1.1	actual configuration [IETF RFC 6871]: An actual configuration specifies which combinations of SDP session parameters and media stream components can be used in the current offer/answer exchange and with what parameters. Use of an actual configuration does not require any further negotiation in the offer/answer exchange. See [IETF RFC 5939] for further details.

3.1.2	configuration [ITU-T H.248.80]: A configuration relates to a set of capabilities, which may be indicated and negotiated via a signalling protocol, or provisioned via a management protocol.

NOTE – This definition is derived from [IETF RFC 5939], which itself does not provide an explicit definition of this term.

3.1.2.1	media configuration [ITU-T H.248.80]: A media configuration typically covers capabilities such as media type, media format, all media format attributes, media transport stack, media transport capacity and all associated parameter values.

NOTE 1 – The notion of 'media' originates from the concept of "IP media-path" (which relates to the ITU-T H.248 bearer-path), as opposed to the "IP signalling-path". The IP packet flow of the IP media-path carries the application data between IP application endpoints and the IP signalling-path carries the correspondent application control information.

NOTE 2 – A media configuration is specified within the SDP media description block. The relationship between the ITU-T H.248 "Media Descriptor" and SDP "Media Description" is illustrated in Figure 10 in [ITU-T H.248.1].

3.1.2.2	session configuration [ITU-T H.248.80]: Defines a combination of multiple media configurations.

NOTE – This definition is derived from [IETF RFC 6871], which itself does not provide an explicit definition of this term.

3.1.3	configuration (codec) list [ITU-T H.248.80]: The term configuration list represents a list of media configurations within the SDP media description block (in the case of SIP-embedded SDP).

3.1.4	latent configuration [IETF RFC 6871]: A latent configuration indicates which combinations of capabilities could be used in a future negotiation for the session and its associated media stream components. Latent configurations are neither ready for use, nor are they offered for actual or potential use in the current offer/answer exchange. Latent configurations merely inform the other side of possible configurations supported by the entity. Those latent configurations may be used to guide subsequent offer/answer exchanges, but they are not offered for use as part of the current offer/answer exchange.

NOTE – Latent configurations are beneficial for fast transitioning between different media configurations during active communication phases. Examples: IP emulation services for PSTN teleservices like "alternate speech-facsimile" or "text telephony" calls.

3.1.5	potential configuration [IETF RFC 6871]: A potential configuration indicates which combinations of capabilities can be used for the session and its associated media stream components. Potential configurations are not ready for use, however they are offered for potential use in the current offer/answer exchange. They provide an alternative that may be used instead of the actual configuration, subject to negotiation in the current offer/answer exchange. See [IETF RFC 5939] for further details.

## 3.2	Terms defined in this Recommendation

This Recommendation defines the following terms:

3.2.1	Bearer-level codec broker (or media-level codec broker):  a codec broker at the level of a media path acting interworking functions, providing (limited) mediation signalling support. Here: MG-level codec broker. Codec broker service is basically feasible, but inherently restricted due to limited access on information typically required for a comprehensive mediation function.

3.2.2	Call-level codec broker:  a codec broker directly embedded in the application and call control signaling path. Here: MGC-level codec broker.

3.2.3	Codec broker:  A control plane mediation function located between two or more communicating media endpoints with the purpose of the negotiation of media configurations during communication establishment process, which targets finally a successful communication. The mediation function takes into account:

- the requested (or offered) list of media configuration(s),

- the supported (or answered) list of media configuration(s),

- the supported list of media configuration(s) of media level interworking functions (such as H.248 media gateways or H.248 media servers),

- media policy rules related to network operator preferences,

- available transport capacity for media information,

- the current network conditions in terms of Grade of Service (GoS), or

- other criteria.

The mediation function results either in a single, common media configuration for all media endpoints within a context, or in different media configurations that imply interworking support by in-path equipment (H.248 media gateways).

3.2.4	codec configuration: A subset of a media configuration which includes the capabilities related to media type, media format and all media format attributes, but excludes other information such as  media transport protocol stack, media transport capacity, etc.

# 4	Abbreviations and acronyms

This Recommendation uses the following abbreviations and acronyms:

| 3GPP | 3rd Generation Partnership Project |
| --- | --- |
| AMR | Adaptive Multirate (codec) |
| B2BUA | Back-to-Back User Agent |
| D | Decoder |
| E | Encoder |
| ETSI | European Telecommunications Standards Institute |
| GoS | Grade of Service |
| GSMA | Global System for Mobile Communications Alliance |
| IMS | IP Multimedia Subsystem |
| IP | Internet Protocol |
| IPv4 | Internet Protocol Version 4 |
| IPv6 | Internet Protocol Version 6 |
| IWF | Interworking Function |
| L3 | Layer three |
| L4 | Layer four |
| L4+ | Above layer four |
| MG | Media Gateway |
| MGC | Media Gateway Controller |
| NB | Narrowband |
| QoE | Quality of Experience |
| QoS | Quality of Service |
| RACS | (ETSI) Resource Admission Control Subsystem |
| RTP | Real-time Transport Protocol |
| SDP | Session Description Protocol |
| SEP | Stream Endpoint |
| SEPP | Stream Endpoint Pair |
| SIP | Session Initiation Protocol |
| TISPAN | (ETSI) Telecommunication and Internet converged Services and Protocols for Advanced Networking |
| UA | (SIP) User Agent |
| UDP | User Datagram Protocol |
| UE | User Equipment |
| WB | Wideband |

# 5	Conventions

This document provides a list of items, labelled as R-x/y, where x refers to the clause number and y a number within that clause. Such items use the following keywords with meanings as prescribed below:

The keywords "is required to" indicate a requirement which must be strictly followed and from which no deviation is permitted if conformance to this document is to be claimed.

The keywords "is prohibited from" indicate a requirement which must be strictly followed and from which no deviation is permitted if conformance to this document is to be claimed.

The keywords "is recommended" indicate a requirement which is recommended but which is not absolutely required.  Thus this requirement need not be present to claim conformance.

The keywords "can optionally" indicate an optional requirement that is permissible, without implying any sense of being recommended. This term is not intended to imply that the vendor’s implementation must provide the option and the feature can be optionally enabled by the network operator/service provider.  Rather, it means the vendor may optionally provide the feature and still claim conformance with the specification.

# 6	Problem statement

## 6.1	Codec handling by H.248 media gateways

Codec is the abbreviation for encoder/decoder and represents in general a media format of a particular media type (such as audio, video, text, etc.). The information is carried in IP packets (typically using the RTP) in the Internet and represents application data because the "codec protocol" belongs to the category of IP application protocols. The terms media and application data are usually synonym in the context of codecs. The main purpose of H.248 media gateways is actually the processing of such media traffic, - and the particular mode of operation is subject of the signaled SDP media descriptions (in case of text encoding mode for H.248 messages) in H.248 descriptors (i.e., the H.248 Local and/or Remote descriptor within the H.248 Stream and Media descriptor, see [ITU-T H.248.1], clause 7.1.8.1.1.2 "Summary – ITU-T H.248 usage of SDP").

There are a plethora of mode of operations for media, - known as media interworking functions -, where the entire spectrum is bounded by the high-level modes from "media-aware" towards "media-agnostic".

Thus, the precise and unambiguous specification of the aimed codec handling mode via SDP is unfortunately not straightforward, rather a well-known challenging task.

## 6.2	Mapping SDP information from call control level to H.248 gateway control

SDP could be used at call control level besides the H.248 interface (Figure 1), leading to the aspect in carefully considering the mapping process of SDP information between the two different protocol categories (Figure 2). This is a well-known topic (see e.g. [b-ETSI TR 183 046]) and not repeated again in this Recommendation. [ITU-T H.248.80] provides for instance detailed information in clauses 6 and 7 concerning the interworking of the call control level SDP Offer/Answer process with H.248 resource configuration procedures.

H.248

Media Gateway

(MG or MGW)

H.248

Media Gateway Controller (MGC)

Network Element handling 
both H.248/SDP and SIP/SDP Signalling

Scope of

[ b-ETSI TR 183 046]

H.248/SDP

Gateway Control Protocol

(text encoding mode)

SIP

e.g., SIP Proxy, SIP User Agent, etc

SDP Mapper

SIP/SDP

SIP-I

Session

Initiation Protocol

Figure 1 – Mapping SDP information between call control signaling and H.248 gateway control signaling (copy of Fig 1/[b-ETSI TR 183 046])

H.248

Media Gateway

(MG or MGW)

Media
Gateway Controller

H.248/SDP

SIP

SDP

Mapper

SIP/SDP

1

2

Figure 2 – SDP mapping directions - SIP-to-H.248 (1) and H.248-to-SIP (2) 
(copy of Fig 4/[ b-ETSI TR 183 046])

However, besides the direct, one-stage mapping of SDPSIP to SDPH.248 (as in the so called IP Multimedia Subsystem, IMS), there are other network architectures with indirect mappings, e.g., the two-stage mapping in ETSI's Resource Admission Control Subsystem (RACS), see Figure 3:

Figure 3 – Half-call model "ETSI TISPAN Resource Admission Control Subsystem"

The chained mapping here of SDPSIP to Diameter to SDPH.248 introduces an additional level of consideration concerning codec configurations in H.248 media gateways.

## 6.3	Codec configuration complexity of modern multi-mode codec types

The first generations of codec technologies were fairly simple in their configuration due to their inherent limited support of modes of operation. For instance, the PCM audio codec [ITU-T G.711] allows only modified configurations concerning support of silence suppression and packetization times. Such an SDP based codec description is simple and straightforward from SDP mapping perspective.

However, modern codec technologies support a plethora of operation modes in order to address the variety of operational environments, given by available network transport capacity (bitrate), varying network grade of service, varying network traffic load, processing capabilities of user equipment, consideration of embedded backward compatibility modes, etc. The signalling (declaration and negotiation) of such codec configurations implies quite extensive SDP media descriptions.

## 6.4	SDP specification level in H.248 profiles

The H.248 profile definition template contains explicit clauses for (mandatory and optional) SDP information (see [ITU-T H.248.1], Appendix III: template sections 6.15 and 6.16). These template clauses are used in standardized H.248 profiles, but typically only for the top-level SDP information. There was just not yet any need to detail SDP information for codec configurations so far. That situation might be still valid for text codec types, but definitely not any more for modern audio and video codecs.

## 6.5	Summary

The usage of modern codecs in H.248 media gateways, - under consideration of the variety of potential modes of operation and interworking functions -, implies a common and ambiguous understanding between MGC and MG, i.e., a sufficient detailed H.248 profile specification. That task results actually in the "profiling of codec configurations based SDP". It is obvious that a Recommendation could only provide some guidance by considering a selection of example codec types. The actual SDP specification (for the supported codec types) is still subject of a concrete H.248 profile definition.

The next clause describes the (non-exhaustive list of) aspects which are considered by this Recommendation.

# 7	Models

## 7.1	Half-call model

The half-call model as illustrated by Figure 4 is used as framework in this Recommendation. The "half-call" reflects a terminal-to-network infrastructure configuration, i.e., an inherent asymmetrical relationship concerning the processing of media traffic (because the served user instance of media information is located in the communication endpoint, but not in the H.248 media gateway).

Figure 4 – Half-call model "IP Multimedia Subsystem"

Purpose of this Recommendation is to provide guidance concerning the SDP profiling (for the H.248 interface) for media configurations. Profiling codec configurations via SDP in H.248 profiles need to take into account at least following aspects:

  - mapping of SDP information between call control signalling (here SIP) and H.248;

- SDP semantics might be further precised for gateway control;

- SDP codec parameters might be omitted in call control signalling (in case of a default), but not in H.248 signalling and vice versa;

- SDP elements (with scope on LocalControl descriptor level) needs to be mapped/duplicated to Local / Remote descriptor level;

- alignment (or not) of SDP information between Local and Remote descriptor;

- symmetry conditions concerning traffic directions in case of bidirectional communication services;

- early, partial establishment of media configuration in parallel already to still ongoing call level end-to-end negotiation process;

  - location of control plane mediation function concerning the end-to-end negotiation fo media configurations ("codec broker");

- H.248 default model: MGC-level codec broker, i.e., all information and decisions (as the usual responsibility of a policy decision point) are made by the master entity, providing then straightforward and unambiguous media configurations down the H.248 MG as slave entity;

- MG-level codec broker support: there might be network operational or service delivery aspects which demand for basic MG support concerning that capability, e.g., the MGC could be offloaded from SDP mapping (and could then just forward SDP unmodified media configurations), or the fast signaling and early as possible establishment of the end-to-end media path;

  - Codec restrictions concerning limited support of codec parameters or/and value settings;

- codec profiles defined for terminal and user equpipment (e.g., such as by GSMA, 3GPP, etc for mobile endpoints);

- codec profiles given by network support or preferences;

  - Media interworking functions (IWF): mode of operation of H.248 MG;

- an H.248 MG might be limited concerning the plethora of possible IWFs between the pure media-agnostic forwarding mode towards full-fledged media-aware type of interworking services.

## 7.2	Specification models

## 7.2.1	Call control centric model (e.g. 3GPP)

The standardization organization 3GPP provides codec parameter specifications for 3GPP-specific H.248 gateways, at the specification level of signalling requirements and principle signalling procedures, see e.g. [b-ETSI TS 123 334]. The primary specification scope is the SIP-level SDP offer/answer process from perspective of a SIP B2BUA function (see Figure 5). Those call control centric specifications consider "H.248 aspects" in the knowledge that supported codec configurations of all associated MG entities are known. Such information is typically known a-priori (e.g., due to auditing procedures or configuration management), therefore stored in a SIP B2BUA local data base (called media (codec) policies here).

Figure 5 – Call control centric parameter specification model

### 7.2.2	Gateway control centric model (ITU-T H.248)

Figure 6 illustrates the gateway control centric specification model as in scope of this Recommendation (for point-to-point communication):

Figure 6 – Gateway control centric parameter specification model

The consideration of a MG-embedded media interworking function for bidirectional communication paths in conjuction with the H.248 specific control method of direction-individual descriptors leads to  more detailed protocol semantical levels (marked as 1, 2 and 3 in Figure 6):

Level 1: isolated view of Local Descriptor and Remote Descriptor.

Level 2: additional consideration of potential relationships between both traffic directions, i.e., semantic of the LD AND RD within the H.248 Stream endpoint (SEP).

Level 3: additional consideration of a unicast end-to-end media stream, i.e., the associated LD AND RD of paired SEPs (i.e., a SEPP).

The 3rd level represents the general case and default for H.248 (T1, T2) connection models. It could be further concluded that the semantic of codec parameters with call control SDP Offer/Answer (Figure 5) is not necessarily identical to gateway control centric semantic (Figure 6).

## 7.3	Codec parameter value assignment model

### 7.3.1	Local source media endpoint (local media encoder)

The MG resource configuration needs basically to follow the request of the remote media endpoint, limited by the supported codec configurations by the MG. The correspondent processing of the H.248 Remote Descriptor is rather straightforward and not discussed in detail here.

## 7.3.2	Local destination media endpoint (local media decoder)

The management of such resources is the prime responsibility of the MG, hence a crucial task in context of policy rules related to end-to-end QoS optimization, minimization of media interworking effort, minization of cost factors related to resources, implementation of service/network provider objectives, etc.

Figure 7 illustrates an example model, taking into account in firstly trying to align codec configuration settings per traffic direction across a H.248 SEPP, and secondly trying to achieve symmetrical settings concerning the bidirectional media stream. And the second rule takes precedence over the first rule in this example.

Figure 7 – Codec parameter value assignment model: example for local destination media endpoint

# 8	Codec profiles

Selection of example media codecs.

## 8.1	Configuration profiles for audio codecs

### 8.1.1	Parameterization of audio codec "AMR-WB"

Table 8.1.1.2 provides in general then H.248 procedural considerations by evaluating the combination of settings in Local and Remote descriptors, in MGC-to-MG direction and vice versa, as well as impact on the resource configuration status in the MG:

Particular focus are the semantics of MG Local and Remote AMR(-WB) parameters in case of matching Local and Remote AMR(-WB) based media formats.

NOTE 1: Here it is assumed that the corresponding AMR(-WB) based media formats in Local and Remote have already been identified as matching.

NOTE 2: If a specific parameter is not present in Local or Remote within the MGC's request, then this table assumes that this parameter is used with ist default value as decribed in RFC 4867.

NOTE  3: 3GPP specific AMR(-WB) parameter descriptions can be found in [ETSI TS 126 236], clause 5.1.1 and in [ETSI TS 129 163], clause B.2.5.

{Editor's note (2014-11): The table should be simplified. It was thought that most cases could be covered by the general statement that codec parameters are bi-directional and should be set in both the Local and Remote descriptors. It was felt that the “No” / “n/a” cells could also be removed as they were largely superfluous.}

Table 8.1– SDP parameters for "audio/AMR-WB" - MG Local and Remote AMR(-WB) parameter semantic

| Index | SDP "AMR-WB" parameter | MGC Request (Add/Modify/Move) |  |  | MG H.248 Reply |  | MG resource configuration |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Local Descript. | Remote Descriptor |  | Local Descript. | Remote Descr. | per traffic direction |  |
|  |  | Value (NOTE 1) | Present Yes/No | Value | Value | Value | Ingress | Egress |
| R-8.1.1.2/1: | octet-align | octet-align_Local; Bandwidth-efficient operation to be expected in Rx direction | Yes | octet-align_Remote == octet-align_Local; Bandwidth-efficient operation to be used in Tx direction; | octet-align_Local | octet-align_Remote == octet-align_Local | octet-align_Local == octet-align_Remote | octet-align_Remote |
|  |  |  | No | n/a |  | n/a | octet-align_Local | n/a "SEP" is ReceiveOnly |
| R-8.1.1.2/2: | mode-set | mode-set_Local; Mode-set to be expected in Rx direction | Yes | mode-set_Remote; Mode-set to be used in Tx direction | mode-set_Local | mode-set_Remote | n/a | mode-set_Remote |
|  |  |  | No | n/a |  | n/a | n/a | n/a "SEP" is ReceiveOnly |
| R-8.1.1.2/3: | mode-change-period | mode-change-period_Local; mode change period to be expected in Rx direction | Yes | mode-change-period_Remote; mode change period to be used in Tx direction | mode-change-period_Local [Thus mode-change-period is returned in replied Local as received in requested Local] | mode-change-period_Remote | n/a | mode-change-period_Remote or always mode-change-period=2 |
|  |  |  | No | n/a |  | n/a | n/a | n/a "SEP" is ReceiveOnly |
| R-8.1.1.2/4: | mode-change-capability | mode-change-capability_Local | Yes | mode-change-capability_Remote | mode-change-capability_Local | mode-change-capability_Remote | n/a | n/a |
|  |  |  | No | n/a |  | n/a | n/a | n/a "SEP" is ReceiveOnly |
| R-8.1.1.2/5: | mode-change-neighbor | mode-change-neighbor_Local; mode-change-neighbor to be expected in Rx direction | Yes | mode-change-neighbor_Remote; mode change neighbor to be used in Tx direction; | mode-change-neighbor_Local | mode-change-neighbor_Remote | n/a | mode-change-neighbor_Remote [mode-change-neighbor_Local is not supervized in Rx direction, rather any mode changes between any two modes within the allowed mode set are accepted] |
|  |  |  | No | n/a |  | n/a | n/a | n/a "SEP" is ReceiveOnly |
| R-8.1.1.2/6: | maxptime | FIXTHIS | Yes | FIXTHIS | FIXTHIS | FIXTHIS | FIXTHIS | FIXTHIS |
|  |  |  | No | n/a |  | n/a | FIXTHIS | FIXTHIS |
| R-8.1.1.2/7: | crc | crc_Local; CRC mode to be expected in Rx direction | Yes | crc_Remote == crc_Local; CRC mode to be used in Tx direction; | crc_Local | crc_Remote == crc_Local | crc_Local == crc_Remote | crc_Remote |
|  |  |  | No | n/a |  | n/a | crc_Local | n/a "SEP" is ReceiveOnly |
| R-8.1.1.2/8: | robust-sorting | robust-sorting_Local; robust sorting to be expected in Rx direction; AMR(-WB) media format with robust-sorting_Local == 1 should be considered as not supported as long as not supported by MG | Yes | robust-sorting_Remote == robust-sorting_Local; robust sorting to be used in Tx direction; AMR(-WB) media format with robust-sorting_Remote == 1 should be considered as not supported as long as not supported by MG | robust-sorting_Local | robust-sorting_Remote == robust-sorting_Local | robust-sorting_Local = robust-sorting_Remote | robust-sorting_Remote |
|  |  |  | No | n/a |  | n/a | robust-sorting_Local | n/a "SEP" is ReceiveOnly |
| R-8.1.1.2/9: | interleaving | interleaving_Local; interleaving to be expected in Rx direction; AMR(-WB) media format with present interleaving attribute in Local should be considered as not supported as long as not supported by MG | Yes | interleaving_Remote == interleaving_Local; interleaving to be used in Tx direction; AMR(-WB) media format with present interleaving attribute in Remote should be considered as not supported as long as not supported by MG | interleaving_Local | interleaving_Remote == interleaving_Local | interleaving_Local = interleaving_Remote | interleaving_Remote |
|  |  |  | No | n/a |  | n/a | interleaving_Local | n/a "SEP" is ReceiveOnly |
| R-8.1.1.2/10: | ptime | FIXTHIS | Yes | FIXTHIS | FIXTHIS | FIXTHIS | FIXTHIS | FIXTHIS |
|  |  |  | No |  |  | n/a | FIXTHIS | FIXTHIS |
| R-8.1.1.2/11: | channels | channels_Local; number of channels to be expected in Rx direction; | Yes | channels_Remote == channels_Local; number of channels to be used in Tx direction; | channels_Local | channels_Remote == channels_Local | channels_Local = channels_Remote | channels_Remote |
|  |  |  | No | n/a |  | n/a | channels_Local | n/a "SEP" is ReceiveOnly |
| R-8.1.1.2/12: | max-red | max-red_Local; redundancy scheme to be expected in Rx direction | Yes | max-red_Remote; redundancy scheme to be used in Tx direction | max-red_Local; max-red should always be added to Local reply even if it was not present in Local request | max-red_Remote | n/a | max-red_Remote |
|  |  |  | No | n/a |  | n/a | n/a | n/a "SEP" is ReceiveOnly |

Recommendations for "AMR(-WB)":

- there are 12 SDP parameters for "AMR(-WB)", and 6-out-of-12 parameters need special consideration concerning alignment between both traffic directions;

- the intworking between an AMR-WB and AMR-NB media endpoint should lead to an optimal AMR-NB setting in the MG (because the AMR-WB configuration is expected to be down-negotiated to AMR-NB because an "AMR-WB to AMR-NB" configuration would result in an audio transcoding IWF; and any WB-to-NB transcoding looks questionable from end-to-end QoS perspective because there would be just "NB type of QoE" for all users)

### 8.1.2	Parameterization of audio codec "OPUS"

See clause 2.1/[b-IETF RFC 6716]:

The Opus codec includes a number of control parameters that can be changed dynamically during regular operation of the codec, without interrupting the audio stream from the encoder to the decoder. These parameters only affect the encoder since any impact they have on the bitstream is signaled in-band such that a decoder can decode any Opus stream without any out-of-band signaling. Any Opus implementation can add or modify these control parameters without affecting interoperability. The most important encoder control parameters in the reference encoder are listed below.

[b-IETF RFC 7587] defines the formal codec parameters (in clause 6.1) and their mapping on SDP elements (in clause 7). Figure 8 illustrates a non-exhaustive list of typical use cases with OPUS, by limiting interworking aspects on codec types, sample rates and mono/stereo characteristics only:

Figure 8 – Example use cases with "OPUS"

Use cases with OPUS "medium-band", "super-wideband" and "fullband" are not illustrated in Figure 8. Use case #5 might be of interest because OPUS media translation to other audio codecs with same support of "band types" could lead to traffic-direction specific media translation configurations because both traffic directions are independent of each other in an "OPUS endpoint". Thus, there might be similar cases for the MG-embedded (media)-IWF, i.e., fully decoupled H.248 media flow directions within the concerned H.248 stream.

In contrary, there might be audio signal processing functions enabled on top of the pure media transcoding which could lead to a coupling of both directions (such as acoustic echo control, automatic gain control, etc in packet-to-packet configurations). Such a coupling degree should lead to a "common denominator" configuration of the (media)-IWF, e.g., both traffic directions should be negotiated (under responsibility of the SIP B2BUA) to NB-to-NB transcoding in order to simplify audio signal processing.

Table 8.x1 provides an inventory of the codec parameters and their significance in general from H.248 perspective.

Table 8.x2 provides general H.248 procedural considerations by evaluating the combination of settings in Local and Remote descriptors, in MGC-to-MG direction and vice versa, as well as impact on the resource configuration status in the MG: particular focus are the semantics of MG Local and Remote OPUS parameters in case of matching Local and Remote OPUS based media formats.

Table 8.x2 is complementary to similar work done by 3GPP: see [b-ETSI TS 123 334], clause 5.13.4 "Handling of the OPUS speech and audio codec for WebRTC". The 3GPP work is complementary because

- focusing primarily on the call control signaling to gateway control signalling mapping part (i.e., mapping of SIP/SDP to H.248/SDP), which is inherently out of scope of ITU-T Recommendations on H.248; and

- limited to WebRTC service in context of 3GPP (IMS) network solutions.

{Editor's note (2015-10): the table content was only started to inserted, but intentionally not yet completed due to potential simplifications, see also the example "Codec parameter value assignment model"in  parallel C-0866}

Table 8.x1 – SDP parameters for "audio/OPUS" [IETF RFC 6716]

| Index | SDP parameter | M/O | Value | Description | Call control: SDP O/A | H.248: supported by MG | Coupling of traffic directions? |
| --- | --- | --- | --- | --- | --- | --- | --- |
| R-8.1.2.1/1: | rate | M | not present | FIXTHIS | FIXTHIS | FIXTHIS | FIXTHIS |
|  |  |  | … | FIXTHIS |  | FIXTHIS |  |
|  |  |  | … | FIXTHIS |  |  |  |
| R-8.1.2.1/2: | maxplaybackrate | O | not present | FIXTHIS | FIXTHIS | FIXTHIS | FIXTHIS |
|  |  |  | … | FIXTHIS |  | FIXTHIS |  |
| R-8.1.2.1/3: | sprop-maxcapturerate | O | not present | FIXTHIS | FIXTHIS | FIXTHIS | FIXTHIS |
|  |  |  | 0 | FIXTHIS |  | FIXTHIS |  |
|  |  |  | 1 | FIXTHIS |  |  |  |
| R-8.1.2.1/4: | maxptime | O | not present | any number of frames may be encapsulated into one RTP packet | FIXTHIS | FIXTHIS | No |
|  |  |  | positive time in milliseconds | maximum amount of media encapsulated in one packet in milliseconds |  | FIXTHIS |  |
| R-8.1.2.1/5: | ptime | O | not present | no recommendation regarding length of time represented by the media in one packet | FIXTHIS | Yes (by default) | No |
|  |  |  | time in milliseconds | recommended length of time in milliseconds represented by media in one packet |  | Yes (profile) |  |
| R-8.1.2.1/6: | minptime | O | not present | FIXTHIS | FIXTHIS | FIXTHIS | No |
|  |  |  | positive time in milliseconds | FIXTHIS |  | FIXTHIS |  |
| R-8.1.2.1/7: | maxaveragebitrate | O | not present | FIXTHIS | FIXTHIS | FIXTHIS | FIXTHIS |
|  |  |  | 0 | FIXTHIS |  | FIXTHIS |  |
|  |  |  | 1 | FIXTHIS |  | FIXTHIS |  |
| R-8.1.2.1/8: | stereo | O | not present | FIXTHIS | FIXTHIS | FIXTHIS | FIXTHIS |
|  |  |  | 0 | mono |  | FIXTHIS |  |
|  |  |  | 1 | stereo |  | FIXTHIS |  |
| R-8.1.2.1/9: | sprop-stereo | O | not present | FIXTHIS | FIXTHIS | FIXTHIS | FIXTHIS |
|  |  |  | 0 | mono |  | FIXTHIS |  |
|  |  |  | 1 | stereo |  | FIXTHIS |  |
| R-8.1.2.1/10: | cbr | O | not present | FIXTHIS | FIXTHIS | FIXTHIS | FIXTHIS |
|  |  |  | 0 | constant bitrate) |  | FIXTHIS |  |
|  |  |  | 1 | variable bitrate |  | FIXTHIS |  |
| R-8.1.2.1/11: | useinbandfec | O | not present | FIXTHIS | FIXTHIS | FIXTHIS | FIXTHIS |
|  |  |  | 0 | no FEC |  | FIXTHIS |  |
|  |  |  | 1 | FEC |  | FIXTHIS |  |
| R-8.1.2.1/12: | usedtx | O | not present | FIXTHIS | FIXTHIS | FIXTHIS | FIXTHIS |
|  |  |  | 0 | no DTX |  | FIXTHIS |  |
|  |  |  | 1 | DTX |  | FIXTHIS |  |

Table 8.x2 – SDP parameters for "audio/OPUS" - MG Local and Remote OPUS parameter semantic

| Index | SDP "OPUS" parameter | MGC Request (Add/Modify/Move) |  |  | MG H.248 Reply |  | MG resource configuration |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Local Descript. | Remote Descriptor |  | Local Descript. | Remote Descr. | per traffic direction |  |
|  |  | Value (NOTE 1) | Present Yes/No | Value | Value | Value | Ingress | Egress |
| R-8.1.2.2/1: | rate | rate_Rx (RTP clock rate in Rx direction) | Yes | rate_Tx (RTP clock rate in Tx direction) | rate_Rx | rate_Tx | actual rate to be derived from RTP packet received | rate_Tx (tbc) |
|  |  |  | No | n/a |  | n/a | ditto | no RTP packet sent |
| R-8.1.2.2/2: | maxplaybackrate | Traffic-direction independent codec parameter, hence LD and RD settings are decoupled. |  |  | Reply settings in LD and RD are independent of each other (NOTE 1). |  | LD value | RD value (if already available) |
| R-8.1.2.2/3: | sprop-maxcapturerate | Traffic-direction independent codec parameter, hence LD and RD settings are decoupled. |  |  | Reply settings in LD and RD are independent of each other (NOTE 2). |  | LD value | RD value (if already available) |
| R-8.1.2.2/4: | maxptime | maxptime_Rx | Yes | maxptime_Tx | maxptime_Rx | Yes | FIXTHIS | FIXTHIS |
|  |  |  | No | n/a |  | No | FIXTHIS | FIXTHIS |
| R-8.1.2.2/5: | ptime | ptime_Rx | Yes | ptime_Tx | ptime_Rx | Yes | FIXTHIS | FIXTHIS |
|  |  |  | No | n/a |  | No | FIXTHIS | FIXTHIS |
| R-8.1.2.2/6: | maxaveragebitrate |  |  |  |  |  |  |  |
| R-8.1.2.2/7: | stereo |  |  |  |  |  |  |  |
| R-8.1.2.2/8: | sprop-stereo |  |  |  |  |  |  |  |
| R-8.1.2.2/9: | cbr |  |  |  |  |  |  |  |
| R-8.1.2.2/10: | useinbandfec |  |  |  |  |  |  |  |
| R-8.1.2.2/11: | usedtx |  |  |  |  |  |  |  |
| NOTE 1 – There is independence in general, however, the MG might decide for symmetrical settings for media encoder and media decoder. The LD value would be then aligned to the RD value due to the media decoder specific parameter, i.e., the local receiver follows the remote receiver settings. (to be confirmed). NOTE 2 – There is independence in general, however, the MG might decide for symmetrical settings for media encoder and media decoder. The LD value would be then aligned to the RD value due to the media encoder specific parameter, i.e., the local sender follows the remote sender settings. (to be confirmed). |  |  |  |  |  |  |  |  |

{Editor's note (2015-10): there are a couple conditions which impact the LD/RD handling semantics:

- Media translation mode: a) media transcoding (OPUS-to-non-OPUS) and b) media modification (OPUS-to-OPUS*)

- Harmonization and alignment of codec configurations (for each half call) already as much as possible on call control level (i.e., by SIP B2BUA) or not?

- H.28 wildcarding of codec parameter values in LD or not?

- (Media)-IWF: independent settings on each H.248 SEP / termination or aiming for alignments (i.e., common denominators)?

- Traffic directions: allowing asymmetry for direction-independent parameters or aiming for symmetry?

Above conditions result in a kind of strategical behaviour and impact primarily the "MG H.248 Reply" columns in above table.

Furthermore: the set of parameters may be divided in two subsets: some which are uncontroversial and some which need specific considerations. The final table may be then simplified …}

## 8.2	Configuration profiles for video codecs

FIXTHIS

## 8.3	Configuration profiles for text codecs

FIXTHIS

# 9	H.248 profile guidelines

This clause provides guidelines for H.248 profile specifications. The structure follows the profile template according to Appendix III/[ITU-T H.248.1].

The template elements that are not applicable in this Recommendation are indicated by "Subject to profile specification".

Any profile guidelines are primarily dependent on the concerned network configuration and use case. The guidelines in this clause are therefore conditional.

## 9.1	Profile Identification

Subject to profile specification.

## 9.2	Summary

Subject to profile specification.

## 9.3	Gateway Control Protocol Version

Subject to profile specification.

## 9.4	Connection Model

Subject to profile specification.

## 9.5	Context Attributes

Subject to profile specification.

## 9.6	Terminations

Subject to profile specification.

## 9.7	Descriptors

### 9.7.1	TerminationState Descriptor

Subject to profile specification.

### 9.7.2	Stream Descriptor

Subject to profile specification.

### 9.7.3 to 9.7.10	Descriptor types

Subject to profile specification.

### 9.7.11	Error Descriptor

Subject to profile specification.

## 9.8	Command API

Subject to profile specification.

## 9.9	Generic command syntax and encoding

The profile guidelines suppose text-encoding mode due to the assumption of SDP based codec parameterization.

## 9.10	Transactions

Subject to profile specification.

## 9.11	Messages

Subject to profile specification.

## 9.12	Transport

Subject to profile specification.

## 9.13	Security

Subject to profile specification.

## 9.14	Packages

Subject to profile specification.

## 9.15	Mandatory support of SDP and ITU-T H.248.1 Annex C information elements

At least the following:

| Supported Annex C and SDP information elements: |  |  |
| --- | --- | --- |
| Information Element | Annex C Support | SDP Support |
| "m="-line <proto> |  | FIXTHIS |
|  |  | FIXTHIS |

Additional tables, related to supported SDP elements concerning the parameterization of media codecs, are recommended. The structure and content of such table is media codec specific and H.248 profile dependent. Clause 8 provides examples for such tables.

## 9.16	Optional support of SDP and ITU-T H.248.1 Annex C information elements

At least the following:

| Supported Annex C and SDP information elements: |  |  |
| --- | --- | --- |
| Information Element | Annex C Support | SDP Support |
| FIXTHIS |  |  |
| FIXTHIS |  |  |

Rest: subject to profile specification.

## 9.17	Procedures

FIXTHIS

Appendix I

Negotiation of codec configurations – Examples

(This appendix does not form an integral part of this Recommendation)

## I.1	Example signalling flow for an outgoing call

Figure I.1 illustrates a typical signalling variant for an outgoing call:

Figure I.1 – Example signalling flow for an outgoing call

There might be a lot of variations, such as

- multiple SDP Offer/Answer cycles at SIP level (instead of just one in Figure I.1);

- MGC might be aware of all supported codec configurations by the MG or not; such information could be then stored in the MGC associated data base of media (codec) policies, which would simplify H.248 resource configuration requests;

- the outgoing SDP Offer towards Y (5) might be already issued in parallel to the first H.248 ADD.req (3), of course then lacking network address information, which would be provided  in a subsequent cycle;

- the call/session control server could follow different strategies, e.g., expect per default

- a media translation mode variant in the MG, i.e., early requesting a media-IWF resource; or

- a media transparent mode, i.e., not requesting early such a resource, rather awaiting the final results of the call level negotiation process;

- MG could just enforce the received codec configurations for the two media endpoints T1 and T2, or could evaluate and align in a smart manner both codec configurations in order to optimize the end-to-end usage.

Bibliography

[b-ETSI TR 183 046]	ETSI TR 183 046 V3.3.1 (08/2009), Telecommunications and Internet converged Services and Protocols for Advanced Networking (TISPAN); SDP Interworking between Call/Session Control Protocols (SIP/SDP, RTSP/SDP; etc.) and the Gateway Control Protocol (H.248/SDP).

[b-ETSI TS 123 334]	ETSI TS 123 334 V12.8.0 (07/2015), Digital cellular telecommunications system (Phase 2+); Universal Mobile Telecommunications System (UMTS); LTE; IP Multimedia Subsystem (IMS) Application Level Gateway (IMS-ALG) - IMS Access Gateway (IMS-AGW) interface: Procedures descriptions (3GPP TS 23.334 version 12.8.0 Release 12)

[b-IETF RFC 6716]	IETF RFC 6716 (2012), Definition of the Opus Audio Codec.

[b-IETF RFC 7587]	IETF RFC 7587 (06/2015), RTP Payload Format for Opus Speech and Audio Codec.

_________________