| INTERNATIONAL TELECOMMUNICATION UNION |  | STUDY GROUP 16 |
| --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  | TD 318 (WP 1/16) |
|  |  | English only Original: English |
| Question(s): | 3/16 | Geneva, 12 - 23 October 2015 |
| TD |  |  |
| Source: | Editor H.Sup.ALTC |  |
| Title: | H.Sup.ALTC "Gateway Control Protocol: Alternate IP connectivity support by H.248 gateway control protocol" (New): Output draft Ed. 0.6 |  |

1.	Introduction

This document contains the editor's output draft ed. 0.6 for draft new H.Sup.ALTC "Gateway Control Protocol: Alternate IP connectivity support by H.248 gateway control protocol" (New).

| Document History |  |
| --- | --- |
| Issue | Notes |
| H.Sup.ALTC Ed. 0.1 | Initial draft (AVD-4655) to WP 1/16 Seoul (KR) meeting (2014-11) Location: http://wftp3.itu.int/av-arch/avc-site/2013-2016/1411_Seo/AVD-4655.zip |
| H.Sup.ALTC Ed. 0.2 | Output (TD-23) from WP 1/16 Seoul (KR) meeting (2014-11) Location: http://wftp3.itu.int/av-arch/avc-site/2013-2016/1411_Seo/TD-23.zip |
| H.Sup.ALTC Ed. 0.3 | Input (TD 221/WP1) to ITU-T SG16 Geneva meeting, Feb 2015 Location: http://www.itu.int/md/T13-SG16-150209-TD-WP1-0221/en |
| H.Sup.ALTC Ed. 0.4 | Output (TD 221R1/WP1) from ITU-T SG16 Geneva meeting, Feb 2015 Location: http://www.itu.int/md/T13-SG16-150209-TD-WP1-0221/en |
| H.Sup.ALTC Ed. 0.5 | Input (AVD-4670) to WP 1/16 Chengdu meeting (2015-06) Location: http://wftp3.itu.int/av-arch/avc-site/2013-2016/1506_Che/AVD-4670.zip |
| H.Sup.ALTC Ed. 0.6 | Output (TD 318/WP1) from ITU-T SG16 Geneva meeting, Oct 2015 Location: http://www.itu.int/md/T13-SG16-151012-TD-WP1-0318/en |
| H.Sup.ALTC Ed. 0.7 |  |
| H.Sup.ALTC Ed. 0.8 |  |
| H.Sup.ALTC Ed. 0.9 |  |
| H.Sup.ALTC Ed. 0.10 |  |

CONTENTS

Page

1 Scope 9 2 References 9 3 Definitions 10 3.1 Terms defined elsewhere 10 3.2 Terms defined in this Supplement 10 4 Abbreviations and acronyms 11 5 Conventions 11 6 Network model 12 6.1 Use cases 12 6.2 Network model with ALTC-enabled H.248 gateway 12 6.2.1 Interworking model for H.248 border gateways (IP-to-IP) 12 6.2.2 Interworking model for H.248 trunking gateways (TDM-to-IP) 13 7 Support of alternate IP connectivity: principle options from H.248 side 13 7.1 Solution options: basic H.248 Context models and H.248 Media Descriptor elements 13 7.1.1 H.248 ALTC Context model "M1": reservation of two H.248 Contexts 13 7.1.2 H.248 ALTC Context model "M2": reservation of two H.248 Terminations 14 7.1.3 H.248 ALTC Context model "M3": reservation of two H.248 Stream endpoints 16 7.1.4 H.248 ALTC Context model "M4": reservation of two H.248 Media Groups 17 7.1.5 H.248 ALTC Context model "M5": reservation of two H.248 Stream Groups 17 7.1.6 H.248 ALTC Context model "M6": reservation of a "dual-stack IP interface" 18 7.2 Solution options: complementary H.248 resource usage indication information 18 7.2.1 H.248 signalling element "S1": SDP-based ALTC call type indication 19 7.2.2 H.248 signalling element "S2": Termination connection model package (H.248.51) 19 7.2.3 H.248 signalling element "S3": usage of media mixer resources (H.248.9) 19 7.2.4 H.248 signalling element "S4": Resource management package(s) (H.248.63) 19 7.2.5 H.248 signalling element "S5": SDP Offer/Answer model (H.248.80) 19 8 Requirements (from H.248 profile perspective) 20 8.1 Communication service: number of media components 20 8.2 Communication service: early end-to-end IP connectivity 20 8.3 Call control signaling: number of negotiation cycles 20 8.4 Gateway control signaling: resource management process in H.248 MG 21 8.5 Gateway control signaling: explicit ALTC "call" tagging 21 8.6 Gateway control signaling: specific resource components in H.248 MG 21 9 Discussion 21 9.1 Call control signalling: examples of SIP SDP offers 22 9.1.1 SIP SDP offer example 1: monomedia audio telephony 22 9.1.2 SIP SDP offer example 2: multimedia WebRTC call 22 9.2 Gateway control signalling: examples 23 9.2.1 Example H.248 signalling in case of "application aware IWF" 23 9.2.2 Example H.248 signalling in case of "application agnostic IWF" 53 10 Profile guidelines 79 10.1 Profile Identification 80 10.2 Summary 80 10.3 Gateway Control Protocol Version 80 10.4 Connection Model 80 10.5 Context Attributes 80 10.6 Terminations 80 10.7 Descriptors 80 10.7.1 TerminationState Descriptor 80 10.7.2 Stream Descriptor 80 10.7.3 Events Descriptor 80 10.7.4 EventBuffer Descriptor 81 10.7.5 Signals Descriptor 81 10.7.6 DigitMap Descriptor 81 10.7.7 Statistics Descriptor 81 10.7.8 ObservedEvents Descriptor 81 10.7.9 Topology Descriptor 81 10.7.10 Error Descriptor 81 10.8 Command API 81 10.8.1 Add 81 10.8.2 Modify 81 10.8.3 Subtract 81 10.8.4 Move 81 10.8.5 AuditValue 81 10.8.6 AuditCapabilities 81 10.8.7 Notify 81 10.8.8 ServiceChange 81 10.8.9 Manipulating and auditing ContextAttributes 81 10.9 Generic command syntax and encoding 82 10.10 Transactions 82 10.11 Messages 82 10.12 Transport 82 10.13 Security 82 10.14 Packages 82 10.14.1 Mandatory Package 82 10.14.2 Optional Package 82 10.14.3 Package usage information 82 10.15 Mandatory support of SDP and ITU-T H.248.1 Annex C information elements 84 10.16 Optional support of SDP and ITU-T H.248.1 Annex C information elements 84 10.17 Procedures 84 Appendix I Discussion of the various H.248 solution options – Signalling examples 85 I.1 #### 85 Appendix II Discussion of the various H.248 solution options – Summary 86 II.1 Summary table 86 II.2 Recommended solution(s) 87 Appendix III Example H.248 profile: 3GPP Iq profile 88 III.1 Summary of stage 2 requirements 88 III.2 Resulting stage 3 specification: H.248 Iq profile 88 Appendix IV Use case specific capability sets – Some examples 88 IV.1 Overview 89 Bibliography 90

List of Tables

Page

Table 9.1.1 – Example command encoding – (SIP) SDP Offer example 1 22 Table 9.1.2 – Example command encoding – (SIP) SDP Offer example 2 22 Table 9.2.1.1/1 – Example command encoding – Step 1 – MGC request (for local connection point only in outgoing direction) 24 Table 9.2.1.1/2 – Example command encoding – Step 2 – MG reply 25 Table 9.2.1.1/3a – Example command encoding – Step 3a – MGC request 26 Table 9.2.1.1/4a – Example command encoding – Step 4a – MG reply 27 Table 9.2.1.1/3b – Example command encoding – Step 3b – MGC request 28 Table 9.2.1.1/4b – Example command encoding – Step 4b – MG reply 29 Table 9.2.1.2/1a – Example command encoding – Step 1a – MGC request (for local connection point only in outgoing direction) 30 Table 9.2.1.2/1b – Example command encoding – Step 1b – MG reply 30 Table 9.2.1.2/2a – Example command encoding – Step 2a – MGC request (for local connection point only in outgoing direction) 31 Table 9.2.1.2/2b – Example command encoding – Step 2b – MG reply 32 Table 9.2.1.2/3a – Example command encoding – Step 3a – MGC request 32 Table 9.2.1.2/4a – Example command encoding – Step 4a – MG reply 33 Table 9.2.1.2/3b – Example command encoding – Step 3b – MGC request 34 Table 9.2.1.2/4b – Example command encoding – Step 4b – MG reply 35 Table 9.2.1.3/1 – Example command encoding – Step 1 – MGC request (for local connection point only in outgoing direction) 36 Table 9.2.1.4/1 – Example command encoding – Step 1 – MGC request (for local connection point only in outgoing direction) 37 Table 9.2.1.4/2 – Example command encoding – Step 2 – MG reply 38 Table 9.2.1.4/3a – Example command encoding – Step 3a – MGC request 39 Table 9.2.1.4/4a – Example command encoding – Step 4a – MG reply 40 Table 9.2.1.4/3b – Example command encoding – Step 3b – MGC request 41 Table 9.2.1.4/4b – Example command encoding – Step 4b – MG reply 42 Table 9.2.1.5/1 – Example command encoding – Step 1 TY.S1 – MGC request (for local connection point only in outgoing direction) 43 Table 9.2.1.5/2 – Example command encoding – Step 2 – MG reply 45 Table 9.2.1.5/3a – Example command encoding – Step 3a – MGC request 46 Table 9.2.1.5/4a – Example command encoding – Step 4a – MG reply 48 Table 9.2.1.5/3b – Example command encoding – Step 3b – MGC request 49 Table 9.2.1.5/4b – Example command encoding – Step 4b – MG reply 51 Table 9.2.2.1/1 – Example command encoding – Step 1 – MGC request 53 Table 9.2.2.1/2 – Example command encoding – Step 2 – MG reply 54 Table 9.2.2.1/3a – Example command encoding – Step 3a – MGC request 55 Table 9.2.2.1/4a – Example command encoding – Step 4a – MG reply 56 Table 9.2.2.1/3b – Example command encoding – Step 3b – MGC request 57 Table 9.2.2.1/4b – Example command encoding – Step 4b – MG reply 58 Table 9.2.2.2/1a – Example command encoding – Step 1a – MGC request (for local connection point only in outgoing direction) 59 Table 9.2.2.2/1b – Example command encoding – Step 1b – MG reply 59 Table 9.2.2.2/2a – Example command encoding – Step 2a – MGC request (for local connection point only in outgoing direction) 60 Table 9.2.2.2/2b – Example command encoding – Step 2b – MG reply 61 Table 9.2.2.2/3a – Example command encoding – Step 3a – MGC request 61 Table 9.2.2.2/4a – Example command encoding – Step 4a – MG reply 62 Table 9.2.2.2/3b – Example command encoding – Step 3b – MGC request 63 Table 9.2.2.2/4b – Example command encoding – Step 4b – MG reply 63 Table 9.2.2.3/1 – Example command encoding – Step 1 – MGC request 64 Table 9.2.2.3/2 – Example command encoding – Step 2 – MG reply 64 Table 9.2.2.3/3 – Example command encoding – Step 3 – MGC request 65 Table 9.2.2.3/4 – Example command encoding – Step 4 – MG reply 65 Table 9.2.2.4/1 – Example command encoding – Step 1 – MGC request 65 Table 9.2.2.4/2 – Example command encoding – Step 2 – MG reply 66 Table 9.2.2.4/3a – Example command encoding – Step 3a – MGC request 67 Table 9.2.2.4/4a – Example command encoding – Step 4a – MG reply 68 Table 9.2.2.4/3b – Example command encoding – Step 3b – MGC request 68 Table 9.2.2.4/4b – Example command encoding – Step 4b – MG reply 69 Table 9.2.2.5/1 – Example command encoding – Step 1 – MGC request (for local connection point only in outgoing direction) 70 Table 9.2.1.5/2 – Example command encoding – Step 2 – MG reply 72 Table 9.2.2.5/3a – Example command encoding – Step 3a – MGC request 73 Table 9.2.2.5/4a – Example command encoding – Step 4a – MG reply 75 Table 9.2.2.5/3b – Example command encoding – Step 3b – MGC request 76 Table 9.2.1.5/4b – Example command encoding – Step 4b – MG reply 78 Table II.1 – Evaluation summary 86 Table IV.1 – Examples of use case specific capability sets (CS) 89

List of Figures

Page

Figure 1 – Network model for alternate IP connectivity from H.248 perspective 12 Figure 2 – H.248 ALTC Context model "M1": reservation of two H.248 Contexts 14 Figure 3 – H.248 ALTC Context model "M2a": reservation of two H.248 Terminations 15 Figure 4 – H.248 ALTC Context model "M2b": reservation of two H.248 Terminations 15 Figure 5 – H.248 ALTC Context model "M3": reservation of two H.248 Stream endpoints 16 Figure 6 – H.248 ALTC Context model "M4": reservation of two H.248 Media Groups 17 Figure 7 – Usage of "Stream grouping" capabilities according [ITU-T H.248.96] 17 Figure 8 – H.248 ALTC Context model "M5": reservation of two H.248 Stream Groups 18 Figure 9 – H.248 ALTC Context model "M6": reservation of dual-stack capable SEPs 18

Draft New Supplement # to the ITU-T H-Series (ex H.Sup.ALTC)

Gateway control protocol: 
Alternate IP connectivity support by H.248 entities

Summary

<Mandatory material>

Keywords

ALTC, ANAT

Introduction

The alternate IP connectivity capability results from transitioning scenarios in evolving IP version 4 networks towards IP version 6. The reality of IPv4-IPv6 coexistence introduces heterogeneous scenarios with combinations of IPv4 and IPv6 nodes, some of which are capable of supporting both IPv4 and IPv6 dual-stack (DS) and some of which are capable of supporting only IPv4 or only IPv6. In this context, H.248 entities could be involved in alternate IP connectivity scenarios, at least when the H.248 media gateway support dual-stack (DS) capable IP interfaces.

{Editor's note (2014-11): the main body (clause 7) of the Supplement should finally document the "recommended solution" …}

# 1	Scope

The capability of alternate IP connectivity (ALTC) is basically supported by H.248 entities, without any need to extend the existing H.248 protocol capabilities. However, there are in principle multiple solutions possible how a particular H.248 profile could define ALTC support.

The purpose of this Supplement is to provide guidance on this topic. The Supplement is structured as follows:

- network model for ALTC with focus on H.248 entities (clause 6);

- discussion of principle ALTC solution options from H.248 side (clause 7);

- typical H.248 signalling requirements related to ALTC (clause 8), i.e., the so-called stage 2 information for H.248 profiles;

- evaluation and discussion of signalling examples in order to challenge H.248 ALTC solutions from clause 7 (clause 9); and

- H.248 profile guidelines, i.e., stage 3 recommendations (clause 10).

The preferred solutions are dependent on recently developed H.248 protocol extensions ([ITU-T H.248.41], [ITU-T H.248.96]).

# 2	References

[ITU-T H.248.1]	Recommendation ITU-T H.248.1 (03/2013), Gateway control protocol: Version 3.

[ITU-T H.248.9]	Recommendation ITU-T H.248.9 (12/2009), Gateway control protocol: Advanced media server packages.

[ITU-T H.248.39]	Recommendation ITU-T H.248.39 (10/2014), Gateway control protocol: H.248 SDP parameter identification and wildcarding.

[ITU-T H.248.41]	Recommendation ITU-T H.248.41 (10/2015), Gateway control protocol: IP domain connection package.

[ITU-T H.248.51]	Recommendation ITU-T H.248.51 (08/2007), Gateway control protocol: Termination connection model package.

[ITU-T H.248.63]	Recommendation ITU-T H.248.63 (03/2009), Gateway control protocol: Resource management packages.

[ITU-T H.248.80]	Recommendation ITU-T H.248.80 (01/2014), Gateway control protocol: Usage of the revised SDP offer/answer model with ITU-T H.248.

[ITU-T H.248.96]	Recommendation ITU-T H.248.96 (10/2015), Gateway control protocol: H.248 Stream grouping and aggregation.

[IETF RFC 6947]	IETF RFC 6947 (05/2013), The Session Description Protocol (SDP) Alternate Connectivity (ALTC) Attribute.

[ETSI TS 183 018]	ETSI TS 183 018 V3.5.2 (01/2010), Telecommunications and Internet converged Services and Protocols for Advanced Networking (TISPAN); Resource and Admission Control: H.248 Profile Version 3 for controlling Border Gateway Functions (BGF) in the Resource and Admission Control Subsystem (RACS); Protocol specification.

[ETSI TR 183 068]	ETSI TR 183 068 V3.1.1 (08/2009), Telecommunications and Internet converged Services and Protocols for Advanced Networks (TISPAN); Guidelines on using Ia H.248 profile for control of Border Gateway Functions (BGF); Border Gateway Guidelines.

# 3	Definitions

## 3.1	Terms defined elsewhere

3.1.1	Data Path Border Element (DBE, [IETF RFC 6947]): denotes a functional element, located at the boundaries of an IP telephony administrative domain, that is responsible for intercepting media/data flows received from UAs and relaying them to another DBE (or media servers, e.g., an announcement server or interactive voice response).

NOTE 1 – An example of a DBE is a media gateway that intercepts RTP flows.

NOTE 2 – The DBE function is subject of the ITU-T H.248 media gateway entity in context of this Supplement.

3.2.1	Signalling Path Border Element (SBE, [IETF RFC 6947]): denotes a functional element, located at the boundaries of an IP telephony administrative domain, that is responsible for intercepting signaling flows received from UAs and relaying them to the core service platform.  An SBE may be located at the access segment (i.e., be the service contact point for UAs), or be located at the interconnection with adjacent domains.  An SBE controls one or more DBEs.  The SBE and DBE may be located in the same device (e.g., the session border controller (SBC) [b-IETF RFC 5853]) or be separated.

NOTE 1 – The SBE function is subject of the ITU-T H.248 media gateway controller entity in context of this Supplement. Thus, only decomposed SBCs according the H.248 gateway architecture are in scope here.

## 3.2	Terms defined in this Supplement

3.2.1	Alternate (IP) Connectivity: mechanism to request initially resources (e.g., from a terminal or gateway) for multiple IP addresses of different address families (e.g., IPv4 and IPv6), and release again immediately after successful connectivity establishment the unused resources.

3.2.2	Early media: unidirectional, end-to-end user plane connectivity from called connection endpoint back to calling connection endpoint, which allows e.g., the calling party to receive application data (i.e., "backward early media") already before the final call establishment phase is completed inclusive a successfully established bidirectional end-to-end user plane connection.

NOTE 1 – From H.248 media gateway perspective the capability of "early media" support implies the Context-internal cut-through of an H.248 media stream. Unidirectional support is achieved by StreamMode settings of "SendOnly" and "RecvOnly" at the concerned H.248 SEPP (or SEPT in general).

NOTE 2 – The notion of "early media" originates from SIP-based call control signalling, see [b-IETF RFC 5009]. The RFC describes the use cases of "backward early media" and "forward early media".

# 4	Abbreviations and acronyms

This Supplement uses the following abbreviations and acronyms:

| ALTC | Alternate (IP) Connectivity |
| --- | --- |
| B2BIH | Back-to-Back IP Host (mode according to [ITU-T H.248.64])] |
| B2BUA | Back-to-Back User Agent |
| DBE | Data Path Border Element |
| DS | Dual-Stack (parallel support of V4 and V6) |
| GCP | Gateway Control Protocol |
| IWF | Interworking Function |
| MG | Media Gateway |
| MGC | Media Gateway Controller |
| NA(P)T-PT | Network Address (and Port) Translation plus (Internet) Protocol (Version) Translation |
| SBC | Session Border Controller |
| SBE | Signaling Path Border Element |
| SEP | Stream Endpoint |
| SEPP | Stream Endpoint Pair |
| SEPT | Stream Endpoint Tuple |
| SIP | Session Initiation Protocol |
| TDM | Time Division Multiplexing |
| UA | (SIP) User Agent |
| UE | User Equipment |
| V4 | Internet Protocol version 4 |
| V6 | Internet Protocol version 6 |

# 5	Conventions

None.

# 6	Network model

## 6.1	Use cases

[IETF RFC 6947], Appendix A provides a set of example ALTC use cases:

- clause A.3.2. "Basic scenario for IPv6 SIP service delivery", i.e., an transitioning scenario from IPv4 to IPv6 migration;

- clause A.3.3.  "Avoiding IPv4/IPv6 interworking";

- clause A.3.4. "Data path element bypass procedure"; and

- clause A.3.5. "Direct communications between IPv6-enabled user agents".

The RFC notion of data path border element (DBE, clause 3.1.1) and signaling path border element (SBE, clause 3.1.2) correlates directly with H.248 entities MG and MGC. All use cases indicated by Appendix A/[IETF RFC 6947] could be satisfied by H.248 gateway based solutions.

## 6.2	Network model with ALTC-enabled H.248 gateway

## 6.2.1	Interworking model for H.248 border gateways (IP-to-IP)

There will be two interconnected IP network domains (labelled as X and Y in Figure 1) in case of ALTC support by H.248 gateways.

Figure 1 – Network model for alternate IP connectivity from H.248 perspective

It is sufficient to consider an end-to-end network configuration where at least one domain supports IPv4 and IPv6 connectivity and dualstack capable IP host entities (here domain Y). The SIP B2BUA entity receives a SDP offer with SDP "a=altc" qualified media descriptions from X or Y, dependent on the call establishment direction. The SIP level SDP offer/answer procedures are subject of [IETF RFC 6947] and out of scope of this Recommendation.

The H.248 IP-to-IP media gateway is most likely configured for a B2BIH mode, especially due to potential IP version translation in case of a final V4-to-V6 configuration. This is also known as NAT46 (of NAT64) mode. IP version translation is only possible in B2BIH mode (and not in IPR mode).

Therefore the assumption of B2BIH mode, despite the fact of scenarios with final V4-only or V6-only end-to-end IP connectivity.

### 6.2.2	Interworking model for H.248 trunking gateways (TDM-to-IP)

Consider TDM as the bearer technology for network domain X in Figure 1. The H.248 media gateway function becomes of type "trunking" (or "TDM access" or "TDM residential"). The call control level ALTC scenario in X-to-Y call establishment direction would still be valid in IP network domain Y. There are the following primary differences:

- Termination T2 provides a full IP host function (hence, complete protocol stack termination) due to the TDM-to-IP connection model.

- The MG resource "interworking function" between T1 and T2 is specified in a "fully media aware" manner, i.e., it is obvious already in the very first H.248 transaction cycle that e.g. a "media transcoder" resource (or whatever else) would be required.

Thus, ALTC in the context of TDM-to-IP interworking does not contribute anything special in comparison to above IP-to-IP interworking.

# 7	Support of alternate IP connectivity: principle options from H.248 side

The SDP based ALTC qualification of media descriptions is firstly only required at the SIP-based call control level. H.248 gateway control signalling for ALTC support does not necessarily require such kind of SDP tagging due to alternative solutions concerning media plane IP resource reservations.

## 7.1	Solution options: basic H.248 Context models and H.248 Media Descriptor elements

The illustrations take the example of a dual media call with audio and video over RTP, inclusive of RTCP control flows.

### 7.1.1	H.248 ALTC Context model "M1": reservation of two H.248 Contexts

Figure 2 illustrates such a model. Termination TX serves the bearer traffic towards UE_X and is out of consideration in this Supplement. The bearer traffic for the preferred IP connection is assigned to Termination TY,P. Both Terminations (TX and TY,P) are created in Context Cz1. The alternate IP connection is assigned to Termination TY,A, which is created in a separate Context Cz2.

Figure 2 – H.248 ALTC Context model "M1": reservation of two H.248 Contexts

When the final IP connection option is negotiated, when either

- Termination TY,P is deleted again (via H.248 SUBtract command) and replaced by Termination TY,A (via H.248 MOVe command); or

- Termination TY,A is deleted (via H.248 SUBtract command).

The 2nd Context Cz2 disappears again in both cases.

The motivation for the "two context" model (M1) might be the isolation of resources associated with the alternate IP connection, given the assumption of an estimated low probability that these resources will be finally used.

NOTE – The final decision between the two options is usually not "50:50" from MGC perspective (due to certain a-priori knowledge about the likely selected IP connection, e.g., due to operator policies, registration information from UE side, etc.). An H.248 profile might benefit from such information by trying to optimize resource usage: e.g., both Terminations TY,P and TY,A are prepared not adequately: T Y,P is configured as usual (with much as needed information) whereas TY,A is created with minimum information only.

Model (M1) looks also fairly attractive from H.248 control perspective because Context Cz1 represents the usual Context configuration, without any ALTC specific aspects.

### 7.1.2	H.248 ALTC Context model "M2": reservation of two H.248 Terminations

The alternate IP connection is assigned to a self-contained Termination TY,A in model M2, similar as in previous model M1. However, there is only a single Context from the very beginning, see Figure 3:

Figure 3 – H.248 ALTC Context model "M2a": reservation of two H.248 Terminations

It is obvious that there are some Context-internal resource management aspects during the temporary phase when both Terminations TY,P and TY,A exists:

- the IWF resources for media processing (here: audio transcoding IWF and video transparent forwarding IWF) might already be prepared and internally connected to the external IP interfaces …

- … raising the question of the internal H.248 Stream topology (e.g., topologies [TX(S1), TY,P(S1), oneway] plus [TX(S1), TY,A(S1), oneway]).

Hence, there might be the option of shared resources (e.g., all three SEPs S1 are connected to the same audio transcoder entity), or separate resources (i.e., for "preferred" and "alternate" SEPs S1) are prepared. This is a remarkable aspect of model M2a, but is implementation specific, hence out of scope of this Supplement.

The purpose of model M2a is to serve the possible requirement of early media (see R-8.2/2 in clause 8). If such a requirement does not exist in an H.248 profile, then the consideration of a simpler model might be sufficient, see model M2b in Figure 4:

Figure 4 – H.248 ALTC Context model "M2b": reservation of two H.248 Terminations

When the final IP connection option is negotiated, when either

- Termination TY,P is deleted again (via H.248 SUBtract command) or

- Termination TY,A is deleted (via H.248 SUBtract command).

The Context internal cut-through occurs either "as soon as possible" (M2a) or with the final Context configuration (M2b).

Both Context models (M2a and M2b) require the usage of the H.248 Topology Descriptor in order to specify the topology relations between all three Terminations TX, TY,P and TY,A (see also signalling example in clause 9.2.1.2.1, Table 9.2.1.2/2a). It has to be noted that there are side effects: the Termination names must be known in order to specify the topologies, which isn't the case in the first signalling cycles when the local connection endpoints are wildcard. Consequently two signalling cycles may be required.

### 7.1.3	H.248 ALTC Context model "M3": reservation of two H.248 Stream endpoints

NOTE – Model M3 is conditional, limited on communication services related to single-Stream-per-Termination structures!

The example of a monomedia call (here: audio) is considered, resulting in a single H.248 Stream (S1) for the preferred IP configuration (Figure 5). The alternate IP connection resources are defined via a separate H.248 Stream Descriptor (see SEP S2). Both SEPs S1 and S2 belong to the same Termination TY:

Figure 5 – H.248 ALTC Context model "M3": reservation of two H.248 Stream endpoints

When the final IP connection option is negotiated, when either

- SEP TY(S1) is deleted again (via H.248 SUBtract command) or

- SEP TY(S2) is deleted (via H.248 SUBtract command).

The interconnection of SEP TY(S1) with SEP TX(S1) is straightforward, however the interconnection of SEP TY(S2) with SEP TX(S1) is actually impossible (see also clause 9.2.1.3). It requires the introduction of a Stream grouping level (see method "M5" in clause 7.1.5).

Model M3 looks attractive due to the simplicity of the use of single Terminations only. However, the Context model does not provide a general purpose solution, hence is not recommended for real network deployments.

### 7.1.4	H.248 ALTC Context model "M4": reservation of two H.248 Media Groups

Another approach may be through the use of the H.248 LocalControl properties ReserveValue and ReserveGroup, which would allow two IP addresses to be indicated. The semantic of these LocalControl properties is that only one resource is used at a time. The unused one can be removed at a later date. There would be two H.248 Media Groups reserved, see Figure 6:

Figure 6 – H.248 ALTC Context model "M4": reservation of two H.248 Media Groups

Model M4 may result in quite extensive H.248 Stream Descriptors (see clause 9.2.1.4).

### 7.1.5	H.248 ALTC Context model "M5": reservation of two H.248 Stream Groups

Figure 7 illustrates the Context-internal grouping principle of H.248 Streams. There would be one H.248 Stream Group per media type:

Figure 7 – Usage of "Stream grouping" capabilities according [ITU-T H.248.96]

Figure 8 shows the synonym H.248 ALTC Context model "M5" according to the style as used for the previous models M1 to M4.

Figure 8 – H.248 ALTC Context model "M5": reservation of two H.248 Stream Groups

### 7.1.6	H.248 ALTC Context model "M6": reservation of a "dual-stack IP interface"

A single H.248 Stream endpoint is qualified by a single resource "dual-stack IP interface", see Figure 9.

Figure 9 – H.248 ALTC Context model "M6": reservation of dual-stack capable SEPs

Context model "M6" would facilitate H.248 "ALTC" signalling because all ALTC-associated changes would be subject of a "single resource" object. However, H.248 text-endcoding mode does not (set) provide any support ("neither the existing RFC 4566 "c="-line syntax, nor H.248.39 "c="-line wildcarding options"). This method has therefore no relevance for ALTC.

## 7.2	Solution options: complementary H.248 resource usage indication information

The qualification of an H.248 Context (or Stream) for "ALTC awareness" might improve the MG level resource management.

### 7.2.1	H.248 signalling element "S1": SDP-based ALTC call type indication

It could be imagined that the occurrence of SDP attribute "a=altc:" in the H.248 Local and/or Remote Descriptor will be assigned the semantic, that the MG could qualify associated resources of type "ALTC". E.g., resources which are only temporarily reserved, but finally not used and released immediately after call establishment completion.

Status:

If call control signalling uses ALTC tagging, then the MGC might use SDP attribute value"ANAT" for the prupose of Stream grouping (according to [ITU-T H.248.96], see also clause 9.2.1.5). H.248 profiles should not allow the usage of SDP attribute "a=altc:" in Local and Remote descriptors.

### 7.2.2	H.248 signalling element "S2": Termination connection model package (H.248.51)

The termination connection model package [ITU-T H.248.51] deals with Termination connection configurations supported by a MG. H.248 property "Supported Termination Connections" (tcm/stc) is not used for the indication of a particular Termination configuration from MGC towards MG, rather applied for auditing purposes only.

However, the tcm package might be a valuable complementary tool for ALTC, particularly when the method with two Terminations (M2) is used. This method has an inherent aspect in clarifying the terminology between all three Terminations. An H.248 profile with an M2-based ALTC support might benefit from the predefinition of ALTC-related termination connection configurations. The analysis of detailed use cases is for further studies.

### 7.2.3	H.248 signalling element "S3": usage of media mixer resources (H.248.9)

### There are similarities of the H.248 Stream structures within the Context (related to the three remote IP connection endpoints) during the ALTC establishment phase with media server connection models. However, the applied control function represents a "1-out-of-2" selection, resulting in a switching structure of H.248 Streams, but not any mixing of Streams.

Thus, ALTC might be supported by an H.248 media server together with [ITU-T H.248.9] packages, but both capabilities would be then orthogonal without any interaction.

### 7.2.4	H.248 signalling element "S4": Resource management package(s) (H.248.63)

### [ITU-T H.248.63] defines three H.248 packages concerning the enhanced management of call-dependent resources at MG level. Particularly, the resource management configuration package (rmc) might be of interest for ALTC because this package is positioned for the temporary phase of context creation during call establishment. It could be imagined that the resource description property (rmc/rd) might be used by the MGC for the indication of the required H.248 Stream-level interworking functions, such as an audio transcoder, video forwarder, etc.

### Hence, [ITU-T H.248.63] might be used together with ALTC in a complementary manner. Detailed use cases are out of scope of this Supplement.

### 7.2.5	H.248 signalling element "S5": SDP Offer/Answer model (H.248.80)

The SDP-based ALTC signalling method at SIP level is independent of the SDP offer/answer model (see clause 4.2.4/[IETF RFC 6947]), thus, the SDP attribute "a=altc:" may be used in the legacy SDP offer/answer (see clause 3.2.3.1/[ITU-T H.248.80]) and revised SDP offer/answer (see clause 3.2.3.2/[ITU-T H.248.80]) syntax.

# 8	Requirements (from H.248 profile perspective)

There are multiple solutions for ALTC support (see previous clause), but some of them are conditional dependent on typically H.248 profile dependent requirements. The purpose of this clause is to highlight such kind of capabilities.

## 8.1	Communication service: number of media components

This aspect affects normally the number of required H.248 Streams (and therefore also the internal context topology).

R-8.1/1: Support of single media descriptions only per H.248 Context ("monomedia call").

R-8.1/2: Support of multiple media descriptions per H.248 Context ("multimedia call").

## 8.2	Communication service: early end-to-end IP connectivity

The ALTC feature is subject of the call establishment phase. There are options how fast the end-to-end bearer establishment is finalized.

R-8.2/1: End-to-end IP connectivity should be available with completion of the end-to-end call control signalling.

R-8.2/2: End-to-end IP connectivity should be available as soon as possible, i.e., the MG should internally cut-through the bearer path as soon as fully specified media and transport descriptions are available for individual traffic directions.

NOTE – Requirement R-8.1/2 is also known as "early media", e.g., the called communication endpoint could already sent media in backward direction to the calling communication endpoint when acknowledging an incoming communication request.

It has to be noted that the temporary phase between a partial H.248 Context configuration ("which allows early media") and the final complete H.248 Context configuration is typically lasting only a few tens or hundreds of milliseconds. Thus, support of requirement R-8.2/1 should be normally more than sufficient for call controlled communication services, inclusive the category of realtime services.

There are two follow-up requirements for the early media capability (R-8.2/2):

R-8.2/3: The MG should support unidirectional early media, from the terminating (altc) UE back to the originating UE.

R-8.2/4: The MG should support bidirectional early media between both UEs.

Above requirements impact the Context internal model and topology, e.g. oneway or bothway topology, and may reduce the set Context model options (as outlined in clause 7.1).

## 8.3	Call control signaling: number of negotiation cycles

The indication and negotiation of media configurations at call control signalling level could theoretically subject of multiple (SDP offer/answer) negotiation cycles. The number of cycles is usually only bounded by performance objectives concerning call establishment delays.

R-8.3/1: Network solutions aims for single cycle negotiations, i.e., there is only one SDP offer and one SDP answer at SIP level.

R-8.3/2: Network solutions allow multi cycle negotiations.

Normally, the number of H.248 resource reservation cycles is decoupled and independent from call control procedures, however, incoming SIP messages usually acting as direct stimuli for H.248 signalling actions.

## 8.4	Gateway control signaling: resource management process in H.248 MG

Following resource management models might be worth of consideration:

- model of resource "protocol stack":

- monolithic resource model, i.e., an entire SDP media description is modelled as a single MG resource;

- decomposed resource model by separating the protocol stack in a lower segment of network/transport resources and a upper segment of the actual media-related resources.

NOTE – The decomposed model (1b) is already anchored in the ETSI TISPAN Ia profile (see [ETSI TS 183 018], clause 5.17.1.11 "One-Stage and Two-Stage BGF Resource Reservation") by splitting the protocol stack in SDP "c="-line and SDP "m="-line resource partitions.

It is obvious that there will be trade-off decisions between "early end-to-end connectivity and media path usage" versus "MG resource management complexity". It is not the purpose of this Supplement to evaluate this topic in more detail, but there might be at least following requirements postulated due to H.248 profiles according to [ETSI TS 183 018]:

R-8.4/1: The MGC-to-MG resource reservation process follows the process that available SDP "m="-line is already signalled together with "c="-line information.

R-8.4/2: The MGC-to-MG resource reservation process supports the process that the MGC would provide in a first phase IP connection resources only ("which would be here SDP "c="-line plus "a=altc:"-lines"), without yet media type / media format information. The "m="-line part would be then subjedt of the second phase in case of media aware type of IWF.

## 8.5	Gateway control signaling: explicit ALTC "call" tagging

The ALTC network service might be basically fully transparent to H.248 entities (e.g., in case of a lacking ALTC signalling element at the H.248 interface), however, the MG might support optimized resource management when knowing that a particular call contains "ALTC resource components". Resulting in following requirements:

R-8.5/1: ALTC could be transparent to the H.248 MG. The MG processes "ALTC context configuration" requests as regular communication services.

NOTE – This requirement could result in inefficient MG resource usage and decreased MG capacity.

R-8.5/2: Support of explicit ALTC context tagging (via a ContextAttribute, or Termination-level property, or SDP element, or …).

## 8.6	Gateway control signaling: specific resource components in H.248 MG

R-8.6/1: ALTC should be supported by "conferencing resources".

# 9	Discussion

The various solution options from clause 7 are evaluated by considering example signalling scenarios.

## 9.1	Call control signalling: examples of SIP SDP offers

### 9.1.1	SIP SDP offer example 1: monomedia audio telephony

A typical SIP-level SDP offer is illustrated in Table 9.1.1 (NOTE – the ALTC-specific SDP information is highlighted in yellow).

Table 9.1.1 – Example command encoding – (SIP) SDP Offer example 1

| (SIP) SDP encoding | Comments |
| --- | --- |
| v=0 o=... s=... t=... c=IN IP4 <UE_X_IPv4_addr_data> m=audio <UE_X_IPv4_port_audio> RTP/AVP <fmt> … ; SDP attributes for audio format description(s) a=altc:1 IP6 <UE_X_IPv6_addr_data> <UE_X_IPv6_port_audio> a=altc:2 IP4 <UE_X_IPv4_addr_data> <UE_X_IPv4_port_audio> … | The "altc" attribute is a media-level-only attribute and MUST NOT appear at the SDP session-level. The "c="-line is at session-level in example 1. |

### 9.1.2	SIP SDP offer example 2: multimedia WebRTC call

Consideration of a dual media WebRTC call with media components audio and video. A typical SIP-level SDP offer is illustrated in Table 9.1.2.

Table 9.1.2 – Example command encoding – (SIP) SDP Offer example 2

| (SIP) SDP encoding | Comments |
| --- | --- |
| v=0 o=... s=... t=... a=group:BUNDLE audio video ; m=audio <UE_X_IPv4_port_audio_rtp> RTP/SAVPF 103 104 0 8 106 105 13 126 c=IN IP4 <UE_X_IPv4_addr_audio_rtp> a=rtcp:<UE_X_IPv4_port_audio_rtcp> IN IP4 <UE_X_IPv4_addr&port_audio_rtcp> a=mid:audio a=rtcp-mux [attributes for media formats, RTP media multiplexing, …] a=sendrecv a=altc:1 IP6 <UE_X_IPv6_addr_audio data> <UE_X_IPv6_port_audio> a=altc:2 IP4 <UE_X_IPv4_addr_audio data> <UE_X_IPv4_port_audio> ; m=video <UE_X_IPv4_port_video_rtp> RTP/SAVPF 100 101 102 c=IN IP4 <UE_X_IP_addr_video_rtp> a=rtcp:<UE_X_IPv4_port_video_rtcp> IN IP4 <UE_X_IPv4_addr&port_video_rtcp> a=mid:video a=rtcp-mux [attributes for media formats, RTP media multiplexing, …] a=sendrcv a=altc:1 IP6 <UE_X_IPv6_addr_video data> <UE_X_IPv6_port_video> a=altc:2 IP4 <UE_X_IPv4_addr_video data> <UE_X_IPv4_port_video> … | There are two media descriptions for audio and video. Both are grouped via bundling. The SIP UA provides a multihomed IP host capability (inclusive dual-stack IP interfaces) and wants to use different IP interfaces for each media component. Hence, there are SDP media-level "c="-lines (in contrast to example 1). There are consequently two pairs of "a=altc:"-lines, one per media description. The SDP offer considers that IPv4 is most likely to be used for the session, but IPv6 is preferred. Furthermore, RTCP control flows could be either transport multiplexed (via "a=rtcp-mux") or transport unmultiplexed by allocation of an explicit transport port (via "a=rtcp:"). |

'The combination of "bundling of audio and video" and separate IP connection endpoints for audio and video isn't a contradiction (in the SDP offer). When the SDP answerer doesn't support bundling then just all bundle related SDP ("a=group:BUNDLE" and "a=mid:") would be ignored and media type individual IP connections would be used. When the SDP answerer supports bundling then the first IP connection would be shared by both media components.

Proceeding of evaluation:

The 2nd SDP offer example should be more suitable for the demonstration of ALTC support by H.248 entities due to variety of IP interfaces, alternatives, transport multiplexing and RTCP port allocation options.

It has to be noted that the typical ICE functionality (as part of WebRTC call control signalling) is not considered here. There would not be any added value, see see clause 4.2.3/[IETF RFC 6947] "Interaction with ICE".

## 9.2	Gateway control signalling: examples

(SIP) SDP Offer example 2 is used for below H.248 signalling examples. H.248 signalling illustrates only the "ALTC side" of an H.248 context.

{Editor's note (2015-02): it was commented that the signalling examples might be simplified by indicating a single media type only instead of two (her audio and video).
Background for two media types in the signalling examples:
Idea: "ALTC" relates actually to a vertical split of the "protocol stack" in a lower layer "network" segment and an upper layer "media" segment, hence the rough two classes of SDP "c="- and "m="-line related information. Both classes relate to two categories of MG resources. Any H.248 transaction cycle will lead to reservation, allocation, de-allocation or modification of such resources. It seems to be beneficial to separate also correspondent signalling. E.g., if not separated, then any sub-sequent H.248 transaction cycle must repeat all signalling information related to the "media" segment. There seems to be benefit from avoiding such redundancy.
This is a quantitative criteria. However, there isn't yet any proof of this assumption because the signalling examples are not yet complete.)

### 9.2.1	Example H.248 signalling in case of "application aware IWF"

Thus, there would be fully specified media descriptions.

#### 9.2.1.1	Usage of H.248 ALTC Context model "M1" (two Contexts)

##### 9.2.1.1.1	First transaction cycle: preparation of local IP connection endpoints

The SIP-level SDP offer stimulates an H.248 transaction request from MGC to MG (Table 9.2.1.1/1). The MGC selects the two Context model "M1" (as illustrated in Figure 2).

Notes to SIP (SDP) to H.248 (SDP) mapping:

- Decision (by MGC) concerning the order of preferred IP connectivity:

Resources of local destination (inclusive local source) IP connection endpoints (i.e., LD(A,P) and LS(A,P) are supposed to be managed at MG level, which implies SDP wildcarding of such resources in the H.248 local destination descriptor. The MGC shall therefore request the preferred IP connection endpoint resources in Context Cz,1 (example here: IPv6 due the network operator policy for IPv6 preference in IP domain Y).

- Decision (by MGC) concerning "RTP /RTCP transport multiplexing versus explicit RTCP transport address allocation":

MGC considers that is most likely to be used for the IP connection segment towards UE_B. The SDP "a=rtcp:" information is therefore removed from the SDP media description in H.248 Stream Descriptors.

The first H.248 resource reservation cycle includes the creation of "ALTC MG resources", see Table 9.2.1.1/1 and Table 9.2.1.1/2.

Table 9.2.1.1/1 – Example command encoding – Step 1 – MGC request 
(for local connection point only in outgoing direction)

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MGC to MG: MEGACO/3 [11.9.19.65]:55555 Transaction = 1 { Context = $ { ; Context CZ1 Add = ip/$/$/$ { ; Termination TY,P Media { Stream = 1 { ; SEP S1 LocalControl { ipdc/realm = <realm_Y_IPv6>, Mode = Inactive, ; NOTE 1 ReservedGroup = OFF, ; the default value ReservedValue = OFF, ; the default value ... }, Local { v=0 c=IN IP6 $ m=audio $ RTP/SAVPF 103 104 0 8 106 105 13 126 a=rtcp-mux [attributes for media formats, RTP media multipl., …] … }, Remote { ; still empty (because SDP answer from UE Y not yet received } }, Stream = 2 { ; SEP S2 LocalControl { ipdc/realm = <realm_Y_IPv6>, Mode = Inactive, ; NOTE 1 ReservedGroup = OFF, ; the default value ReservedValue = OFF, ; the default value ... }, Local { v=0 c=IN IP6 $ m=video $ RTP/SAVPF 100 101 102 a=rtcp-mux [attributes for media formats, RTP media multipl., …] , Remote { ; still empty (because SDP answer from UE Y not yet received } … }, Transaction = 2 { Context = $ { ; Context CZ2 Add = ip/$/$/$ { ; Termination TY,A Media { Stream = 1 { ; SEP S1 LocalControl { ipdc/realm = <realm_Y_IPv4>, Mode = Inactive, ; NOTE 2 ReservedGroup = OFF, ; the default value ReservedValue = OFF, ; the default value ... }, Local { v=0 c=IN IP4 $ m=audio $ RTP/SAVPF 103 104 0 8 106 105 13 126 a=rtcp-mux [attributes for media formats, RTP media multipl., …] … }, Remote { ; still empty (because SDP answer from UE Y not yet received } }, Stream = 2 { ; SEP S2 LocalControl { ipdc/realm = <realm_Y_IPv4>, Mode = Inactive, ; NOTE 2 ReservedGroup = OFF, ; the default value ReservedValue = OFF, ; the default value ... }, Local { v=0 c=IN IP4 $ m=video $ RTP/SAVPF 100 101 102 a=rtcp-mux [attributes for media formats, RTP media multipl., …] }, Remote { ; still empty (because SDP answer from UE Y not yet received } … } | The ADD.request command for Termination TX is omitted. Some observations: - SDP attributes "a=altc:" are not used in H.248 descriptors. - The IP connection endpoints for IPv4 and IPv6 belong essentially to separate IP address realms. They can't be inherently part of the same IP address space. That characteristic is reflected by two separate codepoints of property ipdc/realm. - The H.248 Context configuration starts in application-aware mode in this example. There might be also the option in delaying the media format specific details to a next transaction cycle, in conjunction with the final selection of the IP connection endpoint. NOTE 1 – The StreamMode property setting of value "Inactive" at SEPs at Termination TY,P prevents support of "early media". NOTE 2 – The StreamMode property setting of value "Inactive" at SEPs at Termination TY,A is required because these SEPs are still isolated in Context Cz.2. |

A possible example MG reply is indicated in Table 9.2.1.1/2.

Table 9.2.1.1/2 – Example command encoding – Step 2 – MG reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MG to MGC: MEGACO/3 [19.65.11.9]:44444 Reply = 1 { Context = Cz1 { ; Context CZ1 Add = ip/y/p/1 { ; Termination TY,P Media { Stream = 1 { ; SEP S1 Local { v=0 c=IN IP6 <IP_Y,P_IPv6_addr_audio> m=audio <IP_Y,P_IPv6_port_audio> RTP/SAVPF 103 a=rtcp-mux … }, Remote { ; still empty (because SDP answer from UE Y not yet received } }, Stream = 2 { ; SEP S2 Local { v=0 c=IN IP6 <IP_Y,P_IPv6_addr_video> m=video <IP_Y,P_IPv6_port_video> RTP/SAVPF 100 a=rtcp-mux … }, Remote { ; still empty (because SDP answer from UE Y not yet received } … }, Reply = 2 { Context = Cz2 { ; Context CZ2 Add = ip/y/a/1 { ; Termination TY,A Media { Stream = 1 { ; SEP S1 Local { v=0 c=IN IP4 <IP_Y,P_IPv4_addr_audio> m=audio <IP_Y,P_IPv4_port_audio> RTP/SAVPF 103 a=rtcp-mux [attributes for media formats, RTP media multipl., …] … }, Remote { ; still empty (because SDP answer from UE Y not yet received } Stream = 2 { ; SEP S2 Local { v=0 c=IN IP4 <IP_Y,P_IPv4_addr_video> m=video <IP_Y,P_IPv4_port_video> RTP/SAVPF 100 a=rtcp-mux ## [attributes for media formats, RTP media multipl., …] }, Remote { ; still empty (because SDP answer from UE Y not yet received } … } | The media format list is reduced to a single media format (here '103' in case of audio) due to ReserveValue setting of FALSE. The decision made by the MG is dependent on the Stream Descriptor settings at Termination TX. |

##### 9.2.1.1.2	Second transaction cycle: decision to use preferred IP connection

The second H.248 resource reservation cycle completes media configurations and removes unused "ALTC MG resources", see Table 9.2.1.1/3a and Table 9.2.1.1/.4a.

Table 9.2.1.1/3a – Example command encoding – Step 3a – MGC request

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MGC to MG: MEGACO/3 [11.9.19.65]:55555 Transaction = 3 { Context = Cz1 { ; Context CZ1 Modify = ip/y/p/1 { ; Termination TY,P Media { Stream = 1 { ; SEP S1 LocalControl { ## ipdc/realm = <realm_Y_IPv6>, Mode = SendRecv, ReservedGroup = OFF, ; the default value ReservedValue = OFF, ; the default value ... }, ; LocalDescriptor don't need to be modified anymore (NOTE 1) Remote { v=0 c=IN IP6 <IP_UE_Y_IPv6_addr_audio> m=audio <IP_UE_Y_IPv6_port_audio> RTP/SAVPF 103 a=rtcp-mux [attributes for media formats, RTP media multipl., …] … }, }, Stream = 2 { ; SEP S2 LocalControl { ## ipdc/realm = <realm_Y_IPv6>, Mode = SendRecv, ReservedGroup = OFF, ; the default value ReservedValue = OFF, ; the default value ... }, ; LocalDescriptor don't need to be modified anymore Remote { v=0 c=IN IP6 <IP_UE_Y_IPv6_addr_video> m=video <IP_UE_Y_IPv6_port_video> RTP/SAVPF 100 a=rtcp-mux ## [attributes for media formats, RTP media multipl., …] } … } Transaction = 4 { Context = Cz2 { ; Context CZ2 Subtract = ip/y/a/1 { ; Termination TY,A } } | ## IPv6 is the selected IP version, hence Termination TY,P is kept and completety specified by addition of the still missing remote destination information (via H.248 MODify command). ## NOTE 1 – The LD with the reduced media format list don't need to be repeated again because still stored (as it should be) at MG side. ## And Termination TY,A is removed via H.248 SUBtract. |

A possible example MG reply is indicated in Table 9.2.1.1/4a.

Table 9.2.1.1/4a – Example command encoding – Step 4a – MG reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MG to MGC: MEGACO/3 [19.65.11.9]:44444 Reply = 3 { Context = Cz1 { ; Context CZ1 Modify = ip/y/p/1 { ; Termination TY,P Media { Stream = 1 { ; SEP S1 Local { v=0 c=IN IP6 <IP_Y,P_IPv6_addr_audio> m=audio <IP_Y,P_IPv6_port_audio> RTP/SAVPF 103 a=rtcp-mux … }, Remote { v=0 c=IN IP6 <IP_UE_Y_IPv6_addr_audio> m=audio <IP_UE_Y_IPv6_port_audio> RTP/SAVPF 103 a=rtcp-mux … } }, Stream = 2 { ; SEP S2 Local { v=0 c=IN IP6 <IP_Y,P_IPv6_addr_video> m=video <IP_Y,P_IPv6_port_video> RTP/SAVPF 100 a=rtcp-mux … }, Remote { v=0 c=IN IP6 <IP_UE_Y_IPv6_addr_video> m=video <IP_UE_Y_IPv6_port_video> RTP/SAVPF 100 a=rtcp-mux } …}, Reply = 4 { Context = Cz2 { Subtract = ip/y/a/1 { Statistics { … } } } } |  |

##### 9.2.1.1.3	Second transaction cycle: decision to use alternate IP connection

IPv4 is the selected IP version, hence Termination TY,P is subtracted and replaced by Termination TY,A (using an H.248 MOVE command). See Table 9.2.1.1/3b and Table 9.2.1.1/.4b.

Table 9.2.1.1/3b – Example command encoding – Step 3b – MGC request

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MGC to MG: MEGACO/3 [11.9.19.65]:55555 Transaction = 3 { Context = Cz1 { ; Context CZ1 ; step 1 – removal of TY,P via SUBtract Subtract = ip/y/p/1 { ; Termination TY,P }, ; step 2 – replacement by TY,A via MOVe Move = ip/y/a/1 { ; Termination TY,A Media { Stream = 1 { ; SEP S1 LocalControl { ipdc/realm = <realm_Y_IPv4>, Mode = SendRecv, ReservedGroup = OFF, ; the default value ReservedValue = OFF, ; the default value ... }, ; LocalDescriptor don't need to be modified anymore Remote { v=0 c=IN IP4 <IP_UE_Y_IPv4_addr_audio> m=audio <IP_UE_Y_IPv4_port_audio> RTP/SAVPF 103 a=rtcp-mux [attributes for media formats, RTP media multipl., …] … } }, Stream = 2 { ; SEP S2 LocalControl { ipdc/realm = <realm_Y_IPv4>, Mode = SendRecv, ReservedGroup = OFF, ; the default value ReservedValue = OFF, ; the default value ... }, ; LocalDescriptor don't need to be modified anymore Remote { v=0 c=IN IP4 <IP_UE_Y_IPv4_addr_video> m=video <IP_UE_Y_IPv4_port_video> RTP/SAVPF 100 101 102 a=rtcp-mux ## [attributes for media formats, RTP media multipl., …] } … ## } | IPv4 is the selected IP version, hence Termination TY,P is subtracted and replaced by Termination TY,A (using an H.248 MOVE command). Both H.248 command requests are subject of Context Cz1. It has to be noted that the Stream identifier values in the moved Termination has to be consistent with the removed Termination. |

A possible example MG reply is indicated in Table 9.2.1.1/4b.

Table 9.2.1.1/4b – Example command encoding – Step 4b – MG reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MG to MGC: MEGACO/3 [19.65.11.9]:44444 Reply = 3 { Context = Cz1 { ; Context CZ1 Subtract = ip/y/p/1 { Statistics { … } }, Move = ip/y/a/1 { ; Termination TY,A Media { Stream = 1 { ; SEP S1 Local { v=0 c=IN IP4 <IP_Y,P_IPv4_addr_audio> m=audio <IP_Y,P_IPv4_port_audio> RTP/SAVPF 103 a=rtcp-mux … }, Remote { v=0 c=IN IP4 <IP_UE_Y_IPv4_addr_audio> m=audio <IP_UE_Y_IPv4_port_audio> RTP/SAVPF 103 a=rtcp-mux … }, Stream = 2 { ; SEP S2 Local { v=0 c=IN IP4 <IP_Y,P_IPv4_addr_video> m=video <IP_Y,P_IPv4_port_video> RTP/SAVPF 100 a=rtcp-mux … }, Remote { v=0 c=IN IP4 <IP_UE_Y_IPv4_addr_video> m=video <IP_UE_Y_IPv4_port_video> RTP/SAVPF 100 a=rtcp-mux } …} |  |

#### 9.2.1.2	Usage of H.248 ALTC Context model "M2" (single Context with two Terminations for ALTC)

##### 9.2.1.2.1	First transaction cycle: preparation of local IP connection endpoints

The option without early media support is selected, i.e., there will be not an early Context internal cut-through of the H.248 Streams. This is achieved be keeping the StreamMode inactive.

The first H.248 resource reservation cycle includes the creation of "ALTC MG resources", see Table 9.2.1.2/1a and Table 9.2.1/1b and Table 9.2.1.2/2a and Table 9.2.1.2/2b.

Table 9.2.1.2/1a – Example command encoding – Step 1a – MGC request 
(for local connection point only in outgoing direction)

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MGC to MG: MEGACO/3 [11.9.19.65]:55555 Transaction = 1 { Context = $ { ; Context CZ1 Add = ip/$/$/$ { ; Termination TY,P Media { Stream = 1 { ; SEP S1 LocalControl { ipdc/realm = <realm_Y_IPv6>, Mode = Inactive, ReservedGroup = OFF, ; the default value ReservedValue = OFF, ; the default value ... }, Local { v=0 c=IN IP6 $ m=audio $ RTP/SAVPF 103 104 0 8 106 105 13 126 a=rtcp-mux [attributes for media formats, RTP media multipl., …] }, Remote { ; still empty (because SDP answer from UE Y not yet received } }, Stream = 2 { ; SEP S2 LocalControl { ipdc/realm = <realm_Y_IPv6>, Mode = Inactive, ReservedGroup = OFF, ; the default value ReservedValue = OFF, ; the default value ... }, Local { v=0 c=IN IP6 $ m=video $ RTP/SAVPF 100 101 102 a=rtcp-mux [attributes for media formats, RTP media multipl., …] }, Remote { ; still empty (because SDP answer from UE Y not yet received } } } } | At first the MGC adds Termination TY,P to the new Context. Termination TY,A is added in a subsequent step (Step 2a) with a topology tuple isolating it from TY,P. |

A possible example MG reply is indicated in Table 9.2.1.2/1b.

Table 9.2.1.2/1b – Example command encoding – Step 1b – MG reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MG to MGC: MEGACO/3 [19.65.11.9]:44444 Reply = 1 { Context = Cz1 { ; Context CZ1 Add = ip/y/p/1 { ; Termination TY,P Media { Stream = 1 { ; SEP S1 Local { v=0 c=IN IP6 <IP_Y,P_IPv6_addr_audio> m=audio <IP_Y,P_IPv6_port_audio> RTP/SAVPF 103 a=rtcp-mux [attributes for media formats, RTP media multipl., …] } }, Stream = 2 { ; SEP S2 Local { v=0 c=IN IP6 <IP_Y,P_IPv6_addr_video> m=video <IP_Y,P_IPv6_port_video> RTP/SAVPF 100 a=rtcp-mux [attributes for media formats, RTP media multipl., …] } } } }}} |  |

The MGC adds the alternate Termination to the Context and isolates this from the preferred Termination, see Table 9.2.1.2/2a and Table 9.2.1.2/2b.

Table 9.2.1.2/2a – Example command encoding – Step 2a – MGC request 
(for local connection point only in outgoing direction)

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MGC to MG: MEGACO/3 [11.9.19.65]:55555 Transaction = 1 { Context = $ { ; Context CZ1 Topology {ip/y/p/1, ip/$/$/$, Isolate}, ; 2nd Termination for the same Context Add = ip/$/$/$ { ; Termination TY,A Media { Stream = 1 { ; SEP S1 LocalControl { ipdc/realm = <realm_Y_IPv4>, Mode = Inactive, ReservedGroup = OFF, ; the default value ReservedValue = OFF, ; the default value ... }, Local { v=0 c=IN IP4 $ m=audio $ RTP/SAVPF 103 104 0 8 106 105 13 126 a=rtcp-mux [attributes for media formats, RTP media multipl., …] }, Remote { ; still empty (because SDP answer from UE Y not yet received } }, Stream = 2 { ; SEP S2 LocalControl { ipdc/realm = <realm_Y_IPv4>, Mode = Inactive, ReservedGroup = OFF, ; the default value ReservedValue = OFF, ; the default value ... }, Local { v=0 c=IN IP4 $ m=video $ RTP/SAVPF 100 101 102 a=rtcp-mux [attributes for media formats, RTP media multipl., …] }, Remote { ; still empty (because SDP answer from UE Y not yet received } } }}}} | The alternate Termination TY,A is added and is isolated from the preferred Termination TY,P. Such an operation mandates usage of the H.248 topology descriptor. |

A possible example MG reply is indicated in Table 9.2.1.2/2b.

Table 9.2.1.2/2b – Example command encoding – Step 2b – MG reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MG to MGC: MEGACO/3 [19.65.11.9]:44444 Reply = 1 { Context = Cz1 { ; Context CZ1 ; 2nd Termination for the same Context Add = ip/y/a/1 { ; Termination TY,A Media { Stream = 1 { ; SEP S1 Local { v=0 c=IN IP4 <IP_Y,P_IPv4_addr_audio> m=audio <IP_Y,P_IPv4_port_audio> RTP/SAVPF 103 a=rtcp-mux [attributes for media formats, RTP media multipl., …] }, }, Stream = 2 { ; SEP S2 Local { v=0 c=IN IP4 <IP_Y,P_IPv4_addr_video> m=video <IP_Y,P_IPv4_port_video> RTP/SAVPF 100 a=rtcp-mux [attributes for media formats, RTP media multipl., …] } } }}}} |  |

##### 9.2.1.2.2	Second transaction cycle: decision to use preferred IP connection

The second H.248 resource reservation cycle completes media configurations and removes unused "ALTC MG resources", see Table 9.2.1.2/3a and Table 9.2.1.2/.4a.

Table 9.2.1.2/3a – Example command encoding – Step 3a – MGC request

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MGC to MG: MEGACO/3 [11.9.19.65]:55555 Transaction = 2 { Context = Cz1 { ; Context CZ1 Modify = ip/y/p/1 { ; Termination TY,P Media { Stream = 1 { ; SEP S1 LocalControl { ipdc/realm = <realm_Y_IPv6>, Mode = SendReceive, ReservedGroup = OFF, ; the default value ReservedValue = OFF, ; the default value ... }, Remote { v=0 c=IN IP6 <IP_UE_Y_IPv6_addr_audio> m=audio <IP_UE_Y_IPv6_port_audio> RTP/SAVPF 103 a=rtcp-mux [attributes for media formats, RTP media multipl., …] } }, Stream = 2 { ; SEP S2 LocalControl { ipdc/realm = <realm_Y_IPv6>, Mode = SendReceive, ReservedGroup = OFF, ; the default value ReservedValue = OFF, ; the default value ... }, Remote { v=0 c=IN IP6 <IP_UE_Y_IPv6_addr_video> m=video <IP_UE_Y_IPv6_port_video> RTP/SAVPF 100 a=rtcp-mux [attributes for media formats, RTP media multipl., …] } } } }, ; 2nd Termination for the same Context Subtract = ip/y/a/1 { ; Termination TY,A } } } | IPv6 is the selected IP version, hence Termination TY,P is kept and completely specified by addition of the still missing remote destination information. The alternate Termination TY,A is removed from the Context. |

A possible example MG reply is indicated in Table 9.2.1.2/4a.

Table 9.2.1.2/4a – Example command encoding – Step 4a – MG reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MG to MGC: MEGACO/3 [19.65.11.9]:44444 Transaction = 2 { Context = Cz1 { ; Context CZ1 Modify = ip/y/p/1 { ; Termination TY,P Media { Stream = 1 { ; SEP S1 Local { v=0 c=IN IP6 <IP_Y,P_IPv6_addr_audio> m=audio <IP_Y,P_IPv6_port_audio> RTP/SAVPF 103 a=rtcp-mux [attributes for media formats, RTP media multipl., …] }, Remote { v=0 c=IN IP6 <IP_UE_Y_IPv6_addr_audio> m=audio <IP_UE_Y_IPv6_port_audio> RTP/SAVPF 103 a=rtcp-mux [attributes for media formats, RTP media multipl., …] } }, Stream = 2 { ; SEP S2 Local { v=0 c=IN IP6 <IP_Y,P_IPv6_addr_video> m=video <IP_Y,P_IPv6_port_video> RTP/SAVPF 100 a=rtcp-mux [attributes for media formats, RTP media multipl., …] }, Remote { v=0 c=IN IP6 <IP_UE_Y_IPv6_addr_video> m=video <IP_UE_Y_IPv6_port_video> RTP/SAVPF 100 a=rtcp-mux [attributes for media formats, RTP media multipl., …] } } } }, ; 2nd Termination for the same Context Subtract = ip/y/a/1 { ; Termination TY,A Statistics { … } } } } |  |

##### 9.2.1.2.3	Second transaction cycle: decision to use alternate IP connection

IPv4 is the selected IP version, hence Termination TY,P is subtracted. See Table 9.2.1.2/3b and Table 9.2.1.2/.4b.

Table 9.2.1.2/3b – Example command encoding – Step 3b – MGC request

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MGC to MG: MEGACO/3 [11.9.19.65]:55555 Transaction = 2 { Context = Cz1 { ; Context CZ1 Subtract = ip/y/p/1 { ; Termination TY,P }, Modify = ip/y/a/1 { ; Termination TY,A Media { Stream = 1 { ; SEP S1 LocalControl { ipdc/realm = <realm_Y_IPv4>, Mode = SendReceive, ReservedGroup = OFF, ; the default value ReservedValue = OFF, ; the default value ... }, Remote { v=0 c=IN IP6 <IP_UE_Y_IPv4_addr_audio> m=audio <IP_UE_Y_IPv4_port_audio> RTP/SAVPF 103 a=rtcp-mux [attributes for media formats, RTP media multipl., …] } } Stream = 2 { ; SEP S2 LocalControl { ipdc/realm = <realm_Y_IPv4>, Mode = SendReceive, ReservedGroup = OFF, ; the default value ReservedValue = OFF, ; the default value ... }, Remote { v=0 c=IN IP6 <IP_UE_Y_IPv4_addr_video> m=video <IP_UE_Y_IPv4_port_video> RTP/SAVPF 100 a=rtcp-mux [attributes for media formats, RTP media multipl., …] } } } } } } | IPv4 is the selected IP version, hence Termination TY,A is kept and completely specified by addition of the still missing remote destination information. The preferred Termination TY,P is removed from the Context. |

A possible example MG reply is indicated in Table 9.2.1.2/4b.

Table 9.2.1.2/4b – Example command encoding – Step 4b – MG reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MG to MGC: MEGACO/3 [19.65.11.9]:44444 Transaction = 2 { Context = Cz1 { ; Context CZ1 Subtract = ip/y/p/1 { ; Termination TY,P Statistics { … } }, Modify = ip/y/a/1 { ; Termination TY,A Media { Stream = 1 { ; SEP S1 Local { v=0 c=IN IP6 <IP_Y,P_IPv4_addr_audio> m=audio <IP_Y,P_IPv4_port_audio> RTP/SAVPF 103 a=rtcp-mux [attributes for media formats, RTP media multipl., …] }, Remote { v=0 c=IN IP6 <IP_UE_Y_IPv4_addr_audio> m=audio <IP_UE_Y_IPv4_port_audio> RTP/SAVPF 103 a=rtcp-mux [attributes for media formats, RTP media multipl., …] } }, Stream = 2 { ; SEP S2 Local { v=0 c=IN IP6 <IP_Y,P_IPv4_addr_video> m=video <IP_Y,P_IPv4_port_video> RTP/SAVPF 100 a=rtcp-mux [attributes for media formats, RTP media multipl., …] }, Remote { v=0 c=IN IP6 <IP_UE_Y_IPv4_addr_video> m=video <IP_UE_Y_IPv4_port_video> RTP/SAVPF 100 a=rtcp-mux [attributes for media formats, RTP media multipl., …] } } } } } } |  |

#### 9.2.1.3	Usage of H.248 ALTC Context model "M3" (single Termination with two SEPs for ALTC)

This method is conditional, applicable for single-Stream-per-Termination structures only, such as an audio call only in this example. The first SEP is used for the resources tight to the preferred IP connection, the second contains resources related to the alternate IP connection. The method is of theoretical nature only and not relevant in practice.

##### 9.2.1.3.1	First transaction cycle: preparation of local IP connection endpoints

The first H.248 resource reservation cycle includes the creation of "ALTC MG resources", see Table 9.2.1.3/1.

Table 9.2.1.3/1 – Example command encoding – Step 1 – MGC request 
(for local connection point only in outgoing direction)

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MGC to MG: MEGACO/3 [11.9.19.65]:55555 Transaction = 1 { Context = $ { ; Context CZ1 Add = ip/$/$/$ { ; Termination TY Media { Stream = 1 { ; SEP S1 LocalControl { ipdc/realm = <realm_Y_IPv6>, Mode = Inactive, ReservedGroup = OFF, ; the default value ReservedValue = OFF, ; the default value ... }, Local { v=0 c=IN IP6 $ m=audio $ RTP/SAVPF 103 104 0 8 106 105 13 126 a=rtcp-mux [attributes for media formats, RTP media multipl., …] }, Remote { ; still empty (because SDP answer from UE Y not yet received } Stream = 2 { ; SEP S2 LocalControl { ipdc/realm = <realm_Y_IPv4>, Mode = Inactive, ReservedGroup = OFF, ; the default value ReservedValue = OFF, ; the default value ... }, Local { v=0 c=IN IP4 $ m=audio $ RTP/SAVPF 103 104 0 8 106 105 13 126 a=rtcp-mux [attributes for media formats, RTP media multipl., …] Remote { ; still empty (because SDP answer from UE Y not yet received } …} | The two SEPs got inherently different StreamID assignments ("TY(S1)" and "TY(S2)") due to the full specification of this protocol parameter value already at MGC level. |

It is obvious that that Context-internally the two SEPs TX(S1) and TY(S2) could not be interconnected in case the alternate IPv4 based transport address is chosen, as long as the MG's IP  transport address resource management is located on MG level (as always assumed here).

Thus, the exercise is stopped because there is not any real chance in providing a correct protocol solutions ("any temporary phases with protocol objects and protocol parameters values in transitioning states are strictly excluded from protocol perspective").

#### 9.2.1.4	Usage of H.248 ALTC Context model "M4" (two media groups for ALTC)

##### 9.2.1.4.1	First transaction cycle: preparation of local IP connection endpoints

There are two media groups (preferred media group and alternate media group) required per media type (audio and video). Such a resource reservation is achieved by a setting of the LocalControl properties combination of {ReserveValue = "False" AND ReserveGroup = "True"}.

The first H.248 resource reservation cycle includes the creation of "ALTC MG resources", see Table 9.2.1.4/1 and Table 9.2.1.4/2.

Table 9.2.1.4/1 – Example command encoding – Step 1 – MGC request 
(for local connection point only in outgoing direction)

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MGC to MG: MEGACO/3 [11.9.19.65]:55555 Transaction = 1 { Context = $ { ; Context CZ1 Add = ip/$/$/$ { ; Termination TY Media { Stream = 1 { ; SEP S1 LocalControl { ipdc/realm = [<realm_Y_IPv6>, <realm_Y_IPv4>], Mode = Inactive, ReservedGroup = ON, ; two media groups! ReservedValue = OFF, ; the default value ... }, Local { v=0 ; 1st media group for audio c=IN IP6 $ m=audio $ RTP/SAVPF 103 104 0 8 106 105 13 126 a=rtcp-mux [attributes for media formats, RTP media multipl., …] ; end of 1st media group v=0 ; 2nd media group for audio c=IN IP4 $ m=audio $ RTP/SAVPF 103 104 0 8 106 105 13 126 a=rtcp-mux [attributes for media formats, RTP media multipl., …] … ; end of 2nd media group }, Remote { ; still empty (because SDP answer from UE Y not yet received } }, Stream = 2 { ; SEP S2 LocalControl { ipdc/realm = [<realm_Y_IPv6>, <realm_Y_IPv4>], Mode = Inactive, ReservedGroup = ON, ; two media groups ReservedValue = OFF, ; the default value ... }, Local { v=0 ; 1st media group for video c=IN IP6 $ m=video $ RTP/SAVPF 100 101 102 a=rtcp-mux [attributes for media formats, RTP media multipl., …] ; end of 1st media group v=0 ; 2nd media group for video c=IN IP4 $ m=video $ RTP/SAVPF 100 101 102 a=rtcp-mux [attributes for media formats, RTP media multipl., …] ; end of 2nd media group }, Remote { ; still empty (because SDP answer from UE Y not yet received } … } | It has to be noted that the indication of IP address realms requires ipdc package version 2 (according to [ITU-T H.248.41]). |

More information about the requirement concerning the indication of IP address realms:

Problem statement:

- the IP domain connection indication (via property ipdc/realm from the IP Domain Connection package) is on LocalControl descriptor level; and

- the IP connection endpoint information in the SDP media description must be consistent with the IP domain indication

- hence, both H.248 media groups require separate IP domain indications.

Background:

- IPv4 and IPv6 belong inherently to two different IP address realms (or briefly IP realms, also known as IP domains in [ITU-T H.248.41]).

- Thus, the preferred IP connection and alternate IP connection are routed through different IP address realms.

- There are two IP domain connection code points within the single H.248 SEP.

- Furthermore, the IP realm identifier is mandatory signalling information due to the assumption of multiple IP address realms in almost all existing standardized H.248 profiles for H.248 IP-to-IP gateway types.

The new, ALTC-specific requirement is in the meanwhile addressed by Version 2 of the IP Domain Connection package.

A possible example MG reply is indicated in Table 9.2.1.4/2.

Table 9.2.1.4/2 – Example command encoding – Step 2 – MG reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MG to MGC: MEGACO/3 [19.65.11.9]:44444 Reply = 1 { Context = Cz1 { ; Context CZ1 Add = ip/y/p/1 { ; Termination TY Media { Stream = 1 { ; SEP S1 Local { v=0 ; 1st media group for audio c=IN IP6 <IP_Y,P_IPv6_addr_audio> m=audio <IP_Y,P_IPv6_port_audio> RTP/SAVPF 103 a=rtcp-mux [attributes for media formats, RTP media multipl., …] ; end of 1st media group v=0 ; 2nd media group for audio c=IN IP4 <IP_Y,P_IPv4_addr_audio> m=audio <IP_Y,P_IPv4_port_audio> RTP/SAVPF 103 a=rtcp-mux [attributes for media formats, RTP media multipl., …] ; end of 2nd media group }, Remote { ; still empty (because SDP answer from UE Y not yet received } }, Stream = 2 { ; SEP S2 Local { v=0 ; 1st media group for video c=IN IP6 <IP_Y,P_IPv6_addr_video> m=video <IP_Y,P_IPv6_port_video> RTP/SAVPF 100 a=rtcp-mux [attributes for media formats, RTP media multipl., …] ; end of 1st media group v=0 ; 2nd media group for video c=IN IP4 <IP_Y,P_IPv4_addr_video> m=video <IP_Y,P_IPv4_port_video> RTP/SAVPF 100 a=rtcp-mux [attributes for media formats, RTP media multipl., …] ; end of 2nd media group }, Remote { ; still empty (because SDP answer from UE Y not yet received } }, …} | The media format list is reduced to a single media format (here '103' in case of audio) due to ReserveValue setting of FALSE. The decision made by the MG is dependent on the Stream Descriptor settings at Termination TX. |

##### 9.2.1.4.2	Second transaction cycle: decision to use preferred IP connection

The second H.248 resource reservation cycle completes media configurations and removes unused "ALTC MG resources", see Table 9.2.1.4/3a and Table 9.2.1.4/.4a.

Table 9.2.1.4/3a – Example command encoding – Step 3a – MGC request

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MGC to MG: MEGACO/3 [11.9.19.65]:55555 Transaction = 2 { Context = Cz1 { ; Context CZ1 Modify = ip/y/p/1 { ; Termination TY Media { Stream = 1 { ; SEP S1 LocalControl { ; NOTE 1 ipdc/realm = <realm_Y_IPv6>, ; NOTE 2 Mode = SendRecv, ; NOTE 3 ReservedGroup = OFF, ; NOTE 4 ReservedValue = OFF, ; the default value ... }, Local { ; NOTE 5 v=0 ; selected media group for audio c=IN IP6 <IP_Y,P_IPv6_addr_audio> m=audio <IP_Y,P_IPv6_port_audio> RTP/SAVPF 103 a=rtcp-mux [attributes for media formats, RTP media multipl., …] ; end of "media group" ; no second media group for audio }, Remote { v=0 ; only a single media group for audio c=IN IP6 <IP_UE_Y_IPv6_addr_audio> m=audio <IP_UE_Y_IPv6_port_audio> RTP/SAVPF 103 a=rtcp-mux [attributes for media formats, RTP media multipl., …] } }, Stream = 2 { ; SEP S2 LocalControl { ipdc/realm = <realm_Y_IPv6>, Mode = Inactive, ReservedGroup = OFF, ; the default value ReservedValue = OFF, ; the default value ... }, Local { ; NOTE 5 v=0 ; selected media group for video c=IN IP6 <IP_Y,P_IPv6_addr_video> m=video <IP_Y,P_IPv6_port_video> RTP/SAVPF 100 a=rtcp-mux [attributes for media formats, RTP media multipl., …] ; end of "media group" ; no second media group for video }, Remote { v=0 ; only a single media group for video c=IN IP6 <IP_UE_Y_IPv6_addr_video> m=video <IP_UE_Y_IPv6_port_video> RTP/SAVPF 100 a=rtcp-mux [attributes for media formats, RTP media multipl., …] } …} | Preferred option is IPv6, i.e., the first H.248 media group remains and the second one is just not included in the transaction request. NOTE 1 – It may be noted that the LCD has to be included again (in contrast to e.g. model M1) due to changed property values NOTE 2 – The IP domain indication is changed now indicating the IPv6 associated realm only. NOTE 3 – Now, "early" media enabled. NOTE 4 – ReserveGroup disabled again due to selection of a single media group. NOTE 5 – It may be noted that the LD has to be completely included (in contrast to e.g. model M1) with the selected H.248 media group. |

A possible example MG reply is indicated in Table 9.2.1.4/4a.

Table 9.2.1.4/4a – Example command encoding – Step 4a – MG reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MG to MGC: MEGACO/3 [19.65.11.9]:44444 Reply = 1 { Context = Cz1 { ; Context CZ1 Modify = ip/y/p/1 { ; Termination TY Media { Stream = 1 { ; SEP S1 Local { v=0 c=IN IP6 <IP_Y,P_IPv6_addr_audio> m=audio <IP_Y,P_IPv6_port_audio> RTP/SAVPF 103 a=rtcp-mux [attributes for media formats, RTP media multipl., …] }, Remote { v=0 c=IN IP6 <IP_UE_Y_IPv6_addr_audio> m=audio <IP_UE_Y_IPv6_port_audio> RTP/SAVPF 103 a=rtcp-mux [attributes for media formats, RTP media multipl., …] } Stream = 2 { ; SEP S2 Local { v=0 c=IN IP6 <IP_Y,P_IPv6_addr_video> m=video <IP_Y,P_IPv6_port_video> RTP/SAVPF 100 a=rtcp-mux [attributes for media formats, RTP media multipl., …] }, Remote { v=0 c=IN IP6 <IP_UE_Y_IPv6_addr_video> m=video <IP_UE_Y_IPv6_port_video> RTP/SAVPF 100 a=rtcp-mux [attributes for media formats, RTP media multipl., …] } …} | The MG acknowledges the modifications of the Stream endpoints. |

##### 9.2.1.4.3	Second transaction cycle: decision to use alternate IP connection

IPv4 is selected. The second H.248 resource reservation cycle completes media configurations and removes unused "ALTC MG resources", see Table 9.2.1.4/3b and Table 9.2.1.4/.4b.

Table 9.2.1.4/3b – Example command encoding – Step 3b – MGC request

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MGC to MG: MEGACO/3 [11.9.19.65]:55555 Transaction = 2 { Context = Cz1 { ; Context CZ1 Modify = ip/y/p/1 { ; Termination TY Media { Stream = 1 { ; SEP S1 LocalControl { ; NOTE 1 ipdc/realm = <realm_Y_IPv4>, ; NOTE 2 Mode = SendRecv, ; NOTE 3 ReservedGroup = OFF, ; NOTE 4 ReservedValue = OFF, ; the default value ... }, Local { ; NOTE 5 v=0 ; selected media group for audio c=IN IP4 <IP_Y,P_IPv4_addr_audio> m=audio <IP_Y,P_IPv4_port_audio> RTP/SAVPF 103 a=rtcp-mux [attributes for media formats, RTP media multipl., …] ; end of "media group" ; no second media group for audio }, Remote { v=0 ; only a single media group for audio c=IN IP4 <IP_UE_Y_IPv4_addr_audio> m=audio <IP_UE_Y_IPv4_port_audio> RTP/SAVPF 103 a=rtcp-mux [attributes for media formats, RTP media multipl., …] } Stream = 2 { ; SEP S2 LocalControl { ipdc/realm = <realm_Y_IPv4>, Mode = Inactive, ReservedGroup = OFF, ; the default value ReservedValue = OFF, ; the default value ... }, Local { ; NOTE 5 v=0 ; selected media group for video c=IN IP4 <IP_Y,P_IPv4_addr_video> m=video <IP_Y,P_IPv4_port_video> RTP/SAVPF 100 a=rtcp-mux [attributes for media formats, RTP media multipl., …] ; end of "media group" ; no second media group for video }, Remote { v=0 ; only a single media group for video c=IN IP4 <IP_UE_Y_IPv4_addr_video> m=video <IP_UE_Y_IPv4_port_video> RTP/SAVPF 100 a=rtcp-mux [attributes for media formats, RTP media multipl., …] } …} | Alternate option is IPv4, i.e., the first H.248 media group remains and the second one is just not included in the transaction request. NOTE 1 – It may be noted that the LCD has to be included again (in contrast to e.g. model M1) due to changed property values NOTE 2 – The IP domain indication is changed now indicating the IPv4 associated realm only.. NOTE 3 – Now, "early" media enabled. NOTE 4 – ReserveGroup disabled again due to selection of a single media group. NOTE 5 – It may be noted that the LD has to be completely included (in contrast to e.g. model M1) with the selected H.248 media group. |

A possible example MG reply is indicated in Table 9.2.1.4/4b.

Table 9.2.1.4/4b – Example command encoding – Step 4b – MG reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MG to MGC: MEGACO/3 [19.65.11.9]:44444 Reply = 1 { Context = Cz1 { ; Context CZ1 Modify = ip/y/p/1 { ; Termination TY Media { Stream = 1 { ; SEP S1 Local { v=0 c=IN IP4 <IP_Y,P_IPv4_addr_audio> m=audio <IP_Y,P_IPv4_port_audio> RTP/SAVPF 103 a=rtcp-mux [attributes for media formats, RTP media multipl., …] }, Remote { v=0 c=IN IP4 <IP_UE_Y_IPv4_addr_audio> m=audio <IP_UE_Y_IPv4_port_audio> RTP/SAVPF 103 a=rtcp-mux [attributes for media formats, RTP media multipl., …] } Stream = 2 { ; SEP S2 Local { v=0 c=IN IP4 <IP_Y,P_IPv4_addr_video> m=video <IP_Y,P_IPv4_port_video> RTP/SAVPF 100 a=rtcp-mux [attributes for media formats, RTP media multipl., …] }, Remote { v=0 c=IN IP4 <IP_UE_Y_IPv4_addr_video> m=video <IP_UE_Y_IPv4_port_video> RTP/SAVPF 100 a=rtcp-mux [attributes for media formats, RTP media multipl., …] } …} | The MG acknowledges the modifications of the Stream endpoints. |

#### 9.2.1.5	Usage of H.248 ALTC Context model "M5" (two Stream Groups)

Per media type there are one aggregation Stream endpoint and two component Stream endpoints representing the IP address types. The association of the aggregation Stream endpoint with these two component Stream endpoints is achieved by setting the mgroup/stragg property in the aggregation Stream endpoint's LocalControl descriptor. The associated aggregation semantic is signalled with the "ANAT" semantic as value of the mgroup/groupse property in the termination's TerminationState descriptor.

The first H.248 resource reservation cycle includes the creation of "ALTC MG resources", see Table 9.2.1.5/1 and Table 9.2.1.5/2.

Table 9.2.1.5/1 – Example command encoding – Step 1 TY.S1 – MGC request 
(for local connection point only in outgoing direction)

| H.248/SDP encoding (shortened H.248 Media Descriptor) | Comments: |
| --- | --- |
| MGC to MG: MEGACO/3 [11.9.19.65]:55555 Transaction = 1 { Context = $ { ; Context CZ1 Add = ip/$/$/$ { ; Termination TY Media{ TerminationState { mgroup/groupse = ["ANAT 3 4"], ; audio group mgroup/groupse = ["ANAT 5 6"]} ; video group }, ; 1st Stream Group Stream = 1 { ; audio stream group LocalControl{ mgroup/stragg = [3, 4] Mode = ReceiveOnly, ; NOTE 4 ReservedGroup = OFF, ; NOTE 5 ReservedValue = OFF }, Local{ v=0 c=- - - … m=audio - RTP/AVP … … } }, ; end of Context-int. aggr. Stream 1 Stream = 3 { ;preferred audio media stream LocalControl{ ipdc/realm = <realm_Y_IPv6>, ; NOTE 3 Mode = ReceiveOnly, ; NOTE 4 ReservedGroup = OFF, ; NOTE 5 ReservedValue = OFF }, Local{ v=0 c=IN IP6 $ m=- $ udp - } }, ; end of Stream 3 Stream = 4 { ;alternate audio media stream LocalControl{ ipdc/realm = <realm_Y_IPv4>, ; NOTE 3 Mode = ReceiveOnly, ; NOTE 4 ReservedGroup = OFF, ; NOTE 5 ReservedValue = OFF }, Local{ v=0 c=IN IP4 $ m=- $ udp - } }, ; end of Stream 4 ; 2nd Stream Group Stream = 2 { ; video stream group LocalControl{ mgroup/stragg = [5, 6], Mode = ReceiveOnly, ; NOTE 4 ReservedGroup = OFF, ; NOTE 5 ReservedValue = OFF }, Local{ v=0 c=- - - … m=video - RTP/AVP … … } }, ; end of Context-int. aggr.Stream 2 Stream = 5 { ;preferred video media stream LocalControl{ ipdc/realm = <realm_Y_IPv6>, ; NOTE 3 Mode = ReceiveOnly, ; NOTE 4 ReservedGroup = OFF, ; NOTE 5 ReservedValue = OFF }, Local{ v=0 c=IN IP6 $ m=- $ udp - } }, ; end of Stream 5 Stream = 6 { ;alternate video media stream LocalControl{ ipdc/realm = <realm_Y_IPv4>, ; NOTE 3 Mode = ReceiveOnly, ; NOTE 4 ReservedGroup = OFF, ; NOTE 5 ReservedValue = OFF }, Local{ v=0 c=IN IP4 $ m=- $ udp - } } ; end of Stream 6 }}}} | The MGC creates transport address agnostic and media type and protocol and media format aware aggregation Stream endpoints TY.S1 and TY.S2. The associated component Stream endpoints TY.S3 , TY.S4 , TY.S5 and TY.S6 are created transport address aware and media type, protocol and media format agnostically. |
| NOTE 1 – IP address realm indication via the ipdc/realm property (see [b-ITU-T H.248.41]). NOTE 2 – The "grouping semantic" ANAT provides equivalent functionality to ALTC. NOTE 3 – The IP address realm indication element is actually mandated in all H.248 profiles for IP-IP media gateways, therefore part of this example here (due to its tight semantical correlation with ALTC). NOTE 4 – Allows to support "early media". NOTE 5 – ReserveGroup is explicitly disabled because not applied for IP transport resources. However, there might be on top multiple application flows, which could lead to the additional definition of multiple H.248 media groups. |  |

A possible example MG reply is indicated in Table 9.2.1.5/2.

Table 9.2.1.5/2 – Example command encoding – Step 2 – MG reply

| H.248/SDP encoding (shortened H.248 Media Descriptor) | Comments: |
| --- | --- |
| MG to MGC: MEGACO/3 [19.65.11.9]:44444 Reply = 1 { Context = Cz1 { ; Context CZ1 Add = ip/y/p/1 { ; Termination TY Media{ ; 1st Stream Group Stream = 1 { ; audio stream group Local{ v=0 c=- - - … m=audio - RTP/AVP … … } }, ; end of Context-int. aggr. Stream 1 Stream = 3 { ;preferred audio media stream Local{ v=0 c=IN IP6 <IP_Y,P_IPv6_addr_audio> m=- <IP_Y,P_IPv6_port_audio> udp - } }, ; end of Stream 3 Stream = 4 { ;alternate audio media stream Local{ v=0 c=IN IP4 <IP_Y,P_IPv4_addr_audio> m=- <IP_Y,P_IPv4_port_audio> udp - } }, ; end of Stream 4 ; 2nd Stream Group Stream = 2 { ; video stream group Local{ v=0 c=- - - … m=video - RTP/AVP … … } }, ; end of Context-int. aggr. Stream 2 Stream = 5 { ;preferred video media stream Local{ v=0 c=IN IP6 <IP_Y,P_IPv6_addr_video> m=- <IP_Y,P_IPv6_port_video> udp - } }, ; end of Stream 5 Stream = 6 { ;alternate video media stream Local{ v=0 c=IN IP4 <IP_Y,P_IPv4_addr_video> m=- <IP_Y,P_IPv4_port_video> udp - } } ; end of Stream 6 }}}} |  |

##### 9.2.1.5.2	Second transaction cycle: decision to use preferred IP connection

The second H.248 resource reservation cycle completes media configurations and removes unused "ALTC MG resources", see Table 9.2.1.5/3a and Table 9.2.1.5/.4a.

Table 9.2.1.5/3a – Example command encoding – Step 3a – MGC request

| H.248/SDP encoding (shortened H.248 Media Descriptor) | Comments: |
| --- | --- |
| MGC to MG: MEGACO/3 [11.9.19.65]:55555 Transaction = 2 { Context = Cz1 { ; Context CZ1 Modify = ip/y/p/1 { ; Termination TY Media{ TerminationState { mgroup/groupse = ["ANAT 3"], ; audio group mgroup/groupse = ["ANAT 5"]} ; video group }, ; 1st Stream Group Stream = 1 { ; audio stream group LocalControl{ mgroup/stragg = [3] Mode = SendReceive, ; NOTE 4 ReservedGroup = OFF, ; NOTE 5 ReservedValue = OFF }, Local{ v=0 c=- - - … m=audio - RTP/AVP … … }, Remote{ v=0 c=- - - … m=audio - RTP/AVP … … } }, ; end of Context-int. aggr. Stream 1 Stream = 3 { ; chosen audio media stream LocalControl{ ipdc/realm = <realm_Y_IPv6>, ; NOTE 3 Mode = SendReceive, ; NOTE 4 ReservedGroup = OFF, ; NOTE 5 ReservedValue = OFF }, Local{ v=0 c=IN IP6 <IP_Y,P_IPv6_addr_audio> m=- <IP_Y,P_IPv6_port_audio> udp - }, Remote{ v=0 c=IN IP6 <UE_Y_IPv6_addr_audio> m=- <UE_Y_IPv6_port_audio> udp - } }, ; end of Stream 3 Stream = 4 { ;alternate audio media stream LocalControl{ ipdc/realm = <realm_Y_IPv4>, ; NOTE 3 ReservedGroup = OFF, ReservedValue = OFF }, Local{}, Remote{} ; empty Local and Remote descriptors in order to delete this Stream endpoint }, ; end of Stream 4 ; 2nd Stream Group Stream = 2 { ; video stream group LocalControl{ mgroup/stragg = [5] Mode = SendReceive, ; NOTE 4 ReservedGroup = OFF, ; NOTE 5 ReservedValue = OFF }, Local{ v=0 c=- - - … m=video - RTP/AVP … … }, Remote{ v=0 c=- - - … m=audio - RTP/AVP … … } }, ; end of Context-int. aggr. Stream 2 Stream = 5 { ; chosen video media stream LocalControl{ ipdc/realm = <realm_Y_IPv6>, ; NOTE 3 Mode = SendRecv, ; NOTE 4 ReservedGroup = OFF, ; NOTE 5 ReservedValue = OFF }, Local{ v=0 c=IN IP6 <IP_Y,P_IPv6_addr_video> m=- <IP_Y,P_IPv6_port_video> udp - }, Remote{ v=0 c=IN IP6 <UE_Y_IPv6_addr_video> m=- <UE_Y_IPv6_port_video> udp - } }, ; end of Stream 5 Stream = 6 { ;alternate video media stream LocalControl{ ipdc/realm = <realm_Y_IPv4>, ; NOTE 3 ReservedGroup = OFF, ReservedValue = OFF }, Local{}, Remote{} ; empty Local and Remote descriptors in order to delete this Stream endpoint } ; end of Stream 6 }}}} |  |
| NOTE 1 – IP address realm indication via the ipdc/realm property (see [b-ITU-T H.248.41]). NOTE 2 – The "grouping semantic" ANAT provides equivalent functionality to ALTC. NOTE 3 – The IP address realm indication element is actually mandated in all H.248 profiles for IP-IP media gateways, therefore part of this example here (due to its tight semantical correlation with ALTC). NOTE 4 – Allows to support "early media". NOTE 5 – ReserveGroup is explicitly disabled because not applied for IP transport resources. However, there might be on top multiple application flows, which could lead to the additional definition of multiple H.248 media groups. |  |

A possible example MG reply is indicated in Table 9.2.1.5/4a.

Table 9.2.1.5/4a – Example command encoding – Step 4a – MG reply

| H.248/SDP encoding (shortened H.248 Media Descriptor) | Comments: |
| --- | --- |
| MG to MGC: MEGACO/3 [19.65.11.9]:44444 Transaction = 2 { Context = Cz1 { ; Context CZ1 Modify = ip/y/p/1 { ; Termination TY Media{ ; 1st Stream Group Stream = 1 { ; audio stream group Local{ v=0 c=- - - … m=audio - RTP/AVP … … }, Remote{ v=0 c=- - - … m=audio - RTP/AVP … … } }, ; end of Context-int. aggr. Stream 1 Stream = 3 { ; chosen audio media stream Local{ v=0 c=IN IP6 <IP_Y,P_IPv6_addr_audio> m=- IP_Y,P_IPv6_port_audio> udp - }, Remote{ v=0 c=IN IP6 <UE_Y_IPv6_addr_audio> m=- <UE_Y_IPv6_port_audio> udp - } }, ; end of Stream 3 Stream = 4 { ;alternate audio media stream Local{}, Remote{} }, ; end of Stream 4 ; 2nd Stream Group Stream = 2 { ; video stream group Local{ v=0 c=- - - … m=video - RTP/AVP … … }, Remote{ v=0 c=- - - … m=audio - RTP/AVP … … } } ; end of Context-int. aggr. Stream 2 Stream = 5 { ; chosen video media stream Local{ v=0 c=IN IP6 <IP_Y,P_IPv6_addr_video> m=- <IP_Y,P_IPv6_port_video> udp - }, Remote{ v=0 c=IN IP6 <UE_Y_IPv6_addr_video> m=- <UE_Y_IPv6_port_video> udp - } }, ; end of Stream 5 Stream = 6 { ;alternate video media stream Local{}, Remote{} } ; end of Stream 6 }}}} |  |

##### 9.2.1.5.3	Second transaction cycle: decision to use alternate IP connection

IPv4 is selected. The second H.248 resource reservation cycle completes media configurations and removes unused "ALTC MG resources", see Table 9.2.1.5/3b and Table 9.2.1.5/.4b.

Table 9.2.1.5/3b – Example command encoding – Step 3b – MGC request

| H.248/SDP encoding (shortened H.248 Media Descriptor) | Comments: |
| --- | --- |
| MGC to MG: MEGACO/3 [11.9.19.65]:55555 Transaction = 2 { Context = Cz1 { ; Context CZ1 Modify = ip/y/p/1 { ; Termination TY Media{ TerminationState { mgroup/groupse = ["ANAT 4"], ; audio group mgroup/groupse = ["ANAT 6"]} ; video group }, ; 1st Stream Group Stream = 1 { ; audio stream group LocalControl{ mgroup/stragg = [4], Mode = SendReceive, ; NOTE 4 ReservedGroup = OFF, ; NOTE 5 ReservedValue = OFF }, Local{ v=0 c=- - - … m=audio - RTP/AVP … … }, Remote{ v=0 c=- - - … m=audio - RTP/AVP … … } }, ; end of Context-int. aggr. Stream 1 Stream = 3 { ;preferred audio media stream LocalControl{ ipdc/realm = <realm_Y_IPv6>, ; NOTE 3 ReservedGroup = OFF, ReservedValue = OFF }, Local{}, Remote{} ; empty Local and Remote descriptors in order to delete this Stream endpoint }, ; end of Stream 3 Stream = 4 { ; chosen audio media stream LocalControl{ ipdc/realm = <realm_Y_IPv4>, ; NOTE 3 Mode = SendRecv, ; NOTE 4 ReservedGroup = OFF, ; NOTE 5 ReservedValue = OFF }, Local{ v=0 c=IN IP4 <IP_Y,P_IPv4_addr_audio> m=- IP_Y,P_IPv4_port_audio> udp - }, Remote{ v=0 c=IN IP4 <UE_Y_IPv4_addr_audio> m=- <UE_Y_IPv4_port_audio> udp - } }, ; end of Stream 4 ; 2nd Stream Group Stream = 2 { ; video stream group LocalControl{ mgroup/stragg = [6], Mode = SendReceive, ; NOTE 4 ReservedGroup = OFF, ; NOTE 5 ReservedValue = OFF }, Local{ v=0 c=- - - … m=video - RTP/AVP … … }, Remote{ v=0 c=- - - … m=audio - RTP/AVP … … } }, ; end of Context-int. aggr. Stream 2 Stream = 5 { ;preferred video media stream LocalControl{ ipdc/realm = <realm_Y_IPv6>, ; NOTE 3 Mode = SendRecv, ; NOTE 4 ReservedGroup = OFF, ; NOTE 5 ReservedValue = OFF }, Local{}, Remote{} ; empty Local and Remote descriptors in order to delete this Stream endpoint }, ; end of Stream 5 Stream = 6 { ; chosen video media stream LocalControl{ ipdc/realm = <realm_Y_IPv4>, ; NOTE 3 ReservedGroup = OFF, ReservedValue = OFF }, Local{ v=0 c=IN IP4 <IP_Y,P_IPv4_addr_video> m=- <IP_Y,P_IPv4_port_video> udp - }, Remote{ v=0 c=IN IP4 <UE_Y_IPv4_addr_video> m=- <UE_Y_IPv4_port_video> udp - } } ; end of Stream 6 }}}} |  |
| NOTE 1 – IP address realm indication via the ipdc/realm property (see [b-ITU-T H.248.41]). NOTE 2 – The "grouping semantic" ANAT provides equivalent functionality to ALTC. NOTE 3 – The IP address realm indication element is actually mandated in all H.248 profiles for IP-IP media gateways, therefore part of this example here (due to its tight semantical correlation with ALTC). NOTE 4 – Allows to support "early media". NOTE 5 – ReserveGroup is explicitly disabled because not applied for IP transport resources. However, there might be on top multiple application flows, which could lead to the additional definition of multiple H.248 media groups. |  |

A possible example MG reply is indicated in Table 9.2.1.5/4b.

Table 9.2.1.5/4b – Example command encoding – Step 4b – MG reply

| H.248/SDP encoding (shortened H.248 Media Descriptor) | Comments: |
| --- | --- |
| MG to MGC: MEGACO/3 [19.65.11.9]:44444 Transaction = 2 { Context = Cz1 { ; Context CZ1 Modify = ip/y/p/1 { ; Termination TY Media{ ; 1st Stream Group Stream = 1 { ; audio stream group Local{ v=0 c=- - - … m=audio - RTP/AVP … … }, Remote{ v=0 c=- - - … m=audio - RTP/AVP … … } }, ; end of Context-int. aggr. Stream 1 Stream = 3 { ;preferred audio media stream Local{}, Remote{} }, ; end of Stream 3 Stream = 4 { ; chosen audio media stream Local{ v=0 c=IN IP4 <IP_Y,P_IPv4_addr_audio> m=- IP_Y,P_IPv4_port_audio> udp - }, Remote{ v=0 c=IN IP4 <UE_Y_IPv4_addr_audio> m=- <UE_Y_IPv4_port_audio> udp - } }, ; end of Stream 4 ; 2nd Stream Group Stream = 2 { ; video stream group Local{ v=0 c=- - - … m=video - RTP/AVP … … }, Remote{ v=0 c=- - - … m=audio - RTP/AVP … … } }, ; end of Context-int. aggr. Stream 2 Stream = 5 { ;preferred video media stream Local{}, Remote{} }, ; end of Stream 5 Stream = 6 { ; chosen video media stream Local{ v=0 c=IN IP4 <IP_Y,P_IPv4_addr_video> m=- <IP_Y,P_IPv4_port_video> udp - }, Remote{ v=0 c=IN IP4 <UE_Y_IPv4_addr_video> m=- <UE_Y_IPv4_port_video> udp - } } ; end of Stream 6 }}}} |  |

#### 9.2.1.6	Usage of H.248 ALTC Context model "M6" (dual stack IP interface)

For further studies.

### 9.2.2	Example H.248 signalling in case of "application agnostic IWF"

#### 9.2.2.1	Usage of H.248 ALTC Context model "M1" (two Contexts)

The usage of an (initial) application agnostic signalling at the ALTC Context side may result in the huge benefit of fairly short media descriptions within the H.248 Stream Descriptors. The gain depends on the list size of media format types, ReserveValue settings and number of media attributes per media format type.

##### 9.2.2.1.1	First transaction cycle: preparation of local IP connection endpoints

The first H.248 resource reservation cycle includes the creation of "ALTC MG resources", see Table 9.2.2.1/1 and Table 9.2.2.1/2.

Table 9.2.2.1/1 – Example command encoding – Step 1 – MGC request

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MGC to MG: MEGACO/3 [11.9.19.65]:55555 Transaction = 1 { Context = $ { ; Context CZ1 Add = ip/$/$/$ { ; Termination TY,P Media { Stream = 1 { ; SEP S1 LocalControl { ipdc/realm = <realm_Y_IPv6>, Mode = Inactive, ; NOTE 1 ReservedGroup = OFF, ; the default value ReservedValue = OFF, ; the default value ... }, Local { v=0 c=IN IP6 $ m=- $ RTP/SAVPF - ; NOTE 3 a=rtcp-mux }, Remote { ; still empty (because SDP answer from UE Y not yet received } Stream = 2 { ; SEP S2 LocalControl { ipdc/realm = <realm_Y_IPv6>, Mode = Inactive, ; NOTE 1 ReservedGroup = OFF, ; the default value ReservedValue = OFF, ; the default value ... }, Local { v=0 c=IN IP6 $ m=- $ RTP/SAVPF - NOTE 3 a=rtcp-mux }, Remote { ; still empty (because SDP answer from UE Y not yet received } … }, Transaction = 2 { Context = $ { ; Context CZ2 Add = ip/$/$/$ { ; Termination TY,A Media { Stream = 1 { ; SEP S1 LocalControl { ipdc/realm = <realm_Y_IPv4>, Mode = Inactive, ; NOTE 2 ReservedGroup = OFF, ; the default value ReservedValue = OFF, ; the default value ... }, Local { v=0 c=IN IP4 $ m=- $ RTP/SAVPF - NOTE 3 a=rtcp-mux }, Remote { ; still empty (because SDP answer from UE Y not yet received } Stream = 2 { ; SEP S2 LocalControl { ipdc/realm = <realm_Y_IPv4>, Mode = Inactive, ; NOTE 2 ReservedGroup = OFF, ; the default value ReservedValue = OFF, ; the default value ... }, Local { v=0 c=IN IP4 $ m=- $ RTP/SAVPF - NOTE 3 a=rtcp-mux }, Remote { ; still empty (because SDP answer from UE Y not yet received } … } | Additional comments in comparison the the application-aware case (see Table 9.2.1.1/1): NOTE 3 – "Application agnostic" results in a configuration that is: - media type agnostic, - media format agnostic and - transport protocol aware. Transport protocol awareness (RTP/SAVPF) is used as indication for RTP media flow and RTCP control flows due to request of RTP / RTCP transport multiplexing. The ReserveValue property would be essentially not effective in case of a TRUE setting. |

A possible example MG reply is indicated in Table 9.2.2.1/2.

Table 9.2.2.1/2 – Example command encoding – Step 2 – MG reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MG to MGC: MEGACO/3 [19.65.11.9]:44444 Reply = 1 { Context = Cz1 { ; Context CZ1 Add = ip/y/p/1 { ; Termination TY,P Media { Stream = 1 { ; SEP S1 Local { v=0 c=IN IP6 <IP_Y,P_IPv6_addr_audio> m=- <IP_Y,P_IPv6_port_audio> RTP/SAVPF - a=rtcp-mux … }, Remote { ; still empty (because SDP answer from UE Y not yet received } Stream = 2 { ; SEP S2 Local { v=0 c=IN IP6 <IP_Y,P_IPv6_addr_video> m=- <IP_Y,P_IPv6_port_video> RTP/SAVPF - a=rtcp-mux … }, Remote { ; still empty (because SDP answer from UE Y not yet received } … }, Reply = 2 { Context = Cz2 { ; Context CZ2 Add = ip/y/a/1 { ; Termination TY,A Media { Stream = 1 { ; SEP S1 Local { v=0 c=IN IP4 <IP_Y,P_IPv4_addr_audio> m=- <IP_Y,P_IPv4_port_audio> RTP/SAVPF - a=rtcp-mux … }, Remote { ; still empty (because SDP answer from UE Y not yet received } Stream = 2 { ; SEP S2 Local { v=0 c=IN IP4 <IP_Y,P_IPv4_addr_video> m=- <IP_Y,P_IPv4_port_video> RTP/SAVPF - a=rtcp-mux }, Remote { ; still empty (because SDP answer from UE Y not yet received } … } |  |

##### 9.2.2.1.2	Second transaction cycle: decision to use preferred IP connection

The second H.248 resource reservation cycle completes media configurations and removes unused "ALTC MG resources", see Table 9.2.2.1/3a and Table 9.2.2.1/.4a.

Table 9.2.2.1/3a – Example command encoding – Step 3a – MGC request

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MGC to MG: MEGACO/3 [11.9.19.65]:55555 Transaction = 3 { Context = Cz1 { ; Context CZ1 Modify = ip/y/p/1 { ; Termination TY,P Media { Stream = 1 { ; SEP S1 LocalControl { ipdc/realm = <realm_Y_IPv6>, Mode = SendRecv, ReservedGroup = OFF, ; the default value ReservedValue = OFF, ; the default value ... }, ; LocalDescriptor don't need to be modified anymore (NOTE 1) Remote { v=0 c=IN IP6 <IP_UE_Y_IPv6_addr_audio> m=- <IP_UE_Y_IPv6_port_audio> RTP/SAVPF - a=rtcp-mux … }, }, Stream = 2 { ; SEP S2 LocalControl { ipdc/realm = <realm_Y_IPv6>, Mode = SendRecv, ReservedGroup = OFF, ; the default value ReservedValue = OFF, ; the default value … }, ; LocalDescriptor don't need to be modified anymore Remote { v=0 c=IN IP6 <IP_UE_Y_IPv6_addr_video> m=- <IP_UE_Y_IPv6_port_video> RTP/SAVPF - a=rtcp-mux } } … }, Transaction = 4 { Context = Cz2 { ; Context CZ2 Subtract = ip/y/a/1 { ; Termination TY,A } } | IPv6 is the selected IP version, hence Termination TY,P is kept and completety specified by addition of the still missing remote destination information (via H.248 MODify command). NOTE 1 – The LD doesn't need to be repeated again because still stored (as it should be) at MG side. And Termination TY,A is removed via H.248 SUBtract. |

A possible example MG reply is indicated in Table 9.2.2.1/4a.

Table 9.2.2.1/4a – Example command encoding – Step 4a – MG reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MG to MGC: MEGACO/3 [19.65.11.9]:44444 Reply = 3 { Context = Cz1 { ; Context CZ1 Modify = ip/y/p/1 { ; Termination TY,P Media { Stream = 1 { ; SEP S1 Local { v=0 c=IN IP6 <IP_Y,P_IPv6_addr_audio> m=- <IP_Y,P_IPv6_port_audio> RTP/SAVPF - a=rtcp-mux … }, Remote { v=0 c=IN IP6 <IP_UE_Y_IPv6_addr_audio> m=- <IP_UE_Y_IPv6_port_audio> RTP/SAVPF - a=rtcp-mux … } }, Stream = 2 { ; SEP S2 Local { v=0 c=IN IP6 <IP_Y,P_IPv6_addr_video> m=video <IP_Y,P_IPv6_port_video> RTP/SAVPF 100 a=rtcp-mux … }, Remote { v=0 c=IN IP6 <IP_UE_Y_IPv6_addr_video> m=video <IP_UE_Y_IPv6_port_video> RTP/SAVPF 100 a=rtcp-mux } …}, Reply = 4 { Context = Cz2 { Subtract = ip/y/a/1 { Statistics { … } } } } |  |

##### 9.2.2.1.3	Second transaction cycle: decision to use alternate IP connection

IPv4 is the selected IP version, hence Termination TY,P is subtracted and replaced by Termination TY,A (using an H.248 MOVE command). See Table 9.2.2.1/3b and Table 9.2.2.1/.4b.

Table 9.2.2.1/3b – Example command encoding – Step 3b – MGC request

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MGC to MG: MEGACO/3 [11.9.19.65]:55555 Transaction = 3 { Context = Cz1 { ; Context CZ1 ; step 1 – removal of TY,P via SUBtract Subtract = ip/y/p/1 { ; Termination TY,P }, ; step 2 – replacement by TY,A via MOVe Move = ip/y/a/1 { ; Termination TY,A Media { Stream = 1 { ; SEP S1 LocalControl { ipdc/realm = <realm_Y_IPv4>, Mode = SendRecv, ReservedGroup = OFF, ; the default value ReservedValue = OFF, ; the default value … }, ; LocalDescriptor don't need to be modified anymore Remote { v=0 c=IN IP4 <IP_UE_Y_IPv4_addr_audio> m=- <IP_UE_Y_IPv4_port_audio> RTP/SAVPF - a=rtcp-mux … } }, Stream = 2 { ; SEP S2 LocalControl { ipdc/realm = <realm_Y_IPv4>, Mode = SendRecv, ReservedGroup = OFF, ; the default value ReservedValue = OFF, ; the default value … }, ; LocalDescriptor don't need to be modified anymore Remote { v=0 c=IN IP4 <IP_UE_Y_IPv4_addr_video> m=- <IP_UE_Y_IPv4_port_video> RTP/SAVPF - a=rtcp-mux } … ## } | IPv4 is the selected IP version, hence Termination TY,P is subtracted and replaced by Termination TY,A (using an H.248 MOVE command). Both H.248 command requests are subject of Context Cz1. It has to be noted that the Stream identifier values in the moved Termination has to be consistent with the removed Termination. |

A possible example MG reply is indicated in Table 9.2.1.1/4b.

Table 9.2.2.1/4b – Example command encoding – Step 4b – MG reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MG to MGC: MEGACO/3 [19.65.11.9]:44444 Reply = 3 { Context = Cz1 { ; Context CZ1 Subtract = ip/y/p/1 { Statistics { … } }, Move = ip/y/a/1 { ; Termination TY,A Media { Stream = 1 { ; SEP S1 Local { v=0 c=IN IP4 <IP_Y,P_IPv4_addr_audio> m=- <IP_Y,P_IPv4_port_audio> RTP/SAVPF - a=rtcp-mux … }, Remote { v=0 c=IN IP4 <IP_UE_Y_IPv4_addr_audio> m=- <IP_UE_Y_IPv4_port_audio> RTP/SAVPF - a=rtcp-mux … }, Stream = 2 { ; SEP S2 Local { v=0 c=IN IP4 <IP_Y,P_IPv4_addr_video> m=- <IP_Y,P_IPv4_port_video> RTP/SAVPF - a=rtcp-mux … }, Remote { v=0 c=IN IP4 <IP_UE_Y_IPv4_addr_video> m=- <IP_UE_Y_IPv4_port_video> RTP/SAVPF - a=rtcp-mux } …} |  |

#### 9.2.2.2	Usage of H.248 ALTC Context model "M2" (single context with two Terminations for ALTC)

##### 9.2.2.2.1	First transaction cycle: preparation of local IP connection endpoints

The option without early media support is selected, i.e., there will be not an early Context internal cut-through of the H.248 Streams. This is achieved be keeping the StreamMode inactive.

The first H.248 resource reservation cycle includes the creation of "ALTC MG resources", see Table 9.2.2.2/1a and Table 9.2.2.2/1b and Table 9.2.2.2/2a and Table 9.2.2.2/2b.

Table 9.2.2.2/1a – Example command encoding – Step 1a – MGC request 
(for local connection point only in outgoing direction)

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MGC to MG: MEGACO/3 [11.9.19.65]:55555 Transaction = 1 { Context = $ { ; Context CZ1 Add = ip/$/$/$ { ; Termination TY,P Media { Stream = 1 { ; SEP S1 LocalControl { ipdc/realm = <realm_Y_IPv6>, Mode = Inactive, ReservedGroup = OFF, ; the default value ReservedValue = OFF, ; the default value ... }, Local { v=0 c=IN IP6 $ m=- $ RTP/SAVPF - a=rtcp-mux }, Remote { ; still empty (because SDP answer from UE Y not yet received } }, Stream = 2 { ; SEP S2 LocalControl { ipdc/realm = <realm_Y_IPv6>, Mode = Inactive, ReservedGroup = OFF, ; the default value ReservedValue = OFF, ; the default value ... }, Local { v=0 c=IN IP6 $ m=- $ RTP/SAVPF - a=rtcp-mux }, Remote { ; still empty (because SDP answer from UE Y not yet received } } } }}} | At first the MGC adds Termination TY,P to the new Context. Termination TY,A is added in a subsequent step (Step 2a) with a topology tuple isolating it from TY,P. |

A possible example MG reply is indicated in Table 9.2.2.2/1b.

Table 9.2.2.2/1b – Example command encoding – Step 1b – MG reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MG to MGC: MEGACO/3 [19.65.11.9]:44444 Reply = 1 { Context = Cz1 { ; Context CZ1 Add = ip/y/p/1 { ; Termination TY,P Media { Stream = 1 { ; SEP S1 Local { v=0 c=IN IP6 <IP_Y,P_IPv6_addr_audio> m=- <IP_Y,P_IPv6_port_audio> RTP/SAVPF - a=rtcp-mux } }, Stream = 2 { ; SEP S2 Local { v=0 c=IN IP6 <IP_Y,P_IPv6_addr_video> m=- <IP_Y,P_IPv6_port_video> RTP/SAVPF - a=rtcp-mux } } } }}} |  |

The MGC adds the alternate Termination to the Context and isolates this from the preferred Termination, see Table 9.2.2.2/2a and Table 9.2.2.2/2b.

Table 9.2.2.2/2a – Example command encoding – Step 2a – MGC request 
(for local connection point only in outgoing direction)

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MGC to MG: MEGACO/3 [11.9.19.65]:55555 Transaction = 1 { Context = $ { ; Context CZ1 Topology {ip/y/p/1, ip/$/$/$, Isolate}, ; 2nd Termination for the same Context Add = ip/$/$/$ { ; Termination TY,A Media { Stream = 1 { ; SEP S1 LocalControl { ipdc/realm = <realm_Y_IPv4>, Mode = Inactive, ReservedGroup = OFF, ; the default value ReservedValue = OFF, ; the default value ... }, Local { v=0 c=IN IP4 $ m=- $ RTP/SAVPF - a=rtcp-mux }, Remote { ; still empty (because SDP answer from UE Y not yet received } }, Stream = 2 { ; SEP S2 LocalControl { ipdc/realm = <realm_Y_IPv4>, Mode = Inactive, ReservedGroup = OFF, ; the default value ReservedValue = OFF, ; the default value ... }, Local { v=0 c=IN IP4 $ m=- $ RTP/SAVPF - a=rtcp-mux }, Remote { ; still empty (because SDP answer from UE Y not yet received } } }}}} | The alternate Termination TY,A is added and is isolated from the preferred Termination TY,P. |

A possible example MG reply is indicated in Table 9.2.2.2/2b.

Table 9.2.2.2/2b – Example command encoding – Step 2b – MG reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MG to MGC: MEGACO/3 [19.65.11.9]:44444 Reply = 1 { Context = Cz1 { ; Context CZ1 ; 2nd Termination for the same Context Add = ip/y/a/1 { ; Termination TY,A Media { Stream = 1 { ; SEP S1 Local { v=0 c=IN IP4 <IP_Y,P_IPv4_addr_audio> m=- <IP_Y,P_IPv4_port_audio> RTP/SAVPF - a=rtcp-mux }, }, Stream = 2 { ; SEP S2 Local { v=0 c=IN IP4 <IP_Y,P_IPv4_addr_video> m=- <IP_Y,P_IPv4_port_video> RTP/SAVPF - a=rtcp-mux } } }}}} |  |

##### 9.2.2.2.2	Second transaction cycle: decision to use preferred IP connection

The second H.248 resource reservation cycle completes media configurations and removes unused "ALTC MG resources", see Table 9.2.2.2/3a and Table 9.2.2.2/.4a:

Table 9.2.2.2/3a – Example command encoding – Step 3a – MGC request

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MGC to MG: MEGACO/3 [11.9.19.65]:55555 Transaction = 2 { Context = Cz1 { ; Context CZ1 Modify = ip/y/p/1 { ; Termination TY,P Media { Stream = 1 { ; SEP S1 LocalControl { ipdc/realm = <realm_Y_IPv6>, Mode = SendReceive, ReservedGroup = OFF, ; the default value ReservedValue = OFF, ; the default value ... }, Remote { v=0 c=IN IP6 <IP_UE_Y_IPv6_addr_audio> m=- <IP_UE_Y_IPv6_port_audio> RTP/SAVPF - a=rtcp-mux } }, Stream = 2 { ; SEP S2 LocalControl { ipdc/realm = <realm_Y_IPv6>, Mode = SendReceive, ReservedGroup = OFF, ; the default value ReservedValue = OFF, ; the default value ... }, Remote { v=0 c=IN IP6 <IP_UE_Y_IPv6_addr_video> m=- <IP_UE_Y_IPv6_port_video> RTP/SAVPF - a=rtcp-mux } } } }, ; 2nd Termination for the same Context Subtract = ip/y/a/1 { ; Termination TY,A } } } | IPv6 is the selected IP version, hence Termination TY,P is kept and completely specified by addition of the still missing remote destination information. The alternate Termination TY,A is removed from the Context. |

A possible example MG reply is indicated in Table 9.2.2.2/4a.

Table 9.2.2.2/4a – Example command encoding – Step 4a – MG reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MG to MGC: MEGACO/3 [19.65.11.9]:44444 Transaction = 2 { Context = Cz1 { ; Context CZ1 Modify = ip/y/p/1 { ; Termination TY,P Media { Stream = 1 { ; SEP S1 Local { v=0 c=IN IP6 <IP_Y,P_IPv6_addr_audio> m=- <IP_Y,P_IPv6_port_audio> RTP/SAVPF - a=rtcp-mux }, Remote { v=0 c=IN IP6 <IP_UE_Y_IPv6_addr_audio> m=- <IP_UE_Y_IPv6_port_audio> RTP/SAVPF - a=rtcp-mux } }, Stream = 2 { ; SEP S2 Local { v=0 c=IN IP6 <IP_Y,P_IPv6_addr_video> m=- <IP_Y,P_IPv6_port_video> RTP/SAVPF - a=rtcp-mux }, Remote { v=0 c=IN IP6 <IP_UE_Y_IPv6_addr_video> m=- <IP_UE_Y_IPv6_port_video> RTP/SAVPF - a=rtcp-mux } } } }, ; 2nd Termination for the same Context Subtract = ip/y/a/1 { ; Termination TY,A Statistics { … } } } } |  |

##### 9.2.2.2.3	Second transaction cycle: decision to use alternate IP connection

IPv4 is the selected IP version, hence Termination TY,P is subtracted. See Table 9.2.2.2/3b and Table 9.2.2.2/.4b.

Table 9.2.2.2/3b – Example command encoding – Step 3b – MGC request

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MGC to MG: MEGACO/3 [11.9.19.65]:55555 Transaction = 2 { Context = Cz1 { ; Context CZ1 Subtract = ip/y/p/1 { ; Termination TY,P }, Modify = ip/y/a/1 { ; Termination TY,A Media { Stream = 1 { ; SEP S1 LocalControl { ipdc/realm = <realm_Y_IPv4>, Mode = SendReceive, ReservedGroup = OFF, ; the default value ReservedValue = OFF, ; the default value ... }, Remote { v=0 c=IN IP6 <IP_UE_Y_IPv4_addr_audio> m=- <IP_UE_Y_IPv4_port_audio> RTP/SAVPF - a=rtcp-mux } } Stream = 2 { ; SEP S2 LocalControl { ipdc/realm = <realm_Y_IPv4>, Mode = SendReceive, ReservedGroup = OFF, ; the default value ReservedValue = OFF, ; the default value ... }, Remote { v=0 c=IN IP6 <IP_UE_Y_IPv4_addr_video> m=- <IP_UE_Y_IPv4_port_video> RTP/SAVPF - a=rtcp-mux } } } } } } | IPv4 is the selected IP version, hence Termination TY,A is kept and completely specified by addition of the still missing remote destination information. The preferred Termination TY,P is removed from the Context. |

A possible example MG reply is indicated in Table 9.2.2.2/4b.

Table 9.2.2.2/4b – Example command encoding – Step 4b – MG reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MG to MGC: MEGACO/3 [19.65.11.9]:44444 Transaction = 2 { Context = Cz1 { ; Context CZ1 Subtract = ip/y/p/1 { ; Termination TY,P Statistics { … } }, Modify = ip/y/a/1 { ; Termination TY,A Media { Stream = 1 { ; SEP S1 Local { v=0 c=IN IP6 <IP_Y,P_IPv4_addr_audio> m=- <IP_Y,P_IPv4_port_audio> RTP/SAVPF - a=rtcp-mux }, Remote { v=0 c=IN IP6 <IP_UE_Y_IPv4_addr_audio> m=- <IP_UE_Y_IPv4_port_audio> RTP/SAVPF - a=rtcp-mux } }, Stream = 2 { ; SEP S2 Local { v=0 c=IN IP6 <IP_Y,P_IPv4_addr_video> m=- <IP_Y,P_IPv4_port_video> RTP/SAVPF - a=rtcp-mux }, Remote { v=0 c=IN IP6 <IP_UE_Y_IPv4_addr_video> m=- <IP_UE_Y_IPv4_port_video> RTP/SAVPF - a=rtcp-mux } } } } } } |  |

#### 9.2.2.3	Usage of H.248 ALTC Context model "M3" (single Termination with two SEPs for ALTC)

FIXTHIS

The first H.248 resource reservation cycle includes the creation of "ALTC MG resources", see Table 9.2.2.3/1 and Table 9.2.2.3/2.

Table 9.2.2.3/1 – Example command encoding – Step 1 – MGC request

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| FIXTHIS | FIXTHIS |

A possible example MG reply is indicated in Table 9.2.2.3/2.

Table 9.2.2.3/2 – Example command encoding – Step 2 – MG reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| FIXTHIS | FIXTHIS |

The second H.248 resource reservation cycle completes media configurations and removes unused "ALTC MG resources", see Table 9.2.2.3/3 and Table 9.2.2.3/.4.

Table 9.2.2.3/3 – Example command encoding – Step 3 – MGC request

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| FIXTHIS | FIXTHIS |

A possible example MG reply is indicated in Table 9.2.2.3/4.

Table 9.2.2.3/4 – Example command encoding – Step 4 – MG reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| FIXTHIS | FIXTHIS |

#### 9.2.2.4	Usage of H.248 ALTC Context model "M4" (two media groups for ALTC)

There are two media groups (preferred media group and alternate media group) required per media type (audio and video). Such a resource reservation is achieved by a setting of the LocalControl properties combination of {ReserveValue = "False" AND ReserveGroup = "True"}.

The first H.248 resource reservation cycle includes the creation of "ALTC MG resources", see Table 9.2.2.4/1 and Table 9.2.2.4/2.

Table 9.2.2.4/1 – Example command encoding – Step 1 – MGC request

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MGC to MG: MEGACO/3 [11.9.19.65]:55555 Transaction = 1 { Context = $ { ; Context CZ1 Add = ip/$/$/$ { ; Termination TY Media { Stream = 1 { ; SEP S1 LocalControl { ipdc/realm = [<realm_Y_IPv6>, <realm_Y_IPv4>], Mode = Inactive, ReservedGroup = ON, ; two media groups! ReservedValue = OFF, ; the default value ... }, Local { v=0 ; 1st media group for audio c=IN IP6 $ m=- $ RTP/SAVPF - a=rtcp-mux ; end of 1st media group v=0 ; 2nd media group for audio c=IN IP4 $ m=- $ RTP/SAVPF - a=rtcp-mux … ; end of 2nd media group }, Remote { ; still empty (because SDP answer from UE Y not yet received } }, Stream = 2 { ; SEP S2 LocalControl { ipdc/realm = [<realm_Y_IPv6>, <realm_Y_IPv4>], Mode = Inactive, ReservedGroup = ON, ; two media groups ReservedValue = OFF, ; the default value ... }, Local { v=0 ; 1st media group for video c=IN IP6 $ m=- $ RTP/SAVPF - a=rtcp-mux ; end of 1st media group v=0 ; 2nd media group for video c=IN IP4 $ m=- $ RTP/SAVPF - a=rtcp-mux ; end of 2nd media group }, Remote { ; still empty (because SDP answer from UE Y not yet received } … } |  |

A possible example MG reply is indicated in Table 9.2.2.4/2.

Table 9.2.2.4/2 – Example command encoding – Step 2 – MG reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MG to MGC: MEGACO/3 [19.65.11.9]:44444 Reply = 1 { Context = Cz1 { ; Context CZ1 Add = ip/y/p/1 { ; Termination TY Media { Stream = 1 { ; SEP S1 Local { v=0 ; 1st media group for audio c=IN IP6 <IP_Y,P_IPv6_addr_audio> m=- <IP_Y,P_IPv6_port_audio> RTP/SAVPF - a=rtcp-mux ; end of 1st media group v=0 ; 2nd media group for audio c=IN IP4 <IP_Y,P_IPv4_addr_audio> m=- <IP_Y,P_IPv4_port_audio> RTP/SAVPF - a=rtcp-mux ; end of 2nd media group }, Remote { ; still empty (because SDP answer from UE Y not yet received } }, Stream = 2 { ; SEP S2 Local { v=0 ; 1st media group for video c=IN IP6 <IP_Y,P_IPv6_addr_video> m=- <IP_Y,P_IPv6_port_video> RTP/SAVPF - a=rtcp-mux ; end of 1st media group v=0 ; 2nd media group for video c=IN IP4 <IP_Y,P_IPv4_addr_video> m=- <IP_Y,P_IPv4_port_video> RTP/SAVPF - a=rtcp-mux ; end of 2nd media group }, Remote { ; still empty (because SDP answer from UE Y not yet received } } …} |  |

##### 9.2.2.4.2	Second transaction cycle: decision to use preferred IP connection

The second H.248 resource reservation cycle completes media configurations and removes unused "ALTC MG resources", see Table 9.2.2.4/3a and Table 9.2.2.4/.4a.

Table 9.2.2.4/3a – Example command encoding – Step 3a – MGC request

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MGC to MG: MEGACO/3 [11.9.19.65]:55555 Transaction = 2 { Context = Cz1 { ; Context CZ1 Modify = ip/y/p/1 { ; Termination TY Media { Stream = 1 { ; SEP S1 LocalControl { ; NOTE 1 ipdc/realm = <realm_Y_IPv6>, ; NOTE 2 Mode = SendRecv, ; NOTE 3 ReservedGroup = OFF, ; NOTE 4 ReservedValue = OFF, ; the default value ... }, Local { ; NOTE 5 v=0 ; selected media group for audio c=IN IP6 <IP_Y,P_IPv6_addr_audio> m=- <IP_Y,P_IPv6_port_audio> RTP/SAVPF - a=rtcp-mux ; end of "media group" ; no second media group for audio }, Remote { v=0 ; only a single media group for audio c=IN IP6 <IP_UE_Y_IPv6_addr_audio> m=- <IP_UE_Y_IPv6_port_audio> RTP/SAVPF - a=rtcp-mux } }, Stream = 2 { ; SEP S2 LocalControl { ipdc/realm = <realm_Y_IPv6>, Mode = Inactive, ReservedGroup = OFF, ; the default value ReservedValue = OFF, ; the default value ... }, Local { ; NOTE 5 v=0 ; selected media group for video c=IN IP6 <IP_Y,P_IPv6_addr_video> m=- <IP_Y,P_IPv6_port_video> RTP/SAVPF - a=rtcp-mux ; end of "media group" ; no second media group for video }, Remote { v=0 ; only a single media group for video c=IN IP6 <IP_UE_Y_IPv6_addr_video> m=- <IP_UE_Y_IPv6_port_video> RTP/SAVPF - a=rtcp-mux } …} | Preferred option is IPv6, i.e., the first H.248 media group remains and the second one is just not included in the transaction request. NOTE 1 – It may be noted that the LCD has to be included again (in contrast to e.g. model M1) due to changed property values NOTE 2 – The IP domain indication is changed now indicating the IPv6 associated realm only. NOTE 3 – Now, "early" media enabled (TBC). NOTE 4 – ReserveGroup disabled again due to selection of a single media group. NOTE 5 – It may be noted that the LD has to be completely included (in contrast to e.g. model M1) with the selected H.248 media group. |

A possible example MG reply is indicated in Table 9.2.2.4/4a.

Table 9.2.2.4/4a – Example command encoding – Step 4a – MG reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MG to MGC: MEGACO/3 [19.65.11.9]:44444 Reply = 1 { Context = Cz1 { ; Context CZ1 Modify = ip/y/p/1 { ; Termination TY Media { Stream = 1 { ; SEP S1 Local { v=0 c=IN IP6 <IP_Y,P_IPv6_addr_audio> m=- <IP_Y,P_IPv6_port_audio> RTP/SAVPF - a=rtcp-mux }, Remote { v=0 c=IN IP6 <IP_UE_Y_IPv6_addr_audio> m=- <IP_UE_Y_IPv6_port_audio> RTP/SAVPF - a=rtcp-mux } Stream = 2 { ; SEP S2 Local { v=0 c=IN IP6 <IP_Y,P_IPv6_addr_video> m=- <IP_Y,P_IPv6_port_video> RTP/SAVPF - a=rtcp-mux }, Remote { v=0 c=IN IP6 <IP_UE_Y_IPv6_addr_video> m=- <IP_UE_Y_IPv6_port_video> RTP/SAVPF - a=rtcp-mux } …} |  |

##### 9.2.2.4.3	Second transaction cycle: decision to use alternate IP connection

IPv4 is selected. The second H.248 resource reservation cycle completes media configurations and removes unused "ALTC MG resources", see Table 9.2.2.4/3b and Table 9.2.2.4/.4b.

Table 9.2.2.4/3b – Example command encoding – Step 3b – MGC request

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MGC to MG: MEGACO/3 [11.9.19.65]:55555 Transaction = 2 { Context = Cz1 { ; Context CZ1 Modify = ip/y/p/1 { ; Termination TY Media { Stream = 1 { ; SEP S1 LocalControl { ; NOTE 1 ipdc/realm = <realm_Y_IPv4>, ; NOTE 2 Mode = SendRecv, ; NOTE 3 ReservedGroup = OFF, ; NOTE 4 ReservedValue = OFF, ; the default value ... }, Local { ; NOTE 5 v=0 ; selected media group for audio c=IN IP4 <IP_Y,P_IPv4_addr_audio> m=- <IP_Y,P_IPv4_port_audio> RTP/SAVPF - a=rtcp-mux ; end of "media group" ; no second media group for audio }, Remote { v=0 ; only a single media group for audio c=IN IP4 <IP_UE_Y_IPv4_addr_audio> m=- <IP_UE_Y_IPv4_port_audio> RTP/SAVPF - a=rtcp-mux } Stream = 2 { ; SEP S2 LocalControl { ipdc/realm = <realm_Y_IPv4>, Mode = Inactive, ReservedGroup = OFF, ; the default value ReservedValue = OFF, ; the default value ... }, Local { ; NOTE 5 v=0 ; selected media group for video c=IN IP4 <IP_Y,P_IPv4_addr_video> m=- <IP_Y,P_IPv4_port_video> RTP/SAVPF - a=rtcp-mux ; end of "media group" ; no second media group for video }, Remote { v=0 ; only a single media group for video c=IN IP4 <IP_UE_Y_IPv4_addr_video> m=- <IP_UE_Y_IPv4_port_video> RTP/SAVPF - a=rtcp-mux } …} | Alternate option is IPv4, i.e., the first H.248 media group remains and the second one is just not included in the transaction request. NOTE 1 – It may be noted that the LCD has to be included again (in contrast to e.g. model M1) due to changed property values NOTE 2 – The IP domain indication is changed now indicating the IPv4 associated realm only.. NOTE 3 – Now, "early" media enabled (TBC). NOTE 4 – ReserveGroup disabled again due to selection of a single media group. NOTE 5 – It may be noted that the LD has to be completely included (in contrast to e.g. model M1) with the selected H.248 media group. |

A possible example MG reply is indicated in Table 9.2.2.4/4b.

Table 9.2.2.4/4b – Example command encoding – Step 4b – MG reply

| ITU-T H.248 encoding (shortened command) | Comments |
| --- | --- |
| MG to MGC: MEGACO/3 [19.65.11.9]:44444 Reply = 1 { Context = Cz1 { ; Context CZ1 Modify = ip/y/p/1 { ; Termination TY Media { Stream = 1 { ; SEP S1 Local { v=0 c=IN IP4 <IP_Y,P_IPv4_addr_audio> m=- <IP_Y,P_IPv4_port_audio> RTP/SAVPF - a=rtcp-mux }, Remote { v=0 c=IN IP4 <IP_UE_Y_IPv4_addr_audio> m=- <IP_UE_Y_IPv4_port_audio> RTP/SAVPF - a=rtcp-mux } Stream = 2 { ; SEP S2 Local { v=0 c=IN IP4 <IP_Y,P_IPv4_addr_video> m=- <IP_Y,P_IPv4_port_video> RTP/SAVPF - a=rtcp-mux }, Remote { v=0 c=IN IP4 <IP_UE_Y_IPv4_addr_video> m=- <IP_UE_Y_IPv4_port_video> RTP/SAVPF - a=rtcp-mux } …} | The MG acknowledges the modifications of the Stream endpoints. |

#### 9.2.2.5	Usage of H.248 ALTC Context model "M5" (two Stream Groups)

Per media type there are one aggregation Stream endpoint and two component Stream endpoints representing the IP address types. The association of the aggregation Stream endpoint with these two component Stream endpoints is achieved by setting the mgroup/stragg property in the aggregation Stream endpoint's LocalControl descriptor. The associated aggregation semantic is signalled with the "ANAT" semantic as value of the mgroup/groupse property in the termination's TerminationState descriptor.

The first H.248 resource reservation cycle includes the creation of "ALTC MG resources", see Table 9.2.2.5/1 and Table 9.2.2.5/2.

Table 9.2.2.5/1 – Example command encoding – Step 1 – MGC request 
(for local connection point only in outgoing direction)

| H.248/SDP encoding (shortened H.248 Media Descriptor) | Comments: |
| --- | --- |
| MGC to MG: MEGACO/3 [11.9.19.65]:55555 Transaction = 1 { Context = $ { ; Context CZ1 Add = ip/$/$/$ { ; Termination TY Media{ TerminationState { mgroup/groupse = ["ANAT 3 4"], ; audio group mgroup/groupse = ["ANAT 5 6"]} ; video group }, ; 1st Stream Group Stream = 1 { ; audio stream group LocalControl{ mgroup/stragg = [3, 4], Mode = ReceiveOnly, ; NOTE 4 ReservedGroup = OFF, ; NOTE 5 ReservedValue = OFF }, Local{ v=0 c=- - - m=audio - RTP/AVP - } } ; end of Context-int. aggr. Stream 1 Stream = 3 { ;preferred audio media stream LocalControl { ipdc/realm = <realm_Y_IPv6>, ; NOTE 3 Mode = SendRecv, ; NOTE 4 ReservedGroup = OFF, ; NOTE 5 ReservedValue = OFF }, Local { v=0 c=IN IP6 $ m=- $ udp - } }, ; end of Stream 3 Stream = 4 { ;alternate audio media stream LocalControl { ipdc/realm = <realm_Y_IPv4>, ; NOTE 3 Mode = ReceiveOnly, ; NOTE 4 ReservedGroup = OFF, ; NOTE 5 ReservedValue = OFF }, Local{ v=0 c=IN IP4 $ m=- $ udp - } }, ; end of Stream 4 ; 2nd Stream Group Stream = 2 { ; video stream group LocalControl{ mgroup/stragg = [5, 6], Mode = ReceiveOnly, ; NOTE 4 ReservedGroup = OFF, ; NOTE 5 ReservedValue = OFF }, Local{ v=0 c=- - - m=video - RTP/AVP - } } ; end of Context-int. aggr.Stream 2 Stream = 5 { ;preferred video media stream LocalControl{ ipdc/realm = <realm_Y_IPv6>, ; NOTE 3 Mode = SendRecv, ; NOTE 4 ReservedGroup = OFF, ; NOTE 5 ReservedValue = OFF }, Local{ v=0 c=IN IP6 $ m=- $ udp - } }, ; end of Stream 5 Stream = 6 { ;alternate video media stream LocalControl{ ipdc/realm = <realm_Y_IPv4>, ; NOTE 3 Mode = ReceiveOnly, ; NOTE 4 ReservedGroup = OFF, ; NOTE 5 ReservedValue = OFF}, Local{ v=0 c=IN IP4 $ m=- $ udp - } } ; end of Stream 6 }}}} | The MGC creates transport address agnostic and media type and protocol aware and media format agnostic aggregation Stream endpoints TY.S1 and TY.S2. The associated component Stream endpoints TY.S3 , TY.S4 , TY.S5 and TY.S6 are created transport address aware and media type, protocol and media format agnostically. |
| NOTE 1 – IP address realm indication via the ipdc/realm property (see [b-ITU-T H.248.41]). NOTE 2 – The "grouping semantic" ANAT provides equivalent functionality to ALTC. NOTE 3 – The IP address realm indication element is actually mandated in all H.248 profiles for IP-IP media gateways, therefore part of this example here (due to its tight semantical correlation with ALTC). NOTE 4 – Allows to support "early media". NOTE 5 – ReserveGroup is explicitly disabled because not applied for IP transport resources. However, there might be on top multiple application flows, which could lead to the additional definition of multiple H.248 media groups (feasibility to be confirmed). |  |

A possible example MG reply is indicated in Table 9.2.1.5/2.

Table 9.2.1.5/2 – Example command encoding – Step 2 – MG reply

| H.248/SDP encoding (shortened H.248 Media Descriptor) | Comments: |
| --- | --- |
| MG to MGC: MEGACO/3 [19.65.11.9]:44444 Reply = 1 { Context = Cz1 { ; Context CZ1 Add = ip/y/p/1 { ; Termination TY Media{ ; 1st Stream Group Stream = 1 { ; audio stream group Local{ v=0 c=- - - m=audio - RTP/AVP - } } ; end of Context-int. aggr. Stream 1 Stream = 3 { ;preferred audio media stream Local{ v=0 c=IN IP6 <IP_Y,P_IPv6_addr_audio> m=- <IP_Y,P_IPv6_port_audio> udp - } }, ; end of Stream 3 Stream = 4 { ;alternate audio media stream Local{ v=0 c=IN IP4 <IP_Y,P_IPv4_addr_audio> m=- <IP_Y,P_IPv4_port_audio> udp - } }, ; end of Stream 4 ; 2nd Stream Group Stream = 2 { ; video stream group Local{ v=0 c=- - - m=video - RTP/AVP - } } ; end of Context-int. aggr. Stream 2 Stream = 5 { ;preferred video media stream Local{ v=0 c=IN IP6 <IP_Y,P_IPv6_addr_video> m=- <IP_Y,P_IPv6_port_video> udp - } }, ; end of Stream 5 Stream = 6 { ;alternate video media stream Local{ v=0 c=IN IP4 <IP_Y,P_IPv4_addr_video> m=- <IP_Y,P_IPv4_port_video> udp - } } ; end of Stream 6 }}}} |  |

##### 9.2.2.5.2	Second transaction cycle: decision to use preferred IP connection

The second H.248 resource reservation cycle completes media configurations and removes unused "ALTC MG resources", see Table 9.2.2.5/3a and Table 9.2.2.5/.4a.

Table 9.2.2.5/3a – Example command encoding – Step 3a – MGC request

| H.248/SDP encoding (shortened H.248 Media Descriptor) | Comments: |
| --- | --- |
| MGC to MG: MEGACO/3 [11.9.19.65]:55555 Transaction = 2 { Context = Cz1 { ; Context CZ1 Modify = ip/y/p/1 { ; Termination TY Media{ TerminationState { mgroup/groupse = ["ANAT 3"], ; audio group mgroup/groupse = ["ANAT 5"]} ; video group }, ; 1st Stream Group Stream = 1 { ; audio stream group LocalControl{ mgroup/stragg = [3], Mode = SendReceive, ; NOTE 4 ReservedGroup = OFF, ; NOTE 5 ReservedValue = OFF }, Local { v=0 c=- - - m=audio - RTP/AVP - }, Remote { v=0 c=- - - m=audio - RTP/AVP - } } ; end of Context-int. aggr. Stream 1 Stream = 3 { ; chosen audio media stream LocalControl{ ipdc/realm = <realm_Y_IPv6>, ; NOTE 3 Mode = SendReceive, ; NOTE 4 ReservedGroup = OFF, ; NOTE 5 ReservedValue = OFF }, Local{ v=0 c=IN IP6 <IP_Y,P_IPv6_addr_audio> m=- <IP_Y,P_IPv6_port_audio> udp - }, Remote{ v=0 c=IN IP6 <UE_Y_IPv6_addr_audio> m=- <UE_Y_IPv6_port_audio> udp - } }, ; end of Stream 3 Stream = 4 { ;alternate audio media stream LocalControl{ ipdc/realm = <realm_Y_IPv4>, ; NOTE 3 ReservedGroup = OFF, ReservedValue = OFF}, Local{}, Remote{} ; empty Local and Remote descriptors in order to delete this Stream endpoint }, ; end of Stream 4 ; 2nd Stream Group Stream = 2 { ; video stream group LocalControl{ mgroup/stragg = [5], Mode = SendReceive, ; NOTE 4 ReservedGroup = OFF, ; NOTE 5 ReservedValue = OFF }, Local { v=0 c=- - - m=video - RTP/AVP - }, Remote { v=0 c=- - - m=video - RTP/AVP - } }, ; end of Context-int. aggr. Stream 2 Stream = 5 { ; chosen video media stream LocalControl { ipdc/realm = <realm_Y_IPv6>, ; NOTE 3 Mode = SendReceive, ; NOTE 4 ReservedGroup = OFF, ; NOTE 5 ReservedValue = OFF }, Local{ v=0 c=IN IP6 <IP_Y,P_IPv6_addr_video> m=- <IP_Y,P_IPv6_port_video> udp - }, Remote{ v=0 c=IN IP6 <UE_Y_IPv6_addr_video> m=- <UE_Y_IPv6_port_video> udp - } }, ; end of Stream 5 Stream = 6 { ;alternate video media stream LocalControl{ ipdc/realm = <realm_Y_IPv4>, ; NOTE 3 ReservedGroup = OFF, ReservedValue = OFF }, Local{}, Remote{} ; empty Local and Remote descriptors in order to delete this Stream endpoint } ; end of Stream 6 }}}} |  |
| NOTE 1 – IP address realm indication via the ipdc/realm property (see [b-ITU-T H.248.41]). NOTE 2 – The "grouping semantic" ANAT provides equivalent functionality to ALTC. NOTE 3 – The IP address realm indication element is actually mandated in all H.248 profiles for IP-IP media gateways, therefore part of this example here (due to its tight semantical correlation with ALTC). NOTE 4 – Allows to support "early media". NOTE 5 – ReserveGroup is explicitly disabled because not applied for IP transport resources. However, there might be on top multiple application flows, which could lead to the additional definition of multiple H.248 media groups (feasibility to be confirmed). |  |

A possible example MG reply is indicated in Table 9.2.2.5/4a.

Table 9.2.2.5/4a – Example command encoding – Step 4a – MG reply

| H.248/SDP encoding (shortened H.248 Media Descriptor) | Comments: |
| --- | --- |
| MG to MGC: MEGACO/3 [19.65.11.9]:44444 Transaction = 2 { Context = Cz1 { ; Context CZ1 Modify = ip/y/p/1 { ; Termination TY Media{ ; 1st Stream Group Stream = 1 { ; audio stream group Local { v=0 c=- - - m=audio - RTP/AVP - }, Remote { v=0 c=- - - m=audio - RTP/AVP - } }, ; end of Context-int. aggr. Stream 1 Stream = 3 { ; chosen audio media stream Local{ v=0 c=IN IP6 <IP_Y,P_IPv6_addr_audio> m=- IP_Y,P_IPv6_port_audio> udp - }, Remote{ v=0 c=IN IP6 <UE_Y_IPv6_addr_audio> m=- <UE_Y_IPv6_port_audio> udp - } }, ; end of Stream 3 Stream = 4 { ;alternate audio media stream Local { v=0 c=- - - m=video - RTP/AVP - }, Remote { v=0 c=- - - m=video - RTP/AVP - } }, ; end of Stream 4 ; 2nd Stream Group Stream = 2 { ; video stream group Local{}, Remote{} } ; end of Context-int. aggr. Stream 2 Stream = 5 { ; chosen video media stream Local{ v=0 c=IN IP6 <IP_Y,P_IPv6_addr_video> m=- <IP_Y,P_IPv6_port_video> udp - }, Remote{ v=0 c=IN IP6 <UE_Y_IPv6_addr_video> m=- <UE_Y_IPv6_port_video> udp - } }, ; end of Stream 5 Stream = 6 { ;alternate video media stream Local{}, Remote{} } ; end of Stream 6 }}}} |  |

##### 9.2.2.5.3	Second transaction cycle: decision to use alternate IP connection

IPv4 is selected. The second H.248 resource reservation cycle completes media configurations and removes unused "ALTC MG resources", see Table 9.2.2.5/3b and Table 9.2.2.5/.4b.

Table 9.2.2.5/3b – Example command encoding – Step 3b – MGC request

| H.248/SDP encoding (shortened H.248 Media Descriptor) | Comments: |
| --- | --- |
| MGC to MG: MEGACO/3 [11.9.19.65]:55555 Transaction = 2 { Context = Cz1 { ; Context CZ1 Modify = ip/y/p/1 { ; Termination TY Media{ TerminationState { mgroup/groupse = ["ANAT 4"], ; audio group mgroup/groupse = ["ANAT 6"]} ; video group }, ; 1st Stream Group Stream = 1 { ; audio stream group LocalControl{ mgroup/stragg = [4], Mode = SendReceive, ; NOTE 4 ReservedGroup = OFF, ; NOTE 5 ReservedValue = OFF }, Local { v=0 c=- - - m=audio - RTP/AVP - }, Remote { v=0 c=- - - m=audio - RTP/AVP - } }, ; end of Context-int. aggr. Stream 1 Stream = 3 { ;preferred audio media stream LocalControl{ ipdc/realm = <realm_Y_IPv6>, ; NOTE 3 ReservedGroup = OFF, ReservedValue = OFF }, Local{}, Remote{} ; empty Local and Remote descriptors in order to delete this Stream endpoint }, ; end of Stream 3 Stream = 4 { ; chosen audio media stream LocalControl{ ipdc/realm = <realm_Y_IPv4>, ; NOTE 3 Mode = SendReceive, ; NOTE 4 ReservedGroup = OFF, ; NOTE 5 ReservedValue = OFF }, Local{ v=0 c=IN IP4 <IP_Y,P_IPv4_addr_audio> m=- IP_Y,P_IPv4_port_audio> udp - }, Remote{ v=0 c=IN IP4 <UE_Y_IPv4_addr_audio> m=- <UE_Y_IPv4_port_audio> udp - } }, ; end of Stream 4 ; 2nd Stream Group Stream = 2 { ; video stream group LocalControl{ mgroup/stragg = [6], Mode = SendReceive, ; NOTE 4 ReservedGroup = OFF, ; NOTE 5 ReservedValue = OFF }, Local { v=0 c=- - - m=video - RTP/AVP - }, Remote { v=0 c=- - - m=video - RTP/AVP - } }, ; end of Context-int. aggr. Stream 2 Stream = 5 { ;preferred video media stream LocalControl { ipdc/realm = <realm_Y_IPv6>, ; NOTE 3 Mode = SendReceive, ; NOTE 4 ReservedGroup = OFF, ; NOTE 5 ReservedValue = OFF }, Local{}, Remote{} ; empty Local and Remote descriptors in order to delete this Stream endpoint }, ; end of Stream 5 Stream = 6 { ; chosen video media stream LocalControl{ ipdc/realm = <realm_Y_IPv4>, ; NOTE 3 Mode = SendReceive, ; NOTE 4 ReservedGroup = OFF, ReservedValue = OFF }, Local{ v=0 c=IN IP4 <IP_Y,P_IPv4_addr_video> m=- <IP_Y,P_IPv4_port_video> udp - }, Remote{ v=0 c=IN IP4 <UE_Y_IPv4_addr_video> m=- <UE_Y_IPv4_port_video> udp - } } ; end of Stream 6 }}}} |  |
| NOTE 1 – IP address realm indication via the ipdc/realm property (see [b-ITU-T H.248.41]). NOTE 2 – The "grouping semantic" ANAT provides equivalent functionality to ALTC. NOTE 3 – The IP address realm indication element is actually mandated in all H.248 profiles for IP-IP media gateways, therefore part of this example here (due to its tight semantical correlation with ALTC). NOTE 4 – Allows to support "early media". NOTE 5 – ReserveGroup is explicitly disabled because not applied for IP transport resources. However, there might be on top multiple application flows, which could lead to the additional definition of multiple H.248 media groups (feasibility to be confirmed). |  |

A possible example MG reply is indicated in Table 9.2.1.5/4b.

Table 9.2.1.5/4b – Example command encoding – Step 4b – MG reply

| H.248/SDP encoding (shortened H.248 Media Descriptor) | Comments: |
| --- | --- |
| MG to MGC: MEGACO/3 [19.65.11.9]:44444 Transaction = 2 { Context = Cz1 { ; Context CZ1 Modify = ip/y/p/1 { ; Termination TY Media{ ; 1st Stream Group Stream = 1 { ; audio stream group Local { v=0 c=- - - m=audio - RTP/AVP - }, Remote { v=0 c=- - - m=audio - RTP/AVP - } }, ; end of Context-int. aggr. Stream 1 Stream = 3 { ;preferred audio media stream Local{}, Remote{} }, ; end of Stream 3 Stream = 4 { ; chosen audio media stream Local{ v=0 c=IN IP4 <IP_Y,P_IPv4_addr_audio> m=- IP_Y,P_IPv4_port_audio> udp - }, Remote{ v=0 c=IN IP4 <UE_Y_IPv4_addr_audio> m=- <UE_Y_IPv4_port_audio> udp - } }, ; end of Stream 4 ; 2nd Stream Group Stream = 2 { ; video stream group Local { v=0 c=- - - m=video - RTP/AVP - }, Remote { v=0 c=- - - m=video - RTP/AVP - } }, ; end of Context-int. aggr. Stream 2 Stream = 5 { ;preferred video media stream Local{}, Remote{} }, ; end of Stream 5 Stream = 6 { ; chosen video media stream Local{ v=0 c=IN IP4 <IP_Y,P_IPv4_addr_video> m=- <IP_Y,P_IPv4_port_video> udp - }, Remote{ v=0 c=IN IP4 <UE_Y_IPv4_addr_video> m=- <UE_Y_IPv4_port_video> udp - } } ; end of Stream 6 }}}} |  |

# 10	Profile guidelines

This clause provides guidelines for H.248 profile specifications. The structure follows the profile template according to Appendix III/[ITU-T H.248.1].

The template elements which are not applicable in this Recommendation are indicated by "Subject to profile specification".

Any profile guidelines are primarily dependent on the concerned network configuration and use case. The guidelines in this clause are therefore basically conditional. Some exemplary use cases are considered (as described in Appendix IV), termed as capability sets CSA and CSB. Tag 'CS*' indicates unconditional profile elements, which are either generic because basic "ALTC capabilities" or required for all CS examples.

## 10.1	Profile Identification

Subject to profile specification.

## 10.2	Summary

Subject to profile specification.

## 10.3	Gateway Control Protocol Version

Subject to profile specification.

## 10.4	Connection Model

| Maximum number of Contexts: | Subject to profile specification. |
| --- | --- |
| Maximum number of Terminations per Context: | Subject to profile specification. Examples: IF CSA OR CSB THEN "2". NOTE – The "multiple Termination" based method (M2, see clause 7.1.2) would require a profile entry of "3". |
| Allowed Termination type combinations in a Context: | Subject to profile specification. |

## 10.5	Context Attributes

Subject to profile specification.

## 10.6	Terminations

Subject to profile specification.

## 10.7	Descriptors

### 10.7.1	TerminationState Descriptor

IF CSB THEN following table:

| TerminationState: Groupe Semantics (mgroup/groupse) | See clause 10.14.3.10. |
| --- | --- |
| TerminationState: … | … |

All other aspects related to TerminationState Descriptor (e.g., ServiceState, EventBuffer Control) are "Subject to profile specification".

### 10.7.2	Stream Descriptor

Subject to profile specification.

IF CSB THEN the H.248 IP Termination for service ALTC would carry additional H.248 aggregation streams besides the legacy H.248 component streams. There are then additional, dedicated Stream Descriptors at aggregation stream level.

### 10.7.3	Events Descriptor

Subject to profile specification.

### 10.7.4	EventBuffer Descriptor

Subject to profile specification.

### 10.7.5	Signals Descriptor

Subject to profile specification.

### 10.7.6	DigitMap Descriptor

Subject to profile specification.

### 10.7.7	Statistics Descriptor

Subject to profile specification.

### 10.7.8	ObservedEvents Descriptor

Subject to profile specification.

### 10.7.9	Topology Descriptor

Subject to profile specification.

### 10.7.10	Error Descriptor

Subject to profile specification.

## 10.8	Command API

NOTE – It is assumed that an Error Descriptor may be returned in any command reply.

### 10.8.1	Add

Subject to profile specification.

### 10.8.2	Modify

Subject to profile specification.

### 10.8.3	Subtract

Subject to profile specification.

### 10.8.4	Move

Subject to profile specification.

### 10.8.5	AuditValue

Subject to profile specification.

### 10.8.6	AuditCapabilities

Subject to profile specification.

### 10.8.7	Notify

Subject to profile specification.

### 10.8.8	ServiceChange

Subject to profile specification.

### 10.8.9	Manipulating and auditing ContextAttributes

Subject to profile specification.

## 10.9	Generic command syntax and encoding

Subject to profile specification.

## 10.10	Transactions

Subject to profile specification.

NOTE – There is no impact on Transactions.

## 10.11	Messages

Subject to profile specification.

## 10.12	Transport

Subject to profile specification.

## 10.13	Security

Subject to profile specification.

## 10.14	Packages

### 10.14.1	Mandatory Package

Mandatory: specifies the packages that shall be supported in this profile.

Examples:

IF CSA THEN "following table":

| Mandatory packages: |  |  |  |
| --- | --- | --- | --- |
| Package name | PackageID | Version | Termination Types Supported |
| "IP domain connection package" [ITU-T H.248.41] | ipdc (0x009d) | v2 | IP |

IF CSB THEN "following table":

| Mandatory packages: |  |  |  |
| --- | --- | --- | --- |
| Package name | PackageID | Version | Termination Types Supported |
| "Media grouping package" [ITU-T H.248.96] | mgroup (0x011f) | v1 | IP |

NOTE – The H.248 CSB profile could continue with the ipdc v1 package because there is still a 1:1 ratio between H.248 Stream endpoint and associated IP domain.

### 10.14.2	Optional Package

Subject to profile specification.

### 10.14.3	Package usage information

The following is a non-exhaustive list of package usage indications.

#### 10.14.3.1	IP domain connection package

Example:

| Properties | Mandatory/ Optional | Used in command: | Supported values: | Provisioned value: | Termination/ Stream Types Supported: |
| --- | --- | --- | --- | --- | --- |
| IP Realm Identifier (ipdc/realm 0x009d/0x0001) | M | ADD, MOD | ALL (list of two values for ALTC) | - | IP |
| Signals | Mandatory/ Optional | Used in command: |  | Duration provisioned value: |  |
| None. | - | - |  | - |  |
|  | Signal parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
| Events | Mandatory/ Optional | Used in command: |  |  |  |
| None. | - | - |  |  |  |
|  | Event parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
|  | ObservedEvent parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
| Statistics | Mandatory/ Optional | Used in command: | Supported values: |  | Termination/ Stream Types Supported: |
| None. | - | - | - |  | - |
| Error Codes | Mandatory/Optional |  |  |  |  |
| None. | - |  |  |  |  |

#### 10.14.3.2	Media grouping package

Example:

| Properties | Mandatory/ Optional | Used in command: | Supported values: | Provisioned value: | Termination/ Stream Types Supported: |
| --- | --- | --- | --- | --- | --- |
| Group Semantics (mgroup/groupse, 0x011f/0x0001) | M | ADD, MOD | ANAT | None | ALL |
| Stream Aggregation (mgroup/stragg, 0x011f/0x0002) | M | ADD, MOD | ALL | None | ALL |
| Stream De-aggregation (mgroup/strdeagg, 0x011f/0x0003) | O (NOTE 1) | - | - | - | - |
| Signals | Mandatory/ Optional | Used in command: |  | Duration provisioned value: |  |
| None. | - | - |  | - |  |
|  | Signal parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
| Events | Mandatory/ Optional | Used in command: |  |  |  |
| None. | - | - |  |  |  |
|  | Event parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
|  | ObservedEvent parameters | Mandatory/ Optional | Supported values: | Provisioned value: |  |
|  | - | - | - | - |  |
| Statistics | Mandatory/ Optional | Used in command: | Supported values: |  | Termination/ Stream Types Supported: |
| None. | - | - | - |  | - |
| Error Codes | Mandatory/Optional |  |  |  |  |
| #489 | M |  |  |  |  |
| NOTE 1 – Not required for ALTC-based Stream Groups. |  |  |  |  |  |

## 10.15	Mandatory support of SDP and ITU-T H.248.1 Annex C information elements

Subject to profile specification.

## 10.16	Optional support of SDP and ITU-T H.248.1 Annex C information elements

Subject to profile specification.

## 10.17	Procedures

Subject to profile specification.

Appendix I

Discussion of the various H.248 solution options 
 – Signalling examples

## I.1	####

{Editor's note (2014-11): this Appendix is a placeholder for the text from current clause 9. All the signalling examples should be moved from clause 9 to this Appendix. Clause 9 would finally just indicated the preferred / recommended method(s).}

Appendix II

Discussion of the various H.248 solution options 
 – Summary

## II.1	Summary table

Table II.1 summarize aspects which might be positive or not in particular situations. There majority of statements is "soft" (because conditional) besides a few "hard" facts (because in violation with protocol syntax/semantics or limited Context model):

Table II.1 – Evaluation summary

| H.248 solution option | Pro | Con | Comments |
| --- | --- | --- | --- |
| M1: two Context model | - isolation of "alternate MG resources" in separate Context - context of "preferred IP connectivity" is fully identical to the usual non-ALTC Context configuration | - usage of MOVE command - support of early media not possible (because limited on preferred IP connection only) |  |
| M2: two Termination model | - isolation of "alternate MG resources" in separate Termination - support of early media possible, but dependent on topology signalling | - Topology descriptor required - thus, two H.248 transaction cycles | The Stream topology subject could be considered as blocking item. |
| M3: two Stream model | - single Termination | - conditional (only monomedia call types) - static H.248 StreamID values imply removal of S2 AND modification of S1, possibly inclusive IP version change | The StreamID subject could be considered as blocking item. |
| M4: two media groups model | - the semantic of ReserveGroup matches actually the semantic required for ALTC | - protocol issue: obvious contradiction with LCD level IP domain indication versus LD / RD level usage of "two IP address realms" in version 1 of the ipdc package. - No issue with version 2 of the ipdc package. | Existing standardized H.248 profiles for border gateways support only ipdc package version 1. Hence should be upgraded for ipdc v2. |
| M5: two Stream Groups model | - straightforward specification of Context-internal Stream topology (without Topology descriptor); - Context-internal resource configuration ("IWFs") remains unchanged after ALTC decision - Stream grouping allows to decouple the MG internal and MG external resources related to H.248 media streams. - Usage of H.248 media groups on top of H.248 stream grouping possible! | - usage of H.248.96 mgoup package (with semantic ANAT) | Solution available. |
| M6: dualstack IP interface resource | - ATLC decision would result in the simple modification of a single signalling parameter value | - protocol extensions (SDP or/and H.248 required) | Protocol extension subject could be considered as blocking item. |

## II.2	Recommended solution(s)

Three out of six candidates could be most likely rejected: M2, M3 and M6.

Thus, M1, M4 and M5 would be the remaining candidates.

It has to be noted that 3GPP (see Appendix III) selected M2 in correspondent 3GPP H.248 profiles with ALTC support in 3GPP Rel-12 (due to the unavailability of M4 and M5).

Appendix III

Example H.248 profile: 3GPP Iq profile

## III.1	Summary of stage 2 requirements

End of year 2014, 3GPP had to decide an ALTC solution for 3GPP Release 12. The following tables provides a rough summary of the underlying H.248 profile requirements:

## III.2	Resulting stage 3 specification: H.248 Iq profile

See [ETSI TS 129 334], Version 12.5.0.

Appendix IV

Use case specific capability sets – Some examples

(This appendix does not form an integral part of this Recommendation)

## IV.1	Overview

Profile content is mainly use case dependent (given be the inherent motivation of "protocol profiling"). In order to demonstrate profile specification guidelines in clause 10, this Recommendation considers two exemplary use cases (see Table IV.1):

- Capability set 'A' (CSA) – a "lightweight H.248 ALTC gateway", primarily  because the delayed cut-through of end-to-end IP connectivity facilitates MG-internal resource management;

selected method: "reservation of two media groups" (M4, see clause 7.1.4);

- Capability set 'B' (CSB) – a H.248 ALTC gateway which demands more flexibility concerning MG-internal resource management, especially the preparation and interconnection of internal media interworking functions due to early media aspects;

selected method: "reservation of two Stream Groups" (M5, see clause 7.1.5), also driven by the facts that 
a) the underlying H.248 profile is supposed to provide already support of the mgroup package for other services (i.e., ALTC could leverage on existing stream grouping support; and
b) potential future support of "multiple media groups" on top of the "ALTC stream group" structure.

Capability set 'B' (CSB) is therefore a superset of capability set 'A' (CSA).

Table IV.1 – Examples of use case specific capability sets (CS)

| Capability |  | Attribute | Req. Id.: | CSA | CSB |
| --- | --- | --- | --- | --- | --- |
| 1 | Communication service: number of media components | single | R-8.1/1: | X | X |
|  |  | multiple | R-8.1/2: | X | X |
| 2 | Communication service: early end-to-end IP connectivity | delayed | R-8.2/1: | X | X |
|  |  | early | R-8.2/2: | - | X |
|  |  | early unidirectional | R-8.2/3: | - | X |
|  |  | early bidirectional | R-8.2/4: | - | - |
| 3 | Call control signaling: number of negotiation cycles | single | R-8.3/1: | X | X |
|  |  | multiple | R-8.3/2: | - | X |
| 4 | Gateway control signaling: resource management process in H.248 MG | one-stage | R-8.4/1: | X | X |
|  |  | two-stage | R-8.4/2: | - | X |
| 5 | Gateway control signaling: explicit ALTC "call" tagging | no tag | R-8.5/1: | X | X |
|  |  | explicit tag | R-8.5/2: | - | X |
| 6 | Gateway control signaling: specific resource components in H.248 MG | concference unit | R-8.6/1: | - | - |
|  |  | … | R-8.6/1: |  |  |

Bibliography

[b-IETF RFC 5009]	IETF RFC 5009 (09/2007), Private Header (P-Header) Extension to the Session Initiation Protocol (SIP) for Authorization of Early Media.

[b-IETF RFC 5853]	IETF RFC 5853 (04/2010), Requirements from Session Initiation Protocol (SIP) Session Border Control (SBC) Deployment.

[b-ETSI TR 183 046]	ETSI TR 183 046 V3.3.1 (08/2009), Telecommunications and Internet converged Services and Protocols for Advanced Networking (TISPAN); SDP Interworking between Call/Session Control Protocols (SIP/SDP, RTSP/SDP; etc.) and the Gateway Control Protocol (H.248/SDP).

_________________________

See e.g. Figure G.2 in [ETSI TR 183 068].