| Rapporteur Meeting of Question 21/16 |  |  |  | AVD-4781 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 21/16 | Changzhou, 26 - 29 September 2016 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | ETRI |  |  |  |
| Title: | HSTP-DIS-UAV: Proposed to add use cases and service scenarios for flood |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This document contains proposed use cases and service scenarios for flood on disaster information service using unmanned aerial vehicles. The proposed updates are based on the TD 364 (WP1/16), current baseline document of HSTP-DIS-UAV, which was adopted at the last SG16 June 2016 meeting.

- Introduction

Clause 7 of HSTP-DIS-UAV describes use cases and service scenarios of a disaster information service for various disasters. This contribution proposes to add use cases and its service scenarios for flood disaster situation to clause 7.

- Proposal

- Proposes to add contents to clause 7.4 on use cases for flood

[Begin Proposal]

7.4	Use cases for flood

7.4.1	Preparedness phase

The disaster information service using UAVs can be used for detection of localized flooding and an overflowing river. Use-cases are described below.

- The risk for a flood can be estimated from topographic measurement data of flooded areas and cross sections of a river by a UAV carrying a LIDAR sensor.

- The risk for an overflowing river can be estimated from measurements of soil moisture and vegetative state by a UAV carrying a multi-spectral camera.

- A UAV equipped with a visible light camera can detect changes of the surface of an overflowing river in daylight.

Computer vision technologies can be used for automatic detection of floods.

(1)	Service scenario with on-board flood detection

An on-board computer system of a UAV analyses sensor data and detects floods in this service scenario. This scenario requires considerable on-board computing power and battery capacity.

- A UAV or UAVs patrol designated regions.

- The on-board computer system acquires overlapping aerial images of the regions and analyses sensor data.

- If a flood is detected, the sensor data and related information is sent to a flood incident command centre via wireless communication.

(2)	Service scenario with server-based flood detection

A ground computing facility analyses sensor data and detects floods in this service scenario. An on-board computer system of a UAV performs limited processing like resampling to the acquired sensor data.

- A UAV or UAVs patrol designated regions.

- The on-board computer system acquires overlapping aerial images of the regions and processes sensor data.

- The processed data is sent to a ground computing facility via wireless communication.

- The ground computing facility analyses the received sensor data.

- If a flood is detected, the sensor data and related information is sent to a flood incident command centre.

7.4.2	Response and relief phase

UAVs can be used for guiding evacuation and forecasting floods in the response and relief phase. Use-cases are described below.

- Still images and live motion pictures from UAVs can be used for guiding evacuation efforts.

- Sensor data from UAVs can be used for predicting the path of potential flood waters.

- UAVs carrying speakers can be used for alarming people in danger zone.

- UAV can deliver a relief package to isolated people by an overflowing river or stream.

7.5.3 Recovery and reconstruction phase

Images from UAVs can be used for damage assessment.

- Visible photographs, multi-spectral and LIDAR data can be used to assess the extent of the damage by floods and to monitor the recovery of the vegetation near a river.

# End Proposal

__________________