| Rapporteur Meeting of Question 21/16 |  |  |  | AVD-4783 |
| --- | --- | --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2013-2016 |  |  | STUDY GROUP 16 |  |
|  |  |  | Original: English |  |
| Question(s): | 21/16 | Changzhou, 26 - 29 September 2016 |  |  |
| RAPPORTEUR MEETING DOCUMENT |  |  |  |  |
| Source: | China Telecom |  |  |  |
| Title: | H.VSMprot: Proposal to add VAU-VS Reference point in clause 5.8 and revise clause 5 |  |  |  |
| Purpose: | Proposal |  |  |  |

Summary

This contribution proposes to add VAU-VS Reference point in clause 5.8 and revise clause 5 to draft H.VSMprot: "Signalling and Protocols for mobile visual surveillance".

# Introduction

The Recommendation H.VSMprot focus on the Signalling and Protocols of a visual surveillance system with mobile units, such as mobile customer unit or premises unit, as well as on the services flow related to mobile units, such as a mobile customer unit accessing real-time video stream from a visual surveillance system.

# Discussion

Recommendation H.VSMprot defines the reference points, signalling and protocols of the mobile visual surveillance system, based on the architecture defined in the Recommendation ITU-T H.626.1.

This contribution includes following changes:

- Divide clause 5.4 “Reference point Mv: M_CU-VAU/PSS” into “Reference point Vm: M_CU- VAU” and “Reference point Psm: M_CU-PSS”.

- Add description in clause 5.5” Reference point Mv: M_CU-VAU”.

- Add the reference point name “Vp” in clause 5.6” Reference point Vp:VAU-PSS”.

- Add reference point Mdv/Pv/Vs in clause 5.8.

# Conclusion

It is proposed to add VAU-VS Reference point in clause 5.8 and revise clause 5 to draft H.VSMprot: "Signalling and Protocols for mobile visual surveillance".

Attachment
Proposal text

[Begin proposed text]

Signalling and protocols for mobile visual surveillance

# Protocol reference points

Recommendation [ITU-T H.626.1] describes overall architecture framework of mobile Visual Surveillance as the following figure, and it also introduces reference points of architecture. This document gives more detailed description of those reference points as well as some newly defined reference points.

Figure 5-1 – Functional Architecture Framework of mobile Visual Surveillance

The interface protocols which involved in mobile video surveillance include following protocols:

- Reference point Cms: CMU-MSP, uses the Web Service interface protocol (SOAP/HTTP);

- Reference point Msm: MSP-M_CU, uses the HTTP protocol;

- Reference point Mv: M_CU-VAU, uses the UDP communication protocol;

- Reference point Vm: VAU-M_CU, uses the HTTP/RTSP, RTP/RTCP protocol;

- Reference point Psm: PSS-M_CU, uses the HTTP/RTSP, RTP/RTCP protocol;

- Reference point Vs: VAU-SCU, uses the HTTP protocol;

- Reference point Mdv: MDU-VAU, uses the RTP/RTCP protocol;

- Reference point Pv: PU-VAU, uses the RTP/RTCP protocol;

- Reference point Vp: VAU-PSS, uses the RTSP protocol;

- Reference point Wm: WAP GW-MSP, WAP GW puts the MDN number in the head domain of HTTP.

## Reference point Psm: M_CU- PSS

### Interface function

Reference point Psm is between M_CU and PSS.

It is used to transmit information through HTTP and RTSP to:

Send service request, carrying service and user information;

Establish connection and  transmitting the real-time stream of media and real-time video;

Play video according to operations, such as pause, continue, forward and other orders.

### Interface protocol

The Interface is actually between the terminal and the streaming server, it follows the TCP / IP, RTP / RTSP / RTCP protocol, as shown in the Figure 5-2.

| RTP | HTTP | RTSP |  |
| --- | --- | --- | --- |
| UDP | TCP |  | UDP |
| IP |  |  |  |

Figure 5-2 – Interface protocol

And the signalling is shown in Figure 5-3.

M_CU

PSS

RTSP request

Return media stream

Return RTSP

dynamic address

HTTP connection

M_CU

VAU/ PSS

RTSP Request

Media stream

Figure 5-3 – Signalling

### Message format

- HTTP interface

The interface function is realized through the parameters carried by http URL. When user clicks on the URL generated by MSP, the following parameters are acquired and transferred to PSS without processing.

See IETF RFC 2616 for HTTP message.

Table 5-6-1: HTTP Parameters from M_CU to PSS

| M_CU→PSS |  |  |  |
| --- | --- | --- | --- |
| Parameter | Parameter Name | Data Type | Value |
| UserId | User ID | String(BCD20) |  |
| PUID-ChannelNo | Unique identification of surveillance point | String(BCD20) |  |
| PuProperty | Whether PU has PTZ function | Int(4) | Note: Default value is 0 if PU doesn’t have PTZ function. |
| VAUADD | VAU address and PTZ control port | String(BCD20) | Format: xxx.xxx.xxx.xxx:xxx(ip: port) Note: Default value is 0 if PU doesn’t have PTZ function. This parameter is validate only in realtime mode. |

When user requests for PSS service, the http request format is like this:

http://pssIp:port/service?userid=xx&puid-chanelid=xx&puproperty=xx&vauadd=xx。

PSS responds to the M_CU request, and sends the dynamically generated RTSP URL to end user automatically through html page jumping, the address is like this:

<head>

<meta content=”refresh” url=”0,rtsp://xxxxxxxxxxxxxxxxx”/>

</head>

- RTSP interface

RTSP message is for establishment and control of streaming transferring dialog, see RFC2326 for details.

The RTSP request sent by M_CU to PSS should carry following parameters:

Table 5-6-2: RTSP Parameters from M_CU to PSS

| M_CU→PSS |  |  |  |
| --- | --- | --- | --- |
| Parameter | Parameter name | Data type | Value |
| UserId | User ID | String(BCD20) |  |
| PUID-ChannelNo | Unique identification of surveillance point | String(BCD20) |  |
| PlayMetod | Media type | Int(1) | 0: Realtime 1: Video record |
| StartTime | Start time of recording | String(BCD20) | Format: YYYY-MM-DD-HH:MM:SS |
| PuProperty | Whether PU has PTZ function | Int(4) | Note: Default value is 0 if PU doesn’t have PTZ function. |
| VAUADD | VAU address and PTZ control port | String(BCD20) | Format: xxx.xxx.xxx.xxx:xxx(ip: port) Note: Default value is 0 if PU doesn’t have PTZ function. This parameter is validate only in realtime mode. |
| hashtoken | For billing authentication request | String(BCD20) | Used by manufactories for anti-stealing-link, not be processed by M_CU. |

Parameters are transmitted through:

RTSP://localhost:port/service?UserID=×××& PID= XXX……

The server of the RTSP extracts parameters while responding to the request and processes them according to the service logic.

## Reference point Vm: M_CU- VAU

### Interface function

Reference point Vm is between M_CU and VAU.

It is used to transmit the real-time video through RTSP.

### Interface protocol

The interface uses RTSP protocol, M_CU works as the RTSP client, VAU works as the RTSP server which can be configured freely, and the default port is 554. The RTSP message format and parameters are as follow:

Table 5-6-3: Common Parameters from M_CU to VAU

| M_CU→VAU |  |  |  |
| --- | --- | --- | --- |
| Parameter | Parameter Name | Data Type | Value |
| PlayMethod | Media type | String(BCD1) | 0：Realtime |

Table 5-6-4: Realtime Mode Parameters from M_CU to VAU

| M_CU→VAU |  |  |  |
| --- | --- | --- | --- |
| Parameter | Parameter Name | Data Type | Value |
| PUID- ChannelNo | Channel identification, the PUID and channel number in VS system | String(BCD30) | Device ID + Channel Type + Channel Number |

The format of realtime video viewing RTSP URL is like this:

rtsp://vauaddress:port/service?PUID-ChannelNo=XXXXXXXXXXXX-X PlayMethod=0, for instance: rtsp://172.24.202.190/service?PUID-ChannelNo=320101000200000001-1&&PlayMethod=0,  which refers to accessing the real time video stream of first channel of PU NO.320101000200000001.

### Message format

M_CU works as the client, and VAU works as the server, the

interface meets following needs:

- Meet the minimum requirements of client and server basic playback of RFC2326 "Real Time Streaming Protocol (RTSP)".

- In accordance with the requirements of 3GPP TS 26.234, the client and server should support the RTSP DESCRIBE method.

- In accordance with the requirements of 3GPP TS 26.234, the client and server should support the RTSP Range header field;

- In accordance with the requirements of 3GPP TS 26.234, the server need to include the Range field in the responding message to PLAY.

DESCRIBE message

- M_CU sends DESCRIBE message.

- VAU returns 200 OK and messages with correct message body to M_CU, establishing the session. The message body includes default video CODEC of transferred video stream, and related profile-level-id, frame rate, frame size and so on. VAU configures the default value.

SETUP message

- M_CU sends SETUP message, describing the interface which receives the media streaming.

- VAU returns 200 OK to M_CU, describing the interface which sends the media streaming.

PLAY message

- M_CU sends PLAY message.

- VAU starts to play real time video or video record.

- VAU returns  200 OK to M_CU.

- VAU starts to send media stream to M_CU.

PAUSE message

- M_CU sends the correct PAUSE message.

- VAU pauses the real time video or video record playing.

- VAU returns 200 OK to M_CU.

- VAU stops sending media stream.

TEARDOWN message

- M_CU sends TEARDOWN message.

- VAU stops the real time video or video record playing.

- VAU returns 200 OK to M_CU.

- VAU stops sending media stream.

- VAU RTSP SOCK stops.

OPTIONS message

- VAU sends OPTIONS message to M_CU (with Session).

- M_CU searches for the target Session.

- If the target Session exists, M_CU returns 200 OK. If the target Session doesn’t exist, M_CU returns 404 error, and VAU disconnect the Session with CMS and release the source.

## Reference point Mv: M_CU-VAU

### Interface function

Reference point Mv is between M_CU and VAU.

It is used to control PTZ for end-users when browsing real-time video.

### Interface protocol

The communication  between M_CU and  VAU is according to UDP protocol, The default communication port is 5060. The pack for request and response reserves 200 bytes for via fields in head of SIP.

M_CU sends the PTZ control messages to VAU without 200 OK response or resend. But if within the definition time M_CU doesn’t send the stop message to VAU, VAU will send PTZ stop message to the platform automatically.

Considering several M_CUs visit one PUID-ChannelNo at the same time, M_CU will send the PUID-ChannelNo carried by RTSP request to VAU.VAU distinguishes PTZ message target devices according to the PTZ message body.

<IE_HEADER MessageType="MSG_PTZ_SET_REQ"

UserID=”User name”

DestID = "PUID-ChannelNo"

/>

- M_CU

M_CU sends request and receives response .When it receives response, M_CU must check XML.

Message_Type and Sequence_Number.  If the parameter is wrong, the response must be discarded

- VAU

VAU should be able to receive request from any IP address and port, and reply the port from   M_CU request.

### 5.5.3	Message format

generic-message = start-line

*message-header

CRLF

[ message-body ]

1） start-line   start-line = Request-Line / Status-Line

SIP  request:

Request-Line = Method SP Request-URI SP SIP-Version CRLF

SIP response:

Status-Line = SIP-Version SP Status-Code SP Reason-Phrase CRLF

2） message-header

Request  for message-header:

INFO sip:xxx

Content-Type: application/global_eye_v10+xml

Content-Length: xxx

To: xxx

From: xxx

CSeq: xxx INFO

Call-ID: xxx

Max-Forwards: xxx

Via: xxx

Contact: xxx

[CrLF]

Response  to message-header:

200 OK sip:xxx

Content-Type: application/global_eye_v10+xml

Content-Length: xxx

To: xxx

From: xxx

CSeq: xxx INFO

Call-ID: xxx

Max-Forwards: xxx

Via: xxx

Contact: xxx

[CrLF]

3）Message-body

PTZ control

a)	Function: The user platform sends PTZ control message to another platform.

b)	Message:

<!—PTZ control  -->

<?xml version="1.0" encoding="UTF-8" ?>

<Message Verison="1.0">

<IE_HEADER MessageType="MSG_PTZ_SET_REQ"

UserID=”User name”

DestID=”PUID-ChannelNo”

/>

<IE_PTZ OpId="code"

Param1=" parameter 1"

Param2=" parameter 2"

/>

</Message>

This message needs no response.

4）  Code and parameter

1: turn up (1--10)

2: turn down (1--10)

3: turn left (1--10)

4: turn right (1--1)

5: open iris (1--10)

6: close iris   (1--10)

7: zoom in (1--10)

8: zoom out (1--10)

9: focus near (1--10)

10: focus far (1--10)

11: RESERVED

12: RESERVED

13: RESERVED

14: RESERVED

15: stop

20: scan off

21: scan on (1--10)

24: RESERVED

25: RESERVED

26: RESERVED

## Reference point Vp:VAU-PSS

### Interface function

Reference point Vp is between VAU and PSS.

It is used to transfers performance data, alarm data and topological data.

### Interface protocol

Interface protocols are HTPP＋XML, SOAP or CORBA, for  different platforms and systems.

### Message format

#### VAU interface

It is used to control and manage VAU according to the definition of MIB.

- Configuration management

VAU information model as in Table 5-6-5, Table 5-6-6, Table 5-6-7.

Table 5-6-5: VAU interface

| Name | Data type | Read/Write |
| --- | --- | --- |
| VAU device ID | String | R/W |
| Version number | String | R/W |
| Equipment manufacturer code | String | R/W |
| Equipment manufacturer name | String | R/W |
| IP network parameter | String | R/W |
| Maximum number of video input | Integer | R/W |
| Maximum number of video output | Integer | R/W |
| Transcoding type | String |  |
| Platform number | Integer |  |

Table 5-6-6: VAU multi –platform data model

| Name | Data type | Read/Write |
| --- | --- | --- |
| CMS device ID | String | R/W |
| IP address | String | R/W |
| Port | String | R/W |

- state management

Table 5-6-7: VAU state management information in system model

| Name | Date type | Read/Write |
| --- | --- | --- |
| VAU device ID | String | R |
| CPU usage | String | R |
| Memory usage | String | R |
| Current processes number | Integer | R |
| Current thread number | Integer | R |
| Registration status | String | R |
| Current video inputs | Integer | R |
| Current video outputs | Integer | R |
| Current transcoding number | Integer | R |
| VAU heartbeat statue | String(1) | R |

#### PSS interface

It is the MIB definition of PSS management, which can be used to control and manage PSS

- Configuration management

Table 5-6-8: PSS configuration management information in system model

| Name | Data type | Read/Write |
| --- | --- | --- |
| PSS device ID | String | R/W |
| Version Number | String | R/W |
| Equipment manufacturer code | String | R/W |
| Equipment manufacturer name | String | R/W |
| Maximum number of video input | Integer | R/W |
| Maximum number of video output | Integer | R/W |

- Statue management

Table 5-6-9: PSS state management information in system model

| Name | Data type | Read/Write |
| --- | --- | --- |
| PSS device ID | String | R |
| CPU usage | String | R |
| Memory usage | String | R |
| Current processes number | Integer | R |
| Current thread number | Integer | R |
| Registration status | String | R |
| Current video inputs | Integer | R |
| Current transcoding number | Integer | R |
| PSS heartbeat statue | String (1) | R |

## Reference point Mdv/Pv/Vs:VAU-VS system

Reference point Mdv, Pv, Vs are all between VAU and the VS system which are all internal interfaces, there is no specific definition in this recommendation.

_________________