| INTERNATIONAL TELECOMMUNICATION UNION |  | IPTV-GSI |
| --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  | IPTV-GSI-C-503 |
|  |  | English only Original: English |
| Question(s): | 13/16 | Pune, India, 13-17 December 2010 |
| CONTRIBUTION |  |  |
| Source: | Huawei Technologies |  |
| Title: | H.IPTV-AM.0: New section for specification of TD-AMF configuration metadata |  |

Background

Metadata elements have been identified for measurement reports but metadata elements are still missing for audience measurement function configuration in H.IPTV-AM.0.

Summary

A new clause "14.4. Metadata elements for TD-AMF configuration".

Proposal

It is proposed to insert a new section as follows

# 14.4. Configuration package elements for TD-AMF configuration

This clause describes the metadata elements necessary for audience measurement configuration of the IPTV terminal devices.

Information is needed to:

- identify the services to be measured

- specify the elements to be measured and reported

- specify when to measure them (measurement schedule)

- specify how to report measurement (measurement delivery)

- specify recovery mechanisms in case of abnormal situations

The configuration package is structured as follows:

- general elements

- set of measurement requests

# 14.4.1. General elements of the AM configuration package for TD-AMF

These metadata elements apply to the global configuration package, they include in particular default settings that are used by a number of measurement requests.

Table 5 – General metadata elements in configuration package for TD-AMF

| Element | Description | Support/type | Notes or Value Domain |
| --- | --- | --- | --- |
| AMConfPackageId | Identification of the AM configuration package | 1 integer | Editor's note: usefulness to be confirmed. |
| DefaultMeasurementSchedule | TBD | 0-1 |  |
| DefaultDeliveryMechanism | URL to be used to send measurement reports from the AMF | 0-1 |  |
| DefaultRecoveryMechanism | This element describes what to do in case of reporting problems TBD | 0-1 |  |
| DefaultNoNewsReportMode | This element states what to do in case that there is no change since the last measurement sample for the same service Editor's note: To be discussed | 0-1 integer | 0: ignore AM sample 1: create empty AM sample 2: create a complete AM sample |

# 14.4.1. Metadata elements for "measurement requests" in AM configuration package for TD-AMF

The metadata elements which are defined as mandatory in the measurement reports, such as "content identifier", "event time", "measurement report trigger: period or event) are not listed in the list of elements to be reported in the table below.

Table 6 –Metadata elements in "measurement request"

| Element | Description | Support/type | Notes or Value Domain |
| --- | --- | --- | --- |
| RequestId | Identification of the measurement request. Note: This Id, to be included in the measurement reports, may be used by the aggregation function to identify all measurement reports related to the same service. | 1 integer |  |
| ServiceTypeId | Identifier of the type of service to be measured | 1 string | Type already defined in section 14.3. E.g. Linear TV, Content on Demand, x-PVR. |
| ToBeReported | List of the elements to be included in the measurement reports Note: as this list is specific to each service, it is specified in the sections relevant to each of these services | 1 |  |
| MeasurementSourceTypeReq | Requirement for type of the measurement report source to be inserted in measurement report. Note: What is it? Mobile or STB? terminal Id | 0-1 Boolean Default: false |  |
| To be continued |  |  |  |
|  | End of list of elements to be reported. |  |  |
| MeasurementScheduleList | Start of measurement schedule list. | 0-1 |  |
| MeasurementScheduleItem | Start of measurement schedule item. | 1-* |  |
| StartTime | Time of the day at which the audience measurement should start | 0-* time Default 0am |  |
| EndTime | Time of the day at which the audience measurement should stop | 0-* time Default 12pm |  |
| MeasurementTriggerList | Start of measurement trigger list | 1 No default |  |
| MeasurementTriggerItem | Start of measurement trigger item | 1-* |  |
| MeasurementTriggerType | It indicates which type of trigger will trigger a measurement from the AMF Followed by either "EventList" or "Periodicity" | 1 Integer Default: 0 | 0: time 1: event |
| EventList | List of events which will trigger a measurement from the AMF TBD | 1 if MeasurementTriggerType = 1 0 if MeasurementTriggerType = 0 |  |
| Periodicity | Periodicity at which a measurement should be made expressed in seconds. | 0 if MeasurementTriggerType = 1 (0-1) if MeasurementTriggerType = 1 Integer |  |
|  | End of Measurement trigger item |  |  |
|  | End of Measurement trigger list |  |  |
|  | End of measurement schedule item |  |  |
|  | End of measurement schedule list |  |  |
| MeasurementDeliverySchedule | Description of the mechanism to be used to decide on when to send measurement reporting from the TD-AMF | 0-1 |  |
|  | TBD: Parameters for measurement delivery schedule |  |  |
| MeasurementDeliveryMechanism | URL to be used to send measurement reports from the AMF | 0-1 |  |
| RecoveryMechanisms | This element describes what to do in case of reporting problems TBD | 0-1 |  |
| NoNewsReportMode | This element states what to do in case that there is no change since the last measurement snapshot Editor's note: To be discussed | 0-1 integer | 0: ignore AM sample 1: create empty AM sample 2: create a complete AM sample |

Support: 1 = mandatory (one instance), 0-1 = optional (max one instance), 0-* = (optional and multiple instances possible), 1-* = mandatory and multiple instances possible)

Type: string, integer, float, etc.

_____________