| INTERNATIONAL TELECOMMUNICATION UNION |  | IPTV-GSI |
| --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  | IPTV-GSI-C-510 |
|  |  | English only Original: English |
| Question(s): | 13/16 | Pune, India, 13-17 December 2010 |
| CONTRIBUTION |  |  |
| Source: | NEC, NTT |  |
| Title: | Proposed additions of AM protocols to HSTP.IPTV-PITD “Protocols handled by IPTV terminal devices” |  |

Summary

This contribution proposes to add and revise some texts in audience measurement parts of [HSTP.IPTV-PITD] according to [ITU-T IPTV-AM.0] and other standards.

- Introduction

Q13/16 updated [HTSP.IPTV-PITD] according to new Recommendation Q.3040 “IPTV Signalling Control Plane Architecture” at IPTV-GSI meeting (19-23 July, 2010, Geneva).

Q13/16 experts have discussed draft Recommendation concerning audience measurement after the IPTV-GSI.  This contribution adds and revises texts relevant to audience measurement protocol which are to be implemented on IPTV terminal devices.  Almost all texts are derived from an output document [TD 316 (WP 2/16)] of IPTV-GSI (Geneva, 19-30 July 2010).

In addition, [BBF TR-069] relevant items should be added according to [H.770 Amd.2] consented in July.

- Discussion and Proposal

Proposed updates of diagrams and texts to [HTDP.IPTV-PITD] are shown with revision marks:

Start proposed descriptions

Draft Technical Paper HSTP.IPTV-PITD

Protocols handled by IPTV terminal devices

Summary

[To be provided before Approval]

# Scope

This Technical Paper describes the overall protocols for IPTV terminal devices.  Variety models of IPTV terminal devices are considered in this paper (e.g., Basic Model: Linear TV centric, Web-based Model, Mobile Model, Full-fledged Model ).  It also describes methods and use cases using these protocols.

# References

The following ITU-T Recommendations and other references contain provisions, which, through reference in this text, constitute provisions of this Supplement. At the time of publication, the editions indicated were valid. All Recommendations and other references are subject to revision; all users of this Supplement are therefore encouraged to investigate the possibility of applying the most recent edition of the Recommendations and other references listed below.

[ITU-T Y.1901]	Draft Recommendation ITU-T Y.1901 (2008), IPTV Services Requirements

[ITU-T Y.1910]	Recommendation ITU-T Y.1910 (2008), IPTV Architecture

[ITU-T H.701]	Recommendation ITU-T H.720 (2008) (ex H.IPTV-CDER), Content delivery error recovery for IPTV services

[ITU-T H.720]	Recommendation ITU-T H.720 (2008) (ex H.IPTV-TDES.0), Overview of IPTV terminal devices and end systems

[ITU-T H.721]	Recommendation ITU-T H.721 (2009) (ex H.IPTV-TDES.2), IPTV terminal devices, Basic Model

[ITU-T H.770]	Recommendation ITU-T H.770 (2009) (ex H.IPTV-SDC), Mechanisms for service discovery up to consumption for IPTV

[ITU-T X.1191]	Recommendation ITU-T X.1191 (2009), Functional requirements and architecture for IPTV security aspects

[ITU-T Q.3402] 	ITU-T Recommendation Q.3402 (2008), NGN UNI Signalling Profile (Protocol Set 1)

[ITU-T Q.3040] 	ITU-T Recommendation Q.3040 (2010), IPTV Signalling Control Plane Architecture

[ITU-T J.367]	Recommendation ITU-T J.367 (2008), IPCablecom 2 Presence Specification

[ITU-T X.891]	ITU-T Recommendation X.891 (2005) | ISO/IEC 24824-1:2005, Information technology – Generic applications of ASN.1: Fast infoset

[ITU-T X.1301] 	ITU-T Recommendation X.1303(2007), Common alerting protocol (CAP 1.1)

[ISO/IEC 23001]	IISO/IEC 23001-1 (2006), Information technology - MPEG systems technologies - Part 1: Binary MPEG format for XML

[ATIS-0800013]	ATIS standard ATIS-0800013 (2008), Media Formats and Protocols for IPTV services

[ATIS-0800017]	ATIS standard ATIS-0800017 (2008), Network Attachment and Initialization of Devices and Client Discovery of IPTV Services

[ETSI TS 183 063]	ETSI TS 183 063 V2.1.0 (2008), Telecommunications and Internet converged Services and Protocols for Advances Networking (TISPAN); IMS-based IPTV stage 3 specification

[ETSI TS 102 539] 	ETSI TS 1-2 539 V1.2.1 (2008), Digital Video Broadcasting (DVB); Carriage of Broadband Content Guide (BCG) information over Internet Protocol (IP)

[BBFF TR069]		Broadband Forum TR-069 (2007), CPE WAN Management Protocol

[IETF RFC 768]	IETF RFC 768 (1980), User Datagram Protocol (UDP)

[IETF RFC 959]	IETF RFC 959 (1985), File Transfer Protocol (FTP)

[IETF RFC 1350]	IETF RFC1350 (1992), The TFTP Protocol (Revision 2)

[IETF RFC 2131]	IETF RFC 2131(1997), Dynamic Host Configuration Protocol

[IETF RFC 2236]	IETF RFC 2236 (1997), Internet Group Management Protocol, Version 2

[IETF RFC 2250]	IETF RFC 2250 (1998), RTP Payload Format for MPEG1/MPEG2 Video

[IETF RFC 2326]	IETF RFC 2326 (1998), Real Time Streaming Protocol (RTSP)

[IETF RFC 2616]	IETF RFC 2616 (1999), Hypertext Transfer Protocol -- HTTP/1.1

[IETF RFC 2782]		IETF RFC 2782 (2000), A DNS RR for specifying the location of services (DNS SRV)

[IETF RFC 2818]	IETF RFC 2818 (2000), HTTP over TLS

[IETF RFC 3261]		IETF RFC 3261 (2002), SIP: Session Initiation Protocol

[IETF RFC 3376]	IETF RFC 3376 (2002), Internet Group Management Protocol, Version 3

[IETF RFC 3550]	IETF RFC 3550 (2003), RTP: A Transport Protocol for Real-Time Applications

[IETF RFC 3810]	IETF RFC 3810 (2004), Multicast Listener Discovery Version 2 (MLDv2) for IPv6

[IETF RFC3920]	IETF RFC3920 (2004), Extensible Messaging and Presence Protocol (XMPP): Core

[IETF RFC3920]	IETF RFC3920 (2004), Extensible Messaging and Presence Protocol (XMPP): Instant Messaging and Presence

[IETF RFC3926]	IETF RFC3926 (2004), FLUTE - File Delivery over Unidirectional Transport

[IETF RFC4330]	IETF RFC 4330 (), Simple Network Time Protocol (SNTP) Version4 for IPv4, IPv6 and OSI

[IETF RFC4301]	IETF RFC4301 (2005), Security Architecture for the Internet Protocol

[IETF RFC 4825]	IETF RFC 4825 (2007), The Extensible Markup Language (XML) Configuration Access Protocol (XCAP)

[W3C XML]	World Wide Web Consortium (W3C) Recommendation XML1.0 (2008), Extensible Markup Language (XML) 1.0, Fifth Edition

[W3C SOAP]	World Wide Web Consortium (W3C) Recommendation SOAP 1.2 (2007), Simple Object Access Protocol (SOAP) 1.2

# Definitions

## Terms defined elsewhere

This document uses the following terms defined elsewhere:

3.1.1	IPTV [ITU-T Y.1901]: Multimedia services such as television/video/audio/text/graphics /data delivered over IP-based networks managed to support the required level of QoS/QoE, security, interactivity and reliability.

3.1.2	IPTV terminal device [ITU-T Y.1901]: A terminal device which has ITF functionality, e.g. a STB.

3.1.3	IPTV Terminal Function (ITF) [ITU-T Y.1901]: The functionality within the home network that is responsible for terminating the IP signal, and converting the content into a renderable [i.e. enabling to be seen and/or heard] format.

3.1.4	Linear TV [ITU-T Y.1901]: A television service in which a continuous stream flows in real time from the service provider to the terminal device and where the user cannot control the temporal order in which contents are viewed.

3.1.5	SCP [ITU Y.1901]: A combination of service protection and content protection

3.1.6	Service Navigation [ITU-T H.720]: The presentation of information that allows the end-user to discover, select and consume services.

3.1.7	Video-on-Demand (VoD) [ITU-T Y.1901]: A service in which the end-user can , on demand, select and view a video content and where the end-user can control the temporal order in which the video content is viewed (e.g. the ability to start the viewing, pause, fast forward, rewind, etc)

Note: The viewing may occur some time after the selection of the video content.

## Terms Defined within this document

This document defines the following terms

3.2.1	[TBD]: [TBD]

# Abbreviations

| DHCP | Dynamic host configuration protocol |
| --- | --- |
| DVBSTP | DVB SD&S transport protocol |
| FLUTE | File delivery over unidirectional transport |
| HTTP | Hypertext transfer protocol |
| HTTPS | HTTP over TLS |
| IGMP | Internet group management protocol |
| IP | Internet protocol |
| IPsec | Security Architecture for Internet Protocol、 |
| MLD | Multicast listener discovery |
| RSTP | Realtime streaming protocol |
| RSVP | Reservation protocol |
| RTCP | RTP control protocol |
| RTP | Realtime transport protocol |
| SCP | Security and content protection |
| SIP | Session initiation protocol |
| SNTP | Simple network time protocol |
| SOAP | Simple object access protocol |
| TCP | Transmission control protocol |
| TLS | Transport layer security |
| UDP | User datagram protocol |
| XCAP | XML configuration access protocol |
| XML | Extensible markup laungage |
| XMPP | Extensible Messaging and Presence Protocol |

# Protocol Reference Points of IPTV Terminal Device

ITU-T Recommendation [ITU-T H.720] describes overall architecture of IPTV terminal devices’ functions based on [ITU-T Y.1910].  This Recommendation aims to make up for the lack of protocol-relevant information of [ITU-T H.720] according to other ITU-T Recommendations.

NOTE - [ITU-T Y.1910] and [ITU-T Q.3040] introduce basic reference points of IPTV architecture.

Accordingly, Figure 5-1 shows reference points concerning IPTV terminal devices.  Reference points from E1 to E6 fully comply with [ITUT Y.1910] and [ITU-T Q.3040], E7, E8 and E9 are relevant to [ITU-T H.720] ,and E10 are newly defined according to [b_ITU-T H.IPTV-AM.0].  Left side round-square boxes in this figure are server-side or network-side functionalities.

Figure 5-1: Reference points on protocols of IPTV terminal devices

## Reference point E0

The E0 reference point is between SADS client functional block and the server-side SADS functional block.

This reference point is used to discover service provider/services themselves, and select IPTV services and applications.  For details of service discovery, see [H.IPTV-DSC].

## Reference point E1

The E1 reference point is between IPTV terminal devices’ application client functional block and server-side IPTV application functional block.

This reference point is used to support service and application configuration.

## Reference point E2

The E2 reference point is between SCP client functions and server-side SCP functions.

This reference point is used for delivering security information (e.g. rights object or keys) from SCP functions to SCP client functions.  For details of SCP, see [ITU-T X.1191]

## Reference point E3

The E3 reference point is between the control client functional block and the IPTV service control functional block.

This reference point is used to exchange session signalling information, e.g. session establishment, modification, and termination. It can optionally be used to exchange:

- content control messages, such as content recording commands;

- service and application discovery information.

## Reference point E4

The E4 reference point is between error recovery functional block and error recovery client functional block.

This reference point is used to exchange messages for requesting and delivering error recovery information, for example FEC repair data or retransmission data.  For detail of FEC and retransmission, see [ITU-T H.701].

## Reference Point E5

The E5 reference point is between the multicast content delivery client functional block and the multicast control point functional block.

This reference point is used to exchange messages for joining multicast channels, e.g. IGMP messages.

NOTE - This reference point is named as E5-Cm by [ITU-T Q.3040]

## Reference point E6

The E6 reference point is between the unicast content delivery client functional block and the content delivery control functional block.

This reference point is used to exchange content control message, e.g. video recording commands.

NOTE 1- The information exchanged between the unicast content delivery client functional block and the content delivery control functional block can optionally be transferred via the IPTV service control functions, e.g. in the case where the IPTV service control functions proxy all requests between the unicast content delivery client functional block and the content delivery control functional block.

NOTE 2- This reference point is equivalent to E5-Cu of [ITU-T Q.3040]

## Reference point E7

The E7 reference point is between content delivery client functions and the delivery network gateway functional block.

This reference point is used to deliver control messages and content streams.

## Reference point E8

The E8 reference point is between performance monitoring client functions and the relevant server-side functional block.  Details concerning this reference point are for further study.

## Reference point E9

The E9 reference point is between network attachment client functions and the relevant server-side functional block.  Details concerning this reference point are for further study.

## Reference point E10

The E10 reference point is between end-user measurement functions and the relevant server-side functional block.  This reference point is used to deliver audience measurement data (e.g., viewership information) [b_ITU-T H.IPTV-AM.0].

NOTE 3 - End-user measurement functions support the ability to directly ask for an end-user’s permission, store responses, report on permission status, and enable or disable measurement depending upon those responses.  Aggregation functions collect the data through networks.

NOTE 4- This reference point is equivalent to the reference point “A1” of [b_ITU-T H.IPTV-AM.0]

Protocols and methods

Protocols introduced in this clause are used from initialization of IPTV terminal devices up to consumption of IPTV services.

NOTE - Details of requirement levels of each protocol for IPTV services should be consulted with relevant ITU-T Recommendations.

## Protocol stack

Figure 6-1 shows a diagram of typical protocol stack mentioned-below.  Each protocol is deeply related with processes of IPTV services such as an initiation and consumption of IPTV services.

Figure 6-1: Major protocols regarding IPTV terminal devices

## Network Attachment: E9

The followings are protocols for handling entry data of service discovery.  Entry point data is initial address for access to the server-side SADS functions [ATIS-0800017][ETSI TS 183 063];

- Preconfigured methods

  - IPTV terminal device may be preconfigured or manually configured with service discovery information

- DHCP-based methods

  - DHCP[IETF RFC 2131] Container Option can deliver Service Discovery Information when IPTV terminal device acquires network information such as its own IP address from DHCP server

NOTE - There is a lot of variation of the usage of Container Option and detail of these have not been authorised by IETF.

  - Download methods (aka Pull mode)

  - IPTV terminal device may download information using unicast based procedure such as TFTP [IETF RFC 1350], HTTP.

- TR-069-protocol based method

  - Remote management system can provide addressing information of service providers with TR-069 protocol[BBF TR069].

- Multicast delivery method (aka Push mode):

  - SI delivery using IGMP/MLD and/or DVBSTP is the popular way to deliver service discovery information.

  - FLUTE multicast stream can deliver service information files to IPTV devices when using OMA BCAST technology.

- DNS Service Records (SRV)-based method

  - The result of DNS SRV [IETF 2782] lookup can provide available IPTV service servers within the specified Domain Name.

NOTE - This method is often used when SIP server find the locations of services as FQDN.

## Service Discovery:

Service Discovery contains two processes: service provider discovery and service discovery.  Former process used service provider information, later uses detailed service offer information.

### Service Provider Information delivery protocol(s): E0, E3, E5

The recommended transport mechanisms for the delivery of the descriptions of IPTV Service Providers over IP are as follows:

- HTTP version 1.1 [IETF RFC 2616] for "Service Provider Information" delivery over unicast (“pull mode”)

- HTTP over TLS [IETF RFC 2818] for "Service Provider Information" delivery over unicast with secure manner (“pull mode”)

- IGMP version 2  [IETF RFC 2236] for "Service Provider Information" delivery over IPv4 multicast (“push mode”)

- IGMP version 3  [IETF RFC 3376] for "Service Provider Information" delivery over IPv4 multicast (“push mode”)

- MLD version 2 [IETF RFC 3810]  for "Service Provider Information" delivery over IPv6 multicast (“push mode”)

- DVBSTP [ETSI TS 102 034]: a light protocol specified by DVB, used for delivery over multicast (push mode)

- FLUTE [ETSI TS 102 472] for "Service Provider Information" delivery over IPv4/IPv6 multicast (push mode)

- SIP [IETF RFC 3261] for "Service Provider Information" delivery over SIP (“push mode” or “pull mode”)

- TR-069[BBF TR069] for “Service Provider Information” and specifies extensions to TR-135 STB data model [b_BBF TR135] to store this kind of information.

### Detailed Service Offer Information delivery protocol: E0, E5, E6

The recommended transport mechanisms for the delivery of the descriptions of IPTV services offered by an IPTV Service Provider are:

- HTTP 1.1 [IETF RFC 2616] for "Detailed Service Offer" delivery over unicast (“pull mode”)

- HTTP over TLS (to be confirmed for the use of HTTPS)

- IGMP version 2  [IETF RFC 2236] for "Detailed Service Offer" delivery over IPv4 multicast (“push mode”)

- IGMP version 3 [IETF RFC 3376]  for "Detailed Service Offer" delivery over IPv4 multicast (“push mode”)

- MLD version 2 [IETF RFC 3810]  for "Detailed Service Offer" delivery over IPv6 multicast (“push mode”)

- DVBSTP [ETSI TS 102 534]: a light protocol specified by DVB, used for delivery over multicast (“push mode”)

- FLUTE [ETSI TS 102 472] for "Detailed Service Offer" delivery over IPv4/IPv6 multicast (push mode)

- SIP [IETF RFC 3261] for "Detailed Service Offer" delivery over SIP (“push mode” or “pull mode”)

- TR-069[BBF TR069] for “Detailed Service Offer” and specifies extensions to TR-135 STB data model [b_BBF TR135] to store this kind of information.

## Service Navigation: E0

Query mechanism is optionally used to access ECG/ESG/IPG information.

- SOAP[W3C SOAP] overHTTP

  - This is a possible approach for acquiring and retrieving service discovery information using Remote Procedure Call (RPC) [ATIS-0800017] [ETSI TS 102 539].

NOTE - W3C specified Simple Object Access Protocol (SOAP 1.2) as a W3C Recommendation for a framework of exchanging XML structured data between peers.

- XCAP [IETF RFC4825] over HTTP

  - XCAP overHTTP is another method for direct manipulation of elements and attributes within service discovery information [ITU-T J.367][ETSI TS 183 063].

NOTE - XCAP is a lightweight protocol based on HTTP for real-time aspect.

## Service Consumption

### Linear TV services

#### Linear TV service delivery protocol: E5

Linear TV services (characterized as the equivalent of the traditional broadcast like TV and radio) are streamed continuously over IP multicast. The element "Service Location" in the "Detailed Service offer" records gives all the information required to access Linear TV services.

Terminal Devices can receive or stop the reception of Linear TV services. simply by issuing the appropriate multicast control messages.

Linear TV services may be encoded as MPEG-2 Transport Streams [, encapsulated either in RTP (Real-time Transport Protocol) or directly in UDP (User Datagram Protocol)].

Relevant standards:

- [ETSI TS 101 154] Implementation guidelines for the use of Video and Audio Coding in Broadcasting Applications based on the MPEG-2 Transport Stream.

- [IETF RFC 768] User Datagram Protocol (UDP)

- [IETF RFC 2250] RTP Payload Format for MPEG1/MPEG2 Video

- [IETF RFC 3550] RTP: A Transport Protocol for Real-Time Applications

- [ITU-T H.610] Full service VDSL - System architecture and customer premises equipment.

#### Linear TV service control protocol: E5

Relevant standards:

- IGMP version 2 [IETF RFC 2236] for Linear TV delivery over multicast (“push mode”)

- IGMP version 3 [IETF RFC 3376]  for Linear TV delivery over multicast (“push mode”)

- MLD version 2 [IETF RFC 3810]  for Linear TV delivery over multicast (“push mode”)

### VoD services: E1, E6

#### Content on Demand service delivery Protocol: E6

Content on Demand services are encoded as MPEG-2 Transport Streams encapsulated either in RTP (Real-time Transport Protocol) or directly in UDP (User Datagram Protocol).

Relevant standards:

- [ETSI TS 101 154]	Implementation guidelines for the use of Video and Audio Coding in Broadcasting Applications based on the MPEG-2 Transport Stream.

- [IETF RFC 768]	User Datagram Protocol (UDP)

- [IETF RFC 2250]	RTP Payload Format for MPEG1/MPEG2 Video

- [IETF RFC 3550]	RTP: A Transport Protocol for Real-Time Applications

- [ITU-T H.610]	Full service VDSL – System architecture and customer premises equipment

#### Content-on-demand service control protocol: E6

Terminal Devices can receive or stop the reception of Content on Demand services simply by issuing the appropriate RTSP messages.

Relevant standards:

- [IETF RFC 2326], (RTSP) Real Time Streaming Protocol

### Download services

Content Download services allow for the download of content items to a local storage of the IPTV terminal. A Content Download service may support two different service roles:

- The push download service mode that is defined as a distribution of content items where the distribution decision is taken by the service provider, without explicit request from the user.

- The pull download service mode provides for download of content items at the explicit request of a user.

This implies the support of both multicast and unicast download.

Download of a file from a single server and download of the file in chunks from multiple servers are supported.

A reception reporting procedure allows the IPTV terminal devices to report the successful download of content.

#### Download service delivery Protocol: E5, E6

Relevant standards:

- [IETF RFC 959]	File Transfer Protocol (FTP)

- [IETF RFC 3926] “FLUTE - File Delivery over Unidirectional Transport”

  - FLUTE protocol may be combined with a file repair mechanisms

- [IETF RFC 2616] "Hypertext Transfer Protocol - HTTP/1.1"

#### Download service control Protocol: E5, E6

Relevant standards:

- [IETF RFC 959]	File Transfer Protocol (FTP)

- [IETF RFC 2616]	Hypertext Transfer Protocol – HTTP/1.1”

### Other services

#### Details of this item are for further study.

[TBD]

# Other relevant-protocols

Well-known protocols independent of specific IPTV services are introduced in this clause.

## Time Synchronization

It is useful to synchronize automatically all timers of IPTV terminal devices connecting a network. Following well-known protocol provides such functionality.

Relevant standards:

- [IETF RFC 4330]	Simple Network Time Protocol (SNTP)

## Security/Privacy

IPTV terminal devices are required to support secure communication depending on types of IPTV services (e.g., e-commerce).

Relevant standards:

- [IETF RFC 2818]	HTTP over TLS (HTTPS)

## Performance monitoring: E6

IPTV terminal devices can periodically send performance monitoring reports to the network side by issuing the appropriate RTSP or RTCP messages (via Interface E6).

Relevant standards:

- [IETF RFC 2326], (RTSP) Real Time Streaming Protocol

- [IETF RFC 3611], (RTCP XR) RTP Control Protocol Extended Reports

## Terminal Device management

Details of this item are for further study.

## Notification

### Common alerting protocol

# The Common Alerting Protocol (CAP) [ITU-T X.1303] is an XML [W3C XML] based data format for exchanging public warnings and emergencies between alerting technologies. CAP allows a warning message to be consistently disseminated simultaneously over many different warning systems to many applications. Detailed relation between CAP and an emergency alert service as an event handling application is consulted in [ITU-T H.740].

# Delivery methods

Relevant standards for format and encoding of application event record or metadata in ITU-T Recommendations are as follows:

- Matadata is encoded in XML [W3C XML].

- Application event record can optionally be encoded in XML.

- Matadata and application event record encoded in XML can optionally be compressed by using Fast Infoset [ITU-T X.891], ZLIB including GZIP format [ETSI TS 102 472] or BiM [ISO/IEC 23001].

Relevant standards of transport mechanisms for the delivery of application event and metadata over IP in ITU-T Recommendation are as follows:

- HTTP version 1.1 [IETF RFC 2616] for application event or metadata delivery over unicast.

- HTTP over TLS [IETF RFC 2818] for application event or metadata delivery over unicast with secure manner.

- TFTP [IETF RFC1350] for application event or metadata delivery over unicast.

These transports mechanisms for the delivery are not limited in the above lists.

## Audience measurement

Audience measurement is measuring end-user behaviour with end-user permission in IPTV services [b_ITU-T H.IPTV-AM.0]. It can be measured passively without end-user’s consciousness by detecting application events in IPTV services with end-user permission. It can also be measured actively by requesting end-user interaction.

The two types of IPTV service supported by audience measurement are:

- Distributed content services. Examples include linear TV, VoD (Video on Demand), n-PVR (network personal video recorder), and c-PVR (consumer personal video recorder)

- Interactive services (see [b_ITU-T Y.Sup5]).  Examples include games, learning, commerce, and information access.

### Audience measurement protocols: E10

Relevant standards:

- SOAP[W3C SOAP] over HTTP/HTTPS for delivery of audience measurement data

- TFTP [IETF RFC1350] for delivery of audience measurement data if the audience measurement data is file-based format

- XMPP [IETF RFC3920][IETF RFC3921] for delivery of audience measurement data. Rather than storing files into batches like TFTP, it transmits small messages in real time

- UDP over IPsec [IETF RFC4301] for delivery of audience measurement data

# Annex A

<Annex Title>

(This annex forms an integral part of this Recommendation)

[TBD]

Appendix I
RTSP methods for VoD

(This appendix does not form an integral part of this Recommendation)

RTSP provides the way of VoD control via reference point E6 in Figure 1-1. [IETF RFC 2236] specifies several methods for controlling contents delivery and also describes requirement levels of those methods. However there are practical sets of the methods for VoD services partially different from the requirement levels.  Following table shows the profiles of RTSP methods for VoD services according to standards [ETSI TS 102 034][ETSI TS 183 063].

Table A-1: Profile of RTSP methods (M: Mandatory, O: Optional, -: No Use)

| Method | Direction C: Client S: Server | RFC2326 | DVB-IPTV Profile | Other Possible Profile | General action of IPTV terminal device per method |
| --- | --- | --- | --- | --- | --- |
| DESCRIBE | C → S | Recommended | Recommended | O (Note 1) | Retrieve description of a presentation |
| ANNOUNCE | C → S | Optional | Optional | - | Post description of a presentation |
| ANNOUNCE | S → C | Optional | Recommended | M | Receipt session description are updated |
| GET_PARAMETER | C→ S | Optional | Recommended | - | Retrieve playback information |
| GET_PARAMETER | S → C | Optional | Optional | - |  |
| OPTIONS | C→ S | Required | Required | O (Note 2) | Get a list of methods implemented in a VOD server |
| OPTIONS | S → C | Optional | Optional | - |  |
| PAUSE | C → S | Recommended | Required | M | Interrupt streaming temporarily |
| PLAY | C → S | Required | Required | M | Start or resume sending data |
| RECORD | C → S | Optional | - | - | Initiate recording range |
| REDIRECT | S → C | Optional | Required (Note3) | - | Connect to another server |
| SETUP | C → S | Required | Required | M | Request the server to allocate resources for a stream and start an RTSP session. |
| SET_PARAMETER | C → S | Optional | - | - | Set playback position parameters |
| SET_PARAMETER | S → C | Optional | - | - |  |
| TEARDOWN | C → S | Required | Required | M | Stop streaming delivery |

NOTE 1 - In the case IPTV terminal devices did not get content information (e.g., URL, delivery protocol), it is required to request description of the media (e.g. SDP [b_IETF RFC4566]) via DESCRIBE message [ETSI TS 183 063].

NOTE 2 - Option methods may not be used in case all the specified methods are mandatory.  The OPTION methods is included simply to allow for future compatibility and easier introduction of new optional methods [ETSI TS 183 063].

NOTE 3 – DVB assumes that REDIRECT method is a way to treat load balancing issues..

Bibliography

[b_BBF TR135]	Broadband Forum TR-135 (2007), Data model for a TR-069 enabled STB

[b_IETF RFC 4566] IETF RFC4566 (2006), SDP: Session Description Protocol

[b_ETSI TS 102 034] ETSI TS 102 034 v1.3.1 (2007), Digital Video Broadcasting (DVB); Transport of MPEG-2 TS Based DVB Services over IP Based Networks.

[b_H.IPTV-AM.0] ITU-T draft Rec. H.IPTV-AM.0 IPTV-GSI-DOC-375 (2010), “IPTV application event handling: Overall aspects of audience measurement for IPTV services

End proposed descriptions

__________________