| INTERNATIONAL TELECOMMUNICATION UNION |  | IPTV-GSI |
| --- | --- | --- |
| TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  | IPTV-GSI-C-518 |
|  |  | English only Original: English |
| Question(s): | 13/16 | Pune, India, 13-17 December 2010 |
| CONTRIBUTION |  |  |
| Source: | NTT |  |
| Title: | Proposed additions to HSTP.IPTV-HRM.2 |  |

Summary

# Introduction

At the last Q13/16 during IPTV GSI event in Singapore, 19- 27 September 2010, the new technical paper “Harmonization of MAFR series with multiple content sources” (HSTP.IPTV-HRM.2) was created.  This contribution proposes to add text to 7.1 and 7.2 to describe how to get MAFR content while watching a linear IPTV as well as how to send MAFR content over DSM-CC carousel.

# Proposal

This contribution proposes the following:

- To add the following text in 7.1. Transmitting links to IPTV MAFR Content using hyperlink descriptors in MPEG-2 TS of the current HSTP.IPTV-HRM2

---  Start of the proposed text -----------------

The following procedure is a typical example to start a MAFR content while watching a linear IPTV content.

- While a linear IPTV content is being watched, pressing a button, (usually a “default” button specifically designated)  the MAFR user agent (“browser”) is started.

- A linear IPTV service provider is identified by a table in the PSI/SI; This table will provide the information (URL) about the location of the portal to be associated with the linear IPTV service provider. This information is expected to be contained in the “hyperlink descriptor”.

- The user agent then make a query to the URL in the hyperlink descriptor with the information about, e.g. the network ID, transport stream ID and service ID.

- The portal MAFR content is expected to be able to obtain the service ID in the query sent from the user agent and also to be able to embed and display the linear TV content corresponding to the service ID in the MAFR content.

---  end of the proposed text ------------------------

- The information on “hyperlink descriptor”, as described above, should be described at least optionally in the clause describing how to discover a service using SI  in the current H.770.

- 7.2  “Transmitting ‘startup’ file with broadcast content” should make references to H.761, where Carousel is described.

---------------------------------