| # ITU - Telecommunications Standardization Sector STUDY GROUP 16 Question 6 Video Coding Experts Group (VCEG) _________________ 15th Meeting: Pattaya, Thailand, 3-7 Dec., 2001 | Document VCEG-O50 Filename: VCEG-O50.doc Generated: 25 November ’01 |
| --- | --- |

| Question: | Q.6/SG16 (VCEG) |  |  |
| --- | --- | --- | --- |
| Source: | # Foo Teck-Wee, Shen Sheng-Mei, # Lee Chak-Joo (Panasonic Singapore Laboratories Pte Ltd) Shinya Kadono (Matsushita Electric Industrial Co., Ltd) | Tel: Fax: Email: | +65-5505466 +65-5505459 twfoo@psl.com.sg shen@psl.com.sg |
| Title: | Medium Complexity Loop-Filter |  |  |
| Purpose: | Proposal |  |  |

- Introduction

This document proposes changes to the TML loop filter in order to reduce its complexity and at the same time, maintained as close as possible the quality achieved by the original high complexity loop filter.

- Loop-filter Proposal

A loop filter may reduce the annoying blocking artefacts inherent in low bitrate coding and produces smaller bitstream because of less prediction error from the deblocked reference frame. However, loop filter introduces extra complexity to the codec.

  - Skip Mode

The conditions for checking which block to deblock will be the same as the current conditions in TML8.5 except for additions skipping for certain modes as explained below. The calculation of Strength is also used as criteria for skip mode but will not be used for the clipping as the clipping function is removed.

  - Concurrent checking

Instead of checking separately for the number of pixels to filter on either side, the proposed loop-filter will check for the number of pixels to filter concurrently. Only one check is made to determine the number of pixels to filter. The number of pixel to filter will be the same for both sides. This helps to reduce the complexity of the loop-filter.

dif1 = p0 – q0		// this will be reused for 1 pixel filtering

dif2 = p1 – q1		// this will be reused for 1 pixel filtering

dif1a = | dif1 |

dif2a = | dif2 |

| Conditions | Cond. A | Cond. B | n |
| --- | --- | --- | --- |
| Cond. 1 | dif1a >=  | dif2a <  | 0 |
| Cond. 2 | dif1a >=  | dif2a <  * 2 | 0 |
| Cond. 3 | dif1a >=  | dif2a >  | 0 |
| Cond. 4 | dif1a <  | dif2a <  | 2 |
| Cond. 5 | dif1a <  | dif2a <  * 2 | 1 |

Table 1: Conditions for determining the n

| QP | 0 1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 20 21 22 23 24 25 26 27 28 29 30 31 |
| --- | --- |
|  | 0 0 0 0 0 0 0 3 4 4 5 6 7 8 9 13 13 15 17 17 20 22 25 27 29 30 33 34 36 45 50 50 |
|  | 0 0 0 0 0 0 0 0 1 1 2 3 4 5 6 9 10 11 13 14 16 18 20 22 24 24 29 29 34 36 35 35 |
|  | 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 0 1 1 2 2 2 |

Table 2: Threshold values used in number of pixels

to filter determination

See Figure 1 for flow-chart of the above module.

Figure 1: Concurrent determination of number of pixels to filter (0,1 or 3 pixels)

2.3	Filtering

The way to filter 3 pixels and 1 pixel are the same as in current implementation. The only difference with current implementation is that there are no 2 pixels filtering.

Figure 2: Filtering operation

The conditions for filtering 0, 1 or 3 pixels are as in Table 3.2.

| Number of pixels to filter | Conditions |
| --- | --- |
| 0 | n = 0 |
| 1 | Condition 1: n==1 or Condition 2: n==2 and dif2a-dif1a >=my or dif1a < 2 |
| 3 | n == 2 ¦dif2a – dif1a¦ < myu dif1a >= 2 |

Table 3: Conditions for filtering 0, 1 or 3 pixels

The operations for filtering 1 and 3 pixels are the same as in the current implementation.

See Figure 2 for flow-chart of the above module.

- Experiments

Experiments were conducted following the conditions recommended in [1] but we used less frames (>100) instead of the number of frames stated in the document due to time constraint. The proposed loop-filter was integrated into the TML-8.5 software to compare against the TML-8.5 codec. Refer to Appendix for the rate distortion curve. More data is in the enclosed file allseq3.xls.

  - Subjective Viewing

The subjective quality of intra-coded frames (I frames) is generally comparable to TML8.5.

For inter-coded frames (like P and B frames), the visual quality is subjectively better than TML8.5.

Side-by-side comparison between TML8.5 and PSL will be shown during the meeting.

  - Complexity Reduction

The complexity for the new schemes was analysed and calculated as described in [2]. The new algorithm reduces the complexity from 50.4% to 32.9% for Intra coded frames, and from 31.0% to 19.9% for Inter coded frames.

| Picture type | TML8.5 | PSL | Reduction |
| --- | --- | --- | --- |
| intra coded | 50.4 | 32.9 | 17.5 |
| inter coded | 31.0 | 19.9 | 11.1 |

Table 4: Complexity of loop-filter in the overall decoding process according

to calculation methods described in [2].

See enclosed file for the complexity calculation (complexity.xls).

Table 5: Complexity study of TML8.5 Intra Coding

Table 6: Complexity Study of PSL Intra Coding

Table 7: Complexity Study of TML8.5 Inter Coding

Table 8: Complexity Study of PSL Inter Coding

  - Execution Speed

Overall complexity of the decode is measured using ANSI C clock() function. Complexity of loop-filter is measured using RDTSC instruction that can be found in Pentium and newer Intel x86 CPUs as used.  Reduction of 6% to 30% is recorded for the new in-loop filter. Kindly refer to exespeed.xls for comparison of execution speed of TML8.5 and PSL in-loop filter.

See attached file (exespeed.xls) for details.

Platform used:

Pentium III 1 GHz

256 MB RAM

30 GB hard disk

- Summary and Suggestion

We have demonstrated the advantages of the proposed loop-filter with its significantly reduced complexity when compared with the original loop-filter, and the picture quality close to the latter. With the proposed loop-filter, real-time implementation could be easily achieved and at the same time contributing to the reduced bit usage and better quality video.

However there is a balance between complexity and coding quality gain, especially to the implementation for the portable device where the cost effective would be the key factor.

As for in-loop filter, its coding gain is not coming from many bits saving or much higher PSNR, but from subjective quality, especially in the case of very low bit rate coding.

In the case of the low bit rate, QP is large enough to result in subjective quality drop so much that lost information cannot be recovered. Even a perfect and deliberate in-loop filter also cannot help, but the cost of such filter is extremely large to a portable device. On the other hand, it is felt that removing the annoying blocking noise with certain extent is enough in such case.

What we suggest that if we can define profiles, the proposed medium in-loop filter can be used in (future) middle complexity Profile(s) targeting on devices with such complexity or capabilities.  The current TML filter is used in Main profile, and the current TML filter with deringing filter is used in High profile for MPEG2 like bitrate, as shown in the following table.

| Profiles/Filter | PSL Medium In-loop Filter | TML8.5 In-loop Filter | Deringing Filter | Notes |
| --- | --- | --- | --- | --- |
| Simple Profile | X |  |  | No 1/8 pel interpolation |
| Main Profile | X* | X |  | With 1/8 pel interpolation |
| High Profile | X* | X | X | With 1/8 pel interpolation |
| Other profiles |  |  |  |  |

Table 9: Profiling with Different Inloop Filter

Note:

X* -- only a small sub-set unit used for filter determination is required to implement on top of the current TML8.5 Inloop filter in the case of Main Profile and High Profile.

We would also like to recommend that the loop-filter in the current TML implementation be removed from the baseline profile due to the complexity reason given above. At the same time, future higher profiles that demand high fidelity (such as but not limited to Studio profile) would definitely need to ensure that the coded video is kept as “close” as possible to the original material.

References

[1]	Gisle Bjontegaard, “Recommended Simulation Conditions for H.26L”, document VCEG-M75, ITU-T Video Coding Experts Group (VCEG) Meeting, Austin, TX, USA, 2-4 April 2001.

[2]	Minhua Zhou, “Benchmark Analysis of H.26L Decoder Functional Blocks”, document VCEG-N23, ITU-T Video Coding Experts Group (VCEG) Meeting, Santa Barbara, CA, USA, 24-28 September 2001.

Appendix

# JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Matsushita Electric Industrial Co., Ltd. |  |  |
| Mailing address | 1006, Kadoma, Kadoma-Shi, Osaka, 571-8501 |  |  |
| Country | Japan |  |  |
| Contact person | Shinya Kadono |  |  |
| Telephone | +81-6-6900-9689 |  |  |
| Fax | +81-6-6900-9674 |  |  |
| Email | kadono@drl.mei.co.jp |  |  |
| Place and date of submission | Pattaya, Thailand, 3 December 2001 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Medium Complexity Loop-Filter |  |
| Contribution number |  | VCEG-050/VCEG-050r1 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | - The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
| X | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: