| # Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 3rd Meeting: Fairfax, Virginia, USA, 6-10 May, 2002 | Document: JVT-C043 Filename: JVT-C043.doc |
| --- | --- |

| Title: | GMVC and GMC with B-picture |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hideaki Kimata NTT Cyber Space Laboratories 1-1 Hikari-no-oka, Yokosuka, JAPAN | Tel: Email: | +81 468 59 3124 kimata.hideaki@lab.ntt.co.jp |
| Source: | NTT |  |  |

_____________________________

1. Introduction

GMC and GMVC have efficient functionality to express global motions. Since global motions exist naturally in captured videos, such functionality is well-suited for coding of natural videos. In the last few JVT meetings, GMC and GMVC for P-pictures have been proposed in various figures [1,2,3]. In [2], the syntax of GMVC is successfully integrated into MB types. And in [3], the high coding efficiency of GMC is demonstrated with other syntax than [2]. In this documents, the upgraded syntax from [2] for GMC and GMVC is proposed for P-pictures. And additionally GMC scheme for B-picture is also proposed.

2. GMC

2.1. ME

At the encoder side, motion estimation of global motions (GME) is needed. In GME, the global motion vector is estimated and the additional reference picture by warping is generated, and such picture is used for usual motion estimation. Warped picture is generated from the original picture with the same method of motion compensation, which is described in MC section. Note that for GMVC, GME only provide the global motion vector. Generation of warped picture is not necessary.

Figure 1. Typical block diagram at the encoder side in GMC

GME is one of essential features on GMC. There are many methods reported mainly in terms of the improvement of global motion accuracy and the reduction of estimation complexity. In [4,5], one of high accuracy but low complexity methods is described. The summary of it is as follows. At first, motion vectors of all MBs between two original pictures are estimated by conventional motion estimation, such as block matching. Then three global motion parameter (vZ,vH,vV) are estimated by statistical calculation from motion vectors of all MBs, Global motion parameter vZ expresses zoom feature, and vH or vV expresses horizontal or vertical pan feature respectively. The relation between the original position (x,y) and the warped point (x’,y’) is as follows.

x'y'=vZxy+vHvV						(1)

And finally the global motion vector, which consists of four parameters (v00,vH0,v0V,vHV), is calculated from (vZ,vH,vV). Global motion vector (v00,vH0,v0V,vHV) expresses motion information of pixels at the corners of the picture. From this global motion vector, the motion information at each pixel (x,y) is calculated by (2),

v(x,y)=r0+roundrxVx+ryHy+rxyxyHV					(2)

where H or V is width or height of picture respectively, and r0,rx,ry,rxyare defined as follows. The exact formula is described in the explanation of the semantic of GMC blocks.

r0=v00rx=vH0−v00ry=v0V−v00rxy=v00−vH0−v0V+vHV						(3)

Figure 2. Global motion vector used for calculating motion vectors at each pixel

Usual motion estimation in the warped reference picture can be additionally applied. From this motion estimation, the motion information at each pixel (x,y) is calculated by (4), instead of (2),

v(x,y)=r0+mv(x,y)+roundrxV(x+mvx(x,y))+ryH(y+mvy(x,y))+rxy(x+mvx(x,y))(y+mvy(x,y))HV 	(4)

where mv(x,y) expresses the motion vector at the Block in which the pixel (x,y) exists. The exact formula is described in the explanation of the semantic of GMC blocks.

Expression of global motion vector as (vZ,vH,vV) is effective on execution of GME. When GME is conducted between two original pictures, global motion vectors between discontinuous pictures can be calculated from addition of several global motion vectors between continuous pictures. For instance, for three subsequent pictures P1, P2, and, P3, the global motion vector between P1 and P3 (GMV13=(v13Z,v13H,v13V)) is calculated by (5), where GMV12=(v12Z,v12H,v12V) or GMV23=(v23Z,v23H,v23V) expresses the global motion vector between P1 and P2, or P2 and P3 respectively.

v13Zv13Hv13V=v12Zv23Zv23Zv12H+v23Hv23Zv12V+v23V						(5)

This scheme is effective for reducing the complexity of GME in case of P2 is coded as B-picture, in a fast motion scene.

Figure 3. GME for discontinuous pictures

In terms of RD optimization including GMC, there seems no confidential paper available. And in the current JM software RD optimization is only applied within the motion estimation and the mode decision of each MB. Therefore currently GMC can be tested under the adverse situation where GMC is not optimized, in contrast to the other MB basis techniques.

2.2. MC

JM1 originally has 1/4 pel MC mode and 1/8 pel MC mode [6]. In 1/4 pel MC, image at 1/2 pel position is at first generated by interpolation of surrounding image at pixel position using the 6 tap filter, and next, image at 1/4 pel position is generated by linear interpolation of surrounding images at 1/2 pel positions. In 1/8 pel MC, image at 1/8 pel position is generated by interpolation of surrounding image at 1/4 pel position using the 8 tap filter.

In the GMC block, warped image is always generated in the 1/8 pel accuracy. When 1/4 pel MC mode, motion information at each pixel is calculated, and luminance image at 1/8 pel position is generated by linear interpolation of surrounding images at 1/2 pel positions. Figure 4 illustrates the arrangement of integer and half pel positions (H1, H2, H3,and H4), and 1/8 pel positions (a,b,c,d,e,f,g,h,i,j,k,l,m,n,o,p,q,r,s,t, and u). Luminance image data at each 1/8 pel position is calculated as follows.

Figure 4. Interpolation for GMC in 1/4 pel MC mode

a = (3 H1 + H2)>>2

b = (H1 + H2)>>1

c = (H1 + 3 H2)>>2

d = (3 H1 + H3)>>2

e = (9 H1 + 3 (H2 + H3)+ H4)>>4

f = (3 (H1 + H2)+ H3 + H4)>>3

g = (9 H2 + 3 (H1 + H3)+ H4)>>4

h = (3 H2 + H4)>>2

i = (H1 + H3)>>1

j = (3 (H1 + H3)+ H2 + H4)>>3

k = (H1 + H2 + H3 + H4)>>2

l = (3 (H2 + H4)+ H1 + H3)>>3

m = (H2 + H4)>>1

n = (H1 + 3 H3)>>2

o = (9 H3 + 3 (H1 + H4)+ H2)>>4

p = (3 (H3 + H4)+ H1 + H2)>>3

q = (9 H4 + 3 (H2 + H3)+ H1)>>4

r = (H2 + 3 H4)>>2

s = (3 H3 + H4)>>2

t = (H3 + H4)>>1

u = (H3 + 3 H4)>>2

And when 1/8 pel MC mode, luminance image at 1/8 pel position is generated by the same method of usual MC.

And chrominance image data is generated in the same way of the normal MC.

The method of motion information calculation is described in the explanation of the semantic of GMC blocks.

2.3. Syntax

2.3.1. Slice layer

At the Slice layer, GMC type (GMCT) to switch whether the GMC block exists or not is defined, and coded just after Picture type (Ptype). And when GMCT is set to 1, global motion vector (r0) is coded sequentially just after GMCT, and when GMCT is set to 2, global motion vector (r0,rx,ry,rxy) is coded sequentially just after GMCT. The accuracy of the global motion vector (r0,rx,ry,rxy) is the same for the normal MC.

Table 1: Code number of GMCT

| Code number | GMCT |
| --- | --- |
| 0 | No GMC block exists |
| 1 | Translated GMC block exists |
| 2 | Warped GMC block exists |

2.3.2. MB layer

The syntax diagram of MB layer is not changed. The modified or additional MB mode is defined. This MB mode is different depending on GMCT.

The MB mode in P-picture when GMCT is on, where code number of GMCT is whether 1 or 2, is defined as listed in Table 2, and when GMCT is off, where code number of GMCT is 0, it is the same as for the JM1. The mode for 8x8 sub partitioning in P-picture is defined in Table 3 for when GMCT is on. And it is defined in Table 4 for when GMCT is off, where this is the same for Mcopy_r1 mode [7].

Table 2: Macroblock modes for P-frames when GMCT is on

| Code number | Macroblock mode | CABAC Binarization |
| --- | --- | --- |
| 0 | Skip | 0 |
| 1 | 16x16 | 1000 |
| 2 | 16x8 | 1011 |
| 3 | 8x16 | 10100 |
| 4 | 8x8 (split) | 1001 |
| 5 (UVLC only) | 8x8 (split, all ref=0) |  |
| 6 | GMC16x16 | 10101 |
| 7 | Intra4x4 | 110 |
| 8 … | Intra16x16 | 111 |

Table 3: Modes for 8x8 sub-partitions in P-frames when GMCT is on

| Code number | 8x8 partition mode | CABAC Binarization |
| --- | --- | --- |
| 0 | 8x8 | 1 |
| 1 | 8x4 | 000 |
| 2 | 4x8 | 0011 |
| 3 | 4x4 | 00100 |
| 4 | Intra | 010 |
| 5 | GMC8x8 | 011 |
| 6 | GMCLMV8x8 | 001010 |
| 7 | GMCLMV4x4 | 001011 |

Table 4: Modes for 8x8 sub-partitions in P-frames when GMCT is off

| Code number | 8x8 partition mode | CABAC Binarization |
| --- | --- | --- |
| 0 | 8x8 | 1 |
| 1 | 8x4 | 000 |
| 2 | 4x8 | 0011 |
| 3 | 4x4 | 0010 |
| 4 | Intra | 010 |
| 5 | 8x8noMV | 011 |

The MB mode in B-picture when GMCT is on is defined as listed in Table 5, and the mode for 8x8 sub partitioning in B-picture is defined in Table 6. When GMCT is off, these are the same as for the JM1.

Table 5: Macroblock modes for B-frames when GMCT is on

| Code number | Macroblock mode | 1. block | 2. block | CABAC Binarization |
| --- | --- | --- | --- | --- |
| 0 | Direct |  |  | 0 |
| 1 | 16x16 | Forw. |  | 1000 |
| 2 | 16x16 | Backw. |  | 101 |
| 3 | 16x16 | Bidirect. |  | 110000 |
| 4 | 16x8 | Forw. | Forw. | 110001 |
| 5 | 8x16 | Forw. | Forw. | 110010 |
| 6 | 16x8 | Backw. | Backw. | 110011 |
| 7 | 8x16 | Backw. | Backw. | 110100 |
| 8 | 16x8 | Forw. | Backw. | 110101 |
| 9 | 8x16 | Forw. | Backw. | 110110 |
| 10 | 16x8 | Backw. | Forw. | 110111 |
| 11 | 8x16 | Backw. | Forw. | 111110 |
| 12 | 16x8 | Forw. | Bidirect. | 1110000 |
| 13 | 8x16 | Forw. | Bidirect. | 1110001 |
| 14 | 16x8 | Backw. | Bidirect. | 1110010 |
| 15 | 8x16 | Backw. | Bidirect. | 1110011 |
| 16 | 16x8 | Bidirect. | Forw. | 1110100 |
| 17 | 8x16 | Bidirect. | Forw. | 1110101 |
| 18 | 16x8 | Bidirect. | Backw. | 1110110 |
| 19 | 8x16 | Bidirect. | Backw. | 1110111 |
| 20 | 16x8 | Bidirect. | Bidirect. | 1111000 |
| 21 | 8x16 | Bidirect. | Bidirect. | 1111001 |
| 22 | 8x8(split) |  |  | 111111 |
| 23 | GMC16x16 |  |  | 1001 |
| 24 | Intra4x4 |  |  | 1111010 |
| 25 … | Intra16x16 |  |  | 1111011 |

Table 6: Modes for 8x8 sub-partitions in B-frames when GMCT is on

| Code number | 8x8 partition mode | Prediction | CABAC Binarization |
| --- | --- | --- | --- |
| 0 | Direct |  | 0 |
| 1 | 8x8 | Forw. | 1000 |
| 2 | 8x8 | Backw. | 101 |
| 3 | 8x8 | Bidirect. | 11000 |
| 4 | 8x4 | Forw. | 11001 |
| 5 | 4x8 | Forw. | 11010 |
| 6 | 8x4 | Backw. | 11011 |
| 7 | 4x8 | Backw. | 111000 |
| 8 | 8x4 | Bidirect. | 111001 |
| 9 | 4x8 | Bidirect. | 111010 |
| 10 | 4x4 | Forw. | 111011 |
| 11 | 4x4 | Backw. | 111100 |
| 12 | 4x4 | Bidirect. | 111101 |
| 13 | Intra |  | 11111 |
| 14 | GMC8x8 |  | 1001 |

2.4. Semantic of Skip MB and no residual Direct MB

2.4.1. P-picture

In P-picture, the semantic of Skip mode is changed depending on premeditated conditions. Skip MB can have motion information calculated from motion vectors of surrounding blocks or from global motion vector depending on premeditated conditions.

2.4.1.1. When GMCT is on

When GMCT is on (code number is whether 1 or 2), Skip mode is treated as follows, where motion information of GMC block is calculated by formula (2).

- When more than half of the six neighbor blocks (two left, one up left, two up, one up right blocks) are coded as the GMC block, then the motion information of the MB is calculated from global motion vector. Otherwise move to (2).

- When more than one of the four neighbor blocks (one left, one up left, one up, one up right blocks) has a zero motion vector, the motion vector of the MB is set to zero. Otherwise move to (3).

- The motion vector of the MB is calculated from the median predicted motion vector the same for Inter 16x16 mode is used.

2.4.1.2. When GMCT is off

When GMCT is off (code number is 0), Skip mode is treated as follows. This is the same operation in Mcopy and Mcopy_r1 techniques [8][9].

- When more than one of the four neighbor blocks (one left, one up left, one up, one up right blocks) has a zero motion vector, the motion vector of the MB is set to zero. Otherwise move to (2).

- The motion vector of the MB is calculated from the median predicted motion vector the same for Inter 16x16 mode is used.

2.4.2. B-picture

In B-picture, the semantic of no residual Direct mode is changed depending on premeditated conditions. No residual Direct mode can have motion information calculated from global motion vector depending on premeditated conditions, when GMCT is on.

When GMCT is on (code number is whether 1 or 2), no residual Direct mode is treated as follows, where motion information of GMC block is calculated by formula (2)

- When more than half of the six neighbor blocks (two left, one up left, two up, one up right blocks) are coded as the GMC block, then the motion information of the MB is calculated from global motion vector. Otherwise move to (2).

- The motion vector of the MB is calculated from the same as for the normal Direct mode.

2.5. Semantic of GMC16x16, GMC8x8, GMCLMV8x8, or GMCLMV4x4 mode

GMC16x16 mode expresses the MB whose motion information for luminance MC is calculated from global motion vector using formula (6). And GMC8x8 mode expresses the sub partition whose motion information for luminance MC is calculated from global motion vector using formula (6). When code number of GMCT is 1, the global motion vector (rx,ry,rxy) is set to zero, then formula (7) is used instead of (6).

v(x,y)=2r0+2(rxVx+ryHy+rxyxy)//HV for 14pel MCv(x,y)=r0+(rxVx+ryHy+rxyxy)//HV for 18pel MC			(6)

v(x,y)=2r0 for 14pel MCv(x,y)=r0 for 18pel MC							(7)

For chrominance MC, motion information is calculated by dividing the motion information for the corresponding upper luminance by two.

For median prediction of motion vector for the normal MC block, the motion information at the top left integer position in the block is used. And for direct mode for the B-picture, the motion information at the top left integer position in the block is used.

In this mode, the ref_frame is not coded explicitly, and it is always set to zero.

GMCLMV8x8 mode expresses the sub partition that has motion vector data per an 8x8 partitioning and its motion information for MC is calculated from global motion vector using formula (8). And GMCLMV4x4 mode expresses the sub partition that has motion vector data per an 4x4 partitioning and its motion information for MC is calculated from global motion vector using formula (8). When code number of GMCT is 1, the global motion vector (rx,ry,rxy) is set to zero, then formula (9) is used instead of (8).

v(x,y)=2r0+2mv(x,y)+2(4rxV(4x+mvx(x,y))+4ryH(4y+mvy(x,y))+rxy(4x+mvx(x,y))(4y+mvy(x,y)))//16HV for 14pel MCv(x,y)=r0+mv(x,y)+(8rxV(8x+mvx(x,y))+8ryH(8y+mvy(x,y))+rxy(8x+mvx(x,y))(8y+mvy(x,y)))//64HV for 18pel MC 			(8)

v(x,y)=2r0+2mv(x,y) for 14pel MCv(x,y)=r0+mv(x,y) for 18pel MC 						(9)

For chrominance MC, motion information is calculated by dividing the motion information for the corresponding upper luminance by two.

For median prediction of motion vector for the normal MC block, zero motion is used. And for direct mode for the B-picture, the motion information at the top left integer position in the block is used.

In this mode, the ref_frame is not coded explicitly, and it is always set to zero.

2.6. Semantic of 8x8noMV mode

8x8noMV mode expresses the sub partition that has no motion vector data, but has transformed coefficient data. This is the same as for Mcopy_r1 technique [8].

2.7. GMVC

2.7.1. GME

Since GMVC does not have warping functionality, in GME for GMVC only global motion vector is calculated.

2.7.2. MC, Syntax, Semantic

This is the same for described in JVT-C021 [7].

3. Evaluation

Experiments to evaluate GMC with GMVC [7], Mcopy [9], and Mcopy_r1 [8], are conducted following almost the same conditions specified in VCEG-N77r1. GMC, GMVC, Mcopy, and Mcopy_r1 are integrated into the JM1.7 software.

As for including B-picture, modified JM1.7 (namely JM1.7+) is used. The encoder of JM1.7 does not have RD optimization of no residual direct mode, so JM1.7+ includes this optimization in order to compare under fair conditions, because GMC_PB in which B-picture has GMC block is evaluated. GMC_P expresses the case where B-picture does not have GMC block. This optimization is done for Mcopy and Mcopy_r1.

The search range of motion estimation, and the initial Inter16x16 motion search for GME are set to –32 to +32. M1 expresses the IPPP structure (no B-picture), and M3 expresses the IBBP structure (2 B-picture between I,P-picture)

3.1. Results

Table 7 shows BD-bitsaving and BD-PSNR in 1/4 pel MC, M1, UVLC cases, and Table 8 shows BD-bitsaving and BD-PSNR in 1/4 pel MC, M1, CABAC cases.

Table 9 shows BD-bitsaving and BD-PSNR in 1/4 pel MC, M3, UVLC cases, and Table 10 shows BD-bitsaving and BD-PSNR in 1/4 pel MC, M3, CABAC cases.

Table 11 shows BD-bitsaving and BD-PSNR in 1/8 pel MC, M1, UVLC cases, and Table 12 shows BD-bitsaving and BD-PSNR in 1/8 pel MC, M3, UVLC cases only for CIF sequence.

These six tables list the results for 1 reference picture case.

And additionally two 5 reference picture case are experimented. Table 13 shows BD-bitsaving and BD-PSNR in 1/4 pel MC, M1, UVLC cases.

All bitrate and PSNR data and PSNR plots are illustrated in JVT-C043.xls.

3.2. Coding efficiency in P-picture (M1)

GMC achieves the highest coding efficiency for all global motion test sequences (which are highlighted by pink), except foreman (CIF) in 1/4 pel MC CABAC case, and GMVC achieves the second highest coding efficiency. Especially for Tempete, about 5.76 % BD-bitsaving is achieved, while Mcopy achieves 2.53 % BD-bitsaving. On average, about 1 % BD-bitsaving over Mcopy is achieved.

3.3. Coding efficiency in P and B-picture (M3)

GMC_PB achieves the highest coding efficiency for most global motion test sequences (which are highlighted by pink), while GMC_P or GMVC achieves the highest for some sequences (the difference from GMC_PB is within 0.1 % range). And GMC_P achieves the second highest coding efficiency and GMVC achieves the third highest coding efficiency. Especially for Stefan, about 9.68 % BD-bitsaving is achieved, while Mcopy achieves 0.32 % BD-bitsaving. On average, about 2.5 % BD-bitsaving over Mcopy is achieved.

4. Conclusion

We propose GMC with which Mcopy_r1 technique is combined. GMC (GMC_PB) achieves the highest coding efficiency with and without B-pictures. GMVC achieves the second highest coding efficiency.

The difference between GMC and GMVC is mainly 2points. One is that the unit of motion vector generation. In GMC motion vector is generated per each pixel, however in GMVC it is generated per each 4x4 block. The other is that GMC has two additional sub partitioning modes. These modes can refine motion information on the globally moved picture. And GMC and GMVC have the functionality to switch frame by frame to on or off. When GMC is off, Mcopy_r1 technique works.

As for decoder complexity, the additional complexity on Mcopy and Mcopy_r1 techniques is negligible. The additional complexity on GMVC is a bit more and it on GMC is larger. However, this is not the case for translation global motion, where motion vector is calculated by just one addition.

Therefore we propose to adopt GMC where additional one flag to switch GMC or GMVC is added in P-picture, and to adopt GMC_PB. And we propose to continue to study complexity of GMC and GMVC.

5. Reference

[1] Hideaki Kmata, “Proposal of Global Motion Compensation enhancing Global Motion Vector Coding Scheme,” VCEG-O29, ITU-T Video Coding Experts Group (VCEG) Meeting, 2001.

[2] Shijun Sun, “Global Motion Vector Coding (GMVC),” JVT-B019, Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) Meeting, 2002.

[3] Hideaki Kimata, “GMVC and GMC switched by MV,” JVT-B046, Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) Meeting, 2002.

[4] Kazuto Kamikura and Hiroshi Watanabe, "Video coding for digital storage media using hierarchical interframe scheme," Visual Communications and Image Processing '90, SPIE, Vol.1360, pp.1540-1550, 1990.

[5] Kumi Jinzenji et al., “Algorithm for Automatically Producing Layered Sprites by detecting Camera Movement,” IEEE ICIP, vol.1, pp.767-770, 1997.

[6] “Working Draft Number 2, Revision 4 (WD-2 rev 4),” JVT-B118r4, Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) Meeting, 2002.

[7] Shijun Sun et al, “Experimental results of GMVC and Motion Copy,” JVT-C021, Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) Meeting, 2002.

[8] Shijun Sun, “Additional zero-motion subblock for Motion Copy,” JVT-C022, Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) Meeting, 2002.

[9] Jani Lainema, “Skip mode motion compensation,” JVT-C027, Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) Meeting, 2002.

Table 7 BD-bitsaving and BD-PSNR in 1/4 pel MC, M1, UVLC

| sequences | BD-bitsave [%] BD-PSNR [dB] | GMC | GMVC | Mcopy | Mcopy_r1 |
| --- | --- | --- | --- | --- | --- |
| container_qcif | bitsave | 0.55 | 0.55 | 0.59 | 0.61 |
|  | PSNR | 0.02 | 0.02 | 0.03 | 0.03 |
| news_qcif | bitsave | 0.42 | 0.42 | 0.45 | 0.47 |
|  | PSNR | 0.03 | 0.03 | 0.03 | 0.03 |
| foreman_qcif | bitsave | 3.95 | 3.58 | 3.11 | 2.98 |
|  | PSNR | 0.22 | 0.20 | 0.17 | 0.17 |
| silent_qcif | bitsave | -0.85 | -0.85 | -0.61 | -0.79 |
|  | PSNR | -0.04 | -0.04 | -0.03 | -0.04 |
| stefan_qcif | bitsave | 3.16 | 1.59 | 0.90 | 1.13 |
|  | PSNR | 0.18 | 0.09 | 0.05 | 0.06 |
| paris_cif | bitsave | 0.37 | 0.37 | -0.06 | 0.38 |
|  | PSNR | 0.02 | 0.02 | 0.00 | 0.02 |
| mobile_cif | bitsave | 5.21 | 3.88 | 3.63 | 3.76 |
|  | PSNR | 0.26 | 0.19 | 0.18 | 0.19 |
| tempete_cif | bitsave | 5.76 | 3.95 | 2.53 | 2.62 |
|  | PSNR | 0.25 | 0.17 | 0.11 | 0.11 |
| coast_cif | bitsave | 4.86 | 4.69 | 4.19 | 4.16 |
|  | PSNR | 0.18 | 0.17 | 0.15 | 0.15 |
| stefan_cif | bitsave | 4.20 | 3.15 | 2.44 | 2.48 |
|  | PSNR | 0.22 | 0.16 | 0.13 | 0.13 |
| foreman_cif | bitsave | 9.20 | 8.99 | 8.30 | 8.13 |
|  | PSNR | 0.46 | 0.45 | 0.41 | 0.40 |
| average | bitsave | 3.35 | 2.76 | 2.31 | 2.36 |
|  | PSNR | 0.16 | 0.13 | 0.11 | 0.11 |
| average (GM) | bitsave | 4.54 | 3.73 | 3.14 | 3.16 |
|  | PSNR | 0.22 | 0.18 | 0.15 | 0.15 |

Table 8 BD-bitsaving and BD-PSNR in 1/4 pel MC, M1, CABAC

| sequences | BD-bitsave [%] BD-PSNR [dB] | GMC | GMVC | Mcopy | Mcopy_r1 |
| --- | --- | --- | --- | --- | --- |
| container_qcif | bitsave | -0.28 | -0.28 | -0.19 | -0.02 |
|  | PSNR | -0.01 | -0.01 | -0.01 | 0.00 |
| news_qcif | bitsave | 0.22 | 0.22 | -0.14 | 0.34 |
|  | PSNR | 0.01 | 0.01 | -0.01 | 0.02 |
| foreman_qcif | bitsave | 2.41 | 2.05 | 1.65 | 1.85 |
|  | PSNR | 0.13 | 0.11 | 0.09 | 0.10 |
| silent_qcif | bitsave | -0.49 | -0.49 | -0.90 | -0.36 |
|  | PSNR | -0.02 | -0.02 | -0.04 | -0.02 |
| stefan_qcif | bitsave | 2.55 | 1.01 | 0.11 | 0.25 |
|  | PSNR | 0.14 | 0.06 | 0.00 | 0.01 |
| paris_cif | bitsave | 0.41 | 0.41 | 0.29 | 0.35 |
|  | PSNR | 0.02 | 0.02 | 0.02 | 0.02 |
| mobile_cif | bitsave | 3.56 | 2.26 | 2.04 | 1.84 |
|  | PSNR | 0.17 | 0.11 | 0.10 | 0.09 |
| tempete_cif | bitsave | 5.15 | 3.61 | 1.37 | 1.52 |
|  | PSNR | 0.22 | 0.15 | 0.06 | 0.06 |
| coast_cif | bitsave | 2.85 | 2.41 | 1.46 | 1.73 |
|  | PSNR | 0.10 | 0.08 | 0.05 | 0.06 |
| stefan_cif | bitsave | 2.82 | 1.77 | 0.95 | 1.12 |
|  | PSNR | 0.14 | 0.09 | 0.05 | 0.06 |
| foreman_cif | bitsave | 3.43 | 3.22 | 3.90 | 4.18 |
|  | PSNR | 0.15 | 0.14 | 0.17 | 0.18 |
| average | bitsave | 2.06 | 1.47 | 0.96 | 1.17 |
|  | PSNR | 0.10 | 0.07 | 0.04 | 0.05 |
| average (GM) | bitsave | 2.85 | 2.04 | 1.44 | 1.56 |
|  | PSNR | 0.13 | 0.09 | 0.07 | 0.07 |

Table 9 BD-bitsaving and BD-PSNR in 1/4 pel MC, M3, UVLC

| sequences | BD-bitsave [%] BD-PSNR [dB] | GMC_P | GMC_PB | GMVC | Mcopy | Mcopy_r1 |
| --- | --- | --- | --- | --- | --- | --- |
| container_qcif | bitsave | 0.42 | 0.32 | 0.42 | 0.37 | 0.46 |
|  | PSNR | 0.02 | 0.01 | 0.02 | 0.02 | 0.02 |
| news_qcif | bitsave | 0.07 | 0.04 | 0.07 | -0.38 | 0.08 |
|  | PSNR | 0.00 | 0.00 | 0.00 | -0.02 | 0.01 |
| foreman_qcif | bitsave | 1.68 | 1.95 | 1.02 | 0.60 | 0.66 |
|  | PSNR | 0.09 | 0.10 | 0.05 | 0.03 | 0.04 |
| silent_qcif | bitsave | 0.22 | 0.19 | 0.22 | -0.02 | 0.24 |
|  | PSNR | 0.01 | 0.01 | 0.01 | 0.00 | 0.01 |
| stefan_qcif | bitsave | 8.39 | 8.91 | 6.25 | 0.01 | 0.19 |
|  | PSNR | 0.47 | 0.50 | 0.35 | 0.00 | 0.01 |
| paris_cif | bitsave | 0.28 | 0.27 | 0.28 | -0.24 | 0.29 |
|  | PSNR | 0.01 | 0.01 | 0.01 | -0.01 | 0.01 |
| mobile_cif | bitsave | 3.51 | 3.49 | 2.27 | 1.61 | 1.94 |
|  | PSNR | 0.14 | 0.14 | 0.09 | 0.06 | 0.08 |
| tempete_cif | bitsave | 4.91 | 4.90 | 3.90 | 1.07 | 1.06 |
|  | PSNR | 0.18 | 0.18 | 0.14 | 0.04 | 0.04 |
| coast_cif | bitsave | 2.66 | 2.76 | 2.29 | 1.40 | 1.52 |
|  | PSNR | 0.08 | 0.08 | 0.07 | 0.04 | 0.05 |
| stefan_cif | bitsave | 9.46 | 9.68 | 7.90 | 0.32 | 0.48 |
|  | PSNR | 0.49 | 0.50 | 0.40 | 0.02 | 0.03 |
| foreman_cif | bitsave | 4.21 | 4.46 | 3.87 | 3.03 | 3.17 |
|  | PSNR | 0.19 | 0.20 | 0.17 | 0.13 | 0.14 |
| average | bitsave | 3.26 | 3.36 | 2.59 | 0.71 | 0.92 |
|  | PSNR | 0.15 | 0.16 | 0.12 | 0.03 | 0.04 |
| average (GM) | bitsave | 4.36 | 4.52 | 3.44 | 1.01 | 1.13 |
|  | PSNR | 0.20 | 0.21 | 0.16 | 0.04 | 0.05 |

Table 10 BD-bitsaving and BD-PSNR in 1/4 pel MC, M3, CABAC

| sequences | BD-bitsave [%] BD-PSNR [dB] | GMC_P | GMC_PB | GMVC | Mcopy | Mcopy_r1 |
| --- | --- | --- | --- | --- | --- | --- |
| container_qcif | bitsave | -0.02 | -0.18 | -0.02 | -0.63 | 0.10 |
|  | PSNR | -0.01 | -0.02 | -0.01 | -0.03 | 0.00 |
| news_qcif | bitsave | -0.50 | -0.56 | -0.50 | -0.43 | -0.46 |
|  | PSNR | -0.03 | -0.03 | -0.03 | -0.02 | -0.02 |
| foreman_qcif | bitsave | 0.42 | 1.53 | 0.60 | -0.16 | 0.29 |
|  | PSNR | 0.02 | 0.08 | 0.03 | -0.01 | 0.01 |
| silent_qcif | bitsave | 0.02 | -0.06 | 0.02 | -0.17 | 0.07 |
|  | PSNR | 0.00 | 0.00 | 0.00 | -0.01 | 0.00 |
| stefan_qcif | bitsave | 8.37 | 9.34 | 5.89 | 0.37 | -0.36 |
|  | PSNR | 0.48 | 0.53 | 0.33 | 0.02 | -0.02 |
| paris_cif | bitsave | 0.65 | 0.63 | 0.65 | 0.20 | 0.66 |
|  | PSNR | 0.03 | 0.03 | 0.03 | 0.01 | 0.03 |
| mobile_cif | bitsave | 2.63 | 2.58 | 1.42 | 0.75 | 0.84 |
|  | PSNR | 0.10 | 0.10 | 0.06 | 0.03 | 0.03 |
| tempete_cif | bitsave | 5.09 | 5.08 | 3.98 | 0.20 | 0.47 |
|  | PSNR | 0.18 | 0.18 | 0.14 | 0.01 | 0.02 |
| coast_cif | bitsave | 1.68 | 1.65 | 1.08 | 0.53 | 0.41 |
|  | PSNR | 0.05 | 0.05 | 0.03 | 0.02 | 0.01 |
| stefan_cif | bitsave | 8.75 | 9.24 | 7.04 | 0.54 | 0.77 |
|  | PSNR | 0.44 | 0.47 | 0.35 | 0.03 | 0.04 |
| foreman_cif | bitsave | 1.66 | 1.68 | 1.77 | 1.31 | 1.50 |
|  | PSNR | 0.07 | 0.07 | 0.07 | 0.05 | 0.06 |
| average | bitsave | 2.61 | 2.81 | 1.99 | 0.23 | 0.39 |
|  | PSNR | 0.12 | 0.13 | 0.09 | 0.01 | 0.01 |
| average (GM) | bitsave | 3.58 | 3.89 | 2.72 | 0.44 | 0.49 |
|  | PSNR | 0.17 | 0.18 | 0.13 | 0.02 | 0.02 |

Table 11 BD-bitsaving and BD-PSNR in 1/8 pel MC, M1, UVLC

| sequences | BD-bitsave BD-PSNR | GMC | Mcopy | Mcopy_r1 |
| --- | --- | --- | --- | --- |
| paris_cif | bitsave | 0.43 | 0.04 | 0.44 |
|  | PSNR | 0.02 | 0.00 | 0.02 |
| mobile_cif | bitsave | 6.48 | 5.60 | 5.64 |
|  | PSNR | 0.35 | 0.31 | 0.31 |
| tempete_cif | bitsave | 7.19 | 3.46 | 3.59 |
|  | PSNR | 0.33 | 0.16 | 0.16 |
| coast_cif | bitsave | 5.34 | 4.48 | 4.50 |
|  | PSNR | 0.20 | 0.17 | 0.17 |
| stefan_cif | bitsave | 4.69 | 2.75 | 2.99 |
|  | PSNR | 0.26 | 0.15 | 0.17 |
| foreman_cif | bitsave | 10.56 | 9.60 | 9.70 |
|  | PSNR | 0.56 | 0.50 | 0.50 |
| average | bitsave | 5.78 | 4.32 | 4.48 |
|  | PSNR | 0.29 | 0.21 | 0.22 |

Table 12 BD-bitsaving and BD-PSNR in 1/8 pel MC, M3, UVLC

| sequences | BD-bitsave BD-PSNR | GMC_P | GMC_PB | Mcopy | Mcopy_r1 |
| --- | --- | --- | --- | --- | --- |
| paris_cif | bitsave | 0.44 | 0.43 | 0.24 | 0.44 |
|  | PSNR | 0.02 | 0.02 | 0.01 | 0.02 |
| mobile_cif | bitsave | 3.40 | 3.42 | 1.84 | 2.18 |
|  | PSNR | 0.14 | 0.14 | 0.08 | 0.09 |
| tempete_cif | bitsave | 7.59 | 7.58 | 0.85 | 0.94 |
|  | PSNR | 0.30 | 0.30 | 0.03 | 0.04 |
| coast_cif | bitsave | 2.52 | 2.60 | 0.91 | 1.32 |
|  | PSNR | 0.08 | 0.08 | 0.03 | 0.04 |
| stefan_cif | bitsave | 11.34 | 11.98 | -1.06 | -0.60 |
|  | PSNR | 0.63 | 0.66 | -0.05 | -0.03 |
| foreman_cif | bitsave | 4.59 | 5.00 | 3.23 | 3.31 |
|  | PSNR | 0.21 | 0.23 | 0.15 | 0.15 |
| average | bitsave | 4.98 | 5.17 | 1.00 | 1.27 |
|  | PSNR | 0.23 | 0.24 | 0.04 | 0.05 |

Table 13 BD-bitsaving and BD-PSNR in 1/4 pel MC, M1, UVLC (5 reference)

| sequences | BD-bitsave [%] BD-PSNR [dB] | GMC | GMVC | Mcopy | Mcopy_r1 |
| --- | --- | --- | --- | --- | --- |
| container_qcif | bitsave | 0.03 | 0.03 | 0.02 | 0.17 |
|  | PSNR | 0.00 | 0.00 | 0.00 | 0.01 |
| news_qcif | bitsave | 0.02 | 0.02 | -0.13 | 0.12 |
|  | PSNR | 0.00 | 0.00 | -0.01 | 0.01 |
| foreman_qcif | bitsave | 3.60 | 3.63 | 2.33 | 3.10 |
|  | PSNR | 0.21 | 0.21 | 0.14 | 0.18 |
| silent_qcif | bitsave | -0.92 | -0.92 | -1.09 | -0.82 |
|  | PSNR | -0.05 | -0.05 | -0.05 | -0.04 |
| stefan_qcif | bitsave | 2.99 | 1.78 | 0.25 | 0.86 |
|  | PSNR | 0.17 | 0.10 | 0.01 | 0.05 |
| paris_cif | bitsave | 0.45 | 0.45 | 0.13 | 0.50 |
|  | PSNR | 0.02 | 0.02 | 0.01 | 0.03 |
| mobile_cif | bitsave | 4.36 | 3.89 | 3.50 | 3.62 |
|  | PSNR | 0.24 | 0.21 | 0.19 | 0.20 |
| tempete_cif | bitsave | 4.96 | 3.47 | 2.38 | 2.65 |
|  | PSNR | 0.23 | 0.16 | 0.11 | 0.12 |
| coast_cif | bitsave | 5.28 | 4.94 | 4.29 | 4.37 |
|  | PSNR | 0.20 | 0.19 | 0.16 | 0.16 |
| stefan_cif | bitsave | 4.23 | 3.38 | 2.46 | 2.55 |
|  | PSNR | 0.23 | 0.19 | 0.14 | 0.14 |
| foreman_cif | bitsave | 9.99 | 10.06 | 8.95 | 8.93 |
|  | PSNR | 0.56 | 0.57 | 0.49 | 0.49 |
| average | bitsave | 3.18 | 2.79 | 2.10 | 2.37 |
|  | PSNR | 0.17 | 0.15 | 0.11 | 0.12 |
| average (GM) | bitsave | 4.43 | 3.89 | 3.02 | 3.26 |
|  | PSNR | 0.23 | 0.20 | 0.15 | 0.17 |

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | NTT |  |  |
| Mailing address | 1-1 Hikari-no-oka, Yokosuka, Kanagawa |  |  |
| Country | JAPAN |  |  |
| Contact person | Yoshiyuki Yashima |  |  |
| Telephone | +81 468 59 2831 |  |  |
| Fax | +81 468 59 2829 |  |  |
| Email | yashima.yoshiyuki@lab.ntt.co.jp |  |  |
| Place and date of submission | Fairfax, Virginia, USA, 6-10 May, 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | GMVC and GMC with B-picture |  |
| Contribution number |  | JVT-C043 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  | Hideaki Kimata, Yoshiyuki Yashima |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: