| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 3rd Meeting: Fairfax, Virginia, USA, 6-10 May, 2002 | Document: JVT-C050r1 Filename: JVT-C050r1.doc |
| --- | --- |

| Title: | Result of Core Experiment on 4:2:0 Interlace Chroma Phase Shift |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Report and Proposal |  |  |
| Author(s) or Contact(s): | Kazushi Sato, Teruhiko Suzuki and Yoichi Yagasaki Gate City Osaki East Tower 20 F 1-11-1 Osaki, Shinagawa-ku Tokyo, 141-0032, Japan | Tel: Fax: Email: | +81-3-5435-3669 +81-3-5435-3891 kazushi@av.crl.sony.co.jp teruhiko@av.crl.sony.co.jp yagasaki@av.crl.sony.co.jp |
| Source: | Sony Corp. |  |  |

_____________________________

- Introduction

Interlaced field scanning was introduced originally as an analog video compression technique.  Although progressive picture scanning is generally regarded as superior for digital compression and display, the use of interlaced field scanning has persisted in many camera and display designs.  In particular, except for movies nearly all entertainment video delivered to home televisions originates in interlaced form. The MPEG-2 standard has been used effectively now for several years to deliver this interlaced video over satellite, cable, and fiber.  However, we are now receiving requirements to deliver that material at bit rates less than 2 Mbits/sec and are finding that MPEG-2 cannot deliver sufficient video quality at those rates.  Since the video compression in H.26L seems likely to be able to provide ample quality at those rates, it is important to make sure that H.26L has adequate syntax to support the interlace needs for video.

In Geneva meeting we proposed chroma phase shift for 4:2:0 interlaced video, which prevents severe image degradation that is typically observed in MPEG-2 video compression. The purpose of this contribution is to verify the impact of chroma phase shifting that we proposed on improvement and coding efficiency.

- Proposed Algorithm

  - Problem with MPEG-2 Frame MC Mode

Fig. 2.1 illustrates phases of luminance and chrominance signals a frame-mc macroblock with mvvertical =±2.0, ±6.0, ±10.0 … when the input signal is in 4:2:0 interlace format. In this case ideal chroma position is in the opposite field of the chroma phase in a frame memory, and it causes significant image degradation not only in the sense of PSNR but also of visual quality when the macloblock contains motion.

In Fig 2.2 and Fig 2.3 image degradations typically observed in the predicted images of MPEG-2 bit-stream due to this problem are shown.

Fig 2.1

(a) Original Image

(b) Predicted Image (MPEG-2 TM-5 Encoder)

Fig 2.2

(a) Original Image

(b) Predicted Image (MPEG-2 TM-5 Encoder)

Fig 2.3

2.2 Solution of the Problem

To fix the problem shown in Fig 3.1, we propose the following semantics change for motion compensation. No syntax change is necessary.

When the value of vertical component of the motion vector mvvertical =±2.0, ±6.0, ±10.0 … , the pixel “X” in Fig 3.1 is generated by intra-field prediction. The following filter is used for this interpolation:

(2.1)

{1, -5, 20, 20, -5, 1} /32

- Simulation Condition

3.1 Visual Quality Evaluation

To verify if the proposed algorithm will improve the visual quality, the simulation whose condition is described in Table 3.1 is executed.

| Codec | JM1.7 JM1.7 with Chroma Phase Shift |
| --- | --- |
| Sequence | “European Market” 1920x1088x30i; Frame Structure |
| GOP Structure | IPPP… |
| QP | QPI=QPP=20 |
| Entropy Coding | UVLC |
| RD Optimization | On |
| Hadamard | Yes |
| Motion Compensation | 1/4-pel accuracy |
| Reference Frames | 1 |
| MV Search Range | ±32 |
| Restricted Search Range | 2 |

Table 3.1

3.2 Coding Efficiency and Computaitonal Complexity Evaluation

To verify if the proposed algorithm will improve the coding efficiency with little computational complexity, the simulation whose condition is described in Table 3.2 is executed.

Computational complexity is evaluated based on the total encoding time. The following PC is used for simulation:

Sony VAIO PCV-RX62

CPU: Pentium4 1.7GHz

Memory: 256MB

OS: Windows 2000 Professional

Sequences in Group A are the ones to verify that the proposed algorithm will have a positive impact in coding efficiency, and the one s in B are fore verifying if the proposed algorithm will not have a negative impact in terms of coding efficiency and computational complexity.

| Codec. | JM1.7 JM1.7 with Chroma Phase Shift |
| --- | --- |
| Sequence | Group A: “Popple” 720x480i, 30Hz “F1” 720x576i, 25Hz Group B: “Flower Garden” 720x480i, 30Hz “Rugby” 720x576i, 25Hz Frame Structure |
| GOP Structure | - IPPP… - IBBPBB… |
| QP | QPI = QPP = 16,20,24,28 QPB = QPI + 2 |
| Entropy Coding | UVLC / CABAC |
| RD Optimization | On |
| Hadamard | Yes |
| Motion Compensation | 1/4-pel accuracy |
| Reference Frames | 1 or 5 |
| MV Search Range | ±16 |
| Restricted Search Range | 2 |

Table 3.2

3.3 Test with Sequences specified at Interlace Common Test Conditions

The following simulations are additionally tested for the purpose of verifying how the proposed algorithm works with sequences specified at interlace common test conditions:

| Codec. | JM1.7 JM1.7 with Chroma Phase Shift |
| --- | --- |
| Sequence | “Tempete” “Football” 720x480i “Canoa” 720x576i |
| GOP Structure | - IPPP… - IBBPBB… |
| QP | QPI = QPP = 16,20,24,28 QPB = QPI + 2 |
| Entropy Coding | UVLC / CABAC |
| RD Optimization | On |
| Hadamard | Yes |
| Motion Compensation | 1/4-pel accuracy |
| Reference Frames | 1 |
| MV Search Range | ±16 |
| Restricted Search Range | 2 |

Table 3.3

- Simulation Result

4.1 Visual Quality Evaluation

We evaluated the visual quality of the decoded image of the simulation described by Table 3.1.

Visual quality degradation as shown in Fig 2.2 or Fig 2.3 is not observed in JM without chroma phase shift. This is due to the fact that in MPEG-2 Test Model, mv search and mode decision are executed only with luminance signal. Therefore motion vector shown in Fig 2.1 might be occasionally chosen and thus causes significant image degradation.

As of JVT, mv search is executed only on the luminance plane, but chroma coefficients are taken into account in RD optimization, so motion vectors shown in Fig 2.1 are rarely selected. However, this problem will prevent the encoder to choose the “truly best” motion vectors, which causes loss in coding efficiency whith sequences which contain specific motion. By fixing this problem we can expect improvement in coding efficiency as described in 4.2.

4.2 Coding Efficiency and Computational Complexity Evaluation

The simulation results are shown in “JVT-C50r1.xls”.

As of results with Group A sequences, significant improvement in coding efficiency is observed by up to 10%. With the results with Group B sequences, the proposed algorithm will not have a negative impact on coding efficiency or on computational complexity.

3.3 Test with Sequences specified at Interlace Common Test Conditions

The simulation results are also shown in “JVT-C50r1.xls”. We can also see that the proposed algorithm will have not have a negative impact for those sequences. As of “Football” coding gain by up to 1% is obtained.

- Conclusion

We have verified 4:2:0 Interlace Chroma Phase Shift that we propose in JVT-B068. Our simulation results show that although visual quality improvement is not observed due to RD optimization, the proposed algorithm will significantly improve coding efficiency with sequence which contains color component and vertical motions like F1 or Popple. We also verified that the proposed algorithm will not have a negative impact in terms of coding efficiency and computational complexity with other sequences.

In JVT field/frame adaptive will be adapted either on picture-level or on MB-level. In either case, the proposed algorithm can be adapted in frame mode and will contribute to the improvement in coding efficciency. We only showed the results with 1/4-pel accuracy MC in this contribution, but it can also be adapted to 1/8-pel accuracy MC.

In addition to the advantages above, the proposed algorithm requires no syntax change and very little semantics change, and it can easily be adapted to the current JVT specification.

We propose the chroma phase shift for 4:2:0 interlace video be adopted to the interlace/broadcast profile in JVT.

References

[1] ISO/IEC 13818-2:2000 Information technology -- Generic coding of moving pictures and associated audio information: Video

[2] “Working Draft Number 2, Revision 4 (WD-2 rev 5)”, T. Wiegand, JVT-B118r7, DRAFT ISO/IEC 14496-10, Geneva, CH, Feb 2002

[3] “New Interlace Coding Tools”, K. Sato, T. Suzuki and Y. Yagasaki, JVT-B068r1, Geneva, CH, Feb 2002

[4] “Core Experiment on Interlace Chroma Phase Shift”, K. Sato, T. Suzuki, Y. Yagasaki and Y. Suzuki, JVT-B116, Geneva, CH, Feb 2002

[5] “Core Experiment on Interlaced Video Coding”, P. Borgwardt, ITU – Telecommunications Standardization Sector STUDY GROUP 16 Question 6, Video Coding Experts Group (VCEG) document, VCEG-N85r1, Septermber 2001

[6] “Calculation of average PSNR difference between RD-curves”, G. Bjontegaard, VCEG-M33, March 2001

[7] “Recommended Simulation Conditions for H.26L”, G. Bjontegaard, VCEG-M75, May 2001

[8] “TestModel5”, ISO/IEC,JTC/SC29/WG11/N0400, 1993

(Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Sony Corp. |  |  |
| Mailing address | Gate City Osaki East Tower 20 F 1-11-1 Osaki Shinagawa-ku, Tokyo, 141-0032 |  |  |
| Country | Japan |  |  |
| Contact person | Kazushi Sato |  |  |
| Telephone | +81 3 5435 3669 |  |  |
| Fax | +81 3 5435 3891 |  |  |
| Email | kazushi@av.crl.sony.co.jp |  |  |
| Place and date of submission | Tokyo, Japan on May 1, 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Result of Core Experiment on 4:2:0 Interlace Chroma Phase Shift |  |
| Contribution number |  | C050 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
| x | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: