| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 3rd Meeting: Fairfax, Virginia, USA, 6-10 May, 2002 | Document: JVT-C105 Filename: JVT-C105.doc |
| --- | --- |

| Title: | ## Block Boundary Mirroring for Motion Interpolation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ulrich Benzler and Michael Wollborn FV/SLM, Robert Bosch GmbH, PO Box 77 77 77 31132 Hildesheim, Germany | Tel: Email: | +49 5121 49-3087 ulrich.benzler@de.bosch.com |
| Source: | Robert Bosch GmbH |  |  |

_____________________________

## Introduction

For motion compensating prediction with subpel resolution, the values in the reference pictures that do not lie on full-pel positions have to be generated by applying a 6-tap (8-tap) FIR interpolation filter in quarter-pel (eigth-pel) motion vector mode.

For the example of a 4x4 motion compensated block to be predicted in quarter-pel motion vector mode, this leads to a block of 9x9 pel which has to be read from the reference picture to perform the interpolation: three pixels (half the filter length) biased in the direction of the subpel position to interpolate the edge pixels on this side, an two pixels (half the filter length minus 1) for the edge pixels on the opposite side of the block. This means that for interpolating a block of 4x4=16 pixels, a reference picture block of 9x9=81 pixels has to be accessed. Compared with bilinear interpolation, where only 5x5=25 reference pixels are needed, this is an increase to more than three times of reference memory access.

## Method description

In MPEG-4, a technique called “block boundary mirroring” was introduced to limit the amount of reference memory access to that of bilinear interpolation, while still being able to take advantage of the high efficiency of FIR filter interpolation for subpel values:

“In the subpel interpolation, for each block of size MxN in the reference picture which position is defined by the decoded motion vector for the block to be predicted, a reference block of size (M+1)x(N+1) biased in the direction of the subpel sample position is read from the reconstructed reference picture. Then, this reference block is symmetrically extended at the block boundaries by two samples (three samples in case of eigth-pel motion vector mode) using block boundary mirroring according to Figure 1.”

Figure 1 -- block boundary mirroring

The subpel values are then calculated from this extended reference block according to the description of fractional sample accuracy in the motion compensation section of the JWD.

## Results

In the MPEG document 3146 (San Jose meeting), results for the implementation of block boundary mirroring into the MPEG-4 VM are shown: compared with conventional FIR filter interpolation, the difference in PSNR is limited to less than 0.05 dB, and the difference in bitrate is less than 3.8%. For the implementation in to the JM, a similar performance is to be expected.

## Conclusion

An efficient method for reducing the amount of reference memory access in the motion interpolation part of the JM is proposed. It has already shown its potential with the successful adoption into the MPEG-4 Visual standard. We propose to tentatively adopt this method into the JVT standard, with a time slot for implementation after the Fairfax meeting.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Co0ding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Robert Bosch GmbH |  |  |
| Mailing address | FV/SLM PO Box 77 77 77 31132 Hildesheim |  |  |
| Country | Germany |  |  |
| Contact person | Ulrich Benzler |  |  |
| Telephone | +49 5121 49-3087 |  |  |
| Fax | +49 5121 49-173087 |  |  |
| Email | Ulrich.benzler@de.bosch.com |  |  |
| Place and date of submission | Hildesheim, April 29th 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Block Boundary Mirroring for Motion Interpolation |  |
| Contribution number |  | JVT-C105 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: