| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 3rd Meeting: Fairfax, Virginia, USA, 6-10 May, 2002 | Document: JVT-C126 Filename: JVT-C126.doc |
| --- | --- |

| Title: | Core experiment results on SP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Xiaoyan Sun, Feng Wu Microsoft Research Asia, 3F Sigma, No49 Zhichun Rd, Haidian, Beijing, 100080, China Ragip Kurceren Nokia Research Center, 6000 Connection Drive, Irving, TX, 75063 | Tel: Email: | +86-10-62617711-3119 fengwu@microsoft.com +86-10-62617711-3119 Ragip.kurceren@nokia.com |
| Source: | Microsoft, Nokia |  |  |

_____________________________

An SP coding scheme [1]  was proposed by Microsoft in the Geneva meeting to improve the coding efficiency of the current SP scheme in JVT [2] [3] . In order to evaluate the proposed techniques, a core experiment about SP frame was established in the meeting [4] . For convenience, the proposed SP coding scheme is referenced as the JVT-B097 scheme hereafter according to the number of the first proposal. Both the JVT SP scheme and the JVT-B097 scheme are described in the appendix of [4] . The experimental results are given in this report.

# Experimental Conditions

For convenience to understand the experimental results, the experimental conditions specified in JVT-B112 are listed as follows:

- Software:			TML 9.4

- CIF Sequences:		Paris(15fps), Mobile(30fps), Tempete(30fps)

- QCIF Sequences:		Silent(15fps), News(10fps), Foreman(10fps),

Container(10fps), Hall(10fps)

- RD optimization:		Enable

- Hadamard transform:		Enable

- Search Range:		16

- MC:				¼ pixel

- Reference number:		2

- B frame:			No

- Inter and Intra mode:		All

- Entropy coding:		UVLC

# Experimental Results

The coding efficiency of two SP coding schemes is compared in this section. When SP-frame is inserted periodically, the PSNR versus bit rate curves of these two schemes are very close because the differences on SP frames are averaged by other P frames. The experimental results in this case are presented in the attached exec file. The experiment 1 specified in JVT-B112 is discussed here in detail. Only the first frame is encoded as I frame, and all other frames are encoded as SP frames. Each sequence is encoded with skip = 0, 1 or 2. Quantization parameters Qp and Qs are given in Table 1, respectively.

Table 1: The quantization steps in coding efficiency comparisons.

| Qp |  |  |  |  | Qs |  |
| --- | --- | --- | --- | --- | --- | --- |
| 12 | 16 | 20 | 24 | 28 | QP-3 | QP |

## Experiment 1: Static tests: Coding efficiency of SP-frames

The reconstructed reference Luma in the JVT SP scheme and the JVT-B097 scheme is compared in Table 2. (1) When Qs = Qp, for Tempete CIF sequence, the JVT-B097 scheme increases 8.89% bits in average. The average increasing bits for all sequences are about 4.05%. In addition, an extra IDCT is needed in the JVT-B097 scheme. (2) When Qs is equal to Qp-3, the JVT-B097 scheme can save 0.25% bits in average for all sequences. However, the reconstructed reference Chroma in the JVT-B097 scheme are around 0.5dB lower when Qp = Qs, and 0.3dB lower when Qp = Qs-3.

Table 2: The comparisons on the reconstructed reference Luma in the JVT SP scheme and the JVT-B097 scheme.

|  | Container | Foreman | News | Silent | Tempete | Mobile |
| --- | --- | --- | --- | --- | --- | --- |
| Qp = Qs | -1.83 | -3.29 | -2.19 | -1.57 | -8.89 | -6.58 |
| Qp=Qs-3 | -0.62 | 0.12 | 0.62 | 0.83 | -0.36 | 0.84 |

# Experiment 2: Different Display Image

For both JVT-B097 and JVT-SP schemes, another high quality image can be reconstructed for display if the decoder is powerful enough. In other words, the display image may be different from the reconstructed reference. The PSNR of the reconstructed reference and display in the JVT-B097 scheme are compared in Table 3, where Qs is equal to Qp. The PSNR gain is up to 1.6dB in Mobile CIF, 1.4dB in Tempete, 1.3dB in Foreman QCIF, 0.6dB in News QCIF, 0.6dB in Container QCIF, 0.3dB in Hall QCIF, and 0.4dB in Silent QCIF. The average PSNR gain for all testing sequences is 0.71dB. The additional complexity for reconstructing the display image is a non-normative post filtering. When Qs is equal to Qp-3, the experimental results are given in Table 4. The maximum PSNR gain is up to 0.9dB in Mobile CIF, 0.9dB in Tempete, 0.8dB in Foreman QCIF, 0.4dB in News QCIF, 0.4dB in Container QCIF, 0.2dB in Hall QCIF, and 0.3dB in Silent QCIF. The average PSNR gain for all testing sequences is 0.43dB.

Table 3: The comparisons on the reconstructed reference and display of the JVT-B097 scheme with Qs = Qp.

| sequences | frame rate | number of frames | QP | PSNRY (Display) | PSNRY (Ref) | PSNR gain(dB) |
| --- | --- | --- | --- | --- | --- | --- |
| foreman_qcif | 10 | 100 | 12 | 38.115 | 36.860 | 1.255 |
|  |  |  | 16 | 35.062 | 33.930 | 1.132 |
|  |  |  | 20 | 32.111 | 31.170 | 0.941 |
|  |  |  | 24 | 29.290 | 28.540 | 0.750 |
|  |  |  | 28 | 26.555 | 26.000 | 0.555 |
| news_qcif | 10 | 100 | 12 | 39.116 | 38.550 | 0.566 |
|  |  |  | 16 | 35.936 | 35.400 | 0.536 |
|  |  |  | 20 | 32.720 | 32.250 | 0.470 |
|  |  |  | 24 | 29.773 | 29.360 | 0.413 |
|  |  |  | 28 | 26.837 | 26.480 | 0.357 |
| container_qcif | 10 | 100 | 12 | 38.485 | 37.850 | 0.635 |
|  |  |  | 16 | 35.661 | 35.030 | 0.631 |
|  |  |  | 20 | 32.785 | 32.170 | 0.615 |
|  |  |  | 24 | 29.870 | 29.320 | 0.550 |
|  |  |  | 28 | 27.022 | 26.560 | 0.462 |
| hall_qcif | 10 | 100 | 12 | 39.531 | 39.270 | 0.261 |
|  |  |  | 16 | 36.666 | 36.370 | 0.296 |
|  |  |  | 20 | 33.727 | 33.420 | 0.307 |
|  |  |  | 24 | 30.454 | 30.150 | 0.304 |
|  |  |  | 28 | 27.461 | 27.150 | 0.311 |
| silent_qcif | 15 | 150 | 12 | 38.092 | 37.650 | 0.442 |
|  |  |  | 16 | 34.980 | 34.620 | 0.360 |
|  |  |  | 20 | 32.065 | 31.820 | 0.245 |
|  |  |  | 24 | 29.479 | 29.280 | 0.199 |
|  |  |  | 28 | 27.014 | 26.810 | 0.204 |
| Mobile_cif | 30 | 300 | 12 | 36.839 | 35.230 | 1.609 |
|  |  |  | 16 | 33.322 | 31.760 | 1.562 |
|  |  |  | 20 | 29.801 | 28.380 | 1.421 |
|  |  |  | 24 | 26.411 | 25.200 | 1.211 |
|  |  |  | 28 | 23.235 | 22.280 | 0.955 |
| tempete_cif | 30 | 260 | 12 | 37.535 | 36.120 | 1.415 |
|  |  |  | 16 | 34.182 | 32.860 | 1.322 |
|  |  |  | 20 | 30.895 | 29.770 | 1.125 |
|  |  |  | 24 | 27.807 | 26.910 | 0.897 |
|  |  |  | 28 | 24.976 | 24.320 | 0.656 |
| average |  |  |  |  |  | 0.714 |

Table 4: The comparisons on the reconstructed reference and display of the JVT-B097 scheme with Qs = Qp-3.

| sequences | frame rate | number of frames | QP | PSNRY (TML9.4) | PSNRY (B097) | PSNR gain (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| foreman_qcif | 10 | 100 | 12 | 38.158 | 37.380 | 0.778 |
|  |  |  | 16 | 35.137 | 34.440 | 0.697 |
|  |  |  | 20 | 32.198 | 31.620 | 0.578 |
|  |  |  | 24 | 29.373 | 28.900 | 0.473 |
|  |  |  | 28 | 26.612 | 26.250 | 0.362 |
| news_qcif | 10 | 100 | 12 | 39.053 | 38.690 | 0.363 |
|  |  |  | 16 | 35.897 | 35.580 | 0.317 |
|  |  |  | 20 | 32.777 | 32.520 | 0.257 |
|  |  |  | 24 | 29.792 | 29.560 | 0.232 |
|  |  |  | 28 | 26.914 | 26.720 | 0.194 |
| container_qcif | 10 | 100 | 12 | 38.387 | 38.000 | 0.387 |
|  |  |  | 16 | 35.572 | 35.190 | 0.382 |
|  |  |  | 20 | 32.764 | 32.380 | 0.384 |
|  |  |  | 24 | 29.778 | 29.410 | 0.368 |
|  |  |  | 28 | 26.961 | 26.660 | 0.301 |
| hall_qcif | 10 | 100 | 12 | 39.500 | 39.300 | 0.200 |
|  |  |  | 16 | 36.672 | 36.490 | 0.182 |
|  |  |  | 20 | 33.612 | 33.450 | 0.162 |
|  |  |  | 24 | 30.501 | 30.330 | 0.171 |
|  |  |  | 28 | 27.540 | 27.370 | 0.170 |
| silent_qcif | 15 | 150 | 12 | 38.021 | 37.750 | 0.271 |
|  |  |  | 16 | 34.901 | 34.700 | 0.201 |
|  |  |  | 20 | 31.994 | 31.910 | 0.084 |
|  |  |  | 24 | 29.357 | 29.320 | 0.037 |
|  |  |  | 28 | 26.927 | 26.850 | 0.077 |
| Mobile_cif | 30 | 300 | 12 | 36.872 | 35.930 | 0.942 |
|  |  |  | 16 | 33.355 | 32.430 | 0.925 |
|  |  |  | 20 | 29.819 | 28.970 | 0.849 |
|  |  |  | 24 | 26.423 | 25.670 | 0.753 |
|  |  |  | 28 | 23.262 | 22.660 | 0.602 |
| tempete_cif | 30 | 260 | 12 | 37.562 | 36.690 | 0.872 |
|  |  |  | 16 | 34.208 | 33.400 | 0.808 |
|  |  |  | 20 | 30.897 | 30.210 | 0.687 |
|  |  |  | 24 | 27.806 | 27.250 | 0.556 |
|  |  |  | 28 | 24.985 | 24.570 | 0.415 |
| average |  |  |  |  |  | 0.430 |

The experimental results of chrominance components are given in the attached exec file.

# Conclusions

- If the JVT-B097 scheme uses the reconstructed reference for display, when Qs = Qp, the increasing bit rate is 4.05% in average. In addition, an extra IDCT is needed in the decoder. However, when Qs = Qp-3, the JVT-B097 scheme can save the average bit rate 0.25%. The JVT-B097 scheme has a similar performance on the reconstructed reference as the JVT SP scheme.

- The display image quality can be significantly improved up to 1.6dB while a non-normative post filtering is allowed in the decoder. This technique can be similarly applied to the JVT SP scheme, which is described in the proposal JVT-C114.

Reference:

- X. Sun, F. Wu, S. Li, W. Gao and Y.-Q Zhang, “Improved SP coding technique”, doc JVT-B097, Joint video term group meeting, Geneva, Switzerland, Jan 2002.

- M. Karczewicz and R. Kurceren, “A proposal for SP-frames”, doc VCEG-L27, ITU-T video coding experts group meeting, Eibsee, Germany, Jan 2001.

- R. Kurceren and M.Karczewicz, “Improved SP-frame encoding”, doc VCEG-M73, ITU-T video coding experts group meeting, Austin, USA, April 2001.

- F. Wu, S. Li, R. Kurceren and J. Lainema, “The core experiment on the SP frame coding”, doc JVT-B112, Joint video term group meeting, Geneva, Switzerland, Jan 2002.