| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 3rd Meeting: Fairfax, Virginia, USA, 6-10 May, 2002 | Document: JVT-C140r1 Filename: JVT-C140r1.doc |
| --- | --- |

| Title: | Alternate Coefficient Scanning Patterns for Interlaced ABT Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Limin Wang and David Baylon Motorola, Inc. Broadband Communications Sector 6450 Sequence Drive San Diego, CA 92121 | Tel: Email: | 858-404-3893 liwang@gi.com |
| Source: | Motorola, Inc., BCS |  |  |

_____________________________

Summary

Adaptive Block Transform (ABT) coding for JVT has been proposed as a method which can improve coding performance for interlaced and high resolution video.  In [JVT-B053], “traditional” zigzag scans are used to convert two-dimensional coefficient data into one-dimensional data.  This report looks at the use of alternate coefficient scanning patterns for interlaced material in order to improve the ABT coding efficiency.  Preliminary results with alternate scanning patterns have shown bitrate savings of up to about 8%.

- Introduction

The effectiveness of an entropy coding scheme generally depends on how well the codewords are matched to the statistics of the data.  Since typical video data tends to have significant low frequency content, intra and inter coefficient data tends to have coefficient energy concentrated in lower frequencies.  When the coefficients are scanned for run-length coding, it is well known that by first scanning together these high-energy low frequency coefficients, long run-lengths can be avoided, thereby allowing more efficient coding of the data.

Zig-zag scans, such as those used in [JVT-B053], have this particular scanning property.  For square blocksizes, this “traditional” scan goes through the two-dimensional array without any significant bias towards the horizontal or vertical frequency directions.

Although these “unbiased” scans may be appropriate for progressive material, other scanning patterns may be more suitable for interlaced material.  For example, in [JVT-B068] an alternate 4x4 blocksize scan is proposed for interlaced material which shows compression gains for high motion content.  MPEG-2 and MPEG-4 also provide for alternate scans which can improve interlaced performance.

- Alternate Scanning Patterns for Interlaced Video

When a frame (or macroblock) is coded in field mode, the field data may be considered as a vertically subsampled version from an otherwise progressively scanned frame.  As a consequence, the vertical frequency axis (frequencies in the vertical direction) would be scaled by a factor of two relative to the progressive case, with of course the addition of possible aliasing.  Because of the vertical frequency scaling, it might be argued that for interlaced material, the scanning of coefficients should be somewhat biased in the vertical direction in order to capture more of the non-zero low frequency components.

Experimental results appear to be consistent with this notion.  Simulations with an ABT codec described in [JVT-B053] have shown a distribution of non-zero coefficients skewed in the vertical direction for interlaced material.  From these results, a set of alternate scanning patterns were developed for interlaced material.  We focused on ABT coding only for blocksizes of 4x4, 4x8, 8x4, and 8x8.  Since our data was also consistent with a 4x4 blocksize scan described in [JVT-B068], we used the same 4x4 alternate scan as used there.  That scanning pattern, along with the 4x8, 8x4, and 8x8 alternate scans developed here, are shown below.

| 1 | 3 | 9 | 13 |
| --- | --- | --- | --- |
| 2 | 6 | 10 | 14 |
| 4 | 7 | 11 | 15 |
| 5 | 8 | 12 | 16 |

4x4 alternate scan

| 1 | 5 | 13 | 21 |
| --- | --- | --- | --- |
| 2 | 6 | 14 | 22 |
| 3 | 7 | 15 | 23 |
| 4 | 12 | 20 | 28 |
| 8 | 16 | 24 | 29 |
| 9 | 17 | 25 | 30 |
| 10 | 18 | 26 | 31 |
| 11 | 19 | 27 | 32 |

4x8 alternate scan

| 1 | 3 | 7 | 11 | 15 | 19 | 23 | 27 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | 6 | 10 | 14 | 18 | 22 | 26 | 30 |
| 4 | 8 | 12 | 16 | 20 | 24 | 28 | 31 |
| 5 | 9 | 13 | 17 | 21 | 25 | 29 | 32 |

8x4 alternate scan

| 1 | 4 | 9 | 16 | 23 | 31 | 39 | 53 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2 | 5 | 15 | 22 | 30 | 38 | 46 | 54 |
| 3 | 8 | 17 | 24 | 32 | 40 | 47 | 59 |
| 6 | 10 | 21 | 29 | 37 | 45 | 52 | 60 |
| 7 | 14 | 25 | 33 | 41 | 48 | 55 | 61 |
| 11 | 18 | 26 | 34 | 42 | 49 | 56 | 62 |
| 12 | 19 | 27 | 35 | 43 | 50 | 57 | 63 |
| 13 | 20 | 28 | 36 | 44 | 51 | 58 | 64 |

8x8 alternate scan

- Experimental Results

Simulations were performed using an ABT codec described in [JVT-B053] based on TML9.  The baseline ABT software codec was provided by Mathias Wien.  Interlaced test material was used with field-based encoding under the following conditions:

| Picture Structure (field-based) | IP PP PP PP IP BB BB PP BB BB |
| --- | --- |
| Quantization Parameters | 16, 20, 24, 28 [QP(B) = QP(I/P)+2] |
| RD Optimization | On |
| Motion Resolution | ¼ pel |
| Search Range | 16 pixels |
| Hadamard | On |
| Entropy Coding | CABAC |
| Reference Frames | 2 |
| ABT Encoding | Intra and Inter |

Common encoding conditions

Two sets of scanning patterns were used for the 4x4, 4x8, 8x4, and 8x8 blocksizes.  The first set consisted of the reference patterns described in [JVT-B053], and the second set was composed of the alternate scanning patterns described earlier.  These were used in encoding the following interlaced test sequences:

| Sequence Name | Frame Size | # Frames |
| --- | --- | --- |
| Bus | 720x480 | 150 |
| Football | 720x480 | 260 |
| Tempete | 720x480 | 260 |
| Canoa | 720x576 | 220 |
| Rugby | 720x576 | 220 |
| F1car | 720x576 | 220 |

Interlaced test sequences

The results of the simulations are shown in the following four figures.  In these figures, the bitrate savings by using the alternate scans is plotted vs. the corresponding bitrate using the reference scans.  For any given comparison, both reference scan and alternate scan cases had essentially the same PSNRs.

These results show savings over a wide range of bitrates by using the alternate scans over the reference scans for interlaced material.

Table 1 shows the average bit rate saving in % calculated by using the SW provided by Real Network (Bjontegaard Delta bitrate saving in % [3]).

Table 1. Bit Rate savings (%) by using alternate scanning over zigzag scanning

|  | Bus | Football | Tempete | Canoa | Rugby | F1car |
| --- | --- | --- | --- | --- | --- | --- |
| IPPPPP… | 4.08 | 5.90 | 3.18 | 7.39 | 6.00 | 6.23 |
| IBBPBB... | 4.50 | 5.89 | 3.19 | 8.39 | 6.28 | 6.90 |

- Further Remarks

Although the results in this report were based on an ABT TML9 version, it is expected that comparable performance will be obtained for interlace material even with more recent versions.  One might argue that the effect of alternate scans might be less pronounced at very low or high bitrates.  Nevertheless, experimental results have shown some non-trivial bitrate savings over a wide range of bitrates.  There is some additional memory required in storing the additional alternate scanning patterns at both the encoder and decoder, but there is no significant added complexity.

References

[JVT-B053]	Mathias Wien, ' ABT Coding for Higher Resolution Video ', Document JVT-B053, Joint Video Team (JVT) of ISO/IEC MPEG and ITU-T VCEG, Geneva, Switzerland, 29 Jan -1 Feb.. 2002.

[JVT-B068]	Kazushi Sato, Teruhiko Suzuki and Yoichi Yagasaki, ' New Interlace Coding Tools ', Document JVT-B068, Joint Video Team (JVT) of ISO/IEC MPEG and ITU-T VCEG, Geneva, Switzerland, 29 Jan -1 Feb.. 2002.

[VCEG-M33] 	Gisle Bjontegaard, “Calculation of average PSNR differences between RD-curves”, VDCG-M33, Austin, April 2-4, 2001.

# JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Motorola Inc. |  |  |
| Mailing address | # 6450 Sequence Drive San Diego, CA 92121 |  |  |
| Country | U.S.A. |  |  |
| Contact person | Limin Wang |  |  |
| Telephone | 858-404-3893 |  |  |
| Fax | 858-404-2501 |  |  |
| Email | liwang@gi.com |  |  |
| Place and date of submission | San Diego, CA., USA, 05/01/02 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Alternate Coefficient Scanning Patterns for Interlaced ABT Coding |  |
| Contribution number |  | JVT-C140r1 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| Without having conducted a search or investigation, the submitter/author(s) individually believe(s) they or General Instrument Corporation (Patent Holder) may have granted, pending or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | - The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. see comments below regarding Baseline Profile |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  | Comment to Section 2.2. To support the proposed royalty free JVT Baseline Profile, and in addition to the statement of 2.2 above, Patent Holder is also willing to grant a royalty free license to patents essential to JVT Baseline Profile decoder applications on the condition that any and all other patent holders do the same. |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: