| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 3rd Meeting: Fairfax, Virginia, USA, 6-10 May, 2002 | Document: JVT-C145 Filename: JVT-C145.doc |
| --- | --- |

| Title: | ## Verification results for adaptive coding of transform coefficients (JVT-C088) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Information |  |  |
| Authors: | Satoshi Kondo, Kiyofumi Abe and Shinya Kadono 1006 Kadoma, Kadoma, 571-8501, Japan | Tel: Email: | +81-6-6900-9689 kondo@drl.mei.co.jp abe@drl.mei.co.jp kadono@drl.mei.co.jp |
| Source: | Matsushita Electric Industrial Co., Ltd. |  |  |

_____________________________

## Introduction

In this document, we report the verification results for adaptive coding of transform coefficients proposed in the document JVT-C088 [1]. We implement the proposed method to the JM1.7 software independently. And we verify that it can reduce large amount of bits compared to UVLC coding, and that our results are almost same as the results provided in [1].

## Experimental Conditions

Experimental conditions follow the document VCEG-N81 [3] except QP as follows.

- Software version: JM1.7

- Motion estimation and mode selection: High complexity mode.

- QP: 0,4,8,12,16,20,24,28, and fixed for all over sequences.

- Picture coding type: III… for intra evaluation, IPP… for inter evaluation.

- Number of reference frame: 1 frame.

- Motion vector search range: 32 pels.

- Motion compensation: 1/4-pel and 1/8-pel accuracy for QCIF and CIF resolution sequences, respectively.

- Other coding options: using Hadamard transform, no CABAC, and no slice structure.

- Test sequences: see Table 1.

Table 1: Test sequences.

| Frame Skip | 2 | 1 | 0 |
| --- | --- | --- | --- |
| Sequences | Container, QCIF,300 News, QCIF,300 Foreman, QCIF,300 | Paris, CIF,300 Silent Voice, QCIF,300 | Mobile CIF 300 Tempete CIF 260 |

## Experimental Results

The experimental results are summarized in Table 2 (for intra) and Table 3 (for inter) that indicate the bit rate savings from UVLC calculated by BD-snr. Since performances of the method depend on QP, we calculate BD-snr for two groups, Low (QP = 0,4,8,12) and High (QP = 16,20,24,28). Average bit rate savings of all sequences are 17.10 % (low QPs) and 5.28 % (high QPs) for intra, 9.29 % (low QPs) and 0.92 % (high QPs) for inter. Figure 1 and Figure 2 show the bit rate savings from UVLC for each sequence. Note that the differences of PSNR are not considered in the results shown in Figure 1 and Figure 2. And RD-curves are shown in the Excel file JVT-C145.xls. From these results, we can see that the improvements against UVLC are significantly large. And we verify our results are almost same as the results provided in [1].

Table 2: Bit rate savings for intra.

| Sequence Name | Bit Rate Savings (%) |  |
| --- | --- | --- |
|  | Low | High |
| Container (QCIF, 10fps) | -15.47 | -5.07 |
| News (QCIF, 10fps) | -17.21 | -7.32 |
| Foreman (QCIF, 10fps) | -13.40 | -2.99 |
| Paris (CIF, 15fps) | -17.99 | -5.97 |
| Silent Voice (QCIF, 15fps) | -13.84 | -2.34 |
| Mobile (CIF, 30fps) | -23.51 | -8.28 |
| Tempete (CIF, 30fps) | -18.25 | -5.00 |
| Average | -17.10 | -5.28 |

Table 3: Bit rate savings for inter.

| Sequence Name | Bit Rate Savings (%) |  |
| --- | --- | --- |
|  | Low | High |
| Container (QCIF, 10fps) | -4.62 | 0.19 |
| News (QCIF, 10fps) | -10.41 | -2.69 |
| Foreman (QCIF, 10fps) | -7.19 | -0.58 |
| Paris (CIF, 15fps) | -11.15 | -1.34 |
| Silent Voice (QCIF, 15fps) | -7.63 | -0.94 |
| Mobile (CIF, 30fps) | -12.15 | -0.18 |
| Tempete (CIF, 30fps) | -11.91 | -0.88 |
| Average | -9.29 | -0.92 |

## Conclusion

We evaluate the adaptive coding scheme of transform coefficients proposed in JVT-C088 [1]. The scheme can reduce bits 17.10 % at low QPs for intra, and 5.28 % bit at low QPs for inter. Although the experimental results provided in [1] are obtained by using the JM1.1 software and our verification results are obtained by using the JM1.7 software, we confirm that our verification results are almost same as the results provided in [1]. As a result, we verify the effectiveness of the method proposed in [1], and we recommend that the method should be incorporated into the next JM (CD).

## References

- R.Kurceren  and M.Karczewicz, “Adaptive coding of transform coefficients”, JVT-C088, May. 2002.

- G.Sullivan, “Recommended Simulation Common Conditions for H.26L Coding Efficiency Experiments on Low-Resolution Progressive-Scan Source Material”, VCEG-N81, Sep. 2001.

Figure 1: Bit rate savings relative to JM1.7 UVLC (intra).

Figure 2: Bit rate savings relative to JM1.7 UVLC (inter).