| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 4th Meeting: Klagenfurt, Austria, 22-26 July, 2002 | Document: JVT-D025 Filename: JVT-D025.doc |
| --- | --- |

| Title: | Analysis and Simplification of Intra Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Marta Karczewicz, Jani Lainema Nokia Research Center, 6000 Connection Drive Irving, Texas 75039, USA | Tel: Email: | +972 894 4188 marta.karczewicz@nokia.com |
| Source: | Nokia Research Center |  |  |

## Introduction

In order to perform the spatial prediction, JVT coder offers 9 modes for prediction of 4x4 blocks, including DC prediction and 8 directional modes. To reduce number of bits required to specify prediction mode of each block the correlation of the prediction modes of adjacent blocks is used. When the prediction modes of neighboring already coded upper (A) and left (B) blocks are known an ordering of the most probable, next most probable etc. prediction mode for the currently coded block C is given. The ordering of modes is specified for each combination of prediction modes of A and B. For more efficient coding, information on intra prediction of two 4x4 blocks is coded in one codeword

This method has one major drawback – the memory required to keep ordering of prediction modes for block C given prediction modes of blocks A and B. Since 9 modes are used we have 9x9 possible combinations of modes for blocks A and B. For each combination order of 9 possible modes has to be specified. That means that 9x9x9 = 729 bytes (we assume that one number requires one byte) are needed to specify ordering of prediction modes. In addition table specifying code number as a function of the probability of two consecutive modes requires 9x9 = 81 bytes This is a considerable increase when comparing with the spatial prediction used in JVT coder initially when only 6 modes where used.

## Simplifications

The semantics of the intra-prediction modes are same as those of CD. However, the 9 intra-prediction modes shall be named as follows:

Mode 0: Vertical prediction

Mode 1: Horizontal prediction

Mode 2: DC prediction

Mode 3: Diagonal Down/Left prediction

Mode 4: Diagonal Down/Right prediction

Mode 5: Vertical Left prediction

Mode 6: Horizontal Down prediction

Mode 7: Vertical Right prediction

Mode 8: Horizontal Up prediction

To each combination of modes for blocks A and B only one most probable mode is assigned. If one of the blocks A or B is outside the most probable mode is equal to prediction mode 2. Otherwise it is equal to the minimum of modes used for blocks A and B.

To the decoder first 1 bit is sent indicating whether the most probable mode is used for a 4x4 block. If the bit value is equal to 1 the most probable mode should be used. If the most probable mode m is not selected, a code number from 0 to 7 is sent to indicate which of the remaining 8 modes should be used. The code number is sent as 3 bit codeword, which is its binary representation. When the code number n is received the used prediction mode is equal to:

n, if n<m

n+1, otherwise.

Only two contexts are used when coding prediction modes with CABAC, one for the first bin of the mode and one for the rest. The most probable mode is assigned code number zero and the rest of the modes are assigned codes in their increasing numerical order similarly to the above.

## Results

The proposed method is implemented in JM 2.0 and the results of the simulations are summarized in Table 1 in terms of Bjontegaard Delta bitrate savings in % comparing to JM2.0. In addition results are given Table 2 when the proposed method is used in conjunction with the simplified “Horizontal Up prediction” prediction mode as proposed by Real Networks. The test conditions from VCEG-N81 were followed with the exception that all frames were encoded as Intra. We see that the average Bjontegaard Delta bitrate savings for the proposed simplifications, with respect to the original TML code is negative, indicating that for this data set our proposed simplifications produces better results.

Table 1: Performance improvement (Bjontegaard Delta bitrate savings in %) of the proposed coefficient coding in Intra coding. QP = 0, 4, 8, 12 is used for low QP and QP = 16, 20, 24, 28 for high QP.

| QP | News | Foreman | Container | Silent | Mobile | Paris | Tempete | Average |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Low | -0.461 | -1.224 | -0.611 | -1.341 | -0.915 | -0.471 | -0.924 | -0.84957 |
| High | 0.91 | -0.987 | 0.347 | -1.361 | -1.574 | 0.207 | -0.986 | -0.492 |

Table 2: Performance improvement (Bjontegaard Delta bitrate savings in %) of the proposed coefficient coding in Intra coding. QP = 0, 4, 8, 12 is used for low QP and QP = 16, 20, 24, 28 for high QP.

| QP | News | Foreman | Container | Silent | Mobile | Paris | Tempete | Average |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Low | -0.701 | -1.69 | -0.946 | -1.771 | -1.129 | -0.732 | -1.122 | -1.15586 |
| High | 0.295 | -2.75 | -0.626 | -2.237 | -1.978 | -0.241 | -1.406 | -1.27757 |

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Nokia Inc. |  |  |
| Mailing address | 6000 Connection Drive, Irving, TX 75039 USA |  |  |
| Country | USA |  |  |
| Contact person | Marta Karczewicz |  |  |
| Telephone | +1 972 894 4188 |  |  |
| Fax |  |  |  |
| Email | marta.karczewicz@nokia.com |  |  |
| Place and date of submission | Klagenfurt, Austria, 22/7/2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Analysis and Simplification of Intra Prediction |  |
| Contribution number |  | JVT-D025 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: