| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 4th Meeting: Klagenfurt, Austria, 22-26 July, 2002 | Document: JVT-D030 Filename: JVT-D030.doc Generated: 2002-07-15 |
| --- | --- |

| Title: | Rate Control on JVT Standard |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Siwei Ma Institute of Computing Technology, Chinese Academy of Sciences, Beijing, 100080, China Wen Gao Institute of Computing Technology, Chinese Academy of Sciences, Beijing, 100080, China Yan Lu Department of Computer Science, Harbin Institute of Technology, Harbin, 150001, China | Tel: Email: Email: Email: | +8610 82649015 swma@jdl.ac.cn wgao@jdl.ac.cn ylu@ieee.org |
| Source: | Institute of Computing Technology, CAS, China |  |  |

_____________________________

- Introduction

Rate control plays a very important role in constant bit rate (CBR) video coding. Although it’s not a normative tool for any video coding standard, every video coding standard recommends its own rate control algorithm such as TM5 in MPEG-2 and TMN8 in H.263. Due to high coding efficiency, JVT as a promising video coding standard was already developed to the committee draft (CD) stage. However, unlike the existing video coding standards, rate control in JVT standard becomes quite difficult especially at macroblock level because both rate control and rate-distortion optimization (RDO) will involve the quantization parameters. In this proposal, we develop an efficient rate control algorithm at macroblock level for the coming JVT standard by considering both rate control and RDO. The proposed rate control is developed from TM5 [1], however, it outperforms the original TM5 up to 1.0dB on average PSNR at the same bit rate. In the proposed rate control, the sum of absolution difference (SAD) after intra prediction or motion compensation is used to define the macroblock activity, which can reflect the macroblock activity more accurately than the original macroblock variance used in TM5.

- Proposed Rate Control On JVT

Figure 1 illustrates the block diagram of JVT with the proposed rate control. In the proposed rate control, the sum of absolute difference (SAD) of each macroblock after either motion compensation or intra prediction is used to measure macroblock activity. Firstly, the quantization parameter of the previous macroblock is used as the estimated quantization parameter of the current macroblock for rate-distortion optimization. After the preliminary coding mode is decided by rate-distortion criterion, we can calculate the macroblock activity as well by the selected coding mode and motion vectors. Secondly, according to the macroblock activity and the current virtual buffer occupancy, the quantization parameter for the current macroblock is calculated with the similar model used in TM5. Using the new quantization parameter, RDO is performed again for finding the final optimal coding mode. At last, the obtained residue is encoded with either UVLC or CABAC. The proposed rate control algorithm is described in detail as follows.

Step 1: Bit allocation.

The proposed bit allocation is first performed at picture level, which is the same as that in TM5. Ti, Tp and Tb denote the bits allocated to I, P, B frame, respectively, which are calculated by:

Ti=maxR1+NpXpKpXi+NbXbKbXi,bitrate8×picturerate,

Tp=maxRNp+NbKpXbKbXp,bitrate8×picturerate,

Tb=maxRNb+NpKbXpKpXb,bitrate8×picturerate,

where Kp and Kb [2]are constants. They reflect the complexity portion among I, P, and B frames. In this proposal, we select Kp =1.1 and Kb =1.5. Np and Nb are the numbers of P pictures and B pictures remaining in the current group of picture (GOP) in the encoding order. Xi, Xp and Xb denote the complexity estimation of a certain type picture (I, P, or B). In the proposed rate control, Xi, Xp, Xb are initialized with Xi=(150*bit_rate)/115, Xp=(25*bit_rate)/115, and Xb=(18*bit_rate)/115, respectively. R is the remaining number of bits assigned to the GOP. After encoding a picture, R is updated as follows,

R = R - Si,p,b ,

where Si,p,b is the number of bits consumed in the picture just encoded (picture type may be I, P or B). Before encoding the first picture in a GOP, considering the rest or exceeding bits Rprev of previous GOP, initial R is given as follows,

R = G + Rprev,

where				    G = bit_rate*N/picture_rate,

and N is the number of pictures in the GOP. For the first GOP Rprev= 0.

Step 2: First time rate-distortion optimization.

The preliminary coding mode for the current macroblock m is found by using the quantization parameter Qm-1 of the macroblock m-1. The coding mode is selected by minimizing:

D(s,c,MODE|QP)+λMODER(s,c,MODE|QP),

with

λMODE=0.85×2Qm−13      for I, P frame,

or

λMODE=4×0.85×2Qm−13    for B frame [3][4].

If the picture is P or B, considering the SAD is used as criterion, the lambda in motion estimation is λMOTION=λMODE.

Step 3: Computing the macroblock quantization parameter.

After motion compensation with the selected coding mode and motion vectors, the macroblock activity is calculated by:

Nactm=2×actj+avgactactj+2×avgact,

actm=i,j|s(i,j)−c(i,j)|, i,j=1, 2, … , 16,

where s(i,j) is the luminance of original pixel (i,j), c(i,j) is the prediction of pixel (i,j), and avg_act is the average value of actj in the previously coded same type picture. The avg_act is initialized with avg_acti=1000, avg_actp=900, and avg_actb=800.

The quantization parameter Qm of macroblock m is decided by virtual buffer occupancy[5] and the macroblock activity, i.e.

Qm=dmn×31rNactm,

r=2×bitrateframerate,

dmn=d0n+Bm−1−Tn×(m−1)MBCNT, n = i, p, b,

where dmn is the current virtual buffer occupancy. dmi, dmp, and dmb are the fullness of virtual buffers at macroblock j for each picture type. The final fullness of the virtual buffer (dmi, dmp, dmb : m=MB_CNT) is used as d0i, d0p, and d0b for encoding the next frame. Bm-1 is the number of bits generated by encoding all macroblocks in the picture up to an including macroblock m-1. MB_CNT is the number of macroblocks in a frame.

The initial value for the virtual buffer fullness is:

d0i=10×r31,

d0p=Kp×d0i,

d0b=Kb×d0i.

Step 4: Second rate-distortion computing.

Using the new quantization parameter, RDO is performed again in finding the optimal coding mode for the macroblock to be coded. Afterwards, the block is encoded with the selected coding mode and the decided quantization parameter. The virtual buffer is updated as well.

- Experiments and Results

The proposed rate control is implemented in JVT JM2.0 codec. Since the rate control algorithm is derived from TM5 model, we also have implemented TM5 rate control on JVT JM2.0 as a reference. Comparisons between the results achieved by the proposed rate control and TM5 are discussed in this section. In the test, a GOP is composed of 15 frames. Every GOP contains one I frame. The other test conditions are listed in Table 1.

Table 2 shows the coding results with the proposed rate control on several test sequences. From the table, we can see that the proposed algorithm can accurately control the generated bit rate at different resolutions and frame rates. The error between target bit rate and real bit bate is below 1%. Table 3 shows the average PSNR gain compared between TM5/Proposed rate control and without rate control. The PSNR is calculated by averaging some typical bit-rates. For example, it was sampled at 16, 24, 32, …, 72kbps for the Foreman sequence at 10 fps. Figure 2~7 gives the rate-distortion curves with rate control, no rate control and TM5, respectively. It has been shown that the proposed rate control can still keep high coding efficiencies. In other words, the proposed rate control achieves higher coding efficiency rather than TM5. And also, at high bit rate, the PSNR achieved with the proposed rate control might be higher than that without rate control. At low bit rate, this might lead to a bit lower PSNR than that without rate control. It is reasonable because of the penalty for changing macroblock quantization parameter on RDO. Figure 8 shows the PSNR per frame for the test sequence coded using our proposed rate control and the PSNR per frame for the same test sequence without rate control at the same bit rate.

Furthermore, in Figure 9, multi-iterative results are demonstrated. From the figure, we can see that multiple iterations only get little gain on coding efficiency. To reduce encoding complexity, it is enough to do only one iteration.

Table 1 Test Conditions

| MV resolution | 1/4 pel |
| --- | --- |
| Hadamard | ON |
| RD optimization | ON |
| Search Range | 16 |
| Restrict Search Range | 2 |
| Reference Frames | 2 |
| Symbol Mode | CABAC |
| GOP structure | IPP or IBBP |

| Table 2. Proposed rate control test on several sequences Stream | Type | Skip/Bframe (GOP 15) | Frame rate | Target bit-rate (kbps) | Achieved bit-rate (kbps) | PSNRY(dB) |
| --- | --- | --- | --- | --- | --- | --- |
| Foreman | QCIF | 2 | 10 | 24.00 | 24.05 | 32.91 |
| Mother | QCIF | 2 | 10 | 17.00 | 17.00 | 33.45 |
| Foreman | QCIF | 2 | 30 | 73.00 | 73.11 | 33.02 |
| Coastguard | CIF | 2 | 30 | 401.00 | 401.14 | 30.09 |
| Basket | CIF | 2 | 30 | 531.00 | 531.77 | 30.02 |
| Mobile | CIF | 0 | 30 | 646.00 | 646.87 | 29.37 |

Table 3. Average PSNR gain compare between TM5/Proposed rate control and no rate control

| Stream | Type | Skip/Bframe (GOP 15) | Frame rate | Proposed rate control | TM5 |
| --- | --- | --- | --- | --- | --- |
| Foreman1 | QCIF | 2 | 10 | -0.2128 | -0.4815 |
| Mother2 | QCIF | 2 | 10 | -0.3277 | -0.8670 |
| Foreman3 | QCIF | 2 | 30 | -0.1002 | -0.7737 |
| Coastguard4 | CIF | 2 | 30 | -0.1729 | -0.6065 |
| Basket5 | CIF | 2 | 30 | -0.3162 | -0.6862 |
| Mobile6 | CIF | 0 | 30 | -0.2611 | -1.9938 |

Figure 1. Proposed rate control scheme on JVT

Figure 2. PSNR graph for Foreman_qcif.yuv 30f/s with JVT/JM20 codec

Figure 3. PSNR graph for Foreman_qcif.yuv 10f/s with JVT/JM20 codec

Figure 4. PSNR graph for Mother_daughter_qcif.yuv 10f/s with JVT/JM20 codec

Figure 5. PSNR graph for Coastguard_cif.yuv 30f/s with JVT/JM20 codec

Figure 6. PSNR graph for basket_cif.yuv 30f/s with JVT/JM20 codec

Figure7. PSNR graph for mobile_cif.yuv 30f/s with JVT/JM20 codec

Figure 8. Comparison of PSNR values per frame for Foreman_qcif.yuv 30f/s 146kbps

Figure 9. Comparison of multi-iterative results

Reference

- Test Model 5, www.mpeg.org/MPEG/MSSG/tm5

- Jiro Katto and Mutsumi Ohta, "Mathematical Analysis of MPEG Compression Capability and Its Application to Rate Control", IEEE ICIP’95 Oct.1995.

- Joint Video Team (JVT) of ISO/IEC MPEG and ITU-T VCEG Document JVT-B118R3, 2002-03-25.

- Thomas Wiegand and Bernd Girod, "Parameter Selection In Lagrangian Hybrid Video Coder Control", ICIP2001.

- Yoo-Sok Saw, Peter M. Grant, and John M.Hannah, "Feed-Forward Buffering And Rate Control Based On Scene Change Features For MPEG Video Coder", in EUSIPCO’96, vol.II, pp.727-730

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Institute of Computing Technology |  |  |
| Mailing address | Ke Xue Yuan South Road 6. Haidian, Beijing, 100080, China |  |  |
| Country | China |  |  |
| Contact person | Wen Gao |  |  |
| Telephone | (86-10) 8264-9208 |  |  |
| Fax | (86-10) 8264-9298 |  |  |
| Email | wgao@jdl.ac.cn |  |  |
| Place and date of submission | 7/12/2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Rate Control on JVT Standard |  |
| Contribution number |  | D030 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: