| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 4th Meeting: Klagenfurt, Austria, 22-26 July, 2002 | Document: JVT-D032 Filename: JVT-D032.doc |
| --- | --- |

| Title: | Frame number (PN) continuity at SP-picture |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shinya Kadono, Satoshi Kondo and Kiyofumi Abe 1006 Kadoma, Kadoma Osaka 573-8501 Japan | Tel: Email: | +81-6-6900-9689 kadono@drl.mei.co.jp |
| Source: | Matsushita Electric Industrial Co., Ltd. |  |  |

_____________________________

# Introduction

S-pictures have functionalities for bit stream switching. However, the prediction structure has to be strictly limited to guarantee the uniqueness after bit stream switching, for example PN of stored pictures should be the same and PN should be continuous when stream is switched. Otherwise, we have inconsistency of error detection mechanism and inconsistency of RPSL and MMCO operation results after SP-picture decoding.

We disclosed one solution to solve these problems in JVT-C030 at Fairfax meeting. In this contribution, we propose more sophisticated solution, which introduces a frame_num_s element into picture header syntax of SP-pictures.

# Problems of bit stream switching

Frame number (PN) is incremented by one after stored picture is encoded. Therefore PN is very useful to detect an unintentional picture loss and conceal by checking how many PN is incremented.

Fig. 1 Picture type and PN for two bit streams

However, PN of two bit streams, which have different frame rates, may have different PN at SP-picture as shown in Fig. 1.

After bit stream switching from stream 1 to stream 2 PN of the following S-picture of stream 2 is 6, while PN of the S-picture switched from stream 1 is 10. In this case, PN changed from 10 to 6 after S-picture. Therefore, decoder cannot detect an unintentional picture loss by checking PN. This is especially serious if required_frame_num_update_behaviour is 1, which means a specific decoder behaviour in case of missing frame numbers is mandated.

Another problem is occurred when we use reference picture selection layer (RPSL) and memory management control operation (MMCO). As RPSL and MMCO use PN to indicate the target picture, PN shall be the same for the same picture. However, PN of SP-picture of stream 2 is 5 while PN of SP-picture switched from stream 1 is 10. Therefore, we cannot use RPSL and MMCO to ensure consistency.

To ensure the consistency after switching, the stored picture after SP-picture decoding has to be completely the same in both switched stream and non switched stream. That is, PN of the decoded picture should be continuous before and after SP-picture and PN of the stored picture (decoded SP-picture) should be the same even if the stream is switched at SP-picture.

# Proposed solutions

A new syntax element frame_num_s is introduced in the picture layer RBSP syntax when picture coding type is S-picture. frame_num indicates a frame number for decoding the current SP-picture and frame_num_s indicates the frame number assigned after the current SP-picture decoded. For example, in Fig.1, SP-picture from the stream 1 has frame_num = 10 and frame_num_s = 5, and SP-picture of the stream 2 has frame_num = 5 and frame_num_s = 5.

By using frame_num_s, decoder can check continuity by comparing frame_num_s and the frame_number of the following picture, so error concealment method can apply if the frame number is not continuous.

# Proposed revised text

Picture layer RBSP syntax

| picture_layer_rbsp( ) { | Category | Descriptor |
| --- | --- | --- |
| picture_structure | 3 | e( v ) |
| frame_num | 3 | u( v ) |
| if( coding_type( ) = = SP) |  |  |
| frame_num_s | 3 | u(v) |
| rps_layer( ) |  |  |
| if( coding_type( ) = = Bipred ) { |  |  |
| direct_mv_scale_fwd | 3 | e( v ) |
| direct_mv_scale_bwd | 3 | e( v ) |
| direct_mv_scale_divisor | 3 | e( v ) |
| explicit_bipred_weight_indicator | 3 | e( v ) |
| if( explicit_bipred_weight_indicator > 1 ) |  |  |
| adaptive_B_prediction_coeff_table( ) | 3 |  |
| } |  |  |
| rbsp_trailing_bits( ) | 3 |  |
| } |  |  |

frame_num_s labels the frame. frame_num_s appears when picture coding type is SP. For SP-picture, frame_num is used for decoding the current picture, while frame_num_s is used as the frame_num of the current decoded picture. frame_num_s of a frame to be added to the multi-frame buffer shall not be equal to any other frame_num among the short-term frames in the multi-frame buffer.

# Conclusion

By introducing frame_num_s for SP-picture, problems for S-picture relating PN can be solved. By using this solution, error detection and concealment using PN can be achievable. Therefore we propose to incorporate this modification into FCD.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Matsushita Electric Industrial Co., Ltd. |  |  |
| Mailing address | 1006 Kadoma, Kadoma, Osaka, 573-8501 |  |  |
| Country | Japan |  |  |
| Contact person | Shinya Kadono |  |  |
| Telephone | +81-6-6900-9689 |  |  |
| Fax | +81-6-6900-9674 |  |  |
| Email | kadono@drl.mei.co.jp |  |  |
| Place and date of submission | 4th Meeting: Klagenfurt, Austria, 22-26 July, 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Frame number (PN) continuity at SP-picture |  |
| Contribution number |  | JVT-D032 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: