| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 4th Meeting: Klagenfurt, Austria, 22-26 July, 2002 | Document: JVT-D047 Filename: JVT-D047.doc |
| --- | --- |

| Title: | Verification Results for ABT Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | David Baylon & Limin Wang [presenter] Motorola, Inc. Broadband Communications Sector 6450 Sequence Drive San Diego, CA 92121 | Tel: Email: | 858-404-2285 david.baylon@motorola.com liwang@gi.com |
| Source: | Motorola, Inc., BCS |  |  |

_____________________________

# Summary

This contribution presents some verification results for ABT coding.  In particular, the focus in on the performance of ABT with interlaced sequences using the alternate scans proposed in [JVT-C140].  Simulations were performed using an implementation provided by Mathias Wien based on JM2.1.  Results show significant bitrate savings with ABT coding over non-ABT coding, especially for field coding of interlaced sequences.

# Introduction

This document presents results which show the performance gains that can be achieved with ABT coding over non-ABT coding.  In [JVT-C140] the authors attempted to demonstrate additional gains over zigzag ABT coding with the use of alternate scanning patterns.  In this contribution, the comparison of ABT coding (with alternate scanning patterns) over non-ABT coding is made in order to verify the overall gain with ABT.

The simulation results are given here using software provided by Mathias Wien based on JM2.1.  This version has the adaptive frame/field picture coding tool which is enabled while comparisons are made with ABT on and off.  In contrast, the results reported in [JVT-C140] were obtained from software based on TML9 (also courtesy of Mathias Wien) which did not have frame/field picture coding.  As a consequence, there were some inconsistencies in the data in [JVT-C140] which are no longer present in the results here.

The ABT coefficient scans for frame coding are the zigzag scans shown in [JVT-C107].  The ABT scans for field coding are the alternate scans proposed in [JVT-C140] and repeated here on page 2, except that now the scans start with “0” instead of “1” (to be consistent with the labeling of the baseline 4x4 scan).  It was reported in [JVT-C140] that the 4x4 alternate scan was the same as used in [JVT-B068].  This turns out not to be the case as the two 4x4 scans are in fact different.

If the zigzag scans are considered as having a 45o scan angle, then the alternate scans can be thought of having somewhere between a 45o and 90o (vertical) scan angle.  Some additional alternate scans for ABT were trained on JM2.1 which also had between 45o to 90o angles, but their performance did not significantly increase over the [JVT-C140] alternate scans.

| 0 | 2 | 8 | 12 |
| --- | --- | --- | --- |
| 1 | 5 | 9 | 13 |
| 3 | 6 | 10 | 14 |
| 4 | 7 | 11 | 15 |

4x4 alternate scan

| 0 | 4 | 12 | 20 |
| --- | --- | --- | --- |
| 1 | 5 | 13 | 21 |
| 2 | 6 | 14 | 22 |
| 3 | 11 | 19 | 27 |
| 7 | 15 | 23 | 28 |
| 8 | 16 | 24 | 29 |
| 9 | 17 | 25 | 30 |
| 10 | 18 | 26 | 31 |

4x8 alternate scan

| 0 | 2 | 6 | 10 | 14 | 18 | 22 | 26 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 5 | 9 | 13 | 17 | 21 | 25 | 29 |
| 3 | 7 | 11 | 15 | 19 | 23 | 27 | 30 |
| 4 | 8 | 12 | 16 | 20 | 24 | 28 | 31 |

8x4 alternate scan

| 0 | 3 | 8 | 15 | 22 | 30 | 38 | 52 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 4 | 14 | 21 | 29 | 37 | 45 | 53 |
| 2 | 7 | 16 | 23 | 31 | 39 | 46 | 58 |
| 5 | 9 | 20 | 28 | 36 | 44 | 51 | 59 |
| 6 | 13 | 24 | 32 | 40 | 47 | 54 | 60 |
| 10 | 17 | 25 | 33 | 41 | 48 | 55 | 61 |
| 11 | 18 | 26 | 34 | 42 | 49 | 56 | 62 |
| 12 | 19 | 27 | 35 | 43 | 50 | 57 | 63 |

8x8 alternate scan

# Experimental Results

Simulations using the JM2.1-based version were performed under the coding conditions shown below.  Adaptive frame/field encoding was enabled while ABT was switched on and off.  The case with ABT off formed the reference case.  The six interlaced sequences shown below were tested.  All of these sequences used primarily field-based encoding except Tempete.  Table 1 shows the Bjontegaard Delta bitrate savings (in %) [VCEG-M33] of ABT coding over non-ABT coding.  The RD curves for the II, IP, and IBBP cases are also shown in the plots on the following pages.

| Picture Structure (adaptive frame/field) | II, IP, IBBP |
| --- | --- |
| Quantization Parameters | 16, 20, 24, 28 [QP(B) = QP(I/P)+2] |
| RD Optimization | On |
| Motion Resolution | ¼ pel |
| Search Range | 16 pixels |
| Hadamard | On |
| Entropy Coding | CABAC |
| Reference Frames | 2 |
| Adaptive Frame/Field Encoding | On |
| ABT Encoding | Inter and Intra |

Common encoding conditions

| Sequence Name | Frame Size | # Frames |
| --- | --- | --- |
| Bus | 720x480 | 150 |
| Football | 720x480 | 260 |
| Tempete | 720x480 | 260 |
| Canoa | 720x576 | 220 |
| Rugby | 720x576 | 220 |
| F1car | 720x576 | 220 |

Interlaced test sequences

|  | Bus | Football | Tempete | Canoa | Rugby | F1car |
| --- | --- | --- | --- | --- | --- | --- |
| II IP | 4.94 4.98 | 9.19 10.9 | 8.87 3.41 | 14.0 18.4 | 9.80 12.0 | 6.05 5.66 |
| IBBP | 7.69 | 9.91 | 4.75 | 14.7 | 10.3 | 7.17 |

Table 1.  Bitrate savings (%) of ABT coding over reference (non-ABT) coding

# Conclusion

The results in Table 1 show that there are significant bitrate savings with ABT coding of the interlaced sequences.  With the exception of Tempete, all the sequences used primarily field-based encoding.  The RD plots show that the bitrate savings occur over a wide range of bitrates.

# References

[JVT-C140]	Limin Wang and David Baylon, “Alternate Coefficient Scanning Patterns for Interlaced ABT Coding”, Document JVT-C140, Joint Video Team (JVT) of ISO/IEC MPEG and ITU-T VCEG, Fairfax, Virginia, USA, May 6-10, 2002.

[JVT-C107]	Mathias Wien and Achim Dahlhoff, “16 bit Adaptive Block size Transforms”, Document JVT-C107, Joint Video Team (JVT) of ISO/IEC MPEG and ITU-T VCEG, Fairfax, Virginia, USA, May 6-10, 2002.

[JVT-B068]	Kazushi Sato, Teruhiko Suzuki and Yoichi Yagasaki, “New Interlace Coding

Tools”, Document JVT-B068, Joint Video Team (JVT) of ISO/IEC MPEG and ITU-T VCEG, Geneva, Switzerland, Jan 29 - Feb. 1, 2002.

[VCEG-M33] 	Gisle Bjontegaard, “Calculation of average PSNR differences between RD-curves”, Document VCEG-M33, Austin, Texas, USA, April 2-4, 2001.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Motorola, Inc. |  |  |
| Mailing address | ## 6450 Sequence Drive San Diego, CA 92121 |  |  |
| Country | U.S.A. |  |  |
| Contact person | Limin Wang |  |  |
| Telephone | 858-404-3893 |  |  |
| Fax | 858-404-2501 |  |  |
| Email | liwang@gi.com |  |  |
| Place and date of submission | San Diego, CA, USA, 7/16/02 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Verification Results for ABT Coding |  |
| Contribution number |  | JVT-D047 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. see comments below regarding Baseline Profile |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  | Comment to Section 2.2. To support the proposed royalty free JVT Baseline Profile, and in addition to the statement of 2.2 above, Patent Holder is also willing to grant a royalty free license to patents essential to JVT Baseline Profile decoder applications on the condition that any and all other patent holders do the same. |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: