| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 4th Meeting: Klagenfurt, Austria, 22-26 July, 2002 | Document: JVT-D053 Filename: JVT-D053.doc |
| --- | --- |

| Title: | Changes for ABT |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mathias Wien Institut und Lehrstuhl für Nachrichtentechnik RWTH Aachen 52056 Aachen, Germany | Tel: Email: | ++49-241-80-27681 wien@ient.rwth-aachen.de |
| Source: | Aachen University |  |  |

_____________________________

# Introduction

In this contribution, some clean-up for technical details and some editorial changes are proposed for the description of Adaptive Block-size Transforms (ABT) in the Joint Committee Draft [1]. The changes relative into JVT-D017d0.doc are given in the acompanying document JVT-D053_changes_to_D017d0.doc with change marks on.

The technical changes regard the integration of the useABT flag in the slice header and the harmonization of the scaling with the modified quantization range of the JM. The scaling table used for the quantization of 4x4 blocks corresponds to table used for base line quantization.

# Proposed changes for quantization

The modified scaling table from Clause 14.3.2 is given below. The S4x4 values are the scaling factors from Clause 9.5.2, multiplied by 4 to simplify the description in the ABT section. Since QP factors below  QPnew = = 12  induce a violation of the 16 bit constraint for transform coefficients of the 8x8 blocks, ABT is not used below QPnew = 12.

Table 14-1 – ABT scaling mantissa values

| k | S8x8 | S8x4,4x8 |  | S4x4 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 15 | 9 | 11 | 40 | 64 | 52 |
| 1 | 17 | 10 | 12 | 44 | 72 | 56 |
| 2 | 19 | 11 | 14 | 52 | 80 | 64 |
| 3 | 22 | 12 | 16 | 56 | 92 | 72 |
| 4 | 24 | 14 | 17 | 64 | 100 | 80 |
| 5 | 27 | 15 | 20 | 72 | 116 | 92 |

The application of these new scaling factors results in very minor changes of the coding performance with a slight improvement when using the new scaling factors. Table 3 shows the Bjontegaard Delta measurement [2] comparison for ABT with both, the old and the new scaling table. The performance is improved by 0.02 dB resp. 0.4% bitrate savings for the IPPP sequences structure and by 0.016 dB resp. 0.2% bitrate savings for the IIII sequences structure. The common conditions sequences were encoded in IPPPP and IIII sequence structures. The results were generated with the ABT implementation into JM21 which was distributed via the ABT AhG reflector [3]. The encoder configuration is given in Table 2:

Table 2 – Simulation Parameters

| Parameter | Value |
| --- | --- |
| Sequence Type | IPPP… or IIII… |
| QPFirstFrame | 16, 20, 24, 28 |
| QPRemainingFrame | 16, 20, 24, 28 |
| MVResolution | 1/4 Pel |
| UseHadamard | ON |
| SearchRange | 16 |
| NumberReferenceFrames | 2 |
| MB modes | all |
| NumberBFrames | 0 |
| RDOptimization | ON |

Table 3 – BDSNR comparison: Old Scaling Factors vs. New Scaling Factors

|  |  |  | IP |  | III |  |
| --- | --- | --- | --- | --- | --- | --- |
| Sequence | Freq. | #Frames | SNR | Rate | SNR | Rate |
| Container QCIF | 10Hz | 100 | 0.047 dB | -0.99% | 0.007 dB | -0.11% |
| Foreman QCIF | 10Hz | 100 | 0.024 dB | -0.44% | 0.016 dB | -0.26% |
| News QCIF | 10Hz | 100 | -0.029 dB | 0.51% | 0.004 dB | -0.04% |
| Paris CIF | 15Hz | 150 | 0.009 dB | -0.24% | 0.022 dB | -0.40% |
| Silent QCIF | 15Hz | 150 | 0.024 dB | -0.45% | 0.014 dB | -0.18% |
| Mobile CIF | 30Hz | 300 | 0.035 dB | -0.75% | 0.025 dB | -0.28% |
| Tempete CIF | 30Hz | 300 | 0.030 dB | -0.72% | 0.023 dB | -0.30% |
| Mean  Results: |  |  | 0.020 dB | -0.44% | 0.016 dB | -0.22% |

# References

[1]	Thomas Wiegand (Ed.), 'Joint Committee Draft: Editor's Proposed Text Modifications,' Document JVT-D015d5, this meeting.

[2]	Gisle Bjontegaard, 'Calculation of average PSNR differences between RD curves,' Document VCEG-M33, ITU-T Video Coding Experts Group (VCEG) Meeting, Austin, TX, USA, 2-4 April 2001.

[3]	Mathias Wien, Software implementation of ABT into JM21, 
http://www.ient.rwth-aachen.de/~wien/jvt/jm21abt.zip

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Aachen University |  |  |
| Mailing address | Institut für Nachrichtentechnik RWTH Aachen 53056 Aachen |  |  |
| Country | Germany |  |  |
| Contact person | Mathias Wien |  |  |
| Telephone | ++49-241-80-27681 |  |  |
| Fax | ++49-241-80-22196 |  |  |
| Email | wien@ient.rwth-aachen.de |  |  |
| Place and date of submission | Klagenfurt, July 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Changes for ABT |  |
| Contribution number |  | JVT-D053 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
| x | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| x | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  | Robert Bosch GmbH |  |  |
| Mailing address |  | PO Box 106050 70049 Stuttgart |  |  |
| Country |  | Germany |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: