| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 4th Meeting: Klagenfurt, Austria, 22-26 July, 2002 | Document: JVT-D053 Filename: JVT-D053r1.doc |
| --- | --- |

| Title: | Changes for ABT |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mathias Wien Institut und Lehrstuhl für Nachrichtentechnik RWTH Aachen 52056 Aachen, Germany | Tel: Email: | ++49-241-80-27681 wien@ient.rwth-aachen.de |
| Source: | Aachen University |  |  |

_____________________________

# Introduction

In this contribution, some clean-up for technical details and some editorial changes are proposed for the description of Adaptive Block-size Transforms (ABT) in the Joint Committee Draft [1]. The changes relative into JVT-D017d0.doc are given in the accompanying document JVT-D053_changes_to_D017d0.doc with change marks on.

The technical changes regard the integration of the useABT flag in the slice header and the harmonization of the scaling with the modified quantization range of the JM. The scaling table used for the quantization of 4x4 blocks corresponds to table used for base line quantization.

# Proposed changes for quantization

The modified scaling table from Clause 14.3.2 is given below. The S4x4 values are the scaling factors from Clause 9.5.2, multiplied by 4 to simplify the description in the ABT section. Since QP factors below  QPnew = = 12  induce a violation of the 16 bit constraint for transform coefficients of the 8x8 blocks, ABT is not used below QPnew = 12.

Table 14-1 – ABT scaling mantissa values

| k | S8x8 | S8x4,4x8 |  | S4x4 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 15 | 9 | 11 | 40 | 64 | 52 |
| 1 | 17 | 10 | 12 | 44 | 72 | 56 |
| 2 | 19 | 11 | 14 | 52 | 80 | 64 |
| 3 | 22 | 12 | 16 | 56 | 92 | 72 |
| 4 | 24 | 14 | 17 | 64 | 100 | 80 |
| 5 | 27 | 15 | 20 | 72 | 116 | 92 |

The application of these new scaling factors results in very minor changes of the coding performance with a slight improvement when using the new scaling factors. Table 3 shows the Bjontegaard Delta measurement [2] comparison for ABT with both, the old and the new scaling table. The performance is improved by 0.02 dB resp. 0.4% bitrate savings for the IPPP sequences structure and by 0.016 dB resp. 0.2% bitrate savings for the IIII sequences structure. The common conditions sequences were encoded in IPPPP and IIII sequence structures. The results were generated with the ABT implementation into JM21 which was distributed via the ABT AhG reflector [3]. The encoder configuration is given in Table 2:

Table 2 – Simulation Parameters

| Parameter | Value |
| --- | --- |
| Sequence Type | IPPP… or IIII… |
| QPFirstFrame | 16, 20, 24, 28 |
| QPRemainingFrame | 16, 20, 24, 28 |
| MVResolution | 1/4 Pel |
| UseHadamard | ON |
| SearchRange | 16 |
| NumberReferenceFrames | 2 |
| MB modes | all |
| NumberBFrames | 0 |
| RDOptimization | ON |

Table 3 – BDSNR comparison: Old Scaling Factors vs. New Scaling Factors

|  |  |  | IP |  | III |  |
| --- | --- | --- | --- | --- | --- | --- |
| Sequence | Freq. | #Frames | SNR | Rate | SNR | Rate |
| Container QCIF | 10Hz | 100 | 0.047 dB | -0.99% | 0.007 dB | -0.11% |
| Foreman QCIF | 10Hz | 100 | 0.024 dB | -0.44% | 0.016 dB | -0.26% |
| News QCIF | 10Hz | 100 | -0.029 dB | 0.51% | 0.004 dB | -0.04% |
| Paris CIF | 15Hz | 150 | 0.009 dB | -0.24% | 0.022 dB | -0.40% |
| Silent QCIF | 15Hz | 150 | 0.024 dB | -0.45% | 0.014 dB | -0.18% |
| Mobile CIF | 30Hz | 300 | 0.035 dB | -0.75% | 0.025 dB | -0.28% |
| Tempete CIF | 30Hz | 300 | 0.030 dB | -0.72% | 0.023 dB | -0.30% |
| Mean  Results: |  |  | 0.020 dB | -0.44% | 0.016 dB | -0.22% |

# References

[1]	Thomas Wiegand (Ed.), 'Joint Committee Draft: Editor's Proposed Text Modifications,' Document JVT-D015d5, this meeting.

[2]	Gisle Bjontegaard, 'Calculation of average PSNR differences between RD curves,' Document VCEG-M33, ITU-T Video Coding Experts Group (VCEG) Meeting, Austin, TX, USA, 2-4 April 2001.

[3]	Mathias Wien, Software implementation of ABT into JM21, 
http://www.ient.rwth-aachen.de/~wien/jvt/jm21abt.zip

# CCIR Simulation Results

A set of simulation results for the CCIR test sequences is reported to give an update on ABT performance in the JM software. Results are presented relative to JM21 using the corresponding ABT implementation [3]. Results for CABAC and for VLC entropy coding are provided. The setting for the coder are given below.

| Parameter | Value |
| --- | --- |
| Sequence Type | IBBP |
| QPFirstFrame | 12, 16, 20, 24 |
| QPRemainingFrame | 12, 16, 20, 24 |
| QPBFrame | 14, 18, 22, 26 |
| MVResolution | 1/4 Pel |
| UseHadamard | ON |
| SearchRange | 16 |
| NumberReferenceFrames | 2 |
| MB modes | all |
| Adaptive Frame / Field Coding | ON |
| RDOptimization | ON |
| AdaptiveBlockTransorms | Inter and Intra |

Parameter Settings

| Sequence | Size | # Frames | SNR VLC | Rate VLC | SNR CABAC | Rate CABAC |
| --- | --- | --- | --- | --- | --- | --- |
| Bus | 720x480 | 150 | 0.159 dB | -3.31% | 0.256 dB | -5.43% |
| Canoa | 720x576 | 220 | 0.208 dB | -5.59% | 0.263 dB | -7.24% |
| F1_car | 720x576 | 220 | 0.445 dB | -8.83% | 0.517 dB | -10.27% |
| Football | 720x480 | 260 | 0.433 dB | -8.30% | 0.573 dB | -11.12% |
| Rugby | 720x576 | 220 | 0.487 dB | -9.18% | 0.576 dB | -11.04% |
| Tempete | 720x480 | 260 | -0.039 dB | 1.04% | 0.126 dB | -3.39% |

BDSNR Results

## VLC RD Plots

bus ccir jm21 vlc IBBP

QP     Rate	PSNR

24  2302255	28.69

20  4038411	31.28

16  7454209	34.04

12 13448998	36.93

bus ccir abt2 vlc IBBP

QP     Rate	PSNR

24  2511212	29.11

20  4342461	31.67

16  7535743	34.34

12 13054658	37.09

12 13054658	37.09

canoa ccir jm21 vlc IBBP

QP     Rate	PSNR

24  4110794	28.43

20  7750659	30.76

16 13943315	32.95

12 27252236	35.38

canoa ccir abt2 vlc IBBP

QP     Rate	PSNR

24  4039598	28.62

20  7791115	31.05

16 14507518	33.26

12 28620556	35.66

12 28620556	35.66

f1 car ccir jm21 vlc IBBP

QP     Rate	PSNR

24  3415183	26.66

20  7628477	29.56

16 14189818	32.62

12 23919685	35.83

f1 car ccir abt2 vlc IBBP

QP     Rate	PSNR

24  3524292	26.87

20  7751244	29.94

16 13738659	33.12

12 22701075	36.45

12 22701075	36.45

football ccir jm21 vlc IBBP

QP     Rate	PSNR

24  2393219	29.27

20  4194872	31.68

16  7052588	34.38

12 11816612	37.15

football ccir abt2 vlc IBBP

QP     Rate	PSNR

24  2385424	29.57

20  4189186	32.12

16  7019450	34.81

12 11600082	37.51

12 11600082	37.51

rugby ccir jm21 vlc IBBP

QP     Rate	PSNR

24  4237205	29.43

20  7437766	31.91

16 12622516	34.54

12 21018375	37.33

rugby ccir abt2 vlc IBBP

QP     Rate	PSNR

24  4179482	29.66

20  7360094	32.26

16 12387471	35.02

12 20267190	37.86

12 20267190	37.86

tempete ccir jm21 vlc IBBP

QP     Rate	PSNR

24  1557316	29.15

20  3212051	31.58

16  6591898	34.18

12 12971876	36.87

tempete ccir abt2 vlc IBBP

QP     Rate	PSNR

24  1737638	29.47

20  3431859	31.74

16  6721016	34.23

12 12871442	36.85

12 12871442	36.85

## CABAC RD Plots

### IBBP

bus ccir jm21 cabac IBBP

QP     Rate	PSNR

24   982146	28.80

20  1797784	31.33

16  3278261	34.09

12  5862733	36.98

bus ccir abt2 cabac IBBP

QP     Rate	PSNR

24  1034730	29.26

20  1866222	31.75

16  3301217	34.39

12  5781429	37.14

canoa ccir jm21 cabac IBBP

QP     Rate	PSNR

24  1731559	28.54

20  3375383	30.79

16  6221980	32.97

12 12101200	35.42

canoa ccir abt2 cabac IBBP

QP     Rate	PSNR

24  1651250	28.72

20  3335775	31.07

16  6389492	33.27

12 12608824	35.76

f1 car ccir jm21 cabac IBBP

QP     Rate	PSNR

24  1481671	26.72

20  3375235	29.60

16  6292356	32.68

12 10468186	35.93

f1 car ccir abt2 cabac IBBP

QP     Rate	PSNR

24  1468389	26.95

20  3355842	30.02

16  6104629	33.19

12 10006218	36.47

football ccir jm21 cabac IBBP

QP     Rate	PSNR

24  1051765	29.39

20  1848969	31.77

16  3153882	34.44

12  5297137	37.20

football ccir abt2 cabac IBBP

QP     Rate	PSNR

24  1024122	29.74

20  1809433	32.23

16  3049316	34.89

12  5091208	37.57

rugby ccir jm21 cabac IBBP

QP     Rate	PSNR

24  1853962	29.57

20  3303941	31.98

16  5638139	34.58

12  9317021	37.38

rugby ccir abt2 cabac IBBP

QP     Rate	PSNR

24  1784680	29.79

20  3196338	32.31

16  5416811	35.06

12  8903732	37.91

tempete ccir jm21 cabac IBBP

QP     Rate	PSNR

24   698986	29.19

20  1463314	31.62

16  2979558	34.22

12  5757547	36.91

tempete ccir abt2 cabac IBBP

QP     Rate	PSNR

24   735413	29.57

20  1489781	31.82

16  2956604	34.30

12  5710387	36.91

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Aachen University |  |  |
| Mailing address | Institut für Nachrichtentechnik RWTH Aachen 53056 Aachen |  |  |
| Country | Germany |  |  |
| Contact person | Mathias Wien |  |  |
| Telephone | ++49-241-80-27681 |  |  |
| Fax | ++49-241-80-22196 |  |  |
| Email | wien@ient.rwth-aachen.de |  |  |
| Place and date of submission | Klagenfurt, July 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Changes for ABT |  |
| Contribution number |  | JVT-D053 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
| x | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| x | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  | Robert Bosch GmbH |  |  |
| Mailing address |  | PO Box 106050 70049 Stuttgart |  |  |
| Country |  | Germany |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: