| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 4th Meeting: Klagenfurt, Austria, 22-26 July, 2002 | Document: JVT-D062 Filename: JVT-D062.doc |
| --- | --- |

| Title: | Results of Improved Chroma Spatial Prediction for Intra Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Teruhiko Suzuki, Kazushi Sato and Yoichi Yagasaki Gate City Osaki East Tower 20 F 1-11-1, Osaki, Shinagawa-ku Tokyo, 141-0032, Japan | Tel: Email: | +81-3-5435-3669 teruhiko@av.crl.sony.co.jp kazushi@av.crl.sony.co.jp yagasaki@av.crl.sony.co.jp |
| Source: | Sony Corp. |  |  |

_____________________________

# Introduction

This contribution reports the results of improved chroma spatial prediction for Intra coding. It was proposed in JVT-C118 in Fairfax to improve the coding efficiency of chroma. The test is performed using HDTV (1920x1080 30 Hz interlaced) materials. RD characteristics and the subjective quality are compared.

# Experiments

The test conditions are as follows.

Sequence: Disco (1920x1080, 30 Hz interlaced video) (Very colorful video)

Picture Type: All Intra, and IBBP

QP: 16, 20, 24, 28

Entropy coding: CABAC
RD Optimization: On

ME Search Range: +- 32

Picture Structure: Field structure

Number of Frames: 60

The proposed method is compared with JM2.0.

# Results

- The results are as follows.

|  | QP | SNRY(db) | SNRCb(db) | SNRCr(db) | Bitrate (kbps) |
| --- | --- | --- | --- | --- | --- |
| IBBP |  |  |  |  |  |
| New Chroma Prediction | 16 | 40.62 | 42.42 | 43.15 | 7598 |
|  | 20 | 38.48 | 40.94 | 41.60 | 4744 |
|  | 24 | 36.15 | 39.05 | 39.69 | 2936 |
|  | 28 | 33.78 | 37.79 | 38.55 | 1950 |
| JM2.0 | 16 | 40.63 | 41.91 | 42.60 | 7732 |
|  | 20 | 38.41 | 40.46 | 41.16 | 4610 |
|  | 24 | 36.15 | 38.64 | 39.32 | 2956 |
|  | 28 | 33.67 | 37.50 | 38.21 | 1924 |
| All Intra |  |  |  |  |  |
| New Chroma Prediction | 16 | 42.54 | 43.87 | 44.55 | 13200 |
|  | 20 | 40.66 | 42.54 | 43.10 | 8744 |
|  | 24 | 38.65 | 40.87 | 41.40 | 5614 |
|  | 28 | 36.34 | 39.74 | 40.23 | 3596 |
| JM2.0 | 16 | 42.54 | 43.32 | 43.95 | 13446 |
|  | 20 | 40.62 | 42.01 | 42.62 | 8920 |
|  | 24 | 38.60 | 40.38 | 41.02 | 5718 |
|  | 28 | 36.24 | 39.31 | 39.93 | 3658 |

Table 1: PSNR and Bitrate

|  | BD-PSNR | BD-Bitrate |
| --- | --- | --- |
| IBBP |  |  |
| Y | -0.0037 db | -0.036 % |
| Cb | 0.40 db | -11.17 % |
| Cr | 0.38 db | -10.53 % |
| All Intra |  |  |
| Y | 0.14 db | -2.78 % |
| Cb | 0.56 db | -15.97 % |
| Cr | 0.49 db | -13.82 % |

Table 2: BD-PSNR and BD-SNR

Table 1 shows the comparison of PSNR and Bitrate. Same QP is used for both cases. Table 2 shows the BD-PSNR and BD-Bitrate. In case of Luma, BD-PSNR is improved 0.14db. In case of Chroma, BD-Bitrate is improved about 15% and BD-SNR is improved about 0.5 db.

The results are also compared subjectively. The block artifacts were reduced at flat area.

The results will be demonstrated in Klagenfurt by D1.

# Conclusions

This contribution reports the results of improved chroma spatial prediction for Intra coding. The test is performed using HDTV (1920x1080 30 Hz interlaced) materials. RD characteristics and the subjective quality are compared. It is verified that both RD characteristics and subjective quality were improved by the proposed method. It is very useful especially for high quality applications. Therefore, the improved chroma spatial prediction for Intra coding should be included in JVT.

(Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Sony Corp. |  |  |
| Mailing address | Gate City Osaki East Tower 20F 1-11-1, Osaki, Shinagawa-ku Tokyo, 141-0032 |  |  |
| Country | Japan |  |  |
| Contact person | Teruhiko Suzuki |  |  |
| Telephone | +81-3-5435-3669 |  |  |
| Fax | +81-3-5435-3891 |  |  |
| Email | teruhiko@av.crl.sony.co.jp |  |  |
| Place and date of submission | Tokyo, Japan on July 16th, 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Results of Improved Chroma Spatial Prediction for Intra Coding |  |
| Contribution number |  | D062 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
| X |  |  |  |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
| X | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: