| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 4th Meeting: Klagenfurt, Austria, 22-26 July, 2002 | Document: JVT-D086 Filename: JVT-D086.doc |
| --- | --- |

| Title: | MMCO for interlaced sequences |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Martin Schlockermann Lessingstr. 13 63322 Rödermark Germany Shinya Kadono 1006 Kadoma, Kadoma, Osaka 573-8501 Japan | Tel: Email: Tel: Email: | +49.6103.766-129 schlockermann@panasonic.de +81.6.6900-9689 kadono@drl.mei.co.jp |
| Source: | Panasonic European Laboratories GmbH |  |  |

_____________________________

# 1. Introduction

Reference picture buffering of the CD [1] describes two modes to buffer the currently decoded picture and thus decides how the multi-frame buffer will be modified after the current picture is decoded. The first mode called “Sliding Window” buffering mode and the second “Adaptive Memory Control” buffering mode. In “Sliding Window” mode the memory control is done field based in case of field processing. If in this case the current picture is the first field of a frame, then only the field of the same parity (top or bottom) will be marked as unused in the buffer, so that the second field of the current frame may still reference the other field of the largest relative index. In the “Adaptive Memory Control” mode all operations are done frame based, and the frame numbers referred to in memory management control operation commands are always frame numbers even if the current picture is a field picture.

# 2. Problems of MMCO

Current MMCO commands are not adopted for field processing or adaptive frame/field processing.

# 3. Proposed solution

To solve the problems described in section 2 we propose to use different processing for progressive and interlace sequences.

## 3.1 Progressive sequence

The buffering will be done as specified in the CD.

## 3.2 Interlaced sequence

### 3.2.1 Field mode

In field mode each field can be seen as one independent field picture and the MMCO commands have the following meanings:

| Code Number | Memory Management Control Operation | Associated Data Fields Following |
| --- | --- | --- |
| 0 | End memory_management_control_operation Loop | None (end of RPS layer) |
| 1 | Mark a Short-Term Field as “Unused” | difference_of_pic_nums |
| 2 | Mark a Long-Term Field as “Unused” | long_term_pic_index |
| 3 | Assign a Long-Term Index to a Field | difference_of_pic_nums and long_term_pic_index |
| 4 | Specify the Maximum Long-Term Field Index | max_long_term_pic_index_plus1 |
| 5 | Reset | None |

### 3.2.2 Adaptive frame / field mode

In adaptive frame / field mode each picture is stored as a frame. If the current picture is a frame picture the memory management control operation process one frame. (Identically to processing of progressive sequences). If the current picture is a field the MMCOs “Mark a short-term Picture as “Unused”” and “Mark a Long-Term Picture as “Unused”” process the field with the same parity as the current picture. If in this case the current picture is the first field of a frame, then only the field of the same parity (top or bottom) will be marked as unused in the buffer, as specified by the encoder. The other MMCOs “Assign a Long-Term Index to a Picture” and Specify the Maximum Long-Term Picture Index” are processing the whole frame-picture.

# 4. Proposed revised text

### 8.4.3	Reference picture buffering mode (ref_pic_buffering_mode)

ref_pic_buffering_mode specifies the buffering mode of the currently decoded picture picture and thus decides how the multi-frame buffer will be modified after the current picture is decoded. It follows an remapping_of_pic_nums_indicator “end loop” indication when the picture is not an Intra picture. It is the first element of the RPS layer if the picture is an Intra picture. The values for ref_pic_buffering_mode are defined as follows:

ref_pic_buffering_mode = = “0”:	Sliding Window,

ref_pic_buffering_mode = = “1”:	Adaptive Memory Control.

In the “Sliding Window” buffering mode, the current decoded picture shall be added to the buffer with default relative index 0, and any marking of pictures as “unused” in the buffer is performed automatically in a first-in-first-out fashion among the set of short-term pictures. In this case, if the buffer has sufficient “unused” capacity to store the current picture, no additional pictures shall be marked as “unused” in the buffer. If the buffer does not have sufficient “unused” capacity to store the current picture, the picture with the largest default relative index among the short-term pictures in the buffer shall be marked as “unused”. If in interlace case and adaptive frame / field processing the current picture is the first field of a frame, then only the field of the same parity (top or bottom) will be marked as unused in the buffer, so that the second field of the current frame may still reference the other field of the largest relative index.  In the sliding window buffering mode, no additional information is transmitted to control the buffer contents.

In the "Adaptive Memory Control" buffering mode, the encoder explicitly specifies any addition to the buffer or marking of data as “unused” in the buffer, and may also assign long-term indices to short-term pictures. The current picture and other pictures may be explicitly marked as “unused” in the buffer, as specified by the encoder. This buffering mode requires further information that is controlled by memory management control operation (memory_management_control_operation) parameters.

#### 8.4.3.1	Memory management control operation (memory_management_control_operation)

memory_management_control_operation is present only when ref_pic_buffering_mode indicates “Adaptive Memory Control”, and may occur multiple times if present.  It specifies a control operation to be applied to manage the multi-frame buffer memory.  The memory_management_control_operation parameter is followed by data necessary for the operation specified by the value of memory_management_control_operation, and then an additional memory_management_control_operation parameter follows – until the memory_management_control_operation value indicates the end of the list of such operations.  memory_management_control_operation commands do not affect the buffer contents or the decoding process for the decoding of the current frame – rather, they specify the necessary buffer status for the decoding of subsequent frames in the bitstream.  The values and control operations associated with memory_management_control_operation are defined in Table 8-9.

Note that memory_management_control_operation commands always indicate the same treatment for both fields of a frame, and the frame numbers referred to in memory_management_control_operation commands are always frame numbers even if the current picture is a field picture.

The indication of memory_management_control_operation depends on picture and processing structure. For progressive sequences MMCO commands always indicate a frame picture. For interlace sequences and field pictures MMCO commands always indicate a field picture. For interlace sequences and adaptive frame/field mode the MMCOs “Mark a Short-Term Picture as Unused” and “Mark a Long-Term Picture as “Unused” indicates the field with the same parity as the current field. If in this case the current picture is the first field of a frame, then only the field of the same parity (top or bottom) will be marked as unused in the buffer, so that the second field of the current frame may still reference the other field of the specified index. The MMCOs “Assign a Long-Term Index to a Picture” and “Specify the Maximum Long-Term Picture Index” indicate a frame picture.

If memory_management_control_operation is Reset, all frames in the multi-frame buffer (but not the current frame unless specified separately) shall be marked “unused” (including both short-term and long-term frames). Moreover, the maximum long-term frame index shall be reset to zero.

The frame height and width shall not change within the bitstream except within a picture containing a Reset memory_management_control_operation command.

A “stored picture” does not contain an memory_management_control_operation command in its RPS layer which marks that (entire) picture as “unused”.  If the current picture is not a stored picture, its RPS layer shall not contain any of the following types of memory_management_control_operation commands:

a)	A Reset memory_management_control_operation command,

b)	Any memory_management_control_operation command which marks any other picture (other than the current picture) as “unused” that has not also been marked as “unused” in the RPS layer of a prior stored picture, or

c)	Any memory_management_control_operation command which assigns a long-term index to a picture that has not also been assigned the same long-term index in the RPS layer of a prior stored picture

Table 8-9 – Memory management control operation (memory_management_control_operation) values

| Code Number | Memory Management Control Operation | Associated Data Fields Following |
| --- | --- | --- |
| 0 | End memory_management_control_operation Loop | None (end of RPS layer) |
| 1 | Mark a Short-Term Picture as “Unused” | difference_of_pic_nums |
| 2 | Mark a Long-Term Picture as “Unused” | long_term_pic_index |
| 3 | Assign a Long-Term Index to a Picture | difference_of_pic_nums and long_term_pic_index |
| 4 | Specify the Maximum Long-Term Picture Index | max_long_term_pic_index_plus1 |
| 5 | Reset | None |

# 5. Conclusion

This proposal introduce picture buffering of interlaced sequence for “Adaptive Memory Control”. It is a clean-up of current CD. Therefore we propose to incorporate this modification into FCD.

# 6. References

[1]	Thomas Wiegand, “Joint Committee Draft (CD) ”, JVT-C167, JVT Meeting, Fairfax, Virginia, USA, May 2002.

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Panasonic European Laboratories GmbH |  |  |
| Mailing address | Monzastr. 4c 63225 Langen |  |  |
| Country | Germany |  |  |
| Contact person | Martin Schlockermann |  |  |
| Telephone | +49.6103.766-129 |  |  |
| Fax | +49.6103.766-144 |  |  |
| Email | schlockermann@panasonic.de |  |  |
| Place and date of submission | 4th Meeting: Klagenfurt, Austria, 22-26 July, 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | MMCO for interlaced sequences |  |
| Contribution number |  | JVT-D086 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: