| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 4th Meeting: Klagenfurt, Austria, 22-26 July, 2002 | Document: JVT-D088 Filename: JVT-D088 |
| --- | --- |

| Title: | Verification for JVT-C038 (Bounding Arithmetic Coding Complexity) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Ali Tabatabai, Toby Walker 3300 Zanker Road San Jose, CA USA | Tel: Email: | 1-408-955-6267 ali.tabatabai@am.sony.com |
| Source: | Sony |  |  |

_____________________________

# Summary

We report the results of cross-verifying JVT-C038, bounding arithmetic coding complexity, using the implementation provided by NTT DoCoMo. The results match those reported in JVT-D114.

# Experimental Results

We used the implementation provided to us by NTT DoCoMo (The "JM19_new_CABAC_020603" version), which is based on the JM19_new_CABAC_020603 software developed by Detlev Marpe.. The simulations compared the results of this software with the use of the bounded complexity proposed in JVT-C038.

## Conditions

We ran the NTT DoCoMo software using the following conditions:

| RD Optimization | On |
| --- | --- |
| CABAC | On |
| Structure | IBBPBBPBBP |
| Reference Frames | 3 |
| Motion Compensation | ¼ Pel |

The software used a condition definition, CONSTRAINT_AC, to control enabling of the JVT-C038 modifications to the JM1.9 base software.

## Results

| Sequence | QP | #define CONSTRAINT_AC |  |  |  | //#define CONSTRAINT_AC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BR | SNRY | SNRU | SNRV | BR | SNRY | SNRU | SNRV |
| Container | 16 | 31.67 | 36.03 | 40.50 | 40.08 | 31.67 | 36.03 | 40.50 | 40.08 |
|  | 20 | 18.81 | 33.35 | 38.93 | 38.48 | 18.80 | 33.35 | 38.93 | 38.48 |
|  | 24 | 12.10 | 30.62 | 37.48 | 36.50 | 12.10 | 30.62 | 37.48 | 36.50 |
|  | 28 | 8.08 | 27.92 | 36.68 | 35.63 | 8.06 | 27.92 | 36.68 | 35.63 |
| Foreman | 16 | 116.42 | 35.42 | 38.84 | 39.96 | 116.42 | 35.42 | 38.84 | 39.96 |
|  | 20 | 64.01 | 32.85 | 37.77 | 38.56 | 64.01 | 32.85 | 37.77 | 38.56 |
|  | 24 | 36.85 | 30.40 | 36.59 | 37.04 | 36.85 | 30.40 | 36.59 | 37.04 |
|  | 28 | 22.26 | 28.07 | 35.95 | 36.12 | 22.26 | 28.07 | 35.95 | 36.12 |
| News | 16 | 69.95 | 36.80 | 39.24 | 39.73 | 69.95 | 36.80 | 39.24 | 39.73 |
|  | 20 | 43.42 | 33.75 | 37.64 | 38.36 | 43.42 | 33.75 | 37.64 | 38.36 |
|  | 24 | 25.56 | 30.90 | 35.72 | 36.58 | 25.56 | 30.90 | 35.72 | 36.58 |
|  | 28 | 15.33 | 28.13 | 34.85 | 35.63 | 15.33 | 28.13 | 34.85 | 35.63 |
| Silent | 16 | 76.04 | 35.78 | 38.33 | 39.15 | 76.04 | 35.78 | 38.33 | 39.15 |
|  | 20 | 45.44 | 33.06 | 36.65 | 37.99 | 45.44 | 33.06 | 36.65 | 37.99 |
|  | 24 | 26.18 | 30.50 | 34.95 | 36.90 | 26.18 | 30.50 | 34.95 | 36.90 |
|  | 28 | 14.85 | 27.93 | 34.19 | 36.15 | 14.85 | 27.93 | 34.19 | 36.15 |
| Mobile | 16 | 1223.94 | 33.88 | 35.20 | 35.10 | 1223.94 | 33.88 | 35.20 | 35.10 |
|  | 20 | 603.31 | 30.64 | 33.40 | 33.20 | 603.31 | 30.64 | 33.40 | 33.20 |
|  | 24 | 277.21 | 27.65 | 31.52 | 31.22 | 277.21 | 27.65 | 31.52 | 31.22 |
|  | 28 | 139.92 | 24.88 | 30.46 | 30.05 | 139.92 | 24.88 | 30.46 | 30.05 |

# Conclusion

The results here show that, for the conditions and simulations reported here, there were no significant differences in bit rates when JVT-C038 was used.