| Joint Video Team (JVT) of ISO/IEC MPEG and ITU-T VCEG 4th Meeting: Klagenfurt, Austria, 22-26 July, 2002 | Document JVT-D090 File: JVT-D090.doc Generated: 2002-06-10 |
| --- | --- |

| Title: | Multi-frame interpolative prediction in AFF |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Krit Panusopone and Limin Wang Motorola, Inc. Broadband Communications Sector 6450 Sequence Drive San Diego, CA 92121 | Tel: Email: | 858-404-3618 krit@motorola.com |
| Source: | Motorola, Inc., BCS |  |  |

Abstract

This documentation proposes a coding rule for multi-frame interpolative prediction mode for frame/field picture in AFF coding mode.

Introduction

In JVT CD [1], a B picture (frame or field) is allowed to have two reference pictures (frame or field). The two reference pictures can be both forward (Fig. 1), both backward (Fig. 2), or one forward and one backward (Fig. 3). The prediction signal is a linear interpolation of motion compensated signals as

pred=w1ref1+w2ref2+d	(1)

where ref1 and ref2 are the two reference pictures, and w1 and w2are the weighting factors. d is set to 0 by default. The linear interpolation coefficients, w1,w2,dcan be determined explicitly at the macroblock layer. ref1 is the reference picture closer to the B picture in terms of temporal distance if both ref1 and ref2 are forward, or backward, reference pictures.  For bi-directional, ref1 and ref2 are respectively forward and backward reference pictures.

Both MVs of MFIP MB are coded relative to each other. MV of ref2, MV, for this MB is generated by adding offset DMV to the scaled MV of ref1, MV1, or

MV2=TR2×MV1TR1+DMV	(2)

where TR1 and TR2 are temporal distance between the current picture and the nearest reference picture,  ref1 and the farthest reference picture, ref2, respectively.

The proposed scheme

Multi-frame interpolative prediction mode (MFIP) [2] is a general frame interpolative prediction framework. This document proposes a description for MFIP in field picture. With adaptive switch of frame/field coding at picture level [3], B frame can be coded as one B frame picture or two B field pictures. Rules for handling MFIP in field coding mode (both the current B and its reference pictures are in field structure) are given below:

- Equation 1 is used to generate prediction signal. However, ref1 and ref2 are the fields that are indexed by reference field numbers, ref_idx_fwd and ref_idx_bwd. Notice that ref1 and ref2 can be either top or bottom field. The default weighting factors, w1 and w2, remain the same (½, ½, 0) and (2, -1, 0).

- Equation 2 is used to generate MV. Since both reference pictures are in field structure, TR1 and TR2 is determined based on temporal distance between reference and the current fields.

- Code number for reference field number, ref_idx_fwd and ref_idx_bwd, in MFIP mode follow the normal convention for field picture.

Summary

This document proposes description for MFIP.

Fig. 1. Both reference pictures are forward.

Fig. 2 Both reference pictures are backward.

Fig. 3. One reference picture is forward and another backward.

Reference

- T. Wiegand, “Joint Committee Draft (CD)”, May, 2002.

- Y. Kikuchi, T. Chujoh, and S. Kadono, “Multi-frame interpolative prediction with modified syntax”, JVT-C066, Mar., 2002.

- L. Wang, K. Panusopone, R. Gandhi, Y. Yue and A. Luthra, “Adaptive frame/field coding for JVT”, JVT-B071, Jan., 2002.

- (Append for Proposal Documents)

# JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Motorola Inc. |  |  |
| Mailing address | # 6450 Sequence Drive San Diego, CA 92121 |  |  |
| Country | U.S.A. |  |  |
| Contact person | Limin Wang |  |  |
| Telephone | 858-404-3893 |  |  |
| Fax | 858-404-2501 |  |  |
| Email | liwang@gi.com |  |  |
| Place and date of submission | San Diego, CA., USA, 06/10/02 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Multi-frame interpolative prediction for field picture coding |  |
| Contribution number |  | JVT-D090 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
|  | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| Without having conducted a search or investigation, the submitter/author(s) individually believe(s) they or General Instrument Corporation (Patent Holder) may have granted, pending or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | - The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. see comments below regarding Baseline Profile |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  | Comment to Section 2.2. To support the proposed royalty free JVT Baseline Profile, and in addition to the statement of 2.2 above, Patent Holder is also willing to grant a royalty free license to patents essential to JVT Baseline Profile decoder applications on the condition that any and all other patent holders do the same. |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
|  | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  |  |  |  |
| Mailing address |  |  |  |  |
| Country |  |  |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: