| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 4th Meeting: Klagenfurt, Austria, 22-26 July, 2002 | Document: JVT-D120 Filename: JVT-D120.doc |
| --- | --- |

| Title: | Verification result of JVT-C084 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Chul-Woo Kim Rm. 406 Korea Design Center 344-1 Yatap, Bundang, Seongnam Seoul Korea | Tel: Email: | +82-31-788-7652 charlie@mcubeworks.com |
| Source: | McubeWorks Inc. |  |  |

_____________________________

- Introduction

This contribution is to verify the result of JVT-C084. This contribution analyzed the optimality of Lambda value which is currently used in JCD. It demonstrates that the optimal lambda can be replaced by 0.68 rather than the 0.85 which is currently used in the JM 2.0. Verification results showed that the lambda value of 0.68 gives slightly better performance at QCIF sequences in common conditions while slightly inferior result in CIF sequences. Average BD-BRS is about 0.6 % (0.03 dB PSNR gain) on common conditions. Simulation was done according to the conditions described in common conditions (VCEG-N71).

- Verification Result

Verification results when the value of Lambda = 0.68 were tested in comparison with Lambda = 0.85 which is currently described in JCD. BD-PSNR and BD-BRS values are obtained as in Table 1. Detailed results can be found in JVT-D120.xls

Table 1. Verification result of JVT-C084

| Sequences | L=0.68 vs. JM 2.0 (L=0.85) |  |
| --- | --- | --- |
|  | BD-PSNR [dB] | BD-BRS [%] |
| Foreman [QCIF] | 0.0414 | 0.7625 |
| News [QCIF] | 0.0777 | 1.6752 |
| Container [QCIF] | 0.0079 | 0.1163 |
| Silent [QCIF] | 0.0552 | 1.4208 |
| Paris [QCIF] | 0.0321 | 0.6471 |
| Mobile [CIF] | -0.0021 | -0.0425 |
| Tempete [CIF] | -0.0144 | -0.3455 |
| Average | 0.0283 | 0.6048 |

* (+) values denote the performance gain