| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 4th Meeting: Klagenfurt, Austria, 22-26 July, 2002 | Document: JVT-D137 Filename: JVT-D137.doc |
| --- | --- |

| Title: | Improved Text for SP pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | BTexact Technologies Adastral Park, Martlesham Heath Ipswich, IP5 3RE, UK Nokia Inc. 6000 Connection Drive Irving, TX 75039 | Email: | Mike.Nilsson@bt.com Ragip.Kurceren@nokia.com |
| Source: | BTexact Technologies, Nokia |  |  |

_____________________________

# Summary

This contribution proposes improved text for SP pictures for the FCD, and can be considered to be addressing bug fixes for both the CD and the reference software. No new technical ideas are proposed.

# Bugs in the reference software

## Coding of RUN

The reference software does not code RUN in SP pictures. This is incorrect, as macroblocks can be skipped in SP pictures, and so RUN should be coded in SP pictures.

## SIntra4x4 in SP pictures

Table 9.4, which defines macroblock types for Inter pictures, is also stated to apply to SP pictures. However, the software implements an SIntra4x4 mode in position 0, with all other macroblock types moved down one place.

There is no need for SIntra4x4 in SP pictures, hence the CD text is correct and the software is not.

## 8x8 macroblocks with intra regions

The CD does not give any information about 8x8 macroblocks in SP pictures in which one or more regions is intra. The behaviour of the reference software in this case is strange. Chroma is treated as if the whole macroblock were SP, that is, the prediction is transformed and added to decoded inverse quantised levels. Luma in the intra region is treated as ordinary intra, as if the whole macroblock were 4x4 intra coded.

It is not possible to achieve an exact match between SP and SI pictures when the SP picture is encoded with 8x8 macroblocks with intra regions, as SI pictures do not permit macroblocks with a mixture of Intra4x4 and SIntra4x4.

Consequently, the proposed solution is to disallow 8x8 macroblocks in SP pictures in which one or more regions is intra. This allows the strange code to be removed from the reference software and allows exact matches to be achieved between SP and SI pictures.

# Bugs in the CD text

## Coding of RUN

The CD syntax indicates that RUN is encoded in SI and SP pictures. This is incorrect, as RUN is not required in SI pictures.

## Syntax dependent on coding_type

There are many locations in the syntax and semantics where behaviour depends on coding_type. In most cases, this has not been extended to allow for SI and SP pictures, and in general what applies to Intra also applies to SI, and what applies to Inter also applies to SP.

## 8x8 macroblocks with intra regions

For details of the problem, see section 2.3. The solution is to disallow 8x8 macroblocks in SP pictures in which one or more regions is intra.

## Figure 12-1: the architecture diagram

Figure 12-1 is inconsistent with the text, in that it uses different variable names. Lerr == YQERR; Kpred == YPRED; Lrec == YQREC; Derr, Crec, Drec, R(x,y) and P(x,y) are not used in the text. It is a little misleading, suggesting that all macroblocks of S pictures are decoded as shown. It should be made clear that the diagram does not apply to Intra4x4 or Intra16x16.

## Quantisation equations

Some of the equations in the CD text are not the same as in the reference software, and are clearly wrong.

Equation 11-2 is incorrect: it should use the forward quantisation matrix rather than the inverse quantisation matrix Q(QP%6,i,j). Q needs to be defined for forward quantisation, as in the Test Model description of encoding; and f needs to be defined.

An equation similar to 11-2 for chroma would be useful. In particular, there is an extra factor of two in the inverse quantisation of Lerr (== Yqerr) for DC coefficients that could be easily missed by an implementer.

## Switching stream SI and SP pictures

There is no architecture diagram or description for the decoding of switching stream SI and SP pictures despite this being different to decoding of play stream SI and SP pictures, and despite a flag, sp_for_switch_flag, being present in the bitstream to distinguish these two types of picture.

The method for decoding switching stream SI and SP pictures is described in JVT-C114 section 3.

## Loop filter

The CD text is unclear about the operation of the loop filter. In order to ensure that switching stream and play stream SP and SI pictures can be reconstructed identically, the ordinary quantiser index, rather than the SP quantiser index, is used in the loop filter calculations.

# Improved text for the FCD

The attached document provides improved text for the FCD relating to SP and SI pictures. It is a cut down version of changes relative to D015d5 - all of the bits that are not changed are not there.

The text is written to correspond to the reference software with the bugs described in section 2 fixed. The reference software has been verified, as described in section 6, to be fit for purpose, that is, play stream SP pictures can be decoded identically to switching stream SP pictures.

# Remaining Issues

## SI pictures in play streams

SI pictures were introduced to allow random access into a play stream, providing an exact match for a reconstructed SP picture in the play stream. SI pictures were therefore introduced for use in switching streams.

It is not clear that there is any benefit from having SI pictures in a play stream.

However, the syntax in the CD does not allow sp_for_switch_flag to be indicated for SI pictures, and so limits SI pictures to only occur in switching streams.

Hence a change to the text is needed, either to allow sp_for_switch_flag to be indicated for SI pictures, or to clean up the text of section 12: figure 12-1 would need to have all the intra stuff removed from it, and the wording of 12.2.1 would need tidying up.

## Division in the inverse quantisation

Equation 11-2 in the CD text and equation 12-2 in the attached improved text use division by quantisation numbers to perform inverse quantisation. It would be beneficial if the need for this division could be removed.

Some study is encouraged to determine whether this division could be replaced by multiplication by inverse quantisation numbers.

# Verification

The authors can report that software has implemented to verify the consistency of the proposed text and the reference software.

The independent software, implemented as described in the attached text, can decode bitstreams with SP pictures produced by the reference software, with an exact match of reconstructed pictures. Note this has required implementation in the independent software of the bugs in the reference software as described in section 2.

Independent software has also been developed to take two bitstreams with SP pictures produced by the reference software, and produce one bitstream, the first part of which come from one of the original streams and the second part of which comes from the other, with the link being made by the creation of a switching stream SP picture, implemented as in the attached text. The independent software decoded the resulting single bitstream to create reconstructed pictures that exactly match the corresponding reconstructed pictures of the original two bitstreams.

END