| Joint Video Team (JVT) of ISO/IEC MPEG & ITU-T VCEG (ISO/IEC JTC1/SC29/WG11 and ITU-T SG16 Q.6) 5th Meeting: Geneva, CH, 9-17 October, 2002 | Document: JVT-E025 Filename: JVT-E025.doc |
| --- | --- |

| Title: | Clean-up and improved design consistency for ABT |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVT |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mathias Wien Institut und Lehrstuhl für Nachrichtentechnik RWTH Aachen 52056 Aachen, Germany | Tel: Email: | ++49-241-80-27681 wien@ient.rwth-aachen.de |
| Source: | Aachen University |  |  |

_____________________________

# Introduction

In this document some clean-up and improved design consistency issues are discussed and some simplifications are proposed.

The ABT direct and inverse transform and quantization process as they are used in the JM and specified in the FCD is analyzed. The analysis proves the consistency with the 16-bit design criteria as they were specified by the transform complexity AdHoc group [VCEG-O09].

The syntax for ABT intra block in I slices and P and B slices is harmonized and the specification of the intra prediction modes is modified to make directional intra prediction in blocks of size 4x4, 4x8, 8x4, and 8x8 mode consistent.

Teruhiko Suzuki from Sony Corp pointed out an intra performance problem with the current specification of ABT intra prediction. This problem became obvious especially on HD material in smooth regions. In this document, the application of the base line Intra16x16 in addition to the existing ABT intra block modes is proposed. Introducing Intra16x16 in addition to the ABT intra modes solves this issue at a minimum change to the document as well as to the software, while making the design more consistent with the non-ABT case.

The document is organized as follows. In the first part, the dynamic range analysis for the ABT transform and quantization is given. In the second part, the proposal for the modified intra syntax and intra prediction is presented.

# Dynamic range analysis for ABT transform and quantization

In the AdHoc report of the AHG on transform and quantization complexity reduction at the Pattaya meeting, the conditions for compliant transform and quantization processes were defined [VCEG-O09]. The main issues are

- 16-bit memory for input, intermediate and output

- Arithmetic logic unit supporting 32-bit operations

- 16-bit multiplier.

These constraints were defined for 9-bit residuals.

## Assuming a normalized direct transform

The normalized transform matrix T8 as used in the JM approximates the 8x8 DCT matrix. Before looking at T8, we briefly highlight the dynamics of the DCT. The maximum dynamic increase of the transform is given by

where tij is the (DCT) matrix coefficient at position i in row j. For the DCT, the maximum value is reached for j = 0 (DC) and j = 4. The coefficients of the DC base function are constant and have a value of  .

That means, the dynamic range of a 9 bit input signal is expanded by for each 1D transform. Hence, assuming a quantization of step size 1 in the transform domain, the 2D transform expands the dynamic by a factor of 8, i.e. 3 bit. Therewith, assuming a quantization step size of 1 in the transform domain, the dynamic range of the signal at the encoder side as well as the dynamic range of the re-scaled signal at the decoder side is 12 bit.

## Analysis for the JM 8x8 transform and quantization

In the following, the dynamic range of the forward and inverse transform and quantization process as implemented in the JM software is analyzed.

The 8x8 transform matrix used in the JM is given as

### Direct and inverse transform and quantization process

For an input signal X of size 8x8 samples, the transform and quantization process can be written as follows:

Here, '//' denotes scaling by a factor n ( xq = sign(x)[abs(x) + 2n-1]>>n ).  E contains the transformed and quantized coefficients for the 8x8 block X. The quantization factor is denoted as q. TT is the transposed matrix T. Since the encoder is not specified by the standard, we do not focus on realization of the encoder side in 16 bit dynamic. At the decoder side, the following inverse transform and quantization operations are specified:

After inverse scaling with the inverse scaling factor r and the horizontal inverse transform a first bit shift is introduced. J is the reconstructed signal X.

The forward and inverse scaling factors q and r used for quantization in the FCD double every 6 QP. Therefore, the scaling factors can be separated into a mantissa and an appropriate power of 2. The forward and inverse scaling factors are

Here, QP/6 and QP%6 denote the integer division of Qp and the modulo operation by 6, respectively. (For simplicity reasons we use QP = QPFCD –12 here).  In the FCD, the mantissa tables Q and R are given as

Since the exponent does not influence the dynamic range of the transform process after inverse scaling, it is sufficient to consider the quantization / inverse quantization process for exponent 0.

### Quantizer design

In the JM, the following shift factors ni , i = 0,1,2,3 are used

The squared norm of T8 is . The scaling and inverse scaling factors hold following condition:

For k = 0,1,...,5, and with ni , i = 0,1,2,3, and N8 from above, this gives, in a slight abuse of notation

.

From this property of Q and R it can be concluded, that it is sufficient to analyze the dynamic range for only one scaling / inverse scaling factor pair.

### Dynamic Range

In the following, we give the analysis for Q0 = 2506 and R0 = 15.

The forward and inverse transform can be written as follows

where and .

Applying the transform T8 in one direction boosts the dynamic of the transformed signal by a factor of , hence 6.7 bits are added to the dynamic range. Since, and the dynamic is increased by a factor of  1042 in , only a factor of

is induced by the application of both, the horizontal and vertical inverse transform.

The dynamic range of the outcome of the equations from  and  is given in the table below. It is assumed that the input signal X has a dynamic range of 9 bit.

| Output | dynamic of operation | resulting dynamic [bit] |
| --- | --- | --- |
| X | - | 9 |
| A = T8 X | 9 + 6.7 | 15.7 (16) |
| B = A T8' | 15.7 + 6.7 | 22.4 (23) |
| C = B >> n0 | 22.4 –4 | 18.4 (19) |
| D = q C | 18.4 + 11.3 | 29.7 (30) |
| E = D >> n1 | 29.7 – 19 | 10.7 (11) |
| F = r E | 10.7 + 3.9 | 14.6 (15) |
| G = F T8 | 14.6 + 3.7 | 18.3 (19) |
| H = G >> n2 | 18.3 – 7 | 11.3 (12) |
| I = T8' H | 11.3 + 3.7 | 15.0 (15) |
| J = I >> n3 | 15-6 | 9 |

In order to be compliant with the constraints listed above, all multiplications and memory accesses on the decoder side (F to J) need to be in 16-bit.

- F results from a 16-bit multiplication, the result (that might be stored) does not exceed 16-bit.

- G is derived by adding the results of 16-bit multiplies. The summation might exceed 16-bit but the result is shifted down into the 16-bit range in H before storing to memory.

- I and J stay in the 16-bit range. During the calculation of I, the summation might exceed 16-bit but the result that needs to be stored is reduced to fit into 16-bit dynamics.

## Analysis for the JM 8x4 and 4x8 transform and quantization

The analysis can be given for 4x8 and 8x4 blocks as well.

The JM 4x4 transform and inverse transform matrices are given as

The even basis functions increase the dynamic range of the transformed signal by a factor of 4 (2 bit), while the odd basis functions increase the dynamic by a factor of 6 (2.6 bit). Following the argumentation from equation  onward, the inverse transform does not change the dynamic range of the (already transformed) signal for even base functions. It can be seen, that for odd base functions, the dynamic is reduced by a factor of 5/6 (-0.3 bit).

Due to the variable norm, the analysis has to be done separately for even and odd columns or rows of the 8x4 and 4x8 blocks, respectively. The scaling and inverse scaling factors for these blocks are given as

.

The values for the even rows or columns are given in the first column of Q and R and the values for the odd rows or columns are given in the second column of Q and R, respectively. Again, the first quantizer pair is evaluated. Q = 11030 corresponds to a dynamic increase of 13.4 bit while Q = 7220 gives an increase of 12.8 bit. The inverse scaling factors increase the dynamic by factors of 9 and 11, i.e. 3.2 and 3.5 bit.

### Dynamic Range

For the 4x8 transform:

| Output | dynamic of operation |  | resulting dynamic [bit] |  |
| --- | --- | --- | --- | --- |
| ##### X | even row | odd row | even row | odd row |
| A = T8 X | 9 + 6.7 | 9 + 6.7 | 15.7 | 15.7 |
| B = A T4' | 15.7 + 2 | 15.7 + 2.6 | 17.7 | 18.3 |
| C = B >> n0 | 17.7 – 2 | 18.3 – 4 | 15.7 | 14,3 |
| D = q C | 15.7 + 13.4 | 14.3 + 12.8 | 29.1 | 27.1 |
| E = D >> n1 | 29.1 – 19 | 27.1 – 17 | 10.1 | 10.1 |
| F = r E | 10.1 + 3.2 | 10.1 + 3.5 | 13.3 | 13.6 |
| G = F Tinv4 | 13.3 + 0 | 13.6 – 0.3 | 13.3 | 13.3 |
| H = G >> n2 | 13.3 – 2 | 13.3 – 2 | 11.3 | 11.3 |
| I = T8' H | 11.3 + 3.7 | 11.3 + 3.7 | 15.0 | 15.0 |
| J = I >> n3 | 15.0 - 6 | 15.0 - 6 | 9.0 | 9.0 |

For the 8x4 transform:

| Output | dynamic of operation |  | resulting dynamic [bit] |  |
| --- | --- | --- | --- | --- |
| ##### X | even column | odd column | even column | odd column |
| A = T4 X | 9 + 2 | 9 + 2.6 | 11 | 11.6 |
| C = B T8' | 11.0 + 6.7 | 11.6 + 6.7 | 17.6 | 18.3 |
| B = A >> n0 | 17.6 - 2 | 18.3 - 4 | 15.6 | 14.3 |
| D = q C | 15.6 + 13.4 | 14.3 + 12.8 | 29.0 | 27.1 |
| E = D >> n1 | 29.0 – 19.0 | 27.1 – 17.0 | 10.0 | 10.1 |
| F = r E | 10.0 + 3.2 | 10.1 + 3.5 | 13.2 | 13.6 |
| G = F T8 | 13.2 + 3.7 | 13.6 + 3.7 | 16.9 | 17.3 |
| H = G >> n2 | 16.9 - 2 | 17.3 – 2 | 14.9 | 15.3 |
| I = Tinv4' H | 14.9 - 0.3 | 15.3 – 0.3 | 14.6 (15) | 15.0 (15) |
| J = I >> n3 | 14.6 - 6 | 15.0 - 9 | 8.6 | 9.0 |

As can be seen by applying the argumentation from the 8x8 blocks, both 4x8 and 8x4 suffice the 16-bit memory and multiplication constraints.

# Intra16x16 with ABT

The accompanying modified version of JVT-E022d7 contains the changes to the specification to allow for the Intra16x16 mode to be used when ABT is on.

The changes are

- The indication of the ABT intra block types via  ABTIntra mb_types is removed

- mb_type Intra_4x4 indicates usage of ABT intra for I, P, and B slices when MbABTFlag is 1

- Intra_16x16 can be chosen also for I slices

- intra_block_typeABT is signaled for I, P, and B slices to indicated the ABT intra block type

The missing possibility of having a 16x16 based prediction in the FCD ABT intra specification introduced some weird artifacts, especially on intra coded HD material (thanks again to Teruhiko Suzuki who pointed that out). Allowing for Intra_16x16 in addition to intra coding with the ABT intra block types solves this problem and clearly improves the overall RD performance for ABT Intra on this material. An example for the improved subjective performance is given in the figures below. RD results are given at the end of this document.

(a) Original part of the ShuttleStart sequence		(b) Encoded with JM42 ABT on, QP36

(c) Encoded with JM42 ABT off, QP36	      (d) Encoded with JM42 ABT on +Intra16x16, QP36

Figure 1: Intra coding: Detail of the first frame of the ShuttleStart sequence.

# Harmonized intra prediction modes

For NxM blocks, a pre-filtered prediction is employed. In the FCD, the intra prediction for ABT NxM blocks slightly differed from the specification of the intra prediction for 4x4 blocks. This is mainly due to differences in the rounding operation in the prediction step. Here, we propose to modify the ABT intra prediction make the design for all block sizes more consistent. For this purpose, the intra prediction as specified for 4x4 blocks are extended directly for the most prediction modes. The DC prediction remains unchanged from the FCD.

The accompanying modified version of JVT-E022d7] contains the changes to the specification for the proposed intra prediction in sub clause 12.4.1. Besides prediction modes 0 (vertical), 1 (horizontal), and 7 (vertical right), the base line prediction technique is extended for the larger block sizes. For each of these modes, the extended 4x4 prediction results in almost exactly the same rate / distortion performance as the corresponding intra prediction technique specified in the FCD.

For vertical and horizontal prediction on 4x4 blocks no filtering is applied to the prediction samples. It turned out that for 8x8 blocks filtering of the prediction samples improves the prediction performance compared to the non-filtered prediction. Therefore, filtering is applied for vertical and horizontal prediction on 8x8 blocks. Blocks of size 4x8 and 8x4 use non-filtered prediction.

For 4x8, 8x4, and 8x8 blocks, the vertical right prediction as specified for 4x4 blocks does not perform as well as the vertical right prediction as specified in sub clause 12.4.1.8 of the FCD. Here, the rounding is modified from

( ( ti + ti+1<<1 + ti+2 + 2 ) >> 2  +  ( ti+1 + ti+2<<1 + ti+3 + 2 ) >> 2 ) >> 1

to

( ti + 3ti+1 + 3ti+2 + ti+3 + 4 ) >> 3

in order to reduce the number of operations.

# Simulation Results

[will be given in version r1 of the document]

- (Append for Proposal Documents)

- JVT Patent Disclosure Form

| International Telecommunication Union Telecommunication Standardization Sector | International Organization for Standardization | International Electrotechnical Commission |
| --- | --- | --- |

Joint Video Coding Experts Group - Patent Disclosure Form

(Typically one per contribution and one per Standard | Recommendation)

Please send to:

JVT Rapporteur Gary Sullivan, Microsoft Corp., One Microsoft Way, Bldg. 9, Redmond WA 98052-6399, USA

Email (preferred): Gary.Sullivan@itu.int  Fax: +1 425 706 7329 (+1 425 70MSFAX)

This form provides the ITU-T | ISO/IEC Joint Video Coding Experts Group (JVT) with information about the patent status of techniques used in or proposed for incorporation in a Recommendation | Standard.  JVT requires that all technical contributions be accompanied with this form. Anyone with knowledge of any patent affecting the use of JVT work, of their own or of any other entity (“third parties”), is strongly encouraged to submit this form as well.

This information will be maintained in a “living list” by JVT during the progress of their work, on a best effort basis.  If a given technical proposal is not incorporated in a Recommendation | Standard, the relevant patent information will be removed from the “living list”.  The intent is that the JVT experts should know in advance of any patent issues with particular proposals or techniques, so that these may be addressed well before final approval.

This is not a binding legal document; it is provided to JVT for information only, on a best effort, good faith basis.  Please submit corrected or updated forms if your knowledge or situation changes.

This form is not a substitute for the ITU ISO IEC Patent Statement and Licensing Declaration, which should be submitted by Patent Holders to the ITU TSB Director and ISO Secretary General before final approval.

| Submitting Organization or Person: |  |  |  |
| --- | --- | --- | --- |
| Organization name | Aachen University |  |  |
| Mailing address | Institut für Nachrichtentechnik RWTH Aachen 53056 Aachen |  |  |
| Country | Germany |  |  |
| Contact person | Mathias Wien |  |  |
| Telephone | ++49-241-80-27681 |  |  |
| Fax | ++49-241-80-22196 |  |  |
| Email | wien@ient.rwth-aachen.de |  |  |
| Place and date of submission | Geneva, October 2002 |  |  |
| Relevant Recommendation ¦ Standard and, if applicable, Contribution: |  |  |  |
| Name (ex: “JVT”) |  | JVT |  |
| Title |  | Clean-up and improved design consistency for ABT |  |
| Contribution number |  | JVT-E025 |  |

(Form continues on next page)

| Disclosure information – Submitting Organization/Person (choose one box) |  |  |  |
| --- | --- | --- | --- |
| x | 2.0 The submitter is not aware of having any granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. or, |  |  |
| The submitter (Patent Holder) has granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. In which case, |  |  |  |
|  | 2.1 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a free license to an unrestricted number of applicants on a worldwide, non-discriminatory basis to manufacture, use and/or sell implementations of the above Recommendation ¦ Standard. |  |  |
|  | 2.2 The Patent Holder is prepared to grant – on the basis of reciprocity for the above Recommendation ¦ Standard – a license to an unrestricted number of applicants on a worldwide, non-discriminatory basis and on reasonable terms and conditions to manufacture, use and/ or sell implementations of the above Recommendation ¦ Standard. Such negotiations are left to the parties concerned and are performed outside the ITU ¦ ISO/IEC. |  |  |
|  | 2.2.1 The same as box 2.2 above, but in addition the Patent Holder is prepared to grant a “royalty-free” license to anyone on condition that all other patent holders do the same. |  |  |
|  | 2.3 The Patent Holder is unwilling to grant licenses according to the provisions of either 2.1, 2.2, or 2.2.1 above. In this case, the following information must be provided as part of this declaration: - patent registration/application number; - an indication of which portions of the Recommendation ¦ Standard are affected. - a description of the patent claims covering the Recommendation ¦ Standard; |  |  |
| In the case of any box other than 2.0 above, please provide the following: |  |  |  |
| Patent number(s)/status |  |  |  |
| Inventor(s)/Assignee(s) |  |  |  |
| Relevance to JVT |  |  |  |
| Any other remarks: |  |  |  |
| (please provide attachments if more space is needed) |  |  |  |

(form continues on next page)

Third party patent information – fill in based on your best knowledge of relevant patents granted, pending, or planned by other people or by organizations other than your own.

| Disclosure information – Third Party Patents (choose one box) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | 3.1 The submitter is not aware of any granted, pending, or planned patents held by third parties associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| x | 3.2 The submitter believes third parties may have granted, pending, or planned patents associated with the technical content of the Recommendation ¦ Standard or Contribution. |  |  |  |
| For box 3.2, please provide as much information as is known (provide attachments if more space needed) - JVT will attempt to contact third parties to obtain more information: |  |  |  |  |
| 3rd party name(s) |  | Robert Bosch GmbH |  |  |
| Mailing address |  | PO Box 106050 70049 Stuttgart |  |  |
| Country |  | Germany |  |  |
| Contact person |  |  |  |  |
| Telephone |  |  |  |  |
| Fax |  |  |  |  |
| Email |  |  |  |  |
| Patent number/status |  |  |  |  |
| Inventor/Assignee |  |  |  |  |
| Relevance to JVT |  |  |  |  |

Any other comments or remarks: